3GPP TSG-RAN WG2 Meeting #108	R2-1916230

Reno, USA, November 14th - 18th 2019	Revision of R2-1913622

Agenda item:	6.7.2.2.1

Source: 	Sequans Communications

Title: 	CG/SPS alignment on Resource Boundary Pattern

Document for:	Discussion and Decision

# Introduction

During RAN2#107bis, enhancements to CG to handle misalignment between traffic periodicity and CG periodicity were discussed. There was some support in specifying a pattern such as only the needed resources are configured. In this contribution, we detail further how this can be done in a simple way.

# Discussion

We aim to define a quasi-periodic pattern as in blue in R2-1913622, matching arbitrary periodicities with a given latency. There are several options to realize such pattern. In our view, the existing CG/SPS formula could be easily enhanced such as it selects resources from a boundary resource pattern (used for alignment), as follows.

Baseline formula

It was already agreed that CG periodicities no longer dividing the hyperframe length will be supported. The baseline formula should be the one from LTE, as follows:

After an uplink grant is configured for a configured grant Type 2, the MAC entity shall consider sequentially that the Nth uplink grant occurs associated with the symbol for which:

[(SFN × numberOfSlotsPerFrame × numberOfSymbolsPerSlot) + (slot number in the frame × numberOfSymbolsPerSlot) + symbol number in the slot] = 
[Ref_symbol in HF + N × periodicity] modulo (1024 × numberOfSlotsPerFrame × numberOfSymbolsPerSlot)

where

- 	Ref_symbol in HF: SFNstart time × numberOfSlotsPerFrame × numberOfSymbolsPerSlot + slotstart time × numberOfSymbolsPerSlot + symbolstart time

-	periodicity = 2, 7, n*14, where n={1, 2, 4, 5, 8, 10, 16, 20, 32, 40, 64, 80, 128, 160, 256, 320, 512, 640, 1024, 1280, 2560, 5120} in symbols.

Generalized formula

It is proposed that periodicity can be extended to better match traffic periodicities, e.g. periodicities such as 104Hz or 60Hz. Periodicity would then be a rational number of symbols.

The formula can be modified to select the closest resource from an allowed resource boundary pattern. The resource boundary periodicity allows fulfilling a required latency, while the resource boundary periodicity offset allows to time multiplex CG configurations.

After an uplink grant is configured for a configured grant Type 2, the MAC entity shall consider sequentially that the Nth uplink grant occurs associated with the symbol for which:

[(SFN × numberOfSlotsPerFrame × numberOfSymbolsPerSlot) + (slot number in the frame × numberOfSymbolsPerSlot) + symbol number in the slot] = 
[ Ref_symbol in HF+N × periodicity - RoffsetRperiodicity×RPeriodicity+Roffset] modulo (1024 × numberOfSlotsPerFrame × numberOfSymbolsPerSlot)

Where

-	Ref_symbol in HF: SFNstart time × numberOfSlotsPerFrame × numberOfSymbolsPerSlot + slotstart time × numberOfSymbolsPerSlot + symbolstart time

-	periodicity: CG averaged periodicity, extended to rational number of symbols

-	Rperiodicity: Resource boundary periodicity, in symbols

-	Roffset: Resource boundary offset, in symbols. 0<= Roffset < Rperiodicity

The Ref_symbol in HF + N × periodicity  is now no longer aligned on a symbol boundary. In the figure below it corresponds to time instants Unaligned CG N=0,1, etc. The green part is just rounding up (aligning) this time instant to the next resource boundary from a newly introduced resource boundary pattern.

Such alignment formulas are very common in RAN1 specifications (e.g. to derive VRBs). There is no ambiguity and no complexity associated to it. It is similar to the problem of drawing a line on a screen with arbitrary slope (selecting the closest pixel) which has no computational issues (it can be implemented without division operation).

As a baseline, the resource boundary pattern can be configured with the same granularity of Rel-15 CG.

The generalized formula fallbacks to legacy formula if the ressource boundary pattern matches all symbols (Rperiodicity =1, Roffset =0), i.e. whenever it is not configured.

The HARQ offset formula remains valid with extended periodicities. The different HARQ process IDs are successively used except at SFN rollover. The SFN rollover HARQ process ID issue is not linked to non-integer periods, but to periods not dividing the HF. In LTE it was discussed and considered  scarce enough to not be addressed.

The resource boundary pattern is also useful even for integer periods. Assuming the gNB needs to accommodate a traffic A with periodicity 11ms, maximum latency 5ms. We assume a time ressource granularity of 1ms.

The gNB may use a CG A with the new periodicity of 11ms to avoid ressource waste. However, this would “consume” all the time offsets with a granularity of 1ms (this will be the case for e.g. all prime number periodicities). Assuming now the gNB needs to accommodate a traffic B with periodicity Xms (different from 11ms period or multiple thereof), this is no longer possible without collisions.

Conversaly, the NW can use an enhanced CG A with periodicity 11ms and a  resource boundary pattern of periodicity 5ms. The latency is fullfilled, and the NW may still time multiplex traffic B.

In the Annex, we compare this approach with “cyclic SPS/CG shifting”, and  indicate results of the above formula for different examples.

# Conclusion

We make the following observations and proposals:

Observation 1: A resource boundary pattern can easily address non-integer periodicities for CG/SPS

Observation 2: A resource boundary pattern is also useful for easing time multiplexing while using newly agreed integer periodicities (especially prime numbers periodicities)

Proposal 1: Introduce a resource boundary pattern to align CG/SPS opportunities, according to formula above

Proposal 2: Extend CG/SPS periodicities to better match TSC periodicities (e.g. in Hz or us)

# References

- RP-190728, New WID: Support of NR Industrial Internet of Things (IoT), Nokia, Nokia Shanghai Bell

- R2-1913453, “Support for TSC message periodicities of non-integer multiple of NR CG/SPS periodicities”, Nokia

- R2-1915692, “CG/SPS alignment with TSN periodicity”, Nokia, Nokia Shanghai Bell

# Annex – Comparison with “cyclic SPS/CG shifting”

In [3], an approach called cyclic SPS/CG shifting is proposed.

In this approach, the SPS/CG pattern is configured with an existing integer value periodicity, and shifted according to some conditions in the formula via using new parameters A, B and shift.

This yields an averaged periodicity which can be matched with TSC periodicity, such as (configured fields in green)

TSCPeriodicity  =  IntegerPeriodicity + Shift * A/B

Instead, the approach above would directly configure TSCPeriodicity, as well as Ressource Boundary Period/Offset, but the resulting occasions are the same.

In our view, using TSCPeriodicity, as well as Ressource Boundary Period/Offset is simpler as:

- the “cyclic shifting” formula introduces complexity, e.g. it is recursive and there is no way to e.g. derive quickly “what is the Nth occasion”

- RRC configuration parameters A/B are a bit obscure: as can be seen from the formula above, they are linked to the TSC periodicity, but it makes more sense to just indicate the desired TSC periodicity

- the “cyclic shifting” needs an existing close IntegerPeriodicity already supported. For instance to support 3Hz (333.333ms), then periodicity of 333ms needs to be supported. That would restrict possible periodicities.

- with “cyclic shifting”, the HARQ Process ID formula would use the IntegerPeriodicity. The resulting HARQ Process IDs are not incrementing (modulo #Processes) as expected for each occasion. There is no such problem when using directly the TSCPeriodicity as proposed in this contribution.

# Annex – Examples

A) 104 Hz (9.615ms) , 0.5ms latency

| Numerology |  |
| --- | --- |
| SlotDurationInMs | 1.000 |
| Configured Grant |  |
| Ref (Symbol in HF) | 0.000 |
| Periodicity (Ms) | 9.615 |
| Periodicity (Symbols) | 134.615 |
| Resource Boundary Pattern |  |
| Rperiod (ms) | 0.500 |
| Rperiod (Symbols) | 7.000 |
| Roffset (Symbols) | 0.000 |

| N | UCG (Slot in HF) | CG (Slot in HF) | Delta between CG (ms) | Latency UCG to CG (ms) |
| --- | --- | --- | --- | --- |
| 0 | 0.000 | 0 | 0 | 0.000 |
| 1 | 9.615 | 10 | 10 | 0.385 |
| 2 | 19.231 | 19.5 | 9.5 | 0.269 |
| 3 | 28.846 | 29 | 9.5 | 0.154 |
| 4 | 38.462 | 38.5 | 9.5 | 0.038 |
| 5 | 48.077 | 48.5 | 10 | 0.423 |
| 6 | 57.692 | 58 | 9.5 | 0.308 |
| 7 | 67.308 | 67.5 | 9.5 | 0.192 |
| 8 | 76.923 | 77 | 9.5 | 0.077 |
| 9 | 86.538 | 87 | 10 | 0.462 |
| 10 | 96.154 | 96.5 | 9.5 | 0.346 |
| 11 | 105.769 | 106 | 9.5 | 0.231 |
| 12 | 115.385 | 115.5 | 9.5 | 0.115 |
| 13 | 125.000 | 125 | 9.5 | 0.000 |
| 14 | 134.615 | 135 | 10 | 0.385 |
| 15 | 144.231 | 144.5 | 9.5 | 0.269 |
| 16 | 153.846 | 154 | 9.5 | 0.154 |
| 17 | 163.462 | 163.5 | 9.5 | 0.038 |
| 18 | 173.077 | 173.5 | 10 | 0.423 |
| 19 | 182.692 | 183 | 9.5 | 0.308 |
| 20 | 192.308 | 192.5 | 9.5 | 0.192 |
| 21 | 201.923 | 202 | 9.5 | 0.077 |
| 22 | 211.538 | 212 | 10 | 0.462 |
| 23 | 221.154 | 221.5 | 9.5 | 0.346 |

B) 13 ms, 5ms latency

| Numerology |  |
| --- | --- |
| SlotDurationInMs | 1.000 |
| Configured Grant |  |
| Ref (Symbol in HF) | 0.000 |
| Periodicity (Ms) | 13.000 |
| Periodicity (Symbols) | 182.000 |
| Resource Boundary Pattern |  |
| Rperiod (ms) | 5.000 |
| Rperiod (Symbols) | 70.000 |
| Roffset (Symbols) | 0.000 |

| N | UCG (Slot in HF) | CG (Slot in HF) | Delta between CG (ms) | Latency UCG to CG (ms) |
| --- | --- | --- | --- | --- |
| 0 | 0.000 | 0 | 0 | 0.000 |
| 1 | 13.000 | 15 | 15 | 2.000 |
| 2 | 26.000 | 30 | 15 | 4.000 |
| 3 | 39.000 | 40 | 10 | 1.000 |
| 4 | 52.000 | 55 | 15 | 3.000 |
| 5 | 65.000 | 65 | 10 | 0.000 |
| 6 | 78.000 | 80 | 15 | 2.000 |
| 7 | 91.000 | 95 | 15 | 4.000 |
| 8 | 104.000 | 105 | 10 | 1.000 |
| 9 | 117.000 | 120 | 15 | 3.000 |
| 10 | 130.000 | 130 | 10 | 0.000 |
| 11 | 143.000 | 145 | 15 | 2.000 |
| 12 | 156.000 | 160 | 15 | 4.000 |
| 13 | 169.000 | 170 | 10 | 1.000 |
| 14 | 182.000 | 185 | 15 | 3.000 |
| 15 | 195.000 | 195 | 10 | 0.000 |
| 16 | 208.000 | 210 | 15 | 2.000 |
| 17 | 221.000 | 225 | 15 | 4.000 |
| 18 | 234.000 | 235 | 10 | 1.000 |
| 19 | 247.000 | 250 | 15 | 3.000 |
| 20 | 260.000 | 260 | 10 | 0.000 |

C) 1200Hz, 0.5ms latency

| Numerology |  |
| --- | --- |
| SlotDurationInMs | 1.000 |
| Configured Grant |  |
| Ref (Symbol in HF) | 0.000 |
| Periodicity (Ms) | 0.833 |
| Periodicity (Symbols) | 11.667 |
| Resource Boundary Pattern |  |
| Rperiod (ms) | 0.500 |
| Rperiod (Symbols) | 7.000 |
| Roffset (Symbols) | 0.000 |

| N | UCG (Slot in HF) | CG (Slot in HF) | Delta between CG (ms) | Latency UCG to CG (ms) |
| --- | --- | --- | --- | --- |
| 0 | 0.000 | 0 | 0 | 0.000 |
| 1 | 0.833 | 1 | 1 | 0.167 |
| 2 | 1.667 | 2 | 1 | 0.333 |
| 3 | 2.500 | 2.5 | 0.5 | 0.000 |
| 4 | 3.333 | 3.5 | 1 | 0.167 |
| 5 | 4.167 | 4.5 | 1 | 0.333 |
| 6 | 5.000 | 5 | 0.5 | 0.000 |
| 7 | 5.833 | 6 | 1 | 0.167 |
| 8 | 6.667 | 7 | 1 | 0.333 |
| 9 | 7.500 | 7.5 | 0.5 | 0.000 |
| 10 | 8.333 | 8.5 | 1 | 0.167 |
| 11 | 9.167 | 9.5 | 1 | 0.333 |
| 12 | 10.000 | 10 | 0.5 | 0.000 |
| 13 | 10.833 | 11 | 1 | 0.167 |
| 14 | 11.667 | 12 | 1 | 0.333 |
| 15 | 12.500 | 12.5 | 0.5 | 0.000 |
| 16 | 13.333 | 13.5 | 1 | 0.167 |
| 17 | 14.167 | 14.5 | 1 | 0.333 |
| 18 | 15.000 | 15 | 0.5 | 0.000 |