#### ECID-SignalMeasurementInformationNR

The IE ECID-SignalMeasurementInformationNR is used by the target device to provide various UE NR measurements to the location server.

-- ASN1START

# ECID-SignalMeasurementInformationNR-r16 ::= SEQUENCE {

primaryCellNRMeasuredResults-r16	MeasuredResultsNRElement-r16			OPTIONAL,

measuredResultsNRList-r16			MeasuredResultsNRList-r16

...

}

# MeasuredResultsNRList-r16 ::= SEQUENCE (SIZE(1..32)) OF MeasuredResultsNRElement-r16

# MeasuredResultsNRElement-r16 ::= SEQUENCE {

physCellIdNR-r16				INTEGER (0..1007),

measResultNR-r16				SEQUENCE {

cellResults-r16					SEQUENCE{

resultsSSB-Cell-r16				MeasQuantityResults-r16				OPTIONAL,

resultsCSI-RS-Cell-r16			MeasQuantityResults-r16				OPTIONAL

},

rsIndexResults-r16				SEQUENCE{

resultsSSB-Indexes-r16			ResultsPerSSB-IndexList-r16			OPTIONAL,

resultsCSI-RS-Indexes-r16		ResultsPerCSI-RS-IndexList-r16		OPTIONAL

}																		OPTIONAL

},

cgi-InfoNR-r16					CGI-InfoNR-r16					OPTIONAL

}

MeasQuantityResults-r16 ::= SEQUENCE {

rsrp-r16	INTEGER (0..127)		OPTIONAL,

rsrq-r16	INTEGER (0..127)		OPTIONAL

}

ResultsPerSSB-IndexList-r16::= SEQUENCE (SIZE (1..64)) OF ResultsPerSSB-Index-r16

ResultsPerSSB-Index-r16 ::= SEQUENCE {

ssb-Index-r16					INTEGER (0..63),

ssb-Results-r16					MeasQuantityResults-r16					OPTIONAL

}

ResultsPerCSI-RS-IndexList-r16::= SEQUENCE (SIZE (1..64)) OF ResultsPerCSI-RS-Index-r16

ResultsPerCSI-RS-Index-r16 ::= SEQUENCE {

csi-RS-Index-r16				INTEGER (0..95),

csi-RS-Results-r16				MeasQuantityResults-r16					OPTIONAL

}

CGI-InfoNR-r16 ::=	SEQUENCE {

plmn-IdentityInfoList-r16			OCTET STRING				OPTIONAL,

frequencyBandList-r16				OCTET STRING				OPTIONAL,

noSIB1								SEQUENCE {

ssb-SubcarrierOffset-r16		INTEGER (0..15),

pdcch-ConfigSIB1-r16			OCTET STRING

}																OPTIONAL,

...

}

-- ASN1STOP

| Conditional presence | Explanation |
| --- | --- |

MeasResultNR field descriptions

cellResults Cell level measurement results.

physCellId The physical cell identity of the NR cell for which the reporting is being performed.

resultsSSB-Cell Cell level measurement results based on SS/PBCH related measurements.

resultsSSB-Indexes Beam level measurement results based on SS/PBCH related measurements.

resultsCSI-RS-Cell Cell level measurement results based on CSI-RS related measurements.

resultsCSI-RS-Indexes Beam level measurement results based on CSI-RS related measurements.

rsIndexResults Beam level measurement results.