3GPP TSG-RAN WG2 #108	 TDoc R2-1916456

# Reno, USA, 18th – 22th November 2019

Agenda Item:	6.4.1

Source:	Ericsson

Title:	Open issues on V2X 38.322 running CR

Document for:	Discussion, Decision

# Introduction

R2-1915380	Running CR for 38.322 for NR V2X	Ericsson	draftCR	Rel-16	38.322	15.5.0	B	5G_V2X_NRSL-Core

- [Offline#809]: To update and endorse running CR capturing this meeting agreements (R2-1916449, Ericsson)

In order to address remaining open issues related to V2X 38.322 running CR, companies are kindly invited to provide their views on issues listed in this paper. For the latest version of the running CR, please refer to R2-1915380 [1].

# Discussion

One remaining issue left in the current version of running CR is about whether SCCH is supported per RLC AM entity or RLC UM entity or both.

#### 4.2.1.2 UM RLC entity ##### 4.2.1.2.1 General An UM RLC entity can be configured to submit/receive RLC PDUs through the following logical channels: - DL/UL DTCH, and STCH. Editor’s Note: FFS whether to support SCCH per UM RLC entity. #### 4.2.1.3 AM RLC entity ##### 4.2.1.3.1 General An AM RLC entity can be configured to submit/receive RLC PDUs through the following logical channels: - DL/UL DCCH, DL/UL DTCH, and STCH. Editor’s Note: FFS whether to support SCCH per AM RLC entity.

Note that it has been agreed SCCH is used to carry PC5-S and PC5-RRC signalling. In rapporteur’s understanding, the first PC5-S signalling initiating link establishment, which is broadcast, can only be supported by RLC UM entity. In the first question, companies are invited to confirm such understanding.

Question 1: Companies are asked if the first PC5-S signalling initiating link establishment is sent via a SCCH supported by RLC-UM entity?

| Companies | Yes/No | Comments if any |
| --- | --- | --- |
| Huawei | Yes for Direct communication request | We assume such “first PC5-S signaling initiating link establishment” in the question refers to the Direct communication request message in SA2’s Spec, so it may be inevitably transmitted via RLC-UM, as this PC5-S message is transmitted in a broadcast manner. At the same time, we would like to point out that this message cannot share the same SCCH with any other PC5-S message, with the reason as clarified above that the SCCH used for this message is actually operated as a broadcast SL logical channel (via broadcast addressing), whilst the SCCH used for any other PC5-S message is operated as a unicast SL logical channel (via unicast addressing). |
| Samsung | Yes | The first PC5-S signaling is transmitted in broadcast manner. So RLC-UM is preferred. |
| OPPO | Yes |  |
| LG | Yes | RLC-UM is suitable due to the broadcast nature of the PC5-S signaling initiating link establishment. |
| Apple | Yes | The first message is a broadcast, so UM mode is suitable |
| Qualcomm | Yes |  |
| MediaTek | Yes | Share companies’ view that UM mode should be applied since the first PC5-S signaling is transmitted via broadcast. |
| Nokia | Yes |  |
| CATT | Yes |  |
| ZTE | Yes |  |
| Ericsson | Yes |  |

For the remaining cases, i.e. unicast PC5-S and PC5-RRC signalling, companies are asked if they can be sent via a SCCH supported by RLC-UM entity, or RLC-AM entity, or both. Note that in NR Uu, RLC AM mode is used for SRBs besides SRB0.

Question 2: Which RLC mode can be used to support SCCH for unicast PC5-S and PC5-RRC signalling?

- RLC-AM

- RLC-UM

| Companies | Yes/No | Comments if any |
| --- | --- | --- |
| Huawei | a | Signaling in unicast is appropriately transmitted via RLC-AM. |
| Samsung | a) | We think that one RLC mode i.e., RLC-AM is fine for unicast PC5-S and PC5-RRC. |
| OPPO | a) |  |
| LG | a) | Share the same understanding as Huawei and Samsung. |
| Apple | a) | AM only. |
| Qualcomm | a) |  |
| MediaTek | a) | Share same view with Huawei and Samsung. |
| Nokia | a) |  |
| CATT | a) |  |
| ZTE | a) |  |
| Ericsson | a) |  |

# Conclusion

SCCH configured with UM RLC entity is only used to transmit/receive broadcast PC5-S signalling message (i.e. Direct Communication Request).

AM RLC entity is configured for SCCH to transmit/receive all unicast PC5-RRC and PC5-S signalling message.

# References

[1] R2-1915380	Running CR for 38.322 for NR V2X	Ericsson