3GPP TSG-RAN WG2 #108	Tdoc R2-1916564

Reno, Nevada, USA, 18th – 22nd November 2019

Agenda Item:	7.1.12

Source:	Ericsson (Rapporteur)

Title:	Report of [108#19] When to resume DRBs in UP optimization for 5GC

Document for:	Discussion, Decision

# 1	Introduction

This is for the following email discussion:

- [108#19][eMTC NB-IoT] When to resume DRBs in UP optimization for 5GC (Ericsson) To agree on when to resume DRBs in UP optimization for 5GC Intended outcome: Email discussion report in R2-1916564 Deadline: Thursday 2019-12-05

During RAN2#108 and in the discussions related to running CRs for TS 36.331 before it was brought up that RAN2 have not agreed when to exactly resume DRBs. RAN2 has agreed the following in RAN2#107bis [1]:

RAN2#107bis agreements: Support of eDRX in CM-IDLE and EDT: - For NB-IoT and eMTC connected to 5GC: - NCC is always provided during suspension. - adopt early security activation for resumption from RRC_IDLE.

Before RAN2#108, the details were marked as FFS in the NB-IoT running CR for TS 36.331 [2] and commented on by various companies in the eMTC running CR for TS 36.331 [3].

Note that in LTE TEI16 during RAN2#108 there was a related discussion on early re-activation for security for UP solution for general LTE case, and at what stage DRBs should be resumed for that case. CRs have not yet been agreed for this case either yet.

# 2	Discussion

## 2.1	When to resume DRBs for UP solution in 5GC

At the moment in the specifications (TS 36.331, section 5.3.3), two different alternatives exist for DRB resumption for UP solution:

- Bearers are resumed before transmitting RRCConnectionResumeRequest, i.e. before transmission of Msg 3.

- Bearers are resumed after RRCConnectionResume, i.e., Msg4, has been received.

Option 1. above is used by UP-EDT (both MO and MT), including UP-PUR as so far discussed during Rel-16, as data needs to be transmitted in Msg3 and/or Msg4. The assumption is that any mechanism requiring user plane data transmission in Msg3 or Msg4 should follow 1.

Option 2. is currently used only by resumption from RRC_INACTIVE in eLTE (and NR).

One difference brought up during RAN2#108 between 1. and 2. above is related to ROHC functionality: For 1., in EDT the ROHC is currently only allowed to be continued if it was active before (indicated in previous RRCConnectionRelease) and the resumption is to the same cell, and for 2. ROHC is continued based on indication in Msg4.

Companies are welcome to provide their replies on the discussion points below.

DP1. Please provide company view and motivation on when to resume DRBs when resuming connection in UP solution for 5GC.

| Company | When to resume DRBs | Motivation |
| --- | --- | --- |
| QC | 5GC UP-EDT Resume : Resume DRBs before transmitting Msg 3 5GC UP Resume : activate DRBs after receiving Msg 4 | 5GC UP-EDT Resume : activating DRBs before Msg 3 transmission allows UE to send data in Msg 3 itself. 5GC UP Resume : this behaviour is same as RRC_INACTIVE resume procedure. |
| Intel | Resume DRBs before Msg3. For EDT, re-use drb-ContinueROHC-r15 And for non-EDT, reuse nr-RadioBearerConfig | Issue is not only to resume DRB but the whole resume procedure including ROHC state, SDAP, QoS flow to DRB mapping rule, and full configuration vs restored DRB configuration. For example for ROHC, whether to 1. follow EDT in EPS (decide whether to reset ROHC before Msg3 based on drb-ContinueROHC-r15 in release message) 2. Follow UP CIoT EPS optimization (decide whether to reset ROHC from drb-ContinueROHC-r13 provided in Msg4). 3. follow RRC_INACTIVE (continue ROHC state based on indication in nr-RadioBearerConfig provided in Msg4) There is no issue following EDT as stored DRB configuration can be used for DRB and reseting ROHC is not issue for small UL data transmission. Note that default configuration is used for MAC and PHY. We prefer to have same behavior between resumption from suspended state without UL data (UP 5GS CIoT opt) vs with UL data (EDT, PUR in 5GS) as much as possible. For EDT, drb-ContinueROHC-r15 provided in release message can apply. For non EDT, nr-RadioBearerConfig provided in Msg4 can apply. See next question to handle full configuration provided in Msg4. |
| Huawei/HiSilicon | Option 2 except for UP-EDT and PUR | The issue is related to the ROHC context not being exchanged between eNB. With option 1, the ROHC context will be reset everytime the UE resumes in a different cell, even if the new cell belongs to the same eNB and the context could be retrieved. This leads to unecessary signalling overhead. For UP-EDT and PUR in 5GC, same as for EPS, DRBs should be resumed before sending MSG3 due to the user data. |
| Ericsson | Before transmitting Msg3 | We'd prefer to align the behaviour between (MO/MT) EDT, UP-PUR and the UP solution in general for the 5GC case. This would result in less fragmentation in both implementation and in the specifications when considering the UP solution. It would be beneficial if eNB can include data in Msg4 e.g. if the eNB has data for the UE already or sets up S1 bearers, and gets data from CN, before replying back to UE after resumption. The ROHC issue was discussed during R15 and the solutions were presented where ROHC continuation can be more general, e.g. to be able to continue not only in the same cell, but also to the same eNB. However at that time RAN2 did not want to consider such options further. We are open to reconsideration if companies are interested. |

DP2. Please indicate if there are any other issues or comments you would like to bring up related to above discussion (e.g. related to early re-activation for LTE/EPC case).

| Company | Comments |
| --- | --- |
| Intel | Issue#1: full configuration RAN2#107bis For NB-IoT and eMTC connected to 5GC, full configuration in RRCConnectionResume(-NB) is supported. UE’s action following reception of full configuration remains same as in RRC_INACTIVE. If any UL data is multiplexed with RRCConnectionResumeRequest message (e.g., for EDT, PUR), it can be conisdered as failure. Issue#2: default SRB1 configuration In LTE EDT, we decided that before Msg3, SRB1 is restored. In fact, this may be problematic. For SRB1, defaul configuration for RLC is available to use in section 9.2.1.1. Therefore, as in RRC_INACTIVE, we think for EDT in CIoT 5GS optimization, the SRB1 should use the default NR PDCP and RLC configurations to align UE and target eNB’s SRB1 configuration. |
| Ericsson | For EPC cases for the UP solution, for the same reasons stated in DP1, we prefer resuming DRBs before Msg3. For EPC and UP optimization where early resumption is currently used, there is no precedence of resumption after Msg4 thus we'd like to align with EDT, PUR as was proposed in the CRs submitted to RAN2#108 LTE TEI16. |

# 3	Summary

Four replies have been provided in this discussion: replies from Qualcomm, Intel, Huawei/HiSilicon and Ericsson. Discussion point 1 asks preference on when DRBs should be continued for the 5GC UP solution. For DP1 two companies prefer to resume the DRBs after Msg4 has been received, where one company prefers to align with RRC_INACTIVE and one company brings up the case with ROHC continuation and need for additional signalling otherwise. Two companies prefer to resume DRBs before Msg3 has been transmitted, where the companies prefer to align with EDT, one company mentions this should be discussed together with other aspects related to resumption, and that Msg4 indication could be followed in non-EDT case, and one company says it would be beneficial if there was possibility to transmit data in Msg4.

There is clearly no consensus or any kind of majority, thus the proposal is to continue the discussion of this topic based on the company replies above:

RAN2 to discuss whether to resume DRBs before Msg3 is transmitted or after Msg4 is received for 5GC UP solution.

The common understanding, also commented on in this discussion, is that for EDT-type features (i.e. also 5GC UP-EDT, Rel-16 UP-PUR) the DRBs need to be continued before Msg3 is transmitted. There have been no proposals to change the EDT behaviour, thus no proposals or further discussion should be needed for this aspect.

Discussion point 2 asked about any other issues which were not explicitly in the scope of the email discussion. One company brings up two additional issues: 1) full configuration case, and what should be the exact UE actions and 2) how to restore SRB1 in detail. One company brings up the case for EPC (discussed in TEI16) and prefers to resume DRBs before Msg3 in such case. These topics can be further discussed e.g. based on company contributions.

# References

R2-1914102, "RAN2 agreements for Rel-16 additional enhancements for NB-IoT and eMTC", Blackberry (Rapporteur), RAN2#107bis, Chongqing, China, 2019

R2-1914101, "Introduction of additional enhancements for NB-IoT in 36.331", Huawei, RAN2#107bis, Chongqing, China, 2019

R2-1914038, "Introduction of Rel-16 eMTC enhancements", Qualcomm Inc., RAN2#107bis, Chongqing, China, 2019