3GPP TSG-RAN WG2 Meeting #109 electronic	R2-2001311

24 February – 6 March 2020

Agenda item:	6.18.2

Source:	Qualcomm Incorporated (rapporteur)

Title:	Report for email discussion [108#71][PRN] Running 38.304 CR (Qualcomm)

WID/SID:	Private Network Support for NG-RAN

Document for:	Discussion

# 1	Introduction

This document is for the following email discussion from RAN2#108:

[108#71][PRN] Running 38.304 CR (Qualcomm) Intended outcome: Endorsed running CR reflecting agreements from this meeting. Consider also proposals R2-1915384 Deadline: 2020-01-30

This document aims to

- discuss changes needed for TS 38.304 to capture all previous NPN RAN2 agreements,

- discuss changes to TS 38.304 to support text in TS 23.122 and TS 23.501 related to NPN,

- discuss some additional changes highlighted in yellow for TS 38.304 identified in R2-1915384 [2] and by the rapporteur which may not be based on a previous RAN2 agreement (and thus likely needs more discussion).

  - Not all changes proposed in R2-1915384 are captured in this document and some have been captured with some modifications. Companies are encouraged to review R2-1915384 and comment on the need for any additional changes to be discussed in this document.

Draft running CR R2-1915800 [1] developed as part of email discussion for RAN2#108 has been used as a baseline for several text changes in this document (note that R2-1915800 was not endorsed).

## 1.1	Organization

In the next section, a section of TS 38.304 with proposed changes is presented followed by a table to provide input for the changes. Please use these tables to provide comments for associated sections in TS 38.304.

Section 3 of this document can be used to provide any additional comments.

## 1.2	Timeline

This email discussion will be carried out in two phases:

- Phase 1: to collect input for Sections 2 and 3 of this document.

  - Deadline: Jan 26

- Phase 2: to prepare (i) a draft running CR for TS 38.304 based on input collected during phase 1 and (ii) if needed, prepare a email discussion report as a tdoc for RAN2#109 to discuss any changes/proposals related to open issues identified during phase 1.

  - Deadline: Jan 30

# 2	Discussion

Start of this change

## 3.1	Definitions

For the purposes of the present document, the following terms and definitions apply:

Acceptable Cell: A cell that satisfies certain conditions as specified in 4.5.

Allowed CAG list: a per-PLMN list of CAG Identifiers the UE is allowed to access (see TS 23.501 [3]).

Available PLMN(s): One or more PLMN(s) for which the UE has found at least one cell and read its PLMN identity(ies).

Barred Cell: A cell a UE is not allowed to camp on.

CAG cell: A cell broadcasting at least one Closed Access Group Identifier.

CAG Member Cell: for a UE, a cell broadcasting the identity of the selected PLMN, registered PLMN or equivalent PLMN, and the cell broadcasts a CAG identifier belonging to the Allowed CAG list of the UE for that PLMN.

CAG-only cell: a cell providing access only to CAGs.

Camped on a cell: UE has completed the cell selection/reselection process and has chosen a cell. The UE monitors system information and (in most cases) paging information.

Camped on any cell: UE is in idle mode and has completed the cell selection/reselection process and has chosen a cell irrespective of PLMN identity.

Closed Access Group Identifier: identifies a CAG within a PLMN.

Commercial Mobile Alert System: Public Warning System that delivers Warning Notifications provided by Warning Notification Providers to CMAS capable UEs.

EHPLMN: Any of the PLMN entries contained in the Equivalent HPLMN list TS 23.122 [9].

Equivalent PLMN list: List of PLMNs considered as equivalent by the UE for cell selection, cell reselection, and handover according to the information provided by the NAS.

Home PLMN: A PLMN where the Mobile Country Code (MCC) and Mobile Network Code (MNC) of the PLMN identity are the same as the MCC and MNC of the IMSI.

Network Identifier: identifies an SNPN in combination with a PLMN ID (TS 23.501 [3]).

Non-Public Network: A network deployed for non-public use, as defined in TS 22.261 [12].

Process: A local action in the UE invoked by an RRC procedure or an RRC_IDLE or RRC_INACTIVE state procedure.

Radio Access Technology: Type of technology used for radio access, for instance NR or E-UTRA.

Registration Area: (NAS) registration area is an area in which the UE may roam without a need to perform location registration, which is a NAS procedure.

Registered PLMN: This is the PLMN on which certain Location Registration outcomes have occurred, as specified in TS 23.122 [9].

Registered SNPN: This is the SNPN on which certain Location Registration outcomes have occurred, as specified in TS 23.122 [9].

Reserved Cell: A cell on which camping is not allowed, except for particular UEs, if so indicated in the system information.

Selected PLMN: This is the PLMN that has been selected by the NAS, either manually or automatically.

Selected SNPN: This is the SNPN that has been selected by the NAS, either manually or automatically.

Serving cell: The cell on which the UE is camped.

SNPN Access Mode: mode of operation wherein UE only selects SNPNs (as defined in TS 23.501 [3]).

SNPN identity: an identifier of an SNPN comprising of a PLMN ID and an NID combination.

SNPN-only cell: a cell providing access only to SNPNs.

Strongest cell: The cell on a particular frequency that is considered strongest according to the layer 1 cell search procedure (TS 38.213 [4], TS 38.215 [11]).

Suitable Cell: This is a cell on which a UE may camp. For NR cell, the criteria are defined in clause 4.5, for E-UTRA cell in TS 36.304 [7].

End of this change

3.1-a: Please share your comments for above changes to clause 3.1 in table below.

| Comment # | Company | Detailed description of comment |
| --- | --- | --- |
| 1 | Ericsson | Shouldn’t the definitions of registered/selected SNPN be aligned with that of registered/selected PLMN? |
| @1 | Rapporteur | Please see updated definitions of registered/selected SNPN. |
| 2 | Lenovo | The abbreviation “NID” can be removed as it is already defined in clause 3.2. The following definitions can be added: - Allowed CAG list: A list containing the CAG identifiers the UE is allowed to access. - CAG Member Cell: For a UE, a cell broadcasting the identity of the selected PLMN, registered PLMN or equivalent PLMN, and for that PLMN, a CAG identifier belonging to the Allowed CAG list of the UE for that PLMN. |
| @2 | Rapporteur | Removed the abbreviation “NID”. Added ‘Allowed CAG list’ with some modification to the suggestion above to reuse wording from TS 23.501 and also to highlight that the list is per-PLMN. Added ‘CAG Member Cell’ definition. |
| 3 | Intel | We think NID should be included here since it is providing the context like the CAG ID definition. |
| @3 | Rapporteur | Network Identifier definition has been retained. |
| 4 | Nokia | The running 38.300 CR contains the following two notes: Editor’s Note: definition of CAG-only cell should be added here once agreed in 38.304. Editor’s Note: definition of SNPN-only cell should be added here once agreed in 38.304. Therefore, we think that there is a need for two more definitions. We also agree Ericsson’s comment above. |
| @4 | Rapporteur | I have added the definitions for ‘SNPN-only cell’ and ‘CAG-only cell’. However, I don’t think they are needed in 38.304 as the terms are not currently used anywhere. Also, a precise definition of the terms (based on setting of the SIB1 fields) may be more suited for the RRC specification. Also see comment @1. |
| 5 | ZTE | We suggest to change the definition of Non-Public Network and registered SNPN as follows: Non-Public Network: A network that is intended for non-public use, as defined in TS 22.261 [12]. Registered SNPN: This is the SNPN on which certain Location Registration outcomes have occurred, as specified in TS 23.122 [9]. |
| @5 | Rapporteur | Modified definition of ‘Non-Public Network:’ as suggested with one modification (intended->deployed) to align with wording in TS 23.501 (which says “A Non-Public Network (NPN) is a 5GS deployed for non-public use, see TS 22.261 [2].”). Updated the definition of ‘Registered SNPN’ as suggested. |
| 6 | Huawei | Agree to include NID. Agree to add some descriptions to Non-Public Network and Registered SNPN as ZTE suggested. One small typo: “a SNPN” changed to “an SNPN” in the definition of “SNPN identity”. (Same typo in 3rd paragraph of 4.1) |
| @6 | Rapporteur | Network Identifier definition has been retained. Please see @5. Also, fixed the typo above and in 4.1. |

Next Change

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

AS	Access Stratum

CMAS	Commercial Mobile Alert System

CN	Core Network

CAG	Closed Access Group

CAG-ID	Closed Access Group Identifier

DCI	Downlink Control Information

ETWS	Earthquake and Tsunami Warning System

E-UTRA	Evolved UMTS Terrestrial Radio Access

E-UTRAN	Evolved UMTS Terrestrial Radio Access Network

HRNN	Human-Readable Network Name

IMSI	International Mobile Subscriber Identity

MCC	Mobile Country Code

MICO	Mobile Initiated Connection Only

NAS	Non-Access Stratum

NID	Network Identifier

NPN	Non-Public Network

NR	NR Radio Access

PLMN	Public Land Mobile Network

RAT	Radio Access Technology

RNA	RAN-based Notification Area

RNAU	RAN-based Notification Area Update

RRC	Radio Resource Control

SNPN	Stand-alone Non-Public Network

UAC	Unified Access Control

UE	User Equipment

UMTS	Universal Mobile Telecommunications System

End of this change

3.2-a: Please share your comments for above changes to clause 3.2 in table below.

| Comment # | Company | Detailed description of comment |
| --- | --- | --- |
|  | Ericsson | Changes look ok. |
|  | Lenovo | Changes look ok. |
|  | ZTE | Changes look ok. |
|  | Huawei | Changes look ok. |

Next change

## 4.1	Overview

The RRC_IDLE state and RRC_INACTIVE state tasks can be subdivided into three processes:

-	PLMN selection (for UE not operating in SNPN access mode) or SNPN selection (for UE operating in SNPN access mode);

-	Cell selection and reselection;

-	Location registration and RNA update;

Editor’s note: It needs to be confirmed whether manual CAG selection can be treated as being part of PLMN selection.

PLMN selection, SNPN selection, cell reselection procedures, and location registration are common for both RRC_IDLE state and RRC_INACTIVE state. RNA update is only applicable for RRC_INACTIVE state. When UE selects a new PLMN, UE transitions from RRC_INACTIVE to RRC_IDLE, as specified in TS 24.501 [14].

When a UE is switched on, a public land mobile network (PLMN) or a SNPN is selected by NAS. For the selected PLMN/SNPN, associated RAT(s) may be set, as specified in TS 23.122 [9]. The NAS shall provide a list of equivalent PLMNs, if available, that the AS shall use for cell selection and cell reselection.

With cell selection, the UE searches for a suitable cell of the selected PLMN or selected SNPN, chooses that cell to provide available services, and monitors its control channel. This procedure is defined as “camping on the cell”.

The UE shall, if necessary, then register its presence, by means of a NAS registration procedure, in the tracking area of the chosen cell. As an outcome of a successful Location Registration, the selected PLMN/SNPN then becomes the registered PLMN/SNPN , as specified in TS 23.122 [9].

If the UE finds a more suitable cell, according to the cell reselection criteria, it reselects onto that cell and camps on it. If the new cell does not belong to at least one tracking area to which the UE is registered, location registration is performed. In RRC_INACTIVE state, if the new cell does not belong to the configured RNA, an RNA update procedure is performed.

If necessary, the UE shall search for higher priority PLMNs at regular time intervals as described in TS 23.122 [9] and search for a suitable cell if another PLMN has been selected by NAS.

For UE not operating in SNPN access mode, search of available CAGs may be triggered by NAS to support manual CAG selection. The UE shall report available CAG ID(s) together with their HRNN (if broadcast) and PLMN(s) to the NAS.

If the UE loses coverage of the registered PLMN/SNPN, either a new PLMN/SNPN is selected automatically (automatic mode), or an indication of available PLMNs/SNPNs is given to the user so that a manual selection can be performed (manual mode). As part of manual SNPN selection, the UE shall report available SNPN identifiers together with their HRNN (if broadcast) to the NAS.

Registration is not performed by UEs only capable of services that need no registration.

The purpose of camping on a cell in RRC_IDLE state and RRC_INACTIVE state is fourfold:

a)	It enables the UE to receive system information from the PLMN or the SNPN.

b)	When registered and if the UE wishes to establish an RRC connection or resume a suspended RRC connection, it can do this by initially accessing the network on the control channel of the cell on which it is camped.

c)	If the network needs to send a message or deliver data to the registered UE, it knows (in most cases) the set of tracking areas (in RRC_IDLE state) or RNA (in RRC_INACTIVE state) in which the UE is camped. It can then send a "paging" message for the UE on the control channels of all the cells in the corresponding set of areas. The UE will then receive the paging message and can respond.

d)	It enables the UE to receive ETWS and CMAS notifications.

When the UE is in RRC_IDLE state, upper layers may deactivate AS layer when MICO mode is activated as specified in TS 24.501 [14]. When MICO mode is activated, the AS configuration (e.g. priorities provided by dedicated signalling) is kept and all running timers continue to run but the UE need not perform any idle mode tasks. If a timer expires while MICO mode is activated it is up to the UE implementation whether it performs the corresponding action immediately or the latest when MICO mode is deactivated. When MICO mode is deactivated, the UE shall perform all idle mode tasks.

End of this change

Many changes to clause 4.1 above are based on changes proposed in R2-1915384. Some other changes are based on similar text in TS 36.304 and they are explained using comments above.

4.1-a: Please share your comments for above changes to clause 4.1 in table below.

| Comment # | Company | Detailed description of comment |
| --- | --- | --- |
| 1 | Ericsson | We think it is better to treat CAG selection as part of the PLMN selection and not model it as a separate process. During PLMN selection the PLMN ID and associated CAG IDs should be reported from AS to NAS and then NAS selects a PLMN and CAG ID based on this information. If and how the selected CAG ID is indicated from NAS to AS needs to be further discussed. We see two approaches: - The selected CAG ID is included in the Allowed CAG list and is thereby indicated implicitly/indirectly. This method works for automatic CAG selection since in this case the CAG ID selected by NAS has to be included in the (pre-provisioned) Allowed CAG list. The Allowed CAG list is then used for determining the suitable cells in the cell selection/re-selection. - The selected CAG ID is indicated as a separate parameter from NAS to AS. This method may be more suitable for manual PLMN/CAG selection since in this case the CAG ID selected by NAS (through the user) may not be included in the Allowed CAG list. In this case indicating the selected CAG ID may be necessary to determine which cells are suitable in the cell selection. For the manual SNPN selection we don’t think this should be modelled as a separate process either. Manual and automatic is PLMN selection is treated jointly so we should be able to do the same for SNPNs. Then a general comment/question: rather than using the negative formulation “Ues not operating in SNPN access mode” maybe we could use a positive formulation e.g. “Ues operating in PLMN access mode” or similar. |
| @1 | Rapporteur | I am assuming that the first part of the comment is about manual CAG selection and it is now part of the PLMN selection in this version since several companies supported this. Since there was one company with a different view on this, I have included a proposal below this table for online discussion to confirm this and added an editor’s note to the text changes above. On the issues of indication method of manually selected CAG ID and how it is used, further discussion appears to be needed (and was suggested by others too). So, I have added a proposal below based on the second approach for further discussion. The second approach was supported by another company and seems to be more aligned with following previous agreement: If NAS has selected a CAG and provided this selection to AS, the UE shall search for an acceptable or suitable cell belonging to the selected CAG to camp on. Manual SNPN selection is no longer treated as a separate process in this version. It will be considered as part of SNPN selection. Regarding terminology related to negative formulation: there wasn’t significant support for the proposal and one company opposed it. So, I have added a proposal below for further discussion. |
| 2 | Lenovo | The description can be simplified since there is no need to always mention “not operating in SNPN access mode” and “operating in SNPN access mode” for CAG/SNPN selection: - It is sufficient to have only the text in brackets “(for UE operating in SNPN access mode)” added for “Support for manual SNPN selection” in the first paragraph. - The text “for UE not operating in SNPN access mode” can be removed due to the fact that SNPN access mode is only applicable for an SNPN-enabled UE. There should be no risk of misunderstanding that a normal/legacy and a CAG-capable UE performs only legacy PLMN selection procedure if it is not configured for SNPNs. Therefore, we suggest that whenever “PLMN” is referenced it is sufficient to add “SNPN”, e.g. When a UE is switched on, a public land mobile network (PLMN) or SNPN is selected by NAS. For the selected PLMN/SNPN, associated RAT(s) may be set, as specified in TS 23.122 [9]. We are ok to describe manual CAG selection as a separate process. We think this is beneficial in order to clearly distinguish between normal and CAG-capable UEs. Furthermore, regarding manual CAG selection it is sufficient to say: Search of available CAGs may be triggered by NAS to support manual CAG selection. |
| @2 | Rapporteur | I have applied your suggestions above to reduce the text (even though I am not sure if it is simpler to read now) with following two exceptions: - “PLMN selection (for UE not operating in SNPN access mode) or SNPN selection (for UE operating in SNPN access mode);” - “For UE not operating in SNPN access mode, search of available CAGs may be triggered by NAS to support manual CAG selection.” Rationale for the first exception above is that it is worth clarifying in the Overview section that PLMN selection applies only to UEs not operating in SNPN AM and SNPN selection applies only to UEs operating in SNPN AM. For the second exception, it seems useful to be clear that manual CAG selection does not apply to SNPN AM. Please let me know if you disagree with the exceptions above. Manual CAG selection is now treated as a separate process (based on support from multiple companies). But, it is a tentative change and is subject to further discussion (see related editor’s note above and proposal below). |
| 3 | Intel | We also agree with Lenovo that there is no need to duplicate the text for PLMN also for SNPN and thus there is no need to include “not operating in SNPN access mode” and “operating in SNPN access mode” to separate SNPN and PLMN selection. |
| @3 | Rapporteur | Please see @2. |
| 4 | Nokia | - We think that the CAG selection should be combined with PLMN selection at AS level: AS should discover and report PLMN IDs and the corresponding CAG IDs to NAS together in one step. - Manual SNPN and CAG selection (reporting of HRNNs) should also be captured here. - The need for a “selected CAG ID” at AS is not clear. According to this draft CR it is only used for initial cell selection when manual CAG selection is performed. Our question is whether in case of manual selection the NAS should provide the allowed CAG list or only the selected CAG ID as if the allowed CAG list is also provided the UE may move to cell (found suitable based on the allowed CAG list) that does not support to selected CAG ID immediately after the initial cell selection. (Consider the case when a UE can only find a “weak”, but suitable cell that supports the selected CAG ID, but there are much better cells that supports CAG ID(s) of allowed CAG list.) We think that more discussion is needed on this in RAN2. |
| @4 | Rapporteur | - Manual CAG selection is now part of PLMN selection in this version. Also, see @1. - I added the following above: - " As part of manual SNPN selection, the UE shall report available SNPN identifiers together with their HRNN (if broadcast) to the NAS.” - “The UE shall report available CAG ID(s) together with their HRNN (if broadcast) and PLMN(s) to the NAS.” - Please see related proposal below for further discussion of the issue. |
| 5 | ZTE | We share the same understanding with Lenovo and Intel and prefer to simplify the description. |
| @5 | Rapporteur | Please see @2. |
| 6 | Huawei | 1) Regarding the two approaches proposed by Ericsson, we prefer the second, i.e. after selecting the CAG ID, NAS only indicates the selected CAG ID to AS, not the allowed CAG list. In our understanding, the allowed CAG list should be indicated to AS before PLMN selection. Besides, there is no need to repeatedly transmit the allowed CAG list to AS each time after NAS performs CAG selection. 2) As for the positive/negative formulation, we prefer the current expression used by the rapporteur, because “PLMN access mode” is not defined. 3) Agree with Lenovo to simplify the text. |
| @6 | Rapporteur | - Please see a proposal below for further discussion of the issue. The proposal is based on the second approach. - I have not made any changes. But, it seems useful to discuss it further and added a proposal below. - Please see @2. |
| 7 | LG | SNPN selection: - In 23.122, SNPN selection process is defined as a separate process. Then it seems reasonable to define SNPN selection as a separate process also in 38.304. - However, manual SNPN process does not have to be treated as a separated process (we do not treat manual PLMN selection process as a separate process.) CAG selection: - We do not see the need to define manual CAG selection as a separate process. Instead, manual CAG selection can be treated as part of a generic PLMN selection process, as Ericsson said. |
| @7 | Rapporteur | SNPN selection is mentioned as a separate parallel process. Current version is aligned with your comments on manual SNPN selection and manual CAG selection. Please see @1. |
| @all | Rapporteur | Part c of Proposal 2 is related to comments in clause 4.5 |

Proposals based on above discussion:

Proposal 1:

Void

Proposal 2: RAN2 should discuss the following related to manual CAG selection:

- Selected CAG ID is indicated from NAS to AS as a parameter separate from Allowed CAG list.

- Selected CAG ID is used for cell selection immediately after manual CAG selection and not used subsequently (except if it is part of Allowed CAG list).

- As an outcome of the manual CAG selection procedure the UE is allowed to access an acceptable cell which fulfils the cell selection criteria and is not barred or reserved for operator use for UEs not belonging to Access Identities 11 or 15 and inform NAS that access is possible (for location registration procedure).

Proposal 3: RAN2 should discuss the following two options about terminology:

- Use “UEs not operating in SNPN access mode”.

- Define PLMN access mode, and use “UEs operating in PLMN access mode”  instead of “UEs not operating in SNPN access mode”.

Next change

## 4.2	Functional division between AS and NAS in RRC_IDLE state and RRC_INACTIVE state

Table 4.2-1 presents the functional division between UE non-access stratum (NAS) and UE access stratum (AS) in RRC_IDLE state and RRC_INACTIVE states. The NAS part is specified in TS 23.122 [9] and the AS part in the present document.

Table 4.2-1: Functional division between AS and NAS in RRC_IDLE state and RRC_INACTIVE state

| RRC_IDLE and RRC_INACTIVE state Process | UE Non-Access Stratum | UE Access Stratum |
| --- | --- | --- |
| PLMN Selection | For a UE not operating in SNPN access mode, perform the following: Maintain a list of PLMNs in priority order according to TS 23.122 [9]. Select a PLMN using automatic or manual mode as specified in TS 23.122 [9] and request AS to select a cell belonging to this PLMN. For each PLMN, associated RAT(s) may be set. Evaluate reports of available PLMNs from AS for PLMN selection. Maintain a list of equivalent PLMN identities. To support manual CAG selection, provide request to search for available CAGs and evaluate reports of available CAGs from AS for CAG selection. For a UE operating in SNPN access mode, perform the following: Maintain a list of SNPNs according to TS 23.122 [9]. Select a SNPN using automatic or manual mode as specified in TS 23.122 [9] and request AS to select a cell belonging to this SNPN. Evaluate reports of available SNPNs from AS for SNPN selection. | For a UE not operating in SNPN access mode, search for available PLMNs. If associated RAT(s) is (are) set for the PLMN, search in this (these) RAT(s) and other RAT(s) for that PLMN as specified in TS 23.122 [9]. For a UE operating in SNPN access mode, search for available SNPNs only consider NR cells. Perform measurements to support PLMN/SNPN selection. Synchronise to a broadcast channel to identify found PLMNs/SNPNs. Report available PLMNs with associated RAT(s) to NAS on request from NAS or autonomously. To support manual CAG selection, perform the following: Search for cells broadcasting a CAG-ID. Read the HRNN (if broadcast) for each CAG-ID if a cell broadcasting a CAG-ID is found. Report CAG-ID(s) of found cell(s) broadcasting a CAG ID together with the associated HRNN and PLMN to NAS. On selection of a CAG by NAS, select any acceptable or suitable cell belonging to the selected CAG and give an indication to NAS that access is possible (for the registration procedure) Editor’s note: It is FFS whether the above needs to capture the condition that the cell is “not reserved for operator use for UEs not belonging to AC 11 or 15” To support manual SNPN selection, report available SNPNs together with associated HRNNs (if available) to NAS on request from NAS. |
| Cell Selection | Control cell selection for example by indicating RAT(s) associated with the selected PLMN to be used initially in the search of a cell in the cell selection. Maintain a list of "Forbidden Tracking Areas" and provide the list to AS. For a UE not operating in SNPN access mode: Maintain Allowed CAG list and optional CAG-only indication along with associated PLMN ID(s) on which the UE is allowed access and provide these lists to AS. To support manual CAG selection, select a CAG and request AS to select a cell belonging to this CAG. | Perform measurements needed to support cell selection. Detect and synchronise to a broadcast channel. Receive and handle broadcast information. Forward NAS system information to NAS. Search for a suitable cell. The cells broadcast one or more 'PLMN identity' or ‘SNPN identity’ (for a UE operating in SNPN access mode) in the system information. Respond to NAS whether such cell is found or not. If associated RATs is (are) set for the PLMN, perform the search in this (these) RAT(s) and other RATs for that PLMN as specified in TS 23.122 [9]. If a cell is found which satisfies cell selection criteria, camp on that cell. |
| Cell Reselection | For a UE not operating in SNPN access mode, maintain a list of equivalent PLMN identities and provide the list to AS. Maintain a list of "Forbidden Tracking Areas" and provide the list to AS. For a UE not operating in SNPN access mode, maintain Allowed CAG list and optional CAG-only indication along with associated PLMN ID(s) on which the UE is allowed access and provide these lists to AS. | Perform measurements needed to support cell reselection. Detect and synchronise to a broadcast channel. Receive and handle broadcast information. Forward NAS system information to NAS. Change cell if a more suitable cell is found. |
| Location registration | Register the UE as active after power on. Register the UE's presence in a registration area, for instance regularly or when entering a new tracking area. Deregister UE when shutting down. Maintain a list of "Forbidden Tracking Areas". | Report registration area information to NAS. |
| RAN Notification Area Update | Not applicable. | Register the UE's presence in a RAN-based notification area (RNA), periodically or when entering a new RNA. |

End of this change

Many changes to clause 4.2 above are also based on changes proposed in R2-1915384, though two separate tables have been used instead of one to separately treat UE operating in SNPN AM and not operating in SNPN AM.

The text highlighted in yellow above is based on following in TS 23.122: “In the SNPN selection process, the MS shall consider only the access networks of the NG-RAN access technology.”

Some changes to clause 4.2 above are based on similar text in TS 36.304 and they are explained using comments above.

4.2-a: Please share your comments for above changes to clause 4.2 in table below.

| Comment # | Company | Detailed description of comment |
| --- | --- | --- |
| 1 | Ericsson | For the manual CAG selection, we think it is better to consider this as part of the PLMN selection (see comment to question 4.1.a). So instead of adding a new row for manual CAG selection we could extend the description in the row for PLMN selection. The functional division between AS and NAS for a UE in PLMN access mode and a UE in SNPN access mode seems almost identical except that PLMN is replaced with SNPN and CAG is not supported in SNPN access mode. Considering the large similarities, it doesn’t seem necessary to introduce a separate table for the SNPN access mode case. |
| @1 | Rapporteur | Merged manual CAG selection row into PLMN selection. I have merged two tables into one (also following Intel’s suggestion below). I felt that two tables resulted in much clearer specification even if costed some extra space. I do hope that the combined table is readable. Comments on readability of the table are very welcome. |
| 2 | Lenovo | We share the view from Ericsson that there is no stringent need to add a new Table 4.2-X for SNPN due to its large similarities with existing table 4.2-1. On Table 4.2-1 for CAG: For cell selection/reselection we wonder why there is a need for NAS to provide CAG-only indication (if configured) to AS? In the description below and referring to the discussion to clause 4.5 the condition “or reserved for operator use for UEs not belonging to AC 11 or 15” is missing. On selection of a CAG by NAS, select any acceptable or suitable cell belonging to the selected CAG and give an indication to NAS that access is possible (for the registration procedure). |
| @2 | Rapporteur | Please see @1 about the merged table. CAG-only indication is needed in AS for suitability checks discussed in clause 4.5. I have added an editor’s note with an FFS so that we can revisit this once we conclude on the exception mentioned in clause 4.5. Several companies are proposing to alter/remove the exception. We can perhaps revisit the FFS once clause 4.5 is stable. |
| 3 | Intel | We are also wondering whether 2 tables are really required or we can just quantify using the term ‘UE operating in SNPN access mode’ or UE not operating in SNPN access mode’ within a table to differentiate the different function. |
| @3 | Rapporteur | Please see @1 about the merged table. |
| 4 | Nokia | We are OK with the structure (having two tables), however we have some comments - In Table 4.2-1 “Manual CAG selection” should be merged the with PLMN selection. - We propose to add the following to Table 4.2-X to enable manual SNPN selection “Report available SNPNs with associated HRNNs (if available) to NAS on request from NAS” |
| @3 | Rapporteur | Given many companies prefer to have a single table, I merged the tables. Please see @1 about the merged table. - Done. - Added the suggested sentence with some edits (comments on the edits welcome): “To support manual SNPN selection, report available SNPNs together with associated HRNNs (if available) to NAS on request from NAS.” |
| 5 | ZTE | (1)We also suggest to merge the behavior of UE operating in SNPN access mode and UE not operating in SNPN access mode into one table, i.e. the existing table 4.2-1. (2)Regarding the comment from Lenovo asking why there is a need for NAS to provide the CAG-only indication to AS, we understand that such an indication is useful in cell suitability check as mentioned in 4.5. In our understanding, the cell suitability check is performed in AS layer so it makes sense for NAS to provide the CAG-only indication to AS. |
| @5 | Rapporteur | - Please see @1 about the merged table. - Agree. |
| 6 | Huawei | Having a separate table for SNPN is OK for us. Since CAG related information is not needed for SNPN, having a separate table makes the text easier to read. We are also fine with adding a new row for manual CAG selection. Agree with ZTE that CAG-only indication is needed for AS to check cell suitability. |
| @6 | Rapporteur | Given many companies prefer to have a single table, I merged the tables. Please see @1 about the merged table. |
| 7 | LG | We do not see the benefit of duplicating the tables when the two are almost identical. |
| @7 | Rapporteur | Please see @1 about the merged table. |

Next change

## 4.5	Cell Categories

The cells are categorised according to which services they offer:

acceptable cell:

An "acceptable cell" is a cell on which the UE may camp to obtain limited service (originate emergency calls and receive ETWS and CMAS notifications). Such a cell shall fulfil the following requirements, which is the minimum set of requirements to initiate an emergency call and to receive ETWS and CMAS notification in an NR network:

-	The cell is not barred, see clause 5.3.1;

-	The cell selection criteria are fulfilled, see clause 5.2.3.2.

suitable cell:

For UE not operating in SNPN Access Mode, a cell is considered as suitable if the following conditions are fulfilled:

-	The cell is part of either the selected PLMN or the registered PLMN or PLMN of the Equivalent PLMN list, and for that PLMN either

- the PLMN-ID of that PLMN is broadcast by the cell with no associated CAG-IDs and CAG-only indication in the UE for that PLMN (TS 23.501 [10]) is absent or false;

- Allowed CAG list in the UE for that PLMN (TS 23.501 [10]) includes a CAG-ID broadcast by the cell for that PLMN;

-	The cell selection criteria are fulfilled, see clause 5.2.3.2.

According to the latest information provided by NAS:

-	The cell is not barred, see clause 5.3.1;

-	The cell is part of at least one TA that is not part of the list of "Forbidden Tracking Areas" (TS 22.261 [12]), which belongs to a PLMN that fulfils the first bullet above.

Editor’s note: It is FFS whether the above needs to be updated to consider manually selected CAG ID.

Editor’s note: Consider simplifying the above suitability check using the definition of ‘CAG Member Cell’.

For UE operating in SNPN Access Mode, a cell is considered as suitable if the following conditions are fulfilled:

-	The cell is part of either the selected SNPN or the registered SNPN of the UE;

-	The cell selection criteria are fulfilled, see clause 5.2.3.2;

According to the latest information provided by NAS:

-	The cell is not barred, see clause 5.3.1;

-	The cell is part of at least one TA that is not part of the list of "Forbidden Tracking Areas" which belongs to either the selected SNPN or the registered SNPN of the UE.

barred cell:

A cell is barred if it is so indicated in the system information, as specified in TS 38.331 [3].

reserved cell:

A cell is reserved if it is so indicated in system information, as specified in TS 38.331 [3].

Following exception to these definitions are applicable for UEs:

-	if a UE has an ongoing emergency call, all acceptable cells of that PLMN are treated as suitable for the duration of the emergency call.

-	camped on a cell that belongs to a registration area that is forbidden for regional provision of service; a cell that belongs to a registration area that is forbidden for regional provision service (TS 23.122 [9], TS 24.501 [14]) is suitable but provides only limited service.

NOTE 1:	UE is not required to support manual search and selection of PLMN or CAG or SNPN while in RRC CONNECTED state. The UE may use local release of RRC connection to perform manual search if it is not possible to perform the search while RRC connected.

End of this change

Most of the changes above are based on text converged over email discussion in draft running CR R2-1915800. The note above is based on related RAN2#108 agreement. Text change above highlighted in yellow is similar to text in TS 36.304 copied below:

Following exceptions to these definitions are applicable for UEs: - camped on a cell that belongs to a registration area that is forbidden for regional provision of service; a cell that belongs to a registration area that is forbidden for regional provision service ([5], [16]) is suitable but provides only limited service. - as an outcome of the manual CSG selection procedure the UE is allowed to access an acceptable cell which fulfils the cell selection criteria and is not barred or reserved for operator use for UEs not belonging to AC 11 or 15 and inform NAS that access is possible (for location registration procedure). NOTE: UE is not required to support manual search and selection of PLMN or CSGs while in RRC CONNECTED state. The UE may use local release of RRC connection to perform manual search if it is not possible to perform the search while RRC connected. - if a UE has an ongoing emergency call, all acceptable cells of that PLMN are treated as suitable for the duration of the emergency call. - if the UE in RRC_IDLE fulfils the conditions to support sidelink communication or PS related sidelink discovery in limited service state as specified in TS 23.303 [29, 4.5.6], the UE may perform sidelink communication or PS-related sidelink discovery.

4.5-a: Please share your comments for above changes to clause 4.5 in table below.

| Comment # | Company | Detailed description of comment |
| --- | --- | --- |
| 1 | Ericsson | Instead of introducing an exception to the suitable cell definition in case of manual CAG selection we think it may be better if the selected CAG ID is indicated from NAS to AS and taken into account in the cell suitability check. As described in the comment to question 4.1.a the selected CAG ID can either be included in the Allowed CAG list (in this case we don’t need to update the cell suitability check) or indicated as a separate parameter (in this case the cell suitability check need to be updated). Apart from the comment above the changes look fine. |
| @1 | Rapporteur | Incorporating manually selected CAG ID into suitability check implies that manually selected CAG ID impacts suitability after the initial cell selection (because any cell broadcasting the manually selected CAG ID remains suitable afterwards). This is a major technical change and I would prefer to discuss proposal 2 before making such a change. I have removed the exception also from the CR and it is now part of proposal 2. I have added an editor’s note to make sure that we revisit this. |
| 2 | Lenovo | The changes for CAG on suitability check can be simplified by adding definition for CAG member cell in clause 3.1 and just saying: For a CAG cell, the cell is a CAG member cell for the UE. |
| @2 | Rapporteur | The suitability check related text is not stable yet (see comment 1 above). I hope we can wait for the text to stabilize before compressing it. I have added an editor’s note to make sure that we revisit this. |
| 3 | Nokia | It would be good to avoid special handling of manual CAG ID selection at AS layer. (See also comment in 4.1 about selected CAG ID.) |
| @3 | Rapporteur | As you suggested in comment in 4.1, we probably need more discussions about this. I hope it is okay to discuss proposal 2 before making any major changes. Also, please see @1 above. |
| 4 | CATT | For description “suitable cell” part, “ and for that PLMN either - the PLMN-ID of that PLMN is broadcast by the cell with no associated CAG-IDs and CAG-only indication in the UE for that PLMN (TS 23.501 [10]) is absent or false; - Allowed CAG list in the UE for that PLMN (TS 23.501 [10]) includes a CAG-ID broadcast by the cell for that PLMN; “ Actually if PLMN is configured with “CAG-only indication is absent or false” ,UE can camp on it irrespective of if there is associated CAG IDs, We Suggest to change as following to make the border between two conditions more clear, “ --CAG-only indication in the UE for that PLMN (TS 23.501 [10]) is absent or false. --Allowed CAG list in the UE for that PLMN (TS 23.501 [10]) includes a CAG-ID broadcast by the cell for that PLMN and CAG-only indication in the UE for that PLMN (TS 23.501 [10]) is TRUE” |
| @4 | Rapporteur | I think the current text covers the scenario you described. For a UE configured with absent or FALSE CAG-only indication for a PLMN, a cell broadcasting no CAG-IDs can be suitable (because of the first bullet “the PLMN-ID of that PLMN is broadcast by the cell with no associated CAG-IDs and CAG-only indication in the UE for that PLMN (TS 23.501 [10]) is absent or false;”) and a cell broadcasting CAG-IDs can be suitable (since “Allowed CAG list in the UE for that PLMN (TS 23.501 [10]) includes a CAG-ID broadcast by the cell for that PLMN;” is not conditioned on CAG-only indication). |
| 5 | ZTE | Changes look ok. |
| 6 | Huawei | If I understand Ericson’s comment correctly, their concern is that the CAG ID selected by NAS is not embodied in the suitable cell criterion. Maybe the text could be revised to: • the PLMN-ID of that PLMN is broadcast by the cell with no associated CAG-IDs and CAG-only indication in the UE for that PLMN (TS 23.501 [10]) is absent or false; • Allowed CAG list in the UE for that PLMN (TS 23.501 [10]) includes a CAG-ID broadcast by the cell for that PLMN or the selected CAG ID is broadcast by the cell for that PLMN; |
| @6 | Rapporteur | Thanks for the helpful TP. Please see @1 above. |
| 7 | LG | CAG related text - In our view CAG-ID related condition needs to be added into the suitability criteria. Furthermore we think the adding two bullets on the suitability conditions for CAG are both needed, as the rapporteur suggests, because we have two different suitable cases that are different depending on the value of the CAG-only indication for the concerned PLMN. SNPN related text - Instead of duplicating the text again, we prefer to update the existing text, if possible. |
| @7 | Rapporteur | CAG related text: Please see @1 above. SNPN related text: One SNPN related suitability check, I hope we can keep the current text. We went through several iterations of the text during email discussion prior to last meeting. We originally had a combined text and the current separate treatment of SNPN during the last CR discussion citing complexity of the combined text. |

Next Change

## 5.1	PLMN selection and SNPN selection

In the UE not operating in SNPN access mode, the AS shall report available PLMNs and any associated CAG-IDs to the NAS on request from the NAS or autonomously. In the UE operating in SNPN access mode, the AS shall report available SNPNs to the NAS on request from the NAS or autonomously.

During PLMN selection, based on the list of PLMN identities in priority order, the particular PLMN may be selected either automatically or manually. Each PLMN in the list of PLMN identities is identified by a 'PLMN identity'. In the system information on the broadcast channel, the UE can receive one or multiple 'PLMN identity' in a given cell. The result of the PLMN selection performed by NAS (see TS 23.122 [9]) is an identifier of the selected PLMN.

During SNPN selection, based on the list of SNPN identities, the particular SNPN may be selected either automatically or manually. Each SNPN in the list of SNPN identities is identified by a 'SNPN identity'. In the system information on the broadcast channel, the UE can receive one or multiple 'SNPN identity' in a given cell and optionally may receive associated HRNNs. The result of the SNPN selection performed by NAS (see TS 23.122 [9]) is an identifier of the selected SNPN.

Editor’s note: The HRNN related part in above text may need to be updated based on agreements related to HRNN broadcast.

### 5.1.1	Support for PLMN selection

#### 5.1.1.1	General

On request of the NAS, the AS shall perform a search for available PLMNs and report them to NAS.

#### 5.1.1.2	NR case

The UE shall scan all RF channels in the NR bands according to its capabilities to find available PLMNs and available CAGs. On each carrier, the UE shall search for the strongest cell and read its system information, in order to find out which PLMN(s) the cell belongs to and any associated CAG(s). If the UE can read one or several PLMN identities in the strongest cell, each found PLMN (see the PLMN reading in TS 38.331 [3]) shall be reported to the NAS as a high quality PLMN (but without the RSRP value) and any associated CAG-ID, provided that the following high-quality criterion is fulfilled:

1.	For an NR cell, the measured RSRP value shall be greater than or equal to -110 dBm.

Found PLMNs that do not satisfy the high-quality criterion but for which the UE has been able to read the PLMN identities are reported to the NAS together with their corresponding RSRP values and any associated CAG-ID. The quality measure reported by the UE to NAS shall be the same for each PLMN found in one cell.

The search for PLMNs may be stopped on request from the NAS. The UE may optimise PLMN search by using stored information e.g. frequencies and optionally also information on cell parameters from previously received measurement control information elements.

Once the UE has selected a PLMN, the cell selection procedure shall be performed in order to select a suitable cell of that PLMN to camp on.

To support manual CAG selection, the UE shall report available CAG ID(s) together with their HRNN (if broadcast) and PLMN(s) to the NAS. If NAS has selected a CAG and provided this selection to AS, the UE shall search for an acceptable or suitable cell belonging to the selected CAG to camp on.

#### 5.1.1.3	E-UTRA case

Support for PLMN selection in E-UTRA is described in TS 36.304 [7].

### 5.1.X	Support for SNPN selection

#### 5.1.X.1	General

On request of the NAS, the AS shall perform a search for available SNPNs on only NR cells and report them to NAS.

#### 5.1.X.2	NR case

The UE shall scan all RF channels in the NR bands according to its capabilities to find available SNPNs. On each carrier, the UE shall search for the strongest cell and read its system information, in order to find out which SNPN(s) the cell belongs to. If the UE can read one or several SNPN identities in the strongest cell, each found SNPN (see the SNPN reading in TS 38.331 [3]) shall be reported to the NAS. For manual selection, UE shall report available SNPN identifiers together with their HRNN (if broadcast) to the NAS and the search for available SNPNs may be stopped on request of the NAS.

The search for SNPNs may be stopped on request from the NAS. The UE may optimise SNPN search by using stored information e.g. frequencies and optionally also information on cell parameters from previously received measurement control information elements.

Once the UE has selected a SNPN, the cell selection procedure shall be performed in order to select a suitable cell of that SNPN to camp on.

End of this change

Many changes to clause 5.1 above are based on changes proposed in R2-1915384 with a difference that SNPN selection is discussed separately. This separation is especially useful since SNPN selection is different from PLMN selection in following aspects:

- TS 23.122 does not have a notion of high quality SNPN (whereas there is a notion of high quality PLMN)

- TS 23.122 does not consider RSRP values associated with SNPNs (whereas they are considered for PLMNs that are not high quality PLMNs).

The text-change highlighted in yellow above mimics similar sentences for PLMN selection.

5.1-a: Please share your comments for above changes to clause 5.1 in table below.

| Comment # | Company | Detailed description of comment |
| --- | --- | --- |
| @all | Rapporteur | Changes to clause 5.1.1.2 are based on comments for clause 5.X. |
| 1 | Ericsson | If possible, we would prefer to describe PLMN and SNPN selection in the same section to keep the text shorter. If there are differences these could be highlighted in the common section. Why is there a notion of high-quality cell for PLMNs but not for SNPNs? Is this intentional or is it an oversight in 23.122? |
| @1 | Rapporteur | There are differing views on using a common section. So, I have not changed the structure (especially given the differences between SNPN and PLMN selection mentioned above this table). I am not entirely sure about the thinking behind not having high-quality cell for SNPNs. I am guessing CT1 largely wanted to leave it to implementation. |
| 2 | Lenovo | We share the view from Ericsson that the description of PLMN and SNPN selection can be kept in the same sections due to their large similarities. On the changes in 5.1.: The part (“not operating in SNPN access mode”) can be removed, see our comments to 4.1-a. Regarding the “high quality signal” as defined in TS 23.122: when I read the definition, my understanding is that it is related to the RAT and not the PLMN type. And since SNPN is based on NG-RAN, the “high quality signal” should be supported for SNPN as well. High quality signal: The high quality signal limit is used in the PLMN selection procedure. It is defined in the appropriate AS specification: 3GPP TS 43.022 [35] for the GSM radio access technology, 3GPP TS 25.304 [32] for the UMTS radio access technology (FDD or TDD mode), 3GPP TS 36.304 [43] for the E-UTRAN radio access technology (WB-S1 mode, NB-S1 mode, WB-N1 mode or NB-N1 mode), 3GPP TS 36.304 [43] and and 3GPP TS 38.304 [61] for the NG-RAN radio access technology. |
| @2 | Rapporteur | Please see @1 on the structure. Consideration of high quality SNPNs sounds like a major technical change with cross-WG impacts. We may need to discuss it further. Please see proposal 4. |
| 3 | Intel | Suggest replacing ‘On request of the NAS, the AS shall perform a search for available SNPNs and report them to NAS only considering NR cells.’ with the following: On request of the NAS, the AS shall perform a search for available SNPNs on only NR cell and report them to NAS only considering NR cells. |
| @3 | Rapporteur | Done, with one edit (NR cell->NR cells) |
| 4 | Nokia | We are OK with the structure. We think that reporting about HRNNs is missing from 5.1.X.2 |
| @4 | Rapporteur | Did not change the structure in this version. Merged text from 5.1.X.2 above. |
| 5 | CATT | For the description,“In the UE not operating in SNPN access mode, the AS shall report available PLMNs and any associated CAG-IDs to the NAS on request from the NAS or autonomously” We Suggest to change as following, “In the UE not operating in SNPN access mode, the AS shall report available PLMNs and any associated CAG-IDs in case “allowed CAG list” is not empty to the NAS on request from the NAS or autonomously” And We also suggest adding the following sentence in 5.1.1.2: If a CAG ID is provided by NAS as part of PLMN selection, the UE shall search for an acceptable or suitable cell belonging to the provided CAG ID to camp on. When the UE is no longer camped on a cell with the provided CAG ID, AS shall inform NAS. We can add the following sentence in 5.1.X.2: If a SNPN ID is provided by NAS as part of PLMN selection, the UE shall search for an acceptable or suitable cell belonging to the provided SNPN ID to camp on. When the UE is no longer camped on a cell with the provided SNPN ID, AS shall inform NAS. |
| @5 | Rapporteur | Given the above have not been discussed previously, I have added proposals 5-7 below covering the above. We can perhaps introduce text changes after related proposals are discussed. |
| 6 | ZTE | With regards to defining high-quality cell for SNPNS as mentioned by Ericsson and Lenovo, we do not think it is really needed. The high-quality cell is defined for PLMNs and is mentioned in the description of PLMN selection order, i.e. UE will select and attempt registration PLMN/access technology combinations with received high quality signal in random order before looking at other PLMN/access technology combinations in order of decreasing signal quality. While for SNPN selection, it has been defined in TS23.122 that the MS shall select one of those SNPNs in MS implementation specific order automatically or manually at switch-on or recovery from lack of coverage. For user reselection, UE will select SNPN with the following order: - i)one or more SNPNs, which are available and each of them is identified by an SNPN identity in an entry of the "list of subscriber data" in the ME, excluding the previously selected SNPN. The order of the SNPNs is MS implementation specific. - ii)the previously selected SNPN. - Based on the current description in TS23.122, it seems there is no need to specify high quality cell for SNPNs in 38.304and an LS can be sent to CT1 for confirmation. |
| @6 | Rapporteur | Given there are differing views on this, I have added proposal 4 below. |
| 7 | Huawei | From our perspective, it’s ok to have separate subsections (i.e. 5.1.x) for SNPN, but the overview part (i.e. descriptions directly under 5.1) can avoid using repeated text for PLMN and SNPN. Regarding the high quality, we prefer not to capture related description for SNPN before anything is agreed in RAN2. |
| @7 | Rapporteur | I have retained the separate subsections in this version. Given differing views, I added proposal 4 below. Could you clarify how you were thinking of avoiding repeated text? Was it to replace PLMN with PLMN/SNPN. That approach does not look straightforward due to small differences between PLMN selection and SNPN selection (e.g., no use of CAGs and no “priority order” with SNPNs, and no HRNNs with PLMNs) |
| 8 | LG | We think SNPN selection is essential a kind of PLMN selection, and hence we think high quality threshold should be applied to SNPN selection as well. If required, we can send an LS to CT1 to update their spec. |
| @8 | Rapporteur | Consideration of high quality SNPNs sounds like a major technical change with cross-WG impacts. We may need to discuss it further. Please see proposal 4. |

Proposals based on above discussion:

Proposal 4: Void.

Proposal 5: RAN2 should discuss the following:

For a UE not operating in SNPN access mode, the AS need not report CAG-IDs to NAS in case UE does not have any non-empty “allowed CAG list”.

Proposal 6: RAN2 should discuss the following:

If a CAG ID is provided by NAS as part of PLMN selection, the UE shall search for an acceptable or suitable cell belonging to the provided CAG ID to camp on. When the UE is no longer camped on a cell with the provided CAG ID, AS shall inform NAS.

Proposal 7: RAN2 should discuss the following:

If a SNPN ID is provided by NAS as part of PLMN selection, the UE shall search for an acceptable or suitable cell belonging to the provided SNPN ID to camp on. When the UE is no longer camped on a cell with the provided SNPN ID, AS shall inform NAS.

Next Change

### 5.2.1	Introduction

UE shall perform measurements for cell selection and reselection purposes as specified in TS 38.133 [8].

When evaluating Srxlev and Squal of non-serving cells for reselection evaluation purposes, the UE shall use parameters provided by the serving cell and for the final check on cell selection criterion, the UE shall use parameters provided by the target cell for cell reselection.

The NAS can control the RAT(s) in which the cell selection should be performed, for instance by indicating RAT(s) associated with the selected PLMN, and by maintaining a list of forbidden registration area(s) and a list of equivalent PLMNs. The UE shall select a suitable cell based on RRC_IDLE or RRC_INACTIVE state measurements and cell selection criteria.

In order to expedite the cell selection process, stored information for several RATs, if available, may be used by the UE.

When camped on a cell, the UE shall regularly search for a better cell according to the cell reselection criteria. If a better cell is found, that cell is selected. The change of cell may imply a change of RAT. Details on performance requirements for cell reselection can be found in TS 38.133 [8].

The NAS is informed if the cell selection and reselection result in changes in the received system information relevant for NAS.

For normal service, the UE shall camp on a suitable cell, monitor control channel(s) of that cell so that the UE can:

-	receive system information from the PLMN or SNPN; and

-	receive registration area information from the PLMN or SNPN, e.g., tracking area information; and

-	receive other AS and NAS Information; and

-	if registered:

-	receive paging and notification messages from the PLMN or SNPN; and

-	initiate transfer to Connected mode.

For cell selection in multi-beam operations, measurement quantity of a cell is up to UE implementation.

For cell reselection in multi-beam operations, including inter-RAT reselection from E-UTRA to NR, the measurement quantity of this cell is derived amongst the beams corresponding to the same cell based on SS/PBCH block as follows:

-	if nrofSS-BlocksToAverage (maxRS-IndexCellQual in E-UTRA) is not configured in SIB2/SIB4 (SIB24 in E-UTRA); or

-	if absThreshSS-BlocksConsolidation (threshRS-Index in E-UTRA) is not configured in SIB2/SIB4 (SIB24 in E-UTRA); or

-	if the highest beam measurement quantity value is below or equal to absThreshSS-BlocksConsolidation (threshRS-Index in E-UTRA):

-	derive a cell measurement quantity as the highest beam measurement quantity value, where each beam measurement quantity is described in TS 38.215 [11].

-	else:

-	derive a cell measurement quantity as the linear average of the power values of up to nrofSS-BlocksToAverage (maxRS-IndexCellQual in E-UTRA) of highest beam measurement quantity values above absThreshSS-BlocksConsolidation (threshRS-Index in E-UTRA).

End of this change

All changes to clause above are based on changes proposed in R2-1915384 (except that ‘PLMN’ have not been replaced with ‘network’).

5.2.1-a: Please share your comments for above changes to clause 5.2.1 in table below.

| Comment # | Company | Detailed description of comment |
| --- | --- | --- |
|  | Ericsson | Changes look ok. |
|  | Lenovo | Changes look ok. |
|  | ZTE | Changes look ok. |
|  | Huawei | Changes look ok. |
|  | LG | Changes look ok |

Next Change

### 5.2.2	States and state transitions in RRC_IDLE state and RRC_INACTIVE state

Figure 5.2.2-1 shows the states and state transitions and procedures in RRC_IDLE and RRC_INACTIVE. Whenever a new PLMN selection or new SNPN selection is performed, it causes an exit to number 1.

Editor’s note: The change to the figure below assumes that ‘cell selection by leveraging stored information’ discussed in clause 5.3.1 of TS 38..304 applies to SNPNs also.

Figure 5.2.2-1 RRC_IDLE and RRC_INACTIVE Cell Selection and Reselection

End of this change

Changes to the clause above include following changes to Fig 5.2.2.-1:

- updates to parts referring to PLMN to extend it to be applicable to SNPNs (proposed in R2-1915384),

- updates to parts referring USIM to cover cases involve subscription for SNPNs.

Please note that the changes to the figure assume that ‘cell selection by leveraging stored information’ discussed in clause 5.3.1 of TS 38..304 applies to SNPNs also.

5.2.2-a: Please share your comments for above changes to clause 5.2.2 in table below.

| Comment # | Company | Detailed description of comment |
| --- | --- | --- |
| 1 | Ericsson | Changes look ok. |
| 2 | Lenovo | Emergency calls are not supported in SNPN. We wonder whether it needs to be reflected in Figure 5.2.2-1 or the statement in stage 2 is sufficient. |
| @2 | Rapporteur | I have no strong views. Currently it says “emergency calls only” which is not inaccurate. But, I see your point that it could help to be clear that emergency calls are not supported in SNPN AM at all. If others also see a need for change, we can update the text. |
| 3 | Intel | Just for our understanding. Is USIM not used for SNPN UE? Otherwise, why ‘go here when no USIM in the UE and no subscription for an SNPN’ or ‘USIM inserted or SNPN subscription added’? |
| @3 | Rapporteur | My understanding is that USIM can be used for SNPNs. SNPN credentials can be stored outside USIM also. Clause I.2.2 ‘EAP framework, selection of authentication method, and EAP method credentials’ of TS 33.501 says the following: - “The UE and the serving network may support 5G AKA, EAP-AKA', or any other key-generating EAP authentication method.” - “NOTE 2: How credentials for EAP methods other than EAP AKA’ are stored and processed within the UE are out of the scope for standalone non-public networks. Storage and processing of credentials for EAP AKA’ and 5G AKA is described in clause 6 of the present document.” I have added an editor’s note above so that we can revisit this after companies check this issue. |
| 4 | ZTE | Changes look ok. |
| 5 | Huawei | Several descriptions changed from “PLMN” to “PLMN or SNPN” are OK for us. However, the added descriptions related to “SNPN subscription” alongside USIM seem not needed because in our understanding the USIM contains both PLMN information and SNPN information. |
| @5 | Rapporteur | Please see @3. |
| 6 | LG | Have the similar question as Intel on the added text ‘USIM inserted or SNPN subscription added’. |
| @6 | Rapporteur | Please see @3. |

Next Change

#### 5.2.3.X	CAG cells in Cell Selection

In addition to normal cell selection rules a manual selection of CAGs shall be supported by the UE upon request from higher layers as defined in subclause 5.X.

Editor’s note: FFS whether to keep this section if manual CAG selection is treated as part of PLMN selection.

End of this change

Above new clause is similar to clause 5.2.3.3 ‘CSG cells and Hybrid cells in Cell Selection’ in TS 36.304.

5.2.3.X-a: Please share your comments for new clause 5.2.3.X in table below.

| Comment # | Company | Detailed description of comment |
| --- | --- | --- |
| 1 | Ericsson | This section does not seem needed if manual CAG selection is considered part of the PLMN selection and the selected PLMN ID and CAG ID is reported from NAS to AS. |
| 2 | Lenovo | To have this new section looks ok. |
| 3 | Nokia | We agree with Ericsson that this is not needed if the description of PLMN and CAG selection are merged (we prefer merging them). |
| 4 | ZTE | Changes look ok. |
| 5 | Huawei | Changes look ok. |
| 6 | LG | No strong view but fine with having this as a separate section, like we have the similar for CSG cell in LTE. |
| @all | Rapporteur | Given there are differing views on this, we can revisit this after discussing proposal 1. I have added an editor’s note above as a reminder to revisit this. |

Next Change

#### 5.2.4.1	Reselection priorities handling

Absolute priorities of different NR frequencies or inter-RAT frequencies may be provided to the UE in the system information, in the RRCRelease message, or by inheriting from another RAT at inter-RAT cell (re)selection. In the case of system information, an NR frequency or inter-RAT frequency may be listed without providing a priority (i.e. the field cellReselectionPriority is absent for that frequency). If priorities are provided in dedicated signalling, the UE shall ignore all the priorities provided in system information. If UE is in camped on any cell state, UE shall only apply the priorities provided by system information from current cell, and the UE preserves priorities provided by dedicated signalling and deprioritisationReq received in RRCRelease unless specified otherwise. When the UE in camped normally state, has only dedicated priorities other than for the current frequency, the UE shall consider the current frequency to be the lowest priority frequency (i.e. lower than any of the network configured values).

The UE shall only perform cell reselection evaluation for NR frequencies and inter-RAT frequencies that are given in system information and for which the UE has a priority provided.

In case UE receives RRCRelease with deprioritisationReq, UE shall consider current frequency and stored frequencies due to the previously received RRCRelease with deprioritisationReq or all the frequencies of NR to be the lowest priority frequency (i.e. lower than any of the network configured values) while T325 is running irrespective of camped RAT. The UE shall delete the stored deprioritisation request(s) when a PLMN selection or SNPN selection is performed on request by NAS (TS 23.122 [9]).

NOTE:	UE should search for a higher priority layer for cell reselection as soon as possible after the change of priority. The minimum related performance requirements specified in TS 38.133 [8] are still applicable.

The UE shall delete priorities provided by dedicated signalling when:

-	the UE enters a different RRC state; or

-	the optional validity time of dedicated priorities (T320) expires; or

-	a PLMN selection or SNPN selection is performed on request by NAS (TS 23.122 [9]).

NOTE 2:	Equal priorities between RATs are not supported.

The UE shall not consider any black listed cells as candidate for cell reselection.

The UE in RRC_IDLE state shall inherit the priorities provided by dedicated signalling and the remaining validity time (i.e. T320 in NR and E-UTRA), if configured, at inter-RAT cell (re)selection.

NOTE 3:	The network may assign dedicated cell reselection priorities for frequencies not configured by system information.

End of this change

The above text-change was proposed in R2-1915384.

5.2.4.1-a: Please share your comments for above changes to clause 5.2.4.1 in table below.

| Comment # | Company | Detailed description of comment |
| --- | --- | --- |
| 1 | Ericsson | Change look ok. |
| 2 | Lenovo | The condition “or SNPN” needs to be added in the bullet point below: - a PLMN or SNPN selection is performed on request by NAS (TS 23.122 [9]). |
| 3 | Nokia | We have the same comment as Lenovo |
| 4 | ZTE | Agree to add the change from Lenovo. |
| 5 | Huawei | Agree with Lenovo. |
| 6 | LG | Seems fine. Lenovo’s suggestion seems correct |
| @2-6 | Rapporteur | Implemented Lenovo’s suggested change. |

Next Change

#### 5.2.4.4	Cells with cell reservations, access restrictions or unsuitable for normal camping

For the highest ranked cell (including serving cell) according to cell reselection criteria specified in clause 5.2.4.6, for the best cell according to absolute priority reselection criteria specified in clause 5.2.4.5, the UE shall check if the access is restricted according to the rules in clause 5.3.1.

If that cell and other cells have to be excluded from the candidate list, as stated in clause 5.3.1, the UE shall not consider these as candidates for cell reselection. This limitation shall be removed when the highest ranked cell changes.

If the highest ranked cell or best cell according to absolute priority reselection rules is an intra-frequency or inter-frequency cell which is not suitable due to being part of the "list of 5GS forbidden TAs for roaming" or belonging to a PLMN which is not indicated as being equivalent to the registered PLMN or belonging to the selected SNPN or belonging to the registered SNPN, the UE shall not consider this cell and other cells on the same frequency, as candidates for reselection for a maximum of 300 seconds. If the UE enters into state any cell selection, any limitation shall be removed. If the UE is redirected under NR control to a frequency for which the timer is running, any limitation on that frequency shall be removed.

If the highest ranked cell or best cell according to absolute priority reselection rules is an inter-RAT cell which is not suitable due to being part of the "list of forbidden TAs for roaming" or belonging to a PLMN which is not indicated as being equivalent to the registered PLMN, the UE shall not consider this cell and other cells on the same frequency, as candidates for reselection for a maximum of 300 seconds. If the UE enters into state any cell selection, any limitation shall be removed. If the UE is redirected under NR control to a frequency for which the timer is running, any limitation on that frequency shall be removed.

End of this change

The above text-change is loosely based on text proposal in R2-1915384. One main difference from R2-1915384 proposal is that above text does not extend the following behaviour to SNPN AM:

“the UE shall not consider this cell and other cells on the same frequency, as candidates for reselection for a maximum of 300 seconds”.

Companies are encouraged to provide their views on whether the above behaviour is needed in SNPN AM, considering that multiple SNPNs managed by different operators may be deployed on the same frequency.

5.2.4.4-a: Please share your comments for above changes to clause 5.2.4.4 in table below.

| Comment # | Company | Detailed description of comment |
| --- | --- | --- |
| 1 | Ericsson | Disagree with the change. We think the same behaviour should be applied also to UEs in SNPN AM. If multiple SNPNs deployed on the same frequency is an issue, then this can be resolved by removing any frequency restriction at SNPN selection. A similar approach is used e.g. for dedicated frequency priorities in section 5,2.4.1: “The UE shall delete priorities provided by dedicated signalling when: - the UE enters a different RRC state; or - the optional validity time of dedicated priorities (T320) expires; or - a PLMN selection is performed on request by NAS (TS 23.122 [9]).” |
| @1 | Rapporteur | Undid the change |
| 2 | Lenovo | We share the view from Ericsson. We see no technical reason why the 300 seconds rule shouldn’t be applied for SNPN AM as well, even in RAN sharing scenarios. Furthermore, we think the condition below is missing and should be added (copied from TS 36.304): If the highest ranked cell or best cell according to absolute priority reselection rules is a CAG cell which is not suitable due to not being a CAG member cell, the UE shall not consider this cell as candidate for cell reselection but shall continue considering other cells on the same frequency for cell reselection. |
| @2 | Rapporteur | Please see @1. Given that the condition has not been discussed before and there are alternatives (as mentioned by Intel below), we can perhaps discuss it first as part of a proposal and then incorporate it into the CR. Please see Proposal 8. |
| 3 | Intel | This changes probably require more discussion. Other than no change is needed since it should also apply SNPN UE, with RAN sharing among PLMN, SNPN and CAG cell, there may be a need to change the behaviour such that only the concerned cell is barred but other cells within the same frequency as the concerned cell is not barred. Similar sentence has been specified in TS36.304 for CSG cell: If the highest ranked cell or best cell according to absolute priority reselection rules is a CSG cell which is not suitable due to not being a CSG member cell, the UE shall not consider this cell as candidate for cell reselection but shall continue considering other cells on the same frequency for cell reselection. One way is to follow the above like in LTE. Alternative is to follow the IntraFrequencyReselection in the MIB as in Rel-15. Maybe, it would be good to follow the latter? |
| @3 | Rapporteur | Please see @1 and Proposal 8. |
| 4 | Nokia | - We agree with the comments above that all CAG cells that are known by the UE not to be CAG member cells should be excluded from the reselection. - The deployment of networks in the same frequency can also happen with SNPNs as with PLMNs. Therefore, our view is that the same UE behaviour is needed in SNPN access mode as non-SNPN access mode when the best cell does not belong to the registered/selected SNPN. |
| @4 | Rapporteur | - The first comment is addressed in the text changes discussed below this table. - Please see @1. |
| 5 | CATT | Agree with Ericsson |
| @5 | Rapporteur | Please see @1. |
| 6 | ZTE | We share the same understanding from Intel and prefer to have the concerned cell barred and let UE consider other cells on the same frequency for cell reselection to improve the success rate. |
| @6 | Rapporteur | Please see @3. |
| 7 | Huawei | For CAG, agree to add the text suggested by Lenovo. And we think the similar text could be added for SNPN, i.e. only the cell itself is excluded, other cells on the same frequency can still be candidate cells. As pointed out by the rapporteur and Ericsson, different operators may deploy SNPNs on the same frequency, we think it is the same for CAG. So we don’t see why CAG and SNPN should employ different behaviors. |
| @6 | Rapporteur | Please see @2. Many companies see a need for max-300-seconds barring rule to apply to SNPNs also. So, I have not introduced a condition (for not barring) similar to that proposed by Lenovo for CAGs. Instead, I have added proposal 9 to discuss this. |
| 8 | LG | For SNPN, we think 300s barring for that frequency should be applied to keep consistency between PLMN selection and SNPN selection. For CAG cells, we think the same behaviour as for CSG can be applied. That is, searching for other cell on the same frequency in the concerned case should be allowed. This behaviour would be necessary when two uncoordinated CAGs coexist on the same frequency. Applying 300s barring for the concerned case may deprive UE from the chance of accessing the other CAG as well as the chance of access to the intra-frequency public PLMN cell when CAG-only indication is false/absent for the PLMN. |
| @8 | Rapporteur | Please see @2. |

Proposals based on above discussion:

Proposal 8: RAN2 should discuss the following two options:

Optoin a) UE shall exclude cells on the same frequency as the barred cell for cell selection/reselection based on IntraFrequencyReselection in the MIB.

Option b) If the highest ranked cell or best cell according to absolute priority reselection rules is a CAG cell which is not suitable due to not being a CAG member cell, the UE shall not consider this cell as candidate for cell reselection but shall continue considering other cells on the same frequency for cell reselection.

Proposal 9: RAN2 should discuss the following:

For a UE operating in SNPN AM, if the highest ranked cell or best cell according to absolute priority reselection rules is an intra-frequency or inter-frequency cell which is not suitable due to being part of the "list of 5GS forbidden TAs for roaming" or belonging to the selected SNPN or belonging to the registered SNPN, the UE shall not consider this cell and other cells on the same frequency, as candidates for reselection for a maximum of 300 seconds.

Next Change

#### 5.2.4.6	Intra-frequency and equal priority inter-frequency Cell Reselection criteria

The cell-ranking criterion Rs for serving cell and Rn for neighbouring cells is defined by:

Rs = Qmeas,s +Qhyst - Qoffsettemp Rn = Qmeas,n -Qoffset - Qoffsettemp

where:

| Qmeas | RSRP measurement quantity used in cell reselections. |
| --- | --- |
| Qoffset | For intra-frequency: Equals to Qoffsets,n, if Qoffsets,n is valid, otherwise this equals to zero. For inter-frequency: Equals to Qoffsets,n plus Qoffsetfrequency, if Qoffsets,n is valid, otherwise this equals to Qoffsetfrequency. |
| Qoffsettemp | Offset temporarily applied to a cell as specified in TS 38.331 [3]. |

The UE shall perform ranking of all cells that fulfil the cell selection criterion S, which is defined in 5.2.3.2.

Editor’s note: The above text may need to be updated to reflect any agreed details of PCI list of CAG cells.

The cells shall be ranked according to the R criteria specified above by deriving Qmeas,n and Qmeas,s and calculating the R values using averaged RSRP results.

If rangeToBestCell is not configured, the UE shall perform cell reselection to the highest ranked cell. If this cell is found to be not-suitable, the UE shall behave according to clause 5.2.4.4.

If rangeToBestCell is configured, then the UE shall perform cell reselection to the cell with the highest number of beams above the threshold (i.e. absThreshSS-BlocksConsolidation) among the cells whose R value is within rangeToBestCell of the R value of the highest ranked cell. If there are multiple such cells, the UE shall perform cell reselection to the highest ranked cell among them. If this cell is found to be not-suitable, the UE shall behave according to clause 5.2.4.4.

In all cases, the UE shall reselect the new cell, only if the following conditions are met:

-	the	new cell is better than the serving cell according to the cell reselection criteria specified above during a time interval TreselectionRAT;

-	more than 1 second has elapsed since the UE camped on the current serving cell.

NOTE:	If rangeToBestCell is configured but absThreshSS-BlocksConsolidation is not configured on an NR frequency, the UE considers that there is one beam above the threshold for each cell on that frequency.

End of this change

5.2.4.6-a: Please share any comments for clause 5.2.4.6 in table below.

| Comment # | Company | Detailed description of comment |
| --- | --- | --- |
| 1 | Ericsson | Change look ok. |
| 2 | Lenovo | We wonder how PCI list of CAG cells may affect this section. We think at least the following in bold below can be added to the ranking part: The UE shall perform ranking of all cells that fulfil the cell selection criterion S, which is defined in 5.2.3.2, but may exclude all CAG cells that are known by the UE not to be CAG member cells. |
| @2 | Rapporteur | One company has a different view on addition of the new text. So, I have added proposal 10 to discuss this further (removing “all” in the text). We can perhaps update the specification after the discussion. |
| 3 | ZTE | Changes look ok. |
| 4 | Huawei | Agree with Lenovo that PCI list does not have impact on R criteria. But the second suggestion by Lenovo seems unnecessary because it is clear that UE will not measure non CAG member cells in its implementation, let alone ranking them. |
| @4 | Rapporteur | Given the differing views on the second suggestion by Lenovo, I have added proposal 10 to discuss this further. We can perhaps update the specification after the discussion. |
| 5 | Nokia | We think that the CAG cells that are known by the UE not to be CAG member cells shall be excluded from cell ranking. (Similar view as Lenovo, but this should be a mandatory UE behaviour.) |
| @5 | Rapporteur | Given the differing views on suggestion by Lenovo, I have added proposal 10 to discuss this further. We can perhaps update the specification after the discussion. The issue mandatory vs optional exclusion can also be part of the discussion (and is mentioned in the proposal). |

Proposals based on above discussion:

Proposal 10: RAN2 should discuss whether the exclusion below is mandatory or optional:

The UE shall perform ranking of all cells that fulfil the cell selection criterion S, which is defined in 5.2.3.2, but may exclude CAG cells that are known by the UE not to be CAG member cells.

Next Change

#### 5.2.4.X	Cell reselection with CAG cells

In addition to normal cell reselection, a UE may optionally use an autonomous search function to detect CAG cells on serving and non-serving frequencies. However UE shall follow the cell reselection criteria based on dedicated frequency priorities and only follow the autonomous cell search result if the result fulfils also the existing cell reselection criteria based on dedicated frequency priorities.

End of this change

The above new clause is based on similar clause 5.2.4.8 in TS 36.304 which discussed aspects of autonomous search function for CSGs.

5.2.4.X-a: Please share your comments for new clause 5.2.4.X in table below.

| Comment # | Company | Detailed description of comment |
| --- | --- | --- |
| 1 | Ericsson | We probably need to clarify what is meant by considering dedicated frequency priorities. In our understanding the UE is only allowed to reselect to a CAG cell found using autonomous cell search if the frequency of the CAG cell has a priority which is equal or higher than the serving frequency priority. It might also be good to clarify that only suitable cells are considered during the autonomous cell search, i.e. cells which belong to the selected/registered PLMN or one of its equivalent PLMNs and where the CAG ID is part of the UE’s Allowed CAG list. |
| @1 | Rapporteur | Please see if the updated text suggested by Intel addresses your comments. |
| 2 | Lenovo | We think that description needs to be further extended to cover different cases, e.g. - The UE shall disable the autonomous search function for CAG cells if the UE's Allowed CAG list is empty. - UE may use the autonomous search function for non-serving frequencies as well (e.g. for previously visited CAG member cells). |
| @2 | Rapporteur | Please see if the updated text suggested by Intel addresses your comments. I am not sure if mandatory disabling is needed given the feature is optional and details are left to implementation. Please let me know if you think otherwise. |
| 3 | Intel | Agree with Ericsson that this sentence is not very clear to reflect the agreement below: - Allow autonomous cell search even in situations when frequency priorities are broadcast in system information. - UE follows dedicated frequency priorities as in legacy behaviour. If UE run autonomous cell search and at the same time have dedicated frequency priorities, the result from autonomous cell search should not go against that indicated by dedicated frequency priorities (when they are valid). Maybe change to following: In addition to normal cell reselection, a UE may optionally use an autonomous search function to detect CAG cells. However UE shall follow the cell reselection criteria based on dedicated frequency priorities and only follow the autonomous cell search result if the result fulfils also the existing cell reselection criteria based on dedicated frequency priorities. |
| @3 | Rapporteur | Thanks for the TP. Updated the text using the TP also adding “on serving and non-serving frequencies” to address Lenovo’s comment. |
| 4 | CATT | In RAN2#108 meeting, we agreed that the result from autonomous cell search should not go against that indicated by dedicated frequency priorities (when they are valid), but the wording in 5.2.4.X can’t clearly reflect the agreement. |
| @4 | Rapporteur | Please see if the updated text suggested by Intel addresses your comments. |
| 5 | ZTE | We share the same understanding with Intel. |
| 6 | Huawei | Agree with Intel. |
| 7 | LG | Fine with the current text. But some wording improvement to clarify the intention seems also desirable. |
| @5-7 | Rapporteur | Please see @3. |

Next Change

### 5.3.1	Cell status and cell reservations

Cell status and cell reservations are indicated in the MIB or SIB1 message as specified in TS 38.331 [3] by means of three fields:

-	cellBarred (IE type: "barred" or "not barred") 
Indicated in MIB message. In case of multiple PLMNs or NPNs indicated in SIB1, this field is common for all PLMNs and NPNs

-	cellReservedForOperatorUse (IE type: "reserved" or "not reserved") 
Indicated in SIB1 message. In case of multiple PLMNs or NPNs indicated in SIB1, this field is specified per PLMN or per SNPN.

-	cellReservedForOtherUse (IE type: "true") 
Indicated in SIB1 message. In case of multiple PLMNs indicated in SIB1, this field is common for all PLMNs.

-	cellReservedForFutureUse (IE type: "true") 
Indicated in SIB1 message. In case of multiple PLMNs or NPNs indicated in SIB1, this field is common for all PLMNs and NPNs.

Editor’s note: The above text is based on working assumption that the cellReservedForFutureUse is cell specific. This editor’s note can be updated/removed as the working assumption evolves.

When cell status is indicated as "not barred" and "not reserved" for operator use and not "true" for other use and cellReservedForFutureUse IE is not indicated as “true”,

-	All UEs shall treat this cell as candidate during the cell selection and cell reselection procedures.

When cell broadcasts any CAG IDs or NIDs and the cell status is indicated as "not barred" and "not reserved" for operator use and "true" for other use, and cellReservedForFutureUse IE is not indicated as “true”:

-	All UEs shall treat this cell as candidate during the cell selection and cell reselection procedures.

When cell status is indicated as "true" for other use and cell does not broadcast any CAG-IDs or NIDs,

-	The UE shall treat this cell as if cell status is "barred".

When cellReservedForFutureUse IE is indicated as “true”,

-	The UE shall treat this cell as if cell status is "barred".

When cell status is indicated as "true" for other use and cell does not broadcast any CAG-IDs and the UE is not operating in SNPN Access Mode,

-	The UE shall treat this cell as if cell status is "barred".

When cell status is indicated as "not barred" and "reserved" for operator use for any PLMN and not "true" for other use and cellReservedForFutureUse IE is not indicated as “true”,

-	UEs assigned to Access Identity 11 or 15 operating in their HPLMN/EHPLMN shall treat this cell as candidate during the cell selection and reselection procedures if the field cellReservedForOperatorUse for that PLMN set to "reserved".

-	UEs assigned to Access Identity 11 or 15 shall treat this cell as candidate during the cell selection and reselection procedures if the field cellReservedForOperatorUse for selected/registered SNPN is set to "reserved".

-	UEs assigned to an Access Identity 0, 1, 2 and 12 to 14 shall behave as if the cell status is "barred" in case the cell is "reserved for operator use" for the registered PLMN/SNPN or the selected PLMN/SNPN.

NOTE 1:	Access Identities 11, 15 are only valid for use in the HPLMN/ EHPLMN; Access Identities 12, 13, 14 are only valid for use in the home country as specified in TS 22.261 [12].

When cell status "barred" is indicated or to be treated as if the cell status is "barred",

-	The UE is not permitted to select/reselect this cell, not even for emergency calls.

-	The UE shall select another cell according to the following rule:

-	If the cell is to be treated as if the cell status is "barred" due to being unable to acquire the MIB:

-	the UE may exclude the barred cell as a candidate for cell selection/reselection for up to 300 seconds.

-	the UE may select another cell on the same frequency if the selection criteria are fulfilled.

-	else:

-	If the cell is to be treated as if the cell status is "barred" due to being unable to acquire the SIB1 or due to trackingAreaCode being absent in SIB1 as specified in TS 38.331 [3]:

-	The UE may exclude the barred cell as a candidate for cell selection/reselection for up to 300 seconds.

-	If the field intraFreqReselection in MIB message is set to "allowed", the UE may select another cell on the same frequency if re-selection criteria are fulfilled;

-	The UE shall exclude the barred cell as a candidate for cell selection/reselection for 300 seconds.

-	If the field intraFreqReselection in MIB message is set to "not allowed" the UE shall not re-select a cell on the same frequency as the barred cell;

-	The UE shall exclude the barred cell and the cells on the same frequency as a candidate for cell selection/reselection for 300 seconds.

The cell selection of another cell may also include a change of RAT.

End of this change

Many of the above changes have been made to reflect RAN2#108 agreements.

Note that the text highlighted in yellow covers the case of UE operating in SNPN AM and a cell reserved for operator use.

5.3.1-a: Please share your comments for above changes to clause 5.3.1 in table below.

| Comment # | Company | Detailed description of comment |
| --- | --- | --- |
| 1 | Ericsson | Some comments on the text above: - “PLMNs or NPNs” is a bit unclear since we don’t talk about NPNs in the rest of the document. Should we maybe change to “PLMNs and SNPNs”? - The following condition is incomplete: “When cell status is indicated as "not barred" and "not reserved" for operator use and not "true" for other use” and should be changed to: “When cell status is indicated as "not barred" and "not reserved" for operator use and not "true" for other or future use” - It is a bit unclear what “When cell provides access only to SNPNs” means. Maybe better to say “If the cell is reserved for other use and does not broadcast any CAG-IDs”. |
| @1 | Rapporteur | - I am inclined to retain to say SNPN since latest RRC running CR also uses a similar style, e.g., “cellReservedForFutureUse: Indicates whether the cell is reserved, as defined in 38.304 [20] for future use. The field is applicable to all PLMNs and NPNs.”. - Yes, it was incomplete. Fixed it by adding “cellReservedForFutureUse IE is not indicated as “true”,”. - Updated based on the suggestion using slightly different wording. Please review the updated text. |
| 2 | Lenovo | Due to large similarities with legacy part, we think there is no need to duplicate the description wrt access identities, instead it can be merged with the legacy part. |
| @2 | Rapporteur | Updated the text to create the suggested merged description. |
| 3 | Intel | We are also not sure whether a new text for SNPN access mode is needed (the highlighted part) or just add SNPN to the existing text at places where PLMN is specified |
| @3 | Rapporteur | Please see @2. |
| 4 | Nokia | - We feel that description is ambiguous for the case when cell status is "not barred", "not reserved" for operator use, not "true" for other use, “true” for future use, as “When cell status is indicated as "not barred" and "not reserved" for operator use and not "true" for other use, - All UEs shall treat this cell as candidate during the cell selection and cell reselection procedures.” “When cellReservedForFutureUse IE is indicated as “true”, - The UE shall treat this cell as if cell status is "barred".” Our view is that case should be considered barred case, therefore the condition for the 1st citation above should be extended: “When cell status is indicated as "not barred" and "not reserved" for operator use and not "true" for other use and not “true” for future use,” (This also covers the case when future use flag is not present.) - The case when cell broadcasts any CAG-IDs or NIDs and cell status is "not barred", "not reserved" for operator use, "true" for other use, not “true” for future use is not explicitly defined (the cell should be treated as not barred). We propose to explicitly specify that case explicitly by adding the following: When cell broadcasts any CAG IDs or NIDs and the cell status is indicated as "not barred" and "not reserved" for operator use and "true" for other use, and not “true” for future use: - All UEs shall treat this cell as candidate during the cell selection and cell reselection procedures. |
| @4 | Rapporteur | - Agree with the raised issue. Please see (2) in @1. - I have added the proposed text. Agree that the associated condition was not addressed by previous text. |
| 5 | CATT | “cellReservedForOperatorUse (IE type: "reserved" or "not reserved") Indicated in SIB1 message. In case of multiple PLMNs or NPNs indicated in SIB1, this field is specified per PLMN or per NPN.”we suggest to change it to “this field is specified per PLMN or per SNPN.” The reason for this change is that according to agreement for CAG from RAN2#107bis,” SIB1 allows indication of TAC, RANAC, cellIdentity for each CAG. FFS on other IEs. The fields are indicated per PLMN-ID. FFS whether Rel-15 IEs or Rel-16 IEs are used for the indication.”, it means that field like TAC, RANAC, cellIdentity,(also include cellReservedForOperatorUse) should be indicated per PLMN. |
| @5 | Rapporteur | Updated the text based on the suggestion. |
| 6 | ZTE | We agree with CATT that the cellReservedForOperatorUse is per PLMN or per SNPN. |
| @6 | Rapporteur | Please see @5. |
| 7 | Huawei | Agree with Ericsson’s second comment, no strong view on the others. |
| @7 | Rapporteur | Please see (2) in @1. |

Next Change

## 5.4	Tracking Area registration

In the UE, the AS shall report tracking area information to the NAS.

If the UE reads more than one PLMN identity in the current cell, the UE shall report the found PLMN identities that make the cell suitable in the tracking area information to NAS.

If the UE operating in SNPN access mode reads more than one SNPN identity in the current cell, the UE shall report the found SNPN identities that make the cell suitable in the tracking area information to NAS.

The NAS part of the location registration process is specified in TS 23.122 [9].

End of this change

The above changes are based on text changes proposed in R2-1915384.

5.4-a: Please share your comments for above changes to clause 5.4 in table below.

| Comment # | Company | Detailed description of comment |
| --- | --- | --- |
| 1 | Ericsson | The change looks fine although the case when a cell is suitable for more than one SNPN can probably never occur since equivalent SNPNs is currently not supported. |
| @1 | Rapporteur | I have no strong views. I wonder if the plurality helps when registered and selected SNPN are different. |
| 2 | Lenovo | The condition “not operating in SNPN access mode” can be removed, see our comments to 4.1-a. |
| @2 | Rapporteur | I am not convinced about the change and wonder if we are sacrificing clarity (e.g., is it clear that UE in SNPN AM does not have to report found PLMN identities). But, given the support from other companies below and in 4.1-a, I have made the change. |
| 3 | Intel | Similar view as Lenovo |
| @3 | Rapporteur | Please see @2. |
| 4 | ZTE | Changes look ok. |
| 5 | Huawei | Changes look ok. |

Next Change

End of this change

The above text changes are based on agreements from RAN2#108.

5.XY-a: Please share your comments for new clauses 5.X and 5.Y in table below.

| Comment # | Company | Detailed description of comment |
| --- | --- | --- |
| 1 | Ericsson | Section 5.X and 5.Y are not needed if manual CAG selection and manual SNPN selection are treated as part of the PLMN/SNPN selection. |
| @1 | Rapporteur | Removed the sections following suggestions in comment from Nokia below. |
| 2 | Lenovo | OK to have the new section 5.X for manual CAG selection. The description for manual SNPN selection is mostly covered by new section 5.1.X. Therefore, we see no stringent need to have the new section 5.Y. |
| @2 | Rapporteur | Removed 5.Y. Also, removed 5.X by merging it into 5.1.X since three companies proposed to do it. We can revisit this based on discussion of proposal 1. |
| 3 | Nokia | We think that these new sections are not needed: - 5.X (manual CAG selection) should be added to 5.1.1 as the search for PLMNs and CAGs are expected to be performed in a single step. Note also that we have some concerns on the use of selected CAG ID (see our comment in 4.1). - 5.Y (manual SNPN selection) can easily be merged in 5.1.X (SNPN selection) by adding the optional report about HRNNs |
| @3 | Rapporteur | Implemented the suggestions. Please review the updated sections. |
| 4 | CATT | How the UE behaves in the subsequent cell reselection should be clarified. UE can reselect to any cell support CAG Id in “allowed CAG list”,or UE can only reselect to a cell support the CAG ID selected in previous manual CAG selection? |
| @4 | Rapporteur | Yes, this needs more discussion and is considered in proposal 2. |
| 5 | ZTE | Changes look ok. |
| @5 | Rapporteur | Please note that the text has undergone substantial changes. Please share any comments on the latest text. |
| 6 | Huawei | We wonder why the second paragraph of 5.X is not included in the RAN2 #108 agreements, is it due to oversight? |
| @6 | Rapporteur | Second paragraph of 5.X is included in the RAN2 #108 agreements. I am guessing, the question is why there is not a similar paragraph for SNPNs. I think the reason is that the similar paragraph for SNPNs is not strictly needed since the behaviour with a manually selected SNPN and automatically selected SNPN is same. However, second paragraph behaviour with manually selected CAG needs to be called out as it is not discussed elsewhere. |
| 7 | LG | Since we do not think manual SNPN selection needs to be defined as a separate process, the text on manual SNPN selection can be incorporated into 5.1.x “Support for SNPN selection” Since we do not think manual CAG selection needs to be defined as a separate process, the text on manual CAG selection can be incorporated into 5.1.1 as part of PLMN selection. |
| @7 | Rapporteur | Please see @3. |

# 3	Other comments

Please use the following table to share any other comments.

| Comment # | Clause | Company | Detailed description of comment |
| --- | --- | --- | --- |

# 4	Report

Following proposals were identified as part of the above discusison:

Proposal 1:

Void.

Proposal 2: RAN2 should discuss the following related to manual CAG selection:

- Selected CAG ID is indicated from NAS to AS as a parameter separate from Allowed CAG list.

- Selected CAG ID is used for cell selection immediately after manual CAG selection and not used subsequently (except if it is part of Allowed CAG list).

- As an outcome of the manual CAG selection procedure the UE is allowed to access an acceptable cell which fulfils the cell selection criteria and is not barred or reserved for operator use for UEs not belonging to Access Identities 11 or 15 and inform NAS that access is possible (for location registration procedure).

Proposal 3: RAN2 should discuss the following two options about terminology:

- Use “UEs not operating in SNPN access mode”.

- Define PLMN access mode, and use “UEs operating in PLMN access mode”  instead of “UEs not operating in SNPN access mode”.

Proposal 4: Void.

Proposal 5: RAN2 should discuss the following:

For a UE not operating in SNPN access mode, the AS need not report CAG-IDs to NAS in case UE does not have any non-empty “allowed CAG list”.

Proposal 6: RAN2 should discuss the following:

If a CAG ID is provided by NAS as part of PLMN selection, the UE shall search for an acceptable or suitable cell belonging to the provided CAG ID to camp on. When the UE is no longer camped on a cell with the provided CAG ID, AS shall inform NAS.

Proposal 7: RAN2 should discuss the following:

If a SNPN ID is provided by NAS as part of PLMN selection, the UE shall search for an acceptable or suitable cell belonging to the provided SNPN ID to camp on. When the UE is no longer camped on a cell with the provided SNPN ID, AS shall inform NAS.

Proposal 8: RAN2 should discuss the following two options:

Optoin a) UE shall exclude cells on the same frequency as the barred cell for cell selection/reselection based on IntraFrequencyReselection in the MIB.

Option b) If the highest ranked cell or best cell according to absolute priority reselection rules is a CAG cell which is not suitable due to not being a CAG member cell, the UE shall not consider this cell as candidate for cell reselection but shall continue considering other cells on the same frequency for cell reselection.

Proposal 9: RAN2 should discuss the following:

For a UE operating in SNPN AM, if the highest ranked cell or best cell according to absolute priority reselection rules is an intra-frequency or inter-frequency cell which is not suitable due to being part of the "list of 5GS forbidden TAs for roaming" or belonging to the selected SNPN or belonging to the registered SNPN, the UE shall not consider this cell and other cells on the same frequency, as candidates for reselection for a maximum of 300 seconds.

Proposal 10: RAN2 should discuss whether the exclusion below is mandatory or optional:

The UE shall perform ranking of all cells that fulfil the cell selection criterion S, which is defined in 5.2.3.2, but may exclude CAG cells that are known by the UE not to be CAG member cells.

Rapporteur’s view is that proposals 1, 2 and 8 are relatively more important for completion of the CR and to address key gaps in the CR.

# 5	References

- R2-1915800, Running 38.304 CR for PRN,	Qualcomm Incorporated

- R2-1915384, Network and cell (re)selection in SNPN access mode,	Nokia, Nokia Shanghai Bell

# 6	Appendix

## Agreements from RAN2#108 (Nov 2019)

Agreements:

- Access attempts by Rel-15 UEs for emergency services on CAG cell could be allowed based on operator's preference

- cellReservedForOtherUse is used to prevent Rel-15 UEs to access the cell.

- NPN information is outside PLMN-IdentityInfoList as a new Rel-16 IE for NPN-only cell and PLMN+NPN cell (the total number of network IDs is still 12)

Working assumption:

- The new Rel-16 IE with a role similar to role of cellReservedForOtherUse for Rel-15 UEs is cell specific.

Agreements:

1.	At least one of the following conditions must be satisfied for a cell to be considered as suitable by a Rel-16 UE not in SNPN AM:

a.	Cell is part of either the selected PLMN or the registered PLMN or PLMN of the Equivalent PLMN list of the UE for which the PLMN-ID is broadcast by the cell with no associated CAG-IDs and for which CAG-only indication is absent or false;

b.	Cell is part of either the selected PLMN or the registered PLMN or PLMN of the Equivalent PLMN list of the UE for which Allowed CAG list includes a CAG-ID broadcast by the cell.

2.	Each SNPN-only cell is treated by Rel-16 UEs not in SNPN AM as if cell status is barred.

3.	A CAG cell which is not considered as suitable can be an acceptable cell for a Rel-16 UE not in SNPN AM.

4.	A PLMN-only cell or an SNPN+PLMN cell be an acceptable cell for a Rel-16 UE not in SNPN AM for which CAG-only indication is true for any PLMN-ID broadcast by the cell.

5.	The following are necessary conditions for an SNPN cell to be considered as a suitable cell by a Rel-16 UE in SNPN AM:

a.	the cell is part of either the selected SNPN or the registered SNPN of the UE;

b.	the cell is part of at least one TA that is not part of the list of "Forbidden Tracking Areas" which belongs to either the selected SNPN or the registered SNPN of the UE

c.	the cell is not barred,

d.	the cell selection criteria in clause 5.2.3.2 are fulfilled.

Agreements:

- Add the following note in TS 38.304 :

NOTE:	UE is not required to support manual search and selection of PLMN or CAG or SNPN while in RRC CONNECTED state. The UE may use local release of RRC connection to perform manual search if it is not possible to perform the search while RRC connected.

2.	In the UE on request of NAS, the AS shall scan all RF channels in the NR bands according to its capabilities to find available CAGs. On each carrier, the UE shall at least search for the strongest cell, read its system information and report available CAG ID(s) together with their HRNN (if broadcast) and PLMN(s) to the NAS. The search for available CAGs may be stopped on request of the NAS.

If NAS has selected a CAG and provided this selection to AS, the UE shall search for an acceptable or suitable cell belonging to the selected CAG to camp on.

3.	In the UE on request of NAS, the AS shall scan all RF channels in the NR bands according to its capabilities to find available SNPNs. On each carrier, the UE shall at least search for the strongest cell, read its system information and report available SNPN identifiers together with their HRNN (if broadcast) to the NAS. The search for available SNPNs may be stopped on request of the NAS.”

- All the R16 UEs will treat the cell as barred when the legacy IE cellReservedForOtherUse is set to “True” and this cell does not broadcast any CAG-IDs or NIDs.

Agreements:

- Allow autonomous cell search even in situations when frequency priorities are broadcast in system information.

- UE follows dedicated frequency priorities as in legacy behaviour. If UE run autonomous cell search and at the same time have dedicated frequency priorities, the result from autonomous cell search should not go against that indicated by dedicated frequency priorities (when they are valid).

Agreements:

- From RAN2 point of view there is no requirement for CAG ID to be included in RRC signalling at RRC connection establishment.

- For SNPN, include the SNPN ID in the RRCSetupComplete message. Stage 3 detalls are FFS

- For SNPN, there is no need to include SNPN ID in the RRCResumeComplete message since the UE context is known to the network.

- Send a LS to SA3 with Agreement#1 with SA2 and RAN3 in To.

## Agreements from RAN2#107bis (Oct 2019)

Agreements:

- no new mechanism is introduced to handle the priority of a frequency layer of a CAG cell on which the UE is camped (beyond what cellReselectionPriority provides in SIB4 and in RRCRelease).

- the UE can optionally implement an autonomous search function of CAG cells. FFS on the relationship with dedicated priorities.

- reserving a PCI range for CAG cells is purely a deployment issue (does not need to be reflected in the spec)

- the PCI list of CAG cells can optionally be signalled to UEs. FFS on details of the list

- FFS whether proximity indication in CONNECTED mode is needed

- no preliminary access check for CAG cells in CONNECTED mode. The Allowed CAG list is provided to the gNB by the AMF.

- no new mechanism is introduced to handle the priority of a frequency layer of an SNPN cell on which the UE is camped (beyond what cellReselectionPriority provides in SIB4 and in RRCRelease).

- There is no autonomous search function of SNPN cells.

- reserving a PCI range for SNPN cells is purely a deployment issue (does not need to be reflected in the spec)

- FFS whether PCI range of SNPN cells can optionally be signalled to UEs.

- No proximity indication in CONNECTED mode is needed for SNPN.

- no preliminary access check for SNPN cells in CONNECTED mode.

Agreements:

- SIB1 of NPN-only cell prevents access attempts by Rel-15 UEs for normal services.

- SIB1/MIB supports prevention of access attempts by Rel-15 UEs on a SNPN-only cell for emergency services.

- SIB1/MIB supports prevention of access attempts by Rel-15 UEs on a CAG-only cell for emergency services (this does not mean that access attempts by Rel-15 UEs for emergency services on CAG-only cell are always not allowed. This is still FFS.The feasibility of allowing emergency services on CAG-only for Rel-15 UEs will be discussed in the email discussion on RRC aspects/SIB1 design)

- Access attempts by Rel-15 UEs for emergency services on SNPN-only cell are not allowed.

- In a NPN-only cell, access attempts for normal services by Rel-16 UEs without support for NPN is not allowed.

- In a SNPN-only cell, access attempts for emergency services by Rel-16 UEs without support for SNPNs is not allowed.

- For a PLMN+NPN cell, Rel-15 UEs should be able to access PLMNs associated with the cell for normal and/or limited service.

- A new Rel-16 IE is needed with a role similar to role of cellReservedForOtherUse for Rel-15 UEs (FFS whether this will be PLMN specific)

- SIB1 allows indication of TAC, RANAC, cellIdentity per SNPN (per PLMN ID + NID). FFS on other IEs. FFS whether Rel-15 IEs or Rel-16 IEs are used for the indication.

- SIB1 allows indication of TAC, RANAC, cellIdentity for each CAG. FFS on other IEs. The fields are indicated per PLMN-ID. FFS whether Rel-15 IEs or Rel-16 IEs are used for the indication.

Working assumptions:

- NPN information is outside PLMN-IdentityInfoList as a new Rel-16 IE for NPN-only cell and PLMN+NPN cell (the total number of network IDs is still 12)

- Access attempts by Rel-15 UEs for emergency services on CAG-only cell could be allowed based on operator's preference

## Agreements from RAN2#107 (Aug 2019)

Agreements

1	The SNPNs (identified by PLMN ID + NID) are broadcasted in SIB1,

FFS whether this is achieved by extending the legacy network list or by introducing a new SNPN specific network list or both.

2	The size and format of the NID will not be discussed in RAN2 (we will be informed by other groups)

3	Up to 12 different SNPNs can be broadcasted in a cell.

4	If “mixed” network sharing is allowed (i.e. a cell can contain both PLMNs and NPNs), the total number of networks indicated in SIB1 (i.e. #PLMN + #SNPN + #PNI-NPN) shall not exceed 12.

5	If HRNN are broadcast then the HRNN should a be broadcasted in a separate SIB (i.e. different from SIB1).

6	SNPN selection functions similar to normal PLMN selection: AS reports the found SNPNs (identified by PLMN ID + NID) to NAS which selects the network. In case of manual selection, the human readable network name (if broadcasted) may also be provided from AS to NAS.

7	Once the UE has selected an SNPN, cell selection/re-selection is only performed within the SNPN, i.e. a cell is only considered suitable if the broadcasted SNPN identifier matches the selected SNPN.

Agreements

1	The PNI-NPNs (identified by PLMN ID + CAG ID) are broadcasted in SIB1

FFS whether this is achieved by extending the legacy network list or by introducing a new PNI-NPN specific network list or both

2	The size and format of the CAG ID will not be discussed in RAN2 (we will be informed by other groups)

3	Up to 12 different PNI-NPNs can be broadcasted in a cell.

4	If HRNN are broadcast then the HRNN should a be broadcasted in a separate SIB (i.e. different from SIB1).

5	Network selection is triggered by NAS whereby AS reports the available PNI-NPNs (identified by PLMN ID + CAG ID) to NAS which selects the network to use. In case of manual network selection, the human readable network name (if broadcasted) may also be provided from AS to NAS.

6	The Allowed CAG list and “CAG only” indication received from upper layers are taken into account in the cell suitability check during cell selection/re-reselection.

Agreements

1	There is no issue identified to support E1 for Rel-16 UEs.

2	(Regarding question E2) Rel-16 UEs not supporting the CAG feature can camp on a CAG cell as an acceptable cell to obtain limited service

3	There is no issue identified to support RS1 for Rel-16 UEs

4	RS2 and RS3 can be supported from RAN2 point of view

Excerpt from SA2 LS S2-1906814 [1] describing scenarios E1-E2 and RS1-RS3 mentioned in the above agreements are copied below:

SA2 discussed support of the following features for Rel-16 UEs: 1. Support for Emergency services in CAG cells. 2. RAN sharing between PLMNs and Non-Public Networks, including both Standalone NPNs (SNPNs) and Public Network Integrated Non-Public Networks (PNI-NPNs). Regarding Emergency service in CAG cells: E1: SA2 concluded that the UE should be allowed to camp for Emergency services for the case where UE supports the CAG feature, but is not authorized for any of the advertised CAG IDs. E2: SA2 could not conclude whether Rel-16 UEs not supporting the CAG feature should be allowed to camp in a CAG cell in limited service state. There is no SA2 consensus to support this scenario. Regarding RAN sharing: RS1: SA2 concluded that the system architecture should support RAN sharing between a PLMN and an SNPN. This feature should be applicable to Rel-16 UEs that do not support the SNPN feature. RS2: SA2 discussed support for RAN sharing between a PNI-NPN (with CAG) and an SNPN. This feature would be applicable to Rel-16 UEs that support either PNI-NPN with CAG or SNPN or both. However, concerns were raised about the additional complexity in the access stratum to support this scenario. RS3: SA2 could not conclude whether the system architecture should support RAN sharing between a PLMN and a PNI-NPN with CAG i.e. RAN sharing in a cell that acts as a CAG cell for PLMN1 and as a non-CAG cell for PLMN2. There is no SA2 consensus to support this scenario.