3GPP TSG-RAN WG2 #109-e	R2-2001505

Online, 24th Feb - 06th Mar 2020

Agenda Item	: 7.3.2.1.1 (NR_Mob_enh-Core/LTE_feMob-Core)

Source	: LG Electronics Inc.

Title	: Handling of stored PDCP PDUs for DAPS HO

Document for	: Discussion and Decision

Introduction

This contribution shows our view on how to handle the stored PDCP PDUs received from the source cell in the reception buffer when the upper layer requests the PDCP Reconfiguration to release the ciphering, integrity and ROHC

Discussion

In DAPS HO, the PDCP entity can receive the PDCP PDUs from both the source cell and the target cell during HO. With this reason, in RAN2#108 meeting, some agreements were made as follows.

4 For the change from the normal PDCP to DAPS PDCP, UE establishes a ciphering function, integrity protection function and ROHC protocol stack and applies the security algorithms and keys provided by upper layer. 5 When upper layers request a PDCP reconfiguration and the source protocol is released, UE shall performs PDCP reconfiguration from DAPS PDCP to normal PDCP.

According to the above agreements, the PDCP entity receives the PDCP reconfiguration to release the security function, i.e., ciphering and integrity, and ROHC protocol associated with the source cell from the upper layer when releasing the source cell. However, there may be the stored PDCP PDUs received from source cell when releasing the source cell.

For example, AT T=0, the PDCP entity receives the PDCP PDUs 1, 2, 3 from the source cell and the PDCP PDUs 4, 5, 6 from the target cell but the PDCP entity does not receive the PDCP PDU 0. In this case, the PDCP entity performs the security procedure to the PDCP PDUs 1, 2, 3, 4, 5, 6 and stores the these PDCP PDUs in the reception buffer. In this case, AT T=1, if the PDCP entity receives the PDCP reconfiguration to release a security function and ROHC protocol associated with source cell from the upper layer, the PDCP entity releases the security function and ROHC protocol. After that, AT T=2, if the t-Rerodering expires, the PDCP entity should perform decompression to the PDCP PDU 1, 2, 3. However, the PDCP entity cannot perform decompression to the PDCP PDUs 1, 2, 3. This is because the PDCP entity has released the ROHC protocol associated with source cell.

In order to solve the above issue, the straightforward solution is that if the PDCP entity receives the PDCP reconfiguration to release the security function and ROHC protocol associated with source cell, the UE performs the decompression to the PDCP PDUs received from the source cell. After that, the PDCP entity releases the ROHC protocol associated with the source cell.

With above solution, the PDCP PDUs may be successfully decompressed and may be successfully delivered to the upper layer.

Proposal. The PDCP entity performs the decompression to the PDCP PDUs received from the source cell before releasing the ROHC protocol associated with source cell.

Conclusion

In this contribution, we show our view on how to handle the stored PDCP PDUs received from the source cell in the reception buffer when the upper layer requests the PDCP Reconfiguration to release the ciphering, integrity and ROHC.

Proposal. The PDCP entity performs the decompression to the PDCP PDUs received from the source cell before releasing the ROHC protocol associated with source cell.

Text proposal for 38.323 running CR

- 5.1	PDCP entity handling

- 5.1.X	PDCP entity reconfiguration

When upper layers request a PDCP entity reconfiguration and DAPS is configured for a data radio bearer, UE shall:

-	establish a ciphering function for the radio bearer and apply the ciphering algorithm and key provided by upper layers for the ciphering function;

-	establish an integrity protection function for the radio bearer and apply the integrity protection algorithm and key provided by upper layers for the integrity protection function;

-	establish a header compression protocol for the radio bearer and apply the header compression configuration provided by upper layers for the header compression protocol.

When upper layers request a PDCP entity reconfiguration and the associated RLC entity is released for a radio bearer, UE shall:

-	perform header decompression for all stored PDCP SDUs received from the associated RLC entity;

-	release the ciphering function associated to the released RLC entity for the radio bearer;

-	release the integrity protection function associated to the released RLC entity for the radio bearer;

NOTE:	The state variables which control the transmission and reception operation should not be reset, and the timers including t-Reordering and discardTimer keep running during PDCP entity reconfiguration procedure.