3GPP TSG-RAN WG2 Meeting #109 electronic	R2-2002166

24th Feb – 6th Mar 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.304 | CR | 0150 | rev | 1 | Current version: | 15.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of TS 38.304 to introduce IAB |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2020-02-24 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | RAN2 agreed to introduce the IAB supporting indication in the SIB1 to indicate whether cell is a candidate cell for IAB node. Both support of IAB node(s) and the cell status for IAB node(s) is combined in the IE iab-Support, i.e. if the IE is present, the cell supports IABs and the cell is also considered as a candidate for IABs; if the IE is absent, the cell does not support IAB and/or the cell is barred for IAB. In the TS 38.304, the deterimination of cell status for IAB cell should be clarified using the new defined IE in RRC. |  |  |  |  |  |  |  |  |
| Summary of change: |  | In section 5.3.1: Add “iab-Support” to determine the cell status; |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | IAB node can not determine whether the cell supporting IAB and whether the cell is barred for IAB node. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.2, 5.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

--------------------- [Start of 1st change] ---------------------------------

3.2	 Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

AS	Access Stratum

CMAS	Commercial Mobile Alert System

CN	Core Network

DCI	Downlink Control Information

ETWS	Earthquake and Tsunami Warning System

E-UTRA	Evolved UMTS Terrestrial Radio Access

E-UTRAN	Evolved UMTS Terrestrial Radio Access Network

IAB	Integrated Access and Backhaul

IMSI	International Mobile Subscriber Identity

MCC	Mobile Country Code

MICO	Mobile Initiated Connection Only

NAS	Non-Access Stratum

NR	NR Radio Access

PLMN	Public Land Mobile Network

RAT	Radio Access Technology

RNA	RAN-based Notification Area

RNAU	RAN-based Notification Area Update

RRC	Radio Resource Control

UAC	Unified Access Control

UE	User Equipment

UMTS	Universal Mobile Telecommunications System

--------------------- [End of 1st change] ---------------------------------

--------------------- [Start of 2nd change] ---------------------------------

5.3	Cell Reservations and Access Restrictions

5.3.0	Introduction

There are two mechanisms which allow an operator to impose cell reservations or access restrictions. The first mechanism uses indication of cell status and special reservations for control of cell selection and reselection procedures. The second mechanism, referred to as Unified Access Control as specified in TS 38.331 [3], shall allow preventing selected access categories or access identities from sending initial access messages for load control reasons.

5.3.1	Cell status and cell reservations

Cell status and cell reservations are indicated in the MIB or SIB1 message as specified in TS 38.331 [3] by means of following fields:

-	cellBarred (IE type: "barred" or "not barred") 
Indicated in MIB message. In case of multiple PLMNs indicated in SIB1, this field is common for all PLMNs

-	cellReservedForOperatorUse (IE type: "reserved" or "not reserved") 
Indicated in SIB1 message. In case of multiple PLMNs indicated in SIB1, this field is specified per PLMN.

-	cellReservedForOtherUse (IE type: "true") 
Indicated in SIB1 message. In case of multiple PLMNs indicated in SIB1, this field is common for all PLMNs.

NOTE:	For IAB node, it ignores the cellBarred, cellReservedForOperatorUse and cellReservedForOtherUse as defined in TS 38.331 [3].

Editor Notes: This can be updated based on the possible agreements by discussion of “R2-2002058, Summary of 6.1.5.3: SI Broadcast, cell Restrictions/Reservation and Barring, Initial Access, and Connection Setup, Ericsson”.

-	iab-Support (IE type: "true")

Indicated in SIB1 message. In case of multiple PLMNs indicated in SIB1, this field is specified per PLMN. [This field indicates if the cell is barred for IAB node or the cell does not support IAB node, or both. When this field is absent, the IAB node shall treat this cell as if cell status is barred]

Editor Notes: The need of the text in bracket is FFS.

When cell status is indicated as "not barred" and "not reserved" for operator use and not "true" for other use,

-	All UEs shall treat this cell as candidate during the cell selection and cell reselection procedures.

When cell status is indicated as "true" for other use,

-	The UE shall treat this cell as if cell status is "barred".

When cell status is indicated as "not barred" and "reserved" for operator use for any PLMN and not "true" for other use,

-	UEs assigned to Access Identity 11 or 15 operating in their HPLMN/EHPLMN shall treat this cell as candidate during the cell selection and reselection procedures if the field cellReservedForOperatorUse for that PLMN set to "reserved".

-	UEs assigned to an Access Identity 0, 1, 2 and 12 to 14 shall behave as if the cell status is "barred" in case the cell is "reserved for operator use" for the registered PLMN or the selected PLMN.

NOTE 1:	Access Identities 11, 15 are only valid for use in the HPLMN/ EHPLMN; Access Identities 12, 13, 14 are only valid for use in the home country as specified in TS 22.261 [12].

When cell status "barred" is indicated or to be treated as if the cell status is "barred",

-	The UE is not permitted to select/reselect this cell, not even for emergency calls.

-	The UE shall select another cell according to the following rule:

-	If the cell is to be treated as if the cell status is "barred" due to being unable to acquire the MIB:

-	the UE may exclude the barred cell as a candidate for cell selection/reselection for up to 300 seconds.

-	the UE may select another cell on the same frequency if the selection criteria are fulfilled.

-	else:

-	If operating as an IAB-MT and the cell is to be treated as if the cell status is "barred" due to iab-Support being absent in SIB1:

-	The IAB-MT may exclude the barred cell as a candidate for cell selection/reselection for up to 300 seconds;

Editor Notes: FFS on the need of above text.

-	If the cell is to be treated as if the cell status is "barred" due to being unable to acquire the SIB1 or due to trackingAreaCode being absent in SIB1 as specified in TS 38.331 [3]:

-	The UE may exclude the barred cell as a candidate for cell selection/reselection for up to 300 seconds.

-	If the field intraFreqReselection in MIB message is set to "allowed", the UE may select another cell on the same frequency if re-selection criteria are fulfilled;

-	The UE shall exclude the barred cell as a candidate for cell selection/reselection for 300 seconds.

-	If the field intraFreqReselection in MIB message is set to "not allowed" the UE shall not re-select a cell on the same frequency as the barred cell;

-	The UE shall exclude the barred cell and the cells on the same frequency as a candidate for cell selection/reselection for 300 seconds.

The cell selection of another cell may also include a change of RAT.

--------------------------[End of change] ------------------------------