3GPP TSG-RAN2 Meeting #109-e	R2-2000xxx

# Online, , 24th Feb 2020 - 6th Mar 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.355 | CR | 0247 | rev | 1 | Current version: | 15.0.0 |  |
| For HELPon using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed changeaffects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR of TS 37.355 for introducing NavIC in LTE – core part |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Reliance Jio, CEWiT, Huawei, ISRO, MediaTek Inc., Qualcomm Incorporated, Saankhya Labs Private Limited, Tejas Networks Ltd. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | LCS_NAVIC-Core |  |  |  |  |  | Date: |  |  | 2020-02-28 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Introduce the network-assisted NAVigation with Indian Constellation(NavIC) positioning method in LTE |  |  |  |  |  |  |  |  |
| Summary of change: |  | NavIC Navigation Satellite System position related information is introduced based on NavIC ICD. - ICD specification of NavIC is added in section 2 as reference. - The abbreviation of NavIC Navigation Satellite System is added in section 3.2 - All impacted IEs have been pointed out and main parts have been changed for introducing NavIC in 6.5.2. - New Positioning System Information Block Type 2 added for NavIC in section 7.2 New Clock Model, Orbit Model, Almanac Model, UTC model update, Differential corrections and Ionospheric Model related IEs for NavIC are added in 6.5.2. - Migrated TS 36.355 CR to TS 37.355 - GNSS-ClockModel, GNSS-OrbitModel, & GNSS-Almanac IEs updated to be in sync with CR for BDS B1C inclusion. - NavIC PosSIBType2 numbers updated as per outcome of PosSIB numbering harmonisation exercise. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Network-assisted NAVigation with Indian Constellation (NavIC) positioning method will not be supported in LTE. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 3.2, 6.5.2, 7.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 36.331, TS 36.305 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revision of R2-2000153 |  |  |  |  |  |  |  |  |

----------------------------Start of change----------------------

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 36.305: "Stage 2 functional specification of User Equipment (UE) positioning in E-UTRAN".

[3]	3GPP TS 23.271: "Functional stage 2 description of Location Services (LCS)".

[4]	IS-GPS-200, Revision D, Navstar GPS Space Segment/Navigation User Interfaces, March 7th, 2006.

[5]	IS-GPS-705, Navstar GPS Space Segment/User Segment L5 Interfaces, September 22, 2005.

[6]	IS-GPS-800, Navstar GPS Space Segment/User Segment L1C Interfaces, September 4, 2008.

[7]	IS-QZSS, Quasi Zenith Satellite System Navigation Service Interface Specifications for QZSS, Ver.1.1, July 31, 2009.

[8]	Galileo OS Signal in Space ICD (OS SIS ICD), Issue 1.2, February 2014, European Union.

[9]	Global Navigation Satellite System GLONASS Interface Control Document, Version 5.1, 2008.

[10]	Specification for the Wide Area Augmentation System (WAAS), US Department of Transportation, Federal Aviation Administration, DTFA01-96-C-00025, 2001.

[11]	RTCM-SC104, RTCM Recommended Standards for Differential GNSS Service (v.2.3), August 20, 2001.

[12]	3GPP TS 36.331: "Evolved Universal Terrestrial Radio Access (E-UTRA); "Radio Resource Control (RRC); Protocol specification".

[13]	3GPP TS 25.331: "Radio Resource Control (RRC); Protocol Specification".

[14]	3GPP TS 44.031: "Location Services (LCS); Mobile Station (MS) - Serving Mobile Location Centre (SMLC) Radio Resource LCS Protocol (RRLP)".

[15]	3GPP TS 23.032: "Universal Geographical Area Description (GAD)".

[16]	3GPP TS 36.211: "Evolved Universal Terrestrial Radio Access (E-UTRA); Physical Channels and Modulation".

[17]	3GPP TS 36.214: "Evolved Universal Terrestrial Radio Access (E-UTRA); Physical layer – Measurements".

[18]	3GPP TS 36.133: "Evolved Universal Terrestrial Radio Access (E-UTRA); Requirements for support of radio resource management".

[19]	3GPP TS 23.003: "Numbering, addressing and identification".

[20]	OMA-TS-LPPe-V1_0, LPP Extensions Specification, Open Mobile Alliance.

[21]	3GPP TS 36.101: "Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) radio transmission and reception".

[22]	ITU-T Recommendation X.691 (07/2002) "Information technology - ASN.1 encoding rules: Specification of Packed Encoding Rules (PER)" (Same as the ISO/IEC International Standard 8825-2).

[23]	BDS-SIS-ICD-2.0: "BeiDou Navigation Satellite System Signal In Space Interface Control Document Open Service Signal (Version 2.0)", December 2013.

[24]	ATIS-0500027: "Recommendations for Establishing Wide Scale Indoor Location Performance", May 2015.

[25]	Bluetooth Special Interest Group: "Bluetooth Core Specification v4.2", December 2014.

[26]	IEEE 802.11, Part 11: "Wireless LAN Medium Access Control (MAC) and Physical Layer (PHY) Specifications".

[27]	IETF RFC 6225, "Dynamic Host Configuration Protocol Options for Coordinate-Based Location Configuration Information", July 2011.

[28]	3GPP TS 36.213: "Evolved Universal Terrestrial Radio Access (E-UTRA); Physical layer procedures".

[29]	"Earth Gravitational Model 96 (EGM96)", National Geospatial-Intelligence Agency, NASA.

[30]	RTCM Standard 10403.3: "Differential GNSS (Global Navigation Satellite Systems) Services" – Version 3, October 7, 2016.

[31]	IGS ANTEX: "The Antenna Exchanged Format" – version 1.4, September 15, 2010.

[32]	Federal Information Processing Standards Publication 197, "Specification for the ADVANCED ENCRYPTION STANDARD (AES)", November 26, 2001.

[33]	NIST Special Publication 800-38A, "Recommendation for Block Cipher Modes of Operation Methods and Techniques", 2001.

[34]	3GPP TS 38.101-2: "NR; User Equipment (UE) radio transmission and reception; Part 2: Range 2 Standalone".

[35]	3GPP TS 38.331: "NR; Radio Resource Control (RRC); Protocol specification".

[36]	3GPP TS 38.215: "NR; Physical layer measurements".

[37]	3GPP TS 38.101-1: "NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone".

[xx]	IRNSS Signal-In-Space (SPS) Interface Control Document (ICD) for standard positioning service version 1.1, Aug 2017.

----------------------------the next change----------------------

## 3.2	Abbreviations

For the purposes of the present document, the following abbreviations apply.

ADR	Accumulated Delta-Range

A-GNSS	Assisted-GNSS

AP	Access Point

ARFCN	Absolute Radio Frequency Channel Number

ARP	Antenna Reference Point

BDS	BeiDou Navigation Satellite System

BIPM	Bureau International des Poids et Mesures (International Bureau of Weights and Measures)

BSSID	Basic Service Set Identifier

BTS	Base Transceiver Station (GERAN)

CID	Cell-ID (positioning method)

CNAV	Civil Navigation

CRS	Cell-specific Reference Signals

ECEF	Earth-Centered, Earth-Fixed

ECGI	Evolved Cell Global Identifier

ECI	Earth-Centered-Inertial

E-CID	Enhanced Cell-ID (positioning method)

EGNOS	European Geostationary Navigation Overlay Service

E-SMLC	Enhanced Serving Mobile Location Centre

E-UTRA	Evolved Universal Terrestrial Radio Access

E-UTRAN	Evolved Universal Terrestrial Radio Access Network

EOP	Earth Orientation Parameters

EPDU	External Protocol Data Unit

FDMA	Frequency Division Multiple Access

FEC	Forward Error Correction

FKP	(German) Flächen-Korrektur-Parameter (area correction parameter)

FTA	Fine Time Assistance

GAGAN	GPS Aided Geo Augmented Navigation

GLONASS	GLObal'naya NAvigatsionnaya Sputnikovaya Sistema (Engl.: Global Navigation Satellite System)

GNSS	Global Navigation Satellite System

GPS	Global Positioning System

HA GNSS	High-Accuracy GNSS (RTK, PPP)

ICD	Interface Control Document

IGS	International GNSS Service

IOD	Issue of Data

IRNSS	Indian Regional Navigation Satellite System

IS	Interface Specification

LLA	Latitude Longitude Altitude

LPP	LTE Positioning Protocol

LPPa	LTE Positioning Protocol Annex

LSB	Least Significant Bit

MAC	Master Auxiliary Concept

MBS	Metropolitan Beacon System

MO-LR	Mobile Originated Location Request

MSAS	Multi-functional Satellite Augmentation System

MSB	Most Significant Bit

msd	mean solar day

MT-LR	Mobile Terminated Location Request

NAV	Navigation

NavIC	NAVigation with Indian Constellation

NB-IoT	NarrowBand Internet of Things

NCGI	NR Cell Global Identifier

NICT	National Institute of Information and Communications Technology

NI-LR	Network Induced Location Request

NPRS	Narrowband Positioning Reference Signals

NR	NR Radio Access

NRSRP	Narrowband Reference Signal Received Power

NRSRQ	Narrowband Reference Signal Received Quality

NTSC	National Time Service Center of Chinese Academy of Sciences

OSR	Observation Space Representation

OTDOA	Observed Time Difference Of Arrival

PDU	Protocol Data Unit

PPP	Precise Point Positioning

PRB	Physical Resource Block

PRC	Pseudo-Range Correction

PRS	Positioning Reference Signals

posSIB	Positioning System Information Block

PZ-90	Parametry Zemli 1990 Goda – Parameters of the Earth Year 1990

QZS	Quasi Zenith Satellite

QZSS	Quasi-Zenith Satellite System

QZST	Quasi-Zenith System Time

RF	Radio Frequency

RRC	Range-Rate Correction

Radio Resource Control

RSRP	Reference Signal Received Power

RSRQ	Reference Signal Received Quality

RSTD	Reference Signal Time Difference

RTK	Real-Time Kinematic

RTT	Round Trip Time

RU	Russia

SBAS	Space Based Augmentation System

SET	SUPL Enabled Terminal

SFN	System Frame Number

SLP	SUPL Location Platform

SSID	Service Set Identifier

SSR	State Space Representation

SUPL	Secure User Plane Location

SV	Space Vehicle

TB	Terrestrial Beacon

TBS	Terrestrial Beacon System

TLM	Telemetry

TOD	Time Of Day

TOW	Time Of Week

TP	Transmission Point

UDRE	User Differential Range Error

ULP	User Plane Location Protocol

USNO	US Naval Observatory

UT1	Universal Time No.1

UTC	Coordinated Universal Time

WAAS	Wide Area Augmentation System

WGS-84	World Geodetic System 1984

WLAN	Wireless Local Area Network

----------------------------the next change----------------------

#### 6.5.2.1	GNSS Assistance Data

#### –	GNSS-CommonAssistData

The IE GNSS-CommonAssistData is used by the location server to provide assistance data which can be used for any GNSS (e.g., GPS, Galileo, GLONASS, BDS, NavIC, etc.).

-- ASN1START

# GNSS-CommonAssistData ::= SEQUENCE {

gnss-ReferenceTime				GNSS-ReferenceTime					OPTIONAL,	-- Need ON

gnss-ReferenceLocation			GNSS-ReferenceLocation				OPTIONAL,	-- Need ON

gnss-IonosphericModel			GNSS-IonosphericModel				OPTIONAL,	-- Need ON

gnss-EarthOrientationParameters	GNSS-EarthOrientationParameters		OPTIONAL,	-- Need ON

...,

[[

gnss-RTK-ReferenceStationInfo-r15

GNSS-RTK-ReferenceStationInfo-r15	OPTIONAL,	-- Need ON

gnss-RTK-CommonObservationInfo-r15

GNSS-RTK-CommonObservationInfo-r15	OPTIONAL,	-- Cond RTK

gnss-RTK-AuxiliaryStationData-r15

GNSS-RTK-AuxiliaryStationData-r15	OPTIONAL	-- Need ON

]]

}

-- ASN1STOP

| Conditional presence | Explanation |
| --- | --- |
| RTK | The field is mandatory present if the IE GNSS-RTK-Observations is included in IE GNSS-GenericAssistData; otherwise it is not present. |

#### –	GNSS-GenericAssistData

The IE GNSS-GenericAssistData is used by the location server to provide assistance data for a specific GNSS (e.g., GPS, Galileo, GLONASS, BDS, NavIC, etc.). The specific GNSS for which the provided assistance data are applicable is indicated by the IE GNSS-ID and (if applicable) by the IE SBAS-ID. Assistance for up to 16 GNSSs can be provided.

-- ASN1START

# GNSS-GenericAssistData ::= SEQUENCE (SIZE (1..16)) OF GNSS-GenericAssistDataElement

# GNSS-GenericAssistDataElement ::= SEQUENCE {

gnss-ID							GNSS-ID,

sbas-ID							SBAS-ID							OPTIONAL, 	-- Cond GNSS-ID-SBAS

gnss-TimeModels					GNSS-TimeModelList				OPTIONAL,	-- Need ON

gnss-DifferentialCorrections	GNSS-DifferentialCorrections	OPTIONAL,	-- Need ON

gnss-NavigationModel			GNSS-NavigationModel			OPTIONAL,	-- Need ON

gnss-RealTimeIntegrity			GNSS-RealTimeIntegrity			OPTIONAL,	-- Need ON

gnss-DataBitAssistance			GNSS-DataBitAssistance			OPTIONAL,	-- Need ON

gnss-AcquisitionAssistance		GNSS-AcquisitionAssistance		OPTIONAL,	-- Need ON

gnss-Almanac					GNSS-Almanac					OPTIONAL,	-- Need ON

gnss-UTC-Model					GNSS-UTC-Model					OPTIONAL,	-- Need ON

gnss-AuxiliaryInformation		GNSS-AuxiliaryInformation		OPTIONAL,	-- Need ON

...,

[[

bds-DifferentialCorrections-r12

BDS-DifferentialCorrections-r12	OPTIONAL,	-- Cond	GNSS-ID-BDS

bds-GridModel-r12			BDS-GridModelParameter-r12		OPTIONAL	-- Cond	GNSS-ID-BDS

]],

[[

gnss-RTK-Observations-r15	GNSS-RTK-Observations-r15		OPTIONAL,	-- Need ON

glo-RTK-BiasInformation-r15	GLO-RTK-BiasInformation-r15		OPTIONAL,	-- Cond GNSS-ID-GLO

gnss-RTK-MAC-CorrectionDifferences-r15

GNSS-RTK-MAC-CorrectionDifferences-r15

OPTIONAL,	-- Need ON

gnss-RTK-Residuals-r15		GNSS-RTK-Residuals-r15			OPTIONAL,	-- Need ON

gnss-RTK-FKP-Gradients-r15	GNSS-RTK-FKP-Gradients-r15		OPTIONAL,	-- Need ON

gnss-SSR-OrbitCorrections-r15

GNSS-SSR-OrbitCorrections-r15	OPTIONAL,	-- Need ON

gnss-SSR-ClockCorrections-r15

GNSS-SSR-ClockCorrections-r15	OPTIONAL,	-- Need ON

gnss-SSR-CodeBias-r15		GNSS-SSR-CodeBias-r15			OPTIONAL	-- Need ON

]],

[[

navic-DifferentialCorrections-r16	NavIC-DifferentialCorrections-r16

OPTIONAL,	-- Cond	GNSS-ID-NavIC

navic-GridModel-r16					NavIC-GridModelParameter-r16

OPTIONAL	-- Cond	GNSS-ID-NavIC

]]

}

-- ASN1STOP

| Conditional presence | Explanation |
| --- | --- |
| GNSS-ID-SBAS | The field is mandatory present if the GNSS-ID = sbas; otherwise it is not present. |
| GNSS-ID-BDS | The field may be present if the GNSS-ID = bds; otherwise it is not present. |
| GNSS-ID-GLO | The field may be present if the GNSS ID = glonass; otherwise it is not present. |
| GNSS-ID-NAVIC | The field may be present if the GNSS-ID = navic; otherwise it is not present |

#### –	GNSS-PeriodicAssistData

The IE GNSS-PeriodicAssistData is used by the location server to provide control parameters for a periodic assistance data delivery session (e.g., interval and duration) to the target device.

NOTE:	Omission of a particular assistance data type field in IE GNSS-PeriodicAssistData means that the location server does not provide this assistance data type in a data transaction of a periodic assistance data delivery session, as described in clauses 5.2.1a and 5.2.2a. Inclusion of no assistance data type fields in IE GNSS-PeriodicAssistData means that a periodic assistance data delivery session is terminated.

-- ASN1START

GNSS-PeriodicAssistData-r15 ::= SEQUENCE {

gnss-RTK-PeriodicObservations-r15		GNSS-PeriodicControlParam-r15	OPTIONAL,	-- Need ON

glo-RTK-PeriodicBiasInformation-r15		GNSS-PeriodicControlParam-r15	OPTIONAL,	-- Need ON

gnss-RTK-MAC-PeriodicCorrectionDifferences-r15

GNSS-PeriodicControlParam-r15	OPTIONAL,	-- Need ON

gnss-RTK-PeriodicResiduals-r15			GNSS-PeriodicControlParam-r15	OPTIONAL,	-- Need ON

gnss-RTK-FKP-PeriodicGradients-r15		GNSS-PeriodicControlParam-r15	OPTIONAL,	-- Need ON

gnss-SSR-PeriodicOrbitCorrections-r15

GNSS-PeriodicControlParam-r15	OPTIONAL, 	-- Need ON

gnss-SSR-PeriodicClockCorrections-r15

GNSS-PeriodicControlParam-r15	OPTIONAL, 	-- Need ON

gnss-SSR-PeriodicCodeBias-r15			GNSS-PeriodicControlParam-r15	OPTIONAL, 	-- Need ON

...

}

-- ASN1STOP

#### 6.5.2.2	GNSS Assistance Data Elements

< Unchanged parts are omitted >

#### –	GNSS-SystemTime

-- ASN1START

# GNSS-SystemTime ::= SEQUENCE {

gnss-TimeID						GNSS-ID,

gnss-DayNumber					INTEGER (0..32767),

gnss-TimeOfDay					INTEGER (0..86399),

gnss-TimeOfDayFrac-msec			INTEGER (0..999)		OPTIONAL,	-- Need ON

notificationOfLeapSecond		BIT STRING (SIZE(2))	OPTIONAL,	-- Cond gnss-TimeID-glonass

gps-TOW-Assist					GPS-TOW-Assist			OPTIONAL,	-- Cond gnss-TimeID-gps

...

}

-- ASN1STOP

| Conditional presence | Explanation |
| --- | --- |
| gnss-TimeID-glonass | The field may be present if gnss-TimeID=`glonass′; otherwise it is not present. |
| gnss-TimeID-gps | The field may be present if gnss-TimeID=`gps′; otherwise it is not present. |

GNSS-SystemTime field descriptions

gnss-TimeID This field specifies the GNSS for which the GNSS-SystemTime is provided.

gnss-DayNumber This field specifies the sequential number of days (with day count starting at 0) from the origin of the GNSS System Time as follows: GPS, QZSS, SBAS – Days from January 6th 1980 00:00:00 UTC (USNO); Galileo – Days from Galileo System Time (GST) start epoch, defined as 13 seconds before midnight between 21st August and 22nd August 1999; i.e., GST was equal to 13 seconds at August 22nd 1999 00:00:00 UTC; GLONASS – Days from December 31st 1995 21:00:00 UTC (SU), which is local UTC Moscow January 1st 1996 00:00:00, defined as UTC(SU) + 3 hours in [9]; BDS – Days from January 1st 2006 00:00:00 UTC (NTSC). NavIC – Days from the NavIC System Time start epoch, defined as 13 seconds before midnight between 21st August and 22nd August 1999; i.e., the NavIC System Time was equal to 00:00:00 at August 21st, 1999 23:55:47 UTC (BIPM).

gnss-TimeOfDay This field specifies the integer number of seconds from the GNSS day change.

gnss-TimeOfDayFrac-msec This field specifies the fractional part of the gnssTimeOfDay field in 1-milli-seconds resolution. The total GNSS TOD is gnss-TimeOfDay + gnssTimeOfDayFrac-msec.

notificationOfLeapSecond This field specifies the notification of forthcoming leap second correction, as defined by parameter KP in [9, Table 4.7].

gps-TOW-Assist This field contains several fields in the Telemetry (TLM) Word and Handover Word (HOW) that are currently being broadcast by the respective GPS satellites. Combining this information with GPS TOW enables the target device to know the entire 1.2-second (60-bit) pattern of TLM and HOW that is transmitted at the start of each six-second NAV subframe by the particular GPS satellite.

#### –	GPS-TOW-Assist

-- ASN1START

# GPS-TOW-Assist ::= SEQUENCE (SIZE(1..64)) OF GPS-TOW-AssistElement

# GPS-TOW-AssistElement ::= SEQUENCE {

satelliteID		INTEGER (1..64),

tlmWord			INTEGER (0..16383),

antiSpoof		INTEGER (0..1),

alert			INTEGER (0..1),

tlmRsvdBits		INTEGER (0..3),

...

}

-- ASN1STOP

GPS-TOW-Assist field descriptions

satelliteID This field identifies the satellite for which the GPS-TOW-Assist is applicable. This field is identical to the GPS PRN Signal No. defined in [4].

tlmWord This field contains a 14-bit value representing the Telemetry Message (TLM) being broadcast by the GPS satellite identified by the particular satelliteID, with the MSB occurring first in the satellite transmission, as defined in [4].

antiSpoof This field contains the Anti-Spoof flag that is being broadcast by the GPS satellite identified by satelliteID, as defined in [4].

alert This field contains the Alert flag that is being broadcast by the GPS satellite identified by satelliteID, as defined in [4].

tlmRsvdBits This field contains the two reserved bits in the TLM Word being broadcast by the GPS satellite identified by satelliteID, with the MSB occurring first in the satellite transmission, as defined in [4].

#### –	NetworkTime

-- ASN1START

# NetworkTime ::= SEQUENCE {

secondsFromFrameStructureStart				INTEGER(0..12533),

fractionalSecondsFromFrameStructureStart	INTEGER(0..3999999),

frameDrift									INTEGER (-64..63)	OPTIONAL,	-- Cond GNSSsynch

cellID		CHOICE {

eUTRA		SEQUENCE {

physCellId			INTEGER (0..503),

cellGlobalIdEUTRA	CellGlobalIdEUTRA-AndUTRA	OPTIONAL,	-- Need ON

earfcn				ARFCN-ValueEUTRA,

...,

[[ earfcn-v9a0		ARFCN-ValueEUTRA-v9a0 OPTIONAL	-- Cond EARFCN-max

]]

},

uTRA		SEQUENCE {

mode	CHOICE {

fdd		SEQUENCE {

primary-CPICH-Info	INTEGER (0..511),

...

},

tdd		SEQUENCE {

cellParameters		INTEGER (0..127),

...

}

},

cellGlobalIdUTRA	CellGlobalIdEUTRA-AndUTRA	OPTIONAL,	-- Need ON

uarfcn				ARFCN-ValueUTRA,

...

},

gSM			SEQUENCE {

bcchCarrier			INTEGER (0..1023),

bsic				INTEGER (0..63),

cellGlobalIdGERAN	CellGlobalIdGERAN			OPTIONAL,	-- Need ON

...

},

...,

nBIoT-r14	SEQUENCE {

nbPhysCellId-r14	INTEGER (0..503),

nbCellGlobalId-r14	ECGI						OPTIONAL,	-- Need ON

nbCarrierFreq-r14	CarrierFreq-NB-r14,

...

},

nr-r15		SEQUENCE {

nrPhysCellId-r15	INTEGER (0..1007),

nrCellGlobalID-r15	NCGI-r15					OPTIONAL,	-- Need ON

nrARFCN-r15			ARFCN-ValueNR-r15,

...

}

},

...

}

-- ASN1STOP

| Conditional presence | Explanation |
| --- | --- |
| EARFCN-max | The field is mandatory present if the corresponding earfcn (i.e. without suffix) is set to maxEARFCN. Otherwise the field is not present. |
| GNSSsynch | The field is present and set to 0 if NetworkTime is synchronized to gnss-SystemTime; otherwise the field is optionally present, need OR. |

NetworkTimefield descriptions

secondsFromFrameStructureStart This field specifies the number of seconds from the beginning of the longest frame structure in the corresponding air interface. In case of E-UTRA, the SFN cycle length is 10.24 seconds. In case of UTRA, the SFN cycle length is 40.96 seconds. In case of GSM, the hyperfame length is 12533.76 seconds. In case of NB-IoT, the Hyper-SFN cycle lengths is 10485.76 seconds. In case of NR, the SFN cycle length is 10.24 seconds.

fractionalSecondsFromFrameStructureStart This field specifies the fractional part of the secondsFromFrameStructureStart in 250 ns resolution. The total time since the particular frame structure start is secondsFromFrameStructureStart + fractionalSecondsFromFrameStructureStart

frameDrift This field specifies the drift rate of the GNSS-network time relation with scale factor 2-30 seconds/second, in the range from -5.9605e-8 to +5.8673e-8 sec/sec.

cellID This field specifies the cell for which the GNSS–network time relation is provided.

physCellId This field specifies the physical cell identity of the reference cell (E-UTRA), as defined in TS 36.331 [12], for which the GNSS network time relation is provided.

cellGlobalIdEUTRA This field specifies the Evolved Cell Global Identifier (ECGI), the globally unique identity of a cell in E-UTRA, of the reference cell for the GNSS-network time relation, as defined in TS 36.331 [12].

earfcn This field specifies E-ARFCN of the reference cell for the GNSS-network time relation (E-UTRA). In case the server includes earfcn-v9a0, the server shall set the corresponding earfcn (i.e. without suffix) to maxEARFCN.

primary-CPICH-Info This field specifies the physical cell identity of the reference cell (UTRA) for the GNSS-network time relation, as defined in TS 25.331 [13].

cellParameters This field specifies the physical cell identity of the reference cell (UTRA) for the GNSS-network time relation, as defined in TS 25.331 [13].

cellGlobalIdUTRA The filed specifies the global UTRAN Cell Identifier, the globally unique identity of a cell in UTRA, of the reference cell for the GNSS-network time relation, as defined in TS 25.331 [13].

uarfcn This field specifies ARFCN of the reference cell for the GNSS-network time relation (UTRA).

bcchCarrier This field specifies the absolute GSM RF channel number of the BCCH of the reference base station (GERAN) for the GNSS-network time relation, as defined in TS 44.031 [14].

bsic This field specifies the Base Station Identity Code of the reference base station (GERAN) for the GNSS-network time relation, as defined in TS 44.031 [14].

cellGlobalIdGERAN This field specifies the Cell Global Identification (CGI), the globally unique identity of a cell in GERAN, of the reference base station for the GNSS-network time relation.

nbPhysCellId This field specifies the narrowband physical layer cell identity of the NB-IoT reference cell, as defined in TS 36.331 [12], for which the GNSS network time relation is provided.

nbCellGlobalId This field specifies the global cell identifier of the NB-IoT reference cell for which the GNSS-network time relation is provided, as defined in TS 36.331 [12].

nbCarrierFreq This field specifies the carrier frequency of the NB-IoT reference cell for which the GNSS-network time relation is provided.

nrPhysCellId This field specifies the physical cell identity of the reference cell (NR), as defined in TS 38.331 [35], for which the GNSS network time relation is provided.

nrCellGlobalID This field specifies the NR Cell Global Identifier (NCGI) of the reference cell (NR) for the GNSS-network time relation, as defined in TS 38.331 [35].

nrARFCN This field specifies NR-ARFCN of the reference cell (NR) for the GNSS-network time relation.

#### –	GNSS-ReferenceLocation

The IE GNSS-ReferenceLocation is used by the location server to provide the target device with a-priori knowledge of its location in order to improve GNSS receiver performance. The IE GNSS-ReferenceLocation is provided in WGS-84 reference system.

-- ASN1START

# GNSS-ReferenceLocation ::= SEQUENCE {

threeDlocation			EllipsoidPointWithAltitudeAndUncertaintyEllipsoid,

...

}

-- ASN1STOP

#### –	GNSS-IonosphericModel

The IE GNSS-IonosphericModel is used by the location server to provide parameters to model the propagation delay of the GNSS signals through the ionosphere. Proper use of these fields allows a single-frequency GNSS receiver to remove parts of the ionospheric delay from the pseudorange measurements. Two Ionospheric Models are supported: The Klobuchar model as defined in [4], and the NeQuick model as defined in [8].

-- ASN1START

# GNSS-IonosphericModel ::= SEQUENCE {

klobucharModel			KlobucharModelParameter		OPTIONAL,	-- Need ON

neQuickModel			NeQuickModelParameter		OPTIONAL,	-- Need ON

...

}

-- ASN1STOP

#### –	KlobucharModelParameter

-- ASN1START

# KlobucharModelParameter ::= SEQUENCE {

dataID			BIT STRING (SIZE (2)),

alfa0			INTEGER (-128..127),

alfa1			INTEGER (-128..127),

alfa2			INTEGER (-128..127),

alfa3			INTEGER (-128..127),

beta0			INTEGER (-128..127),

beta1			INTEGER (-128..127),

beta2			INTEGER (-128..127),

beta3			INTEGER (-128..127),

...

}

-- ASN1STOP

KlobucharModelParamater field descriptions

dataID When dataID has the value ′11′ it indicates that the parameters have been generated by QZSS, and the parameters have been specialized and are applicable within the area defined in [7]. When dataID has the value ′01′ it indicates that the parameters have been generated by BDS, and UE shall use these parameters according to the description given in 5.2.4.7 in [23]. When the dataID has the value ′10′, it indicates that the parameters have been generated by the NavIC, and UE shall use these parameters according to the description given in [xx]. When dataID has the value ′00′ it indicates the parameters are applicable worldwide [4], [7].

alpha0 This field specifies the 0 parameter of the Klobuchar model, as specified in [4], [23], [xx]. Scale factor 2-30 seconds.

alpha1 This field specifies the 1 parameter of the Klobuchar model, as specified in [4], [23], [xx]. Scale factor 2-27 seconds/semi-circle.

alpha2 This field specifies the 2 parameter of the Klobuchar model, as specified in [4], [23], [xx]. Scale factor 2-24 seconds/semi-circle2.

alpha3 This field specifies the 3 parameter of the Klobuchar model, as specified in [4], [23], [xx]. Scale factor 2-24 seconds/semi-circle3.

beta0 This field specifies the 0 parameter of the Klobuchar model, as specified in [4], [23], [xx]. Scale factor 211 seconds.

beta1 This field specifies the 1 parameter of the Klobuchar model, as specified in [4], [23], [xx]. Scale factor 214 seconds/semi-circle.

beta2 This field specifies the 2 parameter of the Klobuchar model, as specified in [4], [23], [xx]. Scale factor 216 seconds/semi-circle2.

beta3 This field specifies the 3 parameter of the Klobuchar model, as specified in [4], [23], [xx]. Scale factor 216 seconds/semi-circle3.

< Unchanged parts are omitted >

#### –	GNSS-TimeModelList

The IE GNSS-TimeModelList is used by the location server to provide the GNSS-GNSS system time offset between the GNSS system time indicated by IE GNSS-ID in IE GNSS-GenericAssistDataElement to the GNSS system time indicated by IE gnss-TO-ID. Several GNSS-TimeModelElement IEs can be included with different gnss-TO-ID fields. The location server should provide a GNSS-TimeModelList for the same GNSS-ID as the gnss-TimeID in IE GNSS-SystemTime in GNSS-ReferenceTime assistance. If the location server does not provide a GNSS-TimeModelList for the same GNSS-ID as the gnss-TimeID in IE GNSS-SystemTime in GNSS-ReferenceTime assistance the target device assumes tA1 and tA2 are equal to zero.

-- ASN1START

# GNSS-TimeModelList ::= SEQUENCE (SIZE (1..15)) OF GNSS-TimeModelElement

# GNSS-TimeModelElement ::= SEQUENCE {

gnss-TimeModelRefTime		INTEGER (0..65535),

tA0							INTEGER (-67108864..67108863),

tA1							INTEGER (-4096..4095)					OPTIONAL,	-- Need ON

tA2							INTEGER (-64..63)						OPTIONAL,	-- Need ON

gnss-TO-ID					INTEGER (1..15),

weekNumber					INTEGER (0..8191)						OPTIONAL,	-- Need ON

deltaT						INTEGER (-128..127)						OPTIONAL,	-- Need ON

...

}

-- ASN1STOP

GNSS-TimeModelElement field descriptions

gnss-TimeModelRefTime This field specifies the reference time of week for GNSS-TimeModelElement and it is given in GNSS specific system time. Scale factor 24 seconds.

tA0 This field specifies the bias coefficient of the GNSS-TimeModelElement. Scale factor 2-35 seconds.

tA1 This field specifies the drift coefficient of the GNSS-TimeModelElement. Scale factor of 2-51 seconds/second.

tA2 This field specifies the drift rate correction coefficient of the GNSS-TimeModelElement. Scale factor of 2-68 seconds/second2.

gnss-TO-ID This field specifies the GNSS system time of the GNSS for which the GNSS-TimeModelElement is applicable. GNSS-TimeModelElement contains parameters to convert GNSS system time from the system indicated by GNSS-ID to GNSS system time indicated by gnss-TO-ID. The conversion is defined in [4,5,6]. See table of gnss-TO-ID to Indication relation below. NOTE.

weekNumber This field specifies the reference week of the GNSS-TimeModelElement given in GNSS specific system time. The location server should include this field, if tA1 or tA2 is included. Scale factor 1 week.

deltaT This field specifies the integer number of seconds of the GNSS-GNSS time offset provided in the GNSS-TimeModelElement. Scale factor 1 second.

gnss-TO-ID to Indication relation

| Value of gnss-TO-ID | Indication |
| --- | --- |
| 1 | GPS |
| 2 | Galileo |
| 3 | QZSS |
| 4 | GLONASS |
| 5 | BDS |
| 6 | NavIC |
| 7-15 | reserved |

NOTE:	The time relationship between the system time indicated by GNSS-ID and system time indicated by gnss-TO-ID is given by the following equation:

tGNSS = tE - ( A0GGTO + A1GGTO (tE - tGGTO + 604800 (WN - WNGGTO)) + A2GGTO (tE - tGGTO + 
			604800 (WN - WNGGTO))2 )

where

tGNSS		is the system time of week for the GNSS indicated by gnss-TO-ID.
tE			is the system time of week for the GNSS indicated by GNSS-ID.
WN		is the week number of the GNSS system time indicated by GNSS-ID corresponding to the tE. 
tGGTO		is the system time of week for the time model data in the GNSS time indicated by GNSS-ID
			and given by the gnss-TimeModelRefTime field.
WNGGTO	is the week number for the time model data in the GNSS time indicated by GNSS-ID
			corresponding to the tGGTO and given by the weekNumber field.
A0GGTO	is given by the tA0 field.
A1GGTO	is given by the tA1 field.
A2GGTO	is given by the tA2 field.

If the tA1and tA2are not included in the GNSS-TimeModelElement, the target device assumes A1GGTO and A2GGTO are equal to zero.

The GNSS system times in the IE GNSS-TimeModelList and used in the equation above are all given in Time of Week (TOW) and Week Number (WN) in the indicted GNSS specific system time. For conversion between TOW/WN and Day Number/Time of Day (gnss-DayNumber/gnss-TimeOfDay) a GNSS week consists of 7 days since the origin of the particular GNSS System time (with the week number count starting at 0), and a day consists of 86400 seconds.

#### –	GNSS-DifferentialCorrections

The IE GNSS-DifferentialCorrections is used by the location server to provide differential GNSS corrections to the target device for a specific GNSS. Differential corrections can be provided for up to 3 signals per GNSS.

-- ASN1START

# GNSS-DifferentialCorrections ::= SEQUENCE {

dgnss-RefTime		INTEGER (0..3599),

dgnss-SgnTypeList	DGNSS-SgnTypeList,

...

}

# DGNSS-SgnTypeList ::= SEQUENCE (SIZE (1..3)) OF DGNSS-SgnTypeElement

# DGNSS-SgnTypeElement ::= SEQUENCE {

gnss-SignalID		GNSS-SignalID,

gnss-StatusHealth	INTEGER (0..7),

dgnss-SatList		DGNSS-SatList,

...

}

# DGNSS-SatList ::= SEQUENCE (SIZE (1..64)) OF DGNSS-CorrectionsElement

# DGNSS-CorrectionsElement ::= SEQUENCE {

svID				SV-ID,

iod				BIT STRING (SIZE(11)),

udre				INTEGER (0..3),

pseudoRangeCor		INTEGER (-2047..2047),

rangeRateCor		INTEGER (-127..127),

udreGrowthRate		INTEGER (0..7)			OPTIONAL,	-- Need ON

udreValidityTime	INTEGER (0..7)			OPTIONAL,	-- Need ON

...

}

-- ASN1STOP

GNSS-DifferentialCorrections field descriptions

dgnss-RefTime This field specifies the time for which the DGNSS corrections are valid, modulo 1 hour. dgnss-RefTime is given in GNSS specific system time. Scale factor 1-second.

dgnss-SgnTypeList This list includes differential correction data for different GNSS signal types, identified by GNSS-SignalID.

gnss-StatusHealth This field specifies the status of the differential corrections. The values of this field and their respective meanings are defined as in table gnss-StatusHealth Value to Indication relation below. The first six values in this field indicate valid differential corrections. When using the values described below, the "UDRE Scale Factor" value is applied to the UDRE values contained in the element. The purpose is to indicate an estimate in the amount of error in the corrections. The value "110" indicates that the source of the differential corrections (e.g., reference station or external DGNSS network) is currently not being monitored. The value "111" indicates that the corrections provided by the source are invalid, as judged by the source.

dgnss-SatList This list includes differential correction data for different GNSS satellites, identified by SV-ID.

iod This field specifies the Issue of Data field which contains the identity for the GNSS-NavigationModel.

udre This field provides an estimate of the uncertainty (1-) in the corrections for the particular satellite. The value in this field shall be multiplied by the UDRE Scale Factor in the gnss-StatusHealth field to determine the final UDRE estimate for the particular satellite. The meanings of the values for this field are shown in the table udre Value to Indication relation below.

pseudoRangeCor This field specifies the correction to the pseudorange for the particular satellite at dgnss-RefTime, t0. The value of this field is given in meters and the scale factor is 0.32 meters in the range of ±655.04 meters. The method of calculating this field is described in [11]. If the location server has received a request for GNSS assistance data from a target device which included a request for the GNSS Navigation Model and DGNSS, the location server shall determine, for each satellite, if the navigation model stored by the target device is still suitable for use with DGNSS corrections and if so and if DGNSS corrections are supported the location server should send DGNSS corrections without including the GNSS Navigation Model. The iod value sent for a satellite shall always be the IOD value that corresponds to the navigation model for which the pseudo-range corrections are applicable. The target device shall only use the pseudoRangeCorvalue when the IOD value received matches its available navigation model. Pseudo-range corrections are provided with respect to GNSS specific geodetic datum (e.g., PZ-90.02 if GNSS-ID indicates GLONASS). Scale factor 0.32 meters.

rangeRateCor This field specifies the rate-of-change of the pseudorange correction for the particular satellite, using the satellite ephemeris and clock corrections identified by the iod field. The value of this field is given in meters per second and the resolution is 0.032 meters/sec in the range of ±4.064 meters/sec. For some time t1> t0, the corrections for iod are estimated by PRC(t1,IOD) = PRC(t0, IOD) + RRC(t0,IOD)(t1 - t0), and the target device uses this to correct the pseudorange it measures at t1, PRm(t1,IOD), by PR(t1, IOD) = PRm(t1, IOD) + PRC(t1, IOD) . The location server shall always send the RRC value that corresponds to the PRC value that it sends. The target device shall only use the RRC value when the iod value received matches its available navigation model. Scale factor 0.032 meters/second.

udreGrowthRate This field provides an estimate of the growth rate of uncertainty (1-) in the corrections for the particular satellite identified by SV-ID. The estimated UDRE at time value specified in the udreValidityTimet1 is calculated as follows: UDRE(t0+t1) = UDRE(t0) udreGrowthRate , where t0 is the DGNSS Reference Time dgnss-RefTimefor which the corrections are valid, t1 is the udreValidityTime field, UDRE(t0) is the value of the udre field, and udreGrowthRate field is the factor as shown in the table Value of udreGrowthRate to Indication relation below.

udreValidityTime This field specifies the time when the udreGrowthRate field applies and is included if udreGrowthRate is included. The meaning of the values for this field is as shown in the table Value of udreValidityTimeto Indication relation below.

gnss-StatusHealth Value to Indication relation

| gnss-StatusHealth Value | Indication |
| --- | --- |
| 000 | UDRE Scale Factor = 1.0 |
| 001 | UDRE Scale Factor = 0.75 |
| 010 | UDRE Scale Factor = 0.5 |
| 011 | UDRE Scale Factor = 0.3 |
| 100 | UDRE Scale Factor = 0.2 |
| 101 | UDRE Scale Factor = 0.1 |
| 110 | Reference Station Transmission Not Monitored |
| 111 | Data is invalid - disregard |

udre Value to Indication relation

| udre Value | Indication |
| --- | --- |
| 00 | UDRE 1.0 m |
| 01 | 1.0 m < UDRE 4.0 m |
| 10 | 4.0 m < UDRE 8.0 m |
| 11 | 8.0 m < UDRE |

# Value of udreGrowthRate to Indication relation

| Value of udreGrowthRate | Indication |
| --- | --- |
| 000 | 1.5 |
| 001 | 2 |
| 010 | 4 |
| 011 | 6 |
| 100 | 8 |
| 101 | 10 |
| 110 | 12 |
| 111 | 16 |

# Value of udreValidityTimeto Indication relation

| Value of udreValidityTime | Indication [seconds] |
| --- | --- |
| 000 | 20 |
| 001 | 40 |
| 010 | 80 |
| 011 | 160 |
| 100 | 320 |
| 101 | 640 |
| 110 | 1280 |
| 111 | 2560 |

#### –	GNSS-NavigationModel

The IE GNSS-NavigationModel is used by the location server to provide precise navigation data to the GNSS capable target device. In response to a request from a target device for GNSS Assistance Data, the location server shall determine whether to send the navigation model for a particular satellite to a target device based upon factors like the T-Toe limit specified by the target device and any request from the target device for DGNSS (see also GNSS-DifferentialCorrections). GNSS Orbit Model can be given in Keplerian parameters or as state vector in Earth-Centered Earth-Fixed coordinates, dependent on the GNSS-ID and the target device capabilities. The meaning of these parameters is defined in relevant ICDs of the particular GNSS and GNSS specific interpretations apply. For example, GPS and QZSS use the same model parameters but some parameters have a different interpretation [7].

-- ASN1START

# GNSS-NavigationModel ::= SEQUENCE {

nonBroadcastIndFlag	INTEGER (0..1),

gnss-SatelliteList		GNSS-NavModelSatelliteList,

...

}

# GNSS-NavModelSatelliteList ::= SEQUENCE (SIZE(1..64)) OF GNSS-NavModelSatelliteElement

# GNSS-NavModelSatelliteElement ::= SEQUENCE {

svID				SV-ID,

svHealth			BIT STRING (SIZE(8)),

iod				BIT STRING (SIZE(11)),

gnss-ClockModel		GNSS-ClockModel,

gnss-OrbitModel		GNSS-OrbitModel,

...,

[[	svHealthExt-v1240 BIT STRING (SIZE(4))			OPTIONAL		-- Need ON

]]

}

# GNSS-ClockModel ::= CHOICE {

standardClockModelList	StandardClockModelList,			-- Model-1

nav-ClockModel			NAV-ClockModel,					-- Model-2

cnav-ClockModel			CNAV-ClockModel,				-- Model-3

glonass-ClockModel		GLONASS-ClockModel,				-- Model-4

sbas-ClockModel			SBAS-ClockModel,				-- Model-5

...,

bds-ClockModel-r12		BDS-ClockModel-r12,				-- Model-6

navic-ClockModel-r16	NavIC-ClockModel-r16			-- Model-8

}

# GNSS-OrbitModel ::= CHOICE {

keplerianSet			NavModelKeplerianSet,			-- Model-1

nav-KeplerianSet		NavModelNAV-KeplerianSet,		-- Model-2

cnav-KeplerianSet		NavModelCNAV-KeplerianSet,		-- Model-3

glonass-ECEF			NavModel-GLONASS-ECEF,			-- Model-4

sbas-ECEF				NavModel-SBAS-ECEF,				-- Model-5

...,

bds-KeplerianSet-r12	NavModel-BDS-KeplerianSet-r12,	-- Model-6

navic-KeplerianSet-r16	NavModel-NavIC-KeplerianSet-r16	-- Model-8

}

-- ASN1STOP

GNSS-NavigationModel field descriptions

nonBroadcastIndFlag This field indicates if the GNSS-NavigationModel elements are not derived from satellite broadcast data or are given in a format not native to the GNSS. A value of 0 means the GNSS-NavigationModel data elements correspond to GNSS satellite broadcasted data; a value of 1 means the GNSS-NavigationModel data elements are not derived from satellite broadcast.

gnss-SatelliteList This list provides ephemeris and clock corrections for GNSS satellites indicated by SV-ID.

svHealth This field specifiesthe satellite's current health. The health values are GNSS system specific. The interpretation of svHealth depends on the GNSS-ID and is as shown in table GNSS to svHealth Bit String(8) relation below.

Iod This field specifies the Issue of Data and contains the identity for GNSS Navigation Model. In case of broadcasted GPS NAV ephemeris, the iod contains the IODC as described in [4]. In case of broadcasted Modernized GPS ephemeris, the iod contains the 11-bit parameter toe as defined in [4, Table 30-I] [6, Table 3.5-1]. In case of broadcasted SBAS ephemeris, the iod contains the 8 bits Issue of Data as defined in [10] Message Type 9. In case of broadcasted QZSS QZS-L1 ephemeris, the iod contains the IODC as described in [7]. In case of broadcasted QZSS QZS-L1C/L2C/L5 ephemeris, the iod contains the 11-bit parameter toe as defined in [7]. In case of broadcasted GLONASS ephemeris, the iod contains the parameter tb as defined in [9]. In the case of broadcasted Galileo ephemeris, the iod contains the IOD index as described in [8]. In the case of broadcasted BDS ephemeris, the iod contains 11 MSB bits of the toe as definedin [23]. In the case of broadcasted NavIC ephemeris, the iod contains 11 MSB bits of the toe as defined in [xx]. The interpretation of iod depends on the GNSS-ID and is as shown in table GNSS to iod Bit String(11) relation below.

svHealthExt This field specifiesthe satellite's additional current health. The health values are GNSS system specific. The interpretation of svHealthExt depends on the GNSS-ID and is as shown in table GNSS to svHealthExt Bit String(4) relation below.

# GNSS to svHealth Bit String(8) relation

| GNSS | svHealth Bit String(8) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Bit 1 (MSB) | Bit 2 | Bit 3 | Bit 4 | Bit 5 | Bit 6 | Bit 7 | Bit 8 (LSB) |
| GPS L1/CA(1) | SV Health [4] |  |  |  |  |  | '0' (reserved) | '0' (reserved) |
| Modernized GPS(2) | L1C Health [6] | L1 Health [4,5] | L2 Health [4,5] | L5 Health [4,5] | '0' (reserved) | '0' (reserved) | '0' (reserved) | '0' (reserved) |
| SBAS(3) | Ranging On (0),Off(1) [10] | Corrections On(0),Off(1) [10] | Integrity On(0),Off(1)[10] | '0' (reserved) | '0' (reserved) | '0' (reserved) | '0' (reserved) | '0' (reserved) |
| QZSS(4) QZS-L1 | SV Health [7] |  |  |  |  |  | '0' (reserved) | '0' (reserved) |
| QZSS(5) QZS- L1C/L2C/L5 | L1C Health [7] | L1 Health [7] | L2 Health [7] | L5 Health [7] | '0' (reserved) | '0' (reserved) | '0' (reserved) | '0' (reserved) |
| GLONASS | Bn (MSB) [9, page 30] | FT [9, Table 4.4] |  |  |  | '0' (reserved) | '0' (reserved) | '0' (reserved) |
| Galileo [8, clause 5.1.9.3] | E5a Data Validity Status | E5b Data Validity Status | E1-B Data Validity Status | E5a Signal Health Status |  | '0' (reserved) | '0' (reserved) | '0' (reserved) |
| BDS [23] | B1I Health (SatH1) [23] | '0' (reserved) | '0' (reserved) | '0' (reserved) | '0' (reserved) | '0' (reserved) | '0' (reserved) | '0' (reserved) |
| NavIC | L5 health | ‘0’ (reserved) | ‘0’ (reserved) | ‘0’ (reserved) | ‘0’ (reserved) | ‘0’ (reserved) | ‘0’ (reserved) | ‘0’ (reserved) |
| Note 1: If GNSS-ID indicates 'gps', and GNSS Orbit Model-2 is included, this interpretation of svHealth applies. Note 2: If GNSS-ID indicates 'gps', and GNSS Orbit Model-3 is included, this interpretation of svHealth applies. If a certain signal is not supported on the satellite indicated by SV-ID, the corresponding health bit shall be set to '1' (i.e., signal can not be used). Note 3: svHealth in case of GNSS-ID indicates 'sbas' includes the 5 LSBs of the Health included in GEO Almanac Message Parameters (Type 17) [10]. Note 4: If GNSS-ID indicates 'qzss', and GNSS Orbit Model-2 is included, this interpretation of svHealth applies. Note 5: If GNSS-ID indicates 'qzss', and GNSS Orbit Model-3 is included, this interpretation of svHealth applies. |  |  |  |  |  |  |  |  |

# GNSS to iod Bit String(11) relation

| GNSS | iod Bit String(11) |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Bit 1 (MSB) | Bit 2 | Bit 3 | Bit 4 | Bit 5 | Bit 6 | Bit 7 | Bit 8 | Bit 9 | Bit 10 | Bit 11 (LSB) |
| GPS L1/CA | '0' | Issue of Data, Clock [4] |  |  |  |  |  |  |  |  |  |
| Modernized GPS | toe (seconds, scale factor 300, range 0 – 604500) [4,5,6] |  |  |  |  |  |  |  |  |  |  |
| SBAS | '0' | '0' | '0' | Issue of Data ([10], Message Type 9) |  |  |  |  |  |  |  |
| QZSS QZS-L1 | '0' | Issue of Data, Clock [7] |  |  |  |  |  |  |  |  |  |
| QZSS QZS-L1C/L2C/L5 | toe (seconds, scale factor 300, range 0 – 604500) [7] |  |  |  |  |  |  |  |  |  |  |
| GLONASS | '0' | '0' | '0' | '0' | tb (minutes, scale factor 15) [9] |  |  |  |  |  |  |
| Galileo | '0' | IODnav [8] |  |  |  |  |  |  |  |  |  |
| BDS | 11 MSB bits of toe (seconds, scale factor 512, range 0 – 604672) [23] |  |  |  |  |  |  |  |  |  |  |
| NavIC | 11 MSB bits of toe (seconds, scale factor 512) [xx] |  |  |  |  |  |  |  |  |  |  |

# GNSS to svHealthExt Bit String(4) relation

| GNSS | svHealthExt Bit String(4) |  |  |  |
| --- | --- | --- | --- | --- |
|  | Bit 1 (MSB) | Bit 2 | Bit 3 | Bit 4 (LSB) |
| Galileo [8, clause 5.1.9.3] | E5b Signal Health Status |  | E1-B Signal Health Status |  |

#### –	StandardClockModelList

-- ASN1START

# StandardClockModelList ::= SEQUENCE (SIZE(1..2)) OF StandardClockModelElement

# StandardClockModelElement ::= SEQUENCE {

stanClockToc			INTEGER (0..16383),

stanClockAF2			INTEGER (-32..31),

stanClockAF1			INTEGER (-1048576..1048575),

stanClockAF0			INTEGER (-1073741824..1073741823),

stanClockTgd			INTEGER (-512..511)				OPTIONAL,	-- Need ON

sisa					INTEGER (0..255),

stanModelID				INTEGER (0..1)					OPTIONAL,	-- Need ON

...

}

-- ASN1STOP

StandardClockModelList field descriptions

standardClockModelList gnss-ClockModel Model-1 contains one or two clock model elements. If included, clock Model-1 shall be included once or twice depending on the target device capability. If the target device is supporting multiple Galileo signals, the location server shall include both F/Nav and I/Nav clock models ingnss-ClockModel if the location server assumes the target device to perform location information calculation using multiple signals.

stanClockToc Parameter toc defined in [8]. Scale factor 60 seconds.

stanClockAF2 Parameter af2 defined in [8]. Scale factor 2-59 seconds/second2.

stanClockAF1 Parameter af1 defined in [8]. Scale factor 2-46 seconds/second.

stanClockAF0 Parameter af0 defined in [8]. Scale factor 2-34 seconds.

stanClockTgd Parameter TGD, Broadcast Group Delay (BGD), defined in [8]. Scale factor 2-32 seconds. This field is required if the target device supports only single frequency Galileo signal.

sisa Signal-In-Space Accuracy (SISA), defined in [8] clause 5.1.11.

stanModelID This field specifies the identity of the clock model according to the table Value of stanModelID to Identity relation below. This field is required if the location server includes both F/Nav and I/Nav Galileo clock models in gnss-ClockModel.

# Value of stanModelID to Identity relation

| Value of stanModelID | Identity |
| --- | --- |
| 0 | I/Nav (E1,E5b) |
| 1 | F/Nav (E1,E5a) |

#### –	NAV-ClockModel

-- ASN1START

# NAV-ClockModel ::= SEQUENCE {

navToc			INTEGER (0..37799),

navaf2			INTEGER (-128..127),

navaf1			INTEGER (-32768..32767),

navaf0			INTEGER (-2097152..2097151),

navTgd			INTEGER (-128..127),

...

}

-- ASN1STOP

NAV-ClockModel field descriptions

navToc Parameter toc, time of clock (seconds) [4,7] Scale factor 24 seconds.

navaf2 Parameter af2, clock correction polynomial coefficient (sec/sec2) [4,7]. Scale factor 2-55 seconds/second2.

navaf1 Parameter af1, clock correction polynomial coefficient (sec/sec) [4,7]. Scale factor 2-43 seconds/second.

navaf0 Parameter af0, clock correction polynomial coefficient (seconds) [4,7]. Scale factor 2-31 seconds.

navTgd Parameter TGD, group delay (seconds) [4,7]. Scale factor 2-31 seconds.

#### –	CNAV-ClockModel

-- ASN1START

# CNAV-ClockModel ::= SEQUENCE {

cnavToc			INTEGER (0..2015),

cnavTop			INTEGER (0..2015),

cnavURA0		INTEGER (-16..15),

cnavURA1		INTEGER (0..7),

cnavURA2		INTEGER (0..7),

cnavAf2			INTEGER (-512..511),

cnavAf1			INTEGER (-524288..524287),

cnavAf0			INTEGER (-33554432..33554431),

cnavTgd			INTEGER (-4096..4095),

cnavISCl1cp		INTEGER (-4096..4095)			OPTIONAL,	-- Need ON

cnavISCl1cd		INTEGER (-4096..4095)			OPTIONAL,	-- Need ON

cnavISCl1ca		INTEGER (-4096..4095)			OPTIONAL,	-- Need ON

cnavISCl2c		INTEGER (-4096..4095)			OPTIONAL,	-- Need ON

cnavISCl5i5		INTEGER (-4096..4095)			OPTIONAL,	-- Need ON

cnavISCl5q5		INTEGER (-4096..4095)			OPTIONAL,	-- Need ON

...

}

-- ASN1STOP

CNAV-ClockModel field descriptions

cnavToc Parameter toc, clock data reference time of week (seconds) [4,5,6,7]. Scale factor 300 seconds.

cnavTop Parameter top, clock data predict time of week (seconds) [4,5,6,7]. Scale factor 300 seconds

cnavURA0 Parameter URAoc Index, SV clock accuracy index (dimensionless) [4,5,6,7].

cnavURA1 Parameter URAoc1 Index, SV clock accuracy change index (dimensionless) [4,5,6,7].

cnavURA2 Parameter URAoc2 Index, SV clock accuracy change rate index (dimensionless) [4,5,6,7].

cnavAf2 Parameter af2-n, SV clock drift rate correction coefficient (sec/sec2) [4,5,6,7]. Scale factor 2-60 seconds/second2.

cnavAf1 Parameter af1-n, SV clock drift correction coefficient (sec/sec) [4,5,6,7]. Scale factor 2-48 seconds/second.

cnavAf0 Parameter af0-n, SV clock bias correction coefficient (seconds) [4,5,6,7]. Scale factor 2-35 seconds.

cnavTgd Parameter TGD, Group delay correction (seconds) [4,5,6,7]. Scale factor 2-35 seconds.

cnavISCl1cp Parameter ISCL1CP, inter signal group delay correction (seconds) [6,7]. Scale factor 2-35 seconds. The location server should include this field if the target device is GPS capable and supports the L1C signal.

cnavISCl1cd Parameter ISCL1CD, inter signal group delay correction (seconds) [6,7]. Scale factor 2-35 seconds. The location server should include this field if the target device is GPS capable and supports the L1C signal.

cnavISCl1ca Parameter ISCL1C/A, inter signal group delay correction (seconds) [4,5,7]. Scale factor 2-35 seconds. The location server should include this field if the target device is GPS capable and supports the L1CA signal.

cnavISCl2c Parameter ISCL2C, inter signal group delay correction (seconds) [4,5,7]. Scale factor 2-35 seconds. The location server should include this field if the target device is GPS capable and supports the L2C signal.

cnavISCl5i5 Parameter ISCL5I5, inter signal group delay correction (seconds) [5,7]. Scale factor 2-35 seconds. The location server should include this field if the target device is GPS capable and supports the L5 signal.

cnavISCl5q5 Parameter ISCL5Q5, inter signal group delay correction (seconds) [5,7]. Scale factor 2-35 seconds. The location server should include this field if the target device is GPS capable and supports the L5 signal.

#### –	GLONASS-ClockModel

-- ASN1START

# GLONASS-ClockModel ::= SEQUENCE {

gloTau			INTEGER (-2097152..2097151),

gloGamma		INTEGER (-1024..1023),

gloDeltaTau		INTEGER (-16..15)				OPTIONAL,	-- Need ON

...

}

-- ASN1STOP

GLONASS-ClockModel field descriptions

gloTau Parameter n(tb), satellite clock offset (seconds) [9]. Scale factor 2-30 seconds.

gloGamma Parameter n(tb), relative frequency offset from nominal value (dimensionless) [9]. Scale factor 2-40.

gloDeltaTau Parameter n, time difference between transmission in G2 and G1 (seconds) [9]. Scale factor 2-30 seconds. The location server should include this parameter if the target device is dual frequency GLONASS receiver capable.

#### –	SBAS-ClockModel

-- ASN1START

# SBAS-ClockModel ::= SEQUENCE {

sbasTo			INTEGER (0..5399),

sbasAgfo		INTEGER (-2048..2047),

sbasAgf1		INTEGER (-128..127),

...

}

-- ASN1STOP

SBAS-ClockModel field descriptions

sbasTo Parameter t0 [10]. Scale factor 16 seconds.

sbasAgfo Parameter aGfo [10]. Scale factor 2-31 seconds.

sbasAgf1 Parameter aGf1 [10]. Scale factor 2-40 seconds/second.

#### –	BDS-ClockModel

-- ASN1START

# BDS-ClockModel-r12 ::= SEQUENCE {

bdsAODC-r12			INTEGER (0..31),

bdsToc-r12			INTEGER (0..131071),

bdsA0-r12			INTEGER (-8388608..8388607),

bdsA1-r12			INTEGER (-2097152..2097151),

bdsA2-r12			INTEGER (-1024..1023),

bdsTgd1-r12			INTEGER (-512..511),

...

}

-- ASN1STOP

BDS-ClockModel field descriptions

bdsAODC Parameter Age of Data, Clock (AODC), see [23], Table 5-6.

bdsToc Parameter Toc, Time of clock (seconds) [23]. Scale factor 23 seconds.

bdsA0 Parameter a0, Clock correction polynomial coefficient (seconds) [23]. Scale factor 2-33 seconds.

bdsA1 Parameter a1, Clock correction polynomial coefficient (sec/sec) [23]. Scale factor 2-50 sec/sec.

bdsA2 Parameter a2, Clock correction polynomial coefficient (sec/sec2) [23]. Scale factor 2-66 sec/sec2.

bdsTgd1 Parameter Equipment group delay differential TGD1 [23]. Scale factor is 0.1 nanosecond.

#### –	NavIC-ClockModel

-- ASN1START

# NavIC-ClockModel-r16	 ::= SEQUENCE {

# navic-Toc-r16			INTEGER (0..65535),

# navic-af2-r16			INTEGER (-128..127),

# navic-af1-r16			INTEGER (-32768..32767),

# navic-af0-r16			INTEGER (-2097152..2097151),

# navic-Tgd-r16			INTEGER (-128..127),

# ...

}

-- ASN1STOP

NavIC-ClockModel field descriptions

navic-Toc Parameter toc, time of clock (seconds) Table-11 Ref [xx] Scale factor 24 seconds.

navic-af2 Parameter af2, clock correction polynomial coefficient (sec/sec2) [xx]. Scale factor 2-55 seconds/second2.

navic-af1 Parameter af1, clock correction polynomial coefficient (sec/sec) [xx] Scale factor 2-43 seconds/second.

navic-af0 Parameter af0, clock correction polynomial coefficient (seconds) [xx] Scale factor 2-31 seconds.

navic-Tgd Parameter TGD, group delay (seconds) [xx] Scale factor 2-31 seconds.

< Unchanged parts are omitted >

#### –	GNSS-RealTimeIntegrity

The IE GNSS-RealTimeIntegrity is used by the location server to provide parameters that describe the real-time status of the GNSS constellations. GNSS-RealTimeIntegrity data communicates the health of the GNSS signals to the mobile in real-time.

The location server shall always transmit the GNSS-RealTimeIntegrity with the current list of unhealthy signals (i.e., not only for signals/SVs currently visible at the reference location), for any GNSS positioning attempt and whenever GNSS assistance data are sent. If the number of bad signals is zero, then the GNSS-RealTimeIntegrity IE shall be omitted.

-- ASN1START

# GNSS-RealTimeIntegrity ::= SEQUENCE {

gnss-BadSignalList	GNSS-BadSignalList,

...

}

# GNSS-BadSignalList ::= SEQUENCE (SIZE(1..64)) OF BadSignalElement

# BadSignalElement ::= SEQUENCE {

badSVID			SV-ID,

badSignalID		GNSS-SignalIDs	OPTIONAL,	-- Need OP

...

}

-- ASN1STOP

GNSS-RealTimeIntegrity field descriptions

gnss-BadSignalList This field specifies a list of satellites with bad signal or signals.

badSVID This field specifies the GNSS SV-ID of the satellite with bad signal or signals.

badSignalID This field identifies the bad signal or signals of a satellite. This is represented by a bit string in GNSS-SignalIDs, with a one-value at a bit position means the particular GNSS signal type of the SV is unhealthy; a zero-value means healthy. Absence of this field means that all signals on the specific SV are bad.

#### –	GNSS-DataBitAssistance

The IE GNSS-DataBitAssistance is used by the location server to provide data bit assistance data for specific satellite signals for data wipe-off. The data bits included in the assistance data depends on the GNSS and its signal.

-- ASN1START

# GNSS-DataBitAssistance ::= SEQUENCE {

gnss-TOD				INTEGER (0..3599),

gnss-TODfrac			INTEGER (0..999)		OPTIONAL,	-- Need ON

gnss-DataBitsSatList	GNSS-DataBitsSatList,

...

}

# GNSS-DataBitsSatList ::= SEQUENCE (SIZE(1..64))OF GNSS-DataBitsSatElement

# GNSS-DataBitsSatElement ::= SEQUENCE {

svID					SV-ID,

gnss-DataBitsSgnList	GNSS-DataBitsSgnList,

...

}

# GNSS-DataBitsSgnList ::= SEQUENCE (SIZE(1..8)) OF GNSS-DataBitsSgnElement

# GNSS-DataBitsSgnElement ::= SEQUENCE {

gnss-SignalType			GNSS-SignalID,

gnss-DataBits			BIT STRING (SIZE (1..1024)),

...

}

-- ASN1STOP

GNSS-DataBitAssistance field descriptions

gnss-TOD This field specifies the reference time of the first bit of the data in GNSS-DataBitAssistance in integer seconds in GNSS specific system time, modulo 1 hour. Scale factor 1 second.

gnss-TODfrac This field specifies the fractional part of the gnss-TOD in 1-milli-second resolution. Scale factor 1 millisecond. The total GNSS TOD is gnss-TOD + gnss-TODfrac.

gnss-DataBitsSatList This list specifies the data bits for a particular GNSS satellite SV-ID and signal GNSS-SignalID.

svID This field specifies the GNSS SV-ID of the satellite for which the GNSS-DataBitAssistance is given.

gnss-SignalType This field identifies the GNSS signal type of the GNSS-DataBitAssistance.

gnss-DataBits Data bits are contained in GNSS system and data type specific format. In case of GPS L1 C/A, it contains the NAV data modulation bits as defined in [4] . In case of Modernized GPS L1C, it contains the encoded and interleaved modulation symbols as defined in [6] clause 3.2.3.1. In case of Modernized GPS L2C, it contains either the NAV data modulation bits, the FEC encoded NAV data modulation symbols, or the FEC encoded CNAV data modulation symbols, dependent on the current signal configuration of this satellite as defined in [4, Table 3-III]. In case of Modernized GPS L5, it contains the FEC encoded CNAV data modulation symbols as defined in [5]. In case of SBAS, it contains the FEC encoded data modulation symbols as defined in [10]. In case of QZSS QZS-L1, it contains the NAV data modulation bits as defined in [7] clause 5.2. In case of QZSS QZS-L1C, it contains the encoded and interleaved modulation symbols as defined in [7] clause 5.3. In case of QZSS QZS-L2C, it contains the encoded modulation symbols as defined in [7] clause 5.5. In case of QZSS QZS-L5, it contains the encoded modulation symbols as defined in [7] clause 5.6. In case of GLONASS, it contains the 100 sps differentially Manchester encoded modulation symbols as defined in [9] clause 3.3.2.2. In case of Galileo, it contains the FEC encoded and interleaved modulation symbols. The logical levels 1 and 0 correspond to signal levels -1 and +1, respectively. In case of BDS, it contains the encoded and interleaved modulation symbols as defined in [23, clause 5.1.3]. In case of the NavIC, it contains the FEC encoded and interleaved Navigation symbols as defined in [xx].

< Unchanged parts are omitted >

#### –	GNSS-Almanac

The IE GNSS-Almanac is used by the location server to provide the coarse, long-term model of the satellite positions and clocks. The meaning of these parameters is defined in relevant ICDs of the particular GNSS and GNSS specific interpretations apply. For example, GPS and QZSS use the same model parameters but some parameters have a different interpretation [7]. GNSS-Almanac is useful for receiver tasks that require coarse accuracy, such as determining satellite visibility. The model is valid for up to a few weeks, typically. Since it is a long-term model, the field should be provided for all satellites available in the GNSS constellation (i.e., not only for SVs visible at the reference location and including SVs flagged as unhealthy in almanac). The completeAlmanacProvided field indicates whether or not the location server provided almanacs for the complete GNSS constellation.

-- ASN1START

# GNSS-Almanac ::= SEQUENCE {

weekNumber					INTEGER (0..255)	OPTIONAL,	-- Need ON

toa						INTEGER (0..255)	OPTIONAL,	-- Need ON

ioda						INTEGER (0..3)		OPTIONAL,	-- Need ON

completeAlmanacProvided		BOOLEAN,

gnss-AlmanacList			GNSS-AlmanacList,

...,

[[	toa-ext-v1240			INTEGER (256..1023)	OPTIONAL,	-- Need ON

ioda-ext-v1240			INTEGER (4..15)		OPTIONAL	-- Need ON

]],

[[

weekNumber-ext-r16		INTEGER (256..8191)			OPTIONAL,	-- Need ON

toa-ext2-r16			INTEGER (256..65535)		OPTIONAL	-- Need ON

]]

}

# GNSS-AlmanacList ::= SEQUENCE (SIZE(1..64)) OF GNSS-AlmanacElement

# GNSS-AlmanacElement ::= CHOICE {

keplerianAlmanacSet				AlmanacKeplerianSet,		-- Model-1

keplerianNAV-Almanac			AlmanacNAV-KeplerianSet,	-- Model-2

keplerianReducedAlmanac			AlmanacReducedKeplerianSet,	-- Model-3

keplerianMidiAlmanac			AlmanacMidiAlmanacSet,		-- Model-4

keplerianGLONASS				AlmanacGLONASS-AlmanacSet,	-- Model-5

ecef-SBAS-Almanac				AlmanacECEF-SBAS-AlmanacSet,-- Model-6

...,

keplerianBDS-Almanac-r12		AlmanacBDS-AlmanacSet-r12,	-- Model-7

keplerianNavIC-Almanac-r16		AlmanacNavIC-AlmanacSet-r16	-- Model-8

}

-- ASN1STOP

GNSS-Almanac field descriptions

weekNumber, weekNumber-ext This field specifies the almanac reference week number in GNSS specific system time to which the almanac reference time toa is referenced, modulo 256 weeks. Either weekNumber or weekNumber-ext is required for non-GLONASS GNSS. Note, in case of Galileo, the almanac reference week number WNa natively contains only the 2 LSB's [8], clause 5.1.10]. In case of NavIC, the almanac reference week number is defined in [xx].

toa, toa-ext, toa-ext2 In case of GNSS-ID does not indicate Galileo or NavIC, this field specifies the almanac reference time given in GNSS specific system time, in units of seconds with a scale factor of 212.toa is required for non-GLONASS GNSS when the toa-ext2 is not present. In case of GNSS-ID does indicate Galileo, this field specifies the almanac reference time given in GNSS specific system time, in units of seconds with a scale factor of 600 seconds. Either toa or toa-ext is required for Galileo GNSS. In case of GNSS-ID indicate NavIC, this field specifies the almanac reference time given in GNSS specific system time, in units of seconds with a scale factor of 16 seconds[xx]. Either toa or toa-ext2 is required for NavIC GNSS.

ioda, ioda-ext This field specifies the issue of data. Either ioda or ioda-ext is required for Galileo GNSS.

completeAlmanacProvided If set to TRUE, the gnss-AlmanacList contains almanacs for the complete GNSS constellation indicated by GNSS-ID.

gnss-AlmanacList This list contains the almanac model for each GNSS satellite in the GNSS constellation.

< Unchanged parts are omitted >

#### –	AlmanacNavIC-AlmanacSet

-- ASN1START

# AlmanacNavIC-AlmanacSet-r16	 ::= SEQUENCE {

# svID-r16					SV-ID,

# navic-AlmToa-r16			INTEGER (0..65535)			OPTIONAL,	-- Cond NotSameForAllSV

# navic-AlmE-r16				INTEGER (0..65535),

# navic-AlmOMEGADOT-r16		INTEGER (-32768..32767),

# navic-AlmSqrtA-r16			INTEGER (0..16777215),

# navic-AlmOMEGAo-r16			INTEGER (-8388608..8388607),

# navic-AlmOmega-r16			INTEGER (-8388608..8388607),

# navic-AlmMo-r16				INTEGER (-8388608..8388607),

# navic-Almaf0-r16			INTEGER (-1024..1023),

# navic-Almaf1-r16			INTEGER (-1024..1023),

# ...

}

-- ASN1STOP

| Conditional presence | Explanation |
| --- | --- |
| NotSameForAllSV | This field may be present if the toa is not the same for all SVs; otherwise it is not present and the toa is provided in GNSS-Almanac. |

AlmanacNavIC-AlmanacSetfield descriptions

svID This field identifies the satellite for which the Almanac model is given

navic-AlmToa This field provides the time of almanac set Scale factor 16sec.

navic-AlmE Parameter e, eccentricity, dimensionless Scale factor 2-21.

navic-AlmOMEGADOT Parameter , rate of right ascension, semi-circles/sec Scale factor 2-38 semi-circles/second

navic-AlmSqrtA Parameter , square root of the semi-major axis, meters1/2 Scale factor 2-11 meters1/2.

navic-AlmOMEGAo Parameter 0, longitude of ascending node of orbit plane at weekly epoch, semi-circles Scale factor 2-23 semi-circles.

navic-AlmOmega Parameter , argument of perigee semi-circles Scale factor 2-23 semi-circles.

navic-AlmMo Parameter M0, mean anomaly at reference time semi-circles Scale factor 2-23 semi-circles.

navic-Almaf0 Parameter af0, apparent satellite clock correction seconds Scale factor 2-20 seconds.

navic-Almaf1 Parameter af1, apparent satellite clock correction sec/sec Scale factor 2-38 semi-circles seconds/second.

#### –	GNSS-UTC-Model

The IE GNSS-UTC-Model is used by the location server to provide several sets of parameters needed to relate GNSS system time to Universal Time Coordinate (UTC), as defined in [4], [5], [6], [7], [8], [9], [10], [23], [xx].

The UTC time standard, UTC(k), is GNSS specific. E.g., if GNSS-ID indicates GPS, GNSS-UTC-Model contains a set of parameters needed to relate GPS system time to UTC(USNO); if GNSS-ID indicates QZSS, GNSS-UTC-Model contains a set of parameters needed to relate QZST to UTC(NICT); if GNSS-ID indicates GLONASS, GNSS-UTC-Model contains a set of parameters needed to relate GLONASS system time to UTC(RU); if GNSS-ID indicates SBAS, GNSS-UTC-Model contains a set of parameters needed to relate SBAS network time for the SBAS indicated by SBAS-ID to the UTC standard defined by the UTC Standard ID; if GNSS-ID indicates BDS, GNSS-UTC-Model contains a set of parameters needed to relate BDS system time to UTC (NTSC), if the GNSS-ID indicates the NavIC, the GNSS-UTC-Model contains a set of parameters needed to relate the NavIC system time to the UTC (BIPM).

-- ASN1START

# GNSS-UTC-Model ::= CHOICE {

utcModel1			UTC-ModelSet1,			-- Model-1

utcModel2			UTC-ModelSet2,			-- Model-2

utcModel3			UTC-ModelSet3,			-- Model-3

utcModel4			UTC-ModelSet4,			-- Model-4

...,

utcModel5-r12		UTC-ModelSet5-r12		-- Model-5

}

-- ASN1STOP

< Unchanged parts are omitted >

#### –	UTC-ModelSet2

-- ASN1START

# UTC-ModelSet2 ::= SEQUENCE {

utcA0				INTEGER (-32768..32767),

utcA1				INTEGER (-4096..4095),

utcA2				INTEGER (-64..63),

utcDeltaTls			INTEGER (-128..127),

utcTot				INTEGER (0..65535),

utcWNot				INTEGER (0..8191),

utcWNlsf			INTEGER (0..255),

utcDN				BIT STRING (SIZE(4)),

utcDeltaTlsf		INTEGER (-128..127),

...,

[[

# utcWNlsf-ext-r16	INTEGER (256..8191)	OPTIONAL	-- Need ON

# ]]

}

-- ASN1STOP

UTC-ModelSet2 field descriptions

utcA0 Parameter A0-n, bias coefficient of GNSS time scale relative to UTC time scale (seconds) [4,5,6,7, xx]. Scale factor 2-35 seconds.

utcA1 Parameter A1-n, drift coefficient of GNSS time scale relative to UTC time scale (sec/sec) [4,5,6,7, xx]. Scale factor 2-51 seconds/second.

utcA2 Parameter A2-n, drift rate correction coefficient of GNSS time scale relative to UTC time scale (sec/sec2) [4,5,6,7, xx]. Scale factor 2-68 seconds/second2.

utcDeltaTls Parameter tLS, current or past leap second count (seconds) [4,5,6,7, xx]. Scale factor 1 second.

utcTot Parameter tot, time data reference time of week (seconds) [4,5,6,7, xx]. Scale factor 24 seconds.

utcWNot Parameter WNot, time data reference week number (weeks) [4,5,6,7, xx]. Scale factor 1 week.

UtcWNlsf, utcWNlsf-ext Parameter WNLSF, leap second reference week number (weeks) [4,5,6,7, xx]. If the field utcWNlsf-ext is present, the field utcWNlsf shall be ignored by the receiver. Scale factor 1 week.

utcDN Parameter DN, leap second reference day number (days) [4,5,6,7, xx]. Scale factor 1 day.

utcDeltaTlsf Parameter tLSF, current or future leap second count (seconds) [4,5,6,7, xx]. Scale factor 1 second.

< Unchanged parts are omitted >

#### –	NavIC-DifferentialCorrections

The IE NavIC-DifferentialCorrections parameters provide users with sets of correction terms that apply to the clock and ephemeris data transmitted by other satellites in the AutoNav mode as defined in [xx] under clause 6.2.6.

-- ASN1START

# NavIC-DifferentialCorrections-r16 ::= SEQUENCE {

navic-RefTOWC-r16			INTEGER (0..50400),

navic-CorrectionListAutoNav-r16	NavIC-CorrectionListAutoNav-r16,

...

}

# NavIC-CorrectionListAutoNav-r16 ::= SEQUENCE (SIZE (1..64)) OF NavIC-CorrectionElementAutoNav-r16

# NavIC-CorrectionElementAutoNav-r16 ::= SEQUENCE {

svID						SV-ID,

navic-Tod-r16				INTEGER (0..65535),

navic-iodec-r16				INTEGER (0..255),

navic-UDRAI-r16				INTEGER (-16..15),

navic-UDRArateI-r16			INTEGER (-16..15),

navic-EDC-r16				NavIC-EDC-r16,

navic-CDC-r16				NavIC-CDC-r16,

...

}

# NavIC-EDC-r16 ::= SEQUENCE {

navic-AlphaEDC-r16				INTEGER (-8192..8191),

navic-BetaEDC-r16				INTEGER (-8192..8191),

navic-GammaEDC-r16				INTEGER (-16384..16383),

navic-AoIcorrection-r16			INTEGER (-2048..2047),

navic-AoRAcorrection-r16		INTEGER (-2048..2047),

navic-SemiMajorcorrection-r16	INTEGER (-2048..2047),

...

}

# NavIC-CDC-r16 ::= SEQUENCE {

navic-ClockBiasCorrection-r16	INTEGER (-4096..4095),

navic-ClockDriftCorrection-r16	INTEGER (-128..127),

...

}

-- ASN1STOP

NavIC-DifferentialCorrections field descriptions

navic-RefTOWC The transmission timing of the navigation message provided through the Time of Week Count (TOWC) corresponding to the given set of grid ionospheric parameters. It indicates the number of 12 second counts represented in 17 bits. The TOW count value ranges from 1 to 50400 to cover one entire week. The Time of Week (TOW) in seconds is obtained by multiplying TOWC with 12 as defined in [xx], clause 5.7.

navic-Tod This field indicates the NavIC Time of Differential Correction in seconds. Scale factor 16 seconds

navic-iodec This field indicates Issue of Data Ephemeris and Clock which provides the user with a convenient means of detecting any change in the ephemeris and clock parameters as described under clause 6.2.1.3 in [xx]

navic-UDRAI This field indicates the index for the User Differential Range Accuracy (in meters) value which enables users to estimate the accuracy obtained after differential corrections are applied as described under clause 6.2.6 in [xx]

navic-UDRArateI This field indicates the index for the change rate of User Differential Range Accuracy (meters/sec)value which enables users to estimate the accuracy obtained after differential corrections are applied as described under clause 6.2.6 in [xx]

navic-AlphaEDC This field indicates the Alpha correction to Ephemeris parameter (Δα), which is one of the six keplerian elements defining the ephemeris differential corrections (EDC) for NavIC as defined under clause 6.1.3.5 in [xx]. Scale factor 2–34

navic-BetaEDC This field indicates Beta correction to Ephemeris parameter (Δβ), which is one of the six keplerian elements defining the ephemeris differential corrections (EDC) for NavIC as defined under clause 6.1.3.5 in [xx]. Scale factor 2–34

navic-GammaEDC This field indicates the Gamma correction to Ephemeris parameter (Δγ), which is one of the six keplerian elements defining the ephemeris differential corrections (EDC) for NavIC as defined under clause 6.1.3.5 in [xx]. Scale factor 2–32 semi-circles.

navic-AoIcorrection This field indicates the Angle of inclination correction (Δi), which is one of the six keplerian elements defining the ephemeris differential corrections (EDC) for NavIC as defined under clause 6.1.3.5 in [xx]. Scale factor 2–32 semi-circles.

navic-AoRAcorrection This field indicates the Angle of right ascension correction (ΔΩ), which is one of the six keplerian elements defining the ephemeris differential corrections (EDC) for the NavIC as defined under clause 6.1.3.5 in [xx]. Scale factor 2–32 semi-circles.

navic-SemiMajorcorrection This field indicates the Semi-major correction (ΔA), which is one of the six keplerian elements defining the ephemeris differential corrections (EDC) for the NavIC as defined under clause 6.1.3.5 in [xx]. Scale factor 2–9 meters.

navic-ClockBiasCorrection This field indicates correction to the satellite clock bias coefficient (δaf0), which is one of the two Satellite clock differential corrections (CDC) containing corrections to the NavIC satellite clock polynomial coefficients as defined under clause 6.1.3.5 in [xx]. Scale factor 2–35 seconds.

navic-ClockDriftCorrection This field indicates correction to the satellite clock drift coefficient (δaf1), which is one of the two Satellite clock differential corrections (CDC) containing corrections to the NavIC satellite clock polynomial coefficients as defined under clause 6.1.3.5 in [xx]. Scale factor 2–51 sec / sec.

#### –	NavIC-GridModelParameter

-- ASN1START

NavIC-GridModelParameter-r16 ::= SEQUENCE {

navic-RefTOWC-r16	INTEGER (0..50400),

regionMasked-r16	INTEGER (0..1023),

regionIgpList-r16	RegionIgpList-r16,

...

}

RegionIgpList-r16 ::= SEQUENCE (SIZE (1..16)) OF RegionIgpElement-r16

# RegionIgpElement-r16 ::= SEQUENCE {

regionID-r16	INTEGER (0..15),

givei1-r16		INTEGER (0..15),

givd1-r16		INTEGER (0..511),

givei2-r16		INTEGER (0..15),

givd2-r16		INTEGER (0..511),

givei3-r16		INTEGER (0..15),

givd3-r16		INTEGER (0..511),

givei4-r16		INTEGER (0..15),

givd4-r16		INTEGER (0..511),

givei5-r16		INTEGER (0..15),

givd5-r16		INTEGER (0..511),

givei6-r16		INTEGER (0..15),

givd6-r16		INTEGER (0..511),

givei7-r16		INTEGER (0..15),

givd7-r16		INTEGER (0..511),

givei8-r16		INTEGER (0..15),

givd8-r16		INTEGER (0..511),

givei9-r16		INTEGER (0..15),

givd9-r16		INTEGER (0..511),

givei10-r16		INTEGER (0..15),

givd10-r16		INTEGER (0..511),

givei11-r16		INTEGER (0..15),

givd11-r16		INTEGER (0..511),

givei12-r16		INTEGER (0..15),

givd12-r16		INTEGER (0..511),

givei13-r16		INTEGER (0..15),

givd13-r16		INTEGER (0..511),

givei14-r16		INTEGER (0..15),

givd14-r16		INTEGER (0..511),

givei15-r16		INTEGER (0..15),

givd15-r16		INTEGER (0..511),

...

}

-- ASN1STOP

NavIC-GridModelParameter field descriptions

navic-RefTOWC The transmission timing of the navigation message provided through the Time of Week Count (TOWC) corresponding to the given set of grid ionospheric parameters. It indicates the number of 12 second counts represented in 17 bits. The TOW count value ranges from 1 to 50400 to cover one entire week. The Time of Week (TOW) in seconds is obtained bymultiplying TOWC with 12 as defined in [xx], clause 5.7.

regionMasked Total 90 Ionospheric Grid Points(IGP) are defined in [xx] section 6.2.3 table 25. 15 IGP points are grouped into a single region. The region masked indicates the total number of regions for which the corrections are provided. For the current service area of the IRNSS, regions masked are 6.

regionIgpList This list provides the set of IGPs corresponding to each region. Up to 6 instances (0 to 5) are used in this version of the specification. The values 6 to 15 are reserved for future use.

regionID regionID along with index of the IGPS point corresponding gives the location of IGPS point as defined in [xx], table 25, section 6.2.3.

givei1, give2, .. , give15 This field indicates the Grid Ionospheric Vertical Error Index (GIVEI) which is used to describe the delay correction accuracy at ionospheric grid point indicated by the igp-ID, the mapping between GIVEI and GIVE is defined in [xx], section 6.2.2 and table 27.

givd1, givd2, … , givd15 This field indicates the Grid Ionospheric Vertical Delay (GIVD) as defined in [xx], clause 5.3.3.8.1, i.e. the vertical delay at the corresponding Ionospheric Grid points (IGPs) indicated by igp-ID. The scale factor is 0.125 meter.

< Unchanged parts are omitted >

#### –	GNSS-RTK-MAC-CorrectionDifferences

The IE GNSS-RTK-MAC-CorrectionDifferences is used by the location server to provide dispersive (ionospheric) and non-dispersive (geometric) correction difference components for up to 32 pairs of Auxiliary and Master Reference Stations. The Master Reference Station coordinates are provided in IE GNSS-RTK-ReferenceStationInfo and the Auxiliary Station coordinates are provided in IE GNSS-RTK-AuxiliaryStationData.

The parameters provided in IE GNSS-RTK-MAC-CorrectionDifferences are used as specified for message type 1017 and 1039 in [30] and apply to all GNSS.

-- ASN1START

GNSS-RTK-MAC-CorrectionDifferences-r15 ::= SEQUENCE {

networkID-r15						GNSS-NetworkID-r15,

subNetworkID-r15					GNSS-SubNetworkID-r15				OPTIONAL,	-- Need ON

master-ReferenceStationID-r15		GNSS-ReferenceStationID-r15,

l1-r15								GNSS-FrequencyID-r15				OPTIONAL,	-- Need OP

l2-r15								GNSS-FrequencyID-r15				OPTIONAL,	-- Need OP

rtkCorrectionDifferencesList-r15	RTK-CorrectionDifferencesList-r15,

...

}

RTK-CorrectionDifferencesList-r15 ::= SEQUENCE (SIZE (1..32)) OF

RTK-CorrectionDifferencesElement-r15

RTK-CorrectionDifferencesElement-r15 ::= SEQUENCE {

epochTime-r15							GNSS-SystemTime,

auxiliary-referenceStationID-r15		GNSS-ReferenceStationID-r15,

geometric-ionospheric-corrections-differences-r15

Geometric-Ionospheric-Corrections-Differences-r15,

...

}

Geometric-Ionospheric-Corrections-Differences-r15 ::= SEQUENCE (SIZE(1..64)) OF

Geometric-Ionospheric-Corrections-Differences-Element-r15

Geometric-Ionospheric-Corrections-Differences-Element-r15 ::= SEQUENCE {

svID-r15										SV-ID,

ambiguityStatusFlag-r15							INTEGER (0..3),

non-synch-count-r15								INTEGER (0..7),

geometricCarrierPhaseCorrectionDifference-r15	INTEGER (-65536..65535),

iod-r15											BIT STRING (SIZE(11)),

ionosphericCarrierPhaseCorrectionDifference-r15	INTEGER (-65536..65535),

...

}

-- ASN1STOP

GNSS-RTK-MAC-CorrectionDifferences field descriptions

networkID This field provides the network ID.

subNetworkID This field identifies the subnetwork of a network identified by networkID.

master-ReferenceStationID This field specifies the station ID of the Master Reference Station.

l1, l2 These fields specify the dual-frequency combination of L1 and L2 link/frequencies for which the rtkCorrectionDifferencesList is provided. If the fields are absent, the default interpretation in table 'L1/L2 default interpretation' applies.

rtkCorrectionDifferencesList This field provides the correction differences for Auxiliary-Master Reference Station pairs.

epochTime This field specifies the epoch time of observations used to derive the correction differences. The gnss-TimeID in GNSS-SystemTime shall be the same as the GNSS-ID in IE GNSS-GenericAssistDataElement.

auxiliary-referenceStationID This field specifies the station ID of the Auxiliary Reference Station.

svID This field specifies the satellite for which the data is provided.

ambiguityStatusFlag This field provides the ambiguity status. 'L1' below corresponds to the link indicated by the l1 field; 'L2' below corresponds to the link indicated by the l2 field. 0 - Reserved for future use (artificial observations) 1 - Correct Integer Ambiguity Level for L1 and L2 2 - Correct Integer Ambiguity Level for L1-L2 widelane 3 - Uncertain Integer Ambiguity Level. Only a likely guess is used.

non-synch-count This field provides the count of unrecoverable cycle slips. Whenever an unrecoverable cycle slip occurs this count shall be increased. The counter shall not be increased more than once per minute. Data for satellites with cycle slips more frequent than once per minute should not be provided.

geometricCarrierPhaseCorrectionDifference This field provides the Geometric Carrier Phase Correction Difference (GCPCD), which is the Correction Difference for the geometric part (troposphere and orbits) calculated based on integer leveled L1 and L2 correction differences (L1CD and L2CD). L1CD, L2CD, and ICPCD are presented in meters. 'L1' below corresponds to the link indicated by the l1 field; 'L2' below corresponds to the link indicated by the l2 field. Scale factor 0.5 milli-meter; range ±32.767 meters.

iod This field specifies the IOD value of the broadcast ephemeris used for calculation of Correction Differences (see IE GNSS-NavigationModel).

ionosphericCarrierPhaseCorrectionDifference This field provides the Ionospheric Carrier Phase Correction Difference (ICPCD), which is the Correction Difference for the ionospheric part calculated based on integer leveled L1 and L2 correction differences (L1CD and L2CD). L1CD, L2CD, and ICPCD are presented in meters. 'L1' below corresponds to the link indicated by the l1 field; 'L2' below corresponds to the link indicated by the l2 field. Scale factor 0.5 milli-meter; range ±32.767 meters.

L1/L2 default interpretation

| GNSS | l1 | l2 |
| --- | --- | --- |
| GPS | L1 | L2 |
| SBAS | L1 | L5 |
| QZSS | L1 | L2 |
| Galileo | E1 | E5a |
| GLONASS | G1 | G2 |
| BDS | B1 | B2 |

< Unchanged parts are omitted >

#### 6.5.2.3	GNSS Assistance Data Request

#### –	A-GNSS-RequestAssistanceData

The IE A-GNSS-RequestAssistanceData is used by the target device to request GNSS assistance data from a location server.

< Unchanged parts are omitted >

#### –	GNSS-GenericAssistDataReq

The IE GNSS-GenericAssistDataReq is used by the target device to request assistance data from a location server for one or more specific GNSS (e.g., GPS, Galileo, GLONASS, BDS, etc.). The specific GNSS for which the assistance data are requested is indicated by the IE GNSS-ID and (if applicable) by the IE SBAS-ID. Assistance for up to 16 GNSSs can be requested.

-- ASN1START

# GNSS-GenericAssistDataReq ::= SEQUENCE (SIZE (1..16)) OF GNSS-GenericAssistDataReqElement

# GNSS-GenericAssistDataReqElement ::= SEQUENCE {

gnss-ID							GNSS-ID,

sbas-ID							SBAS-ID						OPTIONAL, -- Cond GNSS-ID-SBAS

gnss-TimeModelsReq				GNSS-TimeModelListReq			OPTIONAL, -- Cond TimeModReq

gnss-DifferentialCorrectionsReq	GNSS-DifferentialCorrectionsReq	OPTIONAL, -- Cond DGNSS-Req

gnss-NavigationModelReq			GNSS-NavigationModelReq			OPTIONAL, -- Cond NavModReq

gnss-RealTimeIntegrityReq		GNSS-RealTimeIntegrityReq		OPTIONAL, -- Cond RTIReq

gnss-DataBitAssistanceReq		GNSS-DataBitAssistanceReq		OPTIONAL, -- Cond DataBitsReq

gnss-AcquisitionAssistanceReq	GNSS-AcquisitionAssistanceReq	OPTIONAL, -- Cond AcquAssistReq

gnss-AlmanacReq					GNSS-AlmanacReq					OPTIONAL, -- Cond AlmanacReq

gnss-UTCModelReq				GNSS-UTC-ModelReq				OPTIONAL, -- Cond UTCModReq

gnss-AuxiliaryInformationReq	GNSS-AuxiliaryInformationReq	OPTIONAL, -- Cond AuxInfoReq

...,

[[

bds-DifferentialCorrectionsReq-r12

BDS-DifferentialCorrectionsReq-r12

OPTIONAL,	-- Cond DBDS-Req

bds-GridModelReq-r12		BDS-GridModelReq-r12			OPTIONAL	-- Cond BDS-GridModReq

]],

[[

gnss-RTK-ObservationsReq-r15

GNSS-RTK-ObservationsReq-r15	OPTIONAL,	-- Cond RTK-OSR-Req

glo-RTK-BiasInformationReq-r15

GLO-RTK-BiasInformationReq-r15	OPTIONAL,	-- Cond GLO-CPB-Req

gnss-RTK-MAC-CorrectionDifferencesReq-r15

GNSS-RTK-MAC-CorrectionDifferencesReq-r15

OPTIONAL,	-- Cond MAC-Req

gnss-RTK-ResidualsReq-r15	GNSS-RTK-ResidualsReq-r15		OPTIONAL,	-- Cond Res-Req

gnss-RTK-FKP-GradientsReq-r15

GNSS-RTK-FKP-GradientsReq-r15	OPTIONAL,	-- Cond FKP-Req

gnss-SSR-OrbitCorrectionsReq-r15

GNSS-SSR-OrbitCorrectionsReq-r15

OPTIONAL, -- Cond OC-Req

gnss-SSR-ClockCorrectionsReq-r15

GNSS-SSR-ClockCorrectionsReq-r15

OPTIONAL, -- Cond CC-Req

gnss-SSR-CodeBiasReq-r15	GNSS-SSR-CodeBiasReq-r15		OPTIONAL -- Cond CB-Req

]],

[[

navic-DifferentialCorrectionsReq-r16

NavIC-DifferentialCorrectionsReq-r16

OPTIONAL,	-- Cond DNavIC-Req

navic-GridModelReq-r16		NavIC-GridModelReq-r16		OPTIONAL	-- Cond NavIC-GridModReq

]]

}

-- ASN1STOP

| Conditional presence | Explanation |
| --- | --- |
| GNSS-ID-SBAS | The field is mandatory present if the GNSS-ID = sbas; otherwise it is not present. |
| TimeModReq | The field is mandatory present if the target device requests GNSS-TimeModelList; otherwise it is not present. |
| DGNSS-Req | The field is mandatory present if the target device requests GNSS-DifferentialCorrections; otherwise it is not present. |
| NavModReq | The field is mandatory present if the target device requests GNSS-NavigationModel; otherwise it is not present. |
| RTIReq | The field is mandatory present if the target device requests GNSS-RealTimeIntegrity; otherwise it is not present. |
| DataBitsReq | The field is mandatory present if the target device requests GNSS-DataBitAssistance; otherwise it is not present. |
| AcquAssistReq | The field is mandatory present if the target device requests GNSS-AcquisitionAssistance; otherwise it is not present. |
| AlmanacReq | The field is mandatory present if the target device requests GNSS-Almanac; otherwise it is not present. |
| UTCModReq | The field is mandatory present if the target device requests GNSS-UTCModel; otherwise it is not present. |
| AuxInfoReq | The field is mandatory present if the target device requests GNSS-AuxiliaryInformation; otherwise it is not present. |
| DBDS-Req | The field is mandatory present if the target device requests BDS-DifferentialCorrections; otherwise it is not present. This field may only be present if gnss-ID indicates 'bds'. |
| BDS-GridModReq | The field is mandatory present if the target device requests BDS-GridModel; otherwise it is not present. This field may only be present if gnss-ID indicates 'bds'. |
| RTK-OSR-Req | The field is mandatory present if the target device requests GNSS-RTK-Observations; otherwise it is not present. |
| GLO-CPB-Req | The field is mandatory present if the target device requests GLO-RTK-BiasInformation; otherwise it is not present. |
| MAC-Req | The field is mandatory present if the target device requests GNSS-RTK-MAC-CorrectionDifferences; otherwise it is not present. |
| Res-Req | The field is mandatory present if the target device requests GNSS-RTK-Residuals; otherwise it is not present. |
| FKP-Req | The field is mandatory present if the target device requests GNSS-RTK-FKP-Gradients; otherwise it is not present. |
| OC-Req | The field is mandatory present if the target device requests GNSS-SSR-OrbitCorrections; otherwise it is not present. |
| CC-Req | The field is mandatory present if the target device requests GNSS-SSR-ClockCorrections; otherwise it is not present. |
| CB-Req | The field is mandatory present if the target device requests GNSS-SSR-CodeBias; otherwise it is not present. |
| DNavIC-Req | The field is mandatory present if the target device requests the NavIC-DifferentialCorrections; otherwise it is not present. This field may only be present if the gnss-ID indicates ‘navic’. |
| NavIC-GridModReq | The field is mandatory present if the target device requests NavIC-GridModel; otherwise it is not present. This field may only be present if the gnss-ID indicates ‘navic’. |

< Unchanged parts are omitted >

#### 6.5.2.4	GNSS Assistance Data Request Elements

< Unchanged parts are omitted >

#### –	GNSS-NavigationModelReq

The IE GNSS-NavigationModelReqis used by the target device to request the GNSS-NavigationModel assistance from the location server.

-- ASN1START

GNSS-NavigationModelReq ::=	CHOICE {

storedNavList		StoredNavListInfo,

reqNavList			ReqNavListInfo,

...

}

# StoredNavListInfo ::= SEQUENCE {

gnss-WeekOrDay			INTEGER (0..4095),

gnss-Toe				INTEGER (0..255),

t-toeLimit				INTEGER (0..15),

satListRelatedDataList	SatListRelatedDataList	OPTIONAL,

...

}

# SatListRelatedDataList ::= SEQUENCE (SIZE (1..64)) OF SatListRelatedDataElement

# SatListRelatedDataElement ::= SEQUENCE {

svID				SV-ID,

iod				BIT STRING (SIZE(11)),

clockModelID		INTEGER (1..8)			OPTIONAL,

orbitModelID		INTEGER (1..8)			OPTIONAL,

...

}

ReqNavListInfo ::=	SEQUENCE {

svReqList				BIT STRING (SIZE (64)),

clockModelID-PrefList	SEQUENCE (SIZE (1..8)) OF	INTEGER (1..8)		OPTIONAL,

orbitModelID-PrefList	SEQUENCE (SIZE (1..8)) OF	INTEGER (1..8)		OPTIONAL,

addNavparamReq			BOOLEAN				OPTIONAL,	-- Cond orbitModelID-2

...

}

-- ASN1STOP

| Conditional presence | Explanation |
| --- | --- |
| orbitModelID-2 | The field is mandatory present if orbitModelID-PrefList is absent or includes a Model-ID = '2'; otherwise it is not present. |

GNSS-NavigationModelReqfield descriptions

storedNavList This list provides information to the location server about which GNSS-NavigationModel data the target device has currently stored for the particular GNSS indicated by GNSS-ID.

reqNavList This list provides information to the location server which GNSS-NavigationModel data are requested by the target device.

gnss-WeekOrDay If GNSS-ID does not indicate 'glonass', this field defines the GNSS Week number of the assistance currently held by the target device. If GNSS-ID is set to 'glonass', this field defines the calendar number of day within the four-year interval starting from 1st of January in a leap year, as defined by the parameter NT in [9] of the assistance currently held by the target device.

gnss-Toe If GNSS-ID does not indicate 'glonass', this field defines the GNSS time of ephemeris in hours of the latest ephemeris set contained by the target device. If GNSS-ID is set to 'glonass', this field defines the time of ephemeris in units of 15 minutes of the latest ephemeris set contained by the target device (range 0 to 95 representing time values between 0 and 1425 minutes). In this case, values 96 to 255 shall not be used by the sender.

t-toeLimit If GNSS-ID does not indicate 'glonass', this IE defines the ephemeris age tolerance of the target device in units of hours. If GNSS-ID is set to 'glonass', this IE defines the ephemeris age tolerance of the target device in units of 30 minutes.

satListRelatedDataList This list defines the clock and orbit models currently held by the target device for each SV. This field is not included if the target device does not have any stored clock and orbit models for any SV.

svID This field identifies the particular GNSS satellite.

iod This field identifies the issue of data currently held by the target device.

clockModelID, orbitModelID These fields define the clock and orbit model number currently held by the target device. If these fields are absent, the default interpretation of the table GNSS-ID to clockModelID & orbitModelID relation below applies.

svReqList This field defines the SV for which the navigation model assistance is requested. Each bit position in this BIT STRING represents a SV-ID. Bit 0 represents SV-ID=0 and bit 63 represents SV-ID=63. A one-value at a bit position means the navigation model data for the corresponding SV-ID is requested, a zero-value means not requested.

clockModelIDPrefList, orbitModelID-PrefList These fields define the Model-IDs of the clock and orbit models that the target device wishes to obtain in the order of preference. The first Model-ID in the list is the most preferred model, the second Model-ID the second most preferred, etc. If these fields are absent, the default interpretation of the table GNSS-ID to clockModelID-PrefList & orbitModelIDPrefList relation below applies.

addNavparamReq This field specifies whether the location server is requested to include the addNAVparam fields in GNSS-NavigationModel IE (NavModel-NAVKeplerianSet field) or not. TRUE means requested.

# GNSS-ID to clockModelID & orbitModelID relation

| GNSS-ID | clockModelID | orbitModelID |
| --- | --- | --- |
| gps | 2 | 2 |
| sbas | 5 | 5 |
| qzss | 2 | 2 |
| galileo | 1 | 1 |
| glonass | 4 | 4 |
| bds | 6 | 6 |
| navic | 8 | 8 |

# GNSS-ID to clockModelID-PrefList & orbitModelID-PrefList relation

| GNSS-ID | clockModelID-PrefList | orbitModelID-PrefList |
| --- | --- | --- |
| gps | Model-2 | Model-2 |
| sbas | Model-5 | Model-5 |
| qzss | Model-2 | Model-2 |
| galileo | Model-1 | Model-1 |
| glonass | Model-4 | Model-4 |
| bds | Model-6 | Model-6 |
| navic | Model-8 | Model-8 |

#### –	GNSS-RealTimeIntegrityReq

The IE GNSS-RealTimeIntegrityReqis used by the target device to request the GNSS-RealTimeIntegrityassistancefrom the location server.

-- ASN1START

GNSS-RealTimeIntegrityReq ::=	SEQUENCE {

...

}

-- ASN1STOP

#### –	GNSS-DataBitAssistanceReq

The IE GNSS-DataBitAssistanceReqis used by the target device to request the GNSS-DataBitAssistanceassistancefrom the location server.

-- ASN1START

GNSS-DataBitAssistanceReq ::=	SEQUENCE {

gnss-TOD-Req		INTEGER (0..3599),

gnss-TOD-FracReq	INTEGER (0..999)		OPTIONAL,

dataBitInterval		INTEGER (0..15),

gnss-SignalType		GNSS-SignalIDs,

gnss-DataBitsReq	GNSS-DataBitsReqSatList	OPTIONAL,

...

}

# GNSS-DataBitsReqSatList ::= SEQUENCE (SIZE(1..64)) OF GNSS-DataBitsReqSatElement

# GNSS-DataBitsReqSatElement ::= SEQUENCE {

svID				SV-ID,

...

}

-- ASN1STOP

GNSS-DataBitAssistanceReqfield descriptions

gnss-TOD-Req This field specifies the reference time for the first data bit requested in GNSS specific system time, modulo 1 hour. Scale factor 1 second.

gnss-TOD-FracReq This field specifies the fractional part of gnss-TOD-Req in 1-milli-second resolution. Scale factor 1 millisecond.

dataBitInterval This field specifies the time length for which the Data Bit Assistance is requested. The GNSS-DataBitAssistance shall be relative to the time interval (gnss-TOD-Req, gnss-TOD-Req + dataBitInterval). The dataBitIntervalr, expressed in seconds, is mapped to a binary number K with the following formula: r =0.1 2 K Value K=15 means that the time interval is not specified.

gnss-SignalType This field specifies the GNSS Signal(s) for which the GNSS-DataBitAssistance are requested. A one-value at a bit position means GNSS-DataBitAssistance for the specific signal is requested; a zero-value means not requested.

gnss-DataBitsReq This list contains the SV-IDs for which the GNSS-DataBitAssistance is requested.

#### –	GNSS-AcquisitionAssistanceReq

The IE GNSS-AcquisitionAssistanceReqis used by the target device to request the GNSS-AcquisitionAssistanceassistancefrom the location server.

-- ASN1START

GNSS-AcquisitionAssistanceReq ::=	SEQUENCE {

gnss-SignalID-Req		GNSS-SignalID,

...

}

-- ASN1STOP

GNSS-AcquisitionAssistanceReqfield descriptions

gnss-SignalID-Req This field specifies the GNSS signal type for which GNSSAcquisitionAssistance is requested.

#### –	GNSS-AlmanacReq

The IE GNSS-AlmanacReqis used by the target device to request the GNSS-Almanacassistancefrom the location server.

-- ASN1START

# GNSS-AlmanacReq ::= SEQUENCE {

modelID				INTEGER(1..8)	OPTIONAL,

...

}

-- ASN1STOP

GNSS-AlmanacReqfield descriptions

modelID This field specifies the Almanac Model ID requested. If this field is absent, the default interpretation as in the table GNSS-ID to modelID relation below applies.

# GNSS-ID to modelID relation

| GNSS-ID | modelID |
| --- | --- |
| gps | 2 |
| sbas | 6 |
| qzss | 2 |
| galileo | 1 |
| glonass | 5 |
| bds | 7 |
| navic | 8 |

#### –	GNSS-UTC-ModelReq

The IE GNSS-UTC-ModelReqis used by the target device to request the GNSS-UTC-Modelassistancefrom the location server.

-- ASN1START

GNSS-UTC-ModelReq ::=	SEQUENCE {

modelID				INTEGER(1..8)	OPTIONAL,

...

}

-- ASN1STOP

GNSS-UTC-ModelReqfield descriptions

modelID This field specifies the GNSS-UTCModel set requested. If this field is absent, the default interpretation as in the table GNSS-ID to modelID relation below applies.

# GNSS-ID to modelID relation

| GNSS-ID | modelID |
| --- | --- |
| gps | 1 |
| sbas | 4 |
| qzss | 1 |
| galileo | 1 |
| glonass | 3 |
| bds | 5 |
| navic | 2 |

< Unchanged parts are omitted >

#### –	NavIC-DifferentialCorrectionsReq

The IE NavIC-DifferentialCorrectionsReq is used by the target device to request the NavIC-DifferentialCorrections assistance from the location server.

-- ASN1START

NavIC-DifferentialCorrectionsReq-r16 ::=	SEQUENCE {

dgnss-SignalsReq-r16			GNSS-SignalIDs-r16,

...

}

-- ASN1STOP

NavIC-DifferentialCorrectionsReqfield descriptions

dgnss-SignalsReq This field specifies the NavIC Signal(s) for which the NavIC-DifferentialCorrections are requested. A one-value at a bit position means the NavIC differential corrections for the specific signal are requested; a zero-value means not requested. The target device shall set a maximum of three bits to value 'one'.

#### –	NavIC-GridModelReq

The IE NavIC-GridModelReq is used by the target device to request the NavIC-GridModel assistance from the location server.

-- ASN1START

NavIC-GridModelReq-r16 ::=	SEQUENCE {

...

}

-- ASN1STOP

< Unchanged parts are omitted >

#### 6.5.2.10	GNSS Capability Information Elements

#### –	GNSS-CommonAssistanceDataSupport

The IE GNSS-CommonAssistanceDataSupport is used by the target device to provide information on supported GNSS common assistance data types to the location server.

< Unchanged parts are omitted >

#### –	GNSS-GenericAssistanceDataSupport

The IE GNSS-GenericAssistanceDataSupportis used by the target device to provide information on supported GNSS generic assistance data types to the location server for each supported GNSS.

-- ASN1START

# GNSS-GenericAssistanceDataSupport ::=

SEQUENCE (SIZE (1..16)) OF GNSS-GenericAssistDataSupportElement

# GNSS-GenericAssistDataSupportElement ::= SEQUENCE {

gnss-ID								GNSS-ID,

sbas-ID								SBAS-ID						OPTIONAL, -- Cond GNSS-ID-SBAS

gnss-TimeModelsSupport				GNSS-TimeModelListSupport

OPTIONAL, -- Cond TimeModSup

gnss-DifferentialCorrectionsSupport	GNSS-DifferentialCorrectionsSupport

OPTIONAL, -- Cond DGNSS-Sup

gnss-NavigationModelSupport			GNSS-NavigationModelSupport

OPTIONAL, -- Cond NavModSup

gnss-RealTimeIntegritySupport		GNSS-RealTimeIntegritySupport

OPTIONAL, -- Cond RTISup

gnss-DataBitAssistanceSupport		GNSS-DataBitAssistanceSupport

OPTIONAL, -- Cond DataBitsSup

gnss-AcquisitionAssistanceSupport	GNSS-AcquisitionAssistanceSupport

OPTIONAL, -- Cond AcquAssistSup

gnss-AlmanacSupport					GNSS-AlmanacSupport

OPTIONAL, -- Cond AlmanacSup

gnss-UTC-ModelSupport				GNSS-UTC-ModelSupport

OPTIONAL, -- Cond UTCModSup

gnss-AuxiliaryInformationSupport	GNSS-AuxiliaryInformationSupport

OPTIONAL, -- Cond AuxInfoSup

...,

[[

bds-DifferentialCorrectionsSupport-r12

BDS-DifferentialCorrectionsSupport-r12

OPTIONAL, -- Cond DBDS-Sup

bds-GridModelSupport-r12		BDS-GridModelSupport-r12

OPTIONAL	-- Cond BDS-GridModSup

]],

[[

gnss-RTK-ObservationsSupport-r15

GNSS-RTK-ObservationsSupport-r15

OPTIONAL,	-- Cond RTK-OSR-Sup

glo-RTK-BiasInformationSupport-r15

GLO-RTK-BiasInformationSupport-r15

OPTIONAL,	-- Cond GLO-CPB-Sup

gnss-RTK-MAC-CorrectionDifferencesSupport-r15

GNSS-RTK-MAC-CorrectionDifferencesSupport-r15

OPTIONAL,	-- Cond MAC-Sup

gnss-RTK-ResidualsSupport-r15	GNSS-RTK-ResidualsSupport-r15

OPTIONAL,	-- Cond Res-Sup

gnss-RTK-FKP-GradientsSupport-r15

GNSS-RTK-FKP-GradientsSupport-r15

OPTIONAL,	-- Cond FKP-Sup

gnss-SSR-OrbitCorrectionsSupport-r15

GNSS-SSR-OrbitCorrectionsSupport-r15

OPTIONAL, -- Cond OC-Sup

gnss-SSR-ClockCorrectionsSupport-r15

GNSS-SSR-ClockCorrectionsSupport-r15

OPTIONAL, -- Cond CC-Sup

gnss-SSR-CodeBiasSupport-r15	GNSS-SSR-CodeBiasSupport-r15

OPTIONAL -- Cond CB-Sup

]],

[[

navic-DifferentialCorrectionsSupport-r16

NavIC-DifferentialCorrectionsSupport-r16

OPTIONAL, 	-- Cond DNavIC-Sup

navic-GridModelSupport-r16		NavIC-GridModelSupport-r16

OPTIONAL	-- Cond NavIC-GridModSup

]]

}

-- ASN1STOP

| Conditional presence | Explanation |
| --- | --- |
| GNSS-ID-SBAS | The field is mandatory present if the GNSS-ID = sbas; otherwise it is not present. |
| TimeModSup | The field is mandatory present if the target device supports GNSS-TimeModelList; otherwise it is not present. |
| DGNSS-Sup | The field is mandatory present if the target device supports GNSS-DifferentialCorrections; otherwise it is not present. |
| NavModSup | The field is mandatory present if the target device supports GNSS-NavigationModel; otherwise it is not present. |
| RTISup | The field is mandatory present if the target device supports GNSS-RealTimeIntegrity; otherwise it is not present. |
| DataBitsSup | The field is mandatory present if the target device supports GNSS-DataBitAssistance; otherwise it is not present. |
| AcquAssistSup | The field is mandatory present if the target device supports GNSS-AcquisitionAssistance; otherwise it is not present. |
| AlmanacSup | The field is mandatory present if the target device supports GNSS-Almanac; otherwise it is not present. |
| UTCModSup | The field is mandatory present if the target device supports GNSS-UTC-Model; otherwise it is not present. |
| AuxInfoSup | The field is mandatory present if the target device supports GNSS-AuxiliaryInformation; otherwise it is not present. |
| DBDS-Sup | The field is mandatory present if the target device supports BDS-DifferentialCorrections; otherwise it is not present. This field may only be present if gnss-ID indicates 'bds'. |
| BDS-GridModSup | The field is mandatory present if the target device supports BDS-GridModel; otherwise it is not present. This field may only be present if gnss-ID indicates 'bds'. |
| RTK-OSR-Sup | The field is mandatory present if the target device supports GNSS-RTK-Observations; otherwise it is not present. Note, support for GNSS-RTK-Observations implies support for GNSS-RTK-CommonObservationInfo as well. |
| GLO-CPB-Sup | The field is mandatory present if the target device supports GLO-RTK-BiasInformation; otherwise it is not present. This field may only be present if gnss-ID indicates 'glonass'. |
| MAC-Sup | The field is mandatory present if the target device supports GNSS-RTK-MAC-CorrectionDifferences; otherwise it is not present. |
| Res-Sup | The field is mandatory present if the target device supports GNSS-RTK-Residuals; otherwise it is not present. |
| FKP-Sup | The field is mandatory present if the target device supports GNSS-RTK-FKP-Gradients; otherwise it is not present. |
| OC-Sup | The field is mandatory present if the target device supports GNSS-SSR-OrbitCorrections; otherwise it is not present. |
| CC-Sup | The field is mandatory present if the target device supports GNSS-SSR-ClockCorrections; otherwise it is not present. |
| CB-Sup | The field is mandatory present if the target device supports GNSS-SSR-CodeBias; otherwise it is not present. |
| DNavIC-Sup | The field is mandatory present if the target device supports the NavIC-DifferentialCorrections; otherwise it is not present. This field may only be present if the gnss-ID indicates ‘navic’. |
| NavIC-GridModSup | The field is mandatory present if the target device supports the NavIC-GridModel; otherwise it is not present. This field may only be present if the gnss-ID indicates ‘navic’. |

< Unchanged parts are omitted >

#### –	NavIC-DifferentialCorrectionsSupport

-- ASN1START

NavIC-DifferentialCorrectionsSupport-r16 ::=	SEQUENCE {

gnssSignalIDs			GNSS-SignalIDs,

...

}

-- ASN1STOP

NavIC-DifferentialCorrectionsSupport field descriptions

gnssSignalIDs This field specifies the NavIC signal types for which differential corrections are supported by the target device. This is represented by a bit string in GNSS-SignalIDs, with a one-value at the bit position means differential corrections for the particular NavIC signal type is supported; a zero-value means not supported.

#### –	NavIC-GridModelSupport

-- ASN1START

NavIC-GridModelSupport-r16 ::=	SEQUENCE {

...

}

-- ASN1STOP

< Unchanged parts are omitted >

#### –	GNSS-NavigationModelSupport

-- ASN1START

# GNSS-NavigationModelSupport ::= SEQUENCE {

clockModel		BIT STRING {	model-1		(0),

model-2		(1),

model-3		(2),

model-4		(3),

model-5		(4),

model-6		(5),

model-8-r16	(7) } (SIZE (1..8))		OPTIONAL,

orbitModel		BIT STRING {	model-1		(0),

model-2		(1),

model-3		(2),

model-4		(3),

model-5		(4),

model-6		(5),

model-8-r16	(7) } (SIZE (1..8))		OPTIONAL,

...

}

-- ASN1STOP

GNSS-NavigationModelSupport field descriptions

clockModel This field specifies the gnss-ClockModel choice(s) in GNSS-NavigationModel IE supported by the target device for the GNSS indicated by GNSS-ID. This is represented by a bit string, with a one-value at the bit position means the particular clock model is supported; a zero-value means not supported. If the target device supports GPS and GNSS-NavigationModel assistance, it shall support clockModel Model-2. If the target device supports SBAS and GNSS-NavigationModel assistance, it shall support clockModel Model-5. If the target device supports QZSS and GNSS-NavigationModel assistance, it shall support clockModel Model-2. If the target device supports Galileo and GNSS-NavigationModel assistance, it shall support clockModel Model-1. If the target device supports GLONASS and GNSS-NavigationModel assistance, it shall support clockModel Model-4. If the target device supports BDS and GNSS-NavigationModel assistance, it shall support clockModel Model-6. If the target device supports NavIC and GNSS-NavigationModel assistance, it shall support clockModel Model-8. If this field is absent, the target device supports the mandatory (native) clockModel choice only as listed above for the GNSS indicated by GNSS-ID.

orbitModel This field specifies the gnss-OrbitModel choice(s) in GNSS-NavigationModel IE supported by the target device for the GNSS indicated by GNSS-ID. This is represented by a bit string, with a one-value at the bit position means the particular orbit model is supported; a zero-value means not supported. If the target device supports GPS and GNSS-NavigationModel assistance, it shall support orbitModel Model-2. If the target device supports SBAS and GNSS-NavigationModel assistance, it shall support orbitModel Model-5. If the target device supports QZSS and GNSS-NavigationModel assistance, it shall support orbitModel Model-2. If the target device supports Galileo and GNSS-NavigationModel assistance, it shall supportorbitModel Model-1. If the target device supports GLONASS and GNSS-NavigationModel assistance, it shall support orbitModel Model-4. If the target device supports BDS and GNSS-NavigationModel assistance, it shall support orbitModel Model-6. If the target device supports NavIC and GNSS-NavigationModel assistance, it shall support orbitModel Model-8. If this field is absent, the target device supports the mandatory (native) orbitModel choice only as listed above for the GNSS indicated by GNSS-ID.

#### 6.5.2.13	Common GNSS Information Elements

#### –	GNSS-FrequencyID

The IE GNSS-FrequencyID is used to indicate a specific GNSS link/frequency. The interpretation of GNSS-FrequencyIDdepends on the GNSS-ID.

-- ASN1START

GNSS-FrequencyID-r15	::= SEQUENCE {

gnss-FrequencyID-r15		INTEGER (0 .. 7),

...

}

-- ASN1STOP

GNSS-FrequencyID field descriptions

gnss-FrequencyID This field specifies a particular GNSS link/frequency. The interpretation of gnss-FrequencyID depends on the GNSS-ID and is as shown in the table Value & Explanation relation below.

# Value & Explanation relation

| System | Value | Explanation |  |
| --- | --- | --- | --- |
|  |  | Link | Centre Frequency [MHz] |
| GPS | 0 | L1 | 1575.42 |
|  | 1 | L2 | 1227.60 |
|  | 2 | L5 | 1176.45 |
|  | 3-7 | reserved |  |
| SBAS | 0 | L1 | 1575.42 |
|  | 1 | L5 | 1176.45 |
|  | 2-7 | reserved |  |
| QZSS | 0 | L1 | 1575.42 |
|  | 1 | L2 | 1227.60 |
|  | 2 | L5 | 1176.45 |
|  | 3-7 | Reserved |  |
| GLONASS k = -7..13 | 0 | G1 | 1602+k×0.5625 |
|  | 1 | G2 | 1246+k×0.4375 |
|  | 2 | G3 | 1202.025 |
|  | 3-7 | Reserved |  |
| Galileo | 0 | E1 | 1575.420 |
|  | 1 | E6 | 1278.750 |
|  | 2 | E5a | 1176.450 |
|  | 3 | E5b | 1207.140 |
|  | 4 | E5 | 1191.795 |
|  | 5-7 | reserved |  |
| BDS | 0 | B1 (Phase II) | 1561.098 |
|  | 1 | B1 (Phase III) | 1575.420 |
|  | 2 | B2 | 1207.140 |
|  | 3 | B3 | 1268.520 |
|  | 4-7 | reserved |  |
| NavIC | 0 | L5 | 1176.450 |
|  | 1-7 | reserved |  |

#### –	GNSS-ID

The IE GNSS-ID is used to indicate a specific GNSS.

-- ASN1START

# GNSS-ID ::= SEQUENCE {

gnss-id				ENUMERATED{ gps, sbas, qzss, galileo, glonass, ..., bds, navic-v16xy },

...

}

-- ASN1STOP

#### –	GNSS-ID-Bitmap

The IE GNSS-ID-Bitmap is used to indicate several GNSSs using a bit map.

-- ASN1START

# GNSS-ID-Bitmap ::= SEQUENCE {

gnss-ids			BIT STRING {	gps			(0),

sbas		(1),

qzss		(2),

galileo		(3),

glonass		(4),

bds			(5),

navic-v16xy	(6) } (SIZE (1..16)),

...

}

-- ASN1STOP

GNSS-ID-Bitmap field descriptions

gnss-ids This field specifies the GNSS(s). This is represented by a bit string, with a one-value at the bit position means the particular GNSS is addressed; a zero-value means not addressed.

#### –	GNSS-Link-CombinationsList

-- ASN1START

GNSS-Link-CombinationsList-r15 ::= SEQUENCE (SIZE(1..8)) OF GNSS-Link-Combinations-r15

GNSS-Link-Combinations-r15 ::= SEQUENCE {

l1-r15		GNSS-FrequencyID-r15,

l2-r15		GNSS-FrequencyID-r15,

...

}

-- ASN1STOP

#### –	GNSS-NavListInfo

-- ASN1START

GNSS-NavListInfo-r15 ::= SEQUENCE (SIZE (1..64)) OF SatListElement-r15

SatListElement-r15 ::= SEQUENCE {

svID-r15		SV-ID,

iod-r15			BIT STRING (SIZE(11)),

...

}

-- ASN1STOP

#### –	GNSS-NetworkID

The IE GNSS-NetworkID defines the reference network and the source of the particular set of reference stations and their observation information. This IE is used for MAC Network RTK as described in [30].

-- ASN1START

# GNSS-NetworkID-r15 ::= SEQUENCE {

networkID-r15					INTEGER (0..255),

...

}

-- ASN1STOP

#### –	GNSS-PeriodicControlParam

The IE GNSS-PeriodicControlParam is used to specify control parameters for a periodic assistance data delivery.

-- ASN1START

# GNSS-PeriodicControlParam-r15 ::= SEQUENCE {

# deliveryAmount-r15			INTEGER (1..32),

# deliveryInterval-r15		INTEGER (1..64),

...

}

-- ASN1STOP

GNSS-PeriodicControlParam field descriptions

deliveryAmount This field specifies the number of periodic assistance data deliveries. Integer values N=1…31 correspond to an amount of 2N. Integer value N=32 indicates an 'infinite/indefinite' amount, which means that the assistance data delivery should continue until a LPP Abort message is received.

deliveryInterval This field specifies the interval between assistance data deliveries in seconds.

#### –	GNSS-ReferenceStationID

The IE GNSS-ReferenceStationID is used to identify a specific GNSS Reference Station.

-- ASN1START

# GNSS-ReferenceStationID-r15 ::= SEQUENCE {

# referenceStationID-r15			INTEGER (0..65535),

# providerName-r15				VisibleString (SIZE (1..32))		OPTIONAL,

...

}

-- ASN1STOP

GNSS-ReferenceStationID field descriptions

referenceStationID This field provides the reference station identity.

providerName This field is associated to a GNSS correction data provider to ensure that the referenceStationID's are unique from a target device perspective.

#### –	GNSS-SignalID

The IE GNSS-SignalID is used to indicate a specific GNSS signal type. The interpretation of GNSS-SignalIDdepends on the GNSS-ID.

-- ASN1START

GNSS-SignalID	::= SEQUENCE {

gnss-SignalID		INTEGER (0 .. 7),

...,

[[

gnss-SignalID-Ext-r15	INTEGER (8..23)			OPTIONAL

]]

}

-- ASN1STOP

GNSS-SignalID field descriptions

gnss-SignalID, gnss-SignalID-Ext This field specifies a particular GNSS signal. The interpretation of gnss-SignalID and gnss-SignalID-Ext depends on the GNSS-ID and is as shown in the table System to Value & Explanation relation below. If the field gnss-SignalID-Ext is present, the gnss-SignalID should be set to value 7 and shall be ignored by the receiver.

# System to Value & Explanation relation

| System | Value | Explanation |
| --- | --- | --- |
| GPS | 0 | GPS L1 C/A |
|  | 1 | GPS L1C |
|  | 2 | GPS L2C |
|  | 3 | GPS L5 |
|  | 4 | GPS L1 P |
|  | 5 | GPS L1 Z-tracking |
|  | 6 | GPS L2 C/A |
|  | 7 | GPS L2 P |
|  | 8 | GPS L2 Z-tracking |
|  | 9 | GPS L2 L2C(M) |
|  | 10 | GPS L2 L2C(L) |
|  | 11 | GPS L2 L2C(M+L) |
|  | 12 | GPS L5 I |
|  | 13 | GPS L5 Q |
|  | 14 | GPS L5 I+Q |
|  | 15 | GPS L1 L1C(D) |
|  | 16 | GPS L1 L1C(P) |
|  | 17 | GPS L1 L1C(D+P) |
|  | 18-23 | Reserved |
| SBAS | 0 | L1 C/A |
|  | 1 | L5 I |
|  | 2 | L5 Q |
|  | 3 | L5 I+Q |
|  | 4-7 | Reserved |
| QZSS | 0 | QZS-L1 C/A |
|  | 1 | QZS-L1C |
|  | 2 | QZS-L2C |
|  | 3 | QZS-L5 |
|  | 4 | QZS-LEX S |
|  | 5 | QZS-LEX L |
|  | 6 | QZS-LEX S+L |
|  | 7 | QZS-L2 L2C(M) |
|  | 8 | QZS-L2 L2C(L) |
|  | 9 | QZS-L2 L2C(M+L) |
|  | 10 | QZS-L5 I |
|  | 11 | QZS-L5 Q |
|  | 12 | QZS-L5 I+Q |
|  | 13 | QZS L1 L1C(D) |
|  | 14 | QZS L1 L1C(P) |
|  | 15 | QZS L1 L1C(D+P) |
|  | 16-23 | Reserved |
| GLONASS | 0 | GLONASS G1 C/A |
|  | 1 | GLONASS G2 C/A |
|  | 2 | GLONASS G3 |
|  | 3 | GLONASS G1 P |
|  | 4 | GLONASS G2 P |
|  | 5-23 | Reserved |
| Galileo | 0 | Galileo E1 |
|  | 1 | Galileo E5A |
|  | 2 | Galileo E5B |
|  | 3 | Galileo E6 |
|  | 4 | Galileo E5A + E5B |
|  | 5 | Galileo E1 C No data |
|  | 6 | Galileo E1 A |
|  | 7 | Galileo E1 B I/NAV OS/CS/SoL |
|  | 8 | Galileo E1 B+C |
|  | 9 | Galileo E1 A+B+C |
|  | 10 | Galileo E6 C |
|  | 11 | Galileo E6 A |
|  | 12 | Galileo E6 B |
|  | 13 | Galileo E6 B+C |
|  | 14 | Galileo E6 A+B+C |
|  | 15 | Galileo E5B I |
|  | 16 | Galileo E5B Q |
|  | 17 | Galileo E5B I+Q |
|  | 18 | Galileo E5(A+B) I |
|  | 19 | Galileo E5(A+B) Q |
|  | 20 | Galileo E5(A+B) I+Q |
|  | 21 | Galileo E5A I |
|  | 22 | Galileo E5A Q |
|  | 23 | Galileo E5A I+Q |
| BDS | 0 | B1 I |
|  | 1 | B1 Q |
|  | 2 | B1 I+Q |
|  | 3 | B3 I |
|  | 4 | B3 Q |
|  | 5 | B3 I+Q |
|  | 6 | B2 I |
|  | 7 | B2 Q |
|  | 8 | B2 I+Q |
|  | 9-23 | Reserved |
| NavIC | 0 | NavIC L5 SPS |
|  | 1-23 | Reserved |

#### –	GNSS-SignalIDs

The IE GNSSSignal-IDs is used to indicate several GNSS signals using a bit map. The interpretation of GNSSSignal-IDs depends on the GNSS-ID.

-- ASN1START

GNSS-SignalIDs	::= SEQUENCE {

gnss-SignalIDs		BIT STRING (SIZE(8)),

...,

[[

gnss-SignalIDs-Ext-r15	BIT STRING (SIZE(16))			OPTIONAL

]]

}

-- ASN1STOP

GNSS-SignalIDs field descriptions

gnss-SignalIDs, gnss-SignalIDs-Ext This field specifies one or several GNSS signals using a bit map. A one-value at the bit position means the particular signal is addressed; a zero-value at the particular bit position means the signal is not addressed. The interpretation of the bit map in gnssSignalIDs and gnss-SignalIDs-Ext depends on the GNSS-ID and is shown in the table below. Unfilled table entries indicate no assignment and shall be set to zero.

interpretation of the bit map in gnssSignalIDs

| GNSS | Bit 1 (MSB) | Bit 2 | Bit 3 | Bit 4 | Bit 5 | Bit 6 | Bit 7 | Bit 8 (LSB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| GPS | L1 C/A | L1C | L2C | L5 | L1P | L1 Z | L2 C/A | L2 P |
| SBAS | L1 C/A | L5 I | L5 Q | L5 I+Q |  |  |  |  |
| QZSS | QZS-L1 C/A | QZS-L1C | QZS-L2C | QZS-L5 | LEX S | LEX L | LEX S+L | L2C(M) |
| GLONASS | G1 C/A | G2 C/A | G3 | G1 P | G2 P |  |  |  |
| Galileo | E1 | E5a | E5b | E6 | E5a+E5b | E1 C No Data | E1 A | E1 B I/NAV OS/CS/SoL |
| BDS | B1 I | B1 Q | B1 I+Q | B3 I | B3 Q | B3 I+Q | B2 I | B2 Q |
| NavIC | L5 SPS |  |  |  |  |  |  |  |

interpretation of the bit map in gnssSignalIDs-Ext

| GNSS | Bit 1 (MSB) | Bit 2 | Bit 3 | Bit 4 | Bit 5 | Bit 6 | Bit 7 | Bit 8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| GPS | L2 Z | L2C(M) | L2C(L) | L2C(M+L) | L5 I | L5 Q | L5 I+Q | L1C(D) |
| SBAS |  |  |  |  |  |  |  |  |
| QZSS | L2C(L) | L2C(M+L) | L5 I | L5 Q | L5 I+Q | L1C(D) | L1C(P) | L1C(D+P) |
| GLONASS |  |  |  |  |  |  |  |  |
| Galileo | E1 B+C | E1 A+B+C | E6C | E6A | E6B | E6 B+C | E6 A+B+C | E5B I |
| BDS | B2 I+Q |  |  |  |  |  |  |  |
| NavIC |  |  |  |  |  |  |  |  |

| GNSS | Bit 9 | Bit 10 | Bit 11 | Bit 12 | Bit 13 | Bit 14 | Bit 15 | Bit 16 (LSB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| GPS | L1C(P) | L1C(D+P) |  |  |  |  |  |  |
| SBAS |  |  |  |  |  |  |  |  |
| QZSS |  |  |  |  |  |  |  |  |
| GLONASS |  |  |  |  |  |  |  |  |
| Galileo | E5B Q | E5B I+Q | E5(A+B) I | E5(A+B) Q | E5(A+B) I+Q | E5A I | E5A Q | E5A I+Q |
| BDS |  |  |  |  |  |  |  |  |
| NavIC |  |  |  |  |  |  |  |  |

#### –	GNSS-SubNetworkID

The IE GNSS-SubNetworkID defines the subnetwork of a network identified by GNSS-NetworkID. This IE is used for MAC Network RTK as described in [30].

-- ASN1START

# GNSS-SubNetworkID-r15 ::= SEQUENCE {

subNetworkID-r15				INTEGER (0..15),

...

}

-- ASN1STOP

#### –	SBAS-ID

The IE SBAS-ID is used to indicate a specific SBAS.

-- ASN1START

# SBAS-ID ::= SEQUENCE {

sbas-id				ENUMERATED { waas, egnos, msas, gagan, ...},

...

}

-- ASN1STOP

#### –	SBAS-IDs

The IE SBAS-IDs is used to indicate several SBASs using a bit map.

-- ASN1START

# SBAS-IDs ::= SEQUENCE {

sbas-IDs		BIT STRING {	waas		(0),

egnos		(1),

msas		(2),

gagan		(3)	} (SIZE (1..8)),

...

}

-- ASN1STOP

SBAS-IDs field descriptions

sbas-IDs This field specifies one or several SBAS(s) using a bit map. A one-value at the bit position means the particular SBAS is addressed; a zero-value at the particular bit position means the SBAS is not addressed.

#### –	SV-ID

The IE SV-ID is used to indicate a specific GNSS satellite. The interpretation of SV-ID depends on the GNSS-ID.

-- ASN1START

# SV-ID ::= SEQUENCE {

satellite-id		INTEGER(0..63),

...

}

-- ASN1STOP

SV-ID field descriptions

satellite-id This field specifies a particular satellite within a specific GNSS. The interpretation of satellite-id depends on the GNSS-ID see the table below.

interpretation of satellite-id

| System | Value of satellite-id | Interpretation of satellite-id |
| --- | --- | --- |
| GPS | '0' – '62' '63' | Satellite PRN Signal No. 1 to 63 Reserved |
| SBAS | '0' – '38' '39' – '63' | Satellite PRN Signal No. 120 to 158 Reserved |
| QZSS | '0' – '4' '5 – '63' | Satellite PRN Signal No. 193 to 197 Reserved |
| GLONASS | '0' – '23' '24 – '63' | Slot Number 1 to 24 Reserved |
| Galileo | '0' – '35' '36' – '63' | Code No. 1 to 36 Reserved |
| BDS | '0' – '36' '37' – '63' | Satellite ranging code number signal No.1 to 37 [23] Reserved |
| NavIC | ‘0’ – ‘13’ ‘14’–‘63’ | Satellite PRN Signal No. 1 to 14 Reserved |

----------------------------the next change----------------------

## 7.2	Mapping of posSibType to assistance data element

The supported posSibType's are specified in Table 7.2-1. The GNSS Common and Generic Assistance Data IEs are defined in clause 6.5.2.2. The OTDOA Assistance Data IEs are defined in clause 7.4.2.

Table 7.2-1: Mapping of posSibType to assistanceDataElement

|  | posSibType [12] | assistanceDataElement |
| --- | --- | --- |
| GNSS Common Assistance Data (clause 6.5.2.2) | posSibType1-1 | GNSS-ReferenceTime |
|  | posSibType1-2 | GNSS-ReferenceLocation |
|  | posSibType1-3 | GNSS-IonosphericModel |
|  | posSibType1-4 | GNSS-EarthOrientationParameters |
|  | posSibType1-5 | GNSS-RTK-ReferenceStationInfo |
|  | posSibType1-6 | GNSS-RTK-CommonObservationInfo |
|  | posSibType1-7 | GNSS-RTK-AuxiliaryStationData |
| GNSS Generic Assistance Data (clause 6.5.2.2) | posSibType2-1 | GNSS-TimeModelList |
|  | posSibType2-2 | GNSS-DifferentialCorrections |
|  | posSibType2-3 | GNSS-NavigationModel |
|  | posSibType2-4 | GNSS-RealTimeIntegrity |
|  | posSibType2-5 | GNSS-DataBitAssistance |
|  | posSibType2-6 | GNSS-AcquisitionAssistance |
|  | posSibType2-7 | GNSS-Almanac |
|  | posSibType2-8 | GNSS-UTC-Model |
|  | posSibType2-9 | GNSS-AuxiliaryInformation |
|  | posSibType2-10 | BDS-DifferentialCorrections |
|  | posSibType2-11 | BDS-GridModelParameter |
|  | posSibType2-12 | GNSS-RTK-Observations |
|  | posSibType2-13 | GLO-RTK-BiasInformation |
|  | posSibType2-14 | GNSS-RTK-MAC-CorrectionDifferences |
|  | posSibType2-15 | GNSS-RTK-Residuals |
|  | posSibType2-16 | GNSS-RTK-FKP-Gradients |
|  | posSibType2-17 | GNSS-SSR-OrbitCorrections |
|  | posSibType2-18 | GNSS-SSR-ClockCorrections |
|  | posSibType2-19 | GNSS-SSR-CodeBias |
|  | posSibType2-24 | NavIC-DifferentialCorrections |
|  | posSibType2-25 | NavIC-GridModelParameter |
| OTDOA Assistance Data (clause 7.4.2) | posSibType3-1 | OTDOA-UE-Assisted |

----------------------------End of change----------------------