3GPP TSG RAN WG2#109bis-e                                             	R2-2002599

Electronic meeting, 20April– 30April 2020

Source:	Quectel

Title:	Discussions on VarRLF-Report Setting

Agenda Item:	6.9.3.2

Document for:	Discussion and Decision

Introduction

At RAN2 #109e meeting, two CRs ([1][2]) were approved for CHO and RLF report respectively. This contribution discusses the VarRLF-Report setting issue when the RLF detection is followed by CHO.

Discussion

  - VarRLF-Report setting issue

In LTE, when RLF occurs, the RRC connection reestablishment procedure will be triggerred via transmitting RRCconnectionReestablishementRequest message. Correspondingly, the reestablishmentCellId in the VarRLF-Reportis set for the selected cell.For NR, a similar mechanism is illustrated in [1] for setting reestablishmentCellId:

The UE shall set the contents of RRCReestablishmentRequest message as follows:

1>	if the procedure was initiated due to radio link failure as specified in 5.3.10.3 or handover failure as specified in 5.3.5.8.3:

2>	set the reestablishmentCellId in the VarRLF-Report to the global cell identity of the selected cell;

However, the RRCReestablishmentRequest  message may not be triggered after RLF detection in NR, due to the introduction offailure recovery via CHO. That is, the reestablishmentCellIdfield in VarRLF-Report may not be set when RLF detection is followed by CHO. The corresponding description is in clause 5.3.7.3([2]) , as below:

1> If the cell selection is triggered by detecting radio link failure of the MCGor re-configuration with sync failure of the MCG, and

1>	if attemptCondReconfig is configured; and

1>	if the selected cell is one of the candidate cells which thereconfigurationWithSync is included in the masterCellGroupinVarCondtionalConfig:

2> apply the stored condRRCReconfigassociated to the selected cell and perform actions as specified in 5.3.5.3;

  - Solution to solve the above issue

In order to ensure the gNB being aware of the targeting cell ID of the CHO, the reestablishmentCellIdfield should be set even if the RRCReestablishmentRequest  message is not transmitted.

  - Proposed text proposal

The below text proposal is suggested based on the agreed version in [2].

*******************************************Start of TP**************************************

5.3.7.3	Actions following cell selection while T311 is running

Upon selecting a suitable NR cell, the UE shall:

1>	ensure having valid and up to date essential system information as specified in clause 5.2.2.2;

1>	stop timer T311;

1>	if T390 is running:

2>	stop timer T390 for all access categories;

2>	perform the actions as specified in 5.3.14.4;

1> If the cell selection is triggered by detecting radio link failure of the MCGor re-configuration with sync failure of the MCG, and

1>	if attemptCondReconfig is configured; and

1>	if the selected cell is one of the candidate cells which thereconfigurationWithSync is included in the masterCellGroupinVarCondtionalConfig:

2>set the reestablishmentCellId in the VarRLF-Report to the global cell identity of the selected cell;

2> apply the stored condRRCReconfigassociated to the selected cell and perform actions as specified in 5.3.5.3;

1>	else:

2>if UE is configured with conditionalReconfiguration:

3>	release spCellConfig, if configured;

3>	suspend all RBs, except SRB0;

2> remove all the entries within VarConditionalConfig, if any;

2>	for each measId, if the associatedreportConfig has areportTypeset to condTriggerConfig:

3>	for the associated reportConfigId:

4>	remove the entry with the matching reportConfigIdfrom the reportConfigList within the VarMeasConfig;

3>	if the associated measObjectId is only associated to a reportConfigwithreportType set to condTriggerConfig:

4>	remove the entry with the matching measObjectIdfrom the measObjectList within the VarMeasConfig;

3>	remove the entry with the matching measId from the measIdList within the VarMeasConfig;

2>	start timer T301;

2>	apply the default L1 parameter values as specified in corresponding physical layer specifications except for the parameters for which values are provided in SIB1;

2>	apply the default MAC Cell Group configuration as specified in 9.2.2;

2>	apply the CCCH configuration as specified in 9.1.1.2;

2>	apply the timeAlignmentTimerCommon included in SIB1;

2>	initiate transmission of the RRCReestablishmentRequest message in accordance with 5.3.7.4;

NOTE 1:	This procedure applies also if the UE returns to the source PCell.

Upon selecting an inter-RAT cell, the UE shall:

1>	perform the actions upon going to RRC_IDLE as specified in 5.3.11, with release cause 'RRC connection failure'.

********************************************End of TP**************************************

Conclusion

In this contribution, we analyze the VarRLF-Report setting issue, and propose the corresponding text proposal.

Reference

- R2-2002003, CR for introducing MDT and SON, Huawei, Ericsson, HiSilicon

- R2-2001767,Introduction of NR mobility enhancement, Intel Corp