3GPP TSG-RAN WG2 Meeting #109bis-e	R2-2002686

# Elbonia, 20 – 30 April 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.321 | CR | 0707 | rev | - | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of FR2 MPE P-MPR reporting |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_RF_FR2_req_enh |  |  |  |  |  | Date: |  |  | 2020-04-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The Maximum Permissible Exposure (MPE) limit set by FCC aims at restricting the UE Tx power averaged over a defined period of time for limiting RF exposure on human body for FR2. UE may be required to apply a large P-MPR value to comply with those requirements. Unless network is aware of the applied P-MPR, radio link failure or connection releases due to significant and unpredictable UE P-MPR may occur. Therefore, UE reporting of the P-MPR used to meet the FR2 MPE limits is required to prevent this. |  |  |  |  |  |  |  |  |
| Summary of change: |  | P-MPR reporting is specified as follows: - New MAC CE for reporting P-MPR due to FR2 MPE is added - P-MPR reporting due to MPE is triggered when the applicable P-MPR exceeds a configured threshold (with a prohibit timer), or when P-MPR falls back to below the threshold (not impacted by prohibit timer). - UL LCID = 35 is reserved for the P-MPR reporting MAC CE |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The MAC procedures for FR2 MPE P-MPR reporting are not specified. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.X (new), 6.1.3.X (new), 6.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | x |  | Other core specifications |  |  |  | TS38.300 CR0210, TS38.306 CR0272, TS38.331 CR1515 |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First Modified Subclause

## 5.X	FR2 MPE P-MPR reporting

If the MAC entity is configured to report FR2 MPE P-MPR, the UE may trigger sending a MAC CE to network. Upon detecting that P-MPR needs to be applied due to FR2 MPE requirements as specified in TS38.101-2 [15], UE creates the corresponding FR2 MPE P-MPR reporting MAC CE and sends it to the network.

The FR2 MPE P-MPR reporting procedure is used to provide the serving gNB with the following information:

-	P-MPR: The P-MPR UE needs to apply to meet the FR2 MPE requirements:

-	Serving cell ID: the FR2 serving cell identity that is impacted by the FR2 MPE requirements;

RRC controls FR2 MPE P-MPR reporting by configuring the following parameters:

-	p-MPR-Threshold;

-	p-MPR-ProhibitTimer;

A FR2 MPE P-MPR peporting shall be triggered if any of the following events occur for a MAC entity where the FR2 MPE P-MPR reporting is configured:

-	the P-MPR due to FR2 MPE is more than or equal to the p-MPR-Threshold dB for at least one activated FR2 Serving Cell of the MAC entity where the P-MPR reporting is configured;

-	the P-MPR due to FR2 MPE is less than p-MPR-Threshold dB for at least one activated FR2 Serving Cell of MAC entity where the P-MPR reporting is configured;

If the FR2 MPE P-MPR reporting is triggered:

1>	if the current P-MPR is more than p-MPR-Threshold dB for an FR2 serving cell of this MAC entity and mpe-ProhibitTimer expires or has expired:

2>	start or restart the p-MPR-ProhibitTimer;

2>	instruct the Multiplexing and Assembly procedure to generate and transmit the P-MPR reporting MAC CE as defined in clause 6.1.3.X based on the P-MPR value reported by the physical layer and the affected serving cell identity.

1>	else if the UE has previously sent P-MPR reporting MAC CE for an FR2 serving cell of this MAC entity and current P-MPR is less than p-MPR-Threshold dB for the FR2 serving cell of this MAC entity:

2>	stop p-MPR-ProhibitTimer, if running.

2>	instruct the Multiplexing and Assembly procedure to generate and transmit the P-MPR reporting MAC CE as defined in clause 6.1.3.X based on the P-MPR value reported by the physical layer and the affected serving cell identity.

Next Modified Subclause

### 6.1.3	MAC Control Elements (CEs)

<UNNECESSARY PARTS OMITTED>

#### 6.1.3.X	FR2 MPE P-MPR reporting MAC CE

The P-MPR reporting MAC CE is identified by a MAC subheader with LCID as specified in Table 6.2.1-2.

It has a fixed size and consists of two octets defined as follows (figure 6.1.3.X-1):

-	R: Reserved bit, set to 0;

-	S: Status of the applied P-MPR due to FR2 MPE requirements. It is set to 1 is the P-MPR is larger than p-MPR-Threshold and set to 0 otherwise.

-	P-MPR: This field indicates the P-MPR UE applies due to FR2 MPE requirements. The length of the field is 6 bits. The reported P-MPR and the corresponding P-MPR levels are shown in Table 6.1.3.X-1 below (the corresponding measured values of P-MPR levels in dB are specified in TS 38.133 [11]);

-	Serving cell ID: This field indicates the serving cell that triggered the P-MPR reporting (as specified in TS 38.101-2 [15]) due to P-MPR exceeding the configured threshold. The serving cell ID corresponds to the ServCellIndex as specified in in TS 38.331 [5]).

Figure 6.1.3.X-1: FR2 MPE P-MPR reporting MAC CE

Table 6.1.3.X-1: FR2 MPE P-MPR levels for FR2 MPE P-MPR reporting

| P-MPR | P-MPR Level |
| --- | --- |
| 0 | P_MPR_0 |
| 1 | P_MPR_1 |
| 2 | P_MPR_2 |
| 3 | P_MPR_3 |
| … | … |
| 60 | P_MPR_60 |
| 61 | P_MPR_61 |
| 62 | P_MPR_62 |
| 63 | P_MPR_63 |

Next Modified Subclause

### 6.2.1	MAC subheader for DL-SCH and UL-SCH

The MAC subheader consists of the following fields:

-	LCID: The Logical Channel ID field identifies the logical channel instance of the corresponding MAC SDU or the type of the corresponding MAC CE or padding as described in Tables 6.2.1-1 and 6.2.1-2 for the DL-SCH and UL-SCH respectively. There is one LCID field per MAC subheader. The LCID field size is 6 bits. If the LCID field is set to 34, one additional octet is present in the MAC subheader containing the eLCID field and follow the octet containing LCID field. If the LCID field is set to 33, two additional octets are present in the MAC subheader containing the eLCID field and these two additional octets follow the octet containing LCID field;

-	eLCID: The extended Logical Channel ID field identifies the logical channel instance of the corresponding MAC SDU as described in tables 6.2.1-1a, 6.2.1-1b, 6.2.1-2a and 6.2.1-2b for the DL-SCH and UL-SCH respectively. The size of the eLCID field is either 8 bits or 16 bits.

NOTE 1:	The extended Logical Channel ID space using two-octet eLCID and the relevant MAC subheader format is used, only when configured, on the NR backhaul links between IAB nodes or between IAB node and IAB Donor.

-	L: The Length field indicates the length of the corresponding MAC SDU or variable-sized MAC CE in bytes. There is one L field per MAC subheader except for subheaders corresponding to fixed-sized MAC CEs, padding, and MAC SDUs containing UL CCCH. The size of the L field is indicated by the F field;

-	F: The Format field indicates the size of the Length field. There is one F field per MAC subheader except for subheaders corresponding to fixed-sized MAC CEs, padding, and MAC SDUs containing UL CCCH. The size of the F field is 1 bit. The value 0 indicates 8 bits of the Length field. The value 1 indicates 16 bits of the Length field;

-	R: Reserved bit, set to 0.

The MAC subheader is octet aligned.

Table 6.2.1-1 Values of LCID for DL-SCH

| Codepoint/Index | LCID values |
| --- | --- |
| 0 | CCCH |
| 1–32 | Identity of the logical channel |
| 33 | Extended logical channel ID field (two-octet eLCID field) |
| 34 | Extended logical channel ID field (one–octet eLCID field) |
| 35 | Reserved |
| 36 | SP Positioning SRS Activation/Deactivation |
| 37 | Duplication RLC Activation/Deactivation |
| 38 | Absolute Timing Advance Command |
| 39 | CC list-based SRS Activation/Deactivation |
| 40 | PUSCH Pathloss Reference RS Activation/Deactivation |
| 41 | SRS Pathloss Reference RS Activation/Deactivation |
| 42 | AP SRS spatial relation Indication |
| 43 | Enhanced PUCCH spatial relation Activation/Deactivation |
| 44 | Enhanced TCI States Activation/Deactivation for UE-specific PDSCH |
| 45 | Number of Provided Guard Symbols |
| 46 | Timing Delta |
| 47 | Recommended bit rate |
| 48 | SP ZP CSI-RS Resource Set Activation/Deactivation |
| 49 | PUCCH spatial relation Activation/Deactivation |
| 50 | SP SRS Activation/Deactivation |
| 51 | SP CSI reporting on PUCCH Activation/Deactivation |
| 52 | TCI State Indication for UE-specific PDCCH |
| 53 | TCI States Activation/Deactivation for UE-specific PDSCH |
| 54 | Aperiodic CSI Trigger State Subselection |
| 55 | SP CSI-RS/CSI-IM Resource Set Activation/Deactivation |
| 56 | Duplication Activation/Deactivation |
| 57 | SCell Activation/Deactivation (four octets) |
| 58 | SCell Activation/Deactivation (one octet) |
| 59 | Long DRX Command |
| 60 | DRX Command |
| 61 | Timing Advance Command |
| 62 | UE Contention Resolution Identity |
| 63 | Padding |

Table 6.2.1-1a Values of two-octet eLCID for DL-SCH

| Index | LCID values |
| --- | --- |
| 320 to (216 + 191) | Identity of the logical channel |
| (216 + 192) to (216 + 319) | Reserved |

Table 6.2.1-1b Values of one-octet eLCID for DL-SCH

| Codepoint | Index | LCID values |
| --- | --- | --- |
| 0 to 255 | 64 to 319 | reserved |

Table 6.2.1-2 Values of LCID for UL-SCH

| Index | LCID values |
| --- | --- |
| 0 | CCCH of size 64 bits (referred to as "CCCH1" in TS 38.331 [5]) |
| 1–32 | Identity of the logical channel |
| 33 | Extended logical channel ID field (two–octet eLCID field) |
| 34 | Extended logical channel ID field (one–octet eLCID field) |
| 35 | FR2 MPE P-MPR reporting |
| 36–39 | Reserved |
| 40 | Sidelink Configured Grant Confirmation |
| 41 | Truncated Sidelink BSR |
| 42 | Sidelink BSR |
| 43 | Multiple Entry Configured Grant Confirmation |
| 44 | LBT failure (four octets) |
| 45 | LBT failure (one octet) |
| 46 | SCell BFR (four octets Ci) |
| 47 | SCell BFR (one octet Ci) |
| 48 | Truncated SCell BFR (four octets Ci) |
| 49 | Truncated SCell BFR (one octet Ci) |
| 50 | Number of Desired Guard Symbols |
| 51 | Pre-emptive BSR |
| 52 | CCCH of size 48 bits (referred to as "CCCH" in TS 38.331 [5]) |
| 53 | Recommended bit rate query |
| 54 | Multiple Entry PHR (four octets Ci) |
| 55 | Configured Grant Confirmation |
| 56 | Multiple Entry PHR (one octet Ci) |
| 57 | Single Entry PHR |
| 58 | C-RNTI |
| 59 | Short Truncated BSR |
| 60 | Long Truncated BSR |
| 61 | Short BSR |
| 62 | Long BSR |
| 63 | Padding |

Table 6.2.1-2a Values of two-octet eLCID for UL-SCH

| Codepoint/IIndex | LCID values |
| --- | --- |
| 320 to (216 + 191) | Identity of the logical channel |
| (216 + 192) to (216 + 319) | Reserved |

Table 6.2.1-2b Values of one-octet eLCID for UL-SCH

| Codepoint | Index | LCID values |
| --- | --- | --- |
| 0 to 255 | 64 to 319 | reserved |

NOTE 2:	For the eLCID space, the 16-bit codepoint 000…00 (all zeros) corresponds to the index value of 320, while the 16-bit codepoint 111…11 (all ones) corresponds to the index value of 216 + 319.