3GPP TSG-RAN WG2 Meeting#109bis electronic					     R2-2002700

e-meeting, 20th - 30th April, 2020

Source: 			ZTE Corporation, Sanechips

Title: 	Support of Inter-RAT handover upon MCG failure recovery

Agenda item:		6.10.7

Document for: 	Discussion and Decision

- Introduction

During RAN2’s email discussion [1], companies discussed whether to support inter-RAT handover upon MCG failure recovery, in this contribution, we discussed this remaining issues, and provided the draft CR in annex.

- Discussion

In email discussion [1], one remaining issue is whether to support of inter-RAT handover upon MCG failure recovery, based on the response from companies, people are comfortable to support any intra-RAT handover scenarios for MCG failure recovery, including inter-system intra-RAT handover. However, companies show different opinions to inter-RAT handover.

Based on TS 37.340, so far, we have the following inter-RAT handover scenarios in Rel-16 (in which source is MR-DC):

- Case 1: (NG)EN-DC to NR;

- Case 2: NE-DC to LTE-EPC;

- Case 3: NE-DC to LTE-5GC;

- Case 4: NR-DC to LTE-EPC;

- Case 5: NR-DC to LTE-5GC;

- Case 6: EN-DC to GERAN/UTRAN;

- Case 7: NE-DC to UTRAN-FDD (i.e. SRVCC from 5G to 3G)

- Case 8: NR-DC to UTRAN-FDD (i.e. SRVCC from 5G to 3G)

Regarding the specification impact, as indicated by some company in [1], all inter-RAT handover scenarios (i.e. Case 1~8) should already be supported when split SRB1 is used, because the Handover Command (i.e. MobilityFromEUTRACommand and MobilityFromNRCommand) is delivered via SCG leg of split SRB1 with no specification change.

Proposal 1   RAN2 to confirm inter-RAT handover (including all scenarios) upon MCG failure recovery is supported when split SRB1 is used for MCG recovery.

While for SRB3 case, some company suggest to not consider it to avoid high standardization effort. However, we think the standardization work is marginal, because the field description can be updated without ASN.1 change (see below example, similar modification can be done in RAN3 spec).

From our perspective, the UE implementation is almost the same no matter the handover command is received from split SRB1 or SRB3.

DLInformationTransferMRDC message

-- ASN1START

-- TAG-DLINFORMATIONTRANSFERMRDC-START

DLInformationTransferMRDC-r16 ::=               SEQUENCE {

criticalExtensions                          CHOICE {

c1                                          CHOICE {

dlInformationTransferMRDC-r16               DLInformationTransferMRDC-r16-IEs,

spare3 NULL, spare2 NULL, spare1 NULL

},

criticalExtensionsFuture            SEQUENCE {}

}

}

DLInformationTransferMRDC-r16-IEs::=           SEQUENCE {

dl-DCCH-MessageNR-r16                       OCTET STRING                    OPTIONAL,

dl-DCCH-MessageEUTRA-r16                    OCTET STRING                    OPTIONAL,

lateNonCriticalExtension                    OCTET STRING                    OPTIONAL,

nonCriticalExtension                        SEQUENCE {}                     OPTIONAL

}

-- TAG-DLINFORMATIONTRANSFERMRDC-STOP

-- ASN1STOP

DLInformationTransferMRDC field descriptions

dl-DCCH-MessageNR Includes the DL-DCCH-Message. In this version of the specification, the field is only used to transfer the NR RRCReconfiguration and, RRCRelease and MobilityFromNRCommand messages.

dl-DCCH-MessageEUTRA Includes the DL-DCCH-Message. In this version of the specification, the field is only used to transfer the E-UTRA RRCConnectionReconfiguration and, RRCConnectionRelease and MobilityFromEUTRACommand messages as specified in TS 36.331 [10].

In addition, if inter-RAT handover is only supported via split SRB1, we are afraid the network will be forced to configure split SRB1 to avoid undesired RRCRelease, therefore, the SRB3 approach becomes meaningless.

Observation 1   If only inter-RAT handover is only supported in case of split SRB1, then network will be forced to always configure split SRB1, then MCG recovery over SRB3 becomes meaningless.

Regarding the inter-RAT scenarios, case 1~5 are inter-RAT handover between NR and EUTRAN (EPC or 5GC). Based on RAN2’s agreement and CRs, UE can deliver the LTE and NR measurement results in MCG failure information, including both MN and SN configured measurements. So from MN’s perspective, upon reception of MCGFailureInformation message, of course MN will first try intra-handover to recovery the connection, however, if no suitable intra-RAT neighbour cell is received from UE, it makes sense to allow network to trigger inter-RAT handover to maintain UE’s connection. Otherwise, once UE is released and reconnect to network, probably, network still has to trigger inter-RAT handover due to the poor quality of intra-RAT neighbour cells.

Proposal 2   Inter-RAT handover between E-UTRAN and NR via SRB3 is supported upon MCG failure recovery, including the following scenarios:

- Case 1: (NG)EN-DC to NR;

- Case 2: NE-DC to LTE-EPC;

- Case 3: NE-DC to LTE-5GC;

- Case 4: NR-DC to LTE-EPC;

- Case 5: NR-DC to LTE-5GC;

For case 6/7/8 via SRB3, those are inter-RAT handover towards 2G/3G system, based on the feedback in [1], companies are divided on these scenarios. The main concern is about extra standardization effort, and some companies also think these scenarios are not urgent in Rel-16. In our opinion, from the perspective of handover signalling, case 6 uses MobiiltyFromEUTRACommand, which is the same as case 1; and case 7/8 use the same signalling (i.e. MobilityFromNRCommand) as case 2/3/4/5. So once Proposal 2 is agreed, case 6/7/8 should also be possible by including different target RRC container. We did not see the necessity to have special treatment (e.g. excluding some scenarios) in specification.

Proposal 3   Inter-RAT handover towards GERAN/UTRAN via SRB3 is supported upon MCG failure recovery, including the following scenarios:

- Case 6: EN-DC to GERAN/UTRAN;

- Case 7: NE-DC to UTRAN-FDD (i.e. SRVCC from 5G to 3G)

- Case 8: NR-DC to UTRAN-FDD (i.e. SRVCC from 5G to 3G)

During email discussion [1], we raised another issue about including 2G/3G measurement results in MCGFailureInformation message. As mentioned above, UE is already able to send 4G/5G measurement results in MCGFailureInformation, but for MN configured 2G/3G inter-RAT measurements, these can not be delivered to network when UE encounters MCG failure. Then the network (source MN) can only select the target 2G/3G cells blindly if it wants to.

We understand the design of MCGFailureInformation message content largely inherit the content of SCGFailureInformation, but we should notice that, upon SCG failure, the MN can either release or change the SN node or PSCell, and the connection of MCG is maintained. So for all MR-DC types, NR and/or EUTRAN measurement results are sufficient to assist network to make decision. However, in case of MCG failure, network needs to try its best to recovery the MCG link as earlier as possible, so except NR or LTE targets, other targets should also be considered.

Observation 2   For SCG failure recovery, MN can either release or change the SN node, so NR and EUTRAN measurement results are sufficient to assist network to make decision.

From our perspective, it is acceptable to us if companies have strong concern on reporting GERAN measurement results, but we do have concern on SRVCC from 5G to 3G (e.g. case 7/8) for following reasons:

- Reason 1: For initial deployment of NR, SRVCC from 5G to 3G is likely be triggered for concerned operator.

- Reason 2: RRCRelease (with or w/o redirection) will result in voice call drop, because SRVCC from 5G to 3G only supports handover approach;

So if inter-RAT handover towards 2G/3G system upon MCG failure is supported, we suggest to at least include available UTRAN-FDD measurement results in MCGFailureInformation message.

Proposal 4   UE can include UTRAN-FDD measurement results in MCG Failure Information message.

The suggested changes to specification 36.331 and 38.331 are shown in Annex. The changes of proposal 2/3 are highlighted in green color, and the changes of proposal 4 are highlighted in yellow color.

Proposal 5   Adopt the draft CR in Annex.

- Conclusion

Based on the discussion in the previous sections we propose the following:

Observation 1   If only inter-RAT handover is only supported in case of split SRB1, then network will be forced to always configure split SRB1, then MCG recovery over SRB3 becomes meaningless.

Observation 2   For SCG failure recovery, MN can either release or change the SN node, so NR and EUTRAN measurement results are sufficient to assist network to make decision.

Proposal 1   RAN2 to confirm inter-RAT handover (including all scenarios) upon MCG failure recovery is supported when split SRB1 is used for MCG recovery.

Proposal 2   Inter-RAT handover between E-UTRAN and NR via SRB3 is supported upon MCG failure recovery, including the following scenarios:

- Case 1: (NG)EN-DC to NR;

- Case 2: NE-DC to LTE-EPC;

- Case 3: NE-DC to LTE-5GC;

- Case 4: NR-DC to LTE-EPC;

- Case 5: NR-DC to LTE-5GC;

Proposal 3   Inter-RAT handover towards GERAN/UTRAN via SRB3 is supported upon MCG failure recovery, including the following scenarios:

- Case 6: EN-DC to GERAN/UTRAN;

- Case 7: NE-DC to UTRAN-FDD (i.e. SRVCC from 5G to 3G)

- Case 8: NR-DC to UTRAN-FDD (i.e. SRVCC from 5G to 3G)

Proposal 4   UE can include UTRAN-FDD measurement results in MCG Failure Information message.

The suggested changes to specification 36.331 and 38.331 are shown in Annex. The changes of proposal 2/3 are highlighted in green color, and the changes of proposal 4 are highlighted in yellow color.

Proposal 5   Adopt the draft CR in Annex.

- References

- RAN2 Email discussion [Post109e#27][DCCA] Fast MCG recovery   Ericsson

- Annex

## Draft CR to TS 38.331

### 5.4.3	Mobility from NR

#### 5.4.3.1	General

Figure 5.4.3.1-1: Mobility from NR, successful

Figure 5.4.3.1-2: Mobility from NR, failure

The purpose of this procedure is to move a UE in RRC_CONNECTED to a cell using other RAT, e.g. E-UTRA. The mobility from NR procedure covers the following type of mobility:

-	handover, i.e. the MobilityFromNRCommand message includes radio resources that have been allocated for the UE in the target cell;

#### 5.4.3.2	Initiation

The network initiates the mobility from NR procedure to a UE in RRC_CONNECTED, possibly in response to a MeasurementReport or MCGFailureInformation message, by sending a MobilityFromNRCommand message. The network applies the procedure as follows:

-	the procedure is initiated only when AS security has been activated, and SRB2 with at least one DRB are setup and not suspended.

Note: The UE may receive the MobilityFromNRCommand via SRB3 within DLInformationTransferMRDC.

5.7	Other

#### 5.7.y.4 Actions related to transmission of MCGFailureInformation message

The UE shall set the contents of the MCGFailureInformation message as follows:

1> include and set failureType in accordance with 5.7.y.3;

1>	for each MeasObjectNR configured by a measConfig associated with the MCG, and for which measurement results are available:

2>	include an entry in measResultFreqList;

2>	if there is a measId configured with the MeasObjectNR and a reportConfig which has rsType set to ssb:

3>	set ssbFrequency in measResultFreqList to the value indicated by ssbFrequency as included in the MeasObjectNR;

2>	if there is a measId configured with the MeasObjectNR and a reportConfig which has rsType set to csi-rs:

3>	set refFreqCSI-RS in measResultFreqList to the value indicated by refFreqCSI-RS as included in the associated measurement object;

2>	if a serving cell is associated with the MeasObjectNR:

3>	set measResultServingCell in measResultFreqList to include the available quantities of the concerned cell and in accordance with the performance requirements in TS 38.133 [14];

2>	set the measResultNeighCellList in measResultFreqList to include the best measured cells, ordered such that the best cell is listed first, and based on measurements collected up to the moment the UE detected the failure, and set its fields as follows;

3>	ordering the cells with sorting as follows:

4>	based on SS/PBCH block if SS/PBCH block measurement results are available and otherwise based on CSI-RS;

4>	using RSRP if RSRP measurement results are available, otherwise using RSRQ if RSRQ measurement results are available, otherwise using SINR;

3>	for each neighbour cell included:

4>	include the optional fields that are available.

1>	for each EUTRA frequency the UE is configured to measure by measConfig for which measurement results are available:

2>	set the measResultFreqListEUTRA to include the best measured cells, ordered such that the best cell is listed first using RSRP to order if RSRP measurement results are available for cells on this frequency, otherwise using RSRQ to order if RSRQ measurement results are available for cells on this frequency, otherwise using SINR to order, and based on measurements collected up to the moment the UE detected the failure, and for each cell that is included, include the optional fields that are available;

1>	for each UTRA-FDD frequency the UE is configured to measure by measConfig for which measurement results are available:

2>	set the measResultFreqListUTRA-FDD to include the best measured cells, ordered such that the best cell is listed first using RSCP to order if RSCP measurement results are available for cells on this frequency, otherwise using EcN0 to order if EcN0 measurement results are available for cells on this frequency, and based on measurements collected up to the moment the UE detected the failure, and for each cell that is included, include the optional fields that are available;

1> if the UE is in NR-DC:

2>	include and set measResultSCG in accordance with 5.7.3.4;

1> if the UE is in NE-DC:

2>	include and set measResultSCG-EUTRA in accordance with TS 36.331 [10] clause 5.6.13.5;

NOTE 1:	The measured quantities are filtered by the L3 filter as configured in the mobility measurement configuration. The measurements are based on the time domain measurement resource restriction, if configured. Blacklisted cells are not required to be reported.

NOTE 2:	Field measResultSCG-Failure is used to report available results for NR frequencies the UE is configured to measure by SCG RRC signalling.

NOTE 3:	Field measResultSCG-EUTRA is used to report available results for E-UTRAN frequencies the UE is configured to measure by E-UTRA RRC signalling.

1>	if SRB1 is configured as split SRB and pdcp-Duplication is not configured:

2>	if the primaryPath for the PDCP entity of SRB1 refers to the MCG:

3> set the primaryPath to refer to the SCG.

The UE shall:

- start timer T316;

1>	if SRB1 is configured as split SRB:

2>	submit the MCGFailureInformation message to lower layers for transmission via SRB1, upon which the procedure ends;

2>	else (i.e. SRB3 configured):

3>	submit the MCGFailureInformation message to lower layers for transmission embedded in NR RRC message ULInformationTransferMRDC via SRB3 as specified in 5.7.2a.3.

- 6.2.2	Message definitions

- –	DLInformationTransferMRDC

The DLInformationTransferMRDC message is used for the downlink transfer of RRC messages (e.g. for transferring NR or E-UTRA RRC connection reconfiguration RRC connection release or inter-RAT handover message) over SRB3 during fast MCG link recovery via SRB3.

Signalling radio bearer: SRB3

RLC-SAP: AM

Logical channel: DCCH

Direction: Network to UE

DLInformationTransferMRDC message

-- ASN1START

-- TAG-DLINFORMATIONTRANSFERMRDC-START

DLInformationTransferMRDC-r16 ::=               SEQUENCE {

criticalExtensions                          CHOICE {

c1                                          CHOICE {

dlInformationTransferMRDC-r16               DLInformationTransferMRDC-r16-IEs,

spare3 NULL, spare2 NULL, spare1 NULL

},

criticalExtensionsFuture            SEQUENCE {}

}

}

DLInformationTransferMRDC-r16-IEs::=           SEQUENCE {

dl-DCCH-MessageNR-r16                       OCTET STRING                    OPTIONAL,

dl-DCCH-MessageEUTRA-r16                    OCTET STRING                    OPTIONAL,

lateNonCriticalExtension                    OCTET STRING                    OPTIONAL,

nonCriticalExtension                        SEQUENCE {}                     OPTIONAL

}

-- TAG-DLINFORMATIONTRANSFERMRDC-STOP

-- ASN1STOP

DLInformationTransferMRDC field descriptions

dl-DCCH-MessageNR Includes the DL-DCCH-Message. In this version of the specification, the field is only used to transfer the NR RRCReconfiguration, RRCRelease and MobilityFromNRCommand messages.

dl-DCCH-MessageEUTRA Includes the DL-DCCH-Message. In this version of the specification, the field is only used to transfer the E-UTRA RRCConnectionReconfiguration, RRCConnectionRelease and MobilityFromEUTRACommand messages as specified in TS 36.331 [10].

- –	MCGFailureInformation

The MCGFailureInformation message is used to provide information regarding NR MCG failures detected by the UE.

Signalling radio bearer: SRB1

RLC-SAP: AM

Logical channel: DCCH

Direction: UE to Network

MCGFailureInformation message

-- ASN1START

-- TAG-MCGFAILUREINFORMATION-START

MCGFailureInformation-r16 ::=             SEQUENCE {

criticalExtensions                         CHOICE {

mcgFailureInformation-r16                MCGFailureInformation-r16-IEs,

criticalExtensionsFuture                 SEQUENCE {}

}

}

MCGFailureInformation-r16-IEs ::=         SEQUENCE {

failureReportMCG-r16                         FailureReportMCG-r16            OPTIONAL,

lateNonCriticalExtension                     OCTET STRING                    OPTIONAL,

nonCriticalExtension                         SEQUENCE {}                     OPTIONAL

}

FailureReportMCG-r16 ::=                   SEQUENCE {

failureType-r16                                ENUMERATED {

t310-Expiry, randomAccessProblem,

rlc-MaxNumRetx, spare},

measResultFreqList-r16                          MeasResultList2NR                                          OPTIONAL,

measResultFreqListEUTRA-r16                    MeasResultList2EUTRA                                      OPTIONAL,

measResultFreqListUTRA-FDD-r16                MeasResultListUTRA-FDD-r16                               OPTIONAL,

measResultSCG-r16                                OCTET STRING (CONTAINING MeasResultSCG-Failure)     OPTIONAL,

measResultSCG-EUTRA-r16                         OCTET STRING                                               OPTIONAL,

...

}

MeasResultList2EUTRA ::=                   SEQUENCE (SIZE (1..maxFreq)) OF MeasResult2EUTRA

-- TAG-MCGFAILUREINFORMATION-STOP

-- ASN1STOP

MCGFailureInformation field descriptions

measResultFreqList The field contains available results of measurements on NR frequencies the UE is configured to measure by the measConfig associated with the MCG.

measResultFreqListEUTRA The field contains available results of measurements on E-UTRA frequencies the UE is configured to measure by measConfig associated with the MCG.

measResultFreqListUTRA-FDD-r16 The field contains available results of measurements on UTRA-FDD frequencies the UE is configured to measure by measConfig associated with the MCG.

measResultSCG The field contains the MeasResultSCG-Failure IE which includes available measurement results on NR frequencies the UE is configured to measure by the measConfig associated with the SCG.

measResultSCG-EUTRA The field contains the EUTRA MeasResultSCG-FailureMRDC IE which includes available results of measurements on E-UTRA frequencies the UE is configured to measure by the E-UTRA RRCConnectionReconfiguration message as specified in TS 36.331 [10].

## Draft CR to TS 36.331

### 5.4.3	Mobility from E-UTRA

#### 5.4.3.1	General

Figure 5.4.3.1-1: Mobility from E-UTRA, successful

Figure 5.4.3.1-2: Mobility from E-UTRA, failure

The purpose of this procedure is to move a UE in RRC_CONNECTED to a cell using another Radio Access Technology (RAT), e.g. GERAN, UTRA, CDMA2000 systems, NR, or handover a UE to an E-UTRA cell connected to another type of CN. The mobility from E-UTRA procedure covers the following type of mobility:

-	handover, i.e. the MobilityFromEUTRACommand message includes radio resources that have been allocated for the UE in the target cell;

-	cell change order, i.e. the MobilityFromEUTRACommand message may include information facilitating access of and/ or connection establishment in the target cell, e.g. system information. Cell change order is applicable only to GERAN; and

-	enhanced CS fallback to CDMA2000 1xRTT, i.e. the MobilityFromEUTRACommand message includes radio resources that have been allocated for the UE in the target cell. The enhanced CS fallback to CDMA2000 1xRTT may be combined with concurrent handover or redirection to CDMA2000 HRPD.

NOTE:	For the case of dual receiver/transmitter enhanced CS fallback to CDMA2000 1xRTT, the DLInformationTransfer message is used instead of the MobilityFromEUTRACommand message (see TS 36.300 [9]).

- 5.4.3.2	Initiation

E-UTRAN initiates the mobility from E-UTRA procedure to a UE in RRC_CONNECTED, possibly in response to a MeasurementReport or MCGFailureInformation message or in response to reception of CS fallback indication for the UE from MME, by sending a MobilityFromEUTRACommand message. E-UTRAN applies the procedure as follows:

-	the procedure is initiated only when AS-security has been activated, and SRB2 with at least one DRB are setup and not suspended;

Note: The UE may receive the MobilityFromNRCommand via SRB3 within DLInformationTransferMRDC.