3GPP TSG-RAN2 Meeting #109-e-Bis	R2-2003267

# Online, 20th Apr 2020 - 30th Apr 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.321 | CR |  | rev | - | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on TA timer maintenance |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation, Sanechips |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NB_IOTenh3-Core |  |  |  |  |  | Date: |  |  | 2020-04-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Before initiating the transmission using PUR, RRC shall make sure that the UE has valid timing alignment. According to the description in 5.3.3.19 of 36.331, RRC shall check whether pur-TimeAlignmentTimer is running as confirmed by lower layers before initiating transmission using PUR. Moreover, pur-TimeAlignmentTimer is maintained in MAC as 5.4.7.2 in 36.321. It can be seen that when a pur-TimeAlignmentTimer expires, MAC send an indication to RRC. However, when TA updated command is received, MAC doesn’t inform RRC. That means, when pur-TimeAlignmentTimer expires, RRC can know that and consider the TA invalid. If later MAC restarts the pur-TimeAlignmentTimer after receiving a TA command, RRC cannot know that and still consider the TA invalid. |  |  |  |  |  |  |  |  |
| Summary of change: |  | One possible way to resolve such inconsistence may be to trigger another indication at the restart of pur-TimeAlignmentTimer. But since anyway RRC shall check whether pur-TimeAlignmentTimer is running with MAC before every time it initiates PUR procedure, we think the MAC-to-RRC indications either at the expiry of PUR TA timer or at the restart of pur-TimeAlignmentTimer would be redundant. Thus, we propose to delete the existing indication sent at the expiry of PUR TA time. Impact Analysis Impacted functionality: TA timer maintenance. Inter-operability: N/A. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | If not, if there has an expiry of TA timer but later this TA timer restarts after receiving a TA command, RRC cannot know that and still consider the TA invalid. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4.7.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of the first change>

5.4.7.2	Maintenance of PUR Uplink Time Alignment

MAC entity may have a configurable timer pur-TimeAlignmentTimer when upper layers have configured Preconfigured Uplink Resource.

The MAC entity shall:

-	when pur-TimeAlignmentTimer configuration is received from upper layers:

-	start pur-TimeAlignmentTimer.

-	if upper layers indicate PUR TA is validated:

-	start or restart the pur-TimeAlignmentTimer.

-	when a Timing Advance Command MAC control element is received or PDCCH indicates timing advance adjustment as specified in TS 36.212 [5]:

-	apply the Timing Advance Command or the timing advance adjustment;

-	start or restart the pur-TimeAlignmentTimer.

Editor's note: How RRC indicates to MAC that TA is valid or instructs MAC to use PUR.

Upon request from upper layers, MAC entity shall indicate if pur-TimeAlignmentTimer is running or not.

Editor's note: FFS whether cell change can be captured in MAC or whether only in RRC and the exact interaction needed.

<End of the first change>