3GPP TSG-RAN WG2 Meeting #109bis Electronic	R2-2003449

# 20th – 30th April, 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR |  | rev |  | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | support of EN-DC FDD+TDD HPUE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | ENDC_UE_PC2_FDD_TDD-Core |  |  |  |  |  | Date: |  |  | 2020-02-12 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The support of high power UE for FDD+TDD BC in ENDC operation is discussed in RAN4 and the following agreements are captured in R4-2002835 [1]: - Two fixed LTE reference configurations of the PC2 FDD-TDD EN-DC, DutyLTE1=70% , DutyLTE2=40% were agreed - Average of two proposed options i.e. [IMD2: 31.9 dB] ; [IMD4: 18.5dB] for DC_3A_n78 combination were agreed - The following capability set was agreed, corresponding to DutyLTE1 and DutyLTE2. - maxNRDuty1 ∈ {30%, 40%, 50%, 60%, 70%, 80%, 90%, 100% , Full_duty_supported} - maxNRDuty2 ∈ {30%, 40%, 50%, 60%, 70%, 80%, 90%, 100% } - Full_duty_supported: no restriction on uplink scheduling for both LTE and NR bands for applicability of PC2 inter-band EN-DC (FDD+TDD) requirements, i.e. SAR compliance will be fulfilled by UE based mechanisms e.g. P-MPR etc. If UE indicate maxNRDuty1= Full_duty_supported, maxNRDuty2 signaling will be overridden i.e. UE will follow Full_duty_supported capability. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add max NR duty report in the MR-DC parameres. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The supported duty cycle is not known by the network when high power UE for FDD+TDD BC operates in ENDC operation |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | Y | Other core specifications |  |  |  | R2-2003450 38.306 |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of modification>

#### –	MRDC-Parameters

The IE MRDC-Parameters contains the band combination parameters specific to MR-DC for a given MR-DC band combination.

MRDC-Parameters information element

-- ASN1START

-- TAG-MRDC-PARAMETERS-START

MRDC-Parameters ::= SEQUENCE {

singleUL-Transmission               ENUMERATED {supported}              OPTIONAL,

dynamicPowerSharingENDC             ENUMERATED {supported}              OPTIONAL,

tdm-Pattern                         ENUMERATED {supported}              OPTIONAL,

ul-SharingEUTRA-NR                  ENUMERATED {tdm, fdm, both}         OPTIONAL,

ul-SwitchingTimeEUTRA-NR            ENUMERATED {type1, type2}           OPTIONAL,

simultaneousRxTxInterBandENDC       ENUMERATED {supported}              OPTIONAL,

asyncIntraBandENDC                  ENUMERATED {supported}              OPTIONAL,

...,

[[

dualPA-Architecture                 ENUMERATED {supported}              OPTIONAL,

intraBandENDC-Support               ENUMERATED {non-contiguous, both}   OPTIONAL,

ul-TimingAlignmentEUTRA-NR          ENUMERATED {required}               OPTIONAL

]],

[[

maxUplinkDutyCycle-interBandENDC-TDD-PC2-r16    SEQUENCE{

eutra-TDD-Config0-r16    ENUMERATED {n20, n40, n50, n60, n70, n80, n90, n100}    OPTIONAL,

eutra-TDD-Config1-r16    ENUMERATED {n20, n40, n50, n60, n70, n80, n90, n100}    OPTIONAL,

eutra-TDD-Config2-r16    ENUMERATED {n20, n40, n50, n60, n70, n80, n90, n100}    OPTIONAL,

eutra-TDD-Config3-r16    ENUMERATED {n20, n40, n50, n60, n70, n80, n90, n100}    OPTIONAL,

eutra-TDD-Config4-r16    ENUMERATED {n20, n40, n50, n60, n70, n80, n90, n100}    OPTIONAL,

eutra-TDD-Config5-r16    ENUMERATED {n20, n40, n50, n60, n70, n80, n90, n100}    OPTIONAL,

eutra-TDD-Config6-r16    ENUMERATED {n20, n40, n50, n60, n70, n80, n90, n100}    OPTIONAL

}        OPTIONAL

]] ,

[[

maxUplinkDutyCycle-BandENDC-FDD-TDD-PC2-r16    CHOICE {

full-duty-supported               ENUMERATED {true},

Partial-duty-supported            SEQUENCE {

dutyLTE1                ENUMERATED {n30, n40, n50, n60,n70,n80,90,100},

dutyLTE2                ENUMERATED {n30, n40, n50, n60,n70,n80,90,100}

}

}      OPTIONAL

]]

}

MRDC-Parameters-v1580 ::= SEQUENCE {

dynamicPowerSharingNEDC             ENUMERATED {supported}              OPTIONAL

}

MRDC-Parameters-v1590 ::=	SEQUENCE {

interBandContiguousMRDC             ENUMERATED {supported}              OPTIONAL

}

-- TAG-MRDC-PARAMETERS-STOP

-- ASN1STOP

<End of modification>