# 3GPP TSG RAN WG2 Meeting #109bis-e      			                          R2- 2003535
E-Conference, 20 April – 30 April, 2020

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.323 | CR | xxxx | rev | - | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR to 38.323 for NR PC5-S and PDCP header |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_V2X_NRSL-Core |  |  |  |  |  |  | Date: |  | 2020-04-02 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | - Update to reflect NR PC5 SL unicast security parameter naming convention, 3GPP references for security establishment per 33.536. - Update to reflect NR PC5-signaling for unicast call establishment per 24.587 - Introduce PDCP PDU format for SLRB based on the following RAN2 agreements: RAN2#108 Agreements on PDCP: 9: For SL unicast, only 12bits pdcp-SN-Size is used for the SL SRB including SL SRBs which carrying PC5 Signalling and PC5 RRC message. FFS on how to handle initial PC5-S signaling for unicast link. RAN2#109e Agreements on PDCP: 3: The PDCP SN size of Direct Communication Request message is 18 bits |  |  |  |  |  |  |  |  |
| Summary of change: |  | Incorporate updates to NR SL unicast security parameter naming. Clarify NR SL unicast ciphering and deciphering, and integrity protection and verification are established using PC5-S Clarify 3GPP references for NR SL unicast ciphering and deciphering, and integrity protection and verification. Update PDCP PDU format for SLRB for unicast, for 12-bit and 18-bit PDCP SN sizes |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The specification will be incomplete in citation of references, potentially leading to incorrect implementation. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.8, 5.9, 6.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

Start of 1st change

## 5.8	Ciphering and deciphering

The ciphering function includes both ciphering and deciphering and is performed in PDCP, if configured. The data unit that is ciphered is the MAC-I (see clause 6.3.4) and the data part of the PDCP Data PDU (see clause 6.3.3) except the SDAP header and the SDAP Control PDU if included in the PDCP SDU. The ciphering is not applicable to PDCP Control PDUs.

For downlink and uplink, the ciphering algorithm and key to be used by the PDCP entity are configured by upper layers TS 38.331 [3] and the ciphering method shall be applied as specified in TS 33.501 [6].

For downlink and uplink, the ciphering function is activated/suspended/resumed by upper layers TS 38.331 [3]. When security is activated and not suspended, the ciphering function shall be applied to all PDCP Data PDUs indicated by upper layers TS 38.331 [3] for the downlink, and uplink.

For downlink and uplink ciphering and deciphering, the parameters that are required by PDCP for ciphering are defined in TS 33.501 [6] and are input to the ciphering algorithm. The required inputs to the ciphering function include the COUNT value, and DIRECTION (direction of the transmission: set as specified in TS 33.501 [6]).The parameters required by PDCP which are provided by upper layers TS 38.331 [3] are listed below:

-	BEARER (defined as the radio bearer identifier in TS 33.501 [6]. It will use the value RB identity –1 as in TS 38.331 [3]);

-	KEY (the ciphering keys for the control plane and for the user plane are KRRCenc and KUPenc, respectively).

For NR sidelink communication, the ciphering algorithm and key to be used by the PDCP entity are configured by PC5-S [CT1 24.587], and the ciphering method shall be applied as specified in TS 33.536 [xx2].

For NR sidelink communication, the ciphering function is activated/suspended/resumed by PC5-S. When security is activated and not suspended, the ciphering function shall be applied to all PDCP Data PDUs indicated by TS 33.536 [xx2].

For NR sidelink communication, the ciphering function as specified in TS 33.501 [6] is applied with KEY (NRPEK), COUNT, BEARER and DIRECTION (which value shall be set is specified in TS 33.536 [xx2]) as input.

End of 1st change

Start of 2nd change

## 5.9   Integrity protection and verification

The integrity protection function includes both integrity protection and integrity verification and is performed in PDCP, if configured. The data unit that is integrity protected is the PDU header and the data part of the PDU before ciphering. The integrity protection is always applied to PDCP Data PDUs of SRBs. The integrity protection is applied to sidelink SRBs except for the sidelink SRB carrying unprotected PC5-S message (e.g. Direct Communication Request). The integrity protection is applied to PDCP Data PDUs of DRBs and sidelink DRBs of NR sidelink communication for unicast for which integrity protection is configured. The integrity protection is not applicable to PDCP Control PDUs.

For downlink and uplink, the integrity protection algorithm and key to be used by the PDCP entity are configured by upper layers TS 38.331 [3] and the integrity protection method shall be applied as specified in TS 33.501 [6].

For downlink and uplink, the integrity protection function is activated/suspended/resumed by upper layers TS 38.331 [3]. When security is activated and not suspended, the integrity protection function shall be applied to all PDUs including and subsequent to the PDU indicated by upper layers TS 38.331 [3] for the downlink, and the uplink.

NOTE:	As the RRC message which activates the integrity protection function is itself integrity protected with the configuration included in this RRC message, this message needs first be decoded by RRC before the integrity protection verification could be performed for the PDU in which the message was received.

NOTE:	As the PC5-S message which activates the integrity protection function is itself integrity protected with the configuration included in this PC5-S message, this message needs first be decoded by upper layer before the integrity protection verification could be performed for the PDU in which the message was received.

For downlink and uplink integrity protection and verification, the parameters that are required by PDCP for integrity protection are defined in TS 33.501 [6] and are input to the integrity protection algorithm. The required inputs to the integrity protection function include the COUNT value, and DIRECTION (direction of the transmission: set as specified in TS 33.501 [6]). The parameters required by PDCP which are provided by upper layers TS 38.331 [3] are listed below:

-	BEARER (defined as the radio bearer identifier in TS 33.501 [6]. It will use the value RB identity –1 as in TS 38.331 [3]);

-	KEY (the integrity protection keys for the control plane and for the user plane are KRRCint and KUPint, respectively).

For NR sidelink communication, the integrity protection algorithm and key to be used by the PDCP entity are configured by upper layers TS 24.587 [xx3] and the integrity protection method shall be applied as specified in TS 33.536 [xx2].

For NR sidelink communication, the integrity protection function is activated/suspended/resumed by PC5-S. When security is activated and not suspended, the integrity protection function shall be applied to all PDUs including and subsequent to the PDU indicated by TS 33.536 [xx2].

For the SLRB that needs integrity protection and verification, the parameters that are required by PDCP for integrity protection are defined in TS 33.536 [xx2] and are input to the integrity protection algorithm. The required inputs to the integrity protection function include the KEY (NRPIK), COUNT, BEARER and DIRECTION (which value shall be set is specified in TS 33.536 [xx2]).

At transmission, the UE computes the value of the MAC-I field and at reception it verifies the integrity of the PDCP Data PDU by calculating the X-MAC based on the input parameters as specified above. If the calculated X-MAC corresponds to the received MAC-I, integrity protection is verified successfully.

End of 2nd change

Start of 3nd change

#### 6.2.2.Y	Data PDU for SLRBs for unicast

Figure 6.2.2.Y-1 shows the format of the PDCP Data PDU with 18 bit PDCP SN. This format is applicable for sidelink DRBs for unicast and for the sidelink SRB carrying the unprotected PC5-S messages (e.g., Direct Communication Request). MAC-I field is used only for the SLRB that needs integrity protection.

Figure 6.2.2.Y-1: PDCP Data PDU format for SLRBs for unicast, 18-bit PDCP SN

Figure 6.2.2.Y-2 shows the format of the PDCP Data PDU with 12 bit PDCP SN. This format is applicable for sidelink DRBs for unicast and for the sidelink SRB carrying PC5-S messages (including Direct Security Mode Command and Direct Security Mode Complete) and PC5-RRC messages. MAC-I field is used only for the SLRB that needs integrity protection.

Figure 6.2.2.Y-2: PDCP Data PDU format for SLRBs for unicast, 12-bit PDCP SN

End of 3nd change