3GPP TSG-RAN WG2 #109bis-e	 	     R2-2003666

April 20th - 30th, 2020

Agenda item:	   6.13.2 (NR_2step_RACH-Core)

Source:	LG Electronics Inc.

Title: 	Further clarifications on parameters for Random Access procedure

Document for:	Discussion and Decision

1.	Introduction

RAN has approved RAN2 CRs for 2-step RACH to TS 38.300, TS 38.321 and TS 38.331. This document discusses the parameters either which have a mismatch between RRC and MAC or which need to be clarified.

2.	Discussion

#1: msgA-RSRP-ThresholdSUL and msgA-RSRP-Threshold

In MAC, these parameters are always used to set UE variable for RA type selection when the carrier to use for Random Access procedure is not explicitly signalled, as described in the section 5.1.1:

TS 38.321: 5.1.1 Random Access procedure initialization

1>	if the carrier to use for the Random Access procedure is explicitly signalled:
2>	select the signalled carrier for performing Random Access procedure;
2>	set the PCMAX to PCMAX,f,c of the signalled carrier.

1>	else if the carrier to use for the Random Access procedure is not explicitly signalled; and

1>	if the Serving Cell for the Random Access procedure is configured with supplementary uplink as specified in TS 38.331 [5]; and

1>	if the RSRP of the downlink pathloss reference is less than rsrp-ThresholdSSB-SUL:
2>	select the SUL carrier for performing Random Access procedure;
2>	set the PCMAX to PCMAX,f,c of the SUL carrier;
2> set the RSRP_THRESHOLD_RA_TYPE_SELECTION to msgA-RSRP-ThresholdSUL.

1>	else:
2>	select the NUL carrier for performing Random Access procedure;
2>	set the PCMAX to PCMAX,f,c of the NUL carrier;
2> set the RSRP_THRESHOLD_RA_TYPE_SELECTION to msgA-RSRP-Threshold.

But, in RRC, these parameters are only present if both 2-step and 4-step RA type are configured for a BWP, as followings:

TS 38.331: 6.3.2 Radio resource control information elements

msgA-RSRP-ThresholdSUL

The UE selects 2-step random access type to perform random access if SUL carrier is selected based on this threshold (see TS 38.321 [3], clause 5.1.1). This field is only present if both 2-step and 4-step RA type are configured for the BWP.

msgA-RSRP-Threshold

The UE selects 2-step random access type to perform random access based on this threshold (see TS 38.321 [3], clause 5.1.1). This field is only present if both 2-step and 4-step RA type are configured for the BWP.

Considering that 4-step and 2-step RA types can be independently configured per BWP, and RA type is selected based on RSRP threshold when both RA types are configured on a BWP, it is desirable for these parameters to be present only for BWP where these RA types are configured, as already described in RRC. Moreover, the parameters msgA-RSRP-ThresholdSUL and msgA-RSRP-Threshold for the BWP can have different values from the parameters for other BWPs. Therefore, setting the UE variable for RA type selection should be performed after performing BWP operation. In addition, these parameters (if configured) should be used to set the UE variable regardless of whether or not the carrier to use for Random Access procedure is explicitly signaled.

Proposal 1. A BWP can be configured with different values of msgA-RSRP-ThresholdSUL and msgA-RSRP-Threshold from other BWPs.

Proposal 2. Adopt the following TP for the section 5.1.1 of TS 38.321:

1>	if the carrier to use for the Random Access procedure is explicitly signalled:
2>	select the signalled carrier for performing Random Access procedure;
2>	set the PCMAX to PCMAX,f,c of the signalled carrier.

1>	else if the carrier to use for the Random Access procedure is not explicitly signalled; and

1>	if the Serving Cell for the Random Access procedure is configured with supplementary uplink as specified in TS 38.331 [5]; and

1>	if the RSRP of the downlink pathloss reference is less than rsrp-ThresholdSSB-SUL:
2>	select the SUL carrier for performing Random Access procedure;
2>	set the PCMAX to PCMAX,f,c of the SUL carrier.

1>	else:
2>	select the NUL carrier for performing Random Access procedure;
2>	set the PCMAX to PCMAX,f,c of the NUL carrier.

1>	perform the BWP operation as specified in clause 5.15;

1>	if the BWP selected for random access procedure is configured with both 2-step and 4-step RA type random access resources:
2> if the selected carrier is the SUL carrier:
  3> set the RSRP_THRESHOLD_RA_TYPE_SELECTION to msgA-RSRP-ThresholdSUL.
2> else if the selected carrier is the NUL carrier:
  3> set the RSRP_THRESHOLD_RA_TYPE_SELECTION to msgA-RSRP-Threshold.

#2: PREAMBLE_POWER_RAMPING

At RAN1#99, RAN1 made the following agreement on power offset to use after RA type is switched to 4-step RA:

After 2-step RACH switches to 4-step RACH, the target power level follow the 4-step RACH formula plus an offset, where the offset is

OFFSET = (N -1) × (PREAMBLE_POWER_RAMPING_STEP_2_step - PREAMBLE_POWER_RAMPING_STEP_4_step)
N is the power ramping counter when switching back to 4-step

In MAC, the power offset is set by using PREAMBLE_POWER_RAMPING_COUNTER, PREAMBLE_POWER_RAMPING_STEP_2_step and PREAMBLE_POWER_RAMPING, as described in the section 5.1.1a:

TS 38.321: 5.1.1a Initialization of variables specific to Random Access type

2> if RA_TYPE is switched from 2-stepRA to 4-stepRA during this random access procedure:
3> set POWER_OFFSET_2STEP_RA to (PREAMBLE_POWER_RAMPING_COUNTER – 1) × (MSGA_PREAMBLE_POWER_RAMPING_STEP - PREAMBLE_POWER_RAMPING).

The highlighted parameters with gray or green are aligned between RAN1 agreement and MAC, whereas the highlighted parameter with yellow is NOT matched between them.

Therefore, the power offset should be set by using PREAMBLE_POWER_RAMPING_STEP, not PREAMBLE_POWER_RAMPING.

Proposal 3. Adopt the following TP for the section 5.1.1a of TS 38.321:

2> if RA_TYPE is switched from 2-stepRA to 4-stepRA during this random access procedure:
3> set POWER_OFFSET_2STEP_RA to (PREAMBLE_POWER_RAMPING_COUNTER – 1) × (MSGA_PREAMBLE_POWER_RAMPING_STEP - PREAMBLE_POWER_RAMPING_STEP).

#3: preambleTransMax and msgA-TransMax for 2-step RA type and preambleTransMax for 4-step RA type

According to TS 38.331, preambleTransMax for 4-step RA type and preambleTransMax for 2-step RA type are always included in RACH-ConfigGeneric and RACH-ConfigGenericTwoStepRA, respectively. msgA-TransMax for 2-step RA type is included in RACH-ConfigGenericTwoStepRA when network allows UE to switch from 2-step RA type to 4-step RA type.

TS 38.331: 6.3.2 Radio resource control information elements

RACH-ConfigGeneric field descriptions

preambleTransMax Max number of RA preamble transmission performed before declaring a failure (see TS 38.321 [3], clauses 5.1.4, 5.1.5).

RACH-ConfigGenericTwoStepRA field descriptions

preambleTransMax Max number of RA preamble transmission performed before declaring a failure (see TS 38.321 [3], clauses 5.1.4, 5.1.5).

msgA-TransMax Max number of MsgA preamble transmissions performed before switching to 4-step random access (see TS 38.321 [3], clauses 5.1.1). This field may only be applicable in case of 2-step and 4-step RA type are configured or switching to 4-step type RA is not supported.

Network can independently configure each preambleTransMax for 4-step and 2-step RA type when both 2-step and 4-step RA type are configured on a BWP. It can mean that these parameters can have different values. Thus, it is necessary to specify the restriction that msgA-TransMax should have a value less than preambleTransMax for 4-step RA type, not preambleTransMax for 2-step RA type. If the restriction is not specified, UE may NOT indicate a Random Access problem to upper layers. For example, in case that preambleTransMax and msgA-TransMax for 2-step RA type and preambleTransMax for 4-step RA type are 10, 8 and 7, respectively, the UE doesn’t indicate a Random Access problem to upper layers because PREAMBLE_TRANSMISSION_COUNTER (i.e., 9) is already larger than preambleTransMax for 4-step RA type (i.e., 7) when the UE switches to 4-step RA type.

Proposal 4. msgA-TransMax should have a value less than preambleTransMax for 4-step RA type, not preambleTransMax for 2-step RA type..

Proposal 5. Adopt the following TP for the section 6.3.2 of TS 38.331:

RACH-ConfigGenericTwoStepRA field descriptions

msgA-TransMax Max number of MsgA preamble transmissions performed before switching to 4-step random access (see TS 38.321 [3], clauses 5.1.1). This field may only be applicable in case of 2-step and 4-step RA type are configured or switching to 4-step type RA is supported. This field has a value less than preambleTransMax included in RACH-ConfigGeneric.

3.	Conclusion

In this contribution, we have discussed Random Access parameters, and our proposals are as follows.

Proposal 1. A BWP can be configured with different values of msgA-RSRP-ThresholdSUL and msgA-RSRP-Threshold from other BWPs.

Proposal 2. Adopt the following TP for the section 5.1.1 of TS 38.321:

1>	if the carrier to use for the Random Access procedure is explicitly signalled:
2>	select the signalled carrier for performing Random Access procedure;
2>	set the PCMAX to PCMAX,f,c of the signalled carrier.

1>	else if the carrier to use for the Random Access procedure is not explicitly signalled; and

1>	if the Serving Cell for the Random Access procedure is configured with supplementary uplink as specified in TS 38.331 [5]; and

1>	if the RSRP of the downlink pathloss reference is less than rsrp-ThresholdSSB-SUL:
2>	select the SUL carrier for performing Random Access procedure;
2>	set the PCMAX to PCMAX,f,c of the SUL carrier.
.

1>	else:
2>	select the NUL carrier for performing Random Access procedure;
2>	set the PCMAX to PCMAX,f,c of the NUL carrier.

1>	perform the BWP operation as specified in clause 5.15;

1>	if the BWP selected for random access procedure is configured with both 2-step and 4-step RA type random access resources:
2> if the selected carrier is the SUL carrier:
  3> set the RSRP_THRESHOLD_RA_TYPE_SELECTION to msgA-RSRP-ThresholdSUL.
2> else if the selected carrier is the NUL carrier:
  3> set the RSRP_THRESHOLD_RA_TYPE_SELECTION to msgA-RSRP-Threshold.

Proposal 3. Adopt the following TP for the section 5.1.1a of TS 38.321:

2> if RA_TYPE is switched from 2-stepRA to 4-stepRA during this random access procedure:
3> set POWER_OFFSET_2STEP_RA to (PREAMBLE_POWER_RAMPING_COUNTER – 1) × (MSGA_PREAMBLE_POWER_RAMPING_STEP - PREAMBLE_POWER_RAMPING_STEP).

Proposal 4. msgA-TransMax should have a value less than preambleTransMax for 4-step RA type, not preambleTransMax for 2-step RA type.

Proposal 5. Adopt the following TP for the section 6.3.2 of TS 38.331:

RACH-ConfigGenericTwoStepRA field descriptions

msgA-TransMax Max number of MsgA preamble transmissions performed before switching to 4-step random access (see TS 38.321 [3], clauses 5.1.1). This field may only be applicable in case of 2-step and 4-step RA type are configured or switching to 4-step type RA is supported. This field has a value less than preambleTransMax included in RACH-ConfigGeneric.