# 3GPP TSG RAN WG2 Meeting #109bis-e      			                          R2-xxxxxx
E-Conference, 20 April – 30 April, 2020

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.321 | CR | xxxx | rev | - | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR to 38.321 on SL process for reception |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  |  | Date: |  | 2019-10-02 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | - UE address matching for unicast and groupcast/broadcast differs. The current text on unicast address matching is incorrect and needs to be updated. Groupcast/broadcast: Rx UE compares the received destination ID (received in the SCI) with the destination ID the UE has for the group (i.e., the group ID the UE is participating in). Unicast: Rx UE compares the received destination ID (received in the SCI) with its own source ID (its own L2 ID), and compares the received source ID with the destination IDs for the unicast link(s) (i.e. the peer UE’s L2 ID). - RAN1 has agreed (38.213) whether to send HARQ feedback is an Upper Layer decision. The MAC spec should incorporate this decision based on successful/unsuccessful packet decoding. - An unsuccessfully decoded packet should be NAKed if the packet is unicast or if the packet is groupcast, and the address-matching criteria are met. - A successfully decoded packet should be ACKed if the packet is unicast and the unicast address-matching criteria are met - In case of unsuccessful decoding, the Rx UE needs to verify the L1 ID before sending the HARQ feedback. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Corrected unicast address matching description. - Add UE decision criteria for sending HARQ feedback based on successful/unsuccessful packet decoding. - Add UE verification of the address before sending HARQ feedback. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The specification will have incomplete and inadequate requirements for NR Sidelink. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Section 5.22.2.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

Start of 1st change

##### 5.22.2.2.2	Sidelink process

For each PSSCH duration where a transmission takes place for the Sidelink process, one TB and the associated HARQ information is received from the Sidelink HARQ Entity.

For each received TB and associated Sidelink transmission information, the Sidelink process shall:

1>	if this is a new transmission:

2>	attempt to decode the received data.

1>	else if this is a retransmission:

2>	if the data for this TB has not yet been successfully decoded:

3>	instruct the physical layer to combine the received data with the data currently in the soft buffer for this TB and attempt to decode the combined data.

1>	if the data which the MAC entity attempted to decode was successfully decoded for this TB; or

1>	if the data for this TB was successfully decoded before:

2>	if this is the first successful decoding of the data for this TB:

3>	if this TB is associated to unicast,  the DST field of the decoded MAC PDU subheader is equal to the 8 MSB of any of the Source Layer-2 ID(s) of the UE for which the 16 LSB are equal to the Destination ID in the corresponding SCI, and if the SRC field of the decoded MAC PDU subheader is equal to the 16 MSB of any of the Destination Layer-2 ID(s) of the UE for which the 8 LSB are equal to the Source ID in the corresponding SCI:

3>	if this TB is associated to groupcast or broadcast and the DST field of the decoded MAC PDU subheader is equal to the 8 MSB of any of the Destination Layer-2 ID(s) of the UE for which the 16 LSB are equal to the Destination ID in the corresponding SCI:

3>	deliver the decoded MAC PDU to the disassembly and demultiplexing entity;

3>	consider the Sidelink process as unoccupied.

1>	else:

2>	instruct the physical layer to replace the data in the soft buffer for this TB with the data which the MAC entity attempted to decode.

1>	if HARQ feedback is enabled by the SCI:

2>	if type 1 groupcast is indicated by the SCI according to clause 8.4.1 of TS 38.212 [9] and distance beteween UE’s location and the central location of the nearest zone indicated by the Zone_id in the SC is smaller or equal to the communication range:

3> if this TB was not successfully decoded:

4> if the 16 LSB of any of the Destination Layer-2 ID(s) of the UE is equal to the Destination ID in the corresponding SCI:

5> instruct the physical layer to generate a negative acknowledgement of the data in this TB

2> if type 1 groupcast is not indicated by the SCI according to clause 8.4.1 of TS 38.212 [9]:

3> if this TB was not successfully decoded:

4> if the 16 LSB of any of the Destination Layer-2 ID(s) of the UE is equal to the Destination ID in the corresponding SCI or:

4>if 16 LSB of any of the Source Layer-2 ID(s) of the UE is equal to the Destination ID in the corresponding SCI AND the 8 LSB of any of the Destination Layer-2 ID(s) of the UE is equal to the Source ID in the corresponding SCI:

5> instruct the physical layer to generate a negative acknowledgement of the data in this TB

3> if this TB was successfully decoded:

4>if the decoded MAC PDU subheader indicates unicast, and if the DST field of the decoded MAC PDU subheader is equal to the 8 MSB of any of the Source Layer-2 ID(s) of the UE for which the 16 LSB are equal to the Destination ID in the corresponding SCI, and if the SRC field of the decoded MAC PDU subheader is equal to the 16 MSB of any of the Destination Layer-2 ID(s) of the UE for which the 8 LSB are equal to the Source ID in the corresponding SCI; or

4> if the decoded MAC PDU subheader indicates it is not unicast

5>	instruct the physical layer to generate acknowledgement(s) of the data in this TB.

End of 1st change