3GPP TSG-RAN WG2 Meeting #110-e	R2-2005108

Online, June 1 – June 12, 2020

Agenda item:	6.8.2.4

Source:	Huawei, HiSilicon

Title:	Remaining issues in LPP ASN.1 issues

Document for:	Discussion and Decision

1	Introduction

After the previous meeting (RAN2#109bis-e), the issues for LPP ASN.1 are gathered and discussed in the email discussion as below.

[Post109bis-e][948][POS] LPP ASN.1 review (Qualcomm)

Scope: Gather and discuss issues and develop a running CR for ASN.1 corrections, with R2-2003981 as a baseline.

Intended outcome: Open issues list and CR to next meeting

Deadline:  Long

However, some issues are not concluded. In this contribution, we propose some remaining issues for LPP ASN.1.

2	Discussion

According to the latest LPP specification [1], we provide our suggestions for correction as follows.

- Change some fields to be optional.

  - E.g.,

- mutingOption2-r16, dl-PRS-MutingPatternList-r16, ssbSubcarrierSpacing-r16,

- nr-DL-TDOA-AdditionalMeasurements-r16.

- Add the conditional presence tag for some fields, e.g.,

  - For DL-PRS-IdInfo-r16 filed, conditional presence tag for the inter-dependency between the three fields (i.e., trp-ID-r16, nr-DL-PRS-ResourceID-List-r16, nr-DL-PRS-ResourceSetId-r16) is required.

- Add/Specify the field description for some IEs, e.g.,

  - The field descriptions of DL-PRS-IdInfo-r16, NR-TimeStamp-r16, , and NR-SelectedDL-PRS-PerFreq-r16 are missing.

  - The field description of dl-PRS-Periodicity-and-ResourceSetSlotOffset needs more clarification, including:

- 1) Whether the scs configuration here should be the same as the scs configuration in frequency layer.

- 2) The meaning of nxxx, e.g., n4-r16.

  - The field description of dl-PRS-ResourceSlotOffset needs more clarification, e.g., what does "1" stand for?

  - The field description of nrARFCNRef-r16 is not correct.

  - The field description of NR-SSB-Config field descriptions needs to be modified according to the following issues：

- 1) Some fields description are missing, e.g., TRP-ID.

- 2) ssb-periodicityServingCell field doesn't exist, which should be deleted.

- 3) The field description of ssb-Index should be more specific.

- Some typos need to be revised, e.g.,

  - nr-dl-PRS-ResourceSetList-r16   SEQUENCE (SIZE (1..nrMaxSetsPerTRP))  DL-PRS-ResourceSet-r16 (in IE NR-DL-PRS-Config) “SEQUENCE … OF”.

  - Bitmap size values: 2, 4, 6, 8, 16, 32 bits (in NR-DL-PRS-Config field descriptions) “The length of bitmap size”.

  - In case of multiple methods, the NR-DL-PRS-ProvideAssistanceData-r16 may only be present in one of the method (in IE NR-SelectedDL-PRS-PerFreq-r16)  “NR-DL-PRS-AssistanceData-r16”.

  - The IE DL-PRS-IdInfo provides IDs provides the IDs of the reference and neighbour TRPs DL-PRS Resources. (in IE DL-PRS-IdInfo)

  - Suggest to change the naming of “NR-UL-SRS-MeasCapability” since UE only transmits SRS, for example, can be revised as “NR-UL-SRS-TransCapability”.

- Other corrections:

  - In IE NR-AdditionalPath, NR-TOAMeasQuality-r16 field is not defined in the spec., which should be nr-TimingMeasQuality.

Proposal 1: Rapporteur to capture the above changes of ASN.1 in the latest 37.355.

3	Conclusions

In this document, we provide our suggested corrections for the LPP ASN.1 issues based on the latest LPP specification [1]. We have the following proposal:

- Proposal 1: Rapporteur to capture the above changes for ASN.1 issues.

4	References

- 3GPP TS 37.355, “LTE Positioning Protocol (LPP)”, V16.0.0 (2020-04).