3GPP TSG-RAN WG2 Meeting #110-e	R2-2005472

Electronic, 1st  – 12th  Jun, 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.322 | CR | 0035 | rev | X | Current version: | 15.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on the reception status of RLC STATUS PDU |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2020-06-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The available size of RLC status PDU is restricted by the total size of RLC PDU(s) indicated by MAC layer. For each missing RLC SDU segment, a set of NACK_SN, SOstart and SOend shall be included in the STATUS PDU, which will occupy 6 bytes (for 12 bit SN) or 7 bytes (for 18 bit SN) space. In case that a RLC SDU has been divided into multiple segments and several scattered segments are unsuccessfully received, tens of bytes of space is needed. If the available size of RLC status PDU is limited, there may exist a case where only partially missing SDU segments for a RLC SDU could be indicated in the RLC status PDU, while some other missing SDU segments for the same RLC SDU couldn’t be indicated. ACK_SN will be set as the next not received RLC SDU which is not reported as missing in the STATUS PDU. However, the transmitting side is not aware of this situation and may regard these missing segments have been successfully transmitted. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add a clarification of interpretation of the missing RLC SDU segment at the transmitting side of RLC AM entity in the STATUS PDU due to truncated NACK SN + SOstart + SOend Note: If the last RLC SDU segment reported as missing in the STATUS PDU is not the last SDU segment for its RLC SDU, the transmitting side of an AM RLC entity may interpret the reception status of data for the RLC SDU following this RLC SDU segment is not reported in the RLC STATUS PDU. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | If the CR is not approved, the missing segments may be flushed in UE’s buffer once the UE decodes the RLC status report successfully and never be retransmit anymore. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.3.10 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X X | Other core specifications |  |  |  | TS/TR ... CR ... TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

START OF CHANGES

6.2.3.10	Acknowledgement SN (ACK_SN) field

Length: 12 bits or 18 bits (configurable).

The ACK_SN field indicates the SN of the next not received RLC SDU which is not reported as missing in the STATUS PDU. When the transmitting side of an AM RLC entity receives a STATUS PDU, it interprets that all RLC SDUs up to but not including the RLC SDU with SN = ACK_SN have been received by its peer AM RLC entity, excluding those RLC SDUs indicated in the STATUS PDU with NACK_SN, portions of RLC SDUs indicated in the STATUS PDU with NACK_SN, SOstart and SOend, RLC SDUs indicated in the STATUS PDU with NACK_SN and NACK_range, and portions of RLC SDUs indicated in the STATUS PDU with NACK_SN, NACK range, SOstart and SOend.

Note: If the last RLC SDU segment reported as missing in the STATUS PDU is not the last SDU segment for its RLC SDU, the transmitting side of an AM RLC entity may interpret the reception status of data for the RLC SDU following this RLC SDU segment is not reported in the RLC STATUS PDU.

END OF CHANGES