# 3GPP TSG-RAN WG2 Meeting #110e					  R2-2005506

# E-meeting, June 1 – June 12, 2020

Agenda item:		6.7.4.1 (NR_IIOT-Core)

Source:	LG Electronics Inc.

Title: 	Indication of PDCP duplication configuration

Document for:	Discussion and Decision

1.	Introduction

In PDCP specification, it is required to indicate whether the PDCP entity is configured with PDCP duplication. In Rel-15, the pdcp-Duplication plays that role. However, in Rel-16, the pdcp-Duplication is absent when moreThanTwoRLC is configured, and it cannot be used to indicate that the PDCP entity is configured with PDCP duplication when more than two RLC entities are associated.

Consequently, the PDCP running CR R2-2004283 endorsed by [Post109bis-e][029] uses moreThanTwoRLC as PDCP duplication configuration parameter for Rel-16, as shown below.

5.11.1 and 5.11.2 For the PDCP entity configured with pdcp-Duplication or moreThanTwoRLC, the transmitting PDCP entity shall:

This document discusses alternative way to indicate PDCP duplication configuration for the PDCP entity associated with more than two RLC entities.

2.	Discussion

Though the current running CR works, it seems better to always use pdcp-Duplication for indicating PDCP duplication configuration from the PDCP point of view. The reason is as follows:

-	It is more intuitively understood that “PDCP configured with pdcp-Duplication” = “PDCP configured with PDCP duplication”. To understand that “PDCP configured with moreThanTwoRLC” = “PDCP configured with PDCP duplication”, readers may have to check further the RRC specification.

-	Currently, the moreThanTwoRLC is configured only for duplicate RB. However, if, in a future, a new type of RB having more than two RLC entities without PDCP duplication is introduced, the moreThanTwoRLC cannot be used for the new RB, and further change is required.

Therefore, we propose that the PDCP entity configured with PDCP duplication is always configured with pdcp-Duplication.

Proposal 1: The PDCP entity configured with PDCP duplication is always configured with pdcp-Duplication.

To support Proposal 1, some changes are needed in RRC specification. In 38.331 v16.0.0, the following descriptions are specified for pdcp-Duplication. The yellow highlighted text is a problematic part, and we propose to remove the text.

pdcp-Duplication Indicates whether or not uplink duplication status at the time of receiving this IE is configured and activated as specified in TS 38.323 [5]. The presence of this field indicates that duplication is configured. PDCP duplication is not configured for CA packet duplication of LTE RLC bearer. The value of this field, when the field is present, indicates the initial state of the duplication. If set to true, duplication is activated. The value of this field is always true, when configured for a SRB. This field is absent, if the field moreThanTwoRLC is present.

Proposal 2: Remove the text “This field is absent, if the field moreThanTwoRLC is present.” from the pdcp-Duplication description.

However, just removing the yellow highlighted text is not enough. This is because, for DRBs, the duplicationState included in the moreThanTwoRLC is used to indicate the initial state of each associated RLC entities. Thus, if the duplicationState is present, the value of pdcp-Duplication shall be ignored for DRBs.

Proposal 3: For DRBs, if the duplicationState is present, the value of pdcp-Duplication shall be ignored.

Finally, for SRBs, it is proposed that the pdcp-Duplication is always present when more than two RLC entities are associated. As the moreThanTwoRLC is absent for SRBs according to the RRC running CR R2-2002703, there is no other parameter to indicate PDCP duplication configuration than the pdcp-Duplication for SRBs.

Proposal 4: For SRBs, the pdcp-Duplication is always present.

3.	Proposals

In this document, we showed alternative way to indicate PDCP duplication configuration for the PDCP entity associated with more than two RLC entities. We have following proposals:

Proposal 1: The PDCP entity configured with PDCP duplication is always configured with pdcp-Duplication.

Proposal 2: Remove the text “This field is absent, if the field moreThanTwoRLC is present.” from the pdcp-Duplication description.

Proposal 3: For DRBs, if the duplicationState is present, the value of pdcp-Duplication shall be ignored.

Proposal 4: For SRBs, the pdcp-Duplication is always present.

The proposed changes to RRC running CR (R2-2002703) are shown below. Actual changes are left up to RRC CR rapporteur.

PDCP-Config field descriptions. moreThanTwoRLC This field configures UL data transmission when more than two RLC entities are associated with the PDCP entity for DRBs.. pdcp-Duplication Indicates whether or not uplink duplication status at the time of receiving this IE is configured and activated as specified in TS 38.323 [5]. The presence of this field indicates that duplication is configured. PDCP duplication is not configured for CA packet duplication of LTE RLC bearer. The value of this field, when the field is present, indicates the initial state of the duplication. If set to true, duplication is activated. This field is always present, and the value of this field is always true, when configured for a SRB. For DRBs, the value of this field shall be ignored if the duplicationState is present..