3GPP TSG RAN WG2 Meeting #110                              R2-2005673

e-Meeting, June 1th – 11th, 2020

Agenda item:	6.1.2

Source:	Samsung

Title:	Minor text correction for CR 38300 for IAB

Document for:	Discussion & Decision

Introduction

We found the minor correction for text in running CR 38.300, which can be corrected as proposed one.

Discussion

Since RRC reestablishment is not the formal terminology, the current written terminology should be changed into the former terminology in RRC spec, as RRC connection re-establishment in all the related part in running CR.

Proposal 1. RAN2 agree the correction from “RRC re-establishment” to “RRC connection re-establishment” as the suggested in the annex.

Conclusion

In this contribution, we proposed to correct the terminology “RRC re-establishment” into “RRC connection re-establishment”.

Annex

TP based on running 38.300CR (R2-2004133).

### 9.2.7	Radio Link Failure

In RRC_CONNECTED, the UE performs Radio Link Monitoring (RLM) in the active BWP based on reference signals (SSB/CSI-RS) and signal quality thresholds configured by the network. SSB-based RLM is based on the SSB associated to the initial DL BWP and can only be configured for the initial DL BWP and for DL BWPs containing the SSB associated to the initial DL BWP. For other DL BWPs, RLM can only be performed based on CSI-RS. In case of DAPS handover, the UE continues the RLM at the source cell until the successful completion of the random access procedure to the target cell.

The UE declares Radio Link Failure (RLF) when one of the following criteria are met:

-	Expiry of a radio problem timer started after indication of radio problems from the physical layer (if radio problems are recovered before the timer is expired, the UE stops the timer); or

-	Expiry of a timer started upon triggering a measurement report for a measurement identity for which the timer has been configured while another radio problem timer is running; or

-	Random access procedure failure; or

-	RLC failure; or

-	Detection of consistent uplink LBT failures for operation with shared spectrum channel access as described in 5.6.1; or

-    For IAB-MT, the reception of BH RLF indication received from its parent node.

In case of regular handover, after RLF is declared, the UE:

-	stays in RRC_CONNECTED;

-	selects a suitable cell and then initiates RRC connection re-establishment;

-	enters RRC_IDLE if a suitable cell was not found within a certain time after RLF was declared.

In case of DAPS handover, if RLF is declared in source cell, the UE:

-	stays in RRC_CONNECTED;

-	stops any data transmission or reception via the source link and releases the source link, but maintains the source RRC configuration;

In case of DAPS handover, when handover failure is declared at the target cell after source cell RLF was declared, the UE:

-	stays in RRC_CONNECTED;

-	selects a suitable cell and then initiates RRC connection re-establishment;

-	enters RRC_IDLE if a suitable cell was not found within a certain time after handover failure was declared.

In case of CHO, after RLF is declared in source cell, the UE:

-	stays in RRC_CONNECTED;

-	selects a suitable cell and if the selected cell is a CHO candidate and if network configured the UE to try CHO after RLF then the UE attempts CHO execution once, otherwise re-establishment is performed;

-	enters RRC_IDLE if a suitable cell was not found within a certain time after RLF was declared.

When RLF occurs at the IAB BH link, the same mechanisms and procedures are applied as for the access link. This includes BH RLF detection and RLF recovery using RRC connection re-establishment procedure.

In case the RRC connection re-establishment procedure fails, the IAB-node may transmit an BH RLF indication to its child nodes. The BH RLF indication is transmitted on the BAP sublayer.