3GPP TSG-RAN WG2 #110-e	R2-2006031

Electronic meeting, 1st – 12th June 2020	Revision of R2-2004622

Agenda Item:	6.2.3

Source:	Ericsson, Samsung, Huawei

Title:	Signalling related to the extended RAR window

Document for:	Discussion, Decision

# 1	Introduction

RAN2 agreed at the last meeting, see also [1]:

8 When msg2 on shared spectrum and msgB, the gNB signals the last 2 bits of SFN

This was implemented in RRC by adding the yellow:

ra-ResponseWindow Msg2 (RAR) window length in number of slots. The network configures a value lower than or equal to 10 ms when Msg2 is transmitted in licensed spectrum and 40 ms when Msg2 is transmitted with shared spectrum channel access (see TS 38.321 [3], clause 5.1.4). UE ignores the field if included in SCellConfig. If ra-ResponseWindow-r16 is signalled, UE shall ignore the ra-ResponseWindow (without suffix). For operation with shared spectrum channel access, the network always includes the two LSB bits of the SFN corresponding to the PRACH occasion where the preamble is received in the DCI scheduling Msg2 (see TS 38.213 [13]).

We think that this has unwanted side-effects and have a proposal for how to update the field description.

# 2	Discussion

The above means that the gNB shall always include the 2 bits of SFN for NR-U, even if the RAR window is small and there is no ambiguity. This would force the gNB to implement and use the new DCI field even for cases when the RAR window is configured with the legacy window length (10 ms or less). That will result in additional implementation burden on the gNB and may delay the deployment of NR-U. Since there is no motivation to include the two LSB bits of the SFN when the RAR window needs no extension, we propose:

The gNB signals the last 2 bits of SFN only when there is risk of ambiguity (i.e. RAR window > 10 ms).

There are two options:

Option a) Specify that the 2 bits of SFN is included when ra-ResponseWindow is larger than 10 ms.

Option b) Specify that the 2 bits of SFN is included when ra-ResponseWindow-r16 is signaled (implying that the RAR window is > 10 ms)

DCI content is captured in TS 38.212, clause 7.3.1 and is under RAN1 responsibility. Therefore, clarifications on when to transmit the 2 LSB bits of the SFN in the DCI should be specified by RAN1.

When to include the 2 LSB bits is removed from the field description in ra-ResponseWindow and clarified in TS 38.212 instead.

ra-ResponseWindow Msg2 (RAR) window length in number of slots. The network configures a value lower than or equal to 10 ms when Msg2 is transmitted in licensed spectrum and 40 ms when Msg2 is transmitted with shared spectrum channel access (see TS 38.321 [3], clause 5.1.4). UE ignores the field if included in SCellConfig. If ra-ResponseWindow-r16 is signalled, UE shall ignore the ra-ResponseWindow (without suffix). For operation with shared spectrum channel access, the network always includes the two LSB bits of the SFN corresponding to the PRACH occasion where the preamble is received in the DCI scheduling Msg2 (see TS 38.213 [13]).

Inform RAN1 that the 2 bits of SFN only need to be signaled in DCI if ra-ResponseWindow is larger than 10 ms, i.e. SFN information is not needed if ra-ResponseWindow is smaller or equal to 10 ms.

Potential changes to TS 38.212 are shown in the annex for both options.

# 3	Conclusion

In the previous sections we made the following observations:

Proposal 1	The gNB signals the last 2 bits of SFN only when there is risk of ambiguity (i.e. RAR window > 10 ms).

Proposal 2	When to include the 2 LSB bits is removed from the field description in ra-ResponseWindow and clarified in TS 38.212 instead.

Proposal 3	Inform RAN1 that the 2 bits of SFN only need to be signaled in DCI if ra-ResponseWindow is larger than 10 ms, i.e. SFN information is not needed if ra-ResponseWindow is smaller or equal to 10 ms.

# 4	References

R2-2002844, “Report for Post109e#40][NR-U] UE capabilities”, Qualcomm Inc., Vivo, RAN2#109bis-e, eMeeting, 20th – 30th  April, 2020

# 4	Annex

Option a: Specify that the 2 bits of SFN is included when ra-ResponseWindow is larger than 10 ms.

<< Start of text proposal for TS 38.212 >>

-	LSBs of SFN – 2 bits for the DCI format 1_0 with CRC scrambled by msgB-RNTI or 2 bits as defined in Clause 8 of [5, TS 38.213] if the random access response window (configured by ra-ResponseWindow or ra-ResponseWindow-r16, respectively)  > 10 ms for operation in a cell with shared spectrum channel access; 0 bit otherwise

-	Reserved bits – 14 bits for the DCI format 1_0 with CRC scrambled by msgB-RNTI or if the random access response window (configured by ra-ResponseWindow or ra-ResponseWindow-r16, respectively)  > 10 ms for operation in a cell with shared spectrum channel access; otherwise 16 bits

<< End of text proposal for TS 38.212  >>

Option b: Specify that the 2 bits of SFN is included when ra-ResponseWindow-r16 is signaled (implying that the RAR window is > 10 ms)

<< Start of text proposal for TS 38.212 >>

-	LSBs of SFN – 2 bits for the DCI format 1_0 with CRC scrambled by msgB-RNTI or 2 bits as defined in Clause 8 of [5, TS 38.213] if ra-ResponseWindow-r16 is present for operation in a cell with shared spectrum channel access; 0 bit otherwise

-	Reserved bits – 14 bits for the DCI format 1_0 with CRC scrambled by msgB-RNTI or if ra-ResponseWindow-r16 is present for operation in a cell with shared spectrum channel access; otherwise 16 bits

<< End of text proposal for TS 38.212  >>