# 3GPP TSG RAN WG2 Meeting #110-e      	                                               R2-200xxxx
E-Conference, 1st – 12th June 2020

Agenda item:	5.4.3.1

Source:	Nokia, Nokia Shanghai Bell

Title:	Summary of [AT110e][014][NR15] UE Cap IPA and email disc last meeting (Nokia)

Document for:	Discussion and Decision

# 1.	Introduction

This is a summary of below offline discussion:

### 5.4.3	UE capabilities and Capability Coordination

#### 5.4.3.0	In-principle Agreed CRs

R2-2005112	Ambiguity in fr1-fr2-Add-UE-NR-Capabilities parameter	Ericsson, NTT Docomo	CR	Rel-15	38.331	15.9.0	1648	-	F	NR_newRAT-Core

R2-2005113	Ambiguity in fr1-fr2-Add-UE-NR-Capabilities parameter	Ericsson, NTT Docomo	CR	Rel-16	38.331	16.0.0	1649	-	A	NR_newRAT-Core

Chair: Why new CR and not a revision?

R2-2005407	SRS Capability report for SRS only Scell	Huawei, HiSilicon	CR	Rel-15	38.331	15.9.0	1559	2	F	NR_newRAT-Core	R2-2004197

R2-2005408	SRS Capability report for SRS only Scell	Huawei, HiSilicon	CR	Rel-16	38.331	16.0.0	1560	2	A	NR_newRAT-Core	R2-2004198

R2-2005409	SRS Capability report for SRS only Scell	Huawei, HiSilicon	CR	Rel-15	38.306	15.9.0	0294	1	F	NR_newRAT-Core	R2-2004199

R2-2005410	SRS Capability report for SRS only Scell	Huawei, HiSilicon	CR	Rel-16	38.306	16.0.0	0295	1	A	NR_newRAT-Core	R2-2004200

R2-2005395	Correction to RequestedCapabilityCommon	Huawei, HiSilicon	CR	Rel-15	38.331	15.9.0	1561	1	F	NR_newRAT-Core	R2-2003463

R2-2005396	Correction to RequestedCapabilityCommon	Huawei, HiSilicon	CR	Rel-16	38.331	16.0.0	1562	1	A	NR_newRAT-Core	R2-2003464

R2-2004842	Missing "Optional features without UE radio access capability parameters"	Ericsson	CR	Rel-15	38.306	15.9.0	0317	-	F	NR_newRAT-Core

R2-2004843	Missing "Optional features without UE radio access capability parameters"	Ericsson	CR	Rel-16	38.306	16.0.0	0318	-	A	NR_newRAT-Core

Chair: Why new CR and not a revision?

All above Treated by email [014]

#### 5.4.3.1	Other

Including Late Drop.

Including outcome of email discussion [Post109bis-e][064][NR15] XDD FRX differentiation (Qualcomm)

Including outcome of email discussion [Post109bis-e][921][NR15] CRs for FR2 CA Fallback (Apple)

Including outcome of email discussion [Post109bis-e][922][NR15] Default values for UE capability (Nokia)

Including outcome of email discussion [Post109bis-e][923][NR15] clarification on codebook parameters for 2-32 (Huawei)

Including outcome of email discussion [Post109bis-e][924][NR15] unnecessary FRx differentiation (ZTE)

[AT110e][014][NR15] UE Cap IPA and email disc last meeting (Nokia)

Scope: Treat all IPA CRs under 5.4.3.0, and from 5.4.3.1: R2-2006021, R2-2006022, R2-2005411, R2-2005412, R2-2005413, R2-2004478, R2-2004479

Part 1: Agree In-principle agreed CRs, for others: Decision whether to make corrections or not, identify agreeable corrections. Deadline: June 4, 0700 UTC.

Part 2: For others, for agreeable parts, continuation to agree CRs. Deadline: June 10, 0700 UTC

# 2. Discussions

## 2.0 Discussion on IPA CRs in 5.4.3.0

| Company | Any comments on IPA CRs? Feedback here with CR number and comment |
| --- | --- |
| Lenovo | Yes, we have some comments to the updated CRs 0317/0318 (R2-2004842/43). 5.1 PWS features In the description of KPAS, EU-Alert the reference to TS 36.304 needs to be corrected to TS 38.304. 5.3 RRC connection The feature name “mo-VoiceCall establishment cause for mobile originating MMTEL video” and its description is not correct. Reason: in NR separate establishment cause values mo-VoiceCall and mo-VideoCall are defined, and for MO MMTEL video the value mo-VideoCall applies. Irrespective of that we think that both establishment cause related features are NAS features and do not need to be defined in TS 38.306. Reason: call establishment is a NAS feature and UE AS sets the establishmentCause acc. to the information received from NAS. |

## 2.1 Discussion on default values

## Including outcome of email discussion [Post109bis-e][922][NR15] Default values for UE capability (Nokia)

## Please provide your comments to the drafts in R2-2006021 (Rel-15) and R2-2006022 (Rel-16 shadow)

- NOTE the drafts are available in the email discussion folder as the revision numbers were acquired after the meeting started

| Company | Comments [R2-2006021 (Rel-15) and R2-2006022 (Rel-16 shadow] |
| --- | --- |
| Nokia | [Proponent] |
| Qualcomm Incorporated | Change to csi-RS-IM-ReceptionForFeedback seems to be motivated to implement the UE minimum requirement from RAN1’s FG2-32. [Nokia] Correct, the discussion is just for alignment. We think it should be enough to set the default for components 2 and 7 to: 8 per CC for each band in FR1 and 2 per CC for each band in FR2, if signalled per band; 8, if signalled per UE, if a UE supports at least one band in FR1; 2, if signalled per UE, if a UE supports bands only in FR2. In FG-32, 8TX for FR1 is only mandated for wideband CSI report, but these UE capability parameters cannot differentiate wideband CSI and subband CSI. [Nokia] Maybe we don’t need to reflect the distinction between WB/SB in 2-33 because this distinction refers to a specific codebook, so it is already correctly captured in the codebookParameters components for Type I. 2-33 indicates only the max number of ports configured/active in a CC or across all CCs, so. It should also be noted that the UE will have to signal additional limitation in csi-RS-IM-ReceptionForFeedbackPerBandComb at BC level in order to make sure the number of active resource is only 1. Since there is no concept of signalling multiple candidates for the following parameters. The UE may end up in only reporting capabilities that are required by FG2-32 and no more than that. [Nokia] I think we are aligned on this. Essentially UE reports here needs to be also in sync with 2-32. - csi-RS-IM-ReceptionForFeedback - csi-RS-IM-ReceptionForFeedbackPerBandComb [Nokia] Overall, from MTK comment as well it seems that we don’t need to capture anything but rely on 2-32 captured by Huawei’s changes. We’re fine with that approach. |

## 2.2 Codebook parameters

Including outcome of email discussion [Post109bis-e][923][NR15] clarification on codebook parameters for 2-32 (Huawei) in R2-2005411.

- Please provide your comments in the table below for R2-2005412 and R2-2005413

| Company | Comments [R2-2005412 (Rel-15) and R2-2005413 (Rel-16 shadow)] |
| --- | --- |
| Qualcomm Incorporated | No comment |

## 2.3 Unnecessary FRx differentiation

Including outcome of email discussion [Post109bis-e][924][NR15] unnecessary FRx differentiation (ZTE) in R2-2004478

- Please provide your comments in the table below for R2-2004479 and R2-2004480

| Company | Comments [R2-2004479 (Rel-15) and R2-2004480 (Rel-16 shadow)] |
| --- | --- |
| Lenovo | To be cross-checked with MCC whether the CR#1605/1606 are correct. It seems that CR# for TS 38.331 were allocated instead for TS 38.306. |
| Qualcomm Incorporated | No comment |

# 3. Conclusion

Summary to be provided at end of the discussion.

# References

#### 5.4.3.0	In-principle Agreed CRs

R2-2005112	Ambiguity in fr1-fr2-Add-UE-NR-Capabilities parameter	Ericsson, NTT Docomo	CR	Rel-15	38.331	15.9.0	1648	-	F	NR_newRAT-Core

R2-2005113	Ambiguity in fr1-fr2-Add-UE-NR-Capabilities parameter	Ericsson, NTT Docomo	CR	Rel-16	38.331	16.0.0	1649	-	A	NR_newRAT-Core

Chair: Why new CR and not a revision?

R2-2005407	SRS Capability report for SRS only Scell	Huawei, HiSilicon	CR	Rel-15	38.331	15.9.0	1559	2	F	NR_newRAT-Core	R2-2004197

R2-2005408	SRS Capability report for SRS only Scell	Huawei, HiSilicon	CR	Rel-16	38.331	16.0.0	1560	2	A	NR_newRAT-Core	R2-2004198

R2-2005409	SRS Capability report for SRS only Scell	Huawei, HiSilicon	CR	Rel-15	38.306	15.9.0	0294	1	F	NR_newRAT-Core	R2-2004199

R2-2005410	SRS Capability report for SRS only Scell	Huawei, HiSilicon	CR	Rel-16	38.306	16.0.0	0295	1	A	NR_newRAT-Core	R2-2004200

R2-2005395	Correction to RequestedCapabilityCommon	Huawei, HiSilicon	CR	Rel-15	38.331	15.9.0	1561	1	F	NR_newRAT-Core	R2-2003463

R2-2005396	Correction to RequestedCapabilityCommon	Huawei, HiSilicon	CR	Rel-16	38.331	16.0.0	1562	1	A	NR_newRAT-Core	R2-2003464

R2-2004842	Missing "Optional features without UE radio access capability parameters"	Ericsson	CR	Rel-15	38.306	15.9.0	0317	-	F	NR_newRAT-Core

R2-2004843	Missing "Optional features without UE radio access capability parameters"	Ericsson	CR	Rel-16	38.306	16.0.0	0318	-	A	NR_newRAT-Core

Chair: Why new CR and not a revision?

All above Treated by email [014]

Default values

Including outcome of email discussion [Post109bis-e][922][NR15] Default values for UE capability (Nokia)

R2-2004454	Default values for UE capability	Nokia, Nokia Shanghai Bell, NTT Docomo	CR	Rel-15	38.306	15.9.0	0176	5	F	NR_newRAT-Core	R2-2002990	Revised

R2-2005709	Default values for UE capability	Nokia, Nokia Shanghai Bell, NTT Docomo	CR	Rel-15	38.306	15.9.0	0176	6	F	NR_newRAT-Core	R2-2004454	Late

=> Revised in R2-2006021

R2-2006021	Default values for UE capability	Nokia, Nokia Shanghai Bell, NTT Docomo	CR	Rel-15	38.306	15.9.0	0176	7	F	NR_newRAT-Core	R2-2004454	Late

R2-2004455	Default values for UE capability	Nokia, Nokia Shanghai Bell, NTT Docomo	CR	Rel-16	38.306	16.0.0	0304	-	A	NR_newRAT-Core	Revised

R2-2005710	Default values for UE capability	Nokia, Nokia Shanghai Bell, NTT Docomo	CR	Rel-16	38.306	16.0.0	0304	1	A	NR_newRAT-Core	R2-2004455	Late

=> Revised in R2-200-6022

R2-2006022	Default values for UE capability	Nokia, Nokia Shanghai Bell, NTT Docomo	CR	Rel-16	38.306	16.0.0	0304	2	A	NR_newRAT-Core	R2-2004455	Late

Codebook parameters

Including outcome of email discussion [Post109bis-e][923][NR15] clarification on codebook parameters for 2-32 (Huawei)

R2-2005411	Summary of [Post109bis-e][923][NR15] clarification on codebook parameters for 2-32	Huawei, HiSilicon	report	NR_newRAT-Core

R2-2005412	on the capability of Basic CSI feedback (2-32)	Huawei, HiSilicon	CR	Rel-15	38.306	15.9.0	0332	-	F	NR_newRAT-Core

R2-2005413	on the capability of Basic CSI feedback (2-32)	Huawei, HiSilicon	CR	Rel-16	38.306	16.0.0	0333	-	F	NR_newRAT-Core

Unnecessary FRx differentiation

Including outcome of email discussion [Post109bis-e][924][NR15] unnecessary FRx differentiation (ZTE)

R2-2004478	Report of [Post109bis-e][924][NR15] Unnecessary FRx differentiation	ZTE Corporation	discussion	Rel-15	NR_newRAT-Core

R2-2004479	CR on unnecessary xDD FRx differentiation	ZTE Corporation, Sanechips	CR	Rel-15	38.331	15.9.0	1605	-	F	NR_newRAT-Core

R2-2004480	CR on unnecessary xDD FRx differentiation	ZTE Corporation, Sanechips	CR	Rel-16	38.331	16.0.0	1606	-	F	NR_newRAT-Core