Implemented in the CR

Same as the current MAC spec (no need to change)

No MAC impact (other spec may change)

Under discussion (postponed)

Other CR will capture

6.7.2.2. Corrections

RAN2 confirm that “up-to 32 CG configurations can be configured per Cell Group across all BWPs” is captured by the constant maxNrofConfiguredGrantConfigMAC-r16.

“Support up to 32 SPS configurations per MAC entity” is not captured in 38.331.

In conditional presence MoreThanTwoRLC, change to “Upon RRC reconfiguration when a PDCP entity is associated with more than two logical channels, this field is optionally present”.

RAN2 confirm moving sps-PUCCH-AN-List from SPS-ConfigList to PUCCH-Config.

Set the status of RIL issue H578 to “ConcReject”

Change the wording “interest in reference time information” to “preference in being provisioned with reference time information”.

6.7.3 MAC Open Issue and Corrections

Remove the current condition “for each uplink grant which is not already a de-prioritized uplink grant”

RAN2 will not specify further on priority of SR triggered by MAC CE in Rel-16. The intention of current MAC text is that such SR has no priority and is handled as lowest priority.

A NOTE for RAN2#109-e agreement on next CG selection for autonomous retransmission to be added. Current proposal is not agreeable, possibly a simplified version can be considered, TBD offline (if no agreement in the end we just skip the Note for now).

(When MAC determines to generate a PDU) MAC entity shall not generate a PDU that cannot be transmitted due to collision with transmission (at least due to equal L1 priority).

Potential agreements (under final review)

Proposal 1. UE continues to use the occasion of the suspended configured grant type 1 when the related UL BWP is activated.

Proposal 2. A de-prioritized SR shall be excluded in prioritization. TP proposed by Fujitsu/Huawei/vivo/OPPO/Nokia can be a baseline.

Proposal 3. For closest N determination, TP in R2-2003586 is adopted.

Proposal 4. RAN2 confirms “CG configurations with the same HARQ process on different BWPs are different and separate CG configurations.”

Proposal 5. A CG cancelled by Cancellation Indicator (CI) is considered as a de-prioritized uplink grant and the MAC entity autonomous transmit the MAC PDU in the subsequent CG

Proposal 6. SR overlapping with uplink grant received in RAR, or addressed to temporary C-RNTI, or with MSGA transmission cannot be transmitted.

Proposal 7. Prioritization between non-overlapping uplink grants is NOT supported in Rel-16.

Proposal 8. TP in R2-2005124 on priority of uplink grant with no data is adopted.

Proposal 9. MAC specification does not have a NOTE to determine the closest N.

Proposal 10. RAN2 confirms that HARQ process ID can be shared between different CGs on different BWPs.

Proposal 11. The autonomous transmission is allowed only for the same CG with the same HARQ process in the same BWP.

Proposal 12. For each uplink grant which can be transmitted by lower layers, the MAC entity evaluates whether it is a prioritized uplink grant. TP of R2-2005647 is a baseline.”.

Proposal 13. A NOTE for RAN2#109-e agreement on next CG selection for autonomous retransmission will be added. The following TP is a baseline:

- “It is up to the UE implementation to determine the closest available next configured grant configured with autonomousTx for the transmission of the MAC PDU of the same HARQ process of the previously de-prioritized uplink grant.”

6.7.3.2 Other

N is non negative (rapporteur to include this in MAC CR discussion whether and how to capture)

The extra CG periodicities of multiple of 2/7 symbols are not introduced in Rel-16

6.7.4.1 PDCP Duplication

The presence of pdcp-Duplication indicates the PDCP duplication configuration (i.e. pdcp-Duplication is always used to indicate the PDCP duplication configuration for both DRBs and SRBs). The 38.331 and 38.323 specifications need to be changed accordingly.

The UE just follows the received MAC CE, even if the RLCi field belongs to the other node. No specification change is required.

PDCP duplication with more than two RLC entities is supported only by NR. It needs to be clarified in 37.340 and 38.331.

Clarify DC+CA duplication in 38.300. 3+1 duplication scenario also needs to be considered. CA duplication may need clarification. Wording to be worked on.

In the description of duplicationState in 38.331, remove “initial” and use “at the time of receiving this IE”.

6.7.4.2 Ethernet Header Compression

Parameter maxCID-EHC is introduced in TS 38.331 to indicate the maximum number of EHC contexts the UE can establish in uplink for a DRB

CID length cannot be reconfigured during the lifetime of the DRB. Field description of ehc-CID-Length is updated by adding a sentence “The value for this field cannot be changed after the initial configuration”

We don’t capture an example of operation on the different Ethernet header structures as an informative text.

Leave trigger in compressor for CID overwriting for implementation (right now the only mandatory trigger is when max CID has been reached).

6.7.6 UE capabilities

Introduce a capability for the UE to indicate whether it supports simultaneous configuration of EHC and RoHC for the same DRB.

If the UE indicates support for RoHC and EHC, but does not indicate support for a new capability as proposed in Proposal Ph1-1, EHC and RoHC may be simultaneously configured for different DRBs.