3GPP TSG-RAN WG2 Meeting #110 electronic				Draft R2-2005751

Elbonia, June 1st  – 12th  2020

Agenda item:		6.9.4

Source:			Intel Corporation

Title:		Summary of discussion][214][MOB] UE capability CRs for NR mobility (Intel)

Document for:	 	Discussion and decision

# Introduction

This is the summary of below offline discussion:

- [AT110-e][214][MOB] UE capability CRs for NR mobility (Intel)

Scope:

    - 38.306 and 38.331 CRs for LTE capabilities based on agreements in this meeting

Intended outcome:

    - Agreed CR to 38.331 CR in R2-2005762 for NR UE capability signalling

    - Agreed CR to 38.306 in R2-2005763 for NR capability descriptions

Deadlines for providing comments and for rapporteur inputs:

    - Deadline for companies' feedback:  Wednesday 2020-06-10 12:00 UTC

    - Deadline for rapporteur's version for agreement:  Thursday 2020-06-11 10:00 UTC

Rapporteur would suggest to resolve open issues first and then check CRs, and therefore setup a early deadline for open issues:

Open issues deadline for companies' feedback:  Friday 2020-06-05 12:00 UTC

# Discussion

## RAN1/4 capabilities

RAN2 have agreed:

Agreements (NR)

12a introduce separate capabilities for intraFreq and interFreq as below:

Per Band/per BC (for intraFreq capabilities), I.e. put under BandParameters-v16xy:

intraFreqDiffSCS-DAPS-r16;

intraFreqAsyncDAPS-r16

intraFreqMultiUL-TransmissionDAPS-r16

Per BC (for interFreq capabilities), i.e. put under CA-ParametersNR-v16xy:

interFreqDiffSCS-DAPS-r16

interFreqAsyncDAPS-r16

interFreqMultiUL-TransmissionDAPS-r16.

12b	All UEs supporting DAPS support these capabilities (can discuss signalling details and naming):

SyncDAPS-r16

SingleUL-TransmissionDAPS-r16

intraFreqTwoTAGs-DAPS-r16  (with 2 TAGs)

(for interFreq since RAN2 agreed to “Reuse CA capability “supportedNumberTAG” for DAPS handover.)

8a	Remove UplinkPowerSharingDAPS-HO

8b	Add separate capabilities for 21-2, 21-2a, 21-2b as semiStaticPowerSharingDAPS-Mode1, semiStaticPowerSharingDAPS-Mode2 and dynamicPowersharingDAPS.

8c	RAN2 thinks that these apply only for multiple UL supporting UEs,

10	Remove pdcch-BlindDetectionSource and pdcch-BlindDetectionTarget from RAN2 agreed capabilities.

11	Add syncDAPS and simultaneous UL transmission based on RAN4 latest capability table.

13	Introduce separate capabilities for intraFreq and interFreq for power sharing capabilities.

- Wait for RAN1 conclusion on ul-TransCancellationDAPS.

The open issue is whether IOT bits are needed for below mandatory features under DAPS, and any comments on the fields name:

SyncDAPS-r16

SingleUL-TransmissionDAPS-r16

intraFreqTwoTAGs-DAPS-r16  (with 2 TAGs)

Question 1: Do companies see the need to have IOT bits for syncDAPS, singleUL-TransmissionDAPS and intraFreqTwoTAGs-DAPS? Any comments on the fields name?

| Company | IOT bits or not? | Remark |
| --- | --- | --- |
| Nokia | Yes | No strong view on the field names. |

## RAN2 capabilities

We discussed RAN2 capability in the meeting. The main open issue is whether we need capability on the support of 2 trigger events for same execution condition. In addition, some companies commented we do not need capability on CHO in FDD-TDD or FR1-FR2 cases since they can be inferred from handoverFDD-TDD, handoverFR1-FR2.

I assume P1/3/4 should be easily agreed.

Proposal 1: the CHO capable UE must support maximum 8 candidate cells; Proposal 3: Introduce cpc-r16 to indicate the support of CPC; Proposal 4: the CPC capable UE must support maximum 8 candidate cells;

Question 2: Do companies agree the P1/3/4 as above in the email discussion 930?

| Company | Yes/No | Remark |
| --- | --- | --- |
| BT | Yes |  |
| Nokia | Yes | Discussed already in the online session and in [930] thread. |

Proposal 2: For CHO, introduce additional capability on the support of 2 trigger events for same execution condition; Proposal 5: For CPC, introduce additional capability on the support of 2 trigger events for same execution condition;

Question 3: Do companies agree the P2/5 as above in the email discussion 930 on the support of 2 trigger events for the same execution condition?

| Company | Yes/No | Remark |
| --- | --- | --- |
| BT | No | We consider two trigger events as part of CHO and not an optimization. We consider CHO and CPC should follow the same approach CPC. |

Proposal 6: For CHO, introduce separate capabilities cho-FDD-TDD-r16 and cho-FR1-FR2-r16;

Question 4: Do companies agree the P6 as above in the email discussion 930 on the support of FDD/TDD HO and FR1/FR2 HO?

| Company | Yes/No | Remark |
| --- | --- | --- |
| BT | No - cho-FDD-TDD-r16 Yes - cho-FR1-FR2-r16 | No for cho-FDD-TDD-r16. We consider handovers a basic feature so for FR1, CHO between FDD and TDD should be mandatory supported if UEs support CHO. Yes for cho-FR1-FR2-r16. For IOT tests. |
| Nokia | No | We did not manage to provide our view on that in [930]. The answer is ‘No’, as we assume any UE supporting CHO and handoverFDD-TDD/handoverFR1-FR2, will also support CHO in FDD-TDD and FR1-FR2 case. Do you assume a different UE implementations? |

# Summary

To be added:

# Reference

[1] R2-2004663	[109b#930] UE capabilities for NR mobility	Intel Corporation

[2] R2-2005311	Report of email discussion [Post109bis-e][963][NR16] UE capabilities	Intel Corporation, NTT DoCoMo