3GPP TSG-RAN WG2 Meeting #110-e	draft_R2-2005822

Online, 1st - 12th  June 2020

Agenda item:	4.2.1

Source:	Huawei (offline email discussion rapporteur)

Title:	Report of 	[AT110-e][402][eMTC] R15 Clarification for CP EDT (Huawei)

Document for:	Report

# 1	Scope of the offline email discussion

This document contains the summary of the offline email discussion “[AT110-e][402][eMTC] R15 Clarification for CP EDT (Huawei)”, as indicated below:

[AT110-e][402][eMTC] R15 Clarification for CP EDT (Huawei)

Scope: Check if there is support and update based on the comments if the CRs are agreeable

Intended outcome: Report provided in R2-2005822 and, if agreeable, updated CR(s).

Deadline: Friday, June 5th 10:00 UTC

# 2	Offline email discussion

R2-2005010	Clarification for CP EDT	Huawei, HiSilicon	CR	Rel-15	36.304	15.5.0	0793	-	F	NB_IOTenh2-Core, LTE_eMTC4-Core

Companies are requested to provide comments in the table below (one row for each new comment to better keep track of the discussion – please don’t edit the previous comments).

| Company | Do you agree with the intent of the CR? | Detailed comments |
| --- | --- | --- |
| Qualcomm | Yes | Change to section 5.2.4.1 is ok. Change to section 5.2.7 is somewhat confusing because UE is supposed to be in RRC_IDLE at start, during and end of EDT, therefore it is not correct to say RRCEarlyDataComplete message causes the UE to return to idle when UE never left idle state. Furthermore, RRCEarlyDataComplete cannot move UE to RRC_INACTIVE. Second paragraph, first change should change as follows: At reception of RRCConnectionRelease message to move the UE into RRC_IDLE or RRC_INACTIVE, or at reception of RRCEarlyDataComplete, or at reception of RRCConnectionRelease during EDT (TS 36.300 [2]), UE shall attempt to camp on a suitable cell according to redirectedCarrierInfo, if included in the RRCConnectionRelease message or RRCEarlyDataComplete message. Second paragraph, second change is ok. Third paragraph change is not needed because EDT cannot be used in state 'camped on any cell' and this change will confuse the reader that EDT is supported when ‘camped on any cell’. Change to section 5.2.7a are not right for similar reason above. First change should be as follows: On transition from RRC_CONNECTED to RRC_IDLE or on completion of EDT (TS 36.300 [2]), UE shall attempt to camp on a suitable cell according to redirectedCarrierInfo, if included in the RRCConnectionRelease-NB message or RRCEarlyDataComplete-NB message. Second change is fine. |

Conclusion:

Proposal:

# 3	Conclusion

Conclusion:

TBC

Agreed CR:

TBC – agreed Rel-15 and Rel-16 shadow CR.

# 4	List of referenced documents

[1] R2-2005010	Clarification for CP EDT	Huawei, HiSilicon	CR	Rel-15	36.304	15.5.0	0793	-	F	NB_IOTenh2-Core, LTE_eMTC4-Core

[2] R2-2005011	Clarification for CP EDT	Huawei, HiSilicon	CR	Rel-16	36.304	16.0.0	0794	-	A	NB_IOTenh2-Core, LTE_eMTC4-Core