3GPP TSG-RAN WG2 Meeting #110 electronic	R2-200xxxx

Online, June 1 – June 12, 2020

Source:	CATT

Title:	Summary of RRM relaxation behaviors

Agenda Item:	6.11.6

Document for:	Discussion and Decision

# Introduction

This contribution captures RAN2 understanding of the UE’s RRM relaxation behaviours across all usecases and configurations.

# Discussion

Table 1 captures RAN2 understanding of the UE’s RRM relaxation behaviours across all usecases and configurations.

Table 1: Comprehensive UE’s RRM relaxation behaviours

| Channel condition | Frequency | NW configures below criteria and when fulfilled | Measurement approach (or with relaxation) | Use case ID |
| --- | --- | --- | --- | --- |
| For inter-frequency / inter-RAT frequency measurement, this part is deduced from RAN4 LS |  |  |  |  |
| When Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ. | Higher priority frequencies | Low mobility (i.e. when NW only configures low mobility, and it is fulfilled) | If highPriorityMeasRelax is configured: UE can stop high priority inter-freq/inter-RAT measurements with a maximum measurement time interval of 1 hour. | A-1 |
|  |  |  | If highPriorityMeasRelax is not configured: The UE shall perform measurements with the requirement of Thigher_priority_search according to TS 38.133. (i.e. legacy behavior) | A-2 |
|  |  | Not at cell edge (i.e. when NW only configures not-at-cell-edge, and it is fulfilled) | The UE shall perform measurements with the requirement of Thigher_priority_search according to TS 38.133. (i.e. legacy behavior) Note: this criterion is always fulfilled, if it is configured. | B-1 |
|  |  | When NW configures both while combineRelaxedMeasCondition is not configured, not at cell edge is fulfilled but low mobility is not fulfilled | The UE shall perform measurements with the requirement of Thigher_priority_search according to TS 38.133. (i.e. legacy behavior) | B-2 |
|  |  | Low mobility and Not at cell edge (i.e. when NW configure both, and both are fulfilled, regardless whether combineRelaxedMeasCondition is configured ) | If highPriorityMeasRelax is configured: UE can stop high priority inter-freq/inter-RAT measurements with a maximum measurement time interval of 1 hour. | C-1 |
|  |  |  | If highPriorityMeasRelax is not configured: The UE shall perform measurements with the requirement of Thigher_priority_search according to TS 38.133. (i.e. legacy behavior) | C-2 |
|  | Equal/Lower priority frequencies | All scenario | UE may choose not to perform measurements of NR inter-frequencies or inter-RAT frequency cells. (According to current TS 38.304). (i.e. legacy behavior) | D |
| When Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ | Higher priority and equal/Lower priority frequencies | Low mobility including: case 1) when NW only configures low mobility, and it is fulfilled; case 2) When NW configure both while combineRelaxedMeasCondition is not configured, low mobility is fulfilled but not at cell edge is not fulfilled; | UE may choose to perform relaxed measurements with longer intervals. | E |
|  |  | Not at cell edge including: case 1) when NW only configures not at cell edge, and it is fulfilled; case 2) When NW configure both while combineRelaxedMeasCondition is not configured, not at cell edge is fulfilled but low mobility is not fulfilled; | UE may choose to perform relaxed measurements with longer intervals. | F |
|  |  | Low mobility and Not at cell edge (i.e. when NW configure both, and both are fulfilled, regardless whether combineRelaxedMeasCondition is configured ) | If highPriorityMeasRelax is configured: UE can stop both equal/low priority and high priority inter-freq/inter-RAT measurements with a maximum measurement time interval of 1 hour. | G-1 |
|  |  |  | If highPriorityMeasRelax is not configured: For higher priority frequencies: UE can stop inter-freq/inter-RAT measurements with a maximum measurement time interval of Thigher_priority_search. For equal/lower priority frequencies: UE can stop inter-freq/inter-RAT measurements with a maximum measurement time interval of 1 hour. | G-2 |
| For intra-frequency measurement, this part is deduced from RAN2/RAN4 conclusion |  |  |  |  |
| Srxlev > SIntraSearchP and Squal > SIntraSearchQ | N/A | All scenarios | UE may choose not to perform intra-frequency measurements. (According to current TS 38.304) (i.e. legacy behavior) | H |
| Srxlev ≤ SIntraSearchP or Squal ≤ SIntraSearchQ | N/A | Low mobility | UE may choose to perform relaxed measurements with longer intervals. | I |
|  | N/A | Not at cell edge | UE may choose to perform relaxed measurements with longer intervals. | J |
|  | N/A | Low mobility and Not at cell edge | UE can stop intra-frequency measurement with a maximum measurement time interval of 1 hour. | K |

# Conclusion

Proposal: Table 1 in this contribution captures RAN2 understanding of the UE’s RRM relaxation behaviours across all usecases and configurations.