3GPP TSG-RAN WG2 Meeting #110-e	R2-2004504

Electronic meeting, 1st - 12th Jun, 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.306 | CR |  | rev |  | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Running CR to 38.306 for NR_SON_MDT |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo, CMCC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SON_MDT-Core |  |  |  |  |  | Date: |  |  | 2020-05-22 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | To capture RAN2 agreements on UE capability for NR_SON_MDT in NR into TS 38.306. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduction of UE capabilities for MDT and SON. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The network does not know which SON/MDT functions capabilities UE supports. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.3 Abbreviations 4.2.X SON parameters 4.2.Y UE-based network performance measurement parameters 5 Optional features without UE radio access capability parameters |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This Running CR is based on the version 16.0.0 of 38.306. |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

START OF CHANGES

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

BC	Band Combination

BT	Bluetooth

DL	Downlink

FS	Feature Set

FSPC	Feature Set Per Component-carrier

MAC	Medium Access Control

MCG	Master Cell Group

MN	Master Node

MR-DC	Multi-RAT Dual Connectivity

PDCP	Packet Data Convergence Protocol

RLC	Radio Link Control

RTT	Round Trip Time

SCG	Secondary Cell Group

SDAP	Service Data Adaptation Protocol

SN	Secondary Node

UL	Uplink

WLAN	Wireless Local Area Network

END OF CHANGES

START OF CHANGES

## 4.2	UE Capability Parameters

<Skip unchanged>

### 4.2.x	SON parameters

| Definitions for parameters | Per | M | FDD-TDD DIFF | FR1-FR2 DIFF |
| --- | --- | --- | --- | --- |
| rach-Report Indicates whether the UE supports delivery of rachReport upon request from the network. | UE | No | No | No |

### 4.2.y	UE-based performance measurement parameters

| Definitions for parameters | Per | M | FDD-TDD DIFF | FR1-FR2 DIFF |
| --- | --- | --- | --- | --- |
| barometerMeasReport Indicates whether UE supports uncompensated barometeric pressure measurement reporting upon request from the network for MDT. | UE | No | No | No |
| immMeasBT Indicates whether the UE supports Bluetooth measurements in RRC_CONNECTED state. | UE | No | No | No |
| immMeasWLAN Indicates whether the UE supports WLAN measurements in RRC_CONNECTED state. | UE | No | No | No |
| loggedMeasBT Indicates whether the UE supports Bluetooth measurements in RRC_IDLE and RRC_INACTIVE state. | UE | No | No | No |
| loggedMeasurements Indicates whether the UE supports logged measurements in RRC_IDLE and RRC_INACTIVE upon request from the network. A UE that supports logged measurements shall support both periodical logging and event-triggered logging. The memory size of MDT logged measurements is 64KB. | UE | No | No | No |
| loggedMeasWLAN Indicates whether the UE supports WLAN measurements in RRC_IDLE and RRC_INACTIVE state. | UE | No | No | No |
| orientationMeasReport Indicates whether the UE supports orientation information reporting upon request from the network for MDT. | UE | No | No | No |
| speedMeasReport Indicates whether the UE supports speed information reporting upon request from the network for MDT. | UE | No | No | No |
| standaloneGNSS-Location Indicates whether the UE is equipped with a standalone GNSS receiver that may be used to provide detailed location information in RRC_CONNECTED and logged measurements in RRC_IDLE and RRC_INACTIVE. | UE | No | No | No |
| ulPDCP-Delay Indicates whether the UE supports UL PDCP Packet Delay measurement and reporting in RRC_CONNECTED state. | UE | No | No | No |

END OF CHANGES

START OF CHANGES

# 5	Optional features without UE radio access capability parameters

Definitions for feature

SU-MIMO Interference Mitigation advanced receiver - R-ML (reduced complexity ML) receivers with enhanced inter-stream interference suppression for SU-MIMO transmissions with rank 2 with 2 RX antennas - R-ML (reduced complexity ML) receivers with enhanced inter-stream interference suppression for SU-MIMO transmissions with rank 2, 3, and 4 with 4 RX antennas UE supporting the feature is required to meet the Enhanced Receiver Type requirements in TS 38.101-4 [18].

Mobility history information storage It is optional for UE to support the storage of mobility history information and the reporting in UEInformationResponse message as specified in TS 38.331 [9]. UE is not required to report this capability.

crossRATRLF-Report Indicates whether the UE supports delivery of EUTRA RLF report to an NR node upon request from the network. UE is not required to report this capability.

END OF CHANGES

Annex (not part of the specification): RAN2 Agreements

## RAN2#108 meeting:

Agreements:

Logged MDT:

1	Introduce loggedMeasurements Capability to indicate whether the UE supports logged measurements in RRC_IDLE and RRC_INACTIVE. A UE that supports logged measurements shall support both periodical logging and event-triggered logging.

Location related capability:

2	locationReport is mandatory supported without UE capability, i.e. if location information is available, UE shall include location information while performing MDT.

3	Reuse standaloneGNSS-Location capability to indicate whether the UE is equipped with a standalone GNSS receiver, same as LTE.

4	Reuse loggedMeasBT, loggedMeasWLAN, immMeasBT, immMeasWLAN capability to indicate whether UE support Bluetooth and WLAN measurements, same as LTE.

5	Introduce barometerMeasReport, speedMeasReport, orientationMeasReport capabilities to indicate whether UE supports sensor data reporting for MDT.

Packet Delay measurement:

6	Reuse ulPDCP-Delay capability to indicate whether the UE supports UL PDCP Packet delay measurement, same as LTE.

SON related capability:

7	CEF reporting and RLF reporting are mandatory supported without UE capability, same as LTE.

8 	Reuse rach-Report capability to indicate whether the UE supports delivery of rachReport upon request from the network, same as LTE.

9	Reuse mobility history information storage capability and no need to report. It is optional for UE to support the storage of mobility history information and the reporting in UEInformationResponse message, same as LTE.

## RAN2#109-e meeting:

Agreements:

1	Introduce a capability in 38.306 for cross-RAT RLF report delivery.

## RAN2#109b-e meeting:

Agreements:

12	If the RAN2 needs to introduce inter-RAT MRO related LTE RLF report in RRC specification based on the RAN3 LS, RAN2 agrees that the support of inter-RAT MRO report associated LTE RLF reporting as an optional feature without UE capability bit.