3GPP TSG-RAN2# 111-E	R2-2007135

# E-meeting, August 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.321 | CR | 0803 | rev | - | Current version: | 15.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on HARQ process ID determination for SPS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | OPPO, Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2020-08-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | For UL CG and DL SPS, similar formula on HARQ process ID determination is specified in Section 5.4.1 and Section 5.3.1 separately. In Section 5.4.1 for UL CG, Note1 is added to describe the used symbol index to determine HARQ process id for a CG bundle, and Note2 is added to explain the range of HARQ process ID for the specific CG. However, similar notes are missing in Section 5.3.1 for DL SPS. Without such notes, - It is unclear the range of HARQ process ID for the specific DL SPS configuration. - It is unclear which slot to be as CURRENT_slot to determine HARQ process if pdsch-AggregationFactor is configured for the DL SPS configuration. Thus, for DL SPS, two new notes similar to UL CG are recommended. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add a Note similar to UL CG to define the range of HARQ process ID for a specific SPS. - Add a Note similar to UL CG to indicate slot index of the first transmission occasion of a SPS bundle as CURRENT_slot. Impact Analysis Impacted 5G architecture options: Standalone, NR-DC, NE-DC, EN-DC, NGEN-DC Impacted functionality: SPS Inter-operability: No inter-operability issue is foreseen. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - Misunderstanding may exist when determining which HARQ process ID is used for the SPS. - Misunderstanding may exist when determining which slot as CURRENT_slot. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

START OF THE CHANGE

5.3.1	DL Assignment reception

Downlink assignments received on the PDCCH both indicate that there is a transmission on a DL-SCH for a particular MAC entity and provide the relevant HARQ information.

When the MAC entity has a C-RNTI, Temporary C-RNTI, or CS-RNTI, the MAC entity shall for each PDCCH occasion during which it monitors PDCCH and for each Serving Cell:

1>	if a downlink assignment for this PDCCH occasion and this Serving Cell has been received on the PDCCH for the MAC entity's C-RNTI, or Temporary C-RNTI:

2>	if this is the first downlink assignment for this Temporary C-RNTI:

3>	consider the NDI to have been toggled.

2>	if the downlink assignment is for the MAC entity's C-RNTI, and if the previous downlink assignment indicated to the HARQ entity of the same HARQ process was either a downlink assignment received for the MAC entity's CS-RNTI or a configured downlink assignment:

3>	consider the NDI to have been toggled regardless of the value of the NDI.

2>	indicate the presence of a downlink assignment and deliver the associated HARQ information to the HARQ entity.

1>	else if a downlink assignment for this PDCCH occasion has been received for this Serving Cell on the PDCCH for the MAC entity's CS-RNTI:

2>	if the NDI in the received HARQ information is 1:

3>	consider the NDI for the corresponding HARQ process not to have been toggled;

3>	indicate the presence of a downlink assignment for this Serving Cell and deliver the associated HARQ information to the HARQ entity.

2>	if the NDI in the received HARQ information is 0:

3>	if PDCCH contents indicate SPS deactivation:

4>	clear the configured downlink assignment for this Serving Cell (if any);

4>	if the timeAlignmentTimer, associated with the TAG containing the Serving Cell on which the HARQ feedback is to be transmitted, is running:

5>	indicate a positive acknowledgement for the SPS deactivation to the physical layer.

3>	else if PDCCH content indicates SPS activation:

4>	store the downlink assignment for this Serving Cell and the associated HARQ information as configured downlink assignment;

4>	initialise or re-initialise the configured downlink assignment for this Serving Cell to start in the associated PDSCH duration and to recur according to rules in clause 5.8.1;

For each Serving Cell and each configured downlink assignment, if configured and activated, the MAC entity shall:

1>	if the PDSCH duration of the configured downlink assignment does not overlap with the PDSCH duration of a downlink assignment received on the PDCCH for this Serving Cell:

2>	instruct the physical layer to receive, in this PDSCH duration, transport block on the DL-SCH according to the configured downlink assignment and to deliver it to the HARQ entity;

2>	set the HARQ Process ID to the HARQ Process ID associated with this PDSCH duration;

2>	consider the NDI bit for the corresponding HARQ process to have been toggled;

2>	indicate the presence of a configured downlink assignment and deliver the stored HARQ information to the HARQ entity.

For configured downlink assignments, the HARQ Process ID associated with the slot where the DL transmission starts is derived from the following equation:

HARQ Process ID = [floor (CURRENT_slot × 10 / (numberOfSlotsPerFrame × periodicity))] modulo nrofHARQ-Processes

where CURRENT_slot = [(SFN × numberOfSlotsPerFrame) + slot number in the frame] and numberOfSlotsPerFrame refers to the number of consecutive slots per frame as specified in TS 38.211 [8].

NOTE 1:	CURRENT_slot refers to the slot index of the first transmission occasion of a bundle of configured downlink assignment that takes place.

NOTE 2:	A HARQ process is configured for a configured downlink assignment if the configured downlink assignment is activated and the associated HARQ process ID is less than nrofHARQ-Processes.

When the MAC entity needs to read BCCH, the MAC entity may, based on the scheduling information from RRC:

1>	if a downlink assignment for this PDCCH occasion has been received on the PDCCH for the SI-RNTI;

2>	indicate a downlink assignment and redundancy version for the dedicated broadcast HARQ process to the HARQ entity.

END OF THE CHANGE