3GPP TSG-RAN WG2 Meeting #111 electronic	R2-2007215

E-Meeting, 17th – 28th August, 2020

Source:	vivo

Title:	Efficient activation/deactivation mechanism for SCG and SCells

Agenda Item:	8.2.2

Document for:	Discussion and Decision

- Introduction

The objective of this work item is to specify enhancements to MR-DC related scenarios [1]. At least the following topics should be considered in the work:

- Support efficient activation/de-activation mechanism for one SCG and SCells - Support for one SCG applies to (NG)EN-DC, and NR-DC [RAN2, RAN3, RAN4] - Support for SCells applies to NR CA, based on RAN1 leading mechanisms [RAN1, RAN2, RAN4] - This objective applies to FR1 and FR2 - Support of conditional PSCell change/addition [RAN2, RAN3] - support scenarios which are not addressed in Rel-16 NR mobility WI

This paper will discuss the efficient activation/deactivation mechanism for SCG and SCells.

- Discussion

  - SCG suspend

If SCG data traffic is totally paused in a period, the network can release the SCG to reduce the UE power consumption, but undesirable long latency is required to add it back when the data traffic of SCG grows, e.g., the latency due to the reception of RRC reconfiguration. Alternatively, the network can keep the SCG instead of release. In this case, the UE is required to monitor the PDCCH of the SCG (at least PSCell) although there is no data at the SCG, increasing the UE power consumption, especially on FR2. It would be better for the network to not release the SCG but also keep the UE power consumption minimized, thus it is agreed in RAN2#107bis meeting to introduce SCG suspended:

R2 assumes the following (can be slightly modified due to progress on Scell dormancy):

The UE supports network-controlled suspension of the SCG in RRC_CONNECTED.

UE behaviour for a suspended SCG is FFS

The UE supports at most one SCG configuration, suspended or not suspended, in Rel16.

In RRC_CONNECTED upon addition of the SCG, the SCG can be either suspended or not suspended by configuration.

This paper will focus on the UE behavior for the suspended SCG and how does the network control the suspension/resumption (or deactivation/activation) of the SCG.

2.1 UE behavior for the suspended SCG

Since SCG suspension and the SCell dormant behavior have the similar intention, i.e., a tradeoff between power saving and activation latency reduction, the existing behavior of SCell (deactivated, activated with dormant BWP) should be considered when discussing that of SCG suspended, to accelerate the progress.

Observation 1: to speed up the progress, the existing behavior of SCell (deactivated, activated with dormant BWP) should be considered when discussing that of SCG suspended, to accelerate the progress.

According to the existing behavior of SCell, the UE behavior for SCG suspended may have the following alternatives, that is, when SCG is suspended,

- Alt1: all serving cells in SCG are deactivated;

- Alt2: all serving cells in SCG are activated with dormant BWP;

- Alt3: PSCell is activated with dormant BWP, and SCells in SCG are deactivated;

- Alt4: PSCell is activated with dormant BWP, while SCells in SCG are deactivated or activated with dormant BWP based on network configuration.

Alt1 is the most power-saving way since the UE can turn off the transceiver for the SCG, and also is the simplest way which does not bring too much network signaling overhead and specification changes. However, Alt1 is not preferred because the activation latency from deactivated to activated may be unbearable when SCG busty traffic occurs. In our understanding, quick start SCG scheduling is also required upon the SCG resumption. For Alt2, it maybe infeasible since not all SCells can be configured with dormant BWP and hence cannot be set to dormant. Although PSCell can be kept in dormant compared to Alt1, Alt3 still may be not enough for quick starting SCG transmission when SCG traffic grows sharply. Alt4 can support starting SCG scheduling as quickly as possible and further provide more flexibility for network configuration, e.g., the network can set the SCells configured with dormant BWP to be dormant while the others to be deactivated upon the SCG is suspended.

Observation 2: quick start SCG scheduling is also required upon the SCG resumption.

Observation 3: Alt4 (PSCell is activated with dormant BWP, while SCells in SCG are deactivated or activated with dormant BWP based on network configuration) can support starting SCG scheduling as quickly as possible and further provide more flexibility for network configuration.

Thus, we have:

Proposal 1: upon SCG is suspended, PSCell is activated with dormant BWP, while SCells in SCG are deactivated or activated with dormant BWP based on network configuration.

Besides, since SCG may experience blocking during SCG suspension, performing RLM on PSCell enables the detection of S-RLF. However, performing RLM on PSCell will cause more power consumption in case of that the radio link quality of PSCell is still stable. It seems that whether the UE perform RLM can be left to network implementation, i.e., UE will perform RLM when the associated RLM-RS is configured for the dormant BWP of the PSCell.

Proposal 2: whether the UE perform RLM on the PSCell during suspension can be left to network implementation, i.e., UE only performs RLM when the associated RLM-RS is configured for the dormant BWP of the PSCell.

2.2 In/Out of the suspended SCG

Following the same logic of SpCell controlled SCell dormant/non-dormant BWP switching, the suspension of SCG shall be controlled by MN. Since SN knows whether there are ongoing data in the SCG, SN should notify the MN via inter-node signaling and then MN to suspend the SCG via MN signaling.

Proposal 3: the suspension of SCG shall be controlled by MN.

How to leave the SCG suspended state should be further considered. The transition from/into dormant BWP is indicated by DCI on the PSCell. However, UE does not monitor PDCCH on both PSCell and SCG SCells during SCG suspension. Thus, leaving the SCG suspended state shall be controlled by MN.

Proposal 4: leaving the SCG suspended state shall be controlled by MN.

An example of leaving the SCG suspended state for a UE in RRC_CONNECTED with an MN, is given below.

- Step 1a: When UE has data to transmit on the SCG, UE sends RRC message to MN, which is a re-activated notification referred to SCG. Or

- Step 1b: When DL data traffic arrives at SN that towards to the UE, SN sends a re-activity notification to the MN to report user plane activity.

- Step 2: MN indicates UE to resume/activate the SCG.

Proposal 5：When UE has data to transmit on the SCG, UE indicates to MN, which is a re-activated notification referred to SCG. When DL data traffic arrives at SN that towards to the UE, SN sends a re-activity notification to the MN to report user plane activity.

For MN terminated SCG bearer, since DL data traffic of SCG arrives at MN, MN can directly indicate the UE to resume the SCG.

Proposal 6：For MN terminated SCG bearer and split bearer, since that DL data traffic of SCG arrives at MN, MN can directly indicate UE to resume the SCG.

2.2 Efficient activation/setup for SCells

- Long SRS transmission on dormant BWP:

In RAN2#110e, an incoming LS from RAN1 pointed out that they see no issue with supporting at least long periodicity P-SRS (e.g. >100ms) for dormant BWP. However, most of companies concerned that RAN2 may not have enough time to sufficiently evaluate and finish it in Rel-16. Thus, the following agreement was made:

Confirm that dormant SCell don’t support SRS or A-CSI.

Since long P-SRS can improve UL quality and reduce the activation latency with the acceptable power consumption, it is worth to revisit the long SRS for dormant BWP in Rel-17.

Proposal 7: Long SRS transmission on dormant BWP can be discussed in Rel-17.

- Maintain CA/DC configuration during RRC re-establishment

According to TS 38.331 [2], upon initiation of the RRC connection re-establishment procedure, if the UE is not configured with conditionalReconfiguration, the UE shall release the configured related DC/CA configuration. However keeping the related DC/CA SCells configuration upon the initiation of the RRC re-establishment, is beneficial for fast cell activation for MR-DC.

Proposal 8: Keeping the related DC/CA SCells configuration upon the initiation of the RRC re-establishment.

- CSI-RS based early measurement:

Only SSB based early measurement is specified in Rel-16. Considering SSB based early measurement can cover almost all cases of fast CA/DC setup and only the limited TU allocated to Rel-16 DC/CA enhancement, CSI-RS based NR early measurement is not pursued in Rel-16. However, for the SCell without SSB, CSI-RS based NR early measurement results may help the network to fast configure these cells to the UE. Besides, RAN4 is about to finalize the requirement definition of CSI-RS based RRM measurement.

Observation 4: for the SCell without SSB, CSI-RS based NR early measurement results may help the network to fast configure these cells to the UE.

Observation 5: RAN4 is about to finalize the requirement definition of CSI-RS based RRM measurement.

Thus, we have the following:

Proposal 9: CSI-RS based NR early measurement can be considered in Rel-17.

- Conclusion

In the contribution, we discussed the issue of xxx, and have made the following observations and proposals:

Observation 1: to speed up the progress, the existing behavior of SCell (deactivated, activated with dormant BWP) should be considered when discussing that of SCG suspended, to accelerate the progress.

Observation 2: quick start SCG scheduling is also required upon the SCG resumption.

Observation 3: Alt4 (PSCell is activated with dormant BWP, while SCells in SCG are deactivated or activated with dormant BWP based on network configuration) can support starting SCG scheduling as quickly as possible and further provide more flexibility for network configuration.

Observation 4: for the SCell without SSB, CSI-RS based NR early measurement results may help the network to fast configure these cells to the UE.

Observation 5: RAN4 is about to finalize the requirement definition of CSI-RS based RRM measurement.

Proposal 1: upon SCG is suspended, PSCell is activated with dormant BWP, while SCells in SCG are deactivated or activated with dormant BWP based on network configuration.

Proposal 2: whether the UE perform RLM on the PSCell during suspension can be left to network implementation, i.e., UE only performs RLM when the associated RLM-RS is configured for the dormant BWP of the PSCell.

Proposal 3: the suspension of SCG shall be controlled by MN.

Proposal 4: leaving the SCG suspended state shall be controlled by MN.

Proposal 5：When UE has data to transmit on the SCG, UE indicates to MN, which is a re-activated notification referred to SCG. When DL data traffic arrives at SN that towards to the UE, SN sends a re-activity notification to the MN to report user plane activity.

Proposal 6：For MN terminated SCG bearer and split bearer, since that DL data traffic of SCG arrives at MN, MN can directly indicate UE to resume the SCG.

Proposal 7: Long SRS transmission on dormant BWP can be discussed in Rel-17.

Proposal 8: Keeping the related DC/CA SCells configuration upon the initiation of the RRC re-establishment.

Proposal 9: CSI-RS based NR early measurement can be considered in Rel-17.

- References

[1] RP-193164.

[2] TS 38.331, V16.1.0.