3GPP TSG-RAN WG2 Meeting #111e	R2-2007252

Elbonia, 17 August – 28 August 2020

Agenda item:	6.4.4

Source:	Nokia, Nokia Shanghai Bell

Title:	Reducing the signalling of peer UE capability transfer in unicast sidelink

WID/SID:	5G_V2X_NRSL-Core - Release 16

Document for:	Discussion and Decision

# 1	Introduction

In the RAN2#109bis-e [1] meeting the problem of UE capability mismatch in unicast sidelink has been discussed in various TDocs [2] based on the preceding offline email [Post109e][20][V2X] [3] and has been further treated in [Post109bis-e][955][V2X] [4]. In the subsequent RAN2#110 meeting it has been agreed to transfer the peer UE capability in a container via the initiating UE to the network.

This contribution discusses further options to reduce the signalling caused by frequent transferring of the peer UE capability to the network.

# 2	Discussion

During RAN2#109bis-e meeting the offline discussion [Post109e][20][V2X] was summarized in R2-2002638, where on the UE capability mismatch problem in sidelink unicast the following has been agreed:

UE capability agreements:

4:	RRC_CONNECTED UE reports the received SL capability via PC5-RRC to network.

That the RRC_CONNECTED UE reports the received SL capability (carrying RX UE capability received via UECapabilityInformationSidelink) via PC5-RRC to network using a container within SidelinkUEInformationNR message is captured in TS38.331 in the following ASN.1 representation marked in yellow:

-- ASN1START

-- TAG-SIDELINKUEINFORMATIONNR-START

SidelinkUEInformationNR-r16::=         SEQUENCE {

criticalExtensions                  CHOICE {

sidelinkUEInformationNR-r16         SidelinkUEInformationNR-r16-IEs,

criticalExtensionsFuture            SEQUENCE {}

}

}

SidelinkUEInformationNR-r16-IEs ::=     SEQUENCE {

sl-RxInterestedFreqList-r16            SL-InterestedFreqList-r16           OPTIONAL,

sl-TxResourceReqList-r16               SL-TxResourceReqList-r16            OPTIONAL,

lateNonCriticalExtension               OCTET STRING                        OPTIONAL,

nonCriticalExtension                   SEQUENCE {}                         OPTIONAL

}

SL-InterestedFreqList-r16 ::=          SEQUENCE (SIZE (1..maxNrofFreqSL-r16)) OF INTEGER (1..maxNrofFreqSL-r16)

SL-TxResourceReqList-r16 ::=           SEQUENCE (SIZE (1..maxNrofSL-Dest-r16)) OF SL-TxResourceReq-r16

SL-TxResourceReq-r16 ::=                SEQUENCE {

sl-DestinationIdentity-r16             SL-DestinationIdentity-r16,

sl-CastType-r16                        ENUMERATED {broadcast, groupcast, unicast, spare1},

sl-RLC-ModeIndicationList-r16          SEQUENCE (SIZE (1.. maxNrofSLRB-r16)) OF SL-RLC-ModeIndication-r16         OPTIONAL,

sl-QoS-InfoList-r16                    SEQUENCE (SIZE (1..maxNrofSL-QFIsPerDest-r16)) OF SL-QoS-Info-r16          OPTIONAL,

sl-Failure-r16                         ENUMERATED {rlf, configFailure, spare2, spare1}                            OPTIONAL,

sl-TypeTxSyncList-r16                  SEQUENCE (SIZE (1..maxNrofFreqSL-r16)) OF SL-TypeTxSync-r16                OPTIONAL,

sl-TxInterestedFreqList-r16            SEQUENCE (SIZE (1..maxNrofFreqSL-r16)) OF INTEGER (1..maxNrofFreqSL-r16)   OPTIONAL,

sl-CapabilityInformationSidelink-r16   OCTET STRING                                                               OPTIONAL

}

SL-QoS-Info-r16 ::=                    SEQUENCE {

sl-QoS-FlowIdentity-r16               SL-QoS-FlowIdentity-r16,

sl-QoS-Profile-r16                    SL-QoS-Profile-r16                                                          OPTIONAL

}

SL-RLC-ModeIndication-r16 ::=          SEQUENCE {

sl-AM-Mode-r16                     SEQUENCE {

sl-AM-Mode-r16                     ENUMERATED {true},

sl-AM-QoS-InfoList-r16             SEQUENCE (SIZE (1..maxNrofSL-QFIsPerDest-r16)) OF SL-QoS-Info-r16

}                                                                                                                 OPTIONAL,

sl-UM-Mode-r16                     SEQUENCE {

sl-UM-Mode-r16                     ENUMERATED {true},

sl-UM-QoS-InfoList-r16             SEQUENCE (SIZE (1..maxNrofSL-QFIsPerDest-r16)) OF SL-QoS-Info-r16

}                                                                                                                 OPTIONAL

}

-- TAG-SIDELINKUEINFORMATIONNR-STOP

-- ASN1STOP

with the field description

sl-CapabilityInformationSidelink

Includes the UECapabilityInformationSidelink message (which can be also included in ueCapabilityInformationSidelink-r16 in UECapabilityEnquirySidelink from peer UE) received from the peer UE.

The UE capabilities are a static set of UE specific properties fixed with the UE hardware, which typically do not change. However, in sidelink the peer UE capability information might be transferred in a container to the network each time SidelinkUEInformationNR is exchanged, although the peer capabilities have not changed. Especially if a pair of UEs is communicating frequently in unicast sidelink, unnecessary signalling overhead is created due to the transfer of peer UE capability.

Observation 1: While peer UE capabilities (typically) do not change they might be reported frequently to the network in SidelinkUEInformationNR message.

During RAN2#110 the offline discussion [Post109bis][955][V2X] on SL UE capabilities was continued and summarized in R2-2004402, where with respect to the transfer of SL peer UE capability in sidelink unicast the following has been agreed:

UE capability agreement

16:	RAN2 not purse signaling overhead optimization for capability transfer procedure for TX-UE forwarding peer-UE SL capability to network via Uu-RRC.

Obviously, the fact that the initiating UE might transfer the peer UE capability unnecessarily if the UE capability of the peer UE is known to the network already or have been transferred already several times, the agreement of RAN2#110 not to pursue any signalling overhead reduction poses a contradiction.

In the following we want to discuss options to reduce the capability signalling without violating the agreement made in RAN2#110. A possible solution is to eliminate the need to transfer the container carrying the peer UE capabilities in the SidelinkUEInformationNR message totally.

In order to achieve this the network stores the capability container [ueCapabilityInformationSidelink-r16] reported from the initiating UE in the SidelinkUEInformationNR message with the sl-DestinationIdentity-r16 L2 identifier or another UE-unique identifier.

Proposal 1: The network should store the peer UE’s capability reported by the initiating UE to the network.

Once the initiating UE has transferred the peer UE capability it does not need to repeat this peer UE capability transfer for the same peer UE, since the peer UE capability does not change, and the network already has the peer UE capability information available.

Proposal 2: The initiating UE does not include the container with the peer UE capability once the initiating UE has reported it to the network for the same peer UE .

Figure 1 Reported sidleink peer UE capability is stored in the network

Figure 2 Network retrieves peer UE’s capability (no need for initiating UE to report peer UE capability)

# 3	Conclusion

Observation 1: While peer UE capabilities (typically) do not change they might be reported frequently to the network in SidelinkUEInformationNR message.

Proposal 1: The network should store the peer UE’s capability reported by the initiating UE to the network.

Proposal 2: The initiating UE does not include the container with the peer UE capability once the initiating UE has reported it to the network for the same peer UE .

# 4	References

[1] R2-2003808 Report from R2#109bis-e session on LTE V2X and NR V2X

[2] R2-2002639Summary of capability related Tdoc submitted to R2#109bis-E

[3] R2-2002638Summary of [Post109e#20] V2X Remaining UE capability issues

[4] R2-2004402 Summary of [Post109bis-e][955][V2X]: UE capabilities

[5] R2-2002661 Further issues on the mismatch of UE capabilities in unicast sidelink