3GPP TSG-RAN WG2 #111-e	Tdoc R2-2008093

Electronic meeting, 2020-08-17 - 2020-08-28

Agenda Item:	6.15

Source:	Ericsson

Title:	Discussion on MPE enhancements

Document for:	Discussion, Decision

# 1	Introduction

RAN2 got LSs from RAN4 where they request RAN2 to specify RAN2 aspects of MPE. Some aspect still remain open in RAN4, but RAN2 can conclude at least some aspects already now.

# 2	Discussion

RAN2 got this LS-text in R4-2002916:

1. Overall Description: RAN4 has agreed further details on Rel-16 FR2 MPE enhancement solutions to mitigate RLF due to sudden and unpredictable large back off caused UE’s actions to ensure compliance with exposure regulation. RAN4 would like to ask RAN2 to develop the following Rel-16 FR2 MPE signalling based on MAC-CE to ensure sufficiently short signalling delays: - at least UE’s P-MPR based event-triggered reporting including also reporting of the actual P-MPR level that UE needs for FR2 MPE reasons. - Network configurable P-MPR reporting threshold - A prohibit timer is enabled to be configured by network to trigger the P-MPR reporting - P-MPR reporting range and reporting granularity are still under discussion in RAN4. RAN4 will provide further details after its next meeting. 2. Actions: To RAN WG2 group. ACTION: RAN4 asks RAN2 group to define Rel-16 MPE signalling based on the above-mentioned information.

And further this LS-text in R4-2005670:

1. Overall Description: RAN4 has agreed further details on the Rel-16 FR2 MPE enhancement signalling, which is used to mitigate RLF due to sudden and unpredictable large back off caused UE’s actions to ensure compliance with MPE regulation. In addition to the previous details provided for the Rel-16 FR2 MPE enhancement signalling RAN4 would like to ask RAN2 to take the following additional details into account when developing MAC-CE based signalling for the FR2 MPE enhancements: - Network configured threshold for event-triggered FR2 MPE P-MPR reporting is defined based P-MPR being higher than a configurable threshold. Whether an additionally relative threshold will be defined is still under discussion in RAN4 and RAN4 will inform RAN2 the outcome in the following meeting - P-MPR reporting range and reporting granularity will be defined in the next RAN4 meeting using [2…5] bits. RAN4 will inform RAN2 the exact reporting range and reporting granularity in its next meeting. - P-MPR is reported by the UE after or on the grant and the exact details are up to UE implementation. 2. Actions: To RAN WG2 group. ACTION: RAN4 asks RAN2 group to take the above-mentioned information into account when defining Rel-16 FR2 MPE signalling

## 2.1	What is "MPE"?

To a RAN2-person the MPE-feature can be summarized as follows. There are regulatory requirements w.r.t. how much output power a UE can use. These requirements depend for example on the distance to a human body: if a user is very close to the antenna the UE has to reduce the output power it uses to ensure that the amount of energy exposed to the human body is below a certain limit.

When the UE performs this type of backoff the communication with the network would be impacted since the amount of available output power of the UE is reduced, and the gNB needs to know how much power is available in the UE so it can decide a suitable number of resource blocks to schedule the UE on, etc.

The amount of power available by the UE is however already reported to the gNB in the PHR MAC CE (the PCMAX,f,c field). The only thing which is not known to the gNB from the current PHR MAC CE alone is why the UE now has less power available.

What RAN4 has requested RAN2 to introduce is MAC signalling to allow the UE to indicate how much back off the UE has applied due to MPE-reasons. It has not been clearly indicated by RAN4 what the gNB should do with this information though.

## 2.2	New MAC CE or reuse PHR MAC CE?

RAN4 indicated that the range and granularity of the P-MRP may require 2-5 bits. Exact number is still FFS. In the last RAN4 meeting, RAN4 narrowed it down to either 2 or 3 bits.

If RAN4 agrees on 2 bits the MPE-field can be placed in the current two R-bits, like in the below:

Single-entry PHR MAC CE

Multi-entry PHR MAC CE

Actually, even if RAN4 agrees that three bits are needed for the MPE reporting, it would be possible to fit it in the current PHR MAC CE. Example shown below. The P-bit which today indicates if the UE did backoff, is redundant if the UE is configured to report MPE. The reason is that the MPE-field indicates how much backoff the UE did. If the MPE-indication indeed is non-zero then the network knows that the UE did backoff, while if the MPE indication is zero, the network knows that the UE did not apply backoff.

Another approach is of course that new octets are added to the PHR MAC CE where the MPE-field is placed, but that results in extra overhead compared to the above. Note that adding a new MAC CE would result in even more overhead due to additional MAC sub-headers.

The PHR MAC CE could fit the MPE-report without adding additional overhead if it is 2 or 3 bits.

RAN2 adds the MPE report to the existing PHR MAC CE.

## 2.3	Triggers for PHR MAC CE due to MPE

RAN4 has agreed that the UE shall report MPE when:

a) P-MPR gets higher than an (absolute) configurable threshold

b) P-MPR changes compared configurable (relative) threshold

For a, a new condition as below should be added to trigger the PHR:

A Power Headroom Report (PHR) shall be triggered if any of the following events occur: - (NEW CONDITION) phr-ProhibitTimer expires or has expired, when the MAC entity has UL resources for new transmission, and the following is true for any of the activated Serving Cells of any MAC entity with configured uplink: - there are UL resources allocated for transmission or there is a PUCCH transmission on this cell, and the required power backoff due to power management (as allowed by P-MPRc as specified in TS 38.101-1 [14], TS 38.101-2 [15], and TS 38.101-3 [16]) for this cell has increased beyond phr-Tx-AbsolutePowerFactorChange dB since the last transmission of a PHR in this MAC entity.

We note that the RAN4 LS does not request that the UE shall report MPE when the back off falls below the absolute threshold.

A new PHR trigger condition is added for the absolute MPE trigger.

For b, current PHR triggered condition for relative power backoff change as below can be reused.

A Power Headroom Report (PHR) shall be triggered if any of the following events occur: - (EXISTING CONDITION) phr-ProhibitTimer expires or has expired, when the MAC entity has UL resources for new transmission, and the following is true for any of the activated Serving Cells of any MAC entity with configured uplink: - there are UL resources allocated for transmission or there is a PUCCH transmission on this cell, and the required power backoff due to power management (as allowed by P-MPRc as specified in TS 38.101-1 [14], TS 38.101-2 [15], and TS 38.101-3 [16]) for this cell has changed more than phr-Tx-PowerFactorChange dB since the last transmission of a PHR when the MAC entity had UL resources allocated for transmission or PUCCH transmission on this cell.

The existing PHR trigger condition covers sufficiently the relative MPE trigger.

## 2.3	Prohibit timer

It was discussed during RAN2#110 whether a new prohibit timer is needed or not. RAN4 stated this regarding prohibit timers:

- A prohibit timer is enabled to be configured by network to trigger the P-MPR reporting

For the PHR MAC CE there is already a prohibit timer. The prohibit timer will stop the UE from sending a PHR MAC CE, by, if it is running not allowing the UE to trigger a PHR MAC CE.

If the MPE indication is included in the PHR MAC CE (as proposed above) it would be possible that the same prohibit timer is applied for this. That would, as requested by RAN4, be "[a] prohibit timer […] enabled to be configured by the network to trigger the P-MPR reporting". RAN4 has not said that this must be a new timer.

The existing PHR prohibit timer covers sufficiently MPE reporting.

## 2.4	Configurability

In order to differentiate the PHR MAC CE format with or without MPE information, network should explicitly enable the UE to use the PHR MAC CE with MPE info by RRC configuration.

PHR MAC CE format with MPE reporting should be enabled via RRC signalling.

# 4	Conclusion

In the previous sections we made the following observations:

Observation 1	The PHR MAC CE could fit the MPE-report without adding additional overhead if it is 2 or 3 bits.

Based on the discussion in the previous sections we propose the following:

Proposal 1	RAN2 adds the MPE report to the existing PHR MAC CE.

Proposal 2	A new PHR trigger condition is added for the absolute MPE trigger.

Proposal 3	The existing PHR trigger condition covers sufficiently the relative MPE trigger.

Proposal 4	The existing PHR prohibit timer covers sufficiently MPE reporting.

Proposal 5	PHR MAC CE format with MPE reporting should be enabled via RRC signalling.

The following CRs implement the approach where 2 bits are used for the MPE report:

- R2-2008094 (MAC)

- R2-2008095 (RRC)

- R2-2008096 (306)