3GPP TSG-RAN WG2 Meeting #111-e	draftR2-2008123

Electronic 17th – 28th August 2020

Agenda Item:	10.3

Source: 		Session Chair (Ericsson)

Title: 			draftReport eMTC breakout session

Document for:	Approval

General

Recording of voice or video at meetings is not used in 3GPP. This applies also to this e-Meeting. At this e-Meeting, no specific actions are taken to prevent the recording of web conferences. Companies that have concerns related to recordings, if any, may express those by email in the main meeting organizational thread [AT111-e][000]

Please see the following Tdocs for e-meeting guidance:

R2-2006500	Agenda for RAN2#111-e	Chairman	agenda

R2-2008391	RAN2#111-e Meeting Guidelines 	ETSI MCC	discussion

Time Schedule 
Please refer to the latest schedule in the RAN2 inbox on the public 3GPP servers.

Access Tools

HTTP Upload Tool:

ETSI IT has created a facility in Inbox and Inbox/Drafts folders on the public 3GPP servers to allow delegates to upload their documents using a web browser (however Internet Explorer is not yet supported). Open your browser and navigate to your chosen folder – for example,

https://www.3gpp.org/ftp/tsg_ran/WG2_RL2/TSGR2_111-e/Inbox

Click the green button to log in using your EOL account. A panel will appear in the upper part of the screen and documents may be dragged and dropped onto this landing pad; this causes them to be uploaded to the folder.

Secure FTP:

Those e-delegates who prefer to use FTP-like access to our e-meeting Inbox & Draft folders but are concerned by their usernames and passwords being sent unencrypted over the internet, ETSI IT has fitted the server with FTPS (SSL) so delegates can connect from their favourite FTP client using the address: ftps.3gpp.org. Please enter your username and password when prompted.

Organizational

- Incoming LSs are noted by default. Contact companies should flag LSs that need to be replied from this meeting.

- Legacy topics will be treated by email only unless indicated explicitly. Please see the list of offline email discussions below.

- Rel-16 (draft) CRs and text proposals will be handled as part of the email discussion on the corresponding CR(s) or the ASN.1 review email discussion if associated with a RIL#.

- All organizational emails and notes will be shared over the following email discussion throughout both meeting weeks:

[AT111-e][400][eMTC/NB-IoT] Organizational Emre’s session

Scope:

- Share plans for the e-meeting and make announcements

- Share status of email discussions

- Share meeting minutes and agreements for review and endorsement

Deadline: Friday, August 28th 10:00 UTC

Status: Started

List and Status of Offline Email Discussions

NOTE: The official kick off date for these email discussions are Monday August 17th at 7:00 UTC. The rapporteurs can share them on the reflector earlier, however companies are not required to participate before the official kick off date. The deadlines refer to the deadline for providing company comments unless stated otherwise.

[AT111-e][401][eMTC R16] Early UE Capability Retrieval (Qualcomm)

Status: Started

Scope: Draft a reply LS to SA2 to inform them about the potential solution to address the 1st concern in the LS and to indicate that it should be up to SA2 to decide whether/how other concerns are addressed.

Intended outcome: Draft reply LS provided in R2-2008231

Deadline: Tuesday 25 1100 UTC.

[AT111-e][402][NB-IoT/eMTC R15] UP EDT for DRB using RLC AM (Huawei)

Status: Started

Scope: Progress the discussion and formulate the common understanding.

Intended outcome: Report from the discussion in R2-2008232

Deadline: Tuesday 25 1100 UTC.

[AT111-e][403][eMTC R16] Clarification on subframe level resource reservation (ZTE)

Status: Started

Scope: Check with RAN1 whether the intention was to use the definition of subframe bitmap reservation and consider introducing a new parameter.

Intended outcome: Report from the discussion in R2-2008233

Deadline: Tuesday 25 1100 UTC.

[AT111-e][404][eMTC R16] Measurement requirements for eMTC UEs in RRC_INACTIVE (ZTE)

Status: Started

Scope: Draft a LS to RAN4 to check if measurement requirements for eMTC UEs in RRC_INACTIVE need to be specified.

Intended outcome: Draft LS in R2-2008234

Deadline: Tuesday 25 1100 UTC.

[AT111-e][405][eMTC R16] Corrections to connection to 5GC for eMTC (Qualcomm)

Status: Started

Scope: Collect feedback from companies and update the CR accordingly

Intended outcome: Agreeable CR in R2-2008235

Deadline: Tuesday 25 1100 UTC.

[AT111-e][406][eMTC R16] Addition of missing capabilities for eMTC (Huawei)

Status: Started

Scope: Collect feedback from companies and update the CR accordingly

Intended outcome: Agreeable CR in R2-2008236

Deadline: Tuesday 25 1100 UTC.

# 4	EUTRA corrections Rel-15 and earlier

See Appendix A for reference to Work items, work item codes and WIDs.

Only essential corrections. No documents should be submitted to 4. Please submit to 4.x

## 4.2	eMTC corrections Rel-15 and earlier

Documents in this agenda item will be handled in a break out session. Common NB-IoT/eMTC parts treated jointly with 4.1. No web conference is planned for this agenda item

R2-2007327	Discussion of UP EDT for DRB using RLC AM	Huawei, HiSilicon	discussion	Rel-15	NB_IOTenh2-Core, LTE_eMTC4-Core

- Ericsson wonders what HW has observed in IODT. HW explains that UL grant is given for the UE to provide the report. The other case is sending the poll bit witn no UL grant.

- Ericsson asks whether the default configuration for PUCCH/PUSCH would still be used for such transnmission. HW thinks that would only be for Msg3.

- QC thinks this may be addressed by stating that UE should not be polled in Msg4 when RRC connection is released.

Proposal 1: RAN2 to confirm that the poll bit shall not be set in the RLC PDU carrying RRCConnectionRelease message for UP-EDT.

Proposal 2: RAN2 to confirm that a positive HARQ feedback (HARQ ACK) is an implicit RLC ACK of all the RLC PDUs included in the UP-EDT DL transmission.

Proposal 3: RAN2 to confirm that the poll bit shall be set in the RLC PDU(s) carrying the DL user data for UP-EDT.

Proposal 4: RAN2 to confirm that the poll bit shall be set in the RLC PDU(s) carrying the UL user data for UP-EDT.

Proposal 5: RAN2 to confirm that a RLC STATUS PDU is included in MSG4 for each RLC PDU included in the uplink transmission.

Proposal 6: Proposals 1..3  also apply to MT-EDT.

Proposal 7: Proposals 1..5 also apply to PUR.

[AT111-e][402][NB-IoT/eMTC R15] UP EDT for DRB using RLC AM (Huawei)

Scope: Progress the discussion.

Intended outcome: Report from the discussion in R2-2008232

Deadline: Tuesday 25 1100 UTC.

R2-2007328	Clarification to UP-EDT	Huawei, HiSilicon	CR	Rel-15	36.300	15.10.0	1298	-	F	NB_IOTenh2-Core, LTE_eMTC4-Core

R2-2007329	Clarification to UP-EDT	Huawei, HiSilicon	CR	Rel-16	36.300	16.2.0	1299	-	A	NB_IOTenh2-Core, LTE_eMTC4-Core

# 7	Rel-16 LTE Work Items

Essential corrections

## 7.2	Additional MTC enhancements for LTE

(LTE_eMTC5-Core; LTE_eMTC5-Core; leading WG: RAN1; REL-16; started: Jun 18; Completed:  June 20; WID: RP192875;)

Documents in this agenda item will be handled in a break out session.

Some sub-items in 7.2 and 7.3 may be treated jointly.

Email max expectation: 5-6 email threads

### 7.2.1	General and Stage 2 corrections

Including incoming LSs

### 7.2.2	Mobile-terminated MT early data transmission EDT corrections

MT Early Data transmission for MTC and NB-IoT is treated jointly under this AI.

### 7.2.3	Scheduling multiple DL/UL transport blocks corrections

Scheduling multiple DL/UL transport blocks for MTC and NB-IoT is treated jointly under this AI.

### 7.2.4	Coexistence with NR corrections

Coexistence with NR for MTC and NB-IoT is treated jointly under this AI.

R2-2006858	Clarification on subframe level resource reservation for eMTC	ZTE Corporation, Sanechips	CR	Rel-16	36.331	16.1.1	4358	-	F	LTE_eMTC5-Core

- Huawei thinks subframe reservation bitmap is already available in SIB1 and wonders why it is not possible to use that. It was RAN1’s intention not to introduce subframe bitmap.

- QC thinks it is possible for the eNB to know the UE capabilities, but this is not the case for the common signalling.

[AT111-e][403][eMTC R16] Clarification on subframe level resource reservation (ZTE)

Scope: Check with RAN1 whether the intention was to use the definition of subframe bitmap reservation and consider introducing a new parameter.

Intended outcome: Report from the discussion in R2-2008233

Deadline: Tuesday 25 1100 UTC.

### 7.2.5	Connection to 5GC corrections

Connection to 5GC for MTC and NB-IoT is treated jointly under this AI.

R2-2006859	Measurement requirement for eMTC UE in RRC_INACTIVE state	ZTE Corporation, Sanechips	discussion	LTE_eMTC5-Core

Proposal 1:  Apply the requirements defined for Category M1/M2 without eDRX configuration in RRC_IDLE state mobility as the requirements for Category M1/M2 in RRC_INACTIVE state.

Proposal 1a: send LS to RAN4 to notify the RAN2 specification sate and request to define the Cell Re-selection Requirements for UE category M1/M2 in RRC_INACTIVE state.

- HW thinks RAN2 spec is complete, but something may be missing in RAN4 specs which should be brought up in that WG.

- QC wonders if there is anything that needs to be specified from RAN4 standpoint considering what has been captured today. HW explains that a link seems to be missing to the existing requirements. For eLTE no new requirements were defined in RAN4, it was rather captured that all existing requirements for LTE applies. It might be missed that RRC_INACTIVE state has been introduced.

[AT111-e][404][eMTC R16] Measurement requirements for eMTC UEs in RRC_INACTIVE (ZTE)

Scope: Draft a LS to RAN4 to check if measurement requirements for eMTC UEs in RRC_INACTIVE need to be specified.

Intended outcome: Draft LS in R2-2008234

Deadline: Tuesday 25 1100 UTC.

R2-2006860	Draft LS to RAN4 on measurement requirement for eMTC UE in RRC_INACTIVE state	ZTE Corporation, Sanechips	LS out	LTE_eMTC5-Core	To:RAN4

R2-2007341	Corrections to connection to 5GC for eMTC	Huawei, HiSilicon	CR	Rel-16	36.331	16.1.0	4381	-	F	LTE_eMTC5-Core

- QC thinks there is a need to formulate the following text which seems to be not clear: “if the UE is a BL UE or UE in CE connected to 5GC:”

[AT111-e][405][eMTC R16] Corrections to connection to 5GC for eMTC (Qualcomm)

Scope: Collect feedback from companies and update the CR accordingly

Intended outcome: Agreeable CR in R2-2008235

Deadline: Tuesday 25 1100 UTC.

### 7.2.6	Other MTC specific corrections

Including corrections related to Quality report in Msg3, MPDCCH performance improvement using CRS, Improvements for non-BL UEs, Stand-alone deployment, Mobility enhancements.

R2-2006792	Early UE capability retrieval enhancements for eMTC connecetd to 5GC	Qualcomm Inc, Sierra Wireless, Thales, Telus, ZTE Corporation, TurkCell	discussion	Rel-16	LTE_eMTC5-Core	R2-2004841

R2-2007695	RRC CR for early UE capability retrieval for eMTC connected to 5GC	Qualcomm Inc,Sierra Wireless, Thales, Telus, ZTE Corporation,TurkCell	CR	Rel-16	36.331	16.1.1	4400	-	F	LTE_eMTC5-Core

R2-2007894	[Draft] Reply LS on early UE capability retrieval for eMTC 	Qualcomm Inc	LS out	Rel-16	LTE_eMTC5-Core	To:SA2	Cc:CT1, RAN3

- Huawei thinks this can not be considered as a correction and Rel-16 is closed. Ericsson and Nokia agree.

- QC thinks this should be a Rel-16 feature regardless of treated as a correction or TEI16.

- Thales thinks it would be useful to introduce this feature considering the support from device vendors.

- ZTE thinks it would be better to consider this in this release to avoid a complicated solution.

- QC thinks RAN2 needs to send a reply to SA2. Huawei thinks this is not the case as no reply is expected.

- Ericsson thinks the reply is not necessarily needed unless RAN2 intends to introduce the feature. The use case is in case the UE does not support security activation and that seems to be not the case for eMTC UEs.

- QC thinks the impact to other groups are not significant as the actual work to be done is in RAN2.

- Nokia thinks that SA2 already raised concerns regarding this issue.

- Huawei thinks the gain may be quite insignificant considering the complexity that the feature introduces. There seems to be no eMTC UEs that support only CP solution.

- SW thinks it would be beneficial to introduce the feature.

RAN2 intends to send a reply LS to SA2 to indicate that there seems to be a potential solution to address the 1st concern from RAN2 standpoint, however it should be up to SA2 to decide whether/how other concerns are addressed.

[AT111-e][401][eMTC R16] Early UE Capability Retrieval (Qualcomm)

Scope: Draft a reply LS to SA2 to inform them about the potential solution to address the 1st concern in the LS and to indicate that it should be up to SA2 to decide whether/how other concerns are addressed.

Intended outcome: Draft reply LS provided in R2-2008231

Deadline: Tuesday 25 1100 UTC.

### 7.2.7	MTC UE capabilities corrections

R2-2007340	Addition of missing capabilities for eMTC R16	Huawei, HiSilicon	CR	Rel-16	36.306	16.1.0	1780	-	F	LTE_eMTC5-Core

[AT111-e][406][eMTC R16] Addition of missing capabilities for eMTC (Huawei)

Scope: Collect feedback from companies and update the CR accordingly

Intended outcome: Agreeable CR in R2-2008236

Deadline: Tuesday 25 1100 UTC.