3GPP TSG-RAN WG2 Meeting #111-e		R2-200xxxx

Electronic, 17th – 28th August 2020

Agenda Item: 	5.4.1.5

Source: 			Huawei, HiSilicon

Title: 	Summary of offline 008 – NR UAI

Document for: 	Discussion and decision

# Introduction

This document summarizes the following offline discussion.

UE assistance information

[AT111-e][008][NR15] NR UAI (Huawei)

Scope: Treat R2-2007792, R2-2007793, R2-2007794, R2-2007795, R2-2006986, R2-2006987, R2-2006997, R2-2006998 (proponents to drive)

Part 1: Decision whether to make corrections, identify agreeable parts. Identify Controversial issues for on-line treatment (if any).

Deadline: Aug 20, 0900 UTC.

Part 2: For agreeable parts, continuation to agree CRs.

Deadline: Aug 26, 0900 UTC.

To be noted, the paper R2-2006997 and R2-2006998 are also included in offline discussion [AT111-e][042][NR15], and I understand these two papers are more suitable to be discussed in offline-042. Thus, these two papers are not included in this email discussion.

# Discussion

## Condition of stopping overheating prohibit timer (R2-2007792/R2-2007793)

The CR [1][2] proposes to that on condition of receiving overheatingAssistanceConfig set to release, the overheating prohibit timer T345 should be stopped.

In current specification, the overheating prohibit timer T345 is stopped upon initiating the connection re-establishment and resume procedures. However, if the overheatingAssistanceConfig is set to release, the overheating prohibit timer is not valid anymore, thus it should be stopped in the UE side.

| T345 | Upon transmitting UEAssistanceInformation message with overheatingAssistance | Upon initiating the connection re-establishment procedure, upon initiating the connection resumption procedure, and upon receiving overheatingAssistanceConfig set to release. | No action. |
| --- | --- | --- | --- |

Q1. Do companies agree the intention and proposed changes of this CR?

| Company | Agree / Disagree | Comments |
| --- | --- | --- |
| MediaTek | Agree | Looks like a straightforward correction. Could be in Rapporteur’s CR. |
| Qcom | Agree |  |
| CATT | Agree |  |
| Vivo | Agree |  |
| Samsung | Agree | Same view as MediaTek i.e. minor so appropriate to capture in rapporteurs CR |

## UE assistance information transmission upon reconfiguration with sync (R2-2007794/R2-2007795, R2-2006986/R2-2006987)

The CRs [3][4] and [5][6] propose to clarify the UE assistance information transmission upon reconfiguration with sync. The proposed changes in these CRs are summarised as below:

Issue 1: In clause 5.3.5.3, after UE performs reconfiguration with sync, UE initiate transmission of a UEAssistanceInformation message in accordance with clause 5.7.4.3. However, in clause 5.7.4.3, UE sets the corresponding contents only if there is an initiation according to 5.7.4.2, without including 5.3.5.3. This may lead to inaccuracy UE assistance information. (Discussed in CRs [3][4] and [5][6])

Issue 2: After UE performs reconfiguration with sync, the UE is not restricted to send the UE assistance information with same value. However, if the UE initiates the transmission of UE assistance information message, the corresponding prohibit timer should be started or restarted which is aligned with clause 5.7.4.2. (Discussed in CRs [3][4] and [5][6])

Issue 3: After UE performs reconfiguration with sync, UE can trigger transmission of a UEAssistanceInformation message that was transmitted during the last 1 second before handover. Based on the current description in the specification, the following erroneous case may happen:

- UE transmits UEAssistanceInformation message with delay budget report during the last 1 second before handover, which satisfies condition of “if the UE transmitted a UEAssistanceInformation message during the last 1 second”

- UE is configured to be able to transmit overheating assistance in the target gNB, which satisfies condition of “and the UE is still configured to provide UE assistance information”

- UE can initiate the transmission of a UEAssistanceInformation with clause 5.7.4.3, UE transmit overheating assistance information to the target gNB but it is not transmitted to the source gNB and UE may even not have any preference for overheating.

To avoid the erroneous case above, UE can only transmit the UEAssistanceInformation with the type (i.e. delay budget report, overheating) that was transmitted during the last 1 second before handover and still configured to be able to transmit in the target gNB. (Discussed in CRs [3][4])

Q2. Do companies agree the identified issue 1, 2 and 3 above?

| Company | Agree / Disagree | Comments |
| --- | --- | --- |
| MediaTek | Agree |  |
| Qcom | Disagree | For issue#1: Based on the CR reason for change is that “the UE sets the corresponding contents if there is an initiation according to 5.7.4.2. But there is no initiation according to 5.7.4.2 as the prohibit timer is running”. This issue can be addressed by simply stopping the T345 prohibit timer once UE determines that assistance information was transmitted 1 sec or less before ReconfigurationWithSync was triggered. Suggested change: Section 5.3.5.3 ###skipped## 2> if reconfigurationWithSync was included in masterCellGroup or secondaryCellGroup; and 2> if the UE transmitted a UEAssistanceInformation message for the corresponding cell group during the last 1 second, and the UE is still configured to provide UE assistance information for the corresponding cell group: 3> stop T345; 3> initiate transmission of a UEAssistanceInformation message for the corresponding cell group in accordance with clause 5.7.4.3; For issue#2: This is not an issue any more once our suggested change proposed in issue#1 is used. For issue#3: We’re not sure why these cases were classified as “erroneous case”??? and we don’t see the need to restrict the UE to transmit assistance info type that was only transmitted on the source cell. If there is a need (given UE is configured to do so) to transmit other info type on the target cell, even if not transmitted on source cell, this info will put into good use by the target cell. Therefore we don’t see the issue here. |
| CATT | Agree with issue 1 and 2. | Agree with the intention of issue 3. But it is not accurate to say that UE can only transmit the UEAssistanceInformation with the type (i.e. delay budget report, overheating) that was transmitted during the last 1 second before handover and still configured to be able to transmit in the target gNB. For example: - UE transmits UEAssistanceInformation message with overheating assistance during the last 1 second before handover, which satisfies condition of “if the UE transmitted a UEAssistanceInformation message during the last 1 second” - UE is configured to provide delay budget report in the handover message. - UE initiates the transmission of a UEAssistanceInformation according to 5.7.4.2, which satisfies condition of “the UE did not transmit a UEAssistanceInformation message with delayBudgetReport since it was configured to provide delay budget report”, no matter whether the UE initiates the transmission of UEAssistanceInformation with the type that was transmitted during the last 1 second before handover and still configured to be able to transmit in the target gNB. So issue 3 can be changed to: UE initiate the transmission of UEAssistanceInformation if the type (i.e. delay budget report, overheating) that was transmitted during the last 1 second before handover are still configured to be able to transmit in the target gNB. |
| vivo | Agree with issue 1 and issue 2. | For issue 3, we don’t think it is an erroneous case. The point here is whether the UAI other than the type that was transmitted during the last 1 second before handover is allowed to be reported when target node configured to transmit: Option1: Yes. This is the intention for CATT’s CR and QC’s suggestion. Option2: No. This is the intention for Huawei’s CR. In our understanding, we prefer option 1, i.e. UE not only can transmit the UEAssistanceInformation with the type that was transmitted during the last 1 second before handover, but also can transmit other UEAssistanceInformation configured by target gNB. Thus, we prefer the change proposed by CATT or QC. Regarding to the QC’s suggestion, we think the last change of Huawei’s CR is also needed: 1> if transmission of the UEAssistanceInformation message is initiated to provide a delay budget report according to 5.7.4.2 or 5.3.5.3; 2> set delayBudgetReport to type1 according to a desired value; 1> if transmission of the UEAssistanceInformation message is initiated to provide overheating assistance information according to 5.7.4.2 or 5.3.5.3; |
| Samsung | Agree | We think that late transmission prior to node change concerns another trigger for repeating transmission of the same UE assistance, except for parts not required by target. As it thus serves the same trigger purpose as 5.7.4.2, we think it is appropriate to call 5.7.4.3 as suggested in Huawei’s CRs We assume that intention of 3 is to clarify that in case of UAI shortly before reconfig with sync, UE merely triggers transmission of the same UE assistance if still configured to provide the concerned assistance (which we agree) |

Q3. If the identified issues above are valid, do companies agree the proposed changes in CRs [3][4](Huawei) or [5][6] (CATT)?

For Issue 1&2, both Huawei CRs and CATT CRs give the possible changes, companies are invited to provide the preference or comments on the CRs.

For Issue 3, Huawei CRs give the possible changes, companies are invited to provide the comments on the CRs.

| Company | Agree / Disagree | Comments |
| --- | --- | --- |
| MediaTek |  | We prefer the Huawei’s CRs, which seems simpler. In addition, it seems that the following change is not necessary. The UE just provide the UE assistance information according to latest configuration. 3> initiate transmission of a UEAssistanceInformation message in accordance with clause 5.7.4.3 to provide UE assistance information corresponding to the content(s) transmitted in the UEAssistanceInformation message; |
| Qcom |  | For issue 1 & 2, We don’t agree with either approach, we provided a simpler solution. Please check Q2. For issue 3, as we mentioned, we don’t see the issue, and need to restrict the UE to a specific behaviour, when UE is providing Assistance Information to target cell. |
| CATT | Either is OK | The two set of CRs seem to result in same intended behaviour, so either is OK with us. It seems useful to clarify that changes proposed by CATT CRs also cover issue 3. The difference between Huawei CRs and CATT CRs is that CATT CRs merge all initiations (including normal initiations and initiation by reconfiguration with sync) in the initiation section (i.e. 5.7.4.2). As mentioned above, the UE may initiate the transmission of UEAssistanceInformation upon receiving a new configuration for UE assistance information in the same RRCReconfiguration message. Hence, in our view it is cleaner and more straightforward to that all initiations are put together. |
| vivo |  | See above |
| Samsung |  | We do not agree to the comment from MediaTek i.e. alike Huawei we think there is a need to clarify for what info UE triggers transmission of the UAI message (alike we do for the other triggers specified in 5.7.4.2) We addressed part of the first change in a somewhat simpler manner in R2-2007232 2> if the UE transmitted a UEAssistanceInformation message during the last 1 second, and the UE is still configured to provide the concerned UE assistance information: 3> initiate transmission of a UEAssistanceInformation message to transfer the concerned UE Assistance information in accordance with clause 5.7.4.3; (or, if people think bullet 2 really needs to be more precise) 2> if the UE transmitted a UEAssistanceInformation message during the last 1 second, and the UE is still configured to provide (some of) the UE assistance information included in that message: |

# Conclusion

…

# Reference

R2-2007792	Correction on condition of stopping overheating prohibit timer	Huawei, HiSilicon	CR	Rel-15	38.331	15.10.0	1905	-	F	NR_newRAT-Core

R2-2007793	Correction on condition of stopping overheating prohibit timer	Huawei, HiSilicon	CR	Rel-16	38.331	16.1.0	1906	-	A	NR_newRAT-Core

R2-2007794	Correction on UE assistance information transmission for handover case	Huawei, HiSilicon	CR	Rel-15	38.331	15.10.0	1907	-	F	NR_newRAT-Core

R2-2007795	Correction on UE assistance information transmission for handover case	Huawei, HiSilicon	CR	Rel-16	38.331	16.1.0	1908	-	A	NR_newRAT-Core

R2-2006986	Further correction on UEAssistanceInformation upon reconfiguration with sync	CATT	CR	Rel-15	38.331	15.10.0	1759	-	F	NR_newRAT-Core

R2-2006987	Further correction on UEAssistanceInformation upon reconfiguration with sync	CATT	CR	Rel-16	38.331	16.1.0	1760	-	F	NR_newRAT-Core