3GPP TSG-RAN WG2 Meeting #111-e		R2-200xxxx

Electronic, 17th – 28th August 2020

Agenda Item: 	6.1.2

Source: 			Huawei, HiSilicon

Title: 	Summary of offline 018 UE cap MR-DC Power Class

Document for: 	Discussion and decision

- Introduction

This document summarizes the following offline discussion for MR-DC Power Class.

[AT111-e][018][NR16] UE cap MR-DC Power Class (Huawei)

Scope: Treat R2-2007112, R2-2007113, R2-2007114, R2-2008077, R2-2008078 (proponents to drive),

Deadlines:

Phase I: solution selection, Wednesday 2020-08-19 07:00 UTC

Phase II: CR details,Friday 2020-08-21 07:00 UTC

- Discussion

  - Background

In RP-201392 it is requested RAN2 to introduce new capability signalling for the below case:

RAN4 has discussed an ambiguity that may arise in regard to the output power available on individual carriers when the UE is configured with an MR-DC configuration. Depending on UE implementation, in some circumstances a UE reporting PC2 for an NR band and PC2 for the MR-DC band combination may provide PC2 in NR part (single NR band or intra-band NR CA) of the MR-DC combination, whereas in other cases the UE would provide PC3 in the NR part of the MR-DC combination. It is ambiguous to the network whether PC2 or PC3 is applicable in the NR part of the MR-DC band combination. To resolve this, RAN#88e agreed that a new power class capability signalling for Rel-16 should be introduced in addition to the existing MR-DC power class.

RAN respectfully requests RAN2 to develop new power class UE capability signalling applicable for the NR part of the MR-DC band combination. This is in addition to the MR-DC power class, i.e. indicates that UE supports the newly indicated power class for the NR part of the MR-DC band combination also in addition the indicated power class for the MR-DC band combination.  The value range for the signalled power class values is { pc1, pc2, pc3, pc5 }.

  - Phase I discussion: Option selection

There are two options to capture the signalling based on contributions:

- Option 1: per BC reporting [1][2]

- Option 2: per band per BC reporting [1][2][3]

It would be good to try to have a consensus to adopt one option, and discuss details.

Q1. Which option is preferred?

| Company | Option 1/Option 2 | Comments |
| --- | --- | --- |

  - Phase II discussion: CR details

To be updated after Phase I discussion

…

- Reference

R2-2008077	Introduction of new PowerClass for NR part in MR-DC	Huawei, HiSilicon, CMCC

R2-2008078	Introduction of new PowerClass for NR part in MR-DC	Huawei, HiSilicon, CMCC

R2-2007112	Discussion on UE capability for power class for NR band in MR-DC combination	Apple

R2-2007113	UE capability for power class for NR band in MR-DC combination		Apple

R2-2007114	UE capability for power class for NR band in MR-DC combination		Apple