3GPP TSG-RAN WG2 Meeting #111 electronic	R2-2008122

Online, August 17th - 28th, 2020

Agenda item:	10.2

Source: 	Vice Chairman (ZTE Corporation)

Title:	Report from Break-out session on R16 eMIMO, CLI, PRN, RACS and R17 NTN and REDCAP

Document for:	Approval

General

Recording of voice or video at meetings is not used in 3GPP. This applies also to this e-Meeting. At this e-Meeting, no specific actions are taken to prevent the recording of web conferences. Companies that have concerns related to recordings, if any, may express those by email in the main meeting organizational thread [AT111e][000]

Organizational

- For R16 items, summary discussion papers might be used during the e-meeting (as indicated in the meeting notes). For R17 items, no summary discusison papers will be used at this meeting.

- All organization emails and notes will be shared over the following email discussion throughout the two meeting weeks:

[AT111e][100] Organizational Sergio's session (eMIMO, CLI, PRN, RACS, NTN, REDCAP)

Scope:

    - Share plans for the meeting and list of ongoing email discussions for the sessions related to eMIMO, CLI and other NR R1 WIs Corrections, PRN, RACS, NTN and REDCAP

    - Share meetings notes and agreements for review and endorsement

Schedule/Plan

eMIMO:

The discussion will initially happen in offline email discussions (101) kicked off at the e-meeting start and will then continue during the web conference call(s).

Tuesday August 18th, 13:30 - 15:00 UTC:

- Check the status of email discussion 101 and decide on next steps

CLI and other NR R1 WIs Corrections:

The discussion will initially happen in offline email discussions (102) kicked off at the e-meeting start and will then continue during the web conference call(s).

Tuesday August 18th, 13:30 - 15:00 UTC:

- Check the status of email discussion 102 and decide on next steps

- Discuss other NR R1 WI corrections

RACS:

The discussion will initially happen in offline email discussions (103) kicked off at the e-meeting start and will then continue during the web conference call(s).

Wednesday August 19th, 13:30 - 15:00 UTC:

- Check the status of email discussion 103 and decide on next steps

PRN:

The discussion will initially happen in offline email discussions (104) kicked off at the e-meeting start and will then continue during the web conference call(s).

Wednesday August 19th, 13:30 - 15:00 UTC:

- Check the status of email discussion 104 and decide on next steps

NTN:

The discussion will initially happen in offline email discussions (105, 106 and 107) kicked off at the e-meeting start and will then continue during the web conference call(s).

Friday August 21th, 3:30 - 5:00 UTC:

- Check the status of email discussion 105, 106 and decide on next steps

- Discuss the incoming LS in R2-2006530 and suggested reply LS

- Continue the discussion on CP aspects

Monday August 24th, 13:00 - 14:30 UTC:

- Check the status of email discussion 107 and decide on next steps

- Continue the discussion on UP aspects

REDCAP:

The discussion will initially happen in offline email discussions (108, 109, 110 and 111) kicked off at the e-meeting start and will then continue during the web conference call(s).

Tuesday August 25th, 13:00 - 16:00 UTC:

- Check the status of email discussion 108, 109, 110, 111 and decide on next steps

- Start the discussion on the other aspects

List and status of offline email discussions

NOTE:  No offline email discussions will be kicked off before Monday August 17th, 07:00 UTC

[AT111e][101][eMIMO] MAC corrections (Samsung)

Initial scope: Discuss the CRs in R2-2006779, R2-2007525, R2-2006797, R2-2007485, R2-2007736, R2-2007526, R2-2007895 and R2-2008053

Initial intended outcome: summary of the offline discussion with e.g.:

    - List of CRs that can be agreed as is

    - List of CRs that can be agreed with some changes (with an indication of the needed changes)

    - List of CRs that require online discussion

    - List of CRs that should not be pursued

Initial deadline (for companies' feedback): Tuesday 2020-08-18 07:00 UTC

Initial deadline (for rapporteur's summary in R2-2008181):  Tuesday 2020-08-18 09:00 UTC

Updated scope: Draft the CR in R2-2008196 and continue the discussion on R2-2008053, e.g. to see whether this issue should be addressed in RAN2 or other groups.

Updated intended outcome: Agreeable CR in R2-2008196 and summary of the discussion in R2-2008197

Updated deadline (for companies' feedback): Wednesday 2020-08-26 07:00 UTC

Updated deadline (for rapporteur's summary in R2-2008197): Wednesday 2020-08-26 09:00 UTC

Status: Ongoing

[AT111e][102][CLI] Reply LS to RAN3 (ZTE)

Scope: Attempt drafting a reply LS to the incoming LS in R2-2006524 based on the related contributions in R2-2006898 and R2-2007355 and draft reply LS proposals in R2-2006899, R2-2007356 and R2-2007851

Initial intended outcome: initial draft reply LS to RAN3 in R2-2008182:

Initial deadline (for companies' feedback): Tuesday 2020-08-18 10:00 UTC

Initial deadline (for initial draft reply LS in R2-2008182):  Tuesday 2020-08-18 12:00 UTC

Updated Scope: Continue the discussion and attempt a revision of the reply LS

Updated intended outcome: revised draft reply LS to RAN3 in R2-2008200

Updated interim deadline (for companies' feedback): Friday 2020-08-21 00:00 UTC

Updated interim deadline (for revised draft reply LS in R2-2008200): Friday 2020-08-21 02:00 UTC

If the draft reply LS in R2-2008200 will be not challenged until Friday 2020-08-21 08:00 UTC, it will be declared as agreed by the session chair. Otherwise the discussion will continue until the CB online session on Wednesday 2020-08-26.

Status: Ongoing

[AT111e][103][RACS] Corrections (Huawei)

Scope: Discuss the CRs in R2-2008104, R2-2007806 and R2-2007807

Initial intended outcome: summary of the offline discussion with e.g.:

    - List of CRs that can be agreed as is

    - List of CRs that can be agreed with some changes (with an indication of the needed changes)

    - List of CRs that require online discussion

    - List of CRs that should not be pursued

Initial deadline (for companies' feedback): Wednesday 2020-08-19 07:00 UTC

Initial deadline (for rapporteur's summary in R2-2008183):  Wednesday 2020-08-19 09:00 UTC

CRs listed as "can be agreed as is" in R2-2008183 and not challenged until Wednesday 2020-08-19 13:00 UTC will be declared as agreed by the session chair. For the other ones, the discussion will continue online.

Status: Ongoing

[AT111e][104][PRN] Stage 3 Corrections (Nokia)

Scope: Discuss the CRs in R2-2006634, R2-2006852, R2-2007841, R2-2008114, R2-2006633, R2-2007842, R2-2006853, R2-2007411 and R2-2008016

Initial intended outcome: summary of the offline discussion with e.g.:

    - List of CRs that can be agreed as is

    - List of CRs that can be agreed with some changes (with an indication of the needed changes)

    - List of CRs that require online discussion

    - List of CRs that should not be pursued

Initial deadline (for companies' feedback): Wednesday 2020-08-19 07:00 UTC

Initial deadline (for rapporteur's summary in R2-2008184):  Wednesday 2020-08-19 09:00 UTC

CRs listed as "can be agreed as is" in R2-2008184 and not challenged until Wednesday 2020-08-19 13:00 UTC will be declared as agreed by the session chair. For the other ones, the discussion will continue online.

Status: Ongoing

[AT111e][105][NTN] Workplan, scope and scenarios (Thales)

Scope: Discuss the workplan in R2-2007565 and the proposals in R2-2007572, R2-2007537,  R2-2006630 (and possibly others from contributions in 8.10.1)

Initial intended outcome: revised workplan and summary of the offline discussion with e.g.:

    - List of agreeable proposals (if any)

    - List of proposals that require online discussions

Initial deadline (for companies' feedback): Thursday 2020-08-20 16:00 UTC

Initial deadline (for rapporteur's summary in R2-2008185):  Thursday 2020-08-20 18:00 UTC

Status: Ongoing

[AT111e][106][NTN] Idle mode issues (ZTE)

Scope: Discuss the proposals in R2-2006872, R2-2006973, R2-2007171 and proposals 1 and 2 in R2-2007574. The intention is to identify design alternatives, collect company views and, whenever possible, also narrow down the proposals.

Initial intended outcome: summary of the offline discussion with e.g.:

    - List of agreeable proposals (if any)

    - List of proposals that require online discussions

Initial deadline (for companies' feedback): Thursday 2020-08-20 16:00 UTC

Initial deadline (for rapporteur's summary in R2-2008187):  Thursday 2020-08-20 18:00 UTC

Status: Ongoing

[AT111e][107][NTN] Pre-compensation and other MAC issues (Interdigital)

Scope: Discuss the proposals in R2-2007615, R2-2007616, R2-2006928, R2-2007590 (and possibly other proposals from contributions in 8.10.2.1 focussing on pre-compensation and offset calculations), as well as proposals 1 to 5 in R2-2007784. The intention is to identify design alternatives, collect company views and, whenever possible, also narrow down the proposals.

Initial intended outcome: summary of the offline discussion with e.g.:

    - List of agreeable proposals (if any)

    - List of proposals that require online discussions

Initial deadline (for companies' feedback): Friday 2020-08-21 08:00 UTC

Initial deadline (for rapporteur's summary in R2-2008188):  Friday 2020-08-21 10:00 UTC

Status: Ongoing

[AT111e][108][REDCAP] Scope and skeleton update (Ericsson)

Scope: Discuss the SI scope in R2-2006910 and the skeleton update in R2-2007366

Initial intended outcome: summary of the offline discussion with e.g.:

    - List of agreeable proposals (if any)

    - List of proposals that require online discussions

and skeleton update

Initial deadline (for companies' feedback): Monday 2020-08-24 16:00 UTC

Initial deadline (for rapporteur's summary in R2-2008189):  Monday 2020-08-24 18:00 UTC

Status: Not yet started

[AT111e][109][REDCAP] Reduced capability signalling framework (Intel)

Scope: Discuss the proposals in R2-2006751, R2-2006911 and R2-2006605. The intention is to identify design alternatives, collect company views and, whenever possible, also narrow down the proposals.

Initial intended outcome: summary of the offline discussion with e.g.:

    - List of agreeable proposals (if any)

    - List of proposals that require online discussions

Initial deadline (for companies' feedback): Monday 2020-08-24 22:00 UTC

Initial deadline (for rapporteur's summary in R2-2008191):  Tuesday 2020-08-25 02:00 UTC

Status: Not yet started

[AT111e][110][REDCAP] Identification and access restriction (Huawei)

Scope: Discuss the proposals in R2-2007345, R2-2006661, R2-2006786 and R2-2007493. The intention is to identify design alternatives, collect company views and, whenever possible, also narrow down the proposals.

Initial intended outcome: summary of the offline discussion with e.g.:

    - List of agreeable proposals (if any)

    - List of proposals that require online discussions

Initial deadline (for companies' feedback): Monday 2020-08-24 22:00 UTC

Initial deadline (for rapporteur's summary in R2-2008192):  Tuesday 2020-08-25 02:00 UTC

Status: Not yet started

[AT111e][111][REDCAP] DRX aspects (CATT)

Scope: Discuss the proposals in R2-2007013, R2-2007346, R2-2007494 as well as proposals 1 to 4 in R2-2006748. The intention is to identify design alternatives, collect company views and, whenever possible, also narrow down the proposals.

Initial intended outcome: summary of the offline discussion with e.g.:

    - List of agreeable proposals (if any)

    - List of proposals that require online discussions

Initial deadline (for companies' feedback): Monday 2020-08-24 22:00 UTC

Initial deadline (for rapporteur's summary in R2-2008193):  Tuesday 2020-08-25 02:00 UTC

Status: Not yet started

[AT111e][112][eMIMO] RRC Corrections (Ericsson)

Scope: Continue the discussion on R2-2007161 and R2-2007577

Intended outcome: Agreeable CRs in R2-2008198 and R2-2008199

Initial deadline (for companies' feedback): Wednesday 2020-08-26 07:00 UTC

Initial deadline (for final CRs): Wednesday 2020-08-26 09:00 UTC

Status: Ongoing

[AT111e][113][CLI] RRC CR (LG)

Scope: Revise the CR in R2-2007989

Intended outcome: Agreeable CR in R2-2008201

Initial deadline (for companies' feedback): Wednesday 2020-08-26 07:00 UTC

Initial deadline (for final CR): Wednesday 2020-08-26 09:00 UTC

Status: Ongoing

[AT111e][114][L1enh_URLLC] RRC CRs (CATT)

Scope: discuss the TP for a possible revision of R2-2007080 and revise R2-2007862

Intended outcome: Agreeable TP for a possible revision of R2-2007080 in R2-2008202

and agreeable CR in R2-2008203

Initial deadline (for companies' feedback): Wednesday 2020-08-26 07:00 UTC

Initial deadline (for TP in R2-2008202 and CR in R2-2008203): Wednesday 2020-08-26 09:00 UTC

Status: Ongoing

## 6.12	NR Other Control Plane WIs

(SRVCC_NR_to_UMTS-Core; leading WG: RAN2; REL-16; started: Dec 18; Completed; Mar 20; WID: RP-190713)

(RACS-RAN-Core, leading WG: RAN2; REL-16; started: Mar 19; completed: Jun 20; WID: RP-191088)

(NG_RAN_PRN-Core; leading WG: RAN3; REL-16; started: Mar 19; completed: June 20; WID: RP-200122)

Documents in this agenda item will be handled in a break out session

Email max expectation: 3 email threads

RACS

R2-2006516	LS reply on RACS multiple radio capability formats (R3-204147; contact: Huawei)	RAN3	LS in	Rel-16	RACS-RAN-Core	To:SA2	Cc:RAN2, CT4, CT3

- Noted

R2-2007805	Correction on the UE Capability presence upon SN addition and SN change	Huawei, HiSilicon	CR	Rel-16	38.331	16.1.0	1911	-	F	RACS-RAN-Core

- Revised in R2-2008104

R2-2008104	Correction on the UE Capability presence upon SN addition and SN change	Huawei, HiSilicon, Ericsson	CR	Rel-16	38.331	16.1.0	1911	1	F	RACS-RAN-Core

- Initially discussed in offline 103

...

R2-2007806	CR on UE capability of segmentation for UE capability information	Huawei, HiSilicon	CR	Rel-16	38.306	16.1.0	0392	-	F	RACS-RAN-Core

- Initially discussed in offline 103

…

R2-2007807	CR on UE capability of segmentation for UE capability information	Huawei, HiSilicon	CR	Rel-16	36.306	16.1.0	1783	-	F	RACS-RAN-Core

- Initially discussed in offline 103

…

[AT111e][103][RACS] Corrections (Huawei)

Scope: Discuss the CRs in R2-2008104, R2-2007806 and R2-2007807

Initial intended outcome: summary of the offline discussion with e.g.:

    - List of CRs that can be agreed as is

    - List of CRs that can be agreed with some changes (with an indication of the needed changes)

    - List of CRs that require online discussion

    - List of CRs that should not be pursued

Initial deadline (for companies' feedback): Wednesday 2020-08-19 07:00 UTC

Initial deadline (for rapporteur's summary in R2-2008183):  Wednesday 2020-08-19 09:00 UTC

CRs listed as "can be agreed as is" in R2-2008183 and not challenged until Wednesday 2020-08-19 13:00 UTC will be declared as agreed by the session chair. For the other ones, the discussion will continue online.

R2-2008183	Summary of offline 103 - RACS corrections	Huawei, HiSilicon	discussion	Rel-16	RACS-RAN-Core

PRN

Stage 2

R2-2006879	Correction to the support of NR-DC for PNI-NPN 	Lenovo, Motorola Mobility	CR	Rel-16	38.300	16.2.0	0261	-	F	NG_RAN_PRN-Core

…

38.304 corrections

R2-2006634	Correction on Naming  of the List of Forbidden Tracking Areas	CATT	CR	Rel-16	38.304	16.1.0	0176	-	F	NG_RAN_PRN-Core

- Initially discussed in offline 104

…

R2-2006852	Cell selection and reselection corrections for NPNs	Nokia, Nokia Shanghai Bell	CR	Rel-16	38.304	16.1.0	0177	-	F	NG_RAN_PRN-Core, NR_unlic-Core

- Initially discussed in offline 104

…

R2-2007841	Correction to 38.304 on any cell seletion in NPN	Huawei, HiSilicon	CR	Rel-16	38.304	16.1.0	0181	-	F	NG_RAN_PRN-Core

- Initially discussed in offline 104

…

R2-2007902	38.304 Correction on UE behavior when the best cell is not suitable	vivo	CR	Rel-16	38.304	16.1.0	0183	-	F	NG_RAN_PRN-Core

=> Revised in R2-2008114

R2-2008114	38.304 Correction on UE behavior when the best cell is not suitable	vivo Nokia,  Nokia Shanghai Bell	CR	Rel-16	38.304	16.1.0	0183	1	F	NG_RAN_PRN-Core

- Initially discussed in offline 104

…

RRC corrections

Clarification on first NPN-Identity

R2-2006633	Correction on First NPN-Identity Usage for SIB Validity	CATT	CR	Rel-16	38.331	16.1.0	1722	-	F	NG_RAN_PRN-Core

- Initially discussed in offline 104

…

R2-2007842	Correction to 38.331 on SIB validity and emergency services for NPN	Huawei, HiSilicon	CR	Rel-16	38.331	16.1.0	1926	-	F	NG_RAN_PRN-Core

- Initially discussed in offline 104

…

PNI-NPN related parameter selection

R2-2006853	Corrections for PNI-NPN related parameter selection	Nokia, Nokia Shanghai Bell	CR	Rel-16	38.331	16.1.0	1742	-	F	NG_RAN_PRN-Core

- Initially discussed in offline 104

…

Emergency services support in SNPN Access Mode

R2-2007411	ims-EmergencySupport interpretation and clarification for SNPN	Ericsson	discussion	Rel-16	NG_RAN_PRN-Core

- Initially discussed in offline 104

…

Other

R2-2008016	Corrections to IntraFreqCAG-CellPerPLMN and InterFreqCAG-CellList in SIB3 and SIB4	Samsung Electronics Co., Ltd	CR	Rel-16	38.331	16.1.0	1973	-	D	NG_RAN_PRN-Core

- Initially discussed in offline 104

…

[AT111e][104][PRN] Stage 3 Corrections (Nokia)

Scope: Discuss the CRs in R2-2006634, R2-2006852, R2-2007841, R2-2008114, R2-2006633, R2-2007842, R2-2006853, R2-2007411 and R2-2008016

Initial intended outcome: summary of the offline discussion with e.g.:

    - List of CRs that can be agreed as is

    - List of CRs that can be agreed with some changes (with an indication of the needed changes)

    - List of CRs that require online discussion

    - List of CRs that should not be pursued

Initial deadline (for companies' feedback): Wednesday 2020-08-19 07:00 UTC

Initial deadline (for rapporteur's summary in R2-2008184):  Wednesday 2020-08-19 09:00 UTC

CRs listed as "can be agreed as is" in R2-2008184 and not challenged until Wednesday 2020-08-19 13:00 UTC will be declared as agreed by the session chair. For the other ones, the discussion will continue online.

R2-2008184	Summary of offline 104 - PRN corrections	Nokia	discussion	Rel-16	NG_RAN_PRN-Core

Other

R2-2007404	Limited services and SNPN Access Mode	Ericsson	discussion	Rel-16	NG_RAN_PRN-Core

…

## 6.13	NR eMIMO

(NR_eMIMO-Core, leading WG: RAN1; REL-16; started: Jun 18; target; Aug 20; WID: RP-200474; R2 part completed)

Documents in this agenda item will be handled in a break out session

Email max expectation: 2 email threads

### 6.13.1	User plane corrections

MAC corrections

Candidate RS ID

R2-2006779	Corrections to description of Candidate RS ID in BFR MAC CE	Samsung Electronics Co., Ltd	CR	Rel-16	38.321	16.1.0	0784	-	F	NR_eMIMO-Core

- Initially discussed in offline 101

- Revised in R2-2008194 with changes agreed in the offline 101

R2-2008194	Corrections to description of Candidate RS ID in BFR MAC CE	Samsung Electronics Co., Ltd	CR	Rel-16	38.321	16.1.0	0784	1	F	NR_eMIMO-Core

- Agreed unseen

R2-2007525	CR on 38.321 for BFR MAC CE design	ZTE Corporation, Sanechips	CR	Rel-16	38.321	16.1.0	0826	-	F	NR_eMIMO-Core

- Initially discussed in offline 101

- Not pursued (as similar changes are covered in R2-2008194)

BFR cancellation

R2-2006797	Clarification on the BFR MAC CE report	vivo	CR	Rel-16	38.321	16.1.0	0785	-	F	NR_eMIMO-Core

- Initially discussed in offline 101

- Vivo would like to clarify that NR is different from LTE and there would be some drawback in not accepting the CR. However they are ok to go for the majority view

- Not pursued

R2-2007485	Correction on the BFR cancellation	Nokia, Nokia Shanghai Bell	CR	Rel-16	38.321	16.1.0	0824	-	F	NR_eMIMO-Core

- Initially discussed in offline 101

- Revised in R2-2008195 with changes agreed in the offline 101

R2-2008195	Correction on the BFR cancellation	Nokia, Nokia Shanghai Bell	CR	Rel-16	38.321	16.1.0	0824	-	F	NR_eMIMO-Core

- Agreed unseen

R2-2007736	BFR Cancellation regarding MAC reset	ASUSTek	CR	Rel-16	38.321	16.1.0	0837	-	F	NR_eMIMO-Core

- Initially discussed in offline 101

- Agreed

BFR procedure

R2-2007526	CR on 38.321 for BFR procedue	ZTE Corporation, Sanechips	CR	Rel-16	38.321	16.1.0	0827	-	F	NR_eMIMO-Core

- Initially discussed in offline 101

- ZTE wonders if all companies agree that with the acronym BFR we refer to Scell Beam Failure Recovery. CATT, Samsung, Qualcomm and Intel think the suggested correction is correct but maybe not essential. Ericsson thinks that this would be misleading in the SpCell case.

- Not pursued

Other corrections

R2-2007895	Correction on AP and SP SRS MAC-CE	Asia Pacific Telecom co. Ltd	discussion	NR_eMIMO-Core

- Initially discussed in offline 101

- Nokia thinks we already have similar text in 5.18.13 so we don't need the change. Samsung is fine with proposal in TP1, at the same time voiding section 5.18.13. Nokia could accept that, but think we need to check all aspects, including the title of the section

- For the second change, Ericsson suggests to use the same wording as in the Rel-15 MAC CE

- Draft an actual CR in R2-2008196 based on the status of the offline discussion (considering both proposals 1&2)

- Continue in the follow-up of offline 101

R2-2008196	Correction on AP and SP SRS MAC-CE	Asia Pacific Telecom co. Ltd	CR	Rel-16 38.321 16.1.0	XXXX	-	F	NR_eMIMO-Core

R2-2008053	Correction on the definition of Ci field in BFR MAC CE	Qualcomm Incorporated	draftCR	Rel-16	38.321	16.1.0	F	NR_eMIMO-Core

- Initially discussed in offline 101

- Vivo thinks the problem described in the CR is related to RAN1 or RAN4 and should be discussed there first.

- Ericsson would not like to change the definition of the Ci field. Also think that this could be address in RAN4 to change the timing requirements.

- Continue in the follow-up of offline 101

[AT111e][101][eMIMO] MAC corrections (Samsung)

Initial scope: Discuss the CRs in R2-2006779, R2-2007525, R2-2006797, R2-2007485, R2-2007736, R2-2007526, R2-2007895 and R2-2008053

Initial intended outcome: summary of the offline discussion with e.g.:

    - List of CRs that can be agreed as is

    - List of CRs that can be agreed with some changes (with an indication of the needed changes)

    - List of CRs that require online discussion

    - List of CRs that should not be pursued

Initial deadline (for companies' feedback): Tuesday 2020-08-18 07:00 UTC

Initial deadline (for rapporteur's summary in R2-2008181):  Tuesday 2020-08-18 09:00 UTC

Updated scope: Draft the CR in R2-2008196 and continue the discussion on R2-2008053, e.g. to see whether this issue should be addressed in RAN2 or other groups.

Updated intended outcome: Agreeable CR in R2-2008196 and summary of the discussion in R2-2008197

New deadline (for companies' feedback): Wednesday 2020-08-26 07:00 UTC

Updated deadline (for rapporteur's summary in R2-2008197): Wednesday 2020-08-26 09:00 UTC

R2-2008181	Summary of offline 101 - MAC corrections for eMIMO	Samsung	discussion	Rel-16	NR_eMIMO-Core

Proposal 1: Agree the CR R2-2006779 with TP as shown below:

“Candidate RS ID: This field is set to the index of an SSB with SS-RSRP above rsrp-ThresholdBFR amongst the SSBs in candidateBeamRSSCellList or to the index of a CSI-RS with CSI-RSRP above rsrp-ThresholdBFR amongst the CSI-RSs in candidateBeamRSSCellList. Index of an SSB or CSI-RS is the index of an entry in candidateBeamRSSCellList corresponding to the SSB or CSI-RS. Index 0 corresponds to the first entry in the candidateBeamRSSCellList, index 1 corresponds to the second entry in the list and so on. The length of this field is 6 bits.”

Proposal 2: Agree the CR R2-2007736 as is.

Proposal 3: Agree the CR R2-2007485 with TP as shown below:

“Pending SR triggered prior to the MAC PDU assembly for beam failure recovery of an SCell shall be cancelled and respective sr-ProhibitTimer shall be stopped when the MAC PDU is transmitted and this PDU includes an BFR MAC CE or Truncated BFR MAC CE which contains beam failure recovery information of that SCell. Pending SR triggered for beam failure recovery of an SCell shall be cancelled upon deactivation of that SCell (as defined in clause 5.9).”

Proposal 4: The CRs CR R2-2006797 and R2-2007526 are not pursued.

Proposal 5: Further discuss the CR R2-2008053 online.

Proposal 6: Approve the Text Proposal 1 (in R2-2007895) for activation/deactivation of SP SRS resource set. The redundant sub-clause 5.18.8 is removed (i.e. can be Voided).

Agreements:

1: Agree the CR R2-2006779 with TP as shown below:

“Candidate RS ID: This field is set to the index of an SSB with SS-RSRP above rsrp-ThresholdBFR amongst the SSBs in candidateBeamRSSCellList or to the index of a CSI-RS with CSI-RSRP above rsrp-ThresholdBFR amongst the CSI-RSs in candidateBeamRSSCellList. Index of an SSB or CSI-RS is the index of an entry in candidateBeamRSSCellList corresponding to the SSB or CSI-RS. Index 0 corresponds to the first entry in the candidateBeamRSSCellList, index 1 corresponds to the second entry in the list and so on. The length of this field is 6 bits.”

2: Agree the CR R2-2007736 as is.

3: Agree the CR R2-2007485 with TP as shown below (also keep the change in 5.17)

“Pending SR triggered prior to the MAC PDU assembly for beam failure recovery of an SCell shall be cancelled and respective sr-ProhibitTimer shall be stopped when the MAC PDU is transmitted and this PDU includes an BFR MAC CE or Truncated BFR MAC CE which contains beam failure recovery information of that SCell. Pending SR triggered for beam failure recovery of an SCell shall be cancelled upon deactivation of that SCell (as defined in clause 5.9).

4.  The CRs CR R2-2006797 and R2-2007526 are not pursued.

R2-2008197	Summary of offline 101 - MAC corrections for eMIMO - second round	Samsung	discussion	Rel-16	NR_eMIMO-Core

On LS exchange with RAN1

R2-2007575	On serving cell set based SRS spatial relation indication MAC CE	Ericsson	discussion	Rel-16	NR_eMIMO-Core

- Comeback during the CB session next week (if a reply LS will be received from RAN1)

Withdrawn

R2-2007544	Correction on the definition of Ci field in BFR MAC CE	Qualcomm Incorporated	draftCR	Rel-16	38.321	16.1.0	F	NR_eMIMO-Core	Withdrawn

### 6.13.2	Control plane corrections

R2-2007161	Correction on number of CORESETs per BWP	OPPO	CR	Rel-16	38.331	16.1.0	1793	-	F	NR_eMIMO-Core

- Ericsson thinks the first change was already in an original TP and lost in the implementation. So it's ok to have. However Ericsson thinks that we should keep the sentence on Rel-15. For Rel-16 we don't need an additional sentence because there is an UE capability for this.

- Huawei thinks we cannot simply change 3 to 5, as this depends on UE capability.

- Continue the discussion in offline 112

- Revised in R2-2008198

R2-2008198	Correction on number of CORESETs per BWP	OPPO	CR	Rel-16	38.331	16.1.0	1793	1	F	NR_eMIMO-Core

R2-2007577	Miscellaneous eMIMO corrections 	Ericsson	CR	Rel-16	38.331	16.1.0	1863	-	F	NR_eMIMO-Core

- Nokia is fine to remove the text in the first change and also remove "that is configured". But they would not like to use the wording "UE may expect to…". QC is fine with the principle of the CR but would like to consider other similar changes for other modes. Apple suggests to indicate which RAN1 spec, not only the sub-clause.

- continue the discussion in offline 112

- Revised in R2-2008199

R2-2008199	Miscellaneous eMIMO corrections 	Ericsson	CR	Rel-16	38.331	16.1.0	1863	-	F	NR_eMIMO-Core

[AT111e][112][eMIMO] RRC Corrections (Ericsson)

Scope: Continue the discussion on R2-2007161 and R2-2007577

Intended outcome: Agreeable CRs in R2-2008198 and R2-2008199

Initial deadline (for companies' feedback): Wednesday 2020-08-26 07:00 UTC

Initial deadline (for final CRs): Wednesday 2020-08-26 09:00 UTC

## 6.14	NR Other R1 WIs

(NR_CLI_RIM; leading WG: RAN1; REL-16; started: Dec 18; Completed: Jun 20; WID: RP-191997;)

(NR_L1enh_URLLC-Core, leading WG: RAN1; REL-16; Completed: June 20; WID: RP-191584)

(R1 Led NR TEI16, Other R1 led items)

Documents in this agenda item will be handled in a break out session

Email max expectation: 5 email threads

### 6.14.1	User plane corrections

### 6.14.2	Control plane corrections

CLI - Reply LS from RAN3 and related discussion

R2-2006524	Response LS on Exchange of information related to SRS-RSRP measurement resource configuration for UE-CLI R3-204399; contact: ZTE)	RAN3	LS in	Rel-16	NR_CLI_RIM	To:RAN2, RAN1	Cc:RAN4

- Discussed in offline 102

- Noted

R2-2006898	Discussion on RAN3 LS about SRS exchange	ZTE Corporation, Sanechips	discussion	Rel-16	NR_CLI_RIM-Core

- Discussed in offline 102

- Noted

R2-2007355	Exchange of SRS Information across GNB for UE CLI	Nokia, Nokia Shanghai Bell	discussion	Rel-16

- Discussed in offline 102

- Noted

R2-2006899	Draft reply LS on exchange of information related to SRS-RSRP measurement resource configuration for UE-CLI	ZTE Corporation	LS out	Rel-16	NR_CLI_RIM-Core	To:RAN3	Cc:RAN1, RAN4

- Revised in R2-2008182 based on the outcome of offline 102

R2-2008182	Draft reply LS on exchange of information related to SRS-RSRP measurement resource configuration for UE-CLI	ZTE Corporation	LS out	Rel-16	NR_CLI_RIM-Core	To:RAN3	Cc:RAN1, RAN4

- QCs think the latest version is a reduced version of what was discussion but it's acceptable for them.

- Huawei and Ericsson think that the frequency is up to gNB implementation and there is no additional information that we can provide.

- LG and ZTE think that for this CLI measurement purpose the SRS configuration would be semi-static. QC agrees.

- Continue the discussion in a follow-up of offline 102

- Revised in R2-2008200

R2-2008200	Draft reply LS on exchange of information related to SRS-RSRP measurement resource configuration for UE-CLI	ZTE Corporation	LS out	Rel-16	NR_CLI_RIM-Core

R2-2007356	[Draft] Reply LS to the LS on Exchange of information related to SRS-RSRP measurement resource configuration for UE-CLI	Nokia, Nokia Shanghai Bell	LS out	Rel-16	NR_CLI_RIM	To:RAN3	Cc:RAN4

- Discussed in offline 102

- Noted

R2-2007851	Draft LS on Update frequency of SRS-RSRP configuration for CLI	Samsung	LS out	Rel-16	NR_CLI_RIM	To:RAN WG3	Cc:RAN WG1, RAN WG4

- Discussed in offline 102

- Noted

[AT111e][102][CLI] Reply LS to RAN3 (ZTE)

Scope: Attempt drafting a reply LS to the incoming LS in R2-2006524 based on the related contributions in R2-2006898 and R2-2007355 and draft reply LS proposals in R2-2006899, R2-2007356 and R2-2007851

Initial intended outcome: initial draft reply LS to RAN3 in R2-2008182:

Initial deadline (for companies' feedback): Tuesday 2020-08-18 10:00 UTC

Initial deadline (for initial draft reply LS in R2-2008182):  Tuesday 2020-08-18 12:00 UTC

Updated Scope: Continue the discussion and attempt a revision of the reply LS

Updated intended outcome: revised draft reply LS to RAN3 in R2-2008200

Updated interim deadline (for companies' feedback): Friday 2020-08-21 00:00 UTC

Updated interim deadline (for revised draft reply LS in R2-2008200): Friday 2020-08-21 02:00 UTC

If the draft reply LS in R2-2008200 will be not challenged until Friday 2020-08-21 08:00 UTC, it will be declared as agreed by the session chair. Otherwise the discussion will continue until the CB online session on Wednesday 2020-08-26.

CLI - other

R2-2007989	CR on CLI configuration	LG Electronics Inc.	CR	Rel-16	38.331	16.1.0	1960	-	F	NR_CLI_RIM

- QC thinks this should have been submitted as a revision of the in-principle agreed CR. Interoperabilty should be added and "other specs affected" could be removed.

- Samsung thinks we should discuss whether to have a NBC change or not. For this CR this is fine. Also Ericsson agrees.

- Revised in R2-2008201, putting a reference in the cover page to the IPA CR that was not included in the final CR in June. Remove "other specs affected". Also include the interoperability part

- Discussed in offline 113

R2-2008201	CR on CLI configuration	LG Electronics Inc.	CR	Rel-16	38.331	16.1.0	1960	1	F	NR_CLI_RIM

[AT111e][113][CLI] RRC CR (LG)

Scope: Revise the CR in R2-2007989

Intended outcome: Agreeable CR in R2-2008201

Initial deadline (for companies' feedback): Wednesday 2020-08-26 07:00 UTC

Initial deadline (for final CR): Wednesday 2020-08-26 09:00 UTC

L1enh_URLLC

R2-2007080	PUCCH configuration with subslotLengthForPUCCH-r16	CATT	CR	Rel-16	38.331	16.1.0	1783	-	F	NR_L1enh_URLLC-Core

- Ericsson thinks this should be captured in RAN1 specs. Huawei thinks this should be captured somewhere and this is not in RAN1 spec at the moment. ZTE thinks the principle is ok but would like to have more time to check where the description should go

- CATT suggests that the topic for the offline could be to discuss the wording for a possible RRC CR *IF* RAN1 will decide that this will not be described in RAN1 specs.

- Continue in offline 114

R2-2007862	Converting suffix ForDCI-Formatx-y for shorter RRC parameter names	Huawei, HiSilicon	CR	Rel-16	38.331	16.1.0	1937	-	F	NR_L1enh_URLLC-Core

- Nokia thinks this a good idea. QC thinks that in the main session it was agreed to do this and also inform RAN1. Ericsson is fine with this.

- Ok with the principle but the CR needs revision.

- Revised in R2-2008203

- Continue in offline 114

R2-2008203	Converting suffix ForDCI-Formatx-y for shorter RRC parameter names	Huawei, HiSilicon	CR	Rel-16	38.331	16.1.0	1937	1	F	NR_L1enh_URLLC-Core

[AT111e][114][L1enh_URLLC] RRC CRs (CATT)

Scope: discuss the TP for a possible revision of R2-2007080 and revise R2-2007862

Intended outcome: Agreeable TP for a possible revision of R2-2007080 in R2-2008202

and agreeable CR in R2-2008203

Initial deadline (for companies' feedback): Wednesday 2020-08-26 07:00 UTC

Initial deadline (for TP in R2-2008202 and CR in R2-2008203): Wednesday 2020-08-26 09:00 UTC

R2-2008202	Summary of offline 113 - TP for PUCCH configuration with subslotLengthForPUCCH-r16	CATT	discussion	NR_L1enh_URLLC-Core

## 8.10	NR Non-Terrestrial Networks (NTN)

(NR_NTN_solutions-Core; leading WG: RAN2; REL-17; WID: RP-201256)

Time budget: 2 TU

Tdoc Limitation: 4 tdocs

Email max expectation: 4 threads

### 8.10.1	Scope, requirements, scenarios, architecture

E.g. understand the WID, confirm the scenarios that shall be addressed, the role of and architecture for Location Service.

Workplan

R2-2007565	NR_NTN_solutions work plan	THALES	Work Plan	Rel-17

- Revised in R2-2008186 based on the outcome of offline 105

R2-2008186	NR_NTN_solutions work plan	THALES	Work Plan	Rel-17

…

R2-2007431	Discussion on NTN workplan	CMCC	discussion	Rel-17	NR_NTN_solutions-Core

Incoming LSs

R2-2006514	Response LS on the “LS out on Location of UEs and associated key issues” (R3-202824; contact: Thales)	RAN3	LS in	Rel-17	FS_5GSAT_ARCH	To:SA2, RAN2, SA3-LI

- Noted

R2-2006532	Response LS on the “LS OUT on Location of UEs and associated key issues” (S3i200056; contact: Rogers)	SA3-LI	LS in	Rel-17	FS_5GSAT_ARCH	To:SA2, RAN2, RAN3	Cc:SA1

- Noted

R2-2006530	LS on SA WG2 assumptions from conclusion of study on architecture aspects for using satellite access in 5G (S2-2004688; contact: Qualcomm)	SA2	LS in	Rel-17	FS_5GSAT_ARCH	To:RAN2, RAN3, CT1

…

R2-2006971	Discussion of SA2 LS on fixed cell identity	Qualcomm Inc	discussion	Rel-17	NR_NTN_solutions-Core

…

R2-2006972	[Draft] LS Reply on SA WG2 assumptions on architecture aspects for using	Qualcomm Inc	LS out	Rel-17	NR_NTN_solutions-Core	To:SA2	Cc:RAN3, CT1

…

Scope & scenarios

R2-2007572	NR NTN Reference scenarios definition for Rel-17 normative phase	THALES	discussion	Rel-17

- Discussed in offline 105

R2-2007537	NTN scope, scenarios, architecture, and requirements	Ericsson	discussion	Rel-17	NR_NTN_solutions-Core

- Discussed in offline 105

R2-2006630	Further Clarifications on the NTN WID	CATT	discussion	Rel-17	NR_NTN_solutions-Core

- Discussed in offline 105

[AT111e][105][NTN] Workplan, scope and scenarios (Thales)

Scope: Discuss the workplan in R2-2007565 and the proposals in R2-2007572, R2-2007537,  R2-2006630 (and possibly others from contributions in 8.10.1)

Initial intended outcome: revised workplan and summary of the offline discussion with e.g.:

    - List of agreeable proposals (if any)

    - List of proposals that require online discussions

Initial deadline (for companies' feedback): Thursday 2020-08-20 16:00 UTC

Initial deadline (for rapporteur's summary in R2-2008185):  Thursday 2020-08-20 18:00 UTC

R2-2008185	Summary of offline 105 - NTN Workplan, scope and scenarios	Thales	discussion	Rel-16	NR_NTN_solutions-Core

R2-2006941	NTN WI- Overall Observations and Proposals  	SAMSUNG	discussion	Rel-17	NR_NTN_solutions

R2-2007143	Discussion on task prioritization for NR NTN	Huawei, HiSilicon	discussion	Rel-17	NR_NTN_solutions-Core

R2-2007363	On the scenarios and simulation assumptions for evaluating NTN mobility	Nokia, Nokia Shanghai Bell	discussion	Rel-17	NR_NTN_solutions-Core

Positioning

R2-2006699	NR-NTN: Positioning Methods	Fraunhofer IIS, Fraunhofer HHI	discussion	Rel-17	38.821

R2-2007185	Location Services in NTN	Sony	discussion	Rel-17	NR_NTN_solutions-Core

### 8.10.2	User Plane

In particular, initial focus on getting a common understanding of pre-compensation and offsets.

#### 8.10.2.1	MAC aspects

R2-2007615	Summary of MAC open issues in NTN	InterDigital	discussion	Rel-17	NR_NTN_solutions-Core

- Discussed in offline 107

R2-2007616	Pre-compensation and offset calculation in NTN	InterDigital	discussion	Rel-17	NR_NTN_solutions-Core

- Discussed in offline 107

R2-2006928	Timing advance for NTN	Intel Corporation	discussion	Rel-17	NR_NTN_solutions-Core

- Discussed in offline 107

R2-2007590	Timing Advance, Random Access and DRX aspects in NTN	Nokia, Nokia Shanghai Bell	discussion	Rel-17	NR_NTN_solutions-Core

- Discussed in offline 107

R2-2007784	Consideration on MAC enhancements for NTN	ZTE Corporation, Sanechips	discussion	Rel-17

- Proposals 1 to 5 discussed in offline 107

[AT111e][107][NTN] Pre-compensation and other MAC issues (Interdigital)

Scope: Discuss the proposals in R2-2007615, R2-2007616, R2-2006928, R2-2007590 (and possibly other proposals from contributions in 8.10.2.1 focussing on pre-compensation and offset calculations), as well as proposals 1 to 5 in R2-2007784. The intention is to identify design alternatives, collect company views and, whenever possible, also narrow down the proposals.

Initial intended outcome: summary of the offline discussion with e.g.:

    - List of agreeable proposals (if any)

    - List of proposals that require online discussions

Initial deadline (for companies' feedback): Friday 2020-08-21 08:00 UTC

Initial deadline (for rapporteur's summary in R2-2008188):  Friday 2020-08-21 10:00 UTC

R2-2008188	Summary of offline 107 - Pre-compensation and other MAC issues	Interdigital	discussion	Rel-16	NR_NTN_solutions-Core

R2-2006631	Discussion on MAC Enhancement and Impact for NTN	CATT	discussion	Rel-17	NR_NTN_solutions-Core

R2-2006638	On Updating MAC Timers in NR-NTN	MediaTek Inc.	discussion

R2-2006702	Enhancements for NTN on MAC Layer – Impact Analysis on TS	Nomor Research GmbH, Thales	discussion	Rel-17

R2-2006781	Consideration on MAC enhancement for NTN	OPPO	discussion	Rel-17	NR_NTN_solutions-Core

R2-2006799	Discussion on DRX and BSR in NTN	PANASONIC R&D Center Germany	discussion

R2-2006927	MAC issues for NTN	Intel Corporation	discussion	Rel-17	NR_NTN_solutions-Core

R2-2006943	MAC User Plane Enhancements for an NTN- Observations and Proposals	SAMSUNG	discussion	Rel-17	NR_NTN_solutions

R2-2006974	UP aspects including Random Access procedure enhancements	Qualcomm Inc	discussion	Rel-17	NR_NTN_solutions-Core

R2-2007056	Introducing offsets in MAC	Spreadtrum Communications	discussion

R2-2007103	On Timing Advance for NTN Networks	Apple	discussion	Rel-17	NR_NTN_solutions-Core

R2-2007104	On Preamble Ambiguity in NTN Networks	Apple	discussion	Rel-17	NR_NTN_solutions-Core

R2-2007105	On User Plane Latency reduction mechanisms in NTN Networks	Apple	discussion	Rel-17	NR_NTN_solutions-Core

R2-2007176	Discussion on UL scheduling enhancement	Beijing Xiaomi Electronics	discussion

R2-2007186	MAC enhancements in NTN	Sony	discussion	Rel-17	NR_NTN_solutions-Core

R2-2007397	Consideration on TA Precompensation	Beijing Xiaomi Mobile Software	discussion	Rel-17

R2-2007428	Discussion of HARQ feedback for NTN	CMCC	discussion	Rel-17	NR_NTN_solutions-Core

R2-2007430	Discussion on TA compensation	CMCC	discussion	Rel-17	NR_NTN_solutions-Core

R2-2007474	Timing advance pre-compensation in NTN	Lenovo, Motorola Mobility	discussion	Rel-17

R2-2007477	Discussion on DRX for NTN	Lenovo, Motorola Mobility	discussion	Rel-17

R2-2007617	RACH preamble ambiguity in NTN	InterDigital	discussion	Rel-17	NR_NTN_solutions-Core

R2-2007712	Impact of pre-compensation on RACH capacity for NTN	NEC Telecom MODUS Ltd.	discussion	Rel-17

R2-2007714	On scheduling, HARQ, DRX, RLC, and PDCP for NTN	Ericsson	discussion	Rel-17	NR_NTN_solutions

R2-2007715	On Random Access in NTN	Ericsson	discussion	Rel-17	NR_NTN_solutions

R2-2007888	Discussion on MAC aspects for NTN	LG Electronics Inc.	discussion	Rel-17	NR_NTN_solutions-Core

R2-2007995	MAC enhancements on the initial access procedures for NTN	ETRI	discussion	Rel-17

R2-2008101	Considerations on RACH procedure enhancements in NTN	CAICT	discussion	Late

Withdrawn

R2-2007519	Impact of pre-compensation on RACH capacity for NTN	NEC Telecom MODUS Ltd.	agenda	Withdrawn

#### 8.10.2.2	Other aspects

R2-2006640	RLC and PDCP Enhancements in NR-NTN	MediaTek Inc.	discussion

R2-2006703	Enhancements for NTN on RLC Control Loops and Timers	Nomor Research GmbH, Thales	discussion	Rel-17

R2-2006705	Enhancements for NTN on PDCP Control Loops and Timers	Nomor Research GmbH, Thales	discussion	Rel-17

R2-2006782	Consideration on RLC and PDCP enhancements for NTN	OPPO	discussion	Rel-17	NR_NTN_solutions-Core

R2-2007172	Discussion on UP enhancement in NTN	Huawei, HiSilicon	discussion	Rel-17	NR_NTN_solutions-Core

R2-2007573	On NTN Feeder link switch over	THALES	discussion

R2-2007785	Consideration on UP timers and RLC/PDCP SN for NTN	ZTE Corporation, Sanechips	discussion	Rel-17

R2-2007889	Discussion on RLC and PDCP aspects for NTN	LG Electronics Inc.	discussion	Rel-17	NR_NTN_solutions-Core

### 8.10.3	Control Plane

Also identify things not covered in the TR that need to be covered, if any.

#### 8.10.3.1	Idle/Inactive mode

Including cell selection/reselection & system information.

R2-2006872	Consideration on system information and cell (re)selection in NTN	ZTE corporation, Sanechips	discussion	Rel-17	NR_NTN_solutions-Core

- Discussed in offline 106

R2-2006973	IDLE mode procedure	Qualcomm Inc	discussion	Rel-17	NR_NTN_solutions-Core

- Discussed in offline 106

R2-2007171	Discussion on RRC_IDLE mode issues in NTN	Huawei, HiSilicon	discussion	Rel-17	NR_NTN_solutions-Core

- Discussed in offline 106

R2-2007574	Considerations on satellite ephemeris	THALES	discussion	Rel-17

- Proposals 1 and 2 discussed in offline 106

[AT111e][106][NTN] Idle mode issues (ZTE)

Scope: Discuss the proposals in R2-2006872, R2-2006973, R2-2007171 and proposals 1 and 2 in R2-2007574. The intention is to identify design alternatives, collect company views and, whenever possible, also narrow down the proposals.

Initial intended outcome: summary of the offline discussion with e.g.:

    - List of agreeable proposals (if any)

    - List of proposals that require online discussions

Initial deadline (for companies' feedback): Thursday 2020-08-20 16:00 UTC

Initial deadline (for rapporteur's summary in R2-2008187):  Thursday 2020-08-20 18:00 UTC

R2-2008187	Summary of offline 106 - Idle mode issues	ZTE corporation	discussion	Rel-16	NR_NTN_solutions-Core

R2-2006628	 Initial Discussion for Idle and Inactive Mode in NTN	CATT	discussion	Rel-17	NR_NTN_solutions-Core

R2-2006642	On Idle Mode Procedures in NR-NTN	MediaTek Inc.	discussion

R2-2006783	Discussion on cell reselection for NTN	OPPO	discussion	Rel-17	NR_NTN_solutions-Core

R2-2006821	Issues of the Fixed Tracking Area in NTN	PANASONIC R&D Center Germany	discussion

R2-2006924	HAPS-Satellite ephemeris broadcast	Loon	discussion	Rel-17

R2-2006925	HAPS-Terrestrial PCI confusion mitigation	Loon and Google	discussion	Rel-17

R2-2006929	Tracking area issue for NTN	Intel Corporation	discussion	Rel-17	NR_NTN_solutions-Core

R2-2006945	Control Plane Enhancements for Idle and Inactive Modes in an NTN- Overall Observations and Proposals	SAMSUNG	discussion	Rel-17	NR_NTN_solutions

R2-2007048	Consideration on Celll Reselection evaluation in NTN	Spreadtrum Communications	discussion

R2-2007175	Control Plane for Idle/Inactive mode UE	Beijing Xiaomi Electronics	discussion

R2-2007184	Idle mode enhancement in NTN	Sony	discussion	Rel-17	NR_NTN_solutions-Core

R2-2007251	Ephemeris data to be included in system information	ITRI	discussion	NR_NTN_solutions-Core

R2-2007362	On Tracking Areas and IDLE mode handling for NTN	Nokia, Nokia Shanghai Bell	discussion	Rel-17	NR_NTN_solutions-Core

R2-2007429	Discussion of cell selection and reselection for NTN	CMCC	discussion	Rel-17	NR_NTN_solutions-Core

R2-2007473	Ephemeris data provision in NTN	Lenovo, Motorola Mobility	discussion	Rel-17

R2-2007558	Idle mode aspects for NTN	Ericsson	discussion	Rel-17	NR_NTN_solutions-Core

R2-2007743	Initial discussion on Idle mode procedures in NR NTN	LG Electronics France	discussion	Rel-17

#### 8.10.3.2	Connected mode

Including mobility management.

R2-2006930	mobility enhacement for NTN	Intel Corporation	discussion	Rel-17	NR_NTN_solutions-Core

R2-2006629	 Initial Discussion for Connected Mode in NTN	CATT	discussion	Rel-17	NR_NTN_solutions-Core

R2-2006643	On Connected Mode Mobility Procedures in NR-NTN	MediaTek Inc.	discussion

R2-2006784	Discussion on mobility management for connected mode UE in NTN	OPPO	discussion	Rel-17	NR_NTN_solutions-Core

R2-2006822	Overhead Reduction for the Handover Procedure in NTN	PANASONIC R&D Center Germany	discussion

R2-2006873	Consideration on mobility enhancement in NTN	ZTE corporation, Sanechips	discussion	Rel-17	NR_NTN_solutions-Core

R2-2006953	Control Plane Enhancements for the Connected Mode in an NTN- Overall Observations and Proposals  	SAMSUNG	discussion	Rel-17	NR_NTN_solutions

R2-2006975	Connected mode mobility enhancements	Qualcomm Inc	discussion	Rel-17	NR_NTN_solutions-Core

R2-2007144	Discussion on enhancements for connected mode in NTN	Huawei, HiSilicon	discussion	Rel-17	NR_NTN_solutions-Core

R2-2007174	Control Plane for Connected mode UE	Beijing Xiaomi Electronics	discussion

R2-2007183	Mobility management in NTN	Sony	discussion	Rel-17	NR_NTN_solutions-Core

R2-2007463	Mobility management in NTN	Lenovo, Motorola Mobility	discussion	Rel-17

R2-2007601	Adjusting timers according to delay variations in NTN	Nokia, Nokia Shanghai Bell	discussion	Rel-17	NR_NTN_solutions-Core

R2-2007618	Location-assisted connected mobility in NTN	InterDigital	discussion	Rel-17	NR_NTN_solutions-Core

R2-2007744	Initial discussion on connected mobility in NR NTN	LG Electronics France	discussion	Rel-17	NR_NTN_solutions-Core

R2-2007955	Discussion on delay difference on measurements for NTN	Asia Pacific Telecom co. Ltd	discussion	NR_NTN_solutions-Core

Late

R2-2006547	Discussion on feeder link hard switch in NTN LEO	CENC	discussion	Late

R2-2006552	Feeder link hard switch triggered HO	CENC	discussion	Late

R2-2006553	Gateway data handling in NTN LEO	CENC	discussion	Late

## 8.12	Reduced Capability SI

(FS_NR_redcap; leading WG: RAN1; REL-17; WID: RP-201386)

Time budget: 2 TU

Tdoc Limitation: 4 tdocs

Email max expectation: 4 threads

### 8.12.1	Organizational and scope

Get a common understanding of the SID, eg. what is RAN2 scope in the RAN1 centric objectives, what is required to be in the TR in order to start a WI.

R2-2006910	Scope of RedCap SI	Ericsson	discussion	FS_NR_redcap

- Discussed in offline 108

R2-2007366	TR38.875 skeleton updates for Study on support of reduced capability NR devices	Ericsson	discussion

- Revised in R2-2008190 based on the outcome of offline 108

R2-2008190	TR38.875 skeleton updates for Study on support of reduced capability NR devices	Ericsson	discussion

[AT111e][108][REDCAP] Scope and skeleton update (Ericsson)

Scope: Discuss the SI scope in R2-2006910 and the skeleton update in R2-2007366

Initial intended outcome: summary of the offline discussion with e.g.:

    - List of agreeable proposals (if any)

    - List of proposals that require online discussions

and skeleton update

Initial deadline (for companies' feedback): Monday 2020-08-24 16:00 UTC

Initial deadline (for rapporteur's summary in R2-2008189):  Monday 2020-08-24 18:00 UTC

R2-2008189	Summary of offline 108 - RedCap scope and skeleton update	Ericsson	discussion	Rel-16	FS_NR_redcap

R2-2006732	General views on Higher-layer impacts for Redcap devices	Xiaomi Communications	discussion

R2-2006753	RAN1-2 work scope discussion on RedCap capability	Intel Corporation	discussion	Rel-17	FS_NR_redcap

R2-2006978	Expected RAN2 scope of RedCap	NEC	discussion	Rel-17	FS_NR_redcap

### 8.12.2	Framework for reduced capabilities

#### 8.12.2.1	Principles for how to define and constrain reduced capabilities

R2-2006751	Reduced capability signalling framework	Intel Corporation	discussion	Rel-17	FS_NR_redcap

- Discussed in offline 109

R2-2006911	Framework and Principles for Reduced Capability	Ericsson	discussion	FS_NR_redcap

- Discussed in offline 109

R2-2006605	Defining and constraining UEs with reduced capabilities	Qualcomm Inc	discussion	Rel-17	FS_NR_redcap

- Discussed in offline 109

[AT111e][109][REDCAP] Reduced capability signalling framework (Intel)

Scope: Discuss the proposals in R2-2006751, R2-2006911 and R2-2006605. The intention is to identify design alternatives, collect company views and, whenever possible, also narrow down the proposals.

Initial intended outcome: summary of the offline discussion with e.g.:

    - List of agreeable proposals (if any)

    - List of proposals that require online discussions

Initial deadline (for companies' feedback): Monday 2020-08-24 22:00 UTC

Initial deadline (for rapporteur's summary in R2-2008191):  Tuesday 2020-08-25 02:00 UTC

R2-2008191	Summary of offline 109 - Reduced capability signalling framework	Intel	discussion	Rel-16	FS_NR_redcap

R2-2006660	Capability and initial access of RedCap UEs	Samsung	discussion	Rel-17	FS_NR_redcap

R2-2006691	UE type and capability for RedCap UEs	vivo, Guangdong Genius	discussion	Rel-17	FS_NR_redcap

R2-2006733	Discussion on UE Capaiblity Issues for reduced capability NR devices	Xiaomi Communications	discussion

R2-2006785	Discussion on definition of RedCap Ues	OPPO	discussion	Rel-17	FS_NR_redcap

R2-2006903	Define and constrain reduced capability	ZTE Corporation, Sanechips	discussion	Rel-17	FS_NR_redcap

R2-2007011	On definition and constraint of reduced capabilities	CATT	discussion	Rel-17	FS_NR_redcap

R2-2007110	RedCap UE characterization and access restriction	Apple	discussion	Rel-17	FS_NR_redcap

R2-2007344	Capability definition of REDCAP UE	Huawei, HiSilicon	discussion	Rel-17	FS_NR_redcap

R2-2007400	Discussion on how to define reduced capability devices	LG Electronics UK	discussion	Rel-17

R2-2007478	The principle to constrain reduced capability NR devices	Lenovo, Motorola Mobility	discussion	Rel-17

R2-2007490	Principles for reduced capabilities	Nokia, Nokia Shanghai Bell	discussion	Rel-17	FS_NR_redcap

R2-2007492	On the definition of a RedCap device type	MediaTek Inc.	discussion	Rel-17	FS_NR_redcap

#### 8.12.2.2	Identification and access restrictions

R2-2007345	Identification and access restriction of REDCAP UE	Huawei, HiSilicon	discussion	Rel-17	FS_NR_redcap

- Discussed in offline 110

R2-2006661	Coexistence between legacy UEs and RedCap UEs	Samsung	discussion	Rel-17	FS_NR_redcap

- Discussed in offline 110

R2-2006786	Discussion on RedCap UE’s identification and access control	OPPO	discussion	Rel-17	FS_NR_redcap

- Discussed in offline 110

R2-2007493	On UE identification and access restrictions	MediaTek Inc.	discussion	Rel-17	FS_NR_redcap

- Discussed in offline 110

[AT111e][110][REDCAP] Identification and access restriction (Huawei)

Scope: Discuss the proposals in R2-2007345, R2-2006661, R2-2006786 and R2-2007493. The intention is to identify design alternatives, collect company views and, whenever possible, also narrow down the proposals.

Initial intended outcome: summary of the offline discussion with e.g.:

    - List of agreeable proposals (if any)

    - List of proposals that require online discussions

Initial deadline (for companies' feedback): Monday 2020-08-24 22:00 UTC

Initial deadline (for rapporteur's summary in R2-2008192):  Tuesday 2020-08-25 02:00 UTC

R2-2008192	Summary of offline 110 - Identification and access restriction	Huawei	discussion	Rel-16	FS_NR_redcap

R2-2006606	Identification and access restriction for RedCap UEs	Qualcomm Inc	discussion	Rel-17	FS_NR_redcap

R2-2006692	Identification and access restrictions for RedCap UEs	vivo, Guangdong Genius	discussion	Rel-17	FS_NR_redcap

R2-2006734	Discussion on Identification and UE access restrictions for Redcap devices	Xiaomi Communications	discussion

R2-2006752	Identification and Access restriction for RedCap devices	Intel Corporation	discussion	Rel-17	FS_NR_redcap

R2-2006904	Identification and access control for Redcap UE	ZTE Corporation, Sanechips	discussion	Rel-17	FS_NR_redcap

R2-2006912	Identification and access restriction for devices with reduced capabilities	Ericsson	discussion	FS_NR_redcap

R2-2006979	Constraint on usage of RedCap functions	NEC	discussion	Rel-17	FS_NR_redcap

R2-2007012	Identification and access restrictions for reduced capability UE	CATT	discussion	Rel-17	FS_NR_redcap

R2-2007399	Access restriction for reduced capability devices	LG Electronics UK	discussion	Rel-17

R2-2007480	Discussion on the identification of Redcap	Lenovo, Motorola Mobility	discussion	Rel-17

R2-2007491	Cell access for REDCAP UE with reduced bandwidth	Nokia, Nokia Shanghai Bell	discussion	Rel-17	FS_NR_redcap

R2-2007560	Cell access restrictions for REDCAP UE	Nokia, Nokia Shanghai Bell	discussion	Rel-17	FS_NR_redcap

### 8.12.3	UE power saving and battery lifetime enhancement

UE power saving and battery lifetime enhancement for reduced capability UEs in applicable use cases (e.g. delay tolerant case).

DRX

R2-2007013	eDRX for NR RRC Inactive and Idle States	CATT	discussion	Rel-17	FS_NR_redcap

- Discussed in offline 111

R2-2007346	Discussion on eDRX for RRC_INACTIVE and RRC_IDLE	Huawei, HiSilicon	discussion	Rel-17	FS_NR_redcap

- Discussed in offline 111

R2-2007494	eDRX for reduced capability UEs	MediaTek Inc.	discussion	Rel-17	FS_NR_redcap

- Discussed in offline 111

R2-2006748	Use cases target to extend paging DRX cycle and relax measurements for stationary devices	Intel Corporation	discussion	Rel-17	FS_NR_redcap

- Proposals 1 to 4 discussed in offline 111

[AT111e][111][REDCAP] DRX aspects (CATT)

Scope: Discuss the proposals in R2-2007013, R2-2007346, R2-2007494 as well as proposals 1 to 4 in R2-2006748. The intention is to identify design alternatives, collect company views and, whenever possible, also narrow down the proposals.

Initial intended outcome: summary of the offline discussion with e.g.:

    - List of agreeable proposals (if any)

    - List of proposals that require online discussions

Initial deadline (for companies' feedback): Monday 2020-08-24 22:00 UTC

Initial deadline (for rapporteur's summary in R2-2008193):  Tuesday 2020-08-25 02:00 UTC

R2-2008193	Summary of offline 111 - DRX aspects	CATT	discussion	Rel-16	FS_NR_redcap

R2-2006607	Power saving enhancements for RedCap UEs	Qualcomm Inc	discussion	Rel-17	FS_NR_redcap

R2-2006694	DRX enhancement for RedCap UEs	vivo, Guangdong Genius	discussion	Rel-17	FS_NR_redcap

R2-2006731	Discussion on UE Power saving for Redcap Devices	Xiaomi Communications	discussion

R2-2006787	Consideration on extended DRX for RedCap	OPPO	discussion	Rel-17	FS_NR_redcap

R2-2006905	Introduction of eDRX for Redcap UE	ZTE Corporation, Sanechips	discussion	Rel-17	FS_NR_redcap

R2-2007111	Impact of power-saving aspects on RedCap UEs	Apple	discussion	Rel-17	FS_NR_redcap

R2-2007346	Discussion on eDRX for RRC_INACTIVE and RRC_IDLE	Huawei, HiSilicon	discussion	Rel-17	FS_NR_redcap

R2-2007393	Introducing Extended DRX for RRC Inactive and/or Idle	Samsung	discussion	FS_NR_redcap

R2-2007401	Extended DRX for reduced capability devices in RRC_IDLE and RRC_INACTIVE	LG Electronics UK	discussion	Rel-17

R2-2007470	eDRX for Idel/inactive-mode UE with reduced capability	Lenovo, Motorola Mobility	discussion	Rel-17

R2-2007561	Power saving and battery lifetime enhancement for REDCAP UE	Nokia, Nokia Shanghai Bell	discussion	Rel-17	FS_NR_redcap

R2-2007653	eDRX for Reduced Capability NR Devices 	Convida Wireless	discussion	Rel-17	FS_NR_redcap

R2-2007654	Discussion on eDRX Configuration	Convida Wireless	discussion	Rel-17	FS_NR_redcap

RRM relaxation

R2-2006913	Reducing power consumption in RedCap devices	Ericsson	discussion	FS_NR_redcap

- Revised in R2-2008130

R2-2008130	Reducing power consumption in RedCap devices	Ericsson	discussion	FS_NR_redcap

Observations 3-8 and proposal 3

…

R2-2007347	RRM measurement relaxation for REDCAP UE	Huawei, HiSilicon	discussion	Rel-17	FS_NR_redcap

R2-2006902	Consideration on RRM relaxation for Redcap UE	ZTE Corporation, Sanechips	discussion	Rel-17	FS_NR_redcap

R2-2006788	Discussion on RRM relaxation	OPPO	discussion	Rel-17	FS_NR_redcap

R2-2006662	RRM relaxation for stationary devices	Samsung	discussion	Rel-17	FS_NR_redcap

R2-2006693	RRM relaxation for power saving	vivo, Guangdong Genius	discussion	Rel-17	FS_NR_redcap

R2-2007471	RRM relaxation for stationary UE with reduced capability	Lenovo, Motorola Mobility	discussion	Rel-17

R2-2007745	Considerations on RRM for reduced capability UEs	LG Electronics France	discussion	Rel-17	FS_NR_redcap

## Summary

TBD