3GPP TSG-RAN WG2 Meeting #111 electronic	R2-2008202

Online, August 17th– 28th, 2020

Source:	CATT

Title:	Summary of offline 114 - TP for PUCCH configuration with subslotLengthForPUCCH-r16

Agenda Item:	6.14.2

Document for:	Discussion and Decision

# Introduction

This contribution provides a summary of the following offline:

[AT111e][114][L1enh_URLLC] RRC CRs (CATT)

Scope: discuss the TP for a possible revision of R2-2007080 and revise R2-2007862

Intended outcome: Agreeable TP for a possible revision of R2-2007080 in R2-2008202

and agreeable CR in R2-2008203

Initial deadline (for companies' feedback): Wednesday 2020-08-26 07:00 UTC

Initial deadline (for TP in R2-2008202 and CR in R2-2008203): Wednesday 2020-08-26 09:00 UTC

# Discussion

# TP for a possible revision of R2-2007080 [1]

As detailed in [1] we have the following configuration restrictions when subslotLengthForPUCCH-r16 is configured in a PUCCH-Config, which are currently captured nowhere:

- PUCCH resource(s) for HARQ-ACK feedback within the PUCCH-Config should be configured within a sub-slot (not across sub-slot boundaries).

- PUCCH resource(s) for SR/CSI with the same priority as the PUCCH-Config should be configured within the sub-slot associated with this PUCCH-Config.

- The determination of priority of HARQ-ACK, SR, CSI and PUCCH-Config are specified in 38.213, clause 9.

The related on-line discussion in RAN2 was as follows:

R2-2007080	PUCCH configuration with subslotLengthForPUCCH-r16	CATT	CR	Rel-16	38.331	16.1.0	1783	-	F	NR_L1enh_URLLC-Core

- Ericsson thinks this should be captured in RAN1 specs. Huawei thinks this should be captured somewhere and this is not in RAN1 spec at the moment. ZTE thinks the principle is ok but would like to have more time to check where the description should go

- CATT suggests that the topic for the offline could be to discuss the wording for a possible RRC CR *IF* RAN1 will decide that this will not be described in RAN1 specs.

- Continue in offline 114

The CR in R2-2007080 also addresses a typo in the 38.213 reference in the field description of subslotLengthForPUCCH:

The parameter subslotLengthForPUCCH is specified in 38.213, clause 9 instead of clause 9.1.

In the meantime, RAN1 discussed potential capture of the above in RAN1 specifications (offline [102-e-NR-L1enh-URLLC-UCI_Enh-02] Misc corrections/clarifications) and just agreed in GTW session:

R1-2007054 Summary#1 of email thread [102-e-NR-L1enh_URLLC-UCI_Enh-02] Moderator (OPPO) […] Agreement The following text proposal is endorsed as part of editor’s CR on TS38.213. --------------------------------- Start of Proposed Text 1 for TS 38.213 ------------------------------- 9 UE procedure for reporting control information A PUSCH or a PUCCH transmission, including repetitions if any, can be of priority index 0 or of priority index 1. For a configured grant PUSCH transmission, a UE determines a priority index from priority, if provided. For a PUCCH transmission with HARQ-ACK information corresponding to a SPS PDSCH reception or a SPS PDSCH release, a UE determines a priority index from harq-CodebookID, if provided. For a PUCCH transmission with SR, a UE determines the corresponding priority as described in subclause 9.2.4. If a priority index is not provided to a UE for a PUSCH or a PUCCH transmission, the priority index is 0. If a PUCCH-Config is provided with a subslotLengthFor PUCCH, the PUCCH resource corresponding to any SR or CSI configuration with the same priority as the PUCCH-Config, should be confined within the sub-slot associated to the PUCCH-Config. --------------------------------- End of Proposed Text 1 for TS 38.213 -------------------------------

Therefore there is no longer a need to capture this in RRC specification, and the CR in [1] reduces to the simple typo fix mentioned above:

subslotLengthForPUCCH Indicate the sub-slot length for sub-slot based PUCCH feedback in number of symbols (see TS 38.213 [13], clause 9.1). Value n2 corresponds to 2 symbols, value n6 corresponding to 6 symbols, value n7 corresponds to 7 symbols. For normal CP, the value is either n2 or n7. For extended CP, the value is either n2 or n6.

Q1: Do you agree with the above typo fix?

| Company | Yes/No | Comments |
| --- | --- | --- |
| CATT | Yes |  |

# Revision of R2-2007862

The related discussions and action point are as below.

R2-2007862	Converting suffix ForDCI-Formatx-y for shorter RRC parameter names	Huawei, HiSilicon	CR	Rel-16	38.331	16.1.0	1937	-	F	NR_L1enh_URLLC-Core

- Nokia thinks this a good idea. QC thinks that in the main session it was agreed to do this and also inform RAN1. Ericsson is fine with this.

- Ok with the principle but the CR needs revision.

- Revised in R2-2008203

- Continue in offline 114

According to the online discussions there may need some changes to the proposed CR, which is the focus of the discussion in the following.

Q2: Do you think any changes are required to the proposed CR in R2-2007862‎?

| Company | Yes/No | Comments (please explain in detail if your answer is yes, or directly in the draft CR in the 114 folder) |
| --- | --- | --- |
| CATT | No |  |

# Conclusions

# Reference

- R2-2007080	 PUCCH configuration with subslotLengthForPUCCH-r16; CATT

- R2-2007862	 Converting suffix ForDCI-Formatx-y for shorter RRC parameter names; Huawei, HiSilicon