3GPP TSG-RAN WG2 Meeting #112 electronic	R2-2009248

e-meeting, 2nd – 13th November 2020

Source: 			ZTE Corporation, Sanechips

Title: 	Consideration on definition and constraining of Reduced Capability

Agenda item:		8.12.2.1

Document for: 	Discussion and Decision

- Introduction

In RAN2#111e meeting, following agreements are reached about the defining and constraining reduced capabilities[1]:

Agreements:

- At least for device type identification and access restriction (including initial access), the network needs to know whether the UE is redCap UE or not. FFS on whether based on explicit or implicit signalling.

- The existing UE capabilities framework is used as baseline to indicate the capabilities of a RedCap UE (this does not imply anything on the reporting of the device type, if the need for a device type will be agreed)

- The number of device types should be minimised, to reduce market fragmentation, and introduced only where essential to control UE accesses and differentiate them from legacy R15/R16 and non-Redcap R17 UEs, (e.g. number of Tx/Rx antennas, maximum supportable BW, etc.). The exact composition of the set of L1 capabilities of the device type can be discussed by RAN1

- Discuss in normative phase on whether to signal (and in case how) a Device type and its associated capabilities (the reduced set of capabilities) is captured in specifications, and whether device type is indicated as part of UE capability;

However it is still not clear how to define RedCap capabilities with current UE capability framework. This contribution provides further consideration on this issue based on the agreements and email discussion after RAN2#111e[2].

- Discussion

In the mail discussion after RAN2@111e[2], majority companies agree that RedCap capabilities can be categorized as mandatory capabilities and optional capabilities. Per our understanding, to have optional capabilities besides mandatory capabilities provides flexibility in market planning and product implementation. Requirement of high end use cases can be covered by more advanced but optional capabilities on top of minimal requirements.Thus from initial access and idle mode procedure point of view, only a limited number of device type, preferably only one device type per FR is needed.

Proposal 1: RedCap capabilities are categorized as mandatory capabilities and optional capabilities.

In the email discussion, a set of scenarios are identified for consideration when design capability signaling and specification:

For the features that are mandatory for non-Redcap UEs, following scenarios are considered:

Case1: The Redcap UE mandatorily supports the feature with the same value;

Case2: The Redcap UE mandatorily supports the feature, but with different value (e.g. bandwidth value);

Case3: The Redcap UE optionally supports the feature;

Case4: The Redcap UE does not support the feature at all;

For the features that are optional for non-Redcap UEs, following scenario is considered:

Case5: The Redcap UE does not support the feature at all;

Besides of cases listed above, three more cases should also be considered

Case 6: the RedCap UE mandatory support the feature;

Case 7: the RedCap UE optionally support the feature with same value;

Case 8: the RedCap UE optionally support the feature with different value;

In the email discussion[2], majority companies think the mandatory features without signaling should be specified clearly in spec. Per our understanding, there are two approaches.

One approach is to list the mandatory capabilities for a RedCap which are different from those for non-RedCap UEs, i.e. list features in case 2 and case 4 (and possibly case 6). Another approach is to simply list all mandatory capabilities explicitly in the spec. This can be done regardless of whether the mandatory capabilities is different or not as those for non-RedCap UE.

Considering the complicated relationship between RedCap UE’s mandatory feature and non-RedCap UE’s mandatory/optional features, we think to list all mandatory features without signaling for RedCap UE is more clear.

Proposal 2: All mandatory features for RedCap UE without signaling are explicitly defined in specification.

In NR, some advanced capability is defined to support bit peak rate use case, such as carrier aggregation/dual connectivity. Some capabilities are defined to support high reliable and low delay use case, such as URLLC, NR-IIoT. It may be meaningful to determine whether these advanced capabilities can be supported by redcap UE.

From prevent market fragmentation point of view, to limit the scope of capabilities will limit number of different UE implementation. And because redcap UE is targeted to design low cost, low complexity and compact device, it seems not all advanced capabilities need to be supported by redcap UE. If the RedCap UE is allowed to support the advanced capability, the scenarios of RedCap UE will be extended if it is allowed by the subscription information (e.g. low-end mobile phone).

Therefore, it is reasonable to have some limitations in capability signaling to prevent redcap UE from indicating some advanced capabilities. FFS which capabilities are not supported by redcap UE.

Proposal 3: Limit the optional capability scope which can be supported by Redcap UE. FFS which capabilities are not supported by Redcap UE.

One solution to limit the capability scope is to simply list or “pick out” the capabilities that are optionally supported for Redcap UE. This solution can be done regardless of these capabilities is mandatory/optional for non-Redcap or not. And all optional capabilities required for Redcap UE can be defined in a container other than current defined containers..

Proposal 4: For all optional capabilities that are required for Redcap UE, introduce signalling fields in an independent container defined specifically for Redcap UE.

- Conclusion

Proposal 1: RedCap capabilities are categorized as mandatory capabilities and optional capabilities.

Proposal 2: All mandatory features for RedCap UE without signaling are explicitly defined in specification.

Proposal 3: Limit the optional capability scope which can be supported by redcap UE.

Proposal 4: For all optional capabilities that are required for Redcap UE, introduce signalling fields in an independent container defined specifically for Redcap UE.

- Reference

- RAN2#111e Chairman notes

- R2-2009004, Report of [Post111-e][913][REDCAP] Definition and constraining of reduced capabilities (Intel)

- 3GPP TS 38.306