3GPP TSG-RAN WG2 Meeting #112-e						R2-20xxxxx

Online, 2-13 November 2020

Source: 	Session Chair (MediaTek)

AI:		10.5

Title:		Report from session on positioning and sidelink relay

# Status of At-Meeting Email Discussions

This subclause is not an Agenda Item. It contains a running summary of the email discussions assigned to take place during the meeting weeks.  This section will be moved to an appendix in the final version of the report.

[AT112-e][600][POS][Relay] Organisational Nathan – Positioning/Relay (MediaTek)

Scope: Organisational discussions and announcements, as needed throughout the meeting weeks

Intended outcome: Well-informed participants

Deadline: Friday 2020-11-13 1000 UTC

[AT112-e][601][Relay] Status update to SA2 (OPPO)

Scope: Generate a summary of RAN2 status on relaying for SA2

- Report status of both L2 and L3 relaying designs as well as architecture-independent aspects (including issues in R2-2008760), in order to coordinate with SA2 for reaching conclusions

- Capture any points where we assume SA2 will resolve an issue

Intended outcome: Approvable LS in R2-2010862

Deadline: Friday 2020-11-13 0000 UTC

[AT112-e][602][POS] Rel-16 positioning UE capabilities in RRC (Intel)

Scope: Accommodate any needed updates to the capabilities, taking into account updates to the feature lists during the meeting.

Intended outcome: Endorsable CR for merge into the capabilities mega CR

Deadline:  Friday 2020-11-13 0000 UTC

[AT112-e][603][POS] Positioning stage 2 corrections (Nokia)

Scope: Conclude on the remaining proposals from R2-2010674: P2/P3/P4/P6/P7/P9/P10/P11/P12.

Intended outcome: Agreeable CR in R2-2010863

Deadline:  Tuesday 2020-11-10 1200 UTC

[AT112-e][604][POS] Positioning RRC proposals (Ericsson)

Scope: Discuss and resolve proposals 1 and 2 from R2-2010709.

Intended outcome: Agreeable CR in R2-2010864

Deadline:  Tuesday 2020-11-10 1200 UTC

[AT112-e][605][POS] LPP proposals (Qualcomm)

Scope: Discuss and resolve the remaining proposals from R2-2010975: P1-P5, P7, P8.

Intended outcome: Agreeable CR in R2-2010865

Deadline:  Tuesday 2020-11-10 1200 UTC

[AT112-e][606][POS] LS to RAN1 on positioning latency (Intel)

Scope: Summarise the latency results and draft an LS to RAN1.  Clarify that the attached results are a checkpoint that has not yet been endorsed as a TP.

Intended outcome: Summary of latency results (R2-2010866) and agreeable LS (R2-2010867) with the summary attached

Deadline:  Thursday 2020-11-05 1200 UTC

[AT112-e][607][POS] Gathering of latency enhancement solutions (CATT)

Scope: Describe and discuss the proposed latency enhancements in a format suitable for developing into a TP.

Intended outcome: Text proposal in R2-2010868

Deadline:  Friday 2020-11-13 0000 UTC

[AT112-e][610][Relay] RRC states for L2 relay (vivo)

Scope: Discuss P6-P9 of R2-2011004.

Intended outcome: Summary in R2-2010869

Deadline:  Friday 2020-11-13 0000 UTC

[AT112-e][611][Relay] Open issues on L2 relay (Huawei)

Scope: Discuss the remaining open issues on L2 relay architecture, including:

- PC5 adaptation layer

- RRC procedures (including paging)

- Remaining issues from email discussion [627]

- Remaining open items in the current TR

Intended outcome: Summary in R2-2010870

Deadline:  Wednesday 2020-11-11 1200 UTC

[AT112-e][612][Relay] Open issues on L3 relay (Qualcomm)

Scope: Discuss the remaining open issues on L3 relay architecture, including:

- NAS transport

- Overhead

- QoS

- RRC states

- Remaining open items in the current TR

Intended outcome: Summary in R2-2010871

Deadline:  Wednesday 2020-11-11 1200 UTC

# 4	EUTRA corrections Rel-15 and earlier

See Appendix A for reference to Work items, work item codes and WIDs.

Only essential corrections. No documents should be submitted to 4. Please submit to 4.x

## 4.4	Positioning corrections Rel-15 and earlier

Documents in this agenda item will be handled by email.  No web conference is planned for this agenda item.

# 5	Rel-15 WI: New Radio (NR) Access Technology

(NR_newRAT-Core; leading WG: RAN1; REL-15; started: Mar. 17; closed: Jun. 19: WID: RP-191971)

Only essential corrections

## 5.5	Positioning corrections

Corrections to both the stage 2 and stage 3 aspects related to positioning. Stage 2 CRs should be discussed with the specification rapporteur before submission.

Documents in this agenda item will be handled in a break out session.

R2-2010138	Corrections to E-CID positioning	Nokia, Nokia Shanghai Bell	CR	Rel-15	38.305	15.6.0	0042	-	F	NR_newRAT-Core

There is no Rel-16 shadow because the affected text is different.

- Agreed

R2-2010274	Correction on OTDOA Positioning support in R15	Huawei, HiSilicon	CR	Rel-16	38.305	16.2.0	0047	-	F	NR_newRAT-Core

Qualcomm are not sure the CR is correct; there is no XnAP signalling for OTDOA, but the text only talks about “signalling access”, and they understand that for option 4 the sentence is correct as written.  Huawei understand that if there is no signalling access between the LMF and ng-eNB, there is no way for the gNB to connect the ng-eNB to the LMF.  Qualcomm think there is generic support on Xn for transport of control plane signalling.

Nokia think in a generic sense such signalling may be possible, but it may not be currently used for any positioning method.

Ericsson would like some extra time to check.

Intel think this change could be made in RAN3.  Qualcomm understand that this is related to LPP routing, not NRPPa, so it isn’t a RAN3 issue.

Come back on Thursday 2020-11-05. [CB]

R2-2010275	Correction on OTDOA Positioning support in R16	Huawei, HiSilicon	CR	Rel-16	38.305	16.2.0	0048	-	A	NR_pos-Core

R2-2010569	Correction of A-GNSS Periodical retrival of Assistance Data	Ericsson	CR	Rel-15	37.355	15.0.0	0277	-	F	NR_newRAT-Core

Qualcomm think the problem scenario is strange; if the serving cell is not known at the LMF, this is bad OAM and the problem is not restricted to periodic assistance data delivery; in their view Rel-15 is not broken.

Ericsson think from the network side there is a problem.  Bad OAM may be the root cause in some cases but it still causes problem scenarios, and they see it as easy for the UE to include the neighbour cell information.

CATT think this is an enhancement, not a correction, and it could be discussed in Rel-17.

Intel have the same view as Qualcomm and think if OAM is bad, the solution doesn’t work because the LMF will be unable to identify other cells too.

Qualcomm think this is not free for the UE since it does not normally decode NCGIs of neighbour cells.

Nokia agree it is not a correction.

- Not pursued

R2-2010570	Correction of A-GNSS Periodical retrival of Assistance Data	Ericsson	CR	Rel-16	37.355	16.2.0	0278	-	F	NR_newRAT-Core

R2-2010571	Correction of hanging ASN.1 code after END	Ericsson	CR	Rel-15	37.355	15.0.0	0279	-	F	NR_newRAT-Core

Qualcomm checked with compilers and did not see a syntax error, but agree the change is backward compatible and the signalling should be properly encapsulated.

- Agreed

R2-2010572	Correction of hanging ASN.1 code after END	Ericsson	CR	Rel-16	37.355	16.2.0	0280	-	F	NR_newRAT-Core

Qualcomm think the coversheet should be updated: UE impact and category F.  Chair notes the “other affected specs” should be filled out.

- Agreed with these changes as R2-2010572

# 6	Rel-16 NR Work Items

Essential corrections. While high maintenance intensity is expected, Rel-16 corrections are treated separately per WI.

## 6.6	NR Positioning Support

(NR_pos-Core; leading WG: RAN1; REL-16; started: Mar 19; target; Jun 20; WID: RP-200218, SR: RP-201342). R2 and R1 parts are 100% complete (NR TEI16 Positioning)

Documents in this agenda item will be handled in a break out session

Limit: 5 email threads

### 6.6.1	General and Stage 2 corrections

Including incoming LSs, Including impact to 36.305 and 38.305. Stage 2 corrections should be discussed with the specification rapporteur before submission.

This agenda item may use a summary document (decision to be made based on submitted tdocs).

Incoming LSs

R2-2008746	Reply LS on positioning SRS during DRX inactive time (R4-2012143; contact: Apple)	RAN4	LS in	Rel-16	NR_pos-Core	To:RAN2	Cc:RAN1

- Noted

R2-2008748	LS on new measurement gap patterns for positioning measurements (R4-2012285; contact: Ericsson)	RAN4	LS in	Rel-16	NR_pos-Core	To:RAN2

- Noted

Summary document

R2-2010674	Summary document for agenda item 6.6.1 - NR Positioning Stage2	Nokia, Nokia Shanghai Bell	discussion	Rel-16	NR_pos-Core	Late

P5:

Chair thinks this may be more of an enhancement.  Ericsson think it is a correction because there are failure modes when the UE is already transmitting.  Qualcomm think there is no activation for periodic SRS, and if the network wants “activation” it should use SP.  Nokia understand there may be interest from RAN3, but see it as somewhat of an enhancement.  Apple generally agree with Qualcomm and think it should be discussed in RAN3 first.  CATT share the same understanding.  Intel also agree with Qualcomm.  Samsung think periodic SRS has less time criticality.  Ericsson clarify they do not intend to introduce a new activation command and this would be used when the LMF is configuring the SRS characteristic.  Huawei have the same view as Qualcomm.  Proposal is noted.

P8:

Qualcomm think this is implementation-dependent and not new in NR; the network can always do this, and they think nothing needs to be captured.  Nokia agree with Qualcomm.  Intel have the same view and think this was discussed before.  Proposal is noted.

P9:

Huawei are not sure the CRs are needed.  CATT think they align the text with the figure.  To be handled in email.

P10:

Qualcomm think the new text does not quite fit into section 5.2 and some editing is needed.  CATT would like the reference for deferred MT-LR clarified.  Huawei understand that the flow is already in an SA2 specification and wonder if there is any relation to RAN.  To be handled in email.

P11:

Nokia think the change is not essential.  Huawei think this text has been there for a long time and it’s not clear what the motivation is to change it now.  Ericsson think this is an alignment CR with SA2.  Intel do not see the need and think nothing is broken in the current text.  To be handled in email.

P12:

Nokia are OK with removing the note.  Ericsson think it is there in Rel-15 as well.  Qualcomm wonder if this could be merged in another CR.  Nokia suggest it could be merged with the aperiodic SRS CR.  To be handled in email.

P1:

Agreed to have the CR from R2-2009000.

[AT112-e][603][POS] Positioning stage 2 corrections (Nokia)

Scope: Conclude on the remaining proposals from R2-2010674: P2/P3/P4/P6/P7/P9/P10/P11/P12.

Intended outcome: Agreeable CR in R2-2010863

Deadline:  Tuesday 2020-11-10 1200 UTC

Proposal 2: [R2-2010992] RAN2 to discuss the text proposals in R2-2010070 and R2-2010267 and decide on a suitable text to capture the addition of aperiodic SRS support to UL methods in TS 38.305.

Proposal 3: [R2-2010992] RAN2 to discuss and decide whether to add the additional step showing the deactivation of SRS transmission in the call flows for multi-RTT, UL-TDOA and UL-AoA positioning methods.

Proposal 4: [R2-2010992, to merge in editorial changes] RAN2 to agree only the editorial changes proposed in R2-2010266 and use proposal 2 and proposal 3 to decide on updates for the addition of aperiodic SRS support to TS 38.305.

Proposal 6: RAN2 to discuss and decide the level of details to add to Annex A in TS 38.305 for SUPL 2.0 support of NR positioning methods.

Proposal 7: RAN2 to discuss and decide the multiple corrections to E-CID and NR E-CID in R2-2010141 since it is a superset of the changes in R2-2010268.

Proposal 9: RAN2 to discuss and agree the CRs in R2-2008803 and R2-2008804 for corrections to NI-LR/MT-LR and MO-LR call flows in TS 36.305 and TS 38.305.

Proposal 10: RAN2 to discuss and decide whether to update the call flow in Figure 5.2-1 in TS 38.305 to include steps corresponding to deferred MT-LR and whether we should add the new section on deferred MT-LR under Section 7.3.x. Also discuss the minor text clarifications to the MO-LR, MT-LR/NI-LR procedures.

Proposal 11: RAN2 to discuss and decide whether to add clarification that the reported geographical co-ordinates is based on the WGS-84 reference frame.

Proposal 12: RAN2 to discuss and decide whether to delete the Editor’s note in Section 8.2.3.2.2.1 in TS 38.305 which is under the Assistance Data Delivery between LMF and ng-eNB section for OTDOA positioning.

Other contributions

R2-2008803	Minor corrections on TS 36.305	CATT	CR	Rel-16	36.305	16.2.0	0093	-	F	NR_newRAT-Core

R2-2008804	Minor corrections on TS 38.305	CATT	CR	Rel-16	38.305	16.2.0	0035	-	F	NR_newRAT-Core

R2-2009000	Remove the NOTE in architecture figure in TS 38.305	Intel Corporation	CR	Rel-16	38.305	16.2.0	0037	-	F	NR_pos-Core

- Agreed

R2-2010067	Activation Time for Periodic UL SRS Transmission	Ericsson	discussion	Rel-16

R2-2010068	Correction to SUPL support for NR positioning methods	Ericsson	CR	Rel-16	38.305	16.2.0	0038	-	F	NR_pos-Core

R2-2010069	Correction of stage 2 positioning architecture aspects	Ericsson	CR	Rel-16	38.305	16.2.0	0039	-	F	NR_pos-Core

R2-2010070	Missing Updates for Aperiodic UL SRS Support	Ericsson	CR	Rel-16	38.305	16.2.0	0040	-	F	NR_pos-Core

R2-2010092	SUPL support for NR positioning methods	Qualcomm Incorporated	CR	Rel-16	38.305	16.2.0	0041	-	F	NR_pos-Core

R2-2010141	Corrections to E-CID and NR E-CID positioning	Nokia, Nokia Shanghai Bell	CR	Rel-16	38.305	16.2.0	0043	-	F	NR_pos-Core

R2-2010266	Miscellaneous corrections for 38305	Huawei, HiSilicon	CR	Rel-16	38.305	16.2.0	0044	-	F	NR_pos-Core

R2-2010267	Correction to stage2 spec for SRS (de-)activaton	Huawei, HiSilicon	CR	Rel-16	38.305	16.2.0	0045	-	F	NR_pos-Core

=> Revised in R2-2010992

R2-2010992	Correction to stage2 spec for SRS (de-)activaton	Huawei, HiSilicon, Ericsson	CR	Rel-16	38.305	16.2.0	0045	1	F	NR_pos-Core

Intel and Nokia would like time to check.

- Handled in email as part of the summary discussion

R2-2010268	Correction to stage2 of NR ECID	Huawei, HiSilicon	CR	Rel-16	38.305	16.2.0	0046	-	F	NR_pos-Core

R2-2010573	Clarification on usage of ECID procedure	Ericsson	CR	Rel-16	38.305	16.2.0	0049	-	F	NR_pos-Core

R2-2010574	Updates on missing deferred location requests	Ericsson	CR	Rel-16	38.305	16.2.0	0050	-	F	NR_pos-Core

R2-2010575	Alignment of the position information format with SA2 specification	Ericsson	CR	Rel-16	38.305	16.2.0	0051	-	F	NR_pos-Core

R2-2010657	Miscellaneous correction for stage 2	Ericsson	CR	Rel-16	38.305	16.2.0	0052	-	F	NR_pos-Core

Withdrawn/Not available

R2-2008805	Correction on the NOTE in architecture figure in TS 38.305	CATT	CR	Rel-16	38.305	16.2.0	0036	-	F	NR_newRAT-Core	Withdrawn

### 6.6.2	RRC corrections

Including impact to 36.331, 38.331, and 38.306.

This agenda item may use a summary document (decision to be made based on submitted tdocs).

Capabilities update

[AT112-e][602][POS] Rel-16 positioning UE capabilities in RRC (Intel)

Scope: Accommodate any needed updates to the capabilities, taking into account updates to the feature lists during the meeting.

Intended outcome: Endorsable CR for merge into the capabilities mega CR

Deadline:  Friday 2020-11-13 0000 UTC

Summary document

R2-2010709	Summary for RRC Corrections for Positioning	Ericsson	discussion

P1: Discussed under R2-2008806; to be handled in email.

P2: Ericsson clarify R2-2010991 is an attempt to resolve this issue.  Nokia think we could merge the different proposals in some form but think R2-2010991 is confusing.  To be handled by email.

P5: Huawei wonder if it is possible for SI messages with and without posSIB to overlap.  Chair thinks the current text indicates they cannot.  vivo wonder if we use the same SI-RNTI how they could be distinguished if they did overlap.  Ericsson understand that this is why they do not overlap.  Nokia do not see the need for a change.

[AT112-e][604][POS] Positioning RRC proposals (Ericsson)

Scope: Discuss and resolve proposals 1 and 2 from R2-2010709.

Intended outcome: Agreeable CR in R2-2010864

Deadline:  Tuesday 2020-11-10 1200 UTC

Proposal 1	RAN2 to agree the posSIB validity inclusion in RRC and review the CR for posSIB validity check provided in R2-2008806 by email discussion.

Proposal 2	RAN2 to provide correction for field description for fields (sfn-Offset and sfn-SSB-Offset) available in SSB-Configuration. The exact changes are captured via email discussion review.

Other contributions

R2-2008806	Corrections on 38.331 to capture agreements of area scope for posSIB validity	CATT,Ericsson	CR	Rel-16	38.331	16.2.0	2014	-	F	NR_newRAT-Core

Huawei think this should not be done in RRC, because the value tag and expiration time are defined in LPP.  Ericsson think we need some guidance in RRC, but they agree it would be good to avoid specifying cross-layer interactions.  CATT think we agreed to reuse the existing area ID, and most companies felt it should be checked in RRC.  Intel agree with the background mentioned by CATT and think we decided RRC was a suitable place to handle it.  Huawei think there was no agreement to specify this in RRC, but could accept a CR to check it in LPP.  Nokia recall that we discussed which layer checks the validity and agreed it was at AS, but they are not totally OK with the proposed text.

- Handled in email [604]

R2-2008807	Corrections on description of sfn-Offset and sfn0-Offset in SSB-Configuration  in TS 38.331	CATT	CR	Rel-16	38.331	16.2.0	2015	-	F	NR_newRAT-Core

R2-2008808	Correction on the missed description of sfn-SSB-Offset in SSB-Configuration  in TS 38.331	CATT	CR	Rel-16	38.331	16.2.0	2016	-	F	NR_newRAT-Core

R2-2010071	Corrections of field descrption of sfn-Offset and sfn-SSB-Offset	Ericsson	CR	Rel-16	38.331	16.2.0	2172	-	F	NR_pos-Core

=> Revised in R2-2010991

R2-2010991	Corrections of field descrption of sfn-Offset and sfn-SSB-Offset	Ericsson	CR	Rel-16	38.331	16.2.0	2172	1	F	NR_pos-Core

R2-2010269	CR on SI window for positioning SI message	Huawei, HiSilicon	CR	Rel-16	38.331	16.2.0	2196	-	F	NR_pos-Core

- Not pursued

R2-2010270	Correction on posSRS configuration	Huawei, HiSilicon	CR	Rel-16	38.331	16.2.0	2197	-	F	NR_pos-Core

R2-2010273	Correction on posSIB broadcastStatus	Huawei, HiSilicon	CR	Rel-16	38.331	16.2.0	2199	-	F	NR_pos-Core

=> Agreed

### 6.6.3	LPP corrections

This agenda item may use a summary document (decision to be made based on submitted tdocs).

Summary document

R2-2010975	Summary of LPP corrections agenda item 6.6.3	Qualcomm Incorporated	discussion

[AT112-e][605][POS] LPP proposals (Qualcomm)

Scope: Discuss and resolve the remaining proposals from R2-2010975: P1-P5, P7, P8.

Intended outcome: Agreeable CR in R2-2010865

Deadline:  Tuesday 2020-11-10 1200 UTC

Proposal 1:	RAN2 to discuss and decide whether any specification changes are needed to clarify that a DL-PRS-ID can be reused across positioning frequency layers.

Proposal 2:	RAN2 to discuss and decide if and how "band combination fallback" is introduced in LPP.

Proposal 3:	RAN2 to discuss and decide whether the IEs NR-DL-PRS-ProcessingCapability and NR-DL-PRS-QCL-ProcessingCapability are provided in only one of the IEs NR-DL-TDOA-ProvideCapabilities, NR-DL-AoD-ProvideCapabilities, and NR-Multi-RTT-ProvideCapabilities, in the case of capabilities for multiple NR positioning methods are provided.

Proposal 4:	RAN2 to discuss and decide whether the UE is required to provide updated UL SRS capabilities in case of UL CA band combination changes during an LPP session.

Proposal 5:	RAN2 to discuss and decide in which RAN2 specification (TS 38.306 and/or TS 37.355) the content of the RAN1 LS in R2-2006103 should be captured.

Proposal 7:	RAN2 to agree a correction is required and check the details of the CR in R2-2010263 [3].

Proposal 8:	RAN2 to agree a correction is required and check the details of the CR in R2-2010264 [4].

Other contributions

R2-2009042	Discussion on whether PRS ID can be reused on different frequency layers	vivo Mobile Communication Co.,	discussion

R2-2010093	Clarification of quality and time stamp for RSTD measurements	Qualcomm Incorporated	CR	Rel-16	37.355	16.2.0	0274	-	F	NR_pos-Core

Nokia think the other quality fields may need similar clarifications, but the proposal is agreeable.

CATT wonder what happens if the RSRP and TOA measurements do not happen at the same time.  Without the CR, their understanding was that the timestamp is for the TOA measurement.  Qualcomm also wondered this but came to the conclusion that they have to be from a single measurement operation, since there is only one timestamp; they understand that the RSRP is a side effect of the TOA measurement and the UE would not do two correlations.  Samsung agree with the proposal.  vivo wonder if the change should also apply to other positioning methods or only DL-TDOA.  Qualcomm think it is clear for the other methods where there is only one measurement (whereas RSTD is a difference of two measurements).

- Agreed

R2-2010263	Correction on PRS configuration	Huawei, HiSilicon	CR	Rel-16	37.355	16.2.0	0275	-	F	NR_pos-Core

Qualcomm think this has aspects related to the DL-PRS-Id reuse across frequency layers.

- Handled in email [605]

R2-2010264	Correction on NR E-CID	Huawei, HiSilicon	CR	Rel-16	37.355	16.2.0	0276	-	F	NR_pos-Core

Ericsson think the CR is not needed.  Qualcomm note there is some added text relative to LTE.

- Handled in email [605]

R2-2010265	LPP corrections on UE capability signaling	Huawei, HiSilicon	discussion	Rel-16	NR_pos-Core

Not available/withdrawn

R2-2008809	Correction on sfn-SSB-Offset in NR-SSB-Config-r16 in TS 37.355	CATT	CR	Rel-16	37.355	16.2.0	0273	-	F	NR_newRAT-Core	Withdrawn

### 6.6.4	MAC corrections

R2-2010271	Correction on SP posSRS (de-)activation MAC CE	Huawei, HiSilicon	CR	Rel-16	38.321	16.2.0	0970	-	F	NR_pos-Core

Qualcomm think the CR is NBC and the coversheet should reflect this in the impact analysis.

- Agreed with revised coversheet in R2-2010861.

R2-2010066	SRS for Positioning transmission in Connected mode DRX	Ericsson	discussion	Rel-16

Huawei think in the RAN1 spec, MIMO SRS and positioning SRS are captured together; they doubt if this is really needed.  CATT have the same view as Huawei.  Samsung also have the same understanding.

- Noted

### 6.6.5	Other

# 7	Rel-16 EUTRA Work Items

Essential corrections

## 7.6	LTE Positioning

(NavIC, LTE TEI16 Positioning)

Documents in this agenda item will be handled by email.  No web conference is planned for this agenda item.

# 8	Rel-17 NR Work Items

## 8.7	NR Sidelink relay SI

(FS_NR_SL_relay; leading WG: RAN2; REL-17; WID: RP-201474)

Time budget: 1.5 TU

Tdoc Limitation: 5 tdocs

Email max expectation: 4 threads

### 8.7.1	Organizational

TR updates, rapporteur inputs, other organizational documents.  Documents in this AI do not count towards the tdoc limitation.

Workplan

R2-2008939	Work planning of R17 SL relay	OPPO	Work Plan	Rel-17	FS_NR_SL_relay

Qualcomm wonder if we have time to address the ENs/FFSs in the TR.

CATT wonder if the table in the work plan will be sent to SA2.  OPPO think not, but we have the email discussion for a status update to SA2 that will reflect progress of this meeting (but not necessarily every component in the TR).  Details can be discussed in the email discussion.

CATT think the “studied” entries could be clearer in the table.

Ericsson wonder if we can reply to SA2 from this meeting without reaching completion; they would rather send an LS from the next meeting.  Nokia think we should only send the LS if we have something to say, and sending status or a draft TR is not really meaningful.  Huawei think we can come back to this after having some progress and see what can be sent to SA2, and it can be discussed offline in the meantime.

- Noted

Incoming LSs

R2-2008760	LS on Direct Discovery and Relay in SA2 (S2-2006587; contact: Oppo)	SA2	LS in	Rel-17	FS_5G_ProSe	To:RAN2	Cc:RAN1

- Noted (to reply from discussion [601])

R2-2010693	LS on SA2 progress on UE-to-Network Relay and UE-to-UE Relay (S2-2007945; contact: OPPO)	SA2	LS in	Rel-17	FS_5G_ProSe	To:RAN2, SA3

Ericsson think RAN2 may be affected regarding the reselection criteria, because the criteria may affect the discovery message format; they wonder if we should inform SA2 now or just capture it in the TR.  Apple think we are only discussing AS layer selection criteria and the relation to discovery messages has to be discussed later, probably in the WI stage.  They see the message formats as a stage 3 issue.

Futurewei note that in the L3 UE-to-NW relay solution, there is no identified solution for service continuity, and a note saying it will be determined by RAN2.

OPPO think we are running into detailed comments about the email discussions and we could note the LS; we use the TR to capture decisions as usual, and decide in the email discussion what we send to SA2.

- Noted

Other contributions

R2-2008926	[Draft] Reply LS on Direct Discovery and Relay	CATT	LS out	Rel-17	5G_V2X_NRSL-Core	To:SA2	Cc:RAN1

R2-2010676	[Draft] Reply LS on Direct Discovery and Relay	OPPO	LS out	Rel-17	FS_NR_SL_relay	To:SA2	Cc:RAN1

[AT112-e][601][Relay] Status update to SA2 (OPPO)

Scope: Generate a summary of RAN2 status on relaying for SA2

- Report status of both L2 and L3 relaying designs as well as architecture-independent aspects (including issues in R2-2008760), in order to coordinate with SA2 for reaching conclusions

- Capture any points where we assume SA2 will resolve an issue

Intended outcome: Approvable LS in R2-2010862

Deadline: Friday 2020-11-13 0000 UTC

### 8.7.2	Scope requirements and scenarios

Refinements to the contents of the TR regarding high-level requirements and assumptions on supported scenarios.

This agenda item may use a summary document (decision to be made based on submitted tdocs).

Summary document

R2-2010984	Summary for AI_8.7.2 Scope requirements and scenarios	vivo	discussion

- Revised in R2-2011004

R2-2011004	Summary for AI_8.7.2 Scope requirements and scenarios	vivo	discussion

Huawei understand the intention for P2 is that the relay and remote UE are controlled by the same cell, since the remote UE may be OOC.

Futurewei note that all these proposals are specific to L2 and they understand the scenarios and requirements should also be applicable to L3.  If the scenarios are important they should be supported by both, otherwise we should not spend the time.  vivo note L3 is covered in P3.

ZTE have some concern for P2 because it does not clarify which cell is in control.  It should be the “remote UE’s serving cell”.

Intel are not sure if P1 adds anything.  They also think the same agreements should hold for L3.

Ericsson think P1 and P2 can be for both L2 and L3.  Also think we should address the RRC state of the remote UE (P6-P9).

Nokia think we do not need to introduce any cell restrictions for the L3 case because the remote UE is not visible in the relay UE’s cell.

Agreements:

Proposal 1	[easy]Confirm for L2 U2N Relay that both Case1.1 and Case 1.2 are supported in this SI, i.e.

-	Case 1.1: Before remote connection via relay UE, relay UE and remote UE are in same cell;

-	Case 1.2: Before remote connection via relay UE, relay UE and remote UE are in different cells.

Proposal 2	[easy]Confirm for L2 U2N Relay that Case 2.1 is supported in this SI as baseline, i.e. after remote UE connection via relay UE, relay UE and remote UE are controlled by the relay UE’s serving cell;

Proposal 3	For L3 U2N Relay, relay UE and remote UE can be in the cell same or different cells, after remote UE connection via Relay UE.

On P4, ZTE understand this should be a L2 CP discussion rather than scenarios.

Lenovo think we should rephrase P8 for clarity.

Ericsson support the proposals and think the idle/inactive relay UE can forward paging messages to the remote UE.  Also think the remote UE could send small data via RACH.

MediaTek think we could agree P6 and P7 but we may need more time on the combinations.

Proposal 6	[easy]For L2 U2N Relay, RAN2 to agree support of RRC_INACTIVE for remote UE.

Proposal 7	[easy]For L2 U2N Relay, RAN2 to agree support of RRC_INACTIVE for relay UE.

Proposal 8	[easy] For L2 U2N Relay, RAN2 to confirm NOT support the following RRC states combination for remote UE and relay UE:

-	(remote RRC_CONNECTED and relay RRC_IDLE or RRC_INACTIVE)

Proposal 9	For L2 U2N Relay, RAN2 to confirm support the following RRC states combination for remote UE and relay UE:

-	(remote RRC_IDLE/RRC_INACTIVE/RRC_CONNECTED and relay RRC_CONNECTED)

-	(remote RRC_IDLE or RRC_INACTIVE, and relay RRC_IDLE or RRC_INACTIVE)

[AT112-e][610][Relay] RRC states for L2 relay (vivo)

Scope: Discuss P6-P9 of R2-2011004.

Intended outcome: Summary in R2-2010869

Deadline:  Friday 2020-11-13 0000 UTC

Ericsson and QC think it is clear that there should be no impact of RRC state for L3.  Nokia and ZTE also.  Futurewei and Huawei do not understand why the L3 relay will not have state issues.

Other contributions

R2-2008779	Left issues on Scenarios for sidelink relay	OPPO	discussion	Rel-17	FS_NR_SL_relay

R2-2008921	Further Clarification on the Scenarios for NR Sidelink Relay	CATT	discussion	Rel-17	FS_NR_SL_relay

R2-2009584	Further discussion on scope and scenarios of SL relay	vivo	discussion	Rel-17

R2-2009693	Coverage Extension using Relays	Lenovo, Motorola Mobility	discussion	Rel-17	FS_NR_SL_relay

R2-2009694	QoS support when using Relays	Lenovo, Motorola Mobility	discussion	FS_NR_SL_relay

R2-2010658	Scenarios for NR sidelink relay	LG Electronics Inc.	discussion	Rel-17

### 8.7.3	Relaying Mechanisms and their characteristics

Start to populate the TR. Put on the table mechanisms, their characteristics at least with respect to aspects A-F for L2 and L3 relay etc.

#### 8.7.3.1	Protocol stacks and procedures

Including report of [Post111-e][627][Relay] Remaining issues on L2 architecture

Email discussion summary

R2-2009122	Email Report of Post111-e 627 Relay Remaining issues on L2 architecture	MediaTek Inc.	discussion	Rel-17	FS_NR_SL_relay

Discussion of “green” proposals from summary:

Qualcomm think for P25, it’s not clear if PC5 bearers with different QoS can be multiplexed into one Uu bearer.  Would like to add an FFS on this point.  Huawei do not think this is needed because it’s a matter of gNB implementation.  Qualcomm’s concern is whether e2e QoS can be guaranteed considering that the relay does not have the full capabilities of a gNB.  Futurewei agree with Huawei that it is a gNB implementation issue, but think this aspect is not discussed in any contribution and we shouldn’t conclude on it now.  Qualcomm think it is not gNB implementation only.  OPPO think we should not spend much time on this.

Ericsson think the bracketed part in P32 about discovery message could be removed.

Agreements:

Proposal-1: [Easy] agree the following description for L2 UE-to-NW relay

For L2 UE-to-NW relay, the Uu adaptation layer at Relay UE supports UL bearer mapping between ingress PC5 RLC channels for relaying and egress Uu RLC channels over the Relay UE Uu path.

Proposal-2: [Easy] agree the following description for L2 UE-to-NW relay

The different RBs of the same Remote UE and/or different Remote UEs can be subject to N:1 mapping and data multiplexing over Uu RLC channel

Proposal-3: [Easy] agree the following description for L2 UE-to-NW relay

For L2 UE-to-NW relay, Uu adaptation layer is used to support Remote UE identification for the UL traffic (multiplexing the data coming from multiple Remote UE).

Proposal-6: [Easy] agree the following description for L2 UE-to-NW relay

The Uu adaptation layer can be used to support DL bearer mapping at gNB to map end-to-end Radio Bearer (SRB, DRB) of Remote UE into Uu RLC channel over Relay UE Uu path

Proposal-15: [Easy] agree the following description for L2 UE-to-UE relay

For L2 UE-to-UE relay, the second hop PC5 adaptation layer can be used to support bearer mapping between the ingress RLC channels over first PC5 hop and egress RLC channels over second PC5 hop at Relay UE.

Proposal-25 [Easy]: agree the following description for L2 UE-to-NW relay

gNB implementation can handle the QoS breakdown over Uu and PC5 for the end-to-end QoS enforcement of a particular session established between Remote UE and network in case of L2 based UE to Network relaying.  Details of handling in case PC5 RLC channels with different e2e QoS are mapped to the same Uu RLC channel can be discussed in WI phase.

Proposal-26 [Easy]: agree the following description for L2 UE-to-UE relay

QoS handling for L2 UE-to-UE Relay is subject to upper layer, e.g. solution 31 within TR23.752 studied by SA2.

Proposal-32 [Easy] [merging P31]: agree the following description for L2 UE-to-NW relay

Relay UE can forward the system information to Remote UE via broadcast, groupcast, or dedicated PC5-RRC signalling. The detailed mechanisms of broadcast, groupcast and PC5-RRC signalling design can be discussed in WI stage.

Proposal-35 [Easy]: agree the following access control check principles for L2 UE-to-NW relay

	The Relay UE may provide UAC parameters to Remote UE

	The access control check is performed at Remote UE using the parameters of the cell it intends to access.

	The UE-to-Network Relay UE does not perform access control check for the Remote UE's data.

Ericsson wonder about the wording “particular Remote UE” in P5 and P7.  Would prefer to delete “particular”.  On P22, they wonder if it identifies the source or the destination.  MediaTek intended that it informs the relay UE of which destination to forward to.  Ericsson think we have agreement only to include the source ID.  MediaTek understand that this aspect is in a later proposal (P20a).

Apple think we should just say “identify traffic” in P22.

Interdigital agree with Apple.

ZTE would like to keep the phrasing “by relay UE” in P5, to align with P10 and P19.

Agreements:

Proposal-5 (merging P4): agree the following description for L2 UE-to-NW relay

The identity information of Remote UE Uu Radio Bearer and Remote UE is included in the Uu adaptation layer at UL in order for gNB to correlate the received data packets for the specific PDCP entity associated with the right Remote UE Uu Radio Bearer of a Remote UE.

Proposal-7: agree the following description for L2 UE-to-NW relay

The Uu adaptation layer can be used to support DL N:1 bearer mapping and data multiplexing between multiple end-to-end Radio Bearers (SRBs, DRBs) of a Remote UE and/or different Remote UEs and one Uu RLC channel over the Relay UE Uu path

Proposal-8: agree the following description for L2 UE-to-NW relay

The Uu adaptation layer needs to support Remote UE identification for Downlink traffic

Proposal-10 (merging P9): agree the following description for L2 UE-to-NW relay

The identity information of Remote UE Uu Radio Bearer and the identity information of Remote UE needs be put into the Uu adaptation layer by gNB at DL in order for Relay UE to map the received data packets from Remote UE Uu Radio Bearer to its associated PC5 RLC channel.

Proposal-21: agree the following description for L2 UE-to-UE relay

Support the N:1 mapping by first hop PC5 adaptation layer between Remote UE SL Radio Bearers and first hop PC5 RLC channels for relaying.

Proposal-22: agree the following description for L2 UE-to-UE relay

Support the adaptation layer over first hop PC5 between Source Remote UE and Relay UE in order to identify traffic destined to different Destination Remote UEs.

On P27, Qualcomm think steps 3 and 6 should have the RLC channel established potentially by the remote UE, since the relay UE does not know whether the remote UE has received the RRCSetup.

Ericsson think the proposed figure is not self-explanatory, and would rather see a full call flow with exchange of individual messages.  Intel agree.

MediaTek think it is a bit difficult to describe this flow as individual messages due to some messages occurring simultaneously or in arbitrary sequence.

OPPO agree with MediaTek and think time is a problem for going into details.  Ericsson think we have message names in the text description.  Chair suggests we take the text description now and the rapporteur will generate an agreeable figure.

Agreement:

Proposal-27: agree the following description for connection establishment procedure of L2 UE-to-NW relay:

Step 1. The Remote and Relay UE perform discovery procedure, and establish PC5-RRC connection using the legacy Rel-16 procedure as a baseline.

Step 2. The Remote UE sends the first RRC message (i.e. RRCSetupRequest) for its connection establishment with gNB via the Relay UE, using a default L2 configuration on PC5.  The gNB responds with an RRCSetup message to Remote UE. The RRCSetup delivery to the Remote UE uses the default configuration for L2 on PC5. If the relay UE had not started in RRC_CONNECTED, it would need to do its own connection establishment as part of this step. The details for Relay UE to forward the RRCSetupRequest/RRCSetup message for Remote UE at this step can be discussed in WI phase.

Step 3. The gNB and Relay UE perform relaying channel setup procedure over Uu. According to the configuration from gNB, the Relay/Remote UE establishes an RLC channel for relaying of SRB1 towards the Remote UE over PC5. This step prepares the relaying channel for SRB1.

Step 4. Remote UE SRB1 message (e.g. an RRCSetupComplete message) is sent to the gNB via the Relay UE using SRB1 relaying channel over PC5. Then the Remote UE is RRC connected over Uu.

Step 5. The Remote UE and gNB establish security following legacy procedure and the security messages are forwarded through the Relay UE.

Step 6. The gNB sets up additional RLC channels between the gNB and Relay UE for traffic relaying. According to the configuration from gNB, the Relay/Remote UE sets up additional RLC channels between the Remote UE and Relay UE for traffic relaying. The gNB sends an RRCReconfiguration to the Remote UE via the Relay UE, to set up the relaying SRB2/DRBs. The Remote UE sends an RRCReconfigurationComplete to the gNB via the Relay UE as a response.

[AT112-e][611][Relay] Open issues on L2 relay (Huawei)

Scope: Discuss the remaining open issues on L2 relay architecture, including:

- PC5 adaptation layer

- RRC procedures (including paging)

- Remaining issues from email discussion [627]

- Remaining open items in the current TR

Intended outcome: Summary in R2-2010870

Deadline:  Wednesday 2020-11-11 1200 UTC

[AT112-e][612][Relay] Open issues on L3 relay (Qualcomm)

Scope: Discuss the remaining open issues on L3 relay architecture, including:

- NAS transport

- Overhead

- QoS

- RRC states

- Remaining open items in the current TR

Intended outcome: Summary in R2-2010871

Deadline:  Wednesday 2020-11-11 1200 UTC

Other contributions

R2-2008777	Left issues on CP procedure for L2 U2N Relay	OPPO	discussion	Rel-17	FS_NR_SL_relay

R2-2008922	On-demand SI Delivery for Remote UE	CATT	discussion	Rel-17	FS_NR_SL_relay

R2-2008962	Discussion on remaining issues of L3 relay	Qualcomm Incorporated	discussion	Rel-17	FS_NR_SL_relay

R2-2008964	Discussion on remaining issues of L2 relay	Qualcomm Incorporated 	discussion	Rel-17	FS_NR_SL_relay

R2-2008966	RRC state and essential RRC procedures in L2 U2N relay	Qualcomm Incorporated	discussion	Rel-17	FS_NR_SL_relay

R2-2008983	Open aspects of L2 relaying	Intel Corporation	discussion	Rel-17	FS_NR_SL_relay

R2-2009030	Discussion on remaining issues on L2 relay architecture	ZTE Corporation, Sanechips	discussion	Rel-17	FS_NR_SL_relay

R2-2009033	Discussion on Remaining issues on L3 relay	ZTE Corporation, Sanechips	discussion	Rel-17	FS_NR_SL_relay

R2-2009123	Adaptation layer  for PC5 at L2 UE-to-Network Relay	MediaTek Inc.	discussion	Rel-17	FS_NR_SL_relay

R2-2009124	Overhead in N3IWF based L3 relaying architecture	MediaTek Inc.	discussion	Rel-17	FS_NR_SL_relay

R2-2009144	Remaining issues on the adaptation layer for Layer-2 Relay	Spreadtrum Communications	discussion	Rel-17	FS_NR_SL_relay

R2-2009202	Control Plane Aspects for UE to NW Relays	InterDigital	discussion	Rel-17	FS_NR_SL_relay

R2-2009203	Connection Establishment and Maintenance for L2 Relays	InterDigital	discussion	Rel-17	FS_NR_SL_relay

R2-2009206	Discussion on L2 Relay Architecture and QoS	InterDigital	discussion	Rel-17	FS_NR_SL_relay

R2-2009230	RAN2 impacts introduced by Layer 2 SL relay	Ericsson	discussion	Rel-17	FS_NR_SL_relay

R2-2009302	QoS Control with Sidelink Relay	Futurewei	discussion	Rel-17	FS_NR_SL_relay

R2-2009525	Discussion on data forwarding mechanisms for Layer 2 UE-to-UE Relay	Apple	discussion	Rel-17	FS_NR_SL_relay

R2-2009526	Discussion on RRC_INACTIVE remote UE	Apple	discussion	Rel-17	FS_NR_SL_relay

R2-2009585	Open issues on Layer-2 relay	vivo	discussion	Rel-17

R2-2009660	L2 relaying open issues	Samsung Electronics GmbH	discussion

R2-2009661	Need for relaying of on-demand SI	Samsung Electronics GmbH	discussion

R2-2009720	Discussion on L3 UE-to-NW relay architecture	Ericsson	discussion	Rel-17	FS_NR_SL_relay

R2-2009891	SL L2 architectrure	Sony	discussion	Rel-17	FS_NR_SL_relay

R2-2009901	Protocol stack design for U2N relay and U2U relay case	Lenovo, Motorola Mobility	discussion	Rel-17	FS_NR_SL_relay

R2-2009939	Discussion on L2 based UE-to-Network	Nokia, Nokia Shanghai Bell	discussion	FS_NR_SL_relay

R2-2010129	Needed Information in Adaptation Layer Header for L2 UE-to-UE Relay	Convida Wireless	discussion	Rel-17

R2-2010344	Remaining issues on protocol stacks and procedures for L2 relay	Huawei, HiSilicon	discussion	Rel-17	FS_NR_SL_relay

R2-2010345	NAS transmission and QoS management in L3 U2N relay	Huawei, HiSilicon	discussion	Rel-17	FS_NR_SL_relay

#### 8.7.3.2	Service continuity

Including report of [Post111-e][621][Relay] Service continuity

Email discussion summary

R2-2010346	Summary email discussion [621][Relay] of Service continuity	Huawei, HiSilicon	report	Rel-17	FS_NR_SL_relay

Other contributions

R2-2008780	Left issues on Service continuity for L2 U2N relay	OPPO	discussion	Rel-17	FS_NR_SL_relay

R2-2008923	Further Clarification on the L2 Service Continuity	CATT	discussion	Rel-17	FS_NR_SL_relay

R2-2008967	Remaining issues on the mobility procedures for L2 relay	Qualcomm Incorporated 	discussion	Rel-17	FS_NR_SL_relay

R2-2009031	Discussion on Service continuity	ZTE Corporation, Sanechips	discussion	Rel-17	FS_NR_SL_relay

R2-2009068	L3 relay enhancements to improve path switching	Nokia, Nokia Shanghai Bell	discussion	Rel-17	FS_NR_SL_relay

R2-2009125	Service Continuity for L2 Relay and L3 Relay	MediaTek Inc.	discussion	Rel-17	FS_NR_SL_relay

R2-2009145	Discussion on service continuity for Layer-2 UE-to-Network Relay	Spreadtrum Communications	discussion	Rel-17	FS_NR_SL_relay

R2-2009171	Service continuity via L3 UE-to-Network relaying	Samsung Electronics	discussion	Rel-17

R2-2009177	Service Continuity Scenarios and AS-Layer Procedures	Fraunhofer HHI, Fraunhofer IIS	discussion	Rel-17

R2-2009271	Further details on Service Continuity for Relaying	Intel Corporation	discussion	Rel-17	FS_NR_SL_relay

R2-2009301	Service Continuity with Sidelink Relay	Futurewei	discussion	Rel-17	FS_NR_SL_relay

R2-2009476	Discussion on service continuity for layer 2 UE to NW relay	Apple	discussion	Rel-17	FS_NR_SL_relay

R2-2009586	Service continuity for L2 and L3 relay	vivo	discussion	Rel-17

R2-2009721	Service continuity procedure and scenarios for sidelink relay	Ericsson	discussion	Rel-17	FS_NR_SL_relay

R2-2009938	Service Continuity for UE2UE Relay	Nokia, Nokia Shanghai Bell	discussion	Rel-17	FS_NR_SL_relay

R2-2010329	Clarification of remote UE mobility	ETRI	discussion	Rel-17	FS_NR_SL_relay

R2-2010469	Discussion on service continuity	Xiaomi communications	discussion

R2-2010588	Service continuity for SL relay	LG Electronics Inc.	discussion	Rel-17	FS_NR_SL_relay

R2-2010659	Service continuity for Remote UE	LG Electronics Inc.	discussion	Rel-17

#### 8.7.3.3	Relay selection

Including report of [Post111-e][622][Relay] Relay selection and reselection

Email discussion summary

R2-2009523	Summary Report of [Post111-e][622][Relay] Relay selection and reselection	Apple	discussion	Rel-17	FS_NR_SL_relay

Other contributions

R2-2008924	Further Discussion on NR Sidelink Relay Selection and Reselection	CATT	discussion	Rel-17	FS_NR_SL_relay

R2-2008987	Further details on relay reselection	Intel Corporation	discussion	Rel-17	FS_NR_SL_relay

R2-2009029	Discussion on Relay initiation and (re-)selection	ZTE Corporation, Sanechips	discussion	Rel-17	FS_NR_SL_relay

R2-2009069	Discussion on relay selection and reselection	Nokia, Nokia Shanghai Bell	discussion	Rel-17	FS_NR_SL_relay

R2-2009148	Discussion on relay selection and reselcetion	Spreadtrum Communications	discussion	Rel-17	FS_NR_SL_relay

R2-2009172	Consideration on relay reselection criteria	Samsung Electronics	discussion	Rel-17

R2-2009176	Relay (re)selection enhancement	MediaTek Inc.	discussion	Rel-17

R2-2009205	Relay Selection and Reselection	InterDigital	discussion	Rel-17	FS_NR_SL_relay

R2-2009229	Remaining aspects for relay selection and reselection	Ericsson	discussion	Rel-17	FS_NR_SL_relay

R2-2009588	SL-RSRP and SD-RSRP comparioson and additional criterion for relay (re-)selection	vivo	discussion	Rel-17

R2-2009634	Considerations on relay selection and reselection	KT Corp.	discussion

R2-2009857	Relay reselection in the failure case	Lenovo, Motorola Mobility	discussion	Rel-17

R2-2009892	SL Relay selection	Sony	discussion	Rel-17	FS_NR_SL_relay

R2-2009972	NR Sidelink Relay (Re-)Selection Criterion and Procedure	Fraunhofer IIS, Fraunhofer HHI	discussion	Rel-17

R2-2010005	Relay reselection based on discovery 	Kyocera	discussion

R2-2010347	Remaining issues on relay selection and reselection	Huawei, HiSilicon	discussion	Rel-17	FS_NR_SL_relay

R2-2010652	PC5 link failure handling for NR sidelink relay	LG Electronics Inc.	discussion	Rel-17

#### 8.7.3.4	Other

This agenda item may use a summary document (decision to be made based on submitted tdocs).

R2-2008778	Left issues on QoS, Security and L23 comparison	OPPO	discussion	Rel-17	FS_NR_SL_relay

R2-2009650	View on Paging Option 2 in L2 relay	ITL	discussion	Rel-17

R2-2009858	Considerations on the UE-to-Network relay and UE-to-UE relay case	Lenovo, Motorola Mobility	discussion	Rel-17

R2-2010104	Release procedure for SL Relaying support  	Intel Corporation	discussion	Rel-17	FS_NR_SL_relay

### 8.7.4	Discovery model and procedure for sidelink relaying

Including report of [Post111-e][623][Relay] Remaining issues on relay discovery

Email discussion summary

R2-2008815	Summary of  [Post111-e][623][Relay]Remaining issues on relay discovery (rapporteur)	OPPO	discussion	Rel-17	FS_NR_SL_relay	Revised

R2-2010661	Summary of  [Post111-e][623][Relay]Remaining issues on relay discovery (rapporteur)	OPPO	discussion	Rel-17	FS_NR_SL_relay	R2-2008815	Late

Other contributions

R2-2008802	Discussion on AS layer protocol of discovery message for SL relay	OPPO	discussion	Rel-17	FS_NR_SL_relay

R2-2008925	Discussion on discovery message	CATT	discussion	Rel-17	FS_NR_SL_relay

R2-2008965	Remaining issues on discovery  and relay (re)selection 	Qualcomm Incorporated	discussion	Rel-17	FS_NR_SL_relay

R2-2008977	Further details on SL discovery for relaying	Intel Corporation	discussion	Rel-17	FS_NR_SL_relay

R2-2009032	Discussion on relay discovery and link management	ZTE Corporation, Sanechips	discussion	Rel-17	FS_NR_SL_relay

R2-2009149	Discussion on remaining issues on relay discovery	Spreadtrum Communications	discussion	Rel-17	FS_NR_SL_relay

R2-2009173	Sidelink relay discovery open issue	Samsung Electronics	discussion	Rel-17

R2-2009204	Discovery Procedure for SL Relays	InterDigital	discussion	Rel-17	FS_NR_SL_relay

R2-2009228	Remaining aspects for discovery	Ericsson	discussion	Rel-17	FS_NR_SL_relay

R2-2009524	Discussion on remaining issues on NR Sidelink Relay discovery	Apple	discussion	Rel-17	FS_NR_SL_relay

R2-2009587	Remaining issues of sidelink relay discovery procedure	vivo	discussion	Rel-17

R2-2009633	Considerations on discovery for sidelink relay	KT Corp.	discussion

R2-2009638	Discussion on differentiation of discovery message	SHARP Corporation	discussion	Rel-17

R2-2009970	NR Sidelink Relaying Discovery	Fraunhofer IIS, Fraunhofer HHI	discussion	Rel-17

R2-2009994	Discovery resources for sidelink relaying 	Kyocera	discussion

R2-2010046	Discussion on relay discovery model and procedure	Nokia, Nokia Shanghai Bell	discussion	Rel-17	FS_NR_SL_relay

R2-2010331	On relay discovery	MediaTek Inc.	discussion	Rel-17	FS_NR_SL_relay

R2-2010348	Remaining issues on relay discovery	Huawei, HiSilicon	discussion	Rel-17	FS_NR_SL_relay

R2-2010349	Discussion on the discovery aspects related to SA2 LS S2-2006587	Huawei, HiSilicon	discussion	Rel-17	FS_NR_SL_relay

R2-2010467	Discussion on scenario regarding non SL relay capable gNB	Xiaomi communications	discussion

R2-2010660	Remaining issues on discovery for NR sidelink relay	LG Electronics Inc.	discussion	Rel-17

## 8.11	NR positioning enhancements SI

(FS_NR_pos_enh; leading WG: RAN1; REL-17; WID: RP-202094)

Time budget: 1 TU

Tdoc Limitation: 4 tdocs

Email max expectation: 3 threads

### 8.11.1	Organizational

Rapporteur inputs and other organizational documents. Documents in this AI do not count towards the tdoc limitation.

Incoming LSs

R2-2008707	LS on Latency of NR Positioning Protocols (R1-2007264; contact: Intel)	RAN1	LS in	Rel-17	FS_NR_pos_enh	To:RAN2	Cc:RAN3, SA2

Qualcomm note that the LS asks for end-to-end latency, not just of the positioning procedures (and the SID and SA1 requirements also talk about end-to-end); so in their view it is misleading to focus only on the positioning procedures.  Nokia think this was discussed as part of the email discussion on latency after RAN2#111-e, and we decided to focus on the RAN2 protocol contribution to the delay.  Ericsson agree with Nokia.  Huawei have the same view and think the tasking for RAN2 is clear, but we can discuss further under the email discussion.  Intel agree this was covered in the email discussion.

- Noted (will reply from the discussion of R2-2009001)

R2-2008766 LS on Requirements on positioning for UAS (S6-200269; contact: InterDigital)   SA6     LS in    Rel-17 FS_UASAPP   To:SA1            Cc:SA2, RAN2

- Noted

TR maintenance

R2-2010577	TP for TR 38.857 Study on NR Positioning	Ericsson, Swift Navigation	report	Rel-17	38.857

R2-2010576	draft LS to capture TP for TR 38.857	Ericsson	LS out	Rel-17	To:RAN1

Ericsson think we can wait for the next meeting to send an update.

Nokia think any related issues can be discussed under the integrity agenda item.

- Noted

### 8.11.2	Enhancements for commercial use cases

Scope and general discussion related to the RAN2 objective on enhancements to support high accuracy, low latency, network efficiency, and device efficiency for commercial use cases.

Including report of [Post111-e][625][POS] End-to-end latency analysis

This agenda item will use a summary document.

Email summary

R2-2009001	Report of [Post111-e][625][POS] End-to-end latency analysis (Intel)	Intel Corporation	discussion	Rel-17	FS_NR_pos_enh

Discussion of P1-P3:

Qualcomm think it does not make sense to focus only on the positioning procedures.  It would mean 0 ms latency for deferred MT-LR.  They also think the RAN2 scope in this discussion has gotten a bit confused and we are making arbitrary distinctions between interfaces in our scope and out of our scope.  They are concerned that we give misleading information if we claim latency numbers that only account for the positioning procedures.  In particular, they think enhancements going beyond the positioning procedures should not be excluded.

Huawei have a comment on P1: If this is agreed, there is some overlap between the RAN1 and RAN2 analyses of latency, so they would prefer if RAN2 excludes this part.  To Qualcomm’s comments, they think RAN1 have already shown what parts of the procedures RAN2 needs to analyse: from reception of PDSCH to transmission of PUSCH, for certain messages; and they don’t see that we need to cover anything outside that range.

Intel clarify that the reason the AMF-LMF interface has to be analysed is because it relates to transfer of LPP/NRPPa messages, whereas the GMLC and UDM interfaces transfer messages that are not in RAN2 scope.  Regarding Huawei’s comment, Intel understand that we do not need to restrict ourselves to strictly exclude things that have been looked at by RAN1.

Nokia think lack of time for the SI is the big issue.  They understand that RAN1 are focussing only on L1 latency, and we as the stage 2 owner could try to coordinate looking at all the signalling delays, but we would need a lot of time.  They understand that we are looking at the interfaces that underly transport of LPP and NRPPa.

Qualcomm think there is no contradiction between RAN1 looking at L1 and RAN2 looking at end-to-end.  They see not so much time effort to extrapolate from the conclusions of the email discussion to cover the end-to-end aspects, and think the SID calls for end-to-end.  They think we should at least capture some guidance in the TR about what we have analysed, and wonder how we will evaluate enhancements such as idle/inactive without considering procedures outside the positioning procedures.

Ericsson agree with Intel and others that what we have set out to do is enough.  They agree readers should be careful but think this is true anyway, as deployment and implementation aspects will also affect the latency.

CATT understand that the purpose of the latency analysis is to find enhancements to reduce the latency; they are fine with considering enhancements that go outside the bounds of the positioning procedures.

vivo agree with Qualcomm that RAN1 is asking for end-to-end latency, and think if no one takes responsibility for end-to-end latency we have no way to judge the enhancements.

Intel agree with Qualcomm that we should note clearly in the minutes or TR what we have analysed.

Agreements:

1: For latency analysis of Rel.16 solutions, RAN2 only consider the latency of positioning procedure, i.e. step 5 in MO-LR/step 12 in MT-LR (involving RRC, LPP, NRPPa, MAC).  A note is added to the TR and in our response to RAN1 to clarify this is what we covered.  We can clarify to RAN1 that more time would be needed for an end-to-end analysis.

2: For latency analysis of Rel.16 solutions, RAN2 only consider the latency caused by UE, gNB, AMF and LMF.

3: For latency analysis of Rel.16 solutions, RAN2 consider both UE-based and UE-assisted.

Handling of call flows in P4-P8:

Qualcomm note that the multi-RTT procedure is not in agreement with the stage 2; it takes DL and UL in sequence, and they think we should use the stage 2 flow as the baseline with some steps in parallel.  Nokia think we need a separate document showing the call flows and latencies.

Intel indicate that they have updated the multi-RTT to account for the UL and DL measurements in parallel, and they wonder if Qualcomm are commenting on the latest version.  Qualcomm find different numbers when checking the multi-RTT in the email discussion.

Call flows from P4-P8 are used as baseline in offline discussion.

Handling of tables in P9-P18:

Tables and contents from P9-P18 are used as baseline in offline discussion.

Qualcomm wonder if we will capture the results as a TP or just provide them to RAN1 for capturing.  Intel suggest that we have a summary of the results and a separate TP.  Nokia would like to see the reply LS and associated document first and then decide on a TP.

Ericsson would like to see the document aligned with an appropriate structure for the TR.  Nokia think we should focus on the content and format it later considering the short time to send the LS.  Intel agree with Nokia and understand that there will be a separate email discussion for a TP in due course.

[AT112-e][606][POS] LS to RAN1 on positioning latency (Intel)

Scope: Summarise the latency results and draft an LS to RAN1.  Clarify that the attached results are a checkpoint that has not yet been endorsed as a TP.

Intended outcome: Summary of latency results (R2-2010866) and agreeable LS (R2-2010867) with the summary attached

Deadline:  Thursday 2020-11-05 1200 UTC

Summary document

R2-2010669	Summary of 8.11.2 Enhancements for commercial use cases	CATT	discussion	Rel-17	FS_NR_pos_enh	Late

Discussion of P2:

Qualcomm think we cannot simply skip the capability procedure, because the LMF needs it.  They understand that deferred MT-LR addresses this issue by avoiding unnecessary sending of the capabilities.  They see no benefit to storing the capabilities in the AMF since the LMF will still need to fetch them.  Huawei have generally the same view: The LMF cannot blindly configure a UE, and they do not see storage in the AMF as necessary since anyway the LMF can keep the capability.

ZTE have the same view as Huawei and Qualcomm, and on P3 they think this may reduce network efficiency and is only applicable to RRC_CONNECTED.

Intel think we should collect and clarify solutions before doing too much down-selection.

Latency enhancements to be discussed further by email.

[AT112-e][607][POS] Gathering of latency enhancement solutions (CATT)

Scope: Describe and discuss the proposed latency enhancements in a format suitable for developing into a TP.

Intended outcome: Text proposal in R2-2010868

Deadline:  Friday 2020-11-13 0000 UTC

Discussion of P8/P9:

Chair understands that RAN1 have agreed positioning support in RRC_INACTIVE.  CATT have the same understanding.  Intel wonder about the relationship to SDT.

Nokia think the support for measurement in a particular state is more a RAN1 decision, and RAN2 do not need to agree on it; we can focus on reporting enhancements.

Huawei have the same view as Nokia, and think there is a concern about SDT since it only carries MO user-plane data, so it may not support our uses.  But they think we have the motivation to resolve these issues and the question should be discussed in this WI.

Qualcomm understand the RAN1 agreement also considers UL and UL+DL methods.  They agree measurement is not really in RAN2 scope, but configuration and reporting are.  On SDT, they think CIoT resolved this same basic issue in Rel-16.  They also see a bit of a disconnect since the state transition aspects are outside the sequences that we agreed to analyse for latency.

CATT think support of idle/inactive is not primarily for latency reduction but for network efficiency.  The reason for proposing DL-PRS as the first priority in RRC_INACTIVE is that for DL-PRS measurement, RAN1 already agreed to support it, and it is valuable for RAN2 to evaluate how to support the DL measurement report in inactive.

Ericsson think we are broadening the scope of the discussion a bit by involving SDT and related subject; do we believe that positioning will drive work to enhance these features?  If so, we should involve additional people.

ZTE agree with CATT and think the main advantage of idle/inactive is for network efficiency and maybe power saving.  About SDT, they think a lot of aspects of this subject still need to be discussed and they would prefer that we discuss this after SDT have progressed.

Qualcomm think we had similar overlap for on-demand SI in connected mode, and we should observe what SDT are doing and see if it’s useful for us.  They consider RRC_INACTIVE as clearly an enhancement for latency, since measurement reports can be piggybacked on RACH and state transitions can be avoided—but this is outside the positioning procedures that we agreed to consider for latency evaluation.

Huawei think we should also consider if the LCS request in an MO-LR can be transmitted in idle/inactive.  They do not see relevance to latency reduction, because the state transitions could be avoided by staying in RRC_CONNECTED all the time.  They point out that the CIoT solution is for LTE and we would need to look at the impact for supporting CP data transmission.

CATT think we should clarify that support of positioning in idle/inactive does not mean all procedures would be supported.  Considering the time budget they would prefer to focus first on DL-PRS case, but also consider other potential solutions besides SDT.

Ericsson are not sure that positioning reports are “small” in the sense of SDT.  We can discuss SDT but they think we do not need to single it out.  They agree with Huawei that this is not for latency reduction.

Intel report that the RAN1 agreement is broad (all positioning methods, UE-based+UE-assisted) and also includes e.g. AD delivery, but does not include the details of how to enable transfer of the signalling; this is all for the WI phase.

Agreements:

Positioning measurement reporting (including location estimates for UE-based) should be supported in RRC_INACTIVE; involvement of SDT is FFS.  Reporting of specific measurements is pending RAN1 decision.

Discussion of P14/P15:

Ericsson think P15 needs some discussion as an LMF may not be able to fulfil the requirement for every UE.

Intel think RAN1 already agreed P14 and we would just be aligning with them.

Intel and CATT clarify RAN1 have not agreed on UE-based multi-RTT.  Nokia understand that they took agreements that imply UE-based multi-RTT would be supported.  Qualcomm have the same understanding as Nokia.

Ericsson think we need to discuss the feasibility of the signalling.

Agreements on on-demand PRS

RAN2 study on-demand PRS mechanism for DL-based, UL&DL-based methods (e.g. multi-RTT), and UE-Based and UE-assisted positioning methods in this SI.

[Post112-e][608][POS] Support of on-demand PRS (Ericsson)

Scope: Discuss potential solutions for on-demand PRS: signalling aspects, which node requests the PRS, which node the request is directed to. Rapporteur is asked to provide update on RAN1 agreements.

Intended outcome: Report to next meeting

Deadline:  Long

[Post112-e][609][POS] Positioning support in RRC_IDLE/RRC_INACTIVE (Huawei)

Scope: Discuss potential solutions for positioning support in RRC_IDLE/RRC_INACTIVE, distinguishing clearly between what can be supported in idle and what can be supported in inactive.  Rapporteur is asked to provide update on RAN1 agreements.

Intended outcome: Report to next meeting

Deadline:  Long

Other contributions

R2-2008775	Discussion on on-demand DL-PRS	OPPO	discussion	Rel-17	FS_NR_pos_enh

R2-2008776	Positioning in RRC_IDLE and RRC_INACTIVE state	OPPO	discussion	Rel-17	FS_NR_pos_enh

R2-2008810	Further discussion on ehancements for commercial use cases	CATT	discussion	Rel-17	FS_NR_pos_enh

R2-2008885	Discussion on Positioning in Idle/Inactive mode	InterDigital, Inc.	discussion	Rel-17	FS_NR_pos_enh

R2-2008886	Discussion on End-to-End Latency Reduction for DL/UL Positioning	InterDigital, Inc.	discussion	Rel-17	FS_NR_pos_enh

R2-2008887	Discussion on On Demand Reference Signals for Positioning 	InterDigital, Inc.	discussion	Rel-17	FS_NR_pos_enh

R2-2009002	Support of positioning in idle/inactive mode	Intel Corporation	discussion	Rel-17

R2-2009023	Solution directions to reduce end-to-end latency	Intel Corporation	discussion	Rel-17	FS_NR_pos_enh

R2-2009039	Discussion on positioning enhancement	vivo Mobile Communication Co.,	discussion	FS_NR_pos_enh

R2-2009040	Procedure of on-demand PRS	vivo Mobile Communication Co.,	discussion	FS_NR_pos_enh

R2-2009041	Positioning in RRC idle and inactive state	vivo Mobile Communication Co.,	discussion	FS_NR_pos_enh

R2-2009137	Discussion on positioning enhancements for commercial use cases	Spreadtrum Communications	discussion	Rel-17	FS_NR_pos_enh

R2-2009286	Reporting movement models	Fraunhofer IIS	discussion	R2-2007238	Revised

R2-2009287	Reporting the situational quality of RAT and RAT-independent technologies	Fraunhofer IIS, Fraunhofer HHI	discussion	R2-2007246

R2-2009574	Discussion on PRS enhancements	Beijing Xiaomi Electronics	discussion	Rel-17

R2-2009577	Positioning enhancements on RRC idle/inactive UE and latency reduction	Beijing Xiaomi Electronics	discussion	Rel-17

R2-2009897	Considerations on potential positioning enhancements	Sony	discussion	Rel-17	FS_NR_pos_enh

R2-2010072	Enhancements for commercial use cases	Ericsson	discussion	Rel-17

R2-2010095	NR Positioning Enhancements	Qualcomm Incorporated	discussion

R2-2010096	NR Positioning Latency Analysis and Enhancements	Qualcomm Incorporated	discussion

R2-2010097	On-Demand PRS	Qualcomm Incorporated	discussion

R2-2010131	Reporting movement models	Fraunhofer IIS, Fraunhofer HHI	discussion	R2-2009286

R2-2010161	On-demand PRS transmission and dynamic PRS resource allocation	Nokia, Nokia Shanghai Bell	discussion	Rel-17	FS_NR_pos_enh	R2-2007128

R2-2010276	Discussion on IDLE INACTIVE pos, on-demand PRS and latency analysis	Huawei, HiSilicon	discussion	Rel-17	FS_NR_pos_enh

R2-2010277	Discussion on R17 positioning enhancement	Huawei, HiSilicon	discussion	Rel-17	FS_NR_pos_enh

R2-2010472	Disucssion on IDLE/INACTIVE mode positioning	ZTE Corporation, Sanechips	discussion	Rel-17	FS_NR_pos_enh

R2-2010473	Discussion on on-demand PRS	ZTE Corporation, Sanechips	discussion	Rel-17	FS_NR_pos_enh

R2-2010627	Discussion on enhancement for commercial use cases	Samsung R&D Institute UK	discussion

R2-2010648	Support for positioning in idle/inactive mode	Samsung R&D Institute UK	discussion

### 8.11.3	Integrity and reliability of assistance data and position information

#### 8.11.3.1	KPIs and use cases

Including report of [Post111-e][626][POS] Integrity use cases and specification impacts

Email discussion summary

R2-2009129	Summary of [Post111-e][626][POS] Email Discussion on integrity use cases and specification impacts	Swift Navigation	discussion

Other contributions

R2-2008811	Discussion on integrity service level	CATT	discussion	Rel-17	FS_NR_pos_enh

R2-2009760	Positioning integrity for Industrial IoT use cases	Nokia, Nokia Shanghai Bell	discussion	Rel-17	FS_NR_pos_enh

R2-2009898	Discussion on Integrity of positioning information	Sony	discussion	Rel-17	FS_NR_pos_enh

R2-2010074	Industrial IoT use-case	Ericsson	discussion	Rel-17

R2-2010090	Integrity and reliability for IIoT positioning use cases	Convida Wireless	discussion	Rel-17	38.857	FS_NR_pos_enh

R2-2010098	Discussion on including PL Availability as an additional integrity KPI	ESA	discussion	Rel-17	FS_NR_pos_enh

R2-2010475	Discussion on integrity&error source factor transmission	ZTE Corporation, Sanechips	discussion	Rel-17	FS_NR_pos_enh

#### 8.11.3.2	Error sources threat models occurrence rates and failure modes

This agenda item may use a summary document (decision to be made based on submitted tdocs).

Summary document

R2-2010700	Summary of 8.11.3.2 Error sources threat models occurrence rates and failure modes	Intel Corporation	discussion	Rel-17	FS_NR_pos_enh

Other contributions

R2-2008812	Discussion on error sources, threat models, occurrence rates and failure modes	CATT	discussion	Rel-17	FS_NR_pos_enh

R2-2009282	Error sources, threat models, occurrence rates and failure modes	Fraunhofer IIS	discussion	Revised

R2-2009331	Discussion on GNSS Integrity Errors	Swift Navigation, Ericsson, Intel Corporation, u-blox	discussion

R2-2010061	Text Proposal on GNSS position integrity error sources	ESA	discussion	Rel-17	38.857	FS_NR_pos_enh

R2-2010073	GNSS position integrity error sources	Ericsson	discussion	Rel-17

R2-2010135	Error sources, threat models, occurrence rates and failure modes	Fraunhofer IIS, Fraunhofer HHI	discussion	R2-2009282

R2-2010278	Discussion on threat models and failure modes	Huawei, HiSilicon	discussion	Rel-17	FS_NR_pos_enh

R2-2010642	Introduction of Integrity monitoring for GNSS and its error	Samsung R&D Institute UK	discussion

#### 8.11.3.3	Methodologies for network-assisted and UE-assisted integrity

This agenda item may use a summary document (decision to be made based on submitted tdocs).

Summary document

R2-2010675	Summary of 8.11.3.3: Methodologies for network-assisted and UE-assisted integrity	InterDigital	discussion	Rel-17	FS_NR_pos_enh

Other contributions

R2-2008774	Discussion on  methodology for integrity	OPPO	discussion	Rel-17	FS_NR_pos_enh

R2-2008813	Discussion on methodologies for network-assisted and UE-assisted integrity	CATT	discussion	Rel-17	FS_NR_pos_enh

R2-2008888	Discussion on methodologies for network-assisted and UE-assisted integrity	InterDigital, Inc.	discussion	Rel-17	FS_NR_pos_enh

R2-2009003	Methodologies for network-assisted and UE-assisted integrity	Intel Corporation, Swift Navigation	discussion	Rel-17

R2-2009043	Integrity signaling and procedures	vivo Mobile Communication Co.,	discussion

R2-2009138	Discussion on integrity methodologies for network-assisted and UE-assisted integrity	Spreadtrum Communications	discussion	Rel-17	FS_NR_pos_enh

R2-2009333	TP for GNSS Integrity Methodologies	Swift Navigation, Ericsson, Intel Corporation, u-blox	discussion

R2-2009530	Discussion on Positioning Integrity	Apple	discussion	Rel-17	FS_NR_pos_enh

R2-2009578	Discussion on methodologies for positioning integrity	Beijing Xiaomi Electronics	discussion	Rel-17

R2-2009761	Signalling for Positioning Integrity Support	Nokia, Nokia Shanghai Bell	discussion	Rel-17	FS_NR_pos_enh

R2-2010075	Methodologies for network-assisted and UE-assisted integrity	Ericsson	discussion	Rel-17

R2-2010279	Discussion for network-assisted and UE-assisted integrity	Huawei, HiSilicon	discussion	Rel-17	FS_NR_pos_enh

R2-2010474	Discussion of the methodologies for network-assisted and UE-assisted integrity	ZTE Corporation, Sanechips	discussion	Rel-17	FS_NR_pos_enh