3GPP TSG-RAN WG2 Meeting #112 electronic	R2-2010701

Online, 02 – 13 November 2020

Agenda item:	10.1

Source: 	Vice Chairman (Nokia)

Title:	Report on LTE legacy, Mobility, DCCA, Multi-SIM and RAN slicing

Document for:	Approval

# Organizational

List of offline email discussions:

NOTE: the email discussion deadlines are meant to allow at least all regions to have one day to comment (other than weekend) and also give rapporteurs time to update their proposals before the meeting)

Organizational

[AT112-e][200] Organizational Tero – LTE legacy, LTE Rel-16 and LTE/NR mobility

Scope:

    - Share plans for the meetings and list of ongoing email discussions for the sessions

    - Share meetings notes and agreements for review and endorsement

    - Flag LSs for presentation (where applicable)

Intended outcome (for LS discussion):

    - General information sharing about the sessions

Deadline for providing comments to LSs:

    - Deadline: EOM

LTE Legacy (kicked off on Monday Nov 2nd)

[AT112-e][201][LTE] LTE Miscellaneous corrections (RAN2 VC)

Scope:

    - Discuss the CRs under AI 4.5, 7.1.X and 7.5 marked for this email discussion

Intended outcome:

    - Discussion summary in R2-2010710 (by email rapporteur)

    - Agreeable CRs by proponents (if revised versions are required, proponents should obtain Tdoc numbers from session chair or RAN2 secretary to provide those)

Deadline for providing comments and for rapporteur inputs:

    - Initial deadline (for companies' feedback):  1st week Fri, UTC 0900

    - Initial deadline (for rapporteur's summary in R2-2010710):  2nd week Mon, UTC 13:00

    - Deadline for CR finalization: 2nd week Thu, UTC 1000

[AT112-e][202][LTE] LTE editorial corrections (RAN2 VC)

Scope:

    - Discuss the CRs under AI 4.5, 7.1.X and 7.5 marked for this email discussion

Intended outcome:

    - Discussion summary in R2-2010711 (by email rapporteur)

    - Agreeable CRs for 36.300, 36.306 and 36.331 (if any) by specification rapporteurs (after online session)

Deadline for providing comments and for rapporteur inputs:

    - Initial deadline (for companies' feedback):  1st week Fri, UTC 0900

    - Initial deadline (for rapporteur's summary in R2-2010711):  2nd week Mon, UTC 13:00

    - Deadline for CR finalization: 2nd week Thu, UTC 1000

[AT112-e][203][LTE] LTE corrections related to RLC out-of-order delivery (Samsung)

Scope:

    - Discuss the CRs under AI 4.5 related to the RLC out-of-order delivery that are marked for this email discussion to determine which changes are acceptable

Intended outcome:

    - Discussion summary in R2-2010714 (by email rapporteur), agreeable CRs (Tdoc numbers can be obtained from session chair if needed)

Deadline for providing comments and for rapporteur inputs:

    - Initial deadline (for companies' feedback):  1st week Fri, UTC 0900

    - Initial deadline (for rapporteur's summary in R2-2010714):  2nd week Mon, UTC 13:00

LTE Legacy (kicked off on Wednesday Nov 4th)

[AT112-e][204][LTE] LTE corrections to TDD/FDD capability differentiation (Huawei)

Scope:

    - Progress the revisions to R2-2009921 and R2-2009922 based on RAN1 LS R2-2011001

Intended outcome:

    - Revised CRs in R2-2010735 (Rel-15, 36.331) and R2-2010736  (Rel-16, 36.331)

Deadline for providing comments and for rapporteur inputs:

    - Initial deadline (for companies' feedback):  2nd week Mon, UTC 13:00

    - Initial deadline (for revised CRs):  2nd week Tue, UTC 13:00

LTE Legacy (kicked off after online session)

TBD

LTE Rel-16 (kicked off after online session)

TBD

LTE/NR Mobility (to be kicked off on Monday Nov 2nd)

[AT112-e][210][MOB] Stage-2 corrections (Nokia)

Scope:

    - Discuss which mobility WI - related Stage-2 corrections (for LTE, MR-DC and NR) are seen necessary

Intended outcome:

    - Discussion summary in R2-2010715 (by email rapporteur).

    - Merged CRs to 36.300 (R2-2010716), 38.300 (R2-2010717) and 37.340 (R2-2010718) (if any)

Deadline for providing comments, for rapporteur inputs, conclusions and CR finalization:

    - Initial deadline (for companies' feedback):  1st week Fri, UTC 0900

    - Initial deadline (for rapporteur's summary in R2-2010715):  2nd week Mon, UTC 13:00

    - Deadline for CR finalization: 2nd week Thu, UTC 1000

[AT112-e][211][MOB] CHO/CPC RRC corrections (Intel)

Scope:

    - Discuss which CHO/CPC corrections for 36.331/38.331 are seen necessary and provide merged CRs with agreeable corrections (if any)

Intended outcome:

    - Discussion summary in R2-2010719 (by email rapporteur).

    - Merged CRs to 36.331 (R2-2010720) and 38.331 (R2-2010721) (if any)

Deadline for providing comments, for rapporteur inputs, conclusions and CR finalization:

    - Initial deadline (for companies' feedback):  1st week Fri, UTC 0900

    - Initial deadline (for rapporteur's summary in R2-2010719):  2nd week Mon, UTC 13:00

    - Deadline for CR finalization: 2nd week Thu, UTC 1000

[AT112-e][212][MOB] Mobility UE capabilities for LTE and NR (Huawei)

Scope:

    - Discuss which UE capability corrections to LTE and NR are seen necessary and provide merged CRs with agreeable corrections (if any)

Intended outcome:

    - Discussion summary in R2-2010722 (by email rapporteur).

    - Merged CRs to 36.306 (R2-2010723), 36.331 (R2-2010724), 38.306 (R2-2010725) and 38.331 (R2-2010726) (if any)

Deadline for providing comments, for rapporteur inputs, conclusions and CR finalization:

    - Initial deadline (for companies' feedback):  1st week Fri, UTC 0900

    - Initial deadline (for rapporteur's summary in R2-2010722):  2nd week Mon, UTC 13:00

    - Deadline for CR finalization: 2nd week Thu, UTC 1000

[AT112-e][213][MOB] DAPS RRC corrections (Ericsson)

Scope:

    - Discuss which DAPS RRC corrections to LTE and NR are seen necessary and provide merged CRs with agreeable corrections (if any)

Intended outcome:

    - Discussion summary in R2-2010727 (by email rapporteur).

    - Merged CRs to 36.331 (R2-2010728) and 38.331 (R2-2010729) (if any)

Deadline for providing comments, for rapporteur inputs, conclusions and CR finalization:

    - Initial deadline (for companies' feedback):  1st week Fri, UTC 0900

    - Initial deadline (for rapporteur's summary in R2-2010727):  2nd week Mon, UTC 13:00

    - Deadline for CR finalization: 2nd week Thu, UTC 1000

LTE Mobility  (only started after online session on Tuesday Nov 3rd)

TBD

NR Mobility  (only started after online session on Tuesday Nov 3rd)

TBD

LTE/NR Rel-16 DCCA (to be kicked off on Monday Monday Nov 2nd)

[AT112-e][220][DCCA] Simple DCCA corrections (Ericsson)

Scope:

    - Discuss DCCA corrections under 6.8.1/6.8.3/6.8.4/6.8.5 marked for the discussion to see which CRs could be agreeable

Intended outcome:

    - Discussion summary in R2-2010730 (by email rapporteur).

Deadline for providing comments, for rapporteur inputs, conclusions and CR finalization:

    - Initial deadline (for companies' feedback):  1st week Fri, UTC 0900

    - Initial deadline (for rapporteur's summary in R2-2010730):  2nd week Mon, UTC 13:00

[AT112-e][221][DCCA] Fast Scell activation and early measurements (Nokia)

Scope:

    - Discuss corrections under 6.8.2/6.8.3 marked for this discussion to see which CRs could be agreeable

Intended outcome:

    - Discussion summary in R2-2010731 (by email rapporteur).

Deadline for providing comments, for rapporteur inputs, conclusions and CR finalization:

    - Initial deadline (for companies' feedback):  1st week Fri, UTC 0900

    - Initial deadline (for rapporteur's summary in R2-2010731):  2nd week Mon, UTC 13:00

[AT112-e][222][DCCA] Miscellaneous DCCA corrections and capabilities (Ericsson?)

Scope:

    - Discuss DCCA corrections under 6.8.4 marked for this discussion to see which CRs could be agreeable

Intended outcome:

    - Discussion summary in R2-2010732 (by email rapporteur).

Deadline for providing comments, for rapporteur inputs, conclusions and CR finalization:

    - Initial deadline (for companies' feedback):  1st week Fri, UTC 0900

    - Initial deadline (for rapporteur's summary in R2-2010732):  2nd week Mon, UTC 13:00

TBA

NR Rel-17 DCCA (only after online session)

[AT112-e][230][eDCCA] Progressing FFS points of efficient SCG activation and deactivation (Huawei)

Scope:

    - Discuss the FFSs from online agreements for Efficient activation deactivation of SCG to understand which alternatives are seen feasible.

    - Can discuss also remaining FFS from email discussion [Post111-e][919] if time allows

Intended outcome:

    - Discussion summary in R2-2010733 (by email rapporteur).

Deadlines:

    - Rapporteur can set an intermediate deadline for company inputs and/or converging the discussion

    - Deadline for email discussion report: 2nd week Thu, UTC 1000

[AT112-e][231][eDCCA] Progressing conditional reconfiguration for SN initiated inter-SN CPC (CATT)

Scope:

    - Discuss the option 1 and option 3 details from P4 of email discussion [Post111-e][920] to better understand the technical details between the alternatives (e.g. signalling flows, signalling load, etc.)

Intended outcome:

    - Discussion summary in R2-2010734 (by email rapporteur).

Deadlines:

    - Rapporteur can set an intermediate deadline for company inputs and/or converging the discussion

    - Deadline for email discussion report: 2nd week Thu, UTC 1000

NR Rel-17 RAN Slicing (only after online session)

TBA

NR Rel-17 Multi-SIM (only after online session)

TBA

[AT112-e][240][Multi-SIM] Reply LS to SA2 (Intel)

Scope:

    - Draft LS reply to SA2 based on online agreements (can also include some analysis from email discussion)

Intended outcome:

    - Discussion summary in R2-2010737 (by email rapporteur) and draft reply LS in R2-2010738.

Deadlines:

    - Deadline for email discussion report: 2nd week Thu, UTC 1000

[AT112-e][241][Multi-SIM] Network switching scenarios (vivo)

Scope:

    - Discuss validity of scenario 3 and scenario 4 from the previous email disucssion

Intended outcome:

    - Discussion summary in R2-2010739 (by email rapporteur).

Deadlines:

    - Deadline for email discussion report: 2nd week Thu, UTC 1000

Dates and deadlines

Oct 22 23.59 PDT	(Oct 23 06.59 UTC) Tdoc number allocation deadline for all tdocs (e.g. including summary tdocs).
General Tdoc Submission Deadline, as usual. Kick off, summaries.

Oct 27 0700 UTC	Tdocs submission deadline for Summaries (baseline version)

Nov 02 0700 UTC	e-Meeting Start (by email) (Nov 03 0700 UTC is first possible email deadline).

Nov 06 1000 UTC 	Suspend decision making in email discussions (= no deadlines etc)

It should be possible for a delegate to take the weekend off, rejoin and not miss decisions.

Nov 09 1000 UTC	Resume decision making in email discussions.

Nov 13 1100 UTC	e-Meeting Stop, no more email comments for AT-meeting email discussions. Decision			confirmations announced within 24h. Session notes for email checking.

Nov 20 1100 UTC		Deadline Short Post112-e email approvals of documents for RP.

Web Conference Schedule

Note that this schedule is indicative and can change. Changes to the schedule will be announced with notice of at least 24h.

| Time Zone UTC | Web Conference R2 - Main | Web Conference R2 - BO1 | Web Conference R2 - BO2 |
| --- | --- | --- | --- |
| Monday |  |  |  |
| 13:00 – 14:30 | General (opportunity for Questions if needed, short 10min) NR16 [6.1.1]:SI acquisition Kick-off NR15 CP (Johan) | NR16 2-step, PowSav (Diana) | NR16 V2X (Kyeognin) |
| 14:30 – 16:00 | NR17 DCCA FEnh (Tero) | NR16 NR-U, Including UE caps for unlicensed (Diana) | LTE16 and earlier IoT (Brian, Emre) |
| Tuesday |  |  |  |
| 13:00 – 14:30 | NR16 [6.1] General and UE caps kick-off (Johan) NR16 [6.15][6.16] | NR17 NTN (Sergio) | NR16 and earlier Pos (Nathan) |
| 14:30 – 16:00 | NR16 IIOT (Johan) | NR16 L1 Centric (Sergio) | NR17 Pos SI (Nathan) |
| Wednesd |  |  |  |
| 13:00 – 14:30 | NR17 Multi-SIM (Tero) | NR17 Red Cap SI (Sergio) | NR17 SL Relay SI (Nathan) |
| 14:30 – 16:00 | NR16 IAB (Johan) NR16 [6.1][6.15][6.16] | NR16 Other CP Centric (Sergio) | LTE17 IoT (Brian) |
| Thursday |  |  |  |
| 13:00 – 14:30 | NR16 DCCA (Tero) | NR17 Small Data Enh (Diana) | NR16 V2X, LTE 16 and earlier V2X SL (Kyeongin) |
| 14:30 – 16:00 | NR17 Multicast (Johan) | LTE16 and earlier IoT (Brian, Emre) | NR16 Pos (Nathan) NR17 Pos SI (Nathan) (if time) |
| Friday |  |  |  |
| 04:30-06:00 | NR17 Multicast (Johan) | NR16 Mob, LTE16 Mob (Tero) | NR17 SON MDT (HuNan) |

| Time Zone UTC | Web Conference R2 - Main | Web Conference R2 - BO1 | Web Conference R2 - BO2 |
| --- | --- | --- | --- |
| Monday |  |  |  |
| 14:00 – 15:30 | NR17 UE Power Saving (Johan) | NR17 RAN Slicing SI (Tero) | NR17 SL enh (Kyeongin) |
| 15:30 – 17:00 | NR16 General, UE caps, R4 items (Johan) | NR17 IIOT URLLC (Diana) | NR16 SON/MDT (HuNan) |
| Tuesday |  |  |  |
| 14:00 – 15:30 | LTE16 and earlier General (Tero) | NR17 NTN (Sergio) | Pos CB (Nathan) |
| 15:30 – 17:00 | NR16 MobEnh (Tero) LTE16 MobEnh (Tero) NR16 DCCA (Tero) | NR17 Small data + CB (Diana) | NR17 SL enh (Kyeongin) |
| Wednesd |  |  |  |
| 14:00 – 15:30 | NR17 IAB (Johan) | CB NR16 (Sergio) | LTE16 IoT (Emre, Brian) |
| 15:30 – 17:00 | CB (Johan) | NR16 2-step PowSav NR-U CB (Diana) | NR17 SL Relay SI + CB (Nathan) |
| Thursday |  |  |  |
| 05:00 – 06:30 | CB (Johan) | CB (Kyeongin) | CB (Brian/Emre) |
| Friday |  |  |  |
| 05:00 – 06:30 | CB (Nathan) CB (HuNan) | CB (Diana) CB (Sergio) | CB (Tero) |

# 4	EUTRA corrections Rel-15 and earlier

See Appendix A for reference to Work items, work item codes and WIDs.

Only essential corrections. No documents should be submitted to 4. Please submit to 4.x

## 4.5	Other LTE corrections Rel-15 and earlier

Documents in this agenda item will be handled in a break out session.

Editorial corrections should be taken up with the specification editor before submitting to avoid CR duplication.

Rel-12: Intra-band contiguous CA capabilities (discussed already several times in previous meetings)

Web Conf (1)

R2-2009428	Clarification to UE capabilities for non-contiguous intra-band CA	Nokia, Nokia Shanghai Bell, Qualcomm Incorporated	CR	Rel-12	36.331	12.19.0	4427	2	F	LTE_CA-Core, TEI12	R2-2008152

R2-2009429	Clarification to UE capabilities for non-contiguous intra-band CA	Nokia, Nokia Shanghai Bell, Qualcomm Incorporated	CR	Rel-13	36.331	13.16.0	4428	2	A	LTE_CA-Core, TEI12	R2-2008153

R2-2009430	Clarification to UE capabilities for non-contiguous intra-band CA	Nokia, Nokia Shanghai Bell, Qualcomm Incorporated	CR	Rel-14	36.331	14.15.0	4429	2	A	LTE_CA-Core, TEI12	R2-2008154

R2-2009431	Clarification to UE capabilities for non-contiguous intra-band CA	Nokia, Nokia Shanghai Bell, Qualcomm Incorporated	CR	Rel-15	36.331	15.11.0	4430	2	A	LTE_CA-Core, TEI12	R2-2008155

R2-2009432	Clarification to UE capabilities for non-contiguous intra-band CA	Nokia, Nokia Shanghai Bell, Qualcomm Incorporated	CR	Rel-16	36.331	16.2.1	4431	2	A	LTE_CA-Core, TEI12	R2-2008156

Online discussion (2nd week Tuesday)

By Email [201] (2)

Rel-13: RRC resume with CIoT:

R2-2009763	Correction to RRC resume for CIoT	Google Inc.	CR	Rel-13	36.331	13.16.0	4484	-	F	TEI13

R2-2009764	Correction to RRC resume for CIoT	Google Inc.	CR	Rel-14	36.331	14.15.0	4485	-	A	TEI13

[AT112-e][201][LTE] LTE Miscellaneous corrections (RAN2 VC)

Scope:

    - Discuss the CRs under AI 4.5, 7.1.X and 7.5 marked for this email discussion

Intended outcome:

    - Discussion summary in R2-2010710 (by email rapporteur)

    - Agreeable CRs by proponents (if revised versions are required, proponents should obtain Tdoc numbers from session chair or RAN2 secretary to provide those)

Deadline for providing comments and for rapporteur inputs:

    - Initial deadline (for companies' feedback):  1st week Fri, UTC 0900

    - Initial deadline (for rapporteur's summary in R2-2010710):  2nd week Mon, UTC 13:00

    - Deadline for CR finalization: 2nd week Thu, UTC 1000

Rel-14: Delay budget reporting

R2-2008901	Removal of DelayBudgetReport message in stage 3	Lenovo, Motorola Mobility	CR	Rel-14	36.331	14.15.0	4450	-	F	LTE_VoLTE_ViLTE_enh-Core

R2-2008902	Removal of DelayBudgetReport message in stage 3	Lenovo, Motorola Mobility	CR	Rel-15	36.331	15.11.0	4451	-	A	LTE_VoLTE_ViLTE_enh-Core

R2-2008903	Removal of DelayBudgetReport message in stage 3	Lenovo, Motorola Mobility	CR	Rel-16	36.331	16.2.1	4452	-	A	LTE_VoLTE_ViLTE_enh-Core

Email 201

By Email [201] (1)

Rel-14: Recommended bitrate query at MAC reset:

R2-2010153	Recommended bit rate query handling at MAC Reset	Ericsson	CR	Rel-14	36.321	14.13.0	1513	-	F	LTE_VoLTE_ViLTE_enh

R2-2010154	Recommended bit rate query handling at MAC Reset	Ericsson	CR	Rel-15	36.321	15.10.0	1514	-	F	LTE_VoLTE_ViLTE_enh

R2-2010155	Recommended bit rate query handling at MAC Reset	Ericsson	CR	Rel-16	36.321	16.2.0	1515	-	F	LTE_VoLTE_ViLTE_enh

Email 201

By Email [203] (3+3+3)

Rel-15: RLC out-of-order delivery impact to PDCP re-establishment (postponed last time)

R2-2009565	PDCP re-establishment for normal DRBs configured with RLC OOD and ROHC	Samsung	discussion	Rel-15	TEI15, LTE_HRLLC-Core

R2-2009566	CR on PDCP re-establishment when t-Reordering is used	Samsung	CR	Rel-15	36.323	15.6.0	0292	-	F	TEI15, LTE_HRLLC-Core

R2-2009567	CR on PDCP re-establishment when t-Reordering is used	Samsung	CR	Rel-16	36.323	16.2.0	0293	-	F	TEI16, LTE_HRLLC-Core

Email 203

[AT112-e][203][LTE] LTE corrections related to RLC out-of-order delivery (Samsung)

Scope:

    - Discuss the CRs under AI 4.5 related to the RLC out-of-order delivery that are marked for this email discussion to determine which changes are acceptable

Intended outcome:

    - Discussion summary in R2-2010714 (by email rapporteur), agreeable CRs (Tdoc numbers can be obtained from session chair if needed)

Deadline for providing comments and for rapporteur inputs:

    - Initial deadline (for companies' feedback):  1st week Fri, UTC 0900

    - Initial deadline (for rapporteur's summary in R2-2010714):  2nd week Mon, UTC 13:00

Rel-15: RoHC configuration with RLC out-of-order delivery:

R2-2009568	Clarification on ROHC configuration	Samsung	discussion	Rel-15	TEI15, LTE_HRLLC-Core

R2-2009569	Correction on ROHC configuration	Samsung	CR	Rel-15	36.331	15.11.0	4470	-	F	TEI15, LTE_HRLLC-Core

R2-2009570	Correction on ROHC configuration	Samsung	CR	Rel-16	36.331	16.2.1	4471	-	F	TEI16, LTE_HRLLC-Core

Email 203

Rel-15: MAC LCH restrictions with RLC out-of-order delivery:

R2-2009571	Correction on lch-CellRestriction	Samsung	CR	Rel-15	36.321	15.10.0	1511	-	F	TEI15, LTE_HRLLC-Core

R2-2009572	Correction on lch-CellRestriction	Samsung	CR	Rel-16	36.321	16.2.0	1512	-	F	TEI16, LTE_HRLLC-Core

Email 203

By Email [202] (1)

Rel-15: Stage-2 rapporteur CR:

R2-2009801	Miscellaneous Stage-2 corrections	Nokia (rapporteur), NEC	CR	Rel-15	36.300	15.11.0	1323	-	F	NB_IOTenh2-Core, LTE_eMTC4-Core, TEI15

Email 202

By Email [202] (1)

R2-2008904	Removal of DelayBudgetReport message in stage 2	Lenovo, Motorola Mobility	CR	Rel-14	36.300	14.12.0	1317	-	F	LTE_VoLTE_ViLTE_enh-Core

R2-2008905	Removal of DelayBudgetReport message in stage 2	Lenovo, Motorola Mobility	CR	Rel-15	36.300	15.11.0	1318	-	A	LTE_VoLTE_ViLTE_enh-Core

R2-2008906	Removal of DelayBudgetReport message in stage 2	Lenovo, Motorola Mobility	CR	Rel-16	36.300	16.3.0	1319	-	A	LTE_VoLTE_ViLTE_enh-Core

Email 202

[AT112-e][202][LTE] LTE editorial corrections (RAN2 VC)

Scope:

    - Discuss the CRs under AI 4.5, 7.1.X and 7.5 marked for this email discussion

Intended outcome:

    - Discussion summary in R2-2010711 (by email rapporteur)

    - Agreeable CRs for 36.300, 36.306 and 36.331 (if any) by specification rapporteurs (after online session)

Deadline for providing comments and for rapporteur inputs:

    - Initial deadline (for companies' feedback):  1st week Fri, UTC 0900

    - Initial deadline (for rapporteur's summary in R2-2010711):  2nd week Mon, UTC 13:00

    - Deadline for CR finalization: 2nd week Thu, UTC 1000

By Email [204] (2)

Rel-15: TDD/FDD capability differentiation (postponed earlier, waiting for RAN1 LS reply):

R2-2009921	Corrections to the field descriptions for TDD/FDD capability differentiation	Huawei, HiSilicon	CR	Rel-15	36.331	15.11.0	4389	2	F	TEI15	R2-2008157

Revised in R2-2010735

R2-2009922	Corrections to the field descriptions for TDD/FDD capability differentiation	Huawei, HiSilicon	CR	Rel-16	36.331	16.2.1	4390	2	A	TEI15	R2-2008158

Revised in R2-2010736

[AT112-e][204][LTE] LTE corrections to TDD/FDD capability differentiation (Huawei)

Scope:

    - Progress the revisions to R2-2009921 and R2-2009922 based on RAN1 LS R2-2011001

Intended outcome:

    - Revised CRs in R2-2010735 (Rel-15, 36.331) and R2-2010736  (Rel-16, 36.331)

Deadline for providing comments and for rapporteur inputs:

    - Initial deadline (for companies' feedback):  2nd week Mon, UTC 13:00

    - Initial deadline (for revised CRs):  2nd week Tue, UTC 13:00

Web Conf (2)

Late LS reply from RAN1:

R2-2011001	Reply LS on Incomplete LTE Physical Layer Capabilities (R1-2009435; contact: Huawei)

Online (should be taken into account in email [204])

R2-2010735	Corrections to the field descriptions for TDD/FDD capability differentiation	Huawei, HiSilicon	CR	Rel-15	36.331	15.11.0	4389	3	F	TEI15	R2-2009921

R2-2010736	Corrections to the field descriptions for TDD/FDD capability differentiation	Huawei, HiSilicon	CR	Rel-16	36.331	16.2.1	4390	3	A	TEI15	R2-2009922

Online

# 6	Rel-16 NR Work Items

Essential corrections. While high maintenance intensity is expected, Rel-16 corrections are treated separately per WI.

## 6.7	NR mobility enhancements

(NR_Mob_enh-Core; leading WG: RAN2; REL-16; started: Jun 18; Completed June 20; WID: RP-192277; SR RP-201273). Documents in this agenda item will be handled in a break out session).

Documents under 6.7 will be treated together with documents in 7.4.

No documents should be submitted to 6.7. Please submit to 6.7.x

Editorial corrections should be taken up with the specification editor before submitting to avoid CR duplication.

Limit: 8 email threads (with 7.4)

### 6.7.1	General and Stage-2 Corrections

Including incoming LSs (if any).

By Email [210] (2+5)

Stage-2 rapporteur inputs

R2-2009312	Miscellaneous corrections to Mobility Enhancements	Nokia (Rapporteur), Ericsson, Intel Corporation, Nokia Shanghai Bell, Sanechips, ZTE	CR	Rel-16	38.300	16.3.0	0305	-	F	NR_Mob_enh-Core

R2-2010354	Miscellaneous corrections for Mobility Enhancements	ZTE Corporation, Sanechips, Ericsson	CR	Rel-16	37.340	16.3.0	0236	-	F	NR_Mob_enh-Core

Email 210

CHO/CPC DAPS Stage-2 corrections:

R2-2009386	Clarification on CHO in LTE-DC	ZTE Corporation, Sanechips	CR	Rel-16	36.300	16.3.0	1321	-	F	LTE_feMob-Core

R2-2009995	Clarification of CHO simultaneous with DAPS	Ericsson	discussion	NR_Mob_enh-Core

R2-2010187	Correction on TS 38.300 for CHO	Huawei, HiSilicon	CR	Rel-16	38.300	16.3.0	0314	-	F	NR_Mob_enh-Core

R2-2010188	Correction on TS 36.300 for CHO	Huawei, HiSilicon	CR	Rel-16	36.300	16.3.0	1326	-	F	LTE_feMob-Core

R2-2010651	Correction to RLF in case of DAPS HO	Samsung Electronics Co., Ltd	CR	Rel-16	38.300	16.3.0	0322	-	F	NR_Mob_enh-Core

Email 210

[AT112-e][210][MOB] Stage-2 corrections (Nokia)

Scope:

    - Discuss which mobility WI - related Stage-2 corrections (for LTE, MR-DC and NR) are seen necessary

Intended outcome:

    - Discussion summary in R2-2010715 (by email rapporteur).

    - Merged CRs to 36.300 (R2-2010716), 38.300 (R2-2010717) and 37.340 (R2-2010718) (if any)

Deadline for providing comments, for rapporteur inputs, conclusions and CR finalization:

    - Initial deadline (for companies' feedback):  1st week Fri, UTC 0900

    - Initial deadline (for rapporteur's summary in R2-2010715):  2nd week Mon, UTC 13:00

    - Deadline for CR finalization: 2nd week Thu, UTC 1000

### 6.7.2	Conditional handover related corrections

This AI jointly addresses corrections to NR and LTE CHO.

Web Conf (1+1+1)

Editorials:

R2-2010229	Support of Rel-16 features for SCG in EN-DC and NR-DC	Huawei, HiSilicon	CR	Rel-16	38.331	16.2.0	2192	-	F	NR_IAB-Core, NR_Mob_enh-Core

Moved from 6.1.3 to 6.2.4, then Moved from 6.2.4 to here

UE compliance check failure for CHO command:

R2-2009998	Inability to comply with conditional reconfiguration	Ericsson	CR	Rel-16	38.331	16.2.0	2154	-	F	NR_Mob_enh-Core

SI reading during CHO recovery:

R2-2010189	Correction on TS 38.331 for CHO	Huawei, HiSilicon	CR	Rel-16	38.331	16.2.0	2185	-	F	NR_Mob_enh-Core

CHO with the "1 second rule" for UE assistance information:

R2-2010253	UE information transmission in NR CHO case	SHARP Corporation, Ericsson	discussion	Rel-16	NR_Mob_enh-Core	R2-2007718

R2-2010251	UE information transmission in LTE CHO case	SHARP Corporation, Ericsson	discussion	Rel-16	LTE_feMob-Core

(moved from 7.4.4)

R2-2010254	Clarification on UE information transmission in CHO case(38.331)	SHARP Corporation, Ericsson	CR	Rel-16	38.331	16.2.0	2194	-	F	NR_Mob_enh-Core

R2-2010252	Clarification on UE information transmission in CHO case(36.331)	SHARP Corporation, Ericsson	CR	Rel-16	36.331	16.2.1	4503	-	F	LTE_feMob-Core

(moved from 7.4.4)

By Email [211] (3+2+2+2+1)

Small corrections, NR RRC affecting ASN.1 parts:

R2-2009996	Missing release of VarConditionalReconfig	Ericsson	CR	Rel-16	38.331	16.2.0	2153	-	F	NR_Mob_enh-Core

R2-2009533	Correction on configuration of triggerCondition for CHO	CATT	CR	Rel-16	36.331	16.2.1	4466	-	F	LTE_feMob-Core

R2-2009848	Correction to attemptCondReconfig in ConditionalReconfiguration	Ericsson	CR	Rel-16	38.331	16.2.0	2140	-	F	NR_Mob_enh-Core

Email 211

Small corrections, NR RRC procedural text

R2-2009640	Correction to remove conditional reconfiguration related measurement configuration	ITRI	CR	Rel-16	38.331	16.2.0	2100	-	F	NR_Mob_enh-Core

R2-2009639	Correction to conditional reconfiguration evaluation	ITRI	CR	Rel-16	38.331	16.2.0	2099	-	F	NR_Mob_enh-Core

Email 211

Small corrections, LTE RRC affecting procedural text:

R2-2009997	Missing release of VarConditionalReconfiguration	Ericsson	CR	Rel-16	36.331	16.2.1	4491	-	F	NR_Mob_enh-Core

R2-2010190	Correction on TS 36.331 for CHO	Huawei, HiSilicon	CR	Rel-16	36.331	16.2.1	4498	-	F	LTE_feMob-Core

Email 211

SRB COUNT during CHO recovery:

R2-2010205	Issue on failure handling of handover without key change for the UE configured with attemptCondReconfig	SHARP Corporation	discussion	Rel-16	NR_Mob_enh-Core

R2-2010206	Correction of reconfiguration with sync failure procedure for the UE configured with attemptCondReconfig	SHARP Corporation	CR	Rel-16	38.331	16.2.0	2190	-	F	NR_Mob_enh-Core

Email 211

Optimizations (adding PCI outside the CHO configuration):

R2-2009472	Target cell ID parsing in CHO and CPAC	Apple	CR	Rel-16	38.331	16.2.0	2080	-	F	NR_Mob_enh-Core

Email 211

[AT112-e][211][MOB] CHO/CPC RRC corrections (Intel)

Scope:

    - Discuss which CHO/CPC corrections for 36.331/38.331 are seen necessary and provide merged CRs with agreeable corrections (if any)

Intended outcome:

    - Discussion summary in R2-2010719 (by email rapporteur).

    - Merged CRs to 36.331 (R2-2010720) and 38.331 (R2-2010721) (if any)

Deadline for providing comments, for rapporteur inputs, conclusions and CR finalization:

    - Initial deadline (for companies' feedback):  1st week Fri, UTC 0900

    - Initial deadline (for rapporteur's summary in R2-2010719):  2nd week Mon, UTC 13:00

    - Deadline for CR finalization: 2nd week Thu, UTC 1000

### 6.7.3	Conditional PSCell change for intra-SN corrections

Including corrections for CPC.

By Email [210] (1)

R2-2009766	Corrections to CPC with and without SRB3 involved	Nokia, Nokia Shanghai Bell, ZTE Corporation (Rapporteur)	CR	Rel-16	37.340	16.3.0	0220	1	F	NR_Mob_enh-Core	R2-2007360

Email 210

By Email [211] (1)

R2-2010589	Correction to CG-Config for CPC	Google Inc.	CR	Rel-16	38.331	16.2.0	2251	-	F	NR_Mob_enh-Core

Email 211

### 6.7.4	UE capability corrections

Including UE capability aspects of NR mobility WI and joint LTE/NR capability corrections..

Including outcome of [Post111-e][921][DAPS] DAPS capability structure clarifications (Huawei)

Web Conf (4+1)

Outcome of [Post111-e][921][DAPS] DAPS capability structure clarifications (Huawei)

R2-2010292	Report of [Post111-e][921][DAPS] DAPS capability structure clarifications (Huawei)	Huawei, HiSilicon	discussion	Rel-16	NR_Mob_enh-Core

R2-2009783	UE Capabilities for Intra-frequency DAPS Handover	MediaTek Inc.	discussion

R2-2010500	Remaining open issues for DAPS capabilities	Ericsson	discussion

R2-2008827	NR DAPS capability corrections	Nokia, Nokia Shanghai Bell	discussion	NR_Mob_enh-Core

R2-2010293	Clarification on NR DAPS UE capability	Huawei, HiSilicon	CR	Rel-16	38.306	16.2.0	0442	-	F	NR_Mob_enh-Core

By Email [212] (1+3)

DAPS capabilities:

R2-2009655	Correction on CA-ParametersNR for DAPS handover	NEC	draftCR	Rel-16	38.331	16.2.0	NR_Mob_enh-Core

Email 212

CHP/CPC capabilities:

R2-2009273	The supported combination among FRx/xDD CHO/CPC capabilities	Intel Corporation	discussion	Rel-16	NR_Mob_enh-Core, LTE_feMob-Core

R2-2009281	Clarification on the setting of FRx&xDD CHO&CPC capabilities	Intel Corporation	CR	Rel-16	38.306	16.2.0	0423	-	F	NR_Mob_enh-Core

R2-2010296	Clarification on CHO and CPC capabilities between different modes	Huawei, HiSilicon	CR	Rel-16	38.306	16.2.0	0443	-	F	NR_Mob_enh-Core

Email 212

[AT112-e][212][MOB] Mobility UE capabilities for LTE and NR (Huawei)

Scope:

    - Discuss which UE capability corrections to LTE and NR are seen necessary and provide merged CRs with agreeable corrections (if any)

Intended outcome:

    - Discussion summary in R2-2010722 (by email rapporteur).

    - Merged CRs to 36.306 (R2-2010723), 36.331 (R2-2010724), 38.306 (R2-2010725) and 38.331 (R2-2010726) (if any)

Deadline for providing comments, for rapporteur inputs, conclusions and CR finalization:

    - Initial deadline (for companies' feedback):  1st week Fri, UTC 0900

    - Initial deadline (for rapporteur's summary in R2-2010722):  2nd week Mon, UTC 13:00

    - Deadline for CR finalization: 2nd week Thu, UTC 1000

Withdrawn:

R2-2008828	NR DAPS capability corrections	Nokia, Nokia Shanghai Bell	CR	Rel-16	38.306	16.2.0	0413	-	F	NR_Mob_enh-Core	Withdrawn

R2-2008829	NR DAPS capability corrections	Nokia, Nokia Shanghai Bell	CR	Rel-16	38.331	16.2.0	2018	-	F	NR_Mob_enh-Core	Withdrawn

### 6.7.5	Other

Including corrections to DAPS that are NR-specific without equivalent LTE impacts

By Email [213] (2)

R2-2009665	Minor corrections to NR mobility enhancements	Lenovo, Motorola Mobility	CR	Rel-16	38.331	16.2.0	2102	-	F	NR_Mob_enh-Core

R2-2010415	Correction on DAPS power configuration	Google Inc.	CR	Rel-16	38.331	16.2.0	2218	-	F	NR_Mob_enh-Core

Email 213

## 6.8	DC and CA enhancements

(LTE_NR_DC_CA_enh-Core; leading WG: RAN2; REL-16; started: Jun 18; Target Aug 20; WI RP-200791, SR: RP-201218) R1 and R2 parts are 100% complete.

No documents should be submitted to 6.8. Please submit to 6.8.x

Editorial corrections should be taken up with the specification editor before submitting to avoid CR duplication.

Limit: 5 email threads

### 6.8.1 	General and Stage-2 Corrections

Including incoming LSs rapporteur inputs, including corrections discussions going beyond a specific TS, cross group discussions.

Web Conf (1+1)

R2-2008706	Reply LS on UL PC for NR-DC (R1-2007261; contact: Apple)	RAN1	LS in	Rel-16	LTE_NR_DC_CA_enh-Core	To:RAN2

R2-2008736	Reply LS on power control for NR-DC (R4-2011721; contact: vivo)	RAN4	LS in	Rel-16	LTE_NR_DC_CA_enh-Core	To:RAN2	Cc:RAN1

Flagging procedure will be used for these LSs (i.e. all will be noted without presentation unless flagged)

Noted (without presentation) - See contributions in AI 6.8.4

R2-2008744	LS response on measurement capability for EMR (R4-2012112; contact: Ericsson)	RAN4	LS in	Rel-16	LTE_NR_DC_CA_enh-Core	To:RAN2

R2-2008750	LS on EMR measurement requirements in NR (R4-2012297; contact: Ericsson)	RAN4	LS in	Rel-16	LTE_NR_DC_CA_enh-Core	To:RAN2

Flagging procedure will be used for these LSs (i.e. all will be noted without presentation unless flagged)

Noted (for information only, no presentation)

Web Conf (2)

R2-2009548	CR for 37.340 on power control  for NR_DC	Nokia, Nokia Shanghai Bell, Samsung, Ericsson	CR	Rel-16	37.340	16.3.0	0235	-	B	LTE_NR_DC_CA_enh-Core

R2-2010647	Miscellaneous corrections for RRC Transfer procedure 	Samsung R&D Institute UK	CR	Rel-16	37.340	16.3.0	0237	-	F	LTE_NR_DC_CA_enh-Core

(moved from 6.8.4)

By Email [22x] (4)

Rapporteur CRs, for merging editorial inputs:

R2-2010018	Misc corrections for Rel-16 DCCA	Ericsson	CR	Rel-16	38.331	16.2.0	2161	-	F	LTE_NR_DC_CA_enh-Core

R2-2010019	Misc corrections for Rel-16 DCCA	Ericsson	CR	Rel-16	36.331	16.2.1	4492	-	F	LTE_NR_DC_CA_enh-Core

R2-2010020	Misc corrections for Rel-16 DCCA	Ericsson	CR	Rel-16	38.300	16.3.0	0312	-	F	LTE_NR_DC_CA_enh-Core	Late

R2-2010021	Misc corrections for Rel-16 DCCA	Ericsson	CR	Rel-16	36.300	16.3.0	1325	-	F	LTE_NR_DC_CA_enh-Core	Late

TBD how to handle the editorial CRs - Only discussed during the 2nd meeting week

### 6.8.2	Fast Scell activation

By Email [221] (5)

SCell dormancy, MAC corrections:

R2-2009549	Dormancy correction	Nokia, Nokia Shanghai Bell	CR	Rel-16	38.321	16.2.0	0934	-	F	LTE_NR_DC_CA_enh-Core

R2-2009573	Corrections on bwp-InactivityTimer	Samsung	CR	Rel-16	38.321	16.2.1	0935	-	F	LTE_NR_DC_CA_enh-Core

R2-2008927	Correction on RA upon BWP switching to dormant BWP	Asia Pacific Telecom co. Ltd	CR	Rel-16	38.321	16.2.1	0901	-	F	LTE_NR_DC_CA_enh-Core

R2-2010022	Timing of direct SCell activation upon RRC configuration	Ericsson	CR	Rel-16	38.321	16.2.1	0956	-	F	LTE_NR_DC_CA_enh-Core

Email 221

SCell dormancy, UE capabilities:

R2-2009550	BWP support for dormancy	Nokia, Nokia Shanghai Bell	discussion	Rel-16	LTE_NR_DC_CA_enh-Core

Email 221

[AT112-e][221][DCCA] Fast Scell activation and early measurements (Nokia)

Scope:

    - Discuss corrections under 6.8.2/6.8.3 marked for this discussion to see which CRs could be agreeable

Intended outcome:

    - Discussion summary in R2-2010731 (by email rapporteur).

Deadline for providing comments, for rapporteur inputs, conclusions and CR finalization:

    - Initial deadline (for companies' feedback):  1st week Fri, UTC 0900

    - Initial deadline (for rapporteur's summary in R2-2010731):  2nd week Mon, UTC 13:00

### 6.8.3	Early measurement reporting

By Email [220] (1)

Editorial corrections:

R2-2009352	Miscellaneous corrections on early measurement reporting in 38.331	CATT	CR	Rel-16	38.331	16.2.0	2056	-	F	LTE_NR_DC_CA_enh-Core

R2-2009353	Miscellaneous corrections on early measurement reporting in 36.331	CATT	CR	Rel-16	36.331	16.2.1	4460	-	F	LTE_NR_DC_CA_enh-Core

Email 220

[AT112-e][220][DCCA] Simple DCCA corrections (Ericsson)

Scope:

    - Discuss DCCA corrections under 6.8.1/6.8.3/6.8.4/6.8.5 marked for the discussion to see which CRs could be agreeable

Intended outcome:

    - Discussion summary in R2-2010730 (by email rapporteur).

Deadline for providing comments, for rapporteur inputs, conclusions and CR finalization:

    - Initial deadline (for companies' feedback):  1st week Fri, UTC 0900

    - Initial deadline (for rapporteur's summary in R2-2010730):  2nd week Mon, UTC 13:00

By Email [221] (2)

Applicability to serving carrier measurements:

R2-2009551	Measurement applicability and validity	Nokia, Nokia Shanghai Bell	CR	Rel-15	36.331	15.11.0	4468	-	F	LTE_euCA-Core

R2-2009552	Measurement applicability and validity	Nokia, Nokia Shanghai Bell	CR	Rel-16	36.331	16.2.1	4469	-	F	LTE_euCA-Core, LTE_NR_DC_CA_enh-Core

R2-2009553	Measurement applicability and validity	Nokia, Nokia Shanghai Bell	CR	Rel-16	38.331	16.2.0	2090	-	F	LTE_NR_DC_CA_enh-Core

Email 221

Clarification to IDLE mode measurement storing procedural text:

R2-2010023	Serving cell results for early measurements	Ericsson	CR	Rel-16	38.331	16.2.0	2162	-	F	LTE_NR_DC_CA_enh-Core

Email 221

By Email [221] (1)

Indication of T331 expiration in measurements (related to RAN4 LS R2-2008750 on EMR requirements):

R2-2010024	Early measurement requirements	Ericsson	discussion	LTE_NR_DC_CA_enh-Core

Email 221

By Email [221] (1)

Usage of SIB indication for early measurements:

R2-2010653	Reporting of dle/inactive measurement not obtained in the current cell	Huawei, HiSilicon	CR	Rel-16	36.331	16.2.1	4528	-	F	LTE_NR_DC_CA_enh-Core

R2-2010654	Reporting of dle/inactive measurement not obtained in the current cell	Huawei, HiSilicon	CR	Rel-16	38.331	16.2.0	2268	-	F	LTE_NR_DC_CA_enh-Core

Email 221

### 6.8.4	Other DCCA corrections

Including NR-NR DC, MCG SCell and SCG configuration with RRC resume, Fast MCG link recovery, and RRC corrections that doesn’t fit under the other headings.

Including outcome of [Post111-e][918][DCCA] SCell SMTC window for Unaligned CA (CMCC)

Including capability signalling based on agreements in RP-202030.

Web Conf (Email disc)

Outcome of [Post111-e][918][DCCA] SCell SMTC window for Unaligned CA (CMCC):

R2-2010378	Summary of [Post111-e][918][R16 DCCA] SCell SMTC window for Unaligned CA (CMCC)	CMCC	discussion	Rel-16	LTE_NR_DC_CA_enh-Core

R2-2010379	CR for Unaligned CA in TS 38.331	CMCC,MediaTek Inc.	CR	Rel-16	38.331	16.2.0	2212	-	C	LTE_NR_DC_CA_enh-Core

R2-2010380	CR for Unaligned CA in TS 38.306	CMCC,MediaTek Inc.	CR	Rel-16	38.306	16.2.0	0447	-	C	LTE_NR_DC_CA_enh-Core

Web Conf (1)

Other unaligned CA corrections:

R2-2008968	Clarification of NR-DC with unaligned CA 	Qualcomm Incorporated 	discussion	Rel-16	LTE_NR_DC_CA_enh-Core

Web Conf (2+1)

Toffset:

R2-2010025	Missing fields for Toffset coordination	Ericsson, Nokia, Nokia Shanghai Bell	CR	Rel-16	38.331	16.2.0	2163	-	F	LTE_NR_DC_CA_enh-Core

R2-2010115	Remaining issues on Toffset for NR-DC power control	Huawei, HiSilicon	discussion	Rel-16	LTE_NR_DC_CA_enh-Core

FR2 maximum power:

R2-2010027	Correction on p-UE-FR2 for NR-DC power control	Ericsson, NTTDOCOMO	CR	Rel-16	38.331	16.2.0	2165	-	F	LTE_NR_DC_CA_enh-Core	Revised

R2-2010112	Correction on p-UE-FR2 for NR-DC power control	Ericsson, NTTDOCOMO	CR	Rel-16	38.331	16.2.0	2165	1	F	LTE_NR_DC_CA_enh-Core	R2-2010027

R2-2010291	Correction on p-UE-FR2 in NR-DC power control	vivo	CR	Rel-16	38.331	16.2.0	2201	-	F	LTE_NR_DC_CA_enh-Core

R2-2010340	Correction on p-UE-FR2 for NR-DC power control in FR2	Huawei, HiSilicon	CR	Rel-16	38.331	16.2.0	2207	-	F	LTE_NR_DC_CA_enh-Core

By Email [220] (3)

Miscellaneous DCCA corrections:

R2-2010026	Correction on sk-counter in RRCResume	Ericsson	CR	Rel-16	38.331	16.2.0	2164	-	F	LTE_NR_DC_CA_enh-Core

R2-2009354	Miscellaneous corrections for Rel-16 DCCA in 38.331	CATT	CR	Rel-16	38.331	16.2.0	2057	-	F	LTE_NR_DC_CA_enh-Core

R2-2010120	Miscellaneous corrections for DCCA	Huawei, HiSilicon	CR	Rel-16	36.331	16.2.1	4497	-	F	LTE_NR_DC_CA_enh-Core

R2-2009415	Correction on tdm-PatternConfig2 configuration upon MR-DC Release	MediaTek Inc.	CR	Rel-16	36.331	16.2.1	4462	-	F	LTE_NR_DC_CA_enh-Core

Email 220

By Email [222] (2)

Resume with SCG:

R2-2010116	Correction on SCG-related fields in RRCConnection Resume	Huawei, HiSilicon	CR	Rel-16	36.331	16.2.1	4495	-	F	LTE_NR_DC_CA_enh-Core

R2-2010121	Corrections for resume with SCG	Huawei, HiSilicon	CR	Rel-16	38.331	16.2.0	2179	-	F	LTE_NR_DC_CA_enh-Core

Email 222

[AT112-e][222][DCCA] Miscellaneous DCCA corrections and capabilities (Ericsson)

Scope:

    - Discuss DCCA corrections under 6.8.4 marked for this discussion to see which CRs could be agreeable

Intended outcome:

    - Discussion summary in R2-2010732 (by email rapporteur).

Deadline for providing comments, for rapporteur inputs, conclusions and CR finalization:

    - Initial deadline (for companies' feedback):  1st week Fri, UTC 0900

    - Initial deadline (for rapporteur's summary in R2-2010732):  2nd week Mon, UTC 13:00

By Email [222] (5)

Fast MCG recovery:

R2-2010117	Correction for fast MCG link recovery via SRB3 in NR-DC	Huawei, HiSilicon	CR	Rel-16	38.331	16.2.0	2177	-	F	LTE_NR_DC_CA_enh-Core

R2-2010566	Clarification on ULInformationTransferMRDC	Google Inc.	CR	Rel-16	38.331	16.2.0	2247	-	F	NR_Mob_enh-Core, LTE_NR_DC_CA_enh-Core

R2-2010650	Corrections on messages encapsulated in ULInformationTransferMRDC 	Samsung R&D Institute UK	CR	Rel-16	36.331	16.2.1	4527	-	F	LTE_NR_DC_CA_enh-Core

R2-2010122	Correction for fast MCG link recovery in (NG)EN-DC	Huawei, HiSilicon	CR	Rel-16	38.331	16.2.0	2180	-	F	LTE_NR_DC_CA_enh-Core

R2-2010255	UE information transmission in LTE fast MCG recovery case	SHARP Corporation	discussion	Rel-16	LTE_NR_DC_CA_enh-Core

R2-2010256	Clarification on UE information transmission in fast MCG recovery case(36.331)	SHARP Corporation	CR	Rel-16	36.331	16.2.1	4504	-	F	LTE_NR_DC_CA_enh-Core

Email 222

By Email [222] (1)

Missing RRC processing delay requirements:

R2-2010028	Processing delay requirements for DLInformationTransferMRDC	Ericsson	CR	Rel-16	38.331	16.2.0	2166	-	F	LTE_NR_DC_CA_enh-Core

R2-2010118	Processing delay requirements for RRC resume	Huawei, HiSilicon	CR	Rel-16	38.331	16.2.0	2178	-	C	LTE_NR_DC_CA_enh-Core

R2-2010119	Processing delay requirements for RRC resume	Huawei, HiSilicon	CR	Rel-16	36.331	16.2.1	4496	-	C	LTE_NR_DC_CA_enh-Core

Email 222

Withdrawn:

R2-2009414	Correction on tdm-PatternConfig2 configuration upon MR-DC Release	MediaTek Inc.	CR	Rel-16	38.331	16.2.0	2072	-	F	LTE_NR_DC_CA_enh-Core	Withdrawn

### 6.8.5	UE capabilities

By Web Conf (2+2)

Beam-level measurement capabilities:

R2-2009437	Capability for beam level NR early measurement reporting	MediaTek Inc.	CR	Rel-16	36.331	16.2.1	4463	-	F	LTE_NR_DC_CA_enh-Core

R2-2009438	Capability for beam level NR early measurement reporting	MediaTek Inc.	CR	Rel-16	36.306	16.2.0	1791	-	F	LTE_NR_DC_CA_enh-Core

R2-2010341	Adding UE capability for beam level early measurement reporting (36331)	Huawei, HiSilicon	CR	Rel-16	36.331	16.2.1	4510	-	F	LTE_NR_DC_CA_enh-Core

R2-2010342	Adding UE capability for beam level early measurement reporting (36306)	Huawei, HiSilicon	CR	Rel-16	36.306	16.2.0	1797	-	F	LTE_NR_DC_CA_enh-Core

By Web Conf (4)

Direct Scell activation capabilities:

R2-2009186	Correction to 36.306 on UE capability of direct SCell activation	Qualcomm Incorporated, Ericsson	CR	Rel-16	36.306	16.2.0	1790	-	F	LTE_NR_DC_CA_enh-Core

R2-2009187	Correction to 36.331 on UE capability of direct SCell activation	Qualcomm Incorporated, Ericsson	CR	Rel-16	36.331	16.2.1	4456	-	F	LTE_NR_DC_CA_enh-Core

R2-2010114	UE capability of direct E-UTRAN SCG SCell activation	Huawei, HiSilicon	discussion	Rel-16	LTE_NR_DC_CA_enh-Core

R2-2009554	Direct Scell activation capability	Nokia, Nokia Shanghai Bell	discussion	Rel-16	LTE_NR_DC_CA_enh-Core

By Web Conf (2)

NR-DC cell group signalling in capabilities:

R2-2010029	Cell group filtering for NR-DC	Ericsson	discussion	LTE_NR_DC_CA_enh-Core

R2-2010593	MCG and SCG differentiation in asynchronous NR-DC	Samsung Electronics	discussion	Rel-16

(moved from 6.1.2)

By Email [222] (2)

Capability naming for EMR:

R2-2010031	Correction on early measurement capabilities	Ericsson	CR	Rel-16	36.306	16.2.0	1795	-	F	LTE_NR_DC_CA_enh-Core

R2-2010032	Correction on early measurement capabilities	Ericsson	CR	Rel-16	36.331	16.2.1	4493	-	F	LTE_NR_DC_CA_enh-Core

Email 222

By Email [222] (2)

R2-2009666	Adding missing field descriptions of Multi-RAT DC and CA enhancements capabilities	Lenovo, Motorola Mobility	CR	Rel-16	36.331	16.2.1	4474	-	F	LTE_NR_DC_CA_enh-Core

R2-2010030	Clarification on cross-carrier A-CSI triggering capability	Ericsson	CR	Rel-16	38.306	16.2.0	0437	-	F	LTE_NR_DC_CA_enh-Core

R2-2010343	Clarification on UE capability of cross-carrier scheduling with different numerologies	Huawei, HiSilicon	discussion	Rel-16	LTE_NR_DC_CA_enh-Core

(moved from 6.1.2)

Email 222

# 7	Rel-16 EUTRA Work Items

Essential corrections

## 7.1	EUTRA Rel-16 General

No documents should be submitted to 7.1. Please submit to.7.1.x

Editorial corrections should be taken up with the specification editor before submitting to avoid CR duplication.

### 7.1.1	Cross WI RRC corrections

Including [Post111-e][928][LTE16] EUTRA Parameter Names Consolidation (Samsung)

Web Conf (1)

Result of [Post111-e][928][LTE16] EUTRA Parameter Names Consolidation (Samsung):

R2-2009608	Updated consolidated parameter list for Rel-16 LTE	Samsung	discussion	Rel-16	LTE_eMTC5-Core, NB_IOTenh3-Core, LTE_DL_MIMO_EE-Core, LTE_terr_bcast-Core

R2-2009609	Reply LS on updated Rel-16 LTE parameter lists	Samsung	LS out	Rel-16	LTE_eMTC5-Core, NB_IOTenh3-Core, LTE_DL_MIMO_EE-Core, LTE_terr_bcast-Core	To:RAN WG1, RAN WG4

Online discussion (2nd week Tuesday)

### 7.1.2	Feature Lists and UE capabilities

Web Conf (1)

LSs from RAN1/RAN4 on UE feature lists for LTE:

R2-2008703	LS on updated Rel-16 RAN1 UE features list for LTE (R1-2007139; contact: NTT DoCoMo, AT&T)	RAN1	LS in	Rel-16	LTE_eMTC5-Core, NB_IOTenh3-Core, LTE_DL_MIMO_EE-Core, LTE_terr_bcast-Core, 5G_V2X_NRSL-Core, TEI16	To:RAN2	Cc:RAN4

R2-2008709	LS on updated Rel-16 RAN1 UE features lists for LTE (R1-2007329; contact: NTT  DoCoMo, AT&T)	RAN1	LS in	Rel-16	LTE_eMTC5-Core, NB_IOTenh3-Core, LTE_DL_MIMO_EE-Core, LTE_terr_bcast-Core, 5G_V2X_NRSL-Core, TEI16	To:RAN2	Cc:RAN4

R2-2008742	LS on Rel-16 updated RAN4 UE features lists for LTE and NR (R4-2011929; contact: CMCC)	RAN4	LS in	Rel-16	To:RAN2	Cc:RAN1

Online

## 7.4	Even further mobility enhancement in E-UTRAN

(LTE_feMob-Core; leading WG: RAN2; REL-16; started: Jun 18; Completed: June 20; WID: RP-190921)

No documents should be submitted to 7.4. Please submit to.7.4.x

Documents under 7.4 will be treated together with documents in 6.7

Editorial corrections should be taken up with the specification editor before submitting to avoid CR duplication.

### 7.4.1	General and Stage-2 Corrections

Including incoming LSs (if any)

Web Conf (1)

R2-2008717	LS response on power sharing for LTE mobility enhancements (R1-2007420; contact: Ericsson)	RAN1	LS in	Rel-16	LTE_feMob-Core	To:RAN2

Contributions submitted under 7.4.3

By Email [210] (4)

DAPS Stage-2:

R2-2010207	Correction for the definition of DAPS handover (36.300)	SHARP Corporation	CR	Rel-16	36.300	16.3.0	1327	-	F	LTE_feMob-Core

R2-2010208	Correction for the definition of DAPS handover (38.300)	SHARP Corporation	CR	Rel-16	38.300	16.3.0	0316	-	F	NR_Mob_enh-Core

R2-2009765	Clarification on no DAPS HO in MR-DC	Nokia, Nokia Shanghai Bell	CR	Rel-16	36.300	16.3.0	1301	1	F	LTE_feMob-Core	R2-2007358

(moved from 7.4.2)

R2-2010507	Clarifications on DAPS and conditional handover for LTE-5GC	Ericsson	CR	Rel-16	36.300	16.3.0	1329	-	F	LTE_feMob-Core

(moved from 7.4.2)

Email 210

### 7.4.2	DAPS handover Corrections

This AI jointly addresses corrections to NR and LTE DAPS.

Including corrections to control and user plane for DAPS HO.

Including discussion on how to avoid mTRP usage during DAPS HO as per RAN#89e discussion.

Web Conf (1)

PHR restrictions for DAPS HO:

R2-2010498	Restriction on PHR for DAPS	Ericsson, China Telecom, LG Electronics Inc., Nokia, Nokia Shanghai-Bell, MediaTek, Vivo, CATT	CR	Rel-16	36.331	16.2.1	4516	-	F	LTE_feMob-Core

(moved from 7.4.3)

Web Conf (4+3)

How to avoid mTRP usage during DAPS HO (as per RAN#89e discussion):

R2-2009770	Prohibiting simultaneous DAPS and multi-TRP operation	Nokia, Nokia Shanghai Bell	discussion	Rel-16	NR_Mob_enh-Core

R2-2009559	Handling of SCells and mTRP during DAPS HO	Qualcomm Incorporated	discussion

R2-2010640	Discussion on releasing source MCG SCells and mTRP	Huawei, HiSilicon	discussion	Rel-16	NR_Mob_enh-Core, LTE_feMob-Core

R2-2009607	Release of mTRP operation before DAPS handover	Samsung	discussion	Rel-16	NR_Mob_enh-Core

(moved from 6.7.5)

R2-2009383	Clarification on no support of multi-TRP with DAPS HO - 38.331	ZTE Corporation, Sanechips, Ericsson	CR	Rel-16	38.331	16.2.0	2061	-	F	NR_Mob_enh-Core

R2-2009384	Clarification on no support of multi-TRP with DAPS HO - 38.300	ZTE Corporation, Sanechips, Ericsson	CR	Rel-16	38.300	16.3.0	0307	-	F	NR_Mob_enh-Core

R2-2010105	Clarification of SCells, mTRP, and DC during DAPS HO	Qualcomm Incorporated	CR	Rel-16	38.331	16.2.0	2176	-	F	NR_Mob_enh-Core

Web Conf (3+4)

Release of SCells during DAPS HO:

R2-2009272	Release SCells/SCG configuration during DAPS HO	Intel Corporation	discussion	Rel-16	NR_Mob_enh-Core, LTE_feMob-Core

R2-2009767	On how to release SCells when DAPS HO is configured	Nokia, Nokia Shanghai Bell	discussion	Rel-16	LTE_feMob-Core

R2-2009380	Discussion on SCells and SCG release in DAPS HO	ZTE Corporation, Sanechips, Ericsson	discussion	Rel-16

R2-2009381	Clarification on SCells and SCG release in DAPS HO - 38.300	ZTE Corporation, Sanechips, Ericsson	CR	Rel-16	38.300	16.3.0	0306	-	F	NR_Mob_enh-Core

R2-2009382	Clarification on SCells and SCG release in DAPS HO - 36.300	ZTE Corporation, Sanechips, Ericsson	CR	Rel-16	36.300	16.3.0	1320	-	F	LTE_feMob-Core

R2-2009768	Draft 38331 CR SCells during DAPS HO	Nokia, Nokia Shanghai Bell	CR	Rel-16	38.331	16.2.0	2126	-	F	NR_Mob_enh-Core

R2-2009769	Draft 36331 CR SCells during DAPS HO	Nokia, Nokia Shanghai Bell	CR	Rel-16	36.331	16.2.1	4486	-	F	LTE_feMob-Core

Web Conf (2)

Key change during DAPS HO:

R2-2009275	Support of DAPS handover without key change	Intel Corporation, Ericsson	discussion	Rel-16	NR_Mob_enh-Core	R2-2006935

R2-2010328	DAPS HO without security key change	LG Electronics Inc.	discussion	LTE_feMob-Core

R2-2010209	Potential security issue on DAPS handover with key change failure	SHARP Corporation	discussion	Rel-16	NR_Mob_enh-Core	R2-2007790

R2-2010210	[Draft] LS to SA3 on security handling for DAPS handover	SHARP Corporation	LS out	Rel-16	NR_Mob_enh-Core	R2-2007791	To:SA3

Web Conf (1)

Timing of source PCell release after HO completion

R2-2010639	Discussion on source release indication	Huawei, HiSilicon	discussion	Rel-16	NR_Mob_enh-Core, LTE_feMob-Core

By Email [213] (1+7)

Rapporteur CRs:

R2-2009276	Miscellaneous corrections for Mobility Enhancements	Intel Corporation (Rapporteur), Ericsson	CR	Rel-16	38.331	16.2.0	2050	-	F	NR_Mob_enh-Core

Miscellaneous RRC CRs:

R2-2010504	Miscellaneous mobility-related corrections	Ericsson, ETRI	CR	Rel-16	36.331	16.2.1	4518	-	F	LTE_feMob-Core

R2-2009535	Corrections on  DAPS in 36.331	CATT,Ericsson	CR	Rel-16	36.331	16.2.1	4467	-	F	LTE_feMob-Core

R2-2009534	Correction on Source Cell Group and Source SpCell on DAPS	CATT,Ericsson	CR	Rel-16	38.331	16.2.0	2087	-	F	NR_Mob_enh-Core

R2-2010297	Correction on reestablishRLC for DAPS	Huawei, HiSilicon	CR	Rel-16	38.331	16.2.0	2203	-	F	NR_Mob_enh-Core

R2-2010505	Release source cell configuration at DAPS handover	Ericsson	CR	Rel-16	38.331	16.2.0	2231	-	F	NR_Mob_enh-Core

R2-2010506	DAPS handover for bearers configured with NR PDCP	Ericsson	CR	Rel-16	36.331	16.2.1	4519	-	F	LTE_feMob-Core

R2-2010435	Correction on DAPS	OPPO	CR	Rel-16	38.331	16.2.0	2222	-	F	NR_Mob_enh-Core

Email [213]

[AT112-e][213][MOB] DAPS RRC corrections (Ericsson)

Scope:

    - Discuss which DAPS RRC corrections to LTE and NR are seen necessary and provide merged CRs with agreeable corrections (if any)

Intended outcome:

    - Discussion summary in R2-2010727 (by email rapporteur).

    - Merged CRs to 36.331 (R2-2010728) and 38.331 (R2-2010729) (if any)

Deadline for providing comments, for rapporteur inputs, conclusions and CR finalization:

    - Initial deadline (for companies' feedback):  1st week Fri, UTC 0900

    - Initial deadline (for rapporteur's summary in R2-2010727):  2nd week Mon, UTC 13:00

    - Deadline for CR finalization: 2nd week Thu, UTC 1000

By Email [213] (2)

DataInactivityTimer:

R2-2009654	Handling of expiry of dataInacticityTimer for DAPS	NEC	discussion	Rel-16	LTE_feMob-Core

R2-2010501	Handling of dataInactivityTimer for DAPS	Ericsson	discussion

Email [213]

By Email [213] (2)

DAPS RLF handing:

R2-2010294	Correction on RLF handling in DAPS	Huawei, HiSilicon	CR	Rel-16	38.331	16.2.0	2202	-	F	NR_Mob_enh-Core

R2-2010295	Correction on RLF handling in DAPS	Huawei, HiSilicon	CR	Rel-16	36.331	16.2.1	4506	-	F	LTE_feMob-Core

R2-2010499	RLF in source during DAPS	Ericsson	discussion

Email [213]

### 7.4.3	UE capability corrections

Including UE capability aspects of LTE mobility WI that are LTE-specific.

By Email [212] (2)

UL power sharing for LTE DAPS:

R2-2010298	Correction on LTE DAPS UE capability	Huawei, HiSilicon	CR	Rel-16	36.331	16.2.1	4507	-	F	LTE_feMob-Core

R2-2010299	Correction on LTE DAPS UE capability	Huawei, HiSilicon	CR	Rel-16	36.306	16.2.0	1796	-	F	LTE_feMob-Core

R2-2010502	Introducing power sharing for DAPS handover	Ericsson, Qualcomm	CR	Rel-16	36.306	16.2.0	1798	-	F	LTE_feMob-Core

=> Revised in R2-2010681

R2-2010681	Introducing power sharing for DAPS handover	Ericsson, Qualcomm, Huawei	CR	Rel-16	36.306	16.2.0	1798	1	F	LTE_feMob-Core

R2-2010503	Introducing power sharing for DAPS handover	Ericsson, Qualcomm	CR	Rel-16	36.331	16.2.1	4517	-	F	LTE_feMob-Core

=> Revised in R2-2010682

R2-2010682	Introducing power sharing for DAPS handover	Ericsson, Qualcomm, Huawei	CR	Rel-16	36.331	16.2.1	4517	1	F	LTE_feMob-Core

Email [212]

By Email [212] (1)

LTE DAPS capabilities:

R2-2009188	Clarifications to LTE DAPS capabilities	Nokia, Nokia Shanghai Bell	discussion	Rel-16	LTE_feMob-Core

Email [212]

### 7.4.4	Other corrections

Only corrections not fitting other agenda items.

Including CHO aspects that are LTE-specific without equivalent NR impacts:

By Email [211] (2)

R2-2010641	Cell selection upon RRCConnectionReestablishment 	Samsung R&D Institute UK	CR	Rel-16	36.331	16.2.1	4525	-	F	LTE_feMob-Core

R2-2010645	Miscellaneous corrections on LTE CHO procedures	Samsung R&D Institute UK	CR	Rel-16	36.331	16.2.1	4526	-	F	LTE_feMob-Core

Email [211]

## 7.5	LTE Other WIs

(LTE_terr_bcast-Core, LTE_DL_MIMO_EE-Core, LTE_high_speed_enh2-Core; LTE TEI16 Non-positioning)

(Documents relating to Rel-16 LTE but for which there is no existing RAN WI/SI, e.g. LSs from CT/SA requesting RAN2 action)

Editorial corrections should be taken up with the specification editor before submitting to avoid CR duplication.

Including TEI16 corrections and issues that do not fit under any other topic.

By Email [202] (2+2)

Stage-2 updates:

R2-2008704	LS on Updates to TS 36.300 on terrestrial broadcast (R1-2007154; contact: Qualcomm)	RAN1	LS in	Rel-16	LTE_terr_bcast-Core	To:RAN2

R2-2009446	CP length and reference signal for MBSFN with sub-carrier spacing of 0.375 KkHz and 2.5 kKHz	Qualcomm Inc	CR	Rel-16	36.300	16.3.0	1322	-	F	LTE_terr_bcast-Core

R2-2009802	Miscellaneous Stage-2 corrections	Nokia (rapporteur), NEC, Lenovo, Motorola Mobility, Intel Corporation, ZTE, Sanechips, Ericsson	CR	Rel-16	36.300	16.3.0	1324	-	F	NB_IOTenh2-Core, LTE_eMTC4-Core, NB_IOTenh3-Core, LTE_eMTC5-Core, LTE_feMob-Core, TEI16

Email 202

By Email [201] (1)

36.306 updates:

R2-2009433	Clarification to Fallback band combination definition	Nokia, Nokia Shanghai Bell	CR	Rel-16	36.306	16.2.0	1782	1	F	TEI16	R2-2007518

Email 201

36.331 updates:

R2-2008908	Corrections to UE capabilities and SIB25	Lenovo, Motorola Mobility, Ericsson	CR	Rel-16	36.331	16.2.1	4453	-	F	LTE_DL_MIMO_EE-Core, LTE_eMTC5-Core, TEI16

R2-2009385	Correction on T312 timer information	ZTE Corporation, Sanechips	CR	Rel-16	36.331	16.2.0	4461	-	F	LTE_feMob-Core

Email 201

By Email [202] (2)

R2-2008907	Corrections to UE capabilities	Lenovo, Motorola Mobility (Rapporteur)	CR	Rel-16	36.306	16.2.0	1789	-	F	NR_IIOT-Core, LTE_DL_MIMO_EE-Core, LTE_eMTC5-Core, TEI16

R2-2009603	Minor changes collected by Rapporteur	Samsung	CR	Rel-16	36.331	16.2.1	4472	-	F	LTE_NR_DC_CA_enh-Core

Email 202

# 8	Rel-17 NR Work Items

## 8.2	MR DC CA further enhancements

(LTE_NR_DC_enh2-Core; leading WG: RAN2; REL-17; WID: RP-201040)

Time budget: 1 TU

Tdoc Limitation: 2 tdocs

Email max expectation: 3 threads

No documents should be submitted to 8.2. Please submit to.8.2.x

### 8.2.1	Organizational Requirements and Scope

Including work plan and any other rapporteur input.

### 8.2.2	Efficient activation deactivation mechanism for one SCG and SCells

Including outcome of  [Post111-e][919][eDCCA] Efficient activation deactivation of SCG (Huawei)

Web Conf (1)

Outcome of  [Post111-e][919][eDCCA] Efficient activation deactivation of SCG (Huawei):

R2-2010123	[Post111-e][919][eDCCA] Efficient activation deactivation of SCG Discussion on SCG deactivation and activation	Huawei	discussion	Rel-17	LTE_NR_DC_enh2-Core

Discussion

P1&2

-	Nokia wonders if this is also possible in HO?

-	CATT thinks this is useful for PSCell change but not addition.

P3

-	Qualcomm wonders if this means PDCCH is not monitored at all? Apple wonders the same - this might cause problems. Nokia also thinks this may cause some problems but is fine to start with this.

P4:

-	Apple wonders if MN-initiated mobility is supportee for PSCell? Ericsson thinks 4a covers this already. LGE agrees and thinks this is the legacy behaviour. OPPO thinks MN-based mobiltiy is also supported. ZTE thinks this was also discussed during email but most companies thought at least Pscell mobility based on SN measurements shuld be supported.

P8:

-	Apple thinks this means that this might impact e.g. beam management, timing and RACH. Will it create problems for performance? Huawei thinks SRS is not the only way for timing and beam management but UL is only possible with timing alignment. CATT indicates dormant Scells do not support this but then TA group is there.

-	Apple wonders if this means UE will always do RACH at SCG activation? Huawei thinks that UL timing might still be accurate as long as TAT is running so RACH is not always needed. There might be other solutions than SRS as well.

Agreements

The work will focus on a single deactivated SCG.

FFS if SCG RRC reconfiguration can select the SCG activation state (activated/deactivated) at PSCell addition/change, RRC resume or HO.

Continue RAN2 work with the assumption that when the SCG is deactivated, the UE does not monitor PDCCH on the PSCell. This assumption can be reconsidered if issues are found.

As a baseline, MN-configured RRM measurement/reporting procedures do not depend on the SCG activation state (deactivated or activated). Further optimisations are not precluded.

While the SCG is deactivated, PSCell mobility is supported. MN- and SN-configured measurements are supported for deactivated SCG.

FFS1: Details on the performed measurements (e.g. all SN configured measurements or subset based on certain criteria, restrictions on inter-frequency/RAT)

FFS2: Support for SCell addition/mobility

FFS3: Reporting procedure

FF4: PSCell mobility procedure

RAN2 assumes that UE will not perform SRS transmission while the SCG is deactivated. This assumption can be reconsidered if issues are found.

FFS if RACH is needed for SCG reactivation

CB: AT-meeting Email discussion (Huawei) on FFS points on agreements: What are the alternatives based on company contributions? If time allows, can also attempt to figure out other FFS points.

[AT112-e][230][eDCCA] Progressing FFS points of efficient SCG activation and deactivation (Huawei)

Scope:

    - Discuss the FFSs from online agreements for Efficient activation deactivation of SCG to understand which alternatives are seen feasible.

    - Can discuss also remaining FFS from email discussion [Post111-e][919] if time allows

Intended outcome:

    - Discussion summary in R2-2010733 (by email rapporteur).

Deadlines:

    - Rapporteur can set an intermediate deadline for company inputs and/or converging the discussion

    - Deadline for email discussion report: 2nd week Thu, UTC 1000

CBF: Report of email discussion [230]

Web Conf (4)

R2-2010062	Efficient SCG (de)activation	Ericsson	discussion	Rel-17

Proposal 15	Both MN and SN initiated procedures supported.

Proposal 6	SCG mobility is supported while SCG is deactivated.

Proposal 8	MCG mobility is supported while SCG is deactivated.

Proposal 1	RAN2 to study the feasibility maintaining DL fine synchronization for deactivated SCG, e.g. through beam or radio link monitoring.

Proposal 4	If SCG is deactivated, UE continues to perform PSCell measurements based on SN configuration.

Proposal 5	If SCG is deactivated, FFS whether UE continues to perform SCG SCell measurements.

Proposal 7	If SCG is deactivated, UE continues to perform SCG measurements for at least the measId(s) associated to events A3/A5. FFS other measurements.

Proposal 9	Discuss, together with RAN4, relaxed RRM measurements requirement for the case the SCG is deactivated.

Proposal 10	If SCG is deactivated, UE performs some level of S-RLM and SCG failure information procedure is supported to report the failure. Exact behaviour to be discussed after beam management and CSI for deactivated SCG is defined.

Proposal 2	Define a reduced processing time for RRC reconfiguration for activating SCG with limited changes to the SCG configuration.

Proposal 3	Send LS to RAN4 to confirm whether Tprocessing = 0ms could be assumed for SCG activation, without cell or frequency change.

Proposal 11	If possible, random access should be avoided upon SCG activation. FFS cases where it is possible to avoid random access upon SCG activation.

Proposal 12	The UE performs BFD monitoring for deactivated SCG. FFS Discuss actions upon BFD while SCG is deactivated.

Proposal 13	Discuss the possibilities to support SCG CSI reporting while SCG is deactivated. FFS how reporting can be enabled e.g. via SCG or MCG.

Proposal 14	UE should at least assume UL TA is accurate for deactivated SCG until TA timer expires. FFS whether to actively maintain TA for deactivated SCG.

R2-2009439	Discussion on SCG suspension	MediaTek Inc.	discussion	Rel-17	LTE_NR_DC_enh2-Core	R2-2007867

Observation 1: With the power saving mechanisms introduced in Rel-16, there is limited power saving gain to have PSCell dormancy.

Observation 2: There is high specification complexity and requires inter-WG discussion between RAN1 and RAN2 to introduce the PSCell dormancy behavior.

Observation 3: For power saving purpose and for thermal protection, PSCell deactivation is simple and efficient.

Observation 4: There is a need to have PSCell in deactivated state upon SCG addition and RRC Resume. Thus RRC control of PSCell deactivation is required.

Proposal 1: Introduce PSCell deactivation behavior in Rel-17. While the PSCell is deactivated, the UE shall

•	Deactivate all SCG SCell(s)

•	Keep the SCG configuration

•	Does not transmit/receive data on the SCG but continue the RRM measurement on SCG cells

•	Suspend the SCG transmission for all radio bearers

Proposal 2: When a PSCell is deactivated

•	The UE does not monitor the PDCCH on that PSCell

•	The UE does not perform RLM/BFD on that PSCell

•	The UE does not maintain the TA value for the SCG

•	The UE does not report CSI on the PSCell or for the PSCell

Proposal 3: While the PSCell is activated from deactivated state, the UE shall

•	Trigger RACH to the PSCell

•	Resume the SCG transmission for all radio bearers

Proposal 4: Introduce new RRC signaling to control the activation and deactivation of PSCell. FFS to use MAC CE to control PSCell activation/deactivation.

R2-2009547	On fast deactivation and activation of one SG and SCells	Nokia, Nokia Shanghai Bell	discussion	Rel-17	LTE_NR_DC_enh2-Core

Proposal 1: Confirm that NE-DC is not part of the efficient activation/deactivation goal of the WI and no solutions are developed for NE-DC case.

Proposal 2: Focus the work on NR-DC SCG activation and deactivation (i.e. NR RRC changes) and only start working on EN-DC use case (i.e. LTE RRC changes) if time allows.

Proposal 9:  SCG SCells are deactivated whenever SCG is deactivated. Either this is done by explicitly or implicitly

Proposal 12: Regular connected mode SIB update mechanisms are used for deactivated SCG

Proposal 10: Support RRM (no spec impact) cell and beam measurements and involve RAN4 regarding performance requirements for the RRM measurements for deactivated SCG

Proposal 11: Do not support radio link measurements for the deactivated SCG as RRM measurements most likely will be able to provide similar information to the network.

Proposal 13: Network needs to be informed about data arrival upon data arrival to bearer(s) mapped to SCG also on deactivated SCG.

Proposal 14: MCG makes final decision to deactivate SCG and sends the deactivation command to the UE

Proposal 15: Support RRC signaling to deactivate SCG as part of RRCReconfiguration message (FFS whether other layer signalling is supported e.g. MAC/DCI)

Proposal 16: Support RRC signaling to activate SCG as part of RRCReconfiguration message.

Proposal 17: Support SR/RACH based SCG activation.

Proposal 18: When SCG is deactivated NW is not required to release SCG configuration

Proposal 19: NW is allowed to reconfigure SCG during deactivation (at least if signaling is done with RRC)

Proposal 3: No PDCCH monitoring for deactivated SCG.

Proposal 4: No need to report CSI/CQI for deactivated SCG.

Proposal 5: No SRS support for deactivated SCG

Proposal 6: No uplink grants (configured or scheduled) supported for deactivated SCG

Proposal 7: Do not introduce mechanism to keep timing alignment up to date on deactivated SCG

Proposal 8: FFS if timing alignment timer is allowed to continue running when SCG is deactivated.

Proposal 20: FFS whether SCG could be directly deactivated when configured (or at handover)

Proposal 21: Study whether lower layer activation signaling would be useful to reduce activation delays without huge efforts to RAN2 (e.g. using MAC/DCI).

R2-2010124	Discussion on SCG deactivation and activation	Huawei, HiSilicon	discussion	Rel-17	LTE_NR_DC_enh2-Core

Proposal 11:	The MN makes the decision on deactivate/activate the SCG.

Proposal 12:	The MN sends the deactivation/activation command to the UE.

Proposal 13:	UE sends an RRC message to the MN to indicate UL data arrival on an SCG bearer

Proposal 14:	Use MAC CE to indicate SCG deactivation/activation to the UE

Proposal 15:	The network can reconfigure the deactivated SCG in order to change the PSCell.

Proposal 16:	The network can keep the SCG to be deactivated when PSCell is changed.

Proposal 17:	The UE does not initiate the RACH when the PSCell is changed and the SCG is deactivated.

Proposal 1:	UE stops the PUSCH transmission when the SCG is deactivated.

Proposal 2:	When the SCG is deactivated, the UE does not perform CSI measurements and reporting on the SCG.

Proposal 8:	When the SCG is deactivated, the UE does not perform BFD/BFR on the PSCell.

Proposal 10:	The UE does not transmit SRS when the SCG is deactivated

Proposal 3:	The network can indicate which RRM measurements the UE continues when the SCG is deactivated.

Proposal 4:	Study the relaxed RRM measurement for the continued RRM measurements.

Proposal 5:	  For the SN-configured RRM measurements, UE reports the results to the SN via the MN.

Proposal 6:	the network controls whether the UE continues to perform RLM on PSCell when the SCG is deactivated.

Proposal 7:	Only RLM based on explicitly configured RS is supported while the SCG is deactivated

Proposal 9:	When the SCG is deactivated while the TAT is running, the UE keeps the TAT running but does not initiate RACH upon TAT expiry. If the SCG is activated while the TAT is still running, the UE may resume normal SCG operation without RACH (details FFS).

R2-2010283	Efficient SCG Activation mechanism	LG Electronics	discussion	Rel-17	R2-2007986

Proposal 1: For power-efficient SCG activation, in the SCG deactivation, the UE doesn’t monitor PDCCH until SCG activation.

Proposal 2: For time-efficient SCG activation, in the SCG deactivation, the UE doesn’t perform synchronisation procedure, i.e. RACH on the way of SCG activation if TA is valid.

Proposal 3: Discuss whether an additional requirement is needed to keep synchronisation with the network even after TA expiry.

Proposal 4: For time-efficient SCG activation, the UE performs RRM which is configured by SN.

Proposal 5: For SN mobility, all SCG radio bearers including SRB3 & DRBs are required to be suspended during SCG deactivation.

R2-2010372	Considerations on SCG activation or deactivation	CMCC	discussion	Rel-17	LTE_NR_DC_enh2-Core

R2-2008920	Considerations on fast (de)active of Scell	KDDI Corporation	discussion

(moved from 6.8.2)

R2-2009590	Discussion on efficient deactivation mechanism for the SCG	China Unicom	discussion	LTE_NR_DC_enh2-Core

R2-2009913	Discussion on efficient SCG activation/deactivation	China Telecommunications	discussion

R2-2010087	Progressing SCG deactivation and resumption for R17	Samsung Telecommunications	discussion	Rel-17	LTE_NR_DC_enh2-Core

=> Revised in R2-2010683

R2-2010683	Progressing SCG deactivation and resumption for R17	Samsung Telecommunications	discussion	Rel-17	LTE_NR_DC_enh2-Core

R2-2009246	Further consideration on SCG activation and deactivation	ZTE Corporation, Sanechips	discussion	Rel-17	LTE_NR_DC_enh2-Core

R2-2009357	Efficient Activation/Deactivation Mechanism for SCG	CATT	discussion	Rel-17	LTE_NR_DC_enh2-Core

R2-2008870	Discussion on SCG suspension or deactivation	OPPO	discussion	Rel-17	LTE_NR_DC_enh2-Core

R2-2009150	Discussion on efficient activation mechanism for one SCG	Spreadtrum Communications	discussion	Rel-17	LTE_NR_DC_enh2-Core

R2-2009259	On Support of Activation/Deactivation for SCG	InterDigital	discussion	Rel-17	LTE_NR_DC_enh2-Core

R2-2009284	Further discuss the issues with SCG fast activation	Futurewei	discussion	Rel-17	LTE_NR_DC_enh2-Core

R2-2009531	Open items on SCG deactivation feature	Apple	discussion	Rel-17	LTE_NR_DC_enh2-Core

R2-2009867	On SCG deactivatoin and activation	Lenovo, Motorola Mobility	discussion	Rel-17

R2-2009942	Signalling for Rel-17 efficient SCG de-activation/re-activation	Intel Corporation	discussion	Rel-17	LTE_NR_DC_enh2-Core

R2-2010132	Efficient SCG activation/deactivation in MR-DC	Qualcomm Incorporated	discussion	Rel-17

R2-2010231	Signalling for SCG activation	SHARP Corporation	discussion	Rel-17	LTE_NR_DC_enh2-Core

R2-2010290	Activation and deactivation mechanism for SCG and SCells	vivo	discussion	Rel-17

R2-2009814	SCG deactivation upon SCG addition	NEC	discussion	Rel-17	LTE_NR_DC_enh2-Core

### 8.2.3	Conditional PSCell change addition

Including outcome of  [Post111-e][920][eDCCA] Condtional PSCell Change and Addition (CATT)

Web Conf (1)

Outcome of  [Post111-e][920][eDCCA] Condtional PSCell Change and Addition (CATT):

R2-2009360	Summary of  [Post111-e][920][eDCCA] Conditional PSCell Change and Addition (CATT)	CATT	discussion	Rel-17	LTE_NR_DC_enh2-Core

Proposals from discussion paper

Proposal 1: The following proposals can be considered for bulk agreements.

Discussion (P1 / bulk agreement points)

-	Intel has concern on P19 within the bulk agreements: Wouldn't want to fix how encapsulation is done.

Bulk Agreement

Proposal Set 1A: general/procedure

- Maintain Rel-15 principle that only one PScell is active at a time even with conditional PScell addition/change.

- Usage of CPAC is decided by the network. The UE evaluates when the condition is valid.

- The baseline operation for CPAC procedure assumes the RRC Reconfiguration message contains SCG addition/change triggering condition(s) and the RRC configuration(s) for candidate target PSCells. The UE accesses the prepared PSCell when the relevant condition is met.

- CPAC execution condition and/or candidate PSCell configuration can be updated by modifying the existing CPAC configuration.

- Support configuration of one or more candidate cells for CPAC.

- UE is not required to continue evaluating the triggering condition of other candidate PSCell(s) during CPC/CPA execution.

- For FR1 and FR2, leave it up to UE implementation to select the candidate PSCell if more than one candidate cell meets the triggering condition. UE may consider beam information in this.

- No additional optimizations with multi-beam operation are introduced to improve RACH performance for CPAC completion with multi-beam operation.

Proposal set 1B: trigger/ condition related

- For conditional PSCell addition, the MN decides on the conditional PSCell addition execution condition. FFS for PSCell Change.

- The execution condition for CPAC is defined by a measurement identity which identifies a measurement configuration.

11	For conditional PSCell change, A3/A5 execution condition should be supported while for conditional PSCell addition, A4/B1 like execution condition should be supported.

12	Allow having multiple triggering conditions (using “and”) for CPAC execution of a single candidate cell. Only single RS type per CPAC candidate is supported. At most two triggering quantities (e.g. RSRP and RSRQ, RSRP and SINR, etc.) can be configured simultaneously.

13	Cell level quality is used as baseline for CPAC execution condition;

14	Only single RS type (SSB or CSI-RS) per candidate PSCell is supported for PSCell change.

15	TTT is supported for CPAC execution condition (as per legacy configuration)

Proposal set 1C: signalling related

16	Reuse the RRCReconfiguration/RRCConnectionReconfiguration procedure to signal CPAC configuration to UE following Rel-16 signalling.

17  Multiple candidate PSCells can be sent in either one or multiple RRC messages.

18	As part of the CPAC configuration to be sent to the UE, the RRC container is used to carry candidate PSCell configuration, and the MN is not allowed to alter any content of the configuration from the PSCell. Moreover, in case of SN change, source SN is not allowed to alter any content of the configuration from the target SN. FFS on which RRC format is used (can be considered in stage-3)

19 For conditional PSCell addition, the MN transmits the final RRCReconfiguration/ RRCConnectionReconfiguration message to the UE. FFS how the encapsulation is done exactly (can be considered in Stage-3).

Proposal 1D: FFS issues

FFS for conditional PSCell change, SN decides on the condition for SN-initiated procedures and MN decides on the condition on MN-initiated procedures

FFS whether we need coordination on exact execution conditions or just measurements.

FFS whether source or target SN knows the condition

FFS in which exact cases the condition needs to be indicated

FFS how many candidate cells (UE and network impacts should be clarified). FFS whether the number of candidate cells for CPAC different from that of CHO.

FFS on UE capability for triggering quantities

Discussion (P2&3)

-	Intel thinks we need to discuss the signalling impact from these. Would like to avoid big RAN3  impacts if we follow CHO principles. CPC would be better from that perspective.

-	Ericsson is fine with P2 but P3 is unclear. CATT thinks large majority supported this.

-	Samsung thinks that in conventional PSCell addition/change, SN chooses the PSCell based on measurement results from MN. Would like to consider similar roles even though Rel-16 CPC didn't do this. OAM coordination could be enough for PSCell selection.

-	OPPO thinks SN would know execution condition better. Intel thinks in R16 the execution condition is forwarded to SN but email only considered P2. QC is not sure this is the case. Also wonders if this is for CPA and CPC? Intel thinks SN should know the execution condition to determine the final message.

-	Intel thinks we haven't considered the Stage-3 consequences of P2/P3.

Show of hands (P2+P3):

Against: 1

For: 15

- In MN initiated inter-SN CPC and CPA, the MN is not required to indicate the execution condition(s) to other involved entities (e.g. target SN, source SN).

- For CPA and MN initiated Inter-SN CPC, the MN generates and transmits the conditional configuration message (i.e. RRCReconfiguration/RRCConnectionReconfiguration message) to the UE.  The RRCReconfiguration provided by the candidate PSCell(s) is encapsulated in the final conditional reconfiguration message to the UE. The MN is not allowed to alter the RRCReconfiguration provided by the candidate PSCell(s).

Proposal 4: For the generation of conditional reconfiguration for SN initiated inter-SN CPC, the following two options should be further discussed.

Option 1:	(16 supporting companies) The MN generates CPC. The source SN sets the execution condition and communicates it to the MN. The MN generates the conditional reconfiguration message including the execution condition(s) provided by the source SN and RRCReconfiguration provided by the candidate PSCell(s).

Option 3:	(6 supporting companies) The source SN generates CPC. The source SN sets the execution condition. The source SN communicates with target SN and receives RRCReconfiguration provided by the candidate PSCell(s). The source SN generates the conditional reconfiguration message and provides it to the MN (possibly in a transparent container) for transmission to the UE.

Discussion (P4)

-	Samsung has changed their mind on P4 and is supportive of option 1 now: MN may need to adjust its configuration based on what target node does (e.g. SK-counter) so MN-generated configuration can be needed.

CB: AT-meeting Email discussion (CATT): Provide signalling flow diagrams for each of the following options in P4 (1 vs. 3), including the consequences of each

[AT112-e][231][eDCCA] Progressing conditional reconfiguration for SN initiated inter-SN CPC (CATT)

Scope:

    - Discuss the option 1 and option 3 details from P4 of email discussion [Post111-e][920] to better understand the technical details between the alternatives (e.g. signalling flows, signalling load, etc.)

Intended outcome:

    - Discussion summary in R2-2010734 (by email rapporteur).

Deadlines:

    - Rapporteur can set an intermediate deadline for company inputs and/or converging the discussion

    - Deadline for email discussion report: 2nd week Thu, UTC 1000

CBF: Report of email discussion [231]

R2-2009359	Introduction of CPA and MN Initiated Inter-SN CPC	CATT	draftCR	Rel-17	37.340	16.3.0	B	LTE_NR_DC_enh2-Core

Web Conf (2)

R2-2010088	Progressing conditional configuration for R17	Samsung Telecommunications	discussion	Rel-17	LTE_NR_DC_enh2-Core

Proposal 1: RAN2 is requested to discuss and confirm that RAN2 progresses based on the assumption that RAN3 inter-node messages do not support addition/ modification of multiple candidates (i.e. use as baseline, only change if serious issue identified)

Proposal 2: RAN2 is requested to discuss and conclude to what extend (T-)SN should have say in measurement condition to be met at CPC execution and select between following options:

a)	Introduce negotiation between S-SN (initiating node) and (target) SN for the conditions

b)	Allow (target) SN to set a separate condition

c)	For R17 its sufficient to have no coordination or use OAM (not UE specific)

Proposal 3: At least in case of SN initiated change of SN, support that UE can apply MN and SN generated configurations at CPC execution. This means that field conditionalReconfiguration should include MN generated fields i.e. at least sk-Counter, radio bearer, cell group and measurement configuration

Proposal 4: At least in case of SN initiated change of SN, the configuration to apply for a candidate at CPC execution is an MN generated reconfiguration message

Proposal 5:	In case of SN initiated change of SN, SN generates the execution condition and it is transferred by a separate field and within an octet string container

Proposal 6:	For all R17 cases i.e. both CPA and CPC, we apply the same conclusion regarding:

o	Support for adding/ modifying multiple candidates in RAN3 inter-node messages

o	How (T-SN) can have say in execution condition e.g. OAM

o	To also support application of MN configuration at execution time

o	The configuration to apply for a candidate at CPC execution is an MN generated reconfiguration message

R2-2010626	Further consideration for Conditional PSCell addition and change	NTT DOCOMO INC.	discussion	Rel-17	LTE_NR_DC_enh2-Core	Late

Observation1: Options 2 and 3 should be considered with the following two sub options

	the communication between S-SN and T-SN is performed directly

	the communication between S-SN and T-SNs occurs via MN

Observation 2: An operation to place PSCells in isolation in macro PCell can be assumed

Proposal 1: Direct communication between S-SN and T-SN should be avoided.

Proposal 2: Execution conditions should be generated by S-SN.

Proposal 3: Conditional PSCell Configuration (i.e., RRC message) should be generated by either S-SN or T-SN.

Proposal 4: Option 2 like is considered one of the options

Option 2 like:	The target SN generates CPC. The source SN sets the execution condition and sends it to the target SN via MN. The target SN generates the conditional configuration message. The target-SN-generated conditional configuration message is provided to the MN (possibly in a transparent container) for transmission to the UE.

Proposal 5:	RAN2 re-consider whether baseline should be that the configurations of all candidates PSCell configurations for CPA and Inter-SN PSCell change are released upon successful completion of CPAC, conventional PSCell change or conventional PSCell addition or not.

R2-2009771	On Rel-17 Conditional PSCell Addition and Change (CPAC)	Nokia, Nokia Shanghai Bell	discussion	Rel-17	LTE_NR_DC_enh2-Core

R2-2010125	Discussion on support of conditional PSCell change/addition	Huawei, HiSilicon	discussion	Rel-17	LTE_NR_DC_enh2-Core

R2-2010373	Discussions about CPAC procedures	CMCC	discussion	Rel-17	LTE_NR_DC_enh2-Core

R2-2009379	Discussion on conditional PSCell addition/change	ZTE Corporation, Sanechips	discussion	Rel-17	LTE_NR_DC_enh2-Core

R2-2009596	Discussion on conditional PSCell change and addition	OPPO	discussion	Rel-17	LTE_NR_DC_enh2-Core

R2-2010003	Conditional PSCell Change / Addition	Ericsson	discussion	LTE_NR_DC_enh2-Core

R2-2010130	Configuration of Conditional PSCell addition/change	Qualcomm Incorporated	discussion	Rel-17

R2-2009868	Issues on inter-SN CPC	Lenovo, Motorola Mobility	discussion	Rel-17

R2-2009592	Discussion on inter-SN conditional PSCell change (SN initiated)	China Unicom	discussion	LTE_NR_DC_enh2-Core

R2-2009358	Discussion on Further CPAC Enhancements	CATT	discussion	Rel-17	LTE_NR_DC_enh2-Core

R2-2009816	Framework of Inter-SN Conditional PSCell change	NEC	discussion	Rel-17	LTE_NR_DC_enh2-Core

R2-2009815	Conditional PSCell addition procedure	NEC	discussion	Rel-17	LTE_NR_DC_enh2-Core

R2-2009088	Conditional PSCell change / addition	vivo	discussion	Rel-17	LTE_NR_DC_enh2-Core

R2-2009158	CPC configuration number restriction	Spreadtrum Communications	discussion	Rel-17	LTE_NR_DC_enh2-Core

R2-2009260	Coexistence of CHO and CPC	InterDigital	discussion	Rel-17	LTE_NR_DC_enh2-Core

R2-2009285	CPAC failure handling discussio	Futurewei	discussion	Rel-17	LTE_NR_DC_enh2-Core

R2-2009475	Discussion on conditional PSCell change and addition	Apple	discussion	Rel-17	LTE_NR_DC_enh2-Core

R2-2010248	Discussion on SN initiated CPC and CPAC Execution	ETRI	discussion	Rel-17	LTE_NR_DC_enh2-Core

R2-2010282	Considerations of CPAC in Rel-17	LG Electronics	discussion	Rel-17	R2-2007985

R2-2010529	Regarding inter MN-SN signaling design for Conditional PSCell Addition	Intel Corporation	discussion	Rel-17	LTE_NR_DC_enh2-Core

## 8.3	Multi SIM

(LTE_NR_MUSIM-Core; leading WG: RAN2; REL-17; WID: RP-201309)

Time budget: 1 TU

Tdoc Limitation: 3 tdocs

Email max expectation: 3 threads

### 8.3.1	Organizational Requirements and Scope

Including work plan and any other rapporteur input.

Including outcome of [Post111-e][917][NR][Multi-SIM] Work prioritization for Multi-SIM (vivo)

Web Conf (2)

R2-2008754	LS on System support for Multi-USIM devices (S2-2006037; contact: Intel)	SA2	LS in	Rel-17	FS_MUSIM	To:RAN2, RAN3, SA3

Noted

R2-2010689	Reply to LS S2-2006037 on System support for Multi-USIM devices (S3-202687; contact: Nokia)	SA3	LS in	Rel-17	FS_MUSIM	To:SA2, RAN2, RAN3

-	OPPO wonders if this is only for RRC_IDLE or also to RRC_INACTIVE? Nokia clarifies that this needs further discussion in RAN2. Should just avoid unsecured transmission. QC thinks this is only for RRC_IDLE.

Noted

Input related to SA2/SA3 LS replies:

R2-2009971	Response to SA2 LS S2-2006037: Paging Repetition in RAN and UE Implementation-based solution aspects	VODAFONE Group Plc	discussion

(moved from 8.3.2)

R2-2009780	Guidance for SA2 on Solution #16 for Key Issue 2 	VODAFONE Group Plc	discussion

(moved from 8.3.2)

R2-2009885	Discussion on Multi SIM	Sony, Convida Wireless	discussion	Rel-17	LTE_NR_MUSIM-Core

R2-2009943	[DRAFT] Reply LS on System support for Multi-USIM devices	Intel Corporation	LS out	Rel-17	LTE_NR_MUSIM-Core	To:SA2	Cc:RAN3

AT112e: Email discussion on reply to SA2 (Intel) (CBF)

[AT112-e][240][Multi-SIM] Reply LS to SA2 (Intel)

Scope:

    - Draft LS reply to SA2 based on online agreements (can also include some analysis from email discussion)

Intended outcome:

    - Discussion summary in R2-2010737 (by email rapporteur) and draft reply LS in R2-2010738.

Deadlines:

    - Deadline for email discussion report: 2nd week Thu, UTC 1000

Web Conf (1)

Outcome of [Post111-e][917][NR][Multi-SIM] Work prioritization for Multi-SIM (vivo):

R2-2009325	Summary of [Post111-e][917][Multi-SIM] Multi-Sim	vivo	discussion

Email discussion proposed conclusions are split to be discussed under respective agenda items

### 8.3.2	Paging collision avoidance

Including discussion on enhancement(s) to address the collision due to reception of paging when the UE is in IDLE/INACTIVE mode in both the networks associated with respective SIMs [RAN2]

Web Conf (1)

Outcome of [Post111-e][917][NR][Multi-SIM] Work prioritization for Multi-SIM (vivo):

R2-2009325	Summary of [Post111-e][917][Multi-SIM] Multi-Sim	vivo	discussion

Online

Proposal 1	(combine the proposal 1-3 and 5 above) From RAN2 point of view, Option 1 , 2a, 2b and 3 are feasible  to solve the paging collision issue in 5GS.

Observation 1	When effectiveness is considered, the option 1 has the following disadvantages:

a) Without UE assistant information, the new assigned 5G-GUTI may still result in PO collisions;

b) Paging collisions may occur after cell reselection in which case UE needs to request new 5G-GUTI again.

Observation 2	When effectiveness is considered, the option 2a has the following disadvantages:

a) Without UE assistant information, the assigned alternative UE_ID may still result in PO collisions;

b) Paging collisions may occur after cell reselection in which case UE needs to request a new alternative UE_ID again.

c) this option would change the legacy way to calculate PF/PO, thus impacts CN, RAN, UE.

Observation 3	When effectiveness is considered, the option 2b has the following disadvantages:

a) Without UE generated offset information, the IMSI offset may still result in PO collisions. R2-2006540 also shows that offset values of {1,2,3,4..} should be effective for dual/tri/quad… SIM;

b) Paging collisions may occur after cell reselection in which case UE needs to request a new UE offset again.

c) this option would change the legacy way to calculate PF/PO, thus impacts on MME, UE.

Proposal 4	more detailed information is needed to judge the feasibility and effectiveness of Option 2c (Calculation of PF/PO based on MUSIM Assistance Information).

Observation 4	When effectiveness is considered, the option 3 has the following disadvantage: this option would increase the signal overhead in the RAN.

Proposal 6	Online discussion is needed if the Option 4 (UE Implementation-based approach) is feasible from RAN2 point of views.

Observation 5	Standardized solution will ensure deterministic and uniform behavior from all UEs, and avoid impact on the paging latency, paging success performance and so on.

Proposal 7	Continue to evaluate the effectiveness of options 1, 2a, 2b, 3.

Proposal 8	For paging collision, “No E-UTRA impact” restriction applies to TS 36.331 at least. Online discussion is needed to determine if “No E-UTRA impact” restriction also applies to TS 36.413, TS 38.413, TS 36.304.

Proposal 9	From RAN2 point of view, when paging reception in one network colliding with data reception in another network is detected, the approach of Access Stratum-based solution with scheduling gap is feasible.

Proposal 1	(combine the proposal 1-3 and 5 above) From RAN2 point of view, Option 1 , 2a, 2b and 3 are feasible  to solve the paging collision issue in 5GS.

Proposal 4	more detailed information is needed to judge the feasibility and effectiveness of Option 2c (Calculation of PF/PO based on MUSIM Assistance Information).

Proposal 6	Online discussion is needed if the Option 4 (UE Implementation-based approach) is feasible from RAN2 point of views.

- From RAN2 point of view, Option 1 , 2a, 2b, and 3 are feasible to solve the paging collision issue in 5GS. Each have different effectiveness (as per analysis during the email discussion). When indicating reply to SA2, indicate both feasibility as well as effectiveness.

- Indicate to SA2 that RAN2 continues to further evaluate the pros and cons of options 1, 2a, 2b, 3.

- Option 4 is still allowed (but RAN2 will not specify UE implementation).

- Clarifying "No E-UTRA impact" can be done in RANP.

- Option 2c can be evaluated later as it doesn't work alone.

Discussion (1, 4, 6)

-	Xiaomi would prefer to use "alleviate" instead of "solve" for P1.

-	Charter thinks "feasible" should also consider the effectiveness of the solution. Option 1 and 2a are not every effective, for instance. QC agrees that we should indicate how well they solve the problem. Apple also agrees we should compare the effectiveness. ZTE agrees and thinks we could provide RAN2 preference. Nokia also agrees.

-	MediaTek thinks not all solutions are feasible in practice, e.g. changing UE ID. Should consider effectiveness first.

-	Vodafone thinks option 1 is for 5GC but not for EPC. Option 2b could work for both but was only intended for EPC in the SA2 LS. So 1 for 5GC and 2b for EPS would be preferred. QC wonders if we could use 2b for 5GC as well? Vodafone thinks it would impact AMF as well.

-	Vodafone thinks that paging repetition (option 3) consumes too many resources in RAN and will have bad impact to non-multi-SIM e.g. on battery life. Apple and ZTE agree. CATT also agrees.

-	MediaTek wonders why we would discus P6 since it's always possible. CATT agress and thinks option 4 is baseline. Nokia thinks this is possible but will not be standardized.

-	Ericsson thinks feasibility is fine for answering to SA2 but would be fine to indicate RAN2 conclusions on drawbacks. P4 seems not needed.

-	CATT thinks paging collision is quite low probability event.

-	Huawei supports P1 and wonders why we would discuss P6.

-	Huawei wonders why option 4 is not taken into account? QC thinks we will not specify UE implementation options. Huawei thinks we should only specify if something is needed.

Web Conf (3)

R2-2009326	Evaluation on Paging Collision Solutions	vivo	discussion

Proposal 1	Option2c(UE assistance information for MUSIM) can be considered later, since it  is not a solution that can work independently, and can be applied with other options.

Proposal 2	Option 4 (UE Implementation-based solution) is not feasible for the UE to solve paging collision issue.

Proposal 3	Enhancement for 5GS should be prioritized, for it can handle paging collision issue in both NR+NR and NR+LTE scenarios.

Proposal 4	Option1 (UE requested 5G-GUTI reassignment) is selected to solve the paging collision issue, since it is the simplest solution and can be applied for 5GS without RAN impact.

Discussion

-	Xiaomi and QC agree with P3. Samsung also agrees.

- Enhancement for 5GS should be prioritized since it can handle paging collision issue in both NR+NR and NR+LTE scenarios.

R2-2009556	Preventing paging collisions for Multi-SIM	Qualcomm Incorporated	discussion

Observation 1: Two USIMs are considered as two independent and separate UEs from NW perspective.

Observation 2: A solution specified on NR side can also resolve the collision for NR + LTE and NR + 3G cases.

Proposal 1: The solutions for paging collision resolution or avoidance should impact only NR specifications.

Observation 3: Paging collision can be resolved only an action taken by the network to resolve this.

Proposal 2: The UE will inform the NW of an existing or possible paging collision. The signaling can also include more information about the collision and UE suggestions to resolve it.

Observation 4: It can be assumed that the signaling is done only for one USIM.

Proposal 3: The signaling to report the paging collision (and possibly UE suggestions) will be done at NAS layer.

Observation 4: The re-allocation of UE ID (e.g. S-TMSI) will not be an efficient solution.

Proposal 4: RAN2 should focus on solutions for paging collision where the gNB will decide and implement the corrective action.

Proposal 5: AMF will inform the gNB of a paging collision and other information reported by the UE.

Proposal 6: RAN2 should discuss gNB initiated mechanisms which allow to page the UE at different times than the existing POs.

Proposal 7: RAN2 to discuss how the UE can determine if a gNB applies paging collision resolution solution, e.g. based on reporting the problem or by broadcasting of this support at cell level.

R2-2010534	Paging collision avoidance	Ericsson	discussion	LTE_NR_MUSIM-Core

Observation 1	It is expected that the probability of Paging Occasions collision in the two PLMNs is rather low and not systematic.

Observation 2	Given the low probability of Paging Occasions collision, a solution based on the UE implementation is expected to be a feasible/effective mean to minimize the impacts on the Paging reception caused by the collision, without introducing any specification change.

Observation 3	An alternative solution is to involve the network, e.g. by introducing an additional offset (generated by the CN and delivered to UE and gNB/eNB) which is used in the SFN and PO calculation formulas, whenever the PO collision occurs.

Proposal 1	RAN2 to agree on one of the two following solutions to cope with the Paging Occasion collision:

1) UE implementation based

2) Network based, by introducing an additional offset used in the SFN and PO calculation

R2-2009659	Consideration on Multi-SIM	China Telecom	discussion

R2-2009851	On Paging Collision Avoidance	Huawei, HiSilicon	discussion	Rel-17

R2-2009505	MUSIM Page Collision Avoidance	Apple	discussion	Rel-17	LTE_NR_MUSIM-Core

R2-2009264	Analysis of solutions for paging collision	Nokia, Nokia Shanghai Bell	discussion	Rel-17

R2-2009538	Effective Solution for Paging Collision Avoidance	Samsung R&D Institute India	discussion

R2-2009622	Consideration on the Paging Collision	ZTE Corporation, Sanechips	discussion	Rel-17	LTE_NR_MUSIM-Core

R2-2008832	Support of UE capabilities coordination for Dual Tx/Dual Rx Multi-USIM UEs	China Telecommunications	discussion

(moved from 8.3)

R2-2008955	Discussion on Paging Collision Avoidance	CATT	discussion	Rel-17	LTE_NR_MUSIM-Core

R2-2008871	Discussion on paging collision issue for multi-SIM	OPPO	discussion	Rel-17	LTE_NR_MUSIM-Core

R2-2009692	Definition and solution for paging collision, RRC Inactive, SI change	Lenovo, Motorola Mobility	discussion	Rel-17	LTE_NR_MUSIM-Core

R2-2009779	Discussion of the paging collision problem	Xiaomi Communications	discussion

R2-2009786	Support for Multi-SIM Devices - Paging Collision	MediaTek Inc.	discussion

R2-2009940	“Effective” solution for paging collision avoidance	Intel Corporation	discussion	Rel-17	LTE_NR_MUSIM-Core

R2-2010284	Consideration of Paging Collision Avoidance	LG Electronics	discussion	Rel-17

R2-2010427	UE indication of paging collision for Multi-SIM	ASUSTeK	discussion	LTE_NR_MUSIM-Core

R2-2010445	Considerations for Paging Collision for Multi-SIM UEs	Charter Communications, Inc	discussion	Rel-17

R2-2010596	RAN2 Impacts of Multi-USIM Paging	Futurewei Technologies	discussion

Withdrawn:

R2-2009739	Guidance for SA2 on Solution #16 for Key Issue 2 	VODAFONE Group Plc	discussion	Withdrawn

R2-2010482	Consideration on slice specific cell selection and reselection	ZTE corporation, Sanechips	discussion	Rel-17	FS_NR_slice	Withdrawn

### 8.3.3	UE notification on network switching for multi-SIM

Including discussion on mechanism for UE to notify Network A of its switch from Network A (for MUSIM purpose)

Web Conf (1)

R2-2009325	Summary of [Post111-e][917][Multi-SIM] Multi-Sim	vivo	discussion

Online

Proposal 10a	Using table 1 as a baseline on the discussion the expected time (in ms) required for UE to send a (NAS) busy indication to Network B.

Proposal 10b	Further online discussions needed to conclude whether scheduling gap is sufficient in network A for the UE to listen to paging and respond with BUSY indication. Table-1 can be considered as starting point for estimation of time delay calculation. This may be revisited based on the agreed signaling mechanism for BUSY indication.

Proposal 11	 From RAN2 point of view, it is feasible (and secure) that the busy indication is sent as an RRC message instead (no NAS message to the CN) i.e. as an RRC response to paging without requiring an RRC connection for RRC Inactive UE. FFS for idle UE.

Proposal 12	It is feasible to define an RRC-based switching/leaving and returning procedure in 5GS/NR.

Proposal 13	For now the changes to 5GS/E-UTRA (Option 5) to support RRC-based switching is not part of RAN Work Item. Online discussion is needed whether having solution in RRC signalling for LTE (Option 5) can be considered if RRC based switching is agreed as one solution for switching in RAN2 for NR RAN.

- Indicate to SA2 that the table 1 is a baseline on the discussion the expected time (in ms) required for UE to send a (NAS) busy indication to Network B.

- From RAN2 point of view, it is feasible that the busy indication is sent as an RRC message with security for RRC_INACTIVE. FFS how this works.

- RAN2 will continue to discuss RRC-based switching/leaving and returning procedure in 5GS/NR when UE is in RRC_CONNECTED. There may be different mechanisms (short/long, leaving/returning, etc.).

Discussion

-	MTK thinks it's fine to indicate NAS times but doesn't support it.

-	Charter thinks that P12 is about short leave only - long leave can be different. QC agrees.

-	LGE agrees with 10a, 11 and 12. Sending busy indication with RRC connection can be disruptive.

-	Nokia thinks P11 still requires security analysis even for RRC_INACTIVE. QC agrees.

Web Conf (3)

R2-2008872	Discussion on graceful leaving and busy indication	OPPO	discussion	Rel-17	LTE_NR_MUSIM-Core

Proposal 1: Case 1,2,3,4,5,6 are considered as events to trigger the leaving action.

-	Case 1: UE response paging message from USIM-B;

-	Case 2: UE will originate the service via USIM-B.

-	Case 3: UE receive updated system information

-	Case 4: UE monitor paging DCI and/or receive paging message.

-	Case 5: UE perform RRM measurement for cell reselection.

-	Case 6: UE perform MO signalling for some purposes, e.g. registration/TAU, check paging cause, send busy indication.

Proposal 2: Both pre-configured periodical duration for predictable leaving and one-shot leaving duration based on UE request are supported.

Proposal 3: For one-shot leaving duration based on UE request, it is up to network decision to make UE enter RRC_IDLE or RRC_INACTIVE or keep UE in RRC_CONNECTED for a pre-configured time duration.

Proposal 4: Both NAS and RRC based leaving indication are considered for different scenarios and RAN2 is kindly asked to discuss all the possible procedures listed above and down selection solution for leaving indication.

Proposal 5: It is up to UE implementation if UE cannot return to perform RRC resume procedure due to T380 expiries or across RNA boundary after leaving.

Proposal 6: NAS based leaving indication is also used for 5GS/E-UTRA (Option 5) case as E-UTRA/EPS case.

Proposal 7: The RRC based busy indication is supported for RRC_INACTIVE mode UE if SA2 agree to support busy indication.

Proposal 8: For RRC_INACTIVE UE, the MSG4 (RRCRelease) is used to confirm the busy indication carried in MSG3 if SA2 agree to support busy indication.

Proposal 9: For RRC_INACTIVE UE, RRC resume procedure without context relocation is supported for busy indication delivery if SA2 agree to support busy indication.

Proposal 10: The NAS based busy indication is supported for RRC_IDLE mode UE if SA2 agree to support busy indication.

Proposal 11: The details of the NAS based indication is up to CT1 and RAN3 if SA2 agree to support busy indication.

Proposal 12: The time duration for busy indication delivery cannot be guaranteed less than a certain period and it depends on UE’s best effort only.

R2-2010246	On coordinated switch from NW for MUSIM device	Huawei, HiSilicon	discussion	Rel-17	LTE_NR_MUSIM-Core

Proposal 1: The scenario 1 and Scenario 2 for UE switching from Network A in [2] are updated as below:

o	Scenario 1:  periodic switching, such as paging reception, measurements

o	Scenario 2:  aperiodic switching, such as TAU, RNAU, MO SMS , VoLTE/VoNR voice call

Proposal 2: The Scenario 3 for UE switching from Network A in [2] is updated as:

-	Dual-Rx /Single-Tx:

o	   Scenario 3: UE in RRC CONNECTED state in network A needs to switch part of RX capability to network B, where the UE is in RRC IDLE or RRC INACTIVE, for DL reception and hence change its RX capability in NW A.

Proposal 3: The issue of the updated Scenario 3 should be considered in this WI.

Proposal 4: A unified solution should be considered for addressing the paging reception issue for Single-Rx/Single-Tx and Dual-Rx/Single-Tx UE.

Proposal 5: UE sends connection release notification via RRC signalling by reusing legacy “ReleasePreference” and it can autonomously release the RRC connection after sending such notification.

R2-2010477	Network Switching for Multi-SIM UEs	Charter Communications, Inc	discussion	Rel-17

Proposal 1: RAN2 should investigate a multi-SIM UE’s co-ordinated leave [1] procedure to allow for both a short- and long- term leave from the network. The upper limit of duration for each leave, and decision entity for classification of such leaves (short- vs long-) can be FFS.

Proposal 2: RAN2 should consider short coordinated leaves where a multi-SIM UE notifies the network of the desired scheduling gaps, e.g. in order to monitor paging occasions in a second network.

Observation 1: In order to evaluate if a scheduling gap on a first network is sufficient for transmission of a busy indication on a second network, RAN2 should consider the total duration required and the expected behaviour from the UE given the paging cause on the second network.

Proposal 3: In RRC inactive state, the busy indication can be sent over RRC. However, in RRC idle state, the busy indication should be sent using a NAS message.

Proposal 4: UE automatously transitioning to RRC idle state possibly impact the first network negatively, hence RAN2 should aim for solutions that properly transition a short leave to a long leave.

Proposal 5: RAN2 may consider an RRC-based procedure for a short-coordinated leave, but for a long-coordinated NAS-based solution should be considered.

R2-2008956	Discussion on UE Notification on Network Switching	CATT	discussion	Rel-17	LTE_NR_MUSIM-Core

R2-2009265	Scenarios and Impact analysis for Switching Notification	Nokia, Nokia Shanghai Bell	discussion	Rel-17

R2-2009623	Consideration on the Switching Notification Procedure	ZTE Corporation, Sanechips	discussion	Rel-17	LTE_NR_MUSIM-Core

R2-2009658	RRC-based coordinated switch for multi-USIM UE	NEC	discussion	Rel-17	LTE_NR_MUSIM-Core

R2-2010350	Discussion on switching mechanism for multi-SIM	Samsung Electronics Co., Ltd	discussion	Rel-17	LTE_NR_MUSIM-Core

R2-2010544	Graceful leaving for a MultiSIM device	Ericsson	discussion	LTE_NR_MUSIM-Core

R2-2007602

R2-2008831	Discussion on various scenarios of UE switching from network for activities on another network	China Telecommunications	discussion	Rel-17

(moved from 8.3)

R2-2009327	UE notification on network switching for multi-SIM	vivo	discussion

R2-2009328	Discussion on Busy Indication Procedure	vivo	discussion

R2-2009506	MUSIM Network Switching	Apple	discussion	Rel-17	LTE_NR_MUSIM-Core

R2-2009557	Switching between two links for Multi-SIM	Qualcomm Incorporated	discussion

R2-2009781	Discussion of the UE switching problem	Xiaomi Communications	discussion

R2-2009787	Support for Multi-SIM Devices - Notification upon Network Switching	MediaTek Inc.	discussion

R2-2009856	Switching Notification in MUSIM	Lenovo, Motorola Mobility	discussion	Rel-17

R2-2009941	Regarding UE notification on network switching for multi-SIM	Intel Corporation	discussion	Rel-17	LTE_NR_MUSIM-Core

R2-2010286	SIM Switching Handling in MUSIM	LG Electronics	discussion	Rel-17

R2-2010428	Mechanism for UE to notify network switching	ASUSTeK	discussion	LTE_NR_MUSIM-Core

R2-2010620	RAN2 impacts of multi-SIM UE notifications on network switching	Futurewei Technologies	discussion

Withdrawn:

R2-2010481	Consideration on the slice specific RACH configuration	ZTE corporation, Sanechips	discussion	Rel-17	FS_NR_slice	Withdrawn

### 8.3.4	Paging with service indication

Including discussions on mechanism for an incoming page to indicate to the UE whether the service is voLTE/VoNR (pending SA2 feedback).

This agenda item may be deprioritized in this meeting.

R2-2009325	Summary of [Post111-e][917][Multi-SIM] Multi-Sim	vivo	discussion

Online

Discussion

- 	QC would be fine to agree to P17-19. Nokia and Apple agree. Xiaomi is fine with 18 and 19 but not 17.

-	Nokia thinks we may have to redo overhead calculations once SA3 provides feedback. Can provide current analysis but further work may be needed.

-	Huawei thinks we shuoldn't distinguish short/long time for SA2 rely.

- Provide SA2 with information on paging cause costs based on the email discussion + contributions. Indicate that this may change if assumptions change.

- From RAN2 perspective, we haven't decided on paging cause feasibility yet.

- RAN will evaluate short/long time switching in this WI

FFS on P20/21 - companies

AT112e: Email discussion on resolving P20/21 (vivo)

[AT112-e][241][Multi-SIM] Network switching scenarios (vivo)

Scope:

    - Discuss validity of scenario 3 and scenario 4 from the previous email disucssion

Intended outcome:

    - Discussion summary in R2-2010739 (by email rapporteur).

Deadlines:

    - Deadline for email discussion report: 2nd week Thu, UTC 1000

Proposal 14a	agree the observation 2.1 to evaluate the paging overlead by paging cause extension.

Observation 2.1	The overhead of paging cause is (1+⌈  〖log〗_2⁡〖number_of_paging cause〗  ⌉) bits per UE in E-UTRA and NR, if parallel list, the extension solution adopted in R16 E-UTRA paging message,  is applied for introducing paging causes.

Proposal 14b	From overhead point of view, it is feasible to have paging cause on Uu for EPS and 5GS.

Observation2.2	if the paging cause (3 bits per UE) is added, the paging message size is generally increased by ~6% for E-UTRA and ~8% for NR.

Proposal 15	Online discussion is needed whether this increasing will impact the real deployment about paging volume and coverage.

Proposal 16	If the paging cause is agreed by SA2, Paging cause is supported Per PLMN as the basline from RAN2 point of view. FFS if other options are needed.

Proposal 17	Address paging collision issue with standard-based solution, However simple solution is preferred.

Proposal 18	The issue of scenario 1 (short time switching, such as paging reception, measurements, TAU, RNAU, MO SMS) is considered in this WI.

Proposal 19	The issue of scenario 2 (Long-time switching, such as VoLTE/VoNR voice call) is considered in this WI.

Proposal 20	Online discussion is needed whether the issue of scenario 3 (UE in RRC CONNECTED state in network A and needs to switch to network B and hence change its RX capability in NW A) is considered in this WI.

Proposal 21	Online discussion is needed whether the issue of scenario 4 (UE in RRC CONNECTED state in network A and needs to switch to network B and hence change its Tx capability in NW A, such as dual connectivity)  is considered in this WI.

Observation 2.2	For now Dual-Tx is not the scope for switching notification bullet in WI.

Proposal 22	Paging cause should be specified in RAN2 after SA2 progress.

Post-meeting Email discussion

NW vendors:

R2-2008957	Discussion on Paging with Service Indication	CATT	discussion	Rel-17	LTE_NR_MUSIM-Core

R2-2009266	On RAN impacts for on paging cause	Nokia, Nokia Shanghai Bell	discussion	Rel-17

R2-2009624	Consideration on the Paging Cause	ZTE Corporation, Sanechips	discussion	Rel-17	LTE_NR_MUSIM-Core

R2-2009852	Discussion on the paging with service indication	Huawei, HiSilicon	discussion	Rel-17

R2-2010250	Discussion on support of paging cause for multi-SIM	Samsung Electronics Co., Ltd	discussion	Rel-17	LTE_NR_MUSIM-Core

R2-2010535	Introduction of a Paging cause indication	Ericsson	discussion	LTE_NR_MUSIM-Core	R2-2007603

UE vendors:

R2-2008873	Discussion on paging cause for multi-SIM	OPPO	discussion	Rel-17	LTE_NR_MUSIM-Core

R2-2009153	Discussion on the open issues of paging transmission	Spreadtrum Communications	discussion	Rel-17	LTE_NR_MUSIM

R2-2009507	MUSIM Paging with Service Indication	Apple	discussion	Rel-17	LTE_NR_MUSIM-Core

R2-2009558	Paging prioritization and response for MUSIM	Qualcomm Incorporated	discussion

R2-2009791	Support for Multi-SIM Devices - Paging Cause	MediaTek Inc.	discussion

R2-2010285	Consideration on Paging Cause Indication	LG Electronics	discussion	Rel-17

R2-2010416	Discussion of the paging cause support for MUSIM	Xiaomi Communications	discussion

## 8.8	RAN slicing SI

(FS_NR_slice; leading WG: RAN2; REL-17; WID: RP-193254)

Time budget: 0.5 TU

Tdoc Limitation: 2 tdocs

Email max expectation: 2 threads

### 8.8.1	Organizational

Including work plan, TR updates and any other rapporteur input.

Including outcome of [Post111-e][916][RAN slicing] RAN slicing study questions (CMCC)

Noted (3)

Incoming LSs with RAN2 in CC-field:

R2-2008732	LS on Enhancement of RAN Slicing (R3-205802; contact: Qualcomm)	RAN3	LS in	Rel-17	FS_NR_slice	To:SA2	Cc:RAN2

R2-2010688	LS on Cell Configuration within TA/RA to Support Allowed NSSAI (C1-206760; contact: Nokia)	CT1	LS in	Rel-17	FS_eNS_Ph2	To:SA2	Cc:RAN2, RAN3

R2-2010695	LS Reply on Enhancement of RAN Slicing (S2-2008240; contact: ZTE)	SA2	LS in	Rel-17	FS_eNS_Ph2	To:RAN3	Cc:RAN2

Flagging procedure will be used for these LSs (i.e. all will be noted without presentation unless flagged)

Noted (without presentation)

Incoming LSs with RAN2 in To-field:

Web Conf (1)

SA2 LS R2-2008759 (configuration within TA/RA to support allowed NSSAI):

R2-2008759	LS on Cell Configuration within TA/RA to Support Allowed NSSAI (S2-2006526; contact: ZTE)	SA2	LS in	Rel-17	FS_eNS_Ph2	To:RAN2, RAN3, CT1

Online

By Email [250] (2)

Draft replies to R2-2008759 (configuration within TA/RA to support allowed NSSAI):

R2-2010488	Reply LS on Cell Configuration within TA/RA to Support Allowed NSSAI	Qualcomm Incorporated 	LS out	Rel-17	FS_NR_slice	To:SA2, RAN3, CT1

R2-2010646	Draft reply LS on Cell Configuration within TA/RA to Support Allowed NSSAI	ZTE corporation, Sanechips	LS out	Rel-17	FS_NR_slice	To:SA2	Cc:CT1, RAN3

By email [250]

Web Conf (1)

SA2 LS R2-2010694 (restricting rate per UE per network slice):

R2-2010694	LS on restricting the rate per UE per network slice (S2-2007946; contact: Nokia)	SA2	LS in	Rel-17	FS_eNS_Ph2	To:RAN2, RAN3

Online

By Email [250] (3)

Draft replies to R2-2010694 (restricting rate per UE per network slice):

R2-2010184	Draft reply LS on restricting the rate per UE per network slice	Huawei	LS out	Rel-17	FS_eNS_Ph2	To:SA2	Cc:RAN3	Late

R2-2010183	Discussion on restricting the rate per UE per network slice	Huawei, HiSilicon	discussion	Rel-17	TEI17	Late

R2-2009669	Considerations on scenarios and solution space of RAN slicing enhancements	Lenovo, Motorola Mobility	discussion	Rel-17	FS_NR_slice

R2-2010987     [DRAFT] Reply LS on restricting the rate per UE per network slice                Nokia	LS out	Rel-17 FS_eNS_Ph2	To:SA2	Cc:RAN3	Late

By email [250]

By Email [250] (not kicked off before online discussion)

[AT112-e][250][Slicing] LS replies to SA2 and RAN3 (??)

Scope:

    - Attempt to create LS reply to the SA2 LSs

Intended outcome:

    - Discussion summary in R2-200xxxx (by email rapporteur).

Deadline for providing comments, for rapporteur inputs, conclusions and CR finalization:

    - Deadline for companies' feedback:  Friday moring 1st week

Web Conf (1)

Work plan update and draft TR:

R2-2010364	Revised Work Plan for RAN Slicing	CMCC, ZTE	Work Plan	Rel-17	FS_NR_slice	R2-2007420

R2-2010365	Draft TR 38.832	CMCC, ZTE	draft TR	Rel-17	38.832	0.2.0	FS_NR_slice

Online

Web Conf (1)

Outcome of [Post111-e][916][RAN slicing] RAN slicing study questions (CMCC):

R2-2010366	Report of [Post111-e][916][Slicing] Open issues for RAN slicing	CMCC	discussion	Rel-17	FS_NR_slice

Online

### 8.8.2	Slice based cell reselection under network control

Including discussion on proposals to address the issues for cell reselection identified in email discussion and whether or to which extent existing mechanisms can address them

Web Conf (2)

Operators:

R2-2008857	Considerations on slice aware cell priority	KDDI Corporation	discussion

R2-2009536	Discussion on slice based cell reselection under network control	China Unicom	discussion	FS_NR_slice

R2-2010367	Discussion on SA2 LS and solutions for slice-based cell reselection	CMCC	discussion	Rel-17	FS_NR_slice

R2-2009288	5G RAN Slicing Framework During Cell Selection / Reselection Phases	MITRE Corporation, DoD, NTIA	discussion	Rel-17	38.832

NW vendors:

R2-2008917	Slice based Cell Reselection under Network Control	CATT	discussion	Rel-17	FS_NR_slice

R2-2009067	Considerations for Slice-based cell (re)selection	Nokia, Nokia Shanghai Bell	discussion	Rel-17	FS_NR_slice

R2-2009174	Slice information for cell reselection	Samsung Electronics	discussion	Rel-17

R2-2009807	Consideration on slice specific cell selection and reselection	ZTE corporation, Sanechips	discussion	Rel-17	FS_NR_slice

R2-2009979	Evaluation of Access delay to slice	Ericsson	discussion	Rel-17	FS_NR_slice

R2-2009986	Solutions for fast access to slice	Ericsson	discussion	Rel-17	FS_NR_slice

R2-2010181	Slice based Cell (re)selection under network control	Huawei, HiSilicon	discussion	Rel-17	FS_NR_slice

UE vendors:

R2-2008949	Cell (re)selection based on preferred frequency(s) per slice	Beijing Xiaomi Software Tech	discussion

R2-2008950	Deployment scenarios of RAN slicing based on SA2 LSout	Beijing Xiaomi Software Tech	discussion

R2-2008963	Further discussion on RAN slicing enhancement	Qualcomm Incorporated	discussion	FS_NR_slice

R2-2009143	Discussion on slice based cell reselection	Spreadtrum Communications	discussion	Rel-17	FS_NR_slice

R2-2009198	Consideration for slice based cell (re)selection	Intel Corporation	discussion	Rel-17	FS_NR_slice

R2-2009473	Discussion on slice based cell selection and re-selection	Apple	discussion	Rel-17	FS_NR_slice

R2-2009542	Consideration on slice-based cell (re)selection	OPPO	discussion	Rel-17	FS_NR_slice

R2-2009644	Assistant information to enable UE fast access network slice	ITRI	discussion	FS_NR_slice

R2-2009689	Remaining issues on slice-based (re)-selection	vivo	discussion	Rel-17	FS_NR_slice

R2-2010063	Cell selection and reselection for RAN slicing	Google	discussion

R2-2010065	Discussion on Network Slicing’s Impact on Cell (Re-)Selection	Convida Wireless	discussion	Rel-17	FS_NR_slice

R2-2010222	Further discussion on how to decide intended slice for idle mobility	LG Electronics UK	discussion	Rel-17

### 8.8.3	Slice based RACH configuration or access barring

Including discussion on proposals to address the issues for RACH/access barring identified in email discussion and whether or to which extent existing mechanisms can address them

Web Conf (2)

R2-2009806	Consideration on the slice specific RACH configuration	ZTE corporation, Sanechips	discussion	Rel-17	FS_NR_slice

Observation 1: Although broadcasting NSSAI/S-NSSAI (or parts of it) is acceptable to some slices without security concern, the NSSAI/S-NSSAI (or parts of it) shall not be exposed in system information for some security/privacy sensitive slices.

Proposal 1: A common solution of slice specific RACH configuration is needed for slice with/without security concern on exposing of NSSAI/S-NSSAI (or parts of it).

Proposal 2: The association between RACH resources and operator defined access categories can be broadcast in system information to link the RACH resources with slices implicitly.

Proposal 3: RA prioritization (including powerRampingStepHighPriority and scalingFactorBI) for operator defined access categories can be introduced in system information to prioritize random access for certain slices implicitly.

R2-2009199	Consideration of Slice based RACH	Intel Corporation	discussion	Rel-17	FS_NR_slice

Use cases and intentions for slice-based RACH:

Observation#1: UAC based on access category may be sufficient to provide access control also for slice, since each slice can correspond to a user defined access category.

Observation#2: RA resource isolation for critical slice or slice group may reduce system capacity and waste precious RACH resource if the resource utilisation for the slices are uneven.

Observation#3: RA prioritization has been applied to critical mission services such as MPS and MCS. It could be beneficial to apply it to some critical slices (e.g. URLLC).

Proposal#1: Request RAN2 to study applying RA prioritization to slice.

Application of RA prioritization for slice-based RACH:

Proposal#2: As baseline, existing RA prioritization with the configured parameters powerRampingStepHighPriority and scalingFactorBI can be supported for critical slice.

Proposal#3: Use the operator defined access categories to provide RA prioritization for slice in MO access case.

Proposal#3_1: Broadcast the operator defined access categories with their corresponding RA prioritization in SIB.

Proposal#3_2: UE AS selects the corresponding RA prioritization based on the operator defined access category provided by NAS for the RA procedure triggered by RRC establishment and resumption from RRC.

R2-2010182	Slice based RACH configuration or access barring	Huawei, HiSilicon	discussion	Rel-17	FS_NR_slice

R2-2009474	Discussion on slice based RACH and cell barring	Apple	discussion	Rel-17	FS_NR_slice

R2-2009175	RACH configuration for RAN slicing	Samsung Electronics	discussion	Rel-17

R2-2009543	Consideration on slice-based RACH	OPPO	discussion	Rel-17	FS_NR_slice

R2-2009688	Remaining issues on RACH and service continuity	vivo	discussion	Rel-17	FS_NR_slice

R2-2010223	Discussion on slice aware overload control	LG Electronics UK	discussion	Rel-17

R2-2009974	RACH enhancements to enable UE fast access to the intended slice	NEC Telecom MODUS Ltd.	discussion

R2-2009423	RACH prioritisation for slices	Nokia, Nokia Shanghai Bell	discussion	Rel-17	FS_NR_slice	Late

# 10	Breakout session reports

No documents shall be submitted to this AI or its sub-AIs. It is only for at-meeting-generated contents.

Breakout session reports will be approved by email.

## 10.1	Session on LTE legacy, Mobility, DCCA, Multi-SIM and RAN slicing

R2-2010701	Report from session on LTE legacy, LTE TEI16 and NR/LTE Rel-16 Mobility	Vice Chairman (Nokia)

# Summary

Agreed CRs

LTE legacy (Rel-15 and earlier)

LTE Rel-16 miscellaneous

Rel-16 LTE/NR mobility

Rel-16 DCCA

CRs merged to another document (fully or partly)

Rel-16 DCCA

Rel-16 LTE/NR mobility

Endorsed CRs

Rel-16 LTE/NR mobility

Endorsed TR

Rel-17 RAN slicing

Approved LS out

Post-meeting email discussions (short)

Post-meeting email discussions (long)