3GPP TSG-RAN WG2 #112e	Tdoc R2-20xxxx

Electronic meeting, November 2nd – 13th 2020

Agenda Item:	6.2.3

Source:	Ericsson

Title:	Summary of [AT112-e][037][IAB] User Plane

Document for:	Discussion, Decision

# 1	Introduction

This paper addresses the following email discussion:

- [AT112-e][037][IAB] User Plane (Ericsson)

Treat tdocs under 6.2.3

Intended outcome: Intermediate: Determine agreeable parts. Final: For agreeable parts, agreed CRs.

Deadline: Intermediate deadline(s) by Rapporteur, Final: Discussion stop at Wed Nov 11, 1200 UTC

The rapporteur would like to set the following deadlines:

Deadline 1: Friday, Nov. 6 EOB for answers to the questionnaire in the summary

Deadline 2: Wed, Nov 11 1200 UTC for CR wording (if needed).

# 2	Discussion

This email discussion handles the following documents submitted at RAN2#112 regarding IAB user plane:

R2-2009745	Correction on Tdelta	Huawei, HiSilicon	CR	Rel-16	38.321	16.2.1	0938	-	F	NR_IAB-Core

R2-2010152	Correction to tDelta	Ericsson	CR	Rel-16	38.321	16.2.1	0963	-	F	NR_IAB-Core

R2-2010150	Pre-emptive BSR handling at MAC Reset	Ericsson	CR	Rel-16	38.321	16.2.1	0962	-	F	NR_IAB-Core

R2-2010419	Correction on the condition check in Pre-emptive BSR procedure	ASUSTeK	CR	Rel-16	38.321	16.2.1	0984	-	F	NR_IAB-Core

R2-2009324	CR to 38.322 on Backhaul RLC Channel	vivo	CR	Rel-16	38.322	16.1.0	0037	-	F	NR_IAB-Core

Moved from 6.2.6

R2-2010684	Summary of Rel-16 IAB UP issues and corrections	Samsung	discussion	Rel-16	NR_IAB-Core

## 2.1	Correction to tDelta

The following CRs handle this issue:

- R2-2009745	Correction on Tdelta	Huawei, HiSilicon	CR	Rel-16	38.321	16.2.1	0938	-	F	NR_IAB-Core

- R2-2010152	Correction to tDelta	Ericsson	CR	Rel-16	38.321	16.2.1	0963	-	F	NR_IAB-Core

Both CRs intend to fix the mismatch between the RAN1 terminology in TS 38.213 and TS 38.321. From TS 38.213 the Tdelta  is the index value, while in MAC specification, the terminology Tdelta and T_delta is used to represent the index value.

The two CRs are very similar. The only difference is that R2-2009745 proposes to remove Tdelta, and T_delta and hence also to change the MAC. Instead, R2-2010152 proposes to keep the legacy MAC CE (with T_delta field) and just remove Tdelta.

Q1: Which option do you prefer to correct this issue?

- The solution in R2-2009745

- The solution in R2-2010152

| Company | Preference (1/2) | Detailed Comments |
| --- | --- | --- |

## 2.2	Correction to Pre-emptive BSR handling at MAC Reset

- R2-2010150	Pre-emptive BSR handling at MAC Reset	Ericsson	CR	Rel-16	38.321	16.2.1	0962	-	F	NR_IAB-Core

The CR is to specify that a triggered pre-emptive BSR shall be cancelled at MAC reset. This is in line with many other triggered procedures, e.g. Buffer Status Reporting procedure, that shall be cancelled at MAC reset.

Q2: Is there any concern with the correction in R2-2010150?

| Company | Preference (Yes/No) | Detailed Comments |
| --- | --- | --- |

## 2.3	Correction on Pre-emptive BSR procedure

- R2-2010419	Correction on the condition check in Pre-emptive BSR procedure	ASUSTeK	CR	Rel-16	38.321	16.2.1	0984	-	F	NR_IAB-Core

The CR is to fix an editorial issue, i.e the word “Pre-emptive” is missing in the Pre-emptive Buffer Status Reporting procedure.

Q3: Is there any concern with the correction in R2-2010419?

| Company | Preference (Yes/No) | Detailed Comments |
| --- | --- | --- |

## 2.4	Correction on Backhaul RLC Channel to TS 38.322

- R2-2009324	CR to 38.322 on Backhaul RLC Channel	vivo	CR	Rel-16	38.322	16.1.0	0037	-	F	NR_IAB-Core

Intention of the CR is to clarify that in case of IAB the data are transferred between RLC and BAP via a single BH RLC channel, rather than via a single RLC channel as it is in legacy non-IAB networks. There is also a second change to clarify that the BAP procedures are specified in TS 38340.

Rapporteur´s view: If RAN2 intends to agree on the intention of this CR, Rapporteur believes that it would be enough to just state that “In case the upper layer is BAP as defined in TS 38.340 [xx], an RLC channel refers to a Backhaul RLC channel”. Otherwise, it might be misleading stating that data are transferred through a single BH RLC channel.

Q4: Do you agree with the intention of the changes in R2-2009324?

| Company | Preference (Yes/No) | Detailed Comments |
| --- | --- | --- |

If the answer to Q4 is “Yes” related to the first change, do you agree with Rapporteur´s proposal to replace the NOTE in R2-2009324 with a NOTE stating “In case the upper layer is BAP as defined in TS 38.340 [xx], an RLC channel refers to a Backhaul RLC channel”

Q5: If the answer to Q4 is yes, do you agree with the Rapporteur proposal to add the following NOTE?

NOTE:	In case the upper layer is BAP as defined in TS 38.340 [xx], an RLC channel refers to a Backhaul RLC channel.

| Company | Preference (Yes/No) | Detailed Comments |
| --- | --- | --- |

# 3	Conclusion