3GPP TSG-RAN WG2 Meeting #112-e	R2-200xxxx

Online, 2nd - 13th  November 2020

Agenda item:	4.2

Source:	Huawei

Title:	Draft summary of	[AT112-e][401][NB-IoT/eMTC R15] UP EDT for DRB using RLC AM (Huawei)

Document for:	Report

# 1	Scope of the offline discussion

This is the offline email discussion “[AT112-e][401][NB-IoT/eMTC R15] UP EDT for DRB using RLC AM”, as indicated below:

[AT112-e][401][NB-IoT/eMTC R15] UP EDT for DRB using RLC AM (Huawei)

Status: Started

Scope: Discuss how to capture the agreement on how the procedure ends at the UE for MO-EDT, MT-EDT or PUR in the specifications.

Intended outcome: Agreed 36.300 and 36.331 CRs in R2-2010810, R2-2010811, R2-2010812, and R2-2010813

Deadline:  Tuesday 2020-11-10 14:00 UTC

# 2	Offline discussion

The summary of offline discussion [Post111-e][922][NB-IoT/eMTC R15] UP EDT for DRB using RLC AM (Huawei) [1] was discussed  and the following agreeement was made:

For MO-EDT, MT-EDT or PUR, the procedure ends at the UE with the transmission of HARQ ACK for Msg4, i.e., without responding to the poll bit, if any, or waiting for 1.25sec/10 sec. FFS how to capture this in the specifications.

This offline discussion is to discuss and agree on what and how to capture in the specification.

In [1], it was proposed to capture the above as follows:

- Proposal 5: Capture in stage 2 that the EDT procedure terminates with the transmission of HARQ ACK for MSG4 which is an implicit acknowledgment of the successful delivery of the DL data. ([2])

- Proposal 6: Capture in RRC specification, that upon reception of RRCConnectionRelease for EDT, the UE can proceed without delay with the release of the resources, regardless of any poll bit. ([4])

Proposal 1: Capture in stage 2 that, for MO-EDT, MT-EDT and PUR, the EDT procedure ends with the transmission of HARQ ACK for MSG4 which is an implicit acknowledgment of the successful delivery of the DL data

Companies to provide their views on the proposal and provide details on the required changes as needed.

| Company | Do you agree? Yes/No | Comments |
| --- | --- | --- |

Conclusion:

Proposal 2: Capture in RRC specification, that upon reception of RRCConnectionRelease for EDT, the UE proceeds without delay with the release of the resources (i.e, regardless of any poll bit).

Companies to provide their views on the proposal and provide details on the required changes as needed.

| Company | Do you agree? Yes/No | Comments |
| --- | --- | --- |

Conclusion:

Companies to provide their views on whether other changes are required and in which specification(s).

| Company | Comments |
| --- | --- |

Conclusion:

# 3	Conclusion

# 4	Reference

R2-2009723	Report of  e-mail discussion [Post111-e][922][NB-IoT/eMTC R15] UP EDT for DRB using RLC AM (Huawei)	Huawei, HiSilicon	report	Rel-15	NB_IOTenh2-Core, LTE_eMTC4-Core

R2-2009724	Clarification to UP-EDT	Huawei, HiSilicon	CR	Rel-15	36.300	15.11.0	1298	1	F	NB_IOTenh2-Core, LTE_eMTC4-Core	R2-2007328

R2-2009725	Clarification to UP-EDT	Huawei, HiSilicon	CR	Rel-16	36.300	16.3.0	1299	1	A	NB_IOTenh2-Core, LTE_eMTC4-Core	R2-2007329

R2-2009726	Clarification to UP-EDT	Huawei, HiSilicon	CR	Rel-15	36.331	15.11.0	4477	-	F	NB_IOTenh2-Core, LTE_eMTC4-Core

R2-2009727	Clarification to UP-EDT	Huawei, HiSilicon	CR	Rel-16	36.331	16.2.1	4478	-	A	NB_IOTenh2-Core, LTE_eMTC4-Core

# 4	Participants

| Company | Name | e-mail address |
| --- | --- | --- |