3GPP TSG-RAN WG2 Meeting #112 electronic				Draft R2-2010866

Elbonia, Nov 2nd – 13th 2020

Agenda item:		8.11.2

Source:			Intel Corporation

Title:		Summary of latency results

Document for:	 	Discussion and decision

- Introduction

This is to summarize the call flow and corresponding latency results for each positioning methods (DL-TDOA, DL AoD, UL-TDoA, UL-AoA, Multi-RTT, DL NR E-CID, UL NR E-CID).

Note: DL-TDOA and DL AoD are handled together in the same call flow and same table, and UL-TDoA and UL-AoA are also handled together in the same call flow and same table.

- Summary

Latency components and values used for evaluation

During the email discussion [R2-2009001], latency assumptions for different processing stages and nodes were derived. These latencies include processing delays of the various involved nodes (UE, gNB, AMF, LMF) and signalling delays between nodes.

Table 1 below summarizes the latency assumptions for the various components, as proposed in [R2-2009001].

Table 1: 	Latency Components

| Label | Latency [ms] | Description |
| --- | --- | --- |
| Processing Latencies |  |  |
| TUEProc-RRCReconf | 10 | RRC Reconfiguration processing |
| TUEProc-RRCDLInfo | 5 | RRC DL information transfer |
| TUEProc-RRCULInfo | 2-5 | RRC UL information transfer |
| TUEProc-RRCLocationMeas | 2-5 | RRC Location Measurement Indication |
| TUEProc-LPPCapab | 10-20 | LPP Provide Capabilities |
| TUEProc-LPPAssi | 10 | LPP Provide Assistance Data |
| TUEProc-LPPLocationRe | 5 | LPP Request/Provide Location Information |
| TUEProc-MAC-SRSAct | 1-3 | MAC-CE SRS Activation/Deactivation |
| TgNBProc-RRC | 3 | RRC Processing |
| TgNBProc-NRPPa | 3 | NRPPa Processing |
| TgNBProc-NAS/LPP | 3 | NAS/LPP Processing |
| TAMFProc | 3 | AMF Processing |
| TLMFProc | 3 | LMF Processing |
| Signalling Propagation Delays between Nodes |  |  |
| TUE-gNB | 0-0.5 |  |
| TgNB-AMF | 3-10 |  |
| TAMF-LMF | 1-10 |  |
| TAMF-GMLC | 3-10 |  |
| Positioning Measurement Latencies |  |  |
| TLMF-Calc | 2-30 | Position Calculation latency |
| TDL-Meas | Wait for RAN1 | PHY DL-PRS measurement time; best possible case |
| TUL-Meas | Wait for RAN1 | PHY UL-PRS measurement time; assume the same value as for DL-PRS. |

Call flow and latency analysis for DL-TDOA/DL-AoD

The figure 1 is used for latency analysis for DL-TDOA and DL-AoD.

Figure 1 procedure for DL-TDOA/DL-AoD

Table 2 summarizes the latency for UE assisted DL-TDOA and DL-AoD.

Table 2: 	Latency for UE assisted DL-TDOA and DL-AoD

| Positioning technique [DL-TDOA/DL-AoD, mode [UE-A] Figure 1 |  |  |
| --- | --- | --- |
| Latency Component | Value Range (ms) | Description of Latency Component |
| Step 1 LPP Request capabilities | 18-34.5 | TLMFProc+ TAMF-LMF+ TAMFProc +TgNB-AMF + TgNBProc-NAS/LPP +TUE-gNB+ TUEProc-RRCDLInfo Processing delays: 14ms - UE: TUEProc-RRCDLInfo= 5ms - gNB: TgNBProc-NAS/LPP= 3ms - AMF: TAMFProc= 3ms - LMF: TLMFProc= 3ms Signalling delay:4-20.5ms - UE-gNB: TUE-gNB= 0-0.5ms - gNB-AMF: TgNB-AMF= 3-10ms - AMF-LMF: TAMF-LMF= 1-10ms Note 1: the LPP capability processing delay is counted together in response message. |
| Step 2 LPP Provide Capabilities | 25-54.5 | TLMFProc+ TAMF-LMF+ TAMFProc +TgNB-AMF + TgNBProc-NAS/LPP +TUE-gNB+ TUEProc-RRCULInfo+ TUEProc-LPPCapab Processing delays: 21-34ms - UE: TUEProc-RRCULInfo= 2-5ms TUEProc-LPPCapab= 10-20ms - gNB: TgNBProc-NAS/LPP= 3ms - AMF: TAMFProc= 3ms - LMF: TLMFProc= 3ms Signalling delay:4-20.5ms - UE-gNB: TUE-gNB= 0-0.5ms - gNB-AMF: TgNB-AMF= 3-10ms - AMF-LMF: TAMF-LMF= 1-10ms |
| Step 3 LPP Provide Assistance Data | 28-44.5 | TLMFProc+ TAMF-LMF+ TAMFProc +TgNB-AMF + TgNBProc-NAS/LPP +TUE-gNB+ TUEProc-RRCDLInfo+ TUEProc-LPPAssi Processing delays: 24ms - UE: TUEProc-RRCDLInfo= 5ms TUEProc-LPPAssi= 10ms - gNB: TgNBProc-NAS/LPP= 3ms - AMF: TAMFProc= 3ms - LMF: TLMFProc= 3ms Signalling delay:4-20.5ms - UE-gNB: TUE-gNB= 0-0.5ms - gNB-AMF: TgNB-AMF= 3-10ms - AMF-LMF: TAMF-LMF= 1-10ms |
| Step 4 LPP Request Location Information | 23-39.5 | TLMFProc+ TAMF-LMF+ TAMFProc +TgNB-AMF + TgNBProc-NAS/LPP +TUE-gNB+ TUEProc-RRCDLInfo+ TUEProc-LPPLocationRe Processing delays: 19ms - UE: TUEProc-RRCDLInfo= 5ms TUEProc-LPPLocationRe = 5ms - gNB: TgNBProc-NAS/LPP= 3ms - AMF: TAMFProc= 3ms - LMF: TLMFProc= 3ms Signalling delay:4-20.5ms - UE-gNB: TUE-gNB= 0-0.5ms - gNB-AMF: TgNB-AMF= 3-10ms - AMF-LMF: TAMF-LMF= 1-10ms |
| Step 5 RRC Location Measurement Indication | 5-8.5 | TUEProc-RRCLocationMeas + TUE-gNB+ TgNBProc-RRC Processing delays: 5-8ms - UE: TUEProc-RRCLocationMeas = 2-5ms - gNB: TgNBProc-RRC= 3ms Signalling delay:0-0.5ms - UE-gNB: TUE-gNB= 0-0.5ms |
| Step 6 RRC Measurement Gap configuration | 13-13.5 | TgNBProc-RRC+ TUE-gNB+ TUEProc-RRCReconf Processing delays: 13ms - UE: TUEProc-RRCReconf = 10ms - gNB: TgNBProc-RRC= 3ms Signalling delay:0-0.5ms - UE-gNB: TUE-gNB= 0-0.5ms |
| Step 7 DL PRS measurement | TDL-Meas | RAN1 inputs |
| Step 8 LPP Provide Location Information | 20-39.5 | TLMFProc+ TAMF-LMF+ TAMFProc +TgNB-AMF + TgNBProc-NAS/LPP +TUE-gNB+ TUEProc-RRCULInfo+ TUEProc-LPPLocationRe Processing delays: 16-19ms - UE: TUEProc-RRCULInfo= 2-5ms TUEProc-LPPLocationRe= 5ms - gNB: TgNBProc-NAS/LPP= 3ms - AMF: TAMFProc= 3ms - LMF: TLMFProc= 3ms Signalling delay:4-20.5ms - UE-gNB: TUE-gNB= 0-0.5ms - gNB-AMF: TgNB-AMF= 3-10ms - AMF-LMF: TAMF-LMF= 1-10ms |
| Step 9 LMF calculation | 2-30 | TLMF-Calc |
| Total values | 134.5-264.5 | Note 2: TDL-Meas is not counted. |

Call flow and latency analysis for UL-TDOA/UL-AoA

The figure 2 is used for latency analysis for UL-TDOA and UL-AoA.

Figure 2 procedure for UL-TDOA and UL-AoA

Table 3 summarizes the latency for UE assisted UL-TDOA and UL-AoA.

Table 3: 	Latency for UE assisted UL-TDOA and UL-AoA

| Positioning technique [UL-TDOA/UL-AoA, mode [UE-A] Figure 2 |  |  |
| --- | --- | --- |
| Latency Component | Value Range (ms) | Description of Latency Component |
| Step 1 LPP Request capabilities | 18-34.5 | TLMFProc+ TAMF-LMF+ TAMFProc +TgNB-AMF + TgNBProc-NAS/LPP +TUE-gNB+ TUEProc-RRCDLInfo Processing delays: 14ms - UE: TUEProc-RRCDLInfo= 5ms - gNB: TgNBProc-NAS/LPP= 3ms - AMF: TAMFProc= 3ms - LMF: TLMFProc= 3ms Signalling delay:4-20.5ms - UE-gNB: TUE-gNB= 0-0.5ms - gNB-AMF: TgNB-AMF= 3-10ms - AMF-LMF: TAMF-LMF= 1-10ms Note: the LPP capability processing delay is counted together in response message. |
| Step 2 LPP Provide Capabilities | 25-54.5 | TLMFProc+ TAMF-LMF+ TAMFProc +TgNB-AMF + TgNBProc-NAS/LPP +TUE-gNB+ TUEProc-RRCULInfo+ TUEProc-LPPCapab Processing delays: 21-34ms - UE: TUEProc-RRCULInfo= 2-5ms TUEProc-LPPCapab= 10-20ms - gNB: TgNBProc-NAS/LPP= 3ms - AMF: TAMFProc= 3ms - LMF: TLMFProc= 3ms Signalling delay:4-20.5ms - UE-gNB: TUE-gNB= 0-0.5ms - gNB-AMF: TgNB-AMF= 3-10ms - AMF-LMF: TAMF-LMF= 1-10ms |
| Step 3 NRPPa POSITIONING INFORMATION REQUEST | 13-29 | TLMFProc+ TAMF-LMF+ TAMFProc +TgNB-AMF + TgNBProc-NRPPa Processing delays: 9 ms - gNB: TgNBProc-NRPPa= 3ms - AMF: TAMFProc= 3ms - LMF: TLMFProc= 3ms Signalling delay:4-20ms - gNB-AMF: TgNB-AMF= 3-10ms - AMF-LMF: TAMF-LMF= 1-10ms |
| Step 4 RRC SRS configuration | 13-13.5 | TgNBProc-RRC+ TUE-gNB+ TUEProc-RRCReconf Processing delays: 13ms - UE: TUEProc-RRCReconf = 10ms - gNB: TgNBProc-RRC= 3ms Signalling delay:0-0.5ms - UE-gNB: TUE-gNB= 0-0.5ms |
| Step 5 NRPPa POSITIONING INFORMATION RESPONSE | 13-29 | TLMFProc+ TAMF-LMF+ TAMFProc +TgNB-AMF + TgNBProc-NRPPa Processing delays: 9 ms - gNB: TgNBProc-NRPPa= 3ms - AMF: TAMFProc= 3ms - LMF: TLMFProc= 3ms Signalling delay:4-20ms - gNB-AMF: TgNB-AMF= 3-10ms - AMF-LMF: TAMF-LMF= 1-10ms |
| Step 6 NRPPa Request UE SRS activation | 13-29 | TLMFProc+ TAMF-LMF+ TAMFProc +TgNB-AMF + TgNBProc-NRPPa Processing delays: 9 ms - gNB: TgNBProc-NRPPa= 3ms - AMF: TAMFProc= 3ms - LMF: TLMFProc= 3ms Signalling delay:4-20ms - gNB-AMF: TgNB-AMF= 3-10ms - AMF-LMF: TAMF-LMF= 1-10ms |
| Step 7 MAC Activate UE SRS transmission | 1-3.5 | TUE-gNB +TUEProc-MAC-SRSAct Processing delays: 13ms - UE: TUEProc-MAC-SRSAct = 1-3ms Signalling delay:0-0.5ms - UE-gNB: TUE-gNB= 0-0.5ms |
| Step 8 NRPPa Request UE SRS activate Response | 13-29 | TLMFProc+ TAMF-LMF+ TAMFProc +TgNB-AMF + TgNBProc-NRPPa Processing delays: 9 ms - gNB: TgNBProc-NRPPa= 3ms - AMF: TAMFProc= 3ms - LMF: TLMFProc= 3ms Signalling delay:4-20ms - gNB-AMF: TgNB-AMF= 3-10ms - AMF-LMF: TAMF-LMF= 1-10ms |
| Step 9 NRPPa MEASUREMENT REQUEST | 13-29 | TLMFProc+ TAMF-LMF+ TAMFProc +TgNB-AMF + TgNBProc-NRPPa Processing delays: 9 ms - gNB: TgNBProc-NRPPa= 3ms - AMF: TAMFProc= 3ms - LMF: TLMFProc= 3ms Signalling delay:4-20ms - gNB-AMF: TgNB-AMF= 3-10ms - AMF-LMF: TAMF-LMF= 1-10ms |
| Step 10 UL SRS measurement | TUL-measc | RAN1 inputs |
| Step 11 NRPPa MEASUREMENT RESPONSE | 13-29 | TLMFProc+ TAMF-LMF+ TAMFProc +TgNB-AMF + TgNBProc-NRPPa Processing delays: 9 ms - gNB: TgNBProc-NRPPa= 3ms - AMF: TAMFProc= 3ms - LMF: TLMFProc= 3ms Signalling delay:4-20ms - gNB-AMF: TgNB-AMF= 3-10ms - AMF-LMF: TAMF-LMF= 1-10ms |
| Step 12 LMF calculation | 2-30 | TLMF-Calc LMF calculation/estimation delay: 2-30 TLMF-Calc |
| Total values | 137.5-310 | Note 1: TUL-measc is not counted; |

Call flow and latency analysis for Multi-RTT

The figure 3 is used for latency analysis for Multi-RTT.

Figure 3 procedure for Multi-RTT

Table 4 summarizes the latency for UE assisted Multi-RTT.

Table 4: 	Latency for UE assisted Multi-RTT

| Positioning technique [Multi-RTT] [UE-A] Figure 3 |  |  |
| --- | --- | --- |
| Latency Component | Value Range (ms) | Description of Latency Component |
| Step 1 LPP Request capabilities | 18-34.5 | TLMFProc+ TAMF-LMF+ TAMFProc +TgNB-AMF + TgNBProc-NAS/LPP +TUE-gNB+ TUEProc-RRCDLInfo Processing delays: 14ms - UE: TUEProc-RRCDLInfo= 5ms - gNB: TgNBProc-NAS/LPP= 3ms - AMF: TAMFProc= 3ms - LMF: TLMFProc= 3ms Signalling delay:4-20.5ms - UE-gNB: TUE-gNB= 0-0.5ms - gNB-AMF: TgNB-AMF= 3-10ms - AMF-LMF: TAMF-LMF= 1-10ms Note 1: the LPP capability processing delay is counted together in response message. |
| Step 2 LPP Provide Capabilities | 25-54.5 | TLMFProc+ TAMF-LMF+ TAMFProc +TgNB-AMF + TgNBProc-NAS/LPP +TUE-gNB+ TUEProc-RRCULInfo+ TUEProc-LPPCapab Processing delays: 21-34ms - UE: TUEProc-RRCULInfo= 2-5ms TUEProc-LPPCapab= 10-20ms - gNB: TgNBProc-NAS/LPP= 3ms - AMF: TAMFProc= 3ms - LMF: TLMFProc= 3ms Signalling delay:4-20.5ms - UE-gNB: TUE-gNB= 0-0.5ms - gNB-AMF: TgNB-AMF= 3-10ms - AMF-LMF: TAMF-LMF= 1-10ms |
| Step 3 NRPPa POSITIONING INFORMATION REQUEST | 13-29 | TLMFProc+ TAMF-LMF+ TAMFProc +TgNB-AMF + TgNBProc-NRPPa Processing delays: 9 ms - gNB: TgNBProc-NRPPa= 3ms - AMF: TAMFProc= 3ms - LMF: TLMFProc= 3ms Signalling delay:4-20ms - gNB-AMF: TgNB-AMF= 3-10ms - AMF-LMF: TAMF-LMF= 1-10ms |
| Step 4 RRC SRS configuration | 13-13.5 | TgNBProc-RRC+ TUE-gNB+ TUEProc-RRCReconf Processing delays: 13ms - UE: TUEProc-RRCReconf = 10ms - gNB: TgNBProc-RRC= 3ms Signalling delay:0-0.5ms - UE-gNB: TUE-gNB= 0-0.5ms |
| Step 5 NRPPa POSITIONING INFORMATION RESPONSE | 13-29 | TLMFProc+ TAMF-LMF+ TAMFProc +TgNB-AMF + TgNBProc-NRPPa Processing delays: 9 ms - gNB: TgNBProc-NRPPa= 3ms - AMF: TAMFProc= 3ms - LMF: TLMFProc= 3ms Signalling delay:4-20ms - gNB-AMF: TgNB-AMF= 3-10ms - AMF-LMF: TAMF-LMF= 1-10ms |
| Step 6 NRPPa Request UE SRS activation | 13-29 | TLMFProc+ TAMF-LMF+ TAMFProc +TgNB-AMF + TgNBProc-NRPPa Processing delays: 9 ms - gNB: TgNBProc-NRPPa= 3ms - AMF: TAMFProc= 3ms - LMF: TLMFProc= 3ms Signalling delay:4-20ms - gNB-AMF: TgNB-AMF= 3-10ms - AMF-LMF: TAMF-LMF= 1-10ms |
| Step 7 MAC Activate UE SRS transmission | 1-3.5 | TUE-gNB +TUEProc-MAC-SRSAct Processing delays: 13ms - UE: TUEProc-MAC-SRSAct = 1-3ms Signalling delay:0-0.5ms - UE-gNB: TUE-gNB= 0-0.5ms |
| Step 8 NRPPa Request UE SRS activate Response | 13-29 | TLMFProc+ TAMF-LMF+ TAMFProc +TgNB-AMF + TgNBProc-NRPPa Processing delays: 9 ms - gNB: TgNBProc-NRPPa= 3ms - AMF: TAMFProc= 3ms - LMF: TLMFProc= 3ms Signalling delay:4-20ms - gNB-AMF: TgNB-AMF= 3-10ms - AMF-LMF: TAMF-LMF= 1-10ms |
| Step 9 NRPPa MEASUREMENT REQUEST | 13-29 | TLMFProc+ TAMF-LMF+ TAMFProc +TgNB-AMF + TgNBProc-NRPPa Processing delays: 9 ms - gNB: TgNBProc-NRPPa= 3ms - AMF: TAMFProc= 3ms - LMF: TLMFProc= 3ms Signalling delay:4-20ms - gNB-AMF: TgNB-AMF= 3-10ms - AMF-LMF: TAMF-LMF= 1-10ms Note 2: Step 9 (NRPPa Measurement Request) can be performed in parallel with Steps 10/11 (LPP signalling). Hence, only the bigger number of the two procedures are considered (i.e., the latency for NRPPa Measurement Request is counted in the summation). |
| Step 10 LPP Provide Assistance Data | 28-44.5 | TLMFProc+ TAMF-LMF+ TAMFProc +TgNB-AMF + TgNBProc-NAS/LPP +TUE-gNB+ TUEProc-RRCDLInfo+ TUEProc-LPPAssi Processing delays: 24ms - UE: TUEProc-RRCDLInfo= 5ms TUEProc-LPPAssi= 10ms - gNB: TgNBProc-NAS/LPP= 3ms - AMF: TAMFProc= 3ms - LMF: TLMFProc= 3ms Signalling delay:4-20.5ms - UE-gNB: TUE-gNB= 0-0.5ms - gNB-AMF: TgNB-AMF= 3-10ms - AMF-LMF: TAMF-LMF= 1-10ms |
| Step 11 LPP Request Location Information | 23-39.5 | TLMFProc+ TAMF-LMF+ TAMFProc +TgNB-AMF + TgNBProc-NAS/LPP +TUE-gNB+ TUEProc-RRCDLInfo+ TUEProc-LPPLocationRe Processing delays: 19ms - UE: TUEProc-RRCDLInfo= 5ms TUEProc-LPPLocationRe = 5ms - gNB: TgNBProc-NAS/LPP= 3ms - AMF: TAMFProc= 3ms - LMF: TLMFProc= 3ms Signalling delay:4-20.5ms - UE-gNB: TUE-gNB= 0-0.5ms - gNB-AMF: TgNB-AMF= 3-10ms - AMF-LMF: TAMF-LMF= 1-10ms |
| Step 12 RRC Location Measurement Indication | 5-8.5 | TUEProc-RRCLocationMeas + TUE-gNB+ TgNBProc-RRC Processing delays: 5-8ms - UE: TUEProc-RRCLocationMeas = 2-5ms - gNB: TgNBProc-RRC= 3ms Signalling delay:0-0.5ms - UE-gNB: TUE-gNB= 0-0.5ms |
| Step 13 RRC Measurement Gap configuration | 13-13.5 | TgNBProc-RRC+ TUE-gNB+ TUEProc-RRCReconf Processing delays: 13ms - UE: TUEProc-RRCReconf = 10ms - gNB: TgNBProc-RRC= 3ms Signalling delay:0-0.5ms - UE-gNB: TUE-gNB= 0-0.5ms |
| Step 14 a DL PRS measurement | TDL-measc | RAN1 inputs |
| Step 14 b UL SRS measurement | TUL-measc | RAN1 inputs |
| Step 15 LPP Provide Location Information | 20-39.5 | TLMFProc+ TAMF-LMF+ TAMFProc +TgNB-AMF + TgNBProc-NAS/LPP +TUE-gNB+ TUEProc-RRCULInfo+ TUEProc-LPPLocationRe Processing delays: 16-19ms - UE: TUEProc-RRCULInfo= 2-5ms TUEProc-LPPLocationRe= 5ms - gNB: TgNBProc-NAS/LPP= 3ms - AMF: TAMFProc= 3ms - LMF: TLMFProc= 3ms Signalling delay:4-20.5ms - UE-gNB: TUE-gNB= 0-0.5ms - gNB-AMF: TgNB-AMF= 3-10ms - AMF-LMF: TAMF-LMF= 1-10ms |
| Step 16 NRPPa MEASUREMENT RESPONSE | 13-29 | TLMFProc+ TAMF-LMF+ TAMFProc +TgNB-AMF + TgNBProc-NRPPa Processing delays: 9 ms - gNB: TgNBProc-NRPPa= 3ms - AMF: TAMFProc= 3ms - LMF: TLMFProc= 3ms Signalling delay:4-20ms - gNB-AMF: TgNB-AMF= 3-10ms - AMF-LMF: TAMF-LMF= 1-10ms Note 3: Step 16 (NRPPa Measurement Response) can be performed in parallel with Step 15 (LPP Provide Location Information). The UL- and DL- measurements are made concurrently, hence the results are send at about the same time. Only the bigger number of the two procedures need to be considered (i.e., the latency for NRPPa Measurement Response is not counted in the summation). |
| Step 17 LMF calculation | 2-30 | TLMF-Calc LMF calculation/estimation delay: 2-30 TLMF-Calc |
| Total values | 220.5-397.5 | Note 4: TDL-measc and TUL-measc are not counted; Note 5: DL PRS related procedure may be performed in parallel with UL SRS related procedure, and therefore we may only need to count the latency caused by DL PRS related procedure, i.e. the latency for Multi-RTT could be similar to the latency of DL-TDOA/DL-AoD |

Call flow and latency analysis for NR E-CID

The figure 4-1 is used for latency analysis for Downlink NR E-CID

Figure 4-1 procedure for Downlink NR E-CID

Table 5 summarizes the latency for UE assisted Downlink NR E-CID.

Table 5: 	Latency for UE assisted Downlink NR E-CID

| Positioning technique [Downlink NR E-CID] [UE-A] Figure 4-1 |  |  |
| --- | --- | --- |
| Latency Component | Value Range (ms) | Description of Latency Component |
| Step 1 LPP Request capabilities | 18-34.5 | TLMFProc+ TAMF-LMF+ TAMFProc +TgNB-AMF + TgNBProc-NAS/LPP +TUE-gNB+ TUEProc-RRCDLInfo Processing delays: 14ms - UE: TUEProc-RRCDLInfo= 5ms - gNB: TgNBProc-NAS/LPP= 3ms - AMF: TAMFProc= 3ms - LMF: TLMFProc= 3ms Signalling delay:4-20.5ms - UE-gNB: TUE-gNB= 0-0.5ms - gNB-AMF: TgNB-AMF= 3-10ms - AMF-LMF: TAMF-LMF= 1-10ms Note: the LPP capability processing delay is counted together in response message. |
| Step 2 LPP Provide Capabilities | 25-54.5 | TLMFProc+ TAMF-LMF+ TAMFProc +TgNB-AMF + TgNBProc-NAS/LPP +TUE-gNB+ TUEProc-RRCULInfo+ TUEProc-LPPCapab Processing delays: 21-34ms - UE: TUEProc-RRCULInfo= 2-5ms TUEProc-LPPCapab= 10-20ms - gNB: TgNBProc-NAS/LPP= 3ms - AMF: TAMFProc= 3ms - LMF: TLMFProc= 3ms Signalling delay:4-20.5ms - UE-gNB: TUE-gNB= 0-0.5ms - gNB-AMF: TgNB-AMF= 3-10ms - AMF-LMF: TAMF-LMF= 1-10ms |
| Step 3 LPP Request Location Information | 23-39.5 | TLMFProc+ TAMF-LMF+ TAMFProc +TgNB-AMF + TgNBProc-NAS/LPP +TUE-gNB+ TUEProc-RRCDLInfo+ TUEProc-LPPLocationRe Processing delays: 19ms - UE: TUEProc-RRCDLInfo= 5ms TUEProc-LPPLocationRe = 5ms - gNB: TgNBProc-NAS/LPP= 3ms - AMF: TAMFProc= 3ms - LMF: TLMFProc= 3ms Signalling delay:4-20.5ms - UE-gNB: TUE-gNB= 0-0.5ms - gNB-AMF: TgNB-AMF= 3-10ms - AMF-LMF: TAMF-LMF= 1-10ms |
| Step 4 UE measurement |  | FFS whether should be counted or not. |
| Step 5 LPP Provide Location Information | 20-39.5 | TLMFProc+ TAMF-LMF+ TAMFProc +TgNB-AMF + TgNBProc-NAS/LPP +TUE-gNB+ TUEProc-RRCULInfo+ TUEProc-LPPLocationRe Processing delays: 16-19ms - UE: TUEProc-RRCULInfo= 2-5ms TUEProc-LPPLocationRe= 5ms - gNB: TgNBProc-NAS/LPP= 3ms - AMF: TAMFProc= 3ms - LMF: TLMFProc= 3ms Signalling delay:4-20.5ms - UE-gNB: TUE-gNB= 0-0.5ms - gNB-AMF: TgNB-AMF= 3-10ms - AMF-LMF: TAMF-LMF= 1-10ms |
| Step 6 LMF calculation | 2-30 | TLMF-Calc LMF calculation/estimation delay: 2-30 TLMF-Calc |
| Total values | 106.5-198 | Note 2: TDL-measc is not counted; |

The figure 4-2 is used for latency analysis for Uplink NR E-CID

Figure 4-2 procedure for Uplink NR E-CID

Table 6 summarizes the latency for UE assisted Uplink NR E-CID.

Table 6:	Latency for UE assisted Uplink NR E-CID

| Positioning technique [Uplink NR E-CID] [UE-A] Figure 4-2 |  |  |
| --- | --- | --- |
| Latency Component | Value Range (ms) | Description of Latency Component |
| Step 1 NRPPa E-CID Measurement Initiation Request | 13-29 | TLMFProc+ TAMF-LMF+ TAMFProc +TgNB-AMF + TgNBProc-NRPPa Processing delays: 9 ms - gNB: TgNBProc-NRPPa= 3ms - AMF: TAMFProc= 3ms - LMF: TLMFProc= 3ms Signalling delay:4-20ms - gNB-AMF: TgNB-AMF= 3-10ms - AMF-LMF: TAMF-LMF= 1-10ms |
| Step 2 RRC Measurement/SRS configuration | 13-13.5 | TgNBProc-RRC+ TUE-gNB+ TUEProc-RRCReconf Processing delays: 13ms - UE: TUEProc-RRCReconf = 10ms - gNB: TgNBProc-RRC= 3ms Signalling delay:0-0.5ms - UE-gNB: TUE-gNB= 0-0.5ms |
| Step 3 MAC Activate UE SRS transmission | 1-3.5 | TUE-gNB +TUEProc-MAC-SRSAct Processing delays: 13ms - UE: TUEProc-MAC-SRSAct = 1-3ms Signalling delay:0-0.5ms - UE-gNB: TUE-gNB= 0-0.5ms |
| Step 4 gNB measurements | TUL-measc | RAN1 inputs |
| Step 5 RRC Measurement report | 5-8.5 | TUEProc-RRCULInfo+ TUE-gNB+ TgNBProc-RRC Processing delays: 5-8ms - UE: TUEProc-RRCULInfo = 2-5ms - gNB: TgNBProc-RRC= 3ms Signalling delay:0-0.5ms - UE-gNB: TUE-gNB= 0-0.5ms |
| Step 6 NRPPa E-CID Measurement Initiation Response | 13-29 | TLMFProc+ TAMF-LMF+ TAMFProc +TgNB-AMF + TgNBProc-NRPPa Processing delays: 9 ms - gNB: TgNBProc-NRPPa= 3ms - AMF: TAMFProc= 3ms - LMF: TLMFProc= 3ms Signalling delay:4-20ms - gNB-AMF: TgNB-AMF= 3-10ms - AMF-LMF: TAMF-LMF= 1-10ms |
| Step 7 LMF calculation | 2-30 | TLMF-Calc LMF calculation/estimation delay: 2-30 TLMF-Calc |
| Total values | 47-113.5 | Note 1: TUL-measc is not counted; |

- Reference

[1] Chairman's Notes RAN1#102-e v022

[2] R1-2007264 LS on Latency of NR Positioning Protocols, RAN1

[3] R2-2006672	Discussion on ehancements for commercial use cases, CATT

[4] R2-2006578	Discussion on R17 positioning enhancement, Huawei, HiSilicon

[5] R2-2006750	Consideration on the support of low latency requirement, Intel Corporation

[6] R2-2007587	End-to-end latency reduction for DL/UL positioning, InterDigital, Inc.

[7] R2-2008261	[AT111-e][612][POS] Assumptions for analysis of commercial use cases (Ericsson)	Ericsson

[8] TS 23.273, 5G System (5GS) Location Services (LCS); Stage 2.

[9] TS37.910

[10]TS38.331

[11] R2-2009001Report of [Post111-e][625][POS] End-to-end latency analysis (Intel)	Intel Corporation

[12] R2-2010096	NR Positioning Latency Analysis and Enhancements	Qualcomm Incorporated