3GPP TSG-RAN WG2 Meeting #113-electronic	 R2-2100138

Online, January 25th – February 5th, 2021

Agenda item:		6.1.3

Source:	vivo

Title:	Remaining Issues on PUSCH Skipping with UCI in Rel-16

Document for:		Discussion and Decision

1 Introduction

In the LS (i.e. R1-2007338) regarding dynamic UL skipping, RAN2 is kindly requested to update the Rel-16 specification TS 38.321 and resolve the capability signaling issue for Rel-16 dynamic UL skipping [1].

Further, in the LS (i.e. R1-2009772) regarding CG PUSCH skipping with UCI, RAN2 is kindly requested to update the Rel-16 specification TS 38.321 and introduce a new capability to differentiate the new UE behavior of CG PUSCH skipping with UCI [2].

In the previous RAN2 meeting, a discussion had been launched on the dynamic UL skipping from the perspective of capability and RRC parameter. The corresponding achieved agreements are list below:

[016] Ph1 General agreements: [016] RAN2 confirms that a new UE capability is introduced for Rel-16 dynamic UL skipping. [016] RAN2 assumes the field name of the new UE capability is skipUplinkTxDynamic-r16. [016] RAN2 assumes that the following is introduced for the field description of the new UE capability (this version is preliminary and the wording may be further updated): - Indicates whether the UE supports skipping UL transmission for a dynamic uplink grant indicated on PDCCH only if no data is available for transmission and no UCI to be multiplexed on the corresponding PUSCH of the uplink grant as specified in TS 38.321 [8]. [016] RAN2 assumes the Rel-16 dynamic UL skipping is per UE level. FFS whether it is mandatory. [016] RAN2 assumes the Rel-16 dynamic UL skipping is FDD/TDD differentiation. [016] RAN2 assumes the Rel-16 dynamic UL skipping is not FR1/FR2 differentiation. [016] The legacy capability bit (i.e. skipUplinkTxDynamic) is not dummified. [016] A new RRC parameter is introduced to enable Rel-16 dynamic UL skipping. FFS the field name. [016] The corresponding 38.321/331/306 CR and reply LS for Rel-16 dynamic UL skipping should be done along with the CG case.

In this contribution, we would like to discuss the remaining issues for PUSCH skipping with overlapping PUCCH, taking both LCH-based intra-UE prioritization and PUSCH repetition/aggregation into account.

Hopefully, RAN2 can output agreeable 38.306/321/331 CRs related to the Rel-16 PUSCH skipping feature in the RAN2#113-e meeting. Otherwise, the corresponding RAN1 correction CR (i.e. R1-2009687) on PUSCH with UL skipping will not be approved in the RAN plenary again due to a lack of corresponding endorsed RAN2 CRs. What’s worse, not only the dynamic PUSCH skipping feature but also the whole Rel-16 URLLC Intra-UE prioritization feature will not be implemented according to the PHY specifications.

2 Discussion

  - Capability aspect of Rel-16 CG PUSCH skipping

In the previous RAN1#103e meeting, whether a UL transmission on the CG PUSCH with overlapping CSI/HARQ-ACK on PUCCH can be skipped in Rel-16 was warmly discussed. The conclusion made by RAN1 is given below [3].

Agreement: Case 1-2: For the case (Case 1-2) where only one or more CG PUSCHs overlapping with PUCCH - In Rel.16, for CA and non-CA case, when Rel-16 LCH based prioritization is not configured and there is a single PHY priority for UL transmissions, and when PUSCH repetition is not applied, in case of one or more CG PUSCHs overlapping with UCI and there is no DG PUSCH overlapping with the UCI and there is no DG PUSCH overlapping with the one or more CG PUSCHs, the CG PUSCH with UCI multiplexing from the one or more CG PUSCHs cannot be skipped. MAC generates MAC PDU for the CG PUSCH and delivers the MAC PDU to PHY and the UCI is multiplexed on the CG PUSCH.

From the achieved agreement, it can be concluded that the desired UE behavior of CG PUSCH skipping with overlapping PUCCH is that the MAC entity is expected to generate the MAC PDU for the CG PUSCH on which there is a UCI to be multiplexed, which is nearly the same or coincide with the UE behavior of Rel-16 dynamic UL skipping. Thus, we think it is natural to introduce a new UE capability and a new RRC parameter to differentiate/enable the new UE behavior as well.

Furthermore, according to the NR UE feature list [4], we can know that the feature of legacy skipping UL transmission for configured UL grant is a Layer-2 feature (i.e. per UE), mandatory without capability signaling, and in no need of “FDD-TDD DIFF” and “FR1-FR2 DIFF”. Considering that the new-defined CG PUSCH skipping feature is directly inherited from the Rel-15 feature, thus, we suggest the new-defined CG PUSCH skipping feature is per UE, mandatory with capability signaling, and in neither “FDD-TDD DIFF” nor “FR1-FR2 DIFF”.

Based on the above, we have the following proposals for the capability of Rel-16 CG PUSCH skipping,

Proposal 1: RAN2 confirms that a new UE capability is introduced for Rel-16 CG PUSCH skipping.

Proposal 2: The Rel-16 CG PUSCH skipping is per UE, mandatory with capability signaling, and in neither “FDD-TDD DIFF” nor “FR1-FR2 DIFF”.

Proposal 3: A new RRC parameter is introduced to enable Rel-16 CG PUSCH skipping.

  - Rel-16 PUSCH skipping with LCH-based prioritization

In Rel-16, LCH-based UE prioritization has been introduced for overlapping PUSCHs carrying UP data of LCHs of different priorities. The MAC entity only generates the MAC PDU for the prioritized uplink grant.  Based on this fact, it is worthy to discuss whether the Rel-16 DG/CG PUSCH skipping feature can cooperatively work with the LCH-based prioritization mechanism.

In Rel-16 with LCH-based UE prioritization, the main preparation steps for the generation of the MAC PDU to be transmitted on DG/CG PUSCH are given as follows:

- The MAC entity receives the UL grants including both dynamic grants and configured grants;

- The MAC entity delivers the UL grants and the associated HARQ information to the HARQ entity;

- The MAC entity determines whether a dynamic grant/configured grant is a prioritized uplink grant;

- The corresponding HARQ entity instructs the multiplexing and assembly entity for the prioritized uplink grant;

- Then the multiplexing and assembly entity performs the Logical Channel Prioritization (LCP), logical channel (LCH) selection, and resource allocation procedures (including determining whether the PUSCH skipping conditions are met or not);

- The corresponding HARQ entity obtains the MAC PDU to transmit from the multiplexing and assembly entity;

- The corresponding HARQ entity triggers a new transmission.

Figure 1. PUSCH overlapping with PUCCH scenario with LCH-based UE prioritization

Specifically, taking the scenario depicted in Figure 1 as an example, the MAC entity may firstly perform LCH-based UE prioritization procedure at the T1 moment, prioritizing the CG and deprioritizing the DG. As we known, the corresponding HARQ entity will not do anything for this deprioritized DG. In other words, no MAC PDU will be generated for this deprioritized DG even if its associated PUSCH is overlapping with PUCCH. This is because the corresponding HARQ entity will not instruct the multiplexing and assembly entity for the deprioritized DG. Consequently, the multiplexing and assembly entity will not be called to go to the step of checking the conditions of Rel-16 dynamic UL skipping for MAC PDU generation. From the PHY layer perspective, as no TB is delivered for the DG PUSCH, the PHY layer does not need to do UCI multiplexing or other PHY procedures, such as PHY intra-UE prioritization. Finally, the UE transmits the CG PUSCH and the PUCCH. At the NW side, once successfully decoded the CG PUSCH, it can know that the DG PUSCH is deprioritized and will detect the UCI on the PUCCH.

Proposal 4: RAN2 confirms that the MAC entity does not generate a MAC PDU for a deprioritized uplink grant (i.e. DG and CG) when its associated PUSCH is overlapping with PUCCH.

Considering that RAN1 is also discussing this issue, if Proposals 4 is agreeable, RAN2 sends an LS [5] to RAN1 informing this agreement for their reference.

  - Rel-16 PUSCH skipping with repetition/aggregation

Next, we will discuss the UE behavior of PUSCH skipping with overlapping PUCCH in the case where repetition/aggregation is configured. Two specific cases are given in the following two figures.

Figure 2: PUCCH overlapping with the first PUSCH transmission

Figure 3: PUCCH overlapping with the HARQ retransmission

For DG PUSCH transmission with repetition/aggregation, according to the following text quoted from TS 38.213, it can be concluded that the DL grant scheduling the PUCCH overlapping with the DG PUSCHs with repetition/aggregation needs to be received before the dynamic grant for DG PUSCH transmission with repetition/aggregation.

TS 38.321 section 9: A UE does not expect to detect a DCI format scheduling a PDSCH reception or a SPS PDSCH release, or a DCI format including a One-shot HARQ-ACK request field with value 1, and indicating a resource for a PUCCH transmission with corresponding HARQ-ACK information in a slot if the UE previously detects a DCI format scheduling a PUSCH transmission in the slot and if the UE multiplexes HARQ-ACK information in the PUSCH transmission.

In other words, the UE can definitely know whether there is a UCI to be multiplexed on any PUSCH transmission within the bundle upon the reception of the DG. For example, in the following Figure 4, at the moment T2, where the MAC entity determines whether dynamic UL skipping, should be performed, the UE has received (at moment T0) other DCI triggering UCI reporting according to the PHY timeline restriction. Since the timeline permits, the UE can generate the MAC PDU for the DG PUSCH transmission with repetition/aggregation and perform UCI multiplexing procedure even though no data is available for transmission. Thus, Rel-16 dynamic UL skipping feature can be adopted in this case without additional spec modification specific to repetition/aggregation.

Besides, considering that no new UE behaviour is defined for the DG PUSCH skipping feature in the case of DG PUSCH transmission with repetition/aggregation, we don’t think another new capability is needed.

Figure 4: PUCCH overlapping with the HARQ retransmission in DG PUSCH repetition/aggregation

Observation 1: The DL grant scheduling the PUCCH overlapping with the DG PUSCHs needs to be received before the DG for PUSCH transmission with repetition/aggregation.

Observation 2:  For DG PUSCH transmission with repetition/aggregation, UE can determine whether there is PUCCH carrying UCI overlapping with any PUSCHs within the bundle upon the reception of the DG.

Proposal 5: For DG PUSCH transmission with repetition/aggregation, when there is PUCCH carrying UCI overlapping with any PUSCHs within the bundle, the MAC entity generates a MAC PDU for the PUSCH on which the UCI is multiplexed.

For CG PUSCH transmission with repetition, the situation becomes a bit complicated. This is because there is no timeline restriction for the DL grant scheduling the PUCCH overlapping with the CG PUSCHs with repetition. As a result, a DL grant scheduling the PUCCH overlapping with a PUSCH for HARQ retransmission within the bundle might be received during the preparation of CG PUSCH transmission with reception, as depicted in the following figure 5.

Figure 5: PUCCH overlapping with the HARQ retransmission in CG PUSCH repetition

In such case, the UE would not generate the MAC PDU for CG PUSCHs with reception when there is no available data because the UE does not know there would be UCI to be multiplexed on the CG PUSCH for HARQ retransmission. Finally, the UE only transmits the UCI on PUCCH. At the NW side, double decoding between CG PUSCH and PUCCH is needed, which is not an intended UE behaviour.

To get rid of the potential increase on both UE and NW complexity, we think the NW should not schedule a PUCCH overlapping with a CG PUSCH for HARQ retransmission within the bundle. With this, taking the following Figure 6 as an example, the UE can definitely know whether there is a UCI to be multiplexed on the initial transmission occasion within the bundle upon the preparation of the CG PUSCH transmission with repetition (at T1 moment). Even there is no available data, the MAC entity can still generate the MAC PDU for UCI multiplexing, helping the NW completely avoid blind decoding.

Furthermore, with making some limitation on NW scheduling, there is no need to define any new UE behavior for the CG PUSCH skipping feature in the case of CG PUSCH transmission with repetition. Consequently, we don’t think another new capability is needed.

Figure 6: PUCCH overlapping with the initial retransmission in CG PUSCH repetition

Proposal 6: UE does not expect to be scheduled a PUCCH overlapping with a CG PUSCH transmission for HARQ retransmission within the bundle.

Proposal 7: For CG PUSCH transmission with repetition, when there is PUCCH carrying UCI overlapping with a PUSCH for initial transmission within the bundle, the MAC entity generates a MAC PDU for the PUSCH on which the UCI is multiplexed.

If Proposals 5/6/7 are agreeable from RAN2 perspective, send an LS [5] to RAN1 checking whether these implementations are feasible and acceptable.

Based on the above discussion, we, at last, have prepared the related text proposals for 38.306, 38.321, and 38.331 in Annex A, respectively.

Proposal 8: Adopt the text proposals in Annex A.

3 Conclusion

In this contribution, we have provided our understanding on the remaining issues related to LCH-based intra-UE prioritization and PUSCH repetition/aggregation for PUSCH skipping with UCI in Rel-16. And our observation and proposals are given as follows,

Observation 1: The DL grant scheduling the PUCCH overlapping with the DG PUSCHs needs to be received before the DG for PUSCH transmission with repetition/aggregation.

Observation 2:  For DG PUSCH transmission with repetition/aggregation, UE can determine whether there is PUCCH carrying UCI overlapping with any PUSCHs within the bundle upon the reception of the DG.

Proposal 1: RAN2 confirms that a new UE capability is introduced for Rel-16 CG PUSCH skipping.

Proposal 2: The Rel-16 CG PUSCH skipping is per UE, mandatory with capability signaling, and in neither “FDD-TDD DIFF” nor “FR1-FR2 DIFF”.

Proposal 3: A new RRC parameter is introduced to enable Rel-16 CG PUSCH skipping.

Proposal 4: RAN2 confirms that the MAC entity does not generate a MAC PDU for a deprioritized uplink grant (i.e. DG and CG) when its associated PUSCH is overlapping with PUCCH.

Proposal 5: For DG PUSCH transmission with repetition/aggregation, when there is PUCCH carrying UCI overlapping with any PUSCHs within the bundle, the MAC entity generates a MAC PDU for the PUSCH on which the UCI is multiplexed.

Proposal 6: UE does not expect to be scheduled a PUCCH overlapping with a CG PUSCH transmission for HARQ retransmission within the bundle.

Proposal 7: For CG PUSCH transmission with repetition, when there is PUCCH carrying UCI overlapping with a PUSCH for initial transmission within the bundle, the MAC entity generates a MAC PDU for the PUSCH on which the UCI is multiplexed.

Proposal 8: Adopt the text proposals in Annex A.

4 References

[1] R1-2007338. LS on PUSCH with UL skipping. vivo.

[2] R1-2009772. LS on PUSCH skipping with UCI in Rel-16. vivo.

[3] Chairman’s Notes, RAN1 #103-e.

[4] 3GPP TS 38.822, “User Equipment (UE) feature list”, V15.0.1.

[5] R2-2010565. Draft reply LS on PUSCH skipping with UCI in Rel-16. vivo.

Annex A

Text proposal for Rel-16 TS 38.321 ver 16.3.0:

START OF the CHANGE

5.4.3.1.3	Allocation of resources

Before the successful completion of the Random Access procedure initiated for DAPS handover, the target MAC entity shall not select the logical channel(s) corresponding to non-DAPS DRB(s) for the uplink grant received in a Random Access Response or the uplink grant for the transmission of the MSGA payload.

The MAC entity shall, when a new transmission is performed:

1>	allocate resources to the logical channels as follows:

2>	logical channels selected in clause 5.4.3.1.2 for the UL grant with Bj > 0 are allocated resources in a decreasing priority order. If the PBR of a logical channel is set to infinity, the MAC entity shall allocate resources for all the data that is available for transmission on the logical channel before meeting the PBR of the lower priority logical channel(s);

2>	decrement Bj by the total size of MAC SDUs served to logical channel j above;

2>	if any resources remain, all the logical channels selected in clause 5.4.3.1.2 are served in a strict decreasing priority order (regardless of the value of Bj) until either the data for that logical channel or the UL grant is exhausted, whichever comes first. Logical channels configured with equal priority should be served equally.

NOTE 1:	The value of Bj can be negative.

If the MAC entity is requested to simultaneously transmit multiple MAC PDUs, or if the MAC entity receives the multiple UL grants within one or more coinciding PDCCH occasions (i.e. on different Serving Cells), it is up to UE implementation in which order the grants are processed.

The UE shall also follow the rules below during the scheduling procedures above:

-	the UE should not segment an RLC SDU (or partially transmitted SDU or retransmitted RLC PDU) if the whole SDU (or partially transmitted SDU or retransmitted RLC PDU) fits into the remaining resources of the associated MAC entity;

-	if the UE segments an RLC SDU from the logical channel, it shall maximize the size of the segment to fill the grant of the associated MAC entity as much as possible;

-	the UE should maximise the transmission of data;

-	if the MAC entity is given a UL grant size that is equal to or larger than 8 bytes while having data available and allowed (according to clause 5.4.3.1) for transmission, the MAC entity shall not transmit only padding BSR and/or padding.

The MAC entity shall:

1>	if the MAC entity is configured with skipUplinkTxDynamicGrant with value true and the grant indicated to the HARQ entity was addressed to a C-RNTI, or the MAC entity is configured with skipUplinkTxConfiguredGrant with value true and the grant indicated to the HARQ entity is a configured uplink grant; and

1>	if there is no aperiodic CSI requested for this PUSCH transmission as specified in TS 38.212 [9]; and

1>	if the MAC PDU includes zero MAC SDUs; and

1>	if the MAC PDU includes only the periodic BSR and there is no data available for any LCG, or the MAC PDU includes only the padding BSR: and

1>	if there is no UCI to be multiplexed on the corresponding PUSCH transmission (including repetitions, if any, as specified in TS 38.214 [7]) as specified in TS 38.213 [6]:

2> not generate a MAC PDU for the HARQ entity.

1> 	elseif the MAC entity is configured with skipUplinkTxDynamic with value true and the grant indicated to the HARQ entity was addressed to a C-RNTI, or the grant indicated to the HARQ entity is a configured uplink grant; and

1> 	if there is no aperiodic CSI requested for this PUSCH transmission as specified in TS 38.212 [9]; and

1> 	if the MAC PDU includes zero MAC SDUs; and

1> 	if the MAC PDU includes only the periodic BSR and there is no data available for any LCG, or the MAC PDU includes only the padding BSR.

2> not generate a MAC PDU for the HARQ entity.

End OF the CHANGE

Text proposal for Rel-16 TS 38.306 ver 16.3.0:

START OF the CHANGE

4.2.6	MAC parameters

| Definitions for parameters | Per | M | FDD-TDD DIFF | FR1-FR2 DIFF |
| --- | --- | --- | --- | --- |
| autonomousTransmission-r16 Indicates whether the UE supports autonomous transmission of the MAC PDU generated for a deprioritized configured uplink grant as specified in TS 38.321 [8]. A UE supporting this feature shall also support lch-priorityBasedPrioritization-r16. | UE | No | No | No |
| directMCG-SCellActivation-r16 Indicates whether the UE supports direct NR MCG SCell activation, as specified in TS 38.321 [8], upon SCell addition, upon reconfiguration with sync of the MCG, as specified in TS 38.331 [9]. | UE | No | No | Yes |
| directMCG-SCellActivationResume-r16 Indicates whether the UE supports direct NR MCG SCell activation, as specified in TS 38.321 [8], upon reception of an RRCResume message, as specified in TS 38.331 [9]. | UE | No | No | Yes |
| directSCG-SCellActivation-r16 Indicates whether the UE supports direct NR SCG SCell activation, as specified in TS 38.321 [8], upon SCell addition and upon reconfiguration with sync of the SCG, both performed via an RRCReconfiguration message received via SRB3 or contained in an RRC(Connection)Reconfiguration message received via SRB1, as specified in TS 38.331 [9] and TS 36.331 [17]. A UE indicating support of directSCG-SCellActivation-r16 shall indicate support of EN-DC or support of NGEN-DC as specified in TS 36.331 [17] or support of nr-dc as specified in TS 38.331 [9]. | UE | No | No | Yes |
| directSCG-SCellActivationResume-r16 Indicates whether the UE supports direct NR SCG SCell activation, as specified in TS 38.321 [8]: - upon reception of an RRCReconfiguration included in an RRCConnectionResume message, as specified in TS 38.331 [9] and TS 36.331 [17], if the UE indicates support of en-dc and of resumeWithSCG-Config-r16 as specified in TS 36.331 [17], - upon reception of an RRCReconfiguration included in an RRCResume message, as specified in TS 38.331 [9], if the UE indicates support of nr-dc and of resumeWithSCG-Config-r16 as specified in TS 38.331 [9]. A UE indicating support of directSCG-SCellActivationResume-r16 shall indicate support of EN-DC or NGEN-DC and support of resumeWithSCG-Config-r16 as specified in TS 36.331 [17] or indicate support of nr-dc and of resumeWithSCG-Config-r16 as specified in TS 38.331 [9]. | UE | No | No | Yes |
| drx-Adaptation-r16 Indicates whether the UE supports DRX adaptation comprised of the following functional components: - Configured ps-Offset for the detection of DCI format 2_6 with CRC scrambling by ps-RNTI and reported MinTimeGap before the start of drx-onDurationTimer of Long DRX - Indication of UE whether or not to start drx-onDurationTimer for the next Long DRX cycle by detection of DCI format 2_6 - Configured UE wakeup or not when DCI format 2_6 is not detected at all monitoring occasions outside Active Time - Configured periodic CSI report apart from L1-RSRP (ps-TransmitOtherPeriodicCSI) when impacted by DCI format 2_6 that drx-onDurationTimer does not start for the next Long DRX cycle - Configured periodic L1-RSRP report (ps-TransmitPeriodicL1-RSRP) when impacted by DCI format 2_6 that drx-onDurationTimer does not start for the next Long DRX cycle The capability signalling includes the minimum time gap between the end of the slot of last DCI format 2_6 monitoring occasion and the beginning of the slot where the UE would start the drx-onDurationTimer of Long DRX for each SCS. The value sl1 indicates 1 slot. The value sl2 indicates 2 slots, and so on. Support of this feature is reported for licensed and unlicensed bands, respectively. When this field is reported, either of sharedSpectrumChAccess-r16 or non-SharedSpectrumChAccess-r16 shall be reported, at least. | UE | No | No | Yes |
| lch-PriorityBasedPrioritization-r16 Indicates whether the UE supports prioritization between overlapping grants and between scheduling request and overlapping grants based on LCH priority as specified in TS 38.321 [8]. | UE | No | No | No |
| lch-ToConfiguredGrantMapping-r16 Indicates whether the UE supports restricting data transmission from a given LCH to a configured (sub-) set of configured grant configurations (see allowedCG-List-r16 in LogicalChannelConfig in TS 38.331 [9]) as specified in TS 38.321 [8]. | UE | No | No | No |
| lch-ToGrantPriorityRestriction-r16 Indicates whether the UE supports restricting data transmission from a given LCH to a configured (sub-) set of dynamic grant priority levels (see allowedPHY-PriorityIndex-r16 in LogicalChannelConfig in TS 38.331 [9]) as specified in TS 38.321 [8]. | UE | No | No | No |
| lch-ToSCellRestriction Indicates whether the UE supports restricting data transmission from a given LCH to a configured (sub-) set of serving cells (see allowedServingCells in LogicalChannelConfig). A UE supporting pdcp-DuplicationMCG-OrSCG-DRB or pdcp-DuplicationSRB (see PDCP-Config) shall also support lch-ToSCellRestriction. | UE | No | No | No |
| lcp-Restriction Indicates whether UE supports the selection of logical channels for each UL grant based on RRC configured restriction. | UE | No | No | No |
| logicalChannelSR-DelayTimer Indicates whether the UE supports the logicalChannelSR-DelayTimer as specified in TS 38.321 [8]. | UE | No | Yes | No |
| longDRX-Cycle Indicates whether UE supports long DRX cycle as specified in TS 38.321 [8]. | UE | Yes | Yes | No |
| multipleConfiguredGrants Indicates whether UE supports more than one configured grant configurations (including both Type 1 and Type 2) in a cell group. For each cell, the UE supports at most one configured grant per BWP and the maximum number of configured grant configurations per cell group is 2. If absent, for each configured cell group, the UE only supports one configured grant configuration on one serving cell. | UE | No | Yes | No |
| multipleSR-Configurations Indicates whether the UE supports 8 SR configurations per PUCCH cell group as specified in TS 38.321 [8]. | UE | No | Yes | No |
| recommendedBitRate Indicates whether the UE supports the bit rate recommendation message from the gNB to the UE as specified in TS 38.321 [8]. | UE | No | No | No |
| recommendedBitRateMultiplier-r16 Indicates whether the UE supports the bit rate multiplier for recommended bit rate MAC CE as specified in TS 38.321 [8], clause 6.1.3.20. This field is only applicable if the UE supports recommendedBitRate. | UE | No | No | No |
| recommendedBitRateQuery Indicates whether the UE supports the bit rate recommendation query message from the UE to the gNB as specified in TS 38.321 [8]. This field is only applicable if the UE supports recommendedBitRate. | UE | No | No | No |
| secondaryDRX-Group-r16 Indicates whether UE supports secondary DRX group as specified in TS 38.321 [8]. | UE | No | Yes | No |
| shortDRX-Cycle Indicates whether UE supports short DRX cycle as specified in TS 38.321 [8]. | UE | Yes | Yes | No |
| singlePHR-P-r16 Indicates whether UE supports the P bit in single PHR MAC CE as specified in TS 38.321 [8]. | UE | No | No | No |
| skipUplinkTxDynamic Indicates whether the UE supports skipping of UL transmission for an uplink grant indicated on PDCCH if no data is available for transmission as specified in TS 38.321 [8]. | UE | No | Yes | No |
| skipUplinkTxDynamic-r16 Indicates whether the UE supports skipping UL transmission for a dynamic uplink grant indicated on PDCCH only if no data is available for transmission and no UCI to be multiplexed on the corresponding PUSCH of the uplink grant as specified in TS 38.321 [8] | UE | Yes | Yes | No |
| skipUplinkTxConfigured-r16 Indicates whether the UE supports skipping UL transmission for a configured uplink grant only if no data is available for transmission and no UCI to be multiplexed on the corresponding PUSCH of the uplink grant as specified in TS 38.321 [8] | UE | Yes | No | No |
| tdd-MPE-P-MPR-Reporting-r16 Indicates whether the UE supports P-MPR reporting for Maximum Permissible Exposure, as specified in TS38.321 [8]. | UE | No | TDD only | FR2 only |
| ul-LBT-FailureDetectionRecovery-r16 Indicates whether the UE supports consistent uplink LBT detection and recovery, as specified in TS 38.321 [8], for cells operating with shared spectrum channel access [8]. This field applies to all serving cells with which the UE is configured with shared spectrum channel access. | UE | No | No | No |

End OF the CHANGE

Text proposal for Rel-16 TS 38.331 ver 16.3.1:

START OF the CHANGE

6.3.2	Radio resource control information elements

MAC-CellGroupConfig

The IE MAC-CellGroupConfig is used to configure MAC parameters for a cell group, including DRX.

MAC-CellGroupConfig information element

-- ASN1START

-- TAG-MAC-CELLGROUPCONFIG-START

MAC-CellGroupConfig ::=             SEQUENCE {

drx-Config                          SetupRelease { DRX-Config }                                     OPTIONAL,   -- Need M

schedulingRequestConfig             SchedulingRequestConfig                                         OPTIONAL,   -- Need M

bsr-Config                          BSR-Config                                                      OPTIONAL,   -- Need M

tag-Config                          TAG-Config                                                      OPTIONAL,   -- Need M

phr-Config                          SetupRelease { PHR-Config }                                     OPTIONAL,   -- Need M

skipUplinkTxDynamic                 BOOLEAN,

...,

[[

csi-Mask                            BOOLEAN                                                         OPTIONAL,   -- Need M

dataInactivityTimer                 SetupRelease { DataInactivityTimer }                            OPTIONAL    -- Cond MCG-Only

]],

[[

usePreBSR-r16                       ENUMERATED {true}                                               OPTIONAL,   -- Need R

schedulingRequestID-LBT-SCell-r16   SchedulingRequestId                                             OPTIONAL,   -- Need R

lch-BasedPrioritization-r16         ENUMERATED {enabled}                                            OPTIONAL,   -- Need R

schedulingRequestID-BFR-SCell-r16   SchedulingRequestId                                             OPTIONAL,   -- Need R

drx-ConfigSecondaryGroup-r16        SetupRelease { DRX-ConfigSecondaryGroup }                       OPTIONAL    -- Need M

]],

[[

skipUplinkTxDynamicGrant-r16        BOOLEAN,

skipUplinkTxConfiguredGrant-r16     BOOLEAN

]]

}

DataInactivityTimer ::=         ENUMERATED {s1, s2, s3, s5, s7, s10, s15, s20, s40, s50, s60, s80, s100, s120, s150, s180}

-- TAG-MAC-CELLGROUPCONFIG-STOP

-- ASN1STOP

MAC-CellGroupConfig field descriptions

usePreBSR If set to true, the MAC entity of the IAB-MT may use the Pre-emptive BSR.

csi-Mask If set to true, the UE limits CSI reports to the on-duration period of the DRX cycle, see TS 38.321 [3].

dataInactivityTimer Releases the RRC connection upon data inactivity as specified in clause 5.3.8.5 and in TS 38.321 [3]. Value s1 corresponds to 1 second, value s2 corresponds to 2 seconds, and so on.

drx-Config Used to configure DRX as specified in TS 38.321 [3].

drx-ConfigSecondaryGroup Used to configure DRX related parameters for the second DRX group as specified in TS 38.321 [3]. The network does not configure secondary DRX group with DCP simultaneously nor secondary DRX group with a dormant BWP simultaneously.

lch-BasedPrioritization If this field is present, the corresponding MAC entity of the UE is configured with prioritization between overlapping grants and between scheduling request and overlapping grants based on LCH priority, see TS 38.321 [3].

schedulingRequestID-BFR-SCell Indicates the scheduling request configuration applicable for BFR on SCell, as specified in TS 38.321 [3].

schedulingRequestID-LBT-SCell Indicates the scheduling request configuration applicable for consistent uplink LBT recovery on SCell, as specified in TS 38.321 [3].

skipUplinkTxDynamic If set to true, the UE skips UL transmissions as described in TS 38.321 [3].

skipUplinkTxDynamicGrant If set to true, the UE skips UL transmissions for dynamic uplink grant as described in TS 38.321 [3].

skipUplinkTxConfiguredGrant If set to true, the UE skips UL transmissions for configured uplink grant as described in TS 38.321 [3].

tag-Config The field is used to configure parameters for a time-alignment group. The field is not present if any DAPS bearer is configured.

END OF the CHANGE

START OF the CHANGE

6.3.3	UE capability information elements

–	MAC-Parameters

The IE MAC-Parameters is used to convey capabilities related to MAC.

MAC-Parameters information element

-- ASN1START

-- TAG-MAC-PARAMETERS-START

MAC-Parameters ::= SEQUENCE {

mac-ParametersCommon            MAC-ParametersCommon        OPTIONAL,

mac-ParametersXDD-Diff          MAC-ParametersXDD-Diff      OPTIONAL

}

MAC-Parameters-v1610 ::= SEQUENCE {

mac-ParametersFRX-Diff-r16      MAC-ParametersFRX-Diff-r16  OPTIONAL

}

MAC-ParametersCommon ::=    SEQUENCE {

lcp-Restriction                         ENUMERATED {supported}      OPTIONAL,

dummy                                   ENUMERATED {supported}      OPTIONAL,

lch-ToSCellRestriction                  ENUMERATED {supported}      OPTIONAL,

...,

[[

recommendedBitRate                      ENUMERATED {supported}      OPTIONAL,

recommendedBitRateQuery                 ENUMERATED {supported}      OPTIONAL

]],

[[

recommendedBitRateMultiplier-r16         ENUMERATED {supported}     OPTIONAL,

preEmptiveBSR-r16                        ENUMERATED {supported}     OPTIONAL,

autonomousTransmission-r16               ENUMERATED {supported}     OPTIONAL,

lch-PriorityBasedPrioritization-r16      ENUMERATED {supported}     OPTIONAL,

lch-ToConfiguredGrantMapping-r16         ENUMERATED {supported}     OPTIONAL,

lch-ToGrantPriorityRestriction-r16       ENUMERATED {supported}     OPTIONAL,

singlePHR-P-r16                          ENUMERATED {supported}     OPTIONAL,

ul-LBT-FailureDetectionRecovery-r16      ENUMERATED {supported}     OPTIONAL,

-- R4 8-1: MPE

tdd-MPE-P-MPR-Reporting-r16              ENUMERATED {supported}     OPTIONAL,

lcid-ExtensionIAB-r16                    ENUMERATED {supported}     OPTIONAL

]],

[[

skipUplinkTxConfigured-r16               ENUMERATED {supported}     OPTIONAL

]]

}

MAC-ParametersFRX-Diff-r16 ::=  SEQUENCE {

directMCG-SCellActivation-r16           ENUMERATED {supported}      OPTIONAL,

directMCG-SCellActivationResume-r16     ENUMERATED {supported}      OPTIONAL,

directSCG-SCellActivation-r16           ENUMERATED {supported}      OPTIONAL,

directSCG-SCellActivationResume-r16     ENUMERATED {supported}      OPTIONAL,

-- R1 19-1: DRX Adaptation

drx-Adaptation-r16          SEQUENCE {

non-SharedSpectrumChAccess-r16      MinTimeGap-r16              OPTIONAL,

sharedSpectrumChAccess-r16          MinTimeGap-r16              OPTIONAL

}                                                                   OPTIONAL,

...

}

MAC-ParametersXDD-Diff ::=  SEQUENCE {

skipUplinkTxDynamic                     ENUMERATED {supported}     OPTIONAL,

logicalChannelSR-DelayTimer             ENUMERATED {supported}     OPTIONAL,

longDRX-Cycle                           ENUMERATED {supported}     OPTIONAL,

shortDRX-Cycle                          ENUMERATED {supported}     OPTIONAL,

multipleSR-Configurations               ENUMERATED {supported}     OPTIONAL,

multipleConfiguredGrants                ENUMERATED {supported}     OPTIONAL,

...,

[[

secondaryDRX-Group-r16                  ENUMERATED {supported}     OPTIONAL

]],

[[

skipUplinkTxDynamic-r16                 ENUMERATED {supported}     OPTIONAL

]]

}

MinTimeGap-r16 ::=    SEQUENCE {

scs-15kHz-r16                         ENUMERATED {sl1, sl3}        OPTIONAL,

scs-30kHz-r16                         ENUMERATED {sl1, sl6}        OPTIONAL,

scs-60kHz-r16                         ENUMERATED {sl1, sl12}       OPTIONAL,

scs-120kHz-r16                        ENUMERATED {sl2, sl24}       OPTIONAL

}

-- TAG-MAC-PARAMETERS-STOP

-- ASN1STOP

END OF the CHANGE