3GPP TSG-RAN WG2 Meeting #113-e	R2-21xxxxx

Online, 25th Jan - 5th Feb, 2021

# Agenda item:	5.3.1

# Source:	Huawei, HiSilicon

# Title:	Report of [AT113-e][003][NR15] User Plane II (Huawei)

# Document for:	Discussion and Agreement

1	Introduction

This is to report the result of the following email discussion in RAN2#113-e Meeting [1].

[AT113-e][003][NR15] User Plane II (Huawei)

Scope: MAC RLC PDCP Treat R2-2101344, R2-2101349, R2-2101773, R2-2101774, R2-2100317, R2-2100315, R2-2100316 R2-2101441, R2-2101442, R2-2101775

Phase 1, determine agreeable parts, Phase 2, for agreeable parts Work on CRs.

Intended outcome: Report and Agreed CRs.

Deadline: Schedule A

A first round with Deadline for comments Thursday Feb 28 1200 UTC to settle scope what is agreeable etc

A Final round with Final deadline Thursday Feb 4 1200 UTC. to settle details / agree CRs etc. Additional check points etc if needed are defined by the Rapporteur. In case some parts of an email discussion need more time, doesn’t converge, need on-line treatment etc Rapporteur please contact chair.

2	Contact Information

| Company | Contact: Name (E-mail) |
| --- | --- |
| Huawei, HiSilicon | Chong Lou (louchong@huawei.com) |
| OPPO | Shi Cong (shicong@oppo.com) |

3	Discussion

3.1	LCP restriction (Rel-15 and 16)

LCP restrictions

R2-2101344	Clarification to LCP restrictions	Ericsson, Mediatek	CR	Rel-15	38.306	15.12.0	0504	-	F	NR_newRAT-Core

R2-2101349	Clarification to LCP restrictions	Ericsson, Mediatek	CR	Rel-16	38.306	16.3.0	0505	-	A	NR_newRAT-Core

These CRs propose to add the clarification of “RRC configured restriction” for LCP in the field description of lcp-Restriction to both Rel-15 and Rel-16, as follows.

| lcp-Restriction Indicates whether UE supports the selection of logical channels for each UL grant based on RRC configured restriction using RRC parameters allowedSCS-List, maxPUSCH-Duration, and configuredGrantType1Allowed. | UE | No | No | No |
| --- | --- | --- | --- | --- |

| Company | Agree as is (from which release); Agree with changes; To capture it in the meeting minutes; Disagree | Detailed Comments |
| --- | --- | --- |
| HW | Disagree | We think the added clarification should have already been the common understanding. For other restrictions that are not indicated, e.g. allowedServingCells, allowedCG-List and allowedPHY-PriorityIndex, each restriction is already associated with a UE capability. Therefore, this lcp-Restriction is only applied to restrictions that are not explicitly indicated from UE. The current spec should already be clear enough, and if needed, this kind of clarification should be captured into the rapporteur CR as there is no functional change. |
| OPPO | Agree | We are ok on this clarification. |

Conclusion:

TBD

3.2	CSI reporting for DRX (Rel-15 and 16)

CSI reporting

R2-2101773	Correction on CSI reporting when CSI masking is setup	Huawei, HiSilicon	CR	Rel-15	38.321	15.11.0	1052	-	F	NR_newRAT-Core

R2-2101774	Correction on CSI reporting when CSI masking is setup	Huawei, HiSilicon	CR	Rel-16	38.321	16.3.0	1053	-	F	NR_newRAT-Core

These CRs propose to add a case that is motivated the the past discussions that the CSI multiplexed with other overlapping UCI maybe reported outside the “DRX Active Time” and it is up to UE implementation whether to report or not. Similar to the CSI mask case, where the p-CSI multiplexed with other overlapping UCI is outside the “On duration” and these CRs propose to align it with above behaviour.

| Company | Agree as is (which CR; from which release); Agree with changes; To capture it in the meeting minutes; Disagree | Detailed Comments |
| --- | --- | --- |
| HW | Agree as is (from Rel-15) | We confirm that the case mentioned in this CR is valid when CSI mask is setup and the UE behaviour should be aligned to other discussed cases. Since it proposes to leave it up to UE implementation, so the backward compatibility issue can be eliminated. |
| OPPO | Disagree | We think the current note may have already capture the case proposed by the CRs, in our minde, “outside DRX Active Time” also includes “outside onduration”. |

Conclusion:

TBD

3.3	MAC inactivity timers at empty scheduling (Rel-16 only)

MAC inactivity timers at empty scheduling

Moved from 6.1.3

R2-2100317	Configuration and capability signaling for not starting MAC timers	Qualcomm Incorporated		CR	Rel-16	38.331	16.3.0	2320	-	F	TEI16

R2-2100315	Correction to MAC timer procedures	Qualcomm Incorporated	CR	Rel-16	38.321	16.3.0	1013	-	F	TEI16

R2-2100316	UE capability for not starting MAC timers	Qualcomm Incorporated	CR	Rel-16	38.306	16.3.0	0484	-	F	TEI16

These CRs propose that UE UE does not re-/start drx-InactivityTimer, bwp-InactivityTimer and sCellDeactivationTimer if it skips a dynamic UL grant for new data or it transmits a MAC PDU without any MAC SDU in Rel-16.

1) please indicate your answer to the MAC CR (R2-2100315)

| Company | Agree as is; Agree with changes; To capture it in the meeting minutes; Disagree | Detailed Comments |
| --- | --- | --- |
| HW | Disagree | Not essential but significant complexity added to UE implementation. The UE has to check each time about the outcome of UL skipping. In addition, it also brings the riks of misalignment between UE and NW with respect to the “timer” status. And the interaction may impact the time point of taking effect of the corresponding timers, e.g. BWP inactivity timer, which should be consulted with RAN1 and RAN4. |
| OPPO | Disagree | We think the current behaviour is clear and the proposed change may bring extra implementation complexity for UE. |

2) If you answered “agree” in 1), please indicate your answer to the RRC CR (R2-2100317)

| Company | Agree as is; Agree with changes; To capture it in the meeting minutes; Disagree | Detailed Comments |
| --- | --- | --- |

3) If you answered “agree” in 1), please indicate your answer to the UE capability CR (R2-2100316)

| Company | Agree as is; Agree with changes; To capture it in the meeting minutes; Disagree | Detailed Comments |
| --- | --- | --- |

Conclusion:

TBD

3.4	Clarification to RLC PDU polling at HO (Rel-15 and16)

Text Enhancement

R2-2101441	Clarification to RLC PDU Polling at Handover	Ericsson	CR	Rel-16	38.322	16.2.0	0038	-	F	NR_newRAT-Core

R2-2101442	Clarification to RLC PDU Polling at Handover	Ericsson	CR	Rel-15	38.322	15.5.0	0039	-	F	NR_newRAT-Core

These CRs propose to reflect the RRC statement “the UE should perform the reconfiguration with sync as soon as possible following the reception of the RRC message triggering the reconfiguration with sync, which could be before confirming successful reception (HARQ and ARQ) of this message” in RLC as well.

| Company | Agree as is (from which release); Agree with changes; To capture it in the meeting minutes; Disagree | Detailed Comments |
| --- | --- | --- |
| HW | Disagree | First there is a CR in this meeting to revise RRC CR to clarify the same thing and we think both CRs are not needed. The exising RRC spec has already specified how to handle the RLC/HARQ feedback for RRC signalling, and the user plane handling depends on the indication of reestablishRLC.Anyway, this kind of clarification should be taken into account in RRC spec and we should not duplicate the text in RLC spec as normally RRC messages and procedures should be transparent to RLC. |
| OPPO | Disagree | If it’s already captured in the RRC, there is no need to further clarify in RLC. |

Conclusion:

TBD

3.5	RoHC handling during PDCP re-establishment (Rel-15 and 16)

R2-2101775	Discussion about RoHC handling during PDCP re-establishment	Huawei, HiSilicon	discussion	Rel-15	NR_newRAT-Core

During PDCP re-establishment, the UE may retransmit the PDCP data from the first missing SDU. However, there is a risk that the receiver side may discard the duplicated PDCP data carrying the new RoHC context. In this case, RoHC context state is misaligned between transmitter and receiver side. This paper would like to identify this issue and propose one solution similar to the DAPS that the transmitter should maintain IR state for retransmitted PDCP SDU during PDCP re-establishment.

1) please indicate your view on the issue identified in this paper (R2-2101775)

| Company | Agree with the issue; Disagree | Detailed Comments |
| --- | --- | --- |
| HW | Agree with the issue | We confirm the issue is valid when, for instance, drb-ContinueROHC is enabled. Without a specific solution in the spec, it is likely that the RoHC context could be misaligned between UE and NW during PDCP retransmissions, i.e. upon PDCP re-establishment. |
| OPPO | Agree with the issue |  |

2) If you answered “agree” in 1), please indicate your views on the proposed solution in the annext TP in this paper (R2-2101775)

| Company | Agree as is (from which release); Agree with changes; To capture it in the meeting minutes; Disagree | Detailed Comments |
| --- | --- | --- |
| HW | Agree as is, But open to other solutions | We understand that, the proposed solution is motivated by the DAPS RoHC handling that has been discussed in Rel-16. So we slightly prefer to extend the solution to the PDCP re-establishment case without introducing a brand new one. However, as long as the issue can be resolved, we are open to other suggested solutions either by NW side or UE side or both sides. |
| OPPO | The note is ok to us |  |

Conclusion:

TBD

4	Conclusion

TBD

5	References

[1]	RAN2 113-e Chairman Notes 2021-01-25 0900 UTC.docx