3GPP TSG-RAN WG2 Meeting #113-e	R2-21xxxxx

Online, 25th Jan - 5th Feb, 2021

# Agenda item:	5.3.1

# Source:	Huawei, HiSilicon

# Title:	Report of [AT113-e][003][NR15] User Plane II (Huawei)

# Document for:	Discussion and Agreement

# 1	Introduction

This is to report the result of the following email discussion in RAN2#113-e Meeting [1].

[AT113-e][003][NR15] User Plane II (Huawei)

Scope: MAC RLC PDCP Treat R2-2101344, R2-2101349, R2-2101773, R2-2101774, R2-2100317, R2-2100315, R2-2100316 R2-2101441, R2-2101442, R2-2101775

Phase 1, determine agreeable parts, Phase 2, for agreeable parts Work on CRs.

Intended outcome: Report and Agreed CRs.

Deadline: Schedule A

A first round with Deadline for comments Thursday Feb 28 1200 UTC to settle scope what is agreeable etc

A Final round with Final deadline Thursday Feb 4 1200 UTC. to settle details / agree CRs etc. Additional check points etc if needed are defined by the Rapporteur. In case some parts of an email discussion need more time, doesn’t converge, need on-line treatment etc Rapporteur please contact chair.

# 2	Contact Information

| Company | Contact: Name (E-mail) |
| --- | --- |
| Huawei, HiSilicon | Chong Lou (louchong@huawei.com) |
| OPPO | Shi Cong (shicong@oppo.com) |
| MediaTek | Guanyu Lin (guanyu.lin@mediatek.com) |
| Xiaomi | Yumin Wu (wuyumin@xiaomi.com) |

# 3	Discussion

## 3.1	LCP restriction (Rel-15 and 16)

LCP restrictions

R2-2101344	Clarification to LCP restrictions	Ericsson, Mediatek	CR	Rel-15	38.306	15.12.0	0504	-	F	NR_newRAT-Core

R2-2101349	Clarification to LCP restrictions	Ericsson, Mediatek	CR	Rel-16	38.306	16.3.0	0505	-	A	NR_newRAT-Core

These CRs propose to add the clarification of “RRC configured restriction” for LCP in the field description of lcp-Restriction to both Rel-15 and Rel-16, as follows.

| lcp-Restriction Indicates whether UE supports the selection of logical channels for each UL grant based on RRC configured restriction using RRC parameters allowedSCS-List, maxPUSCH-Duration, and configuredGrantType1Allowed. | UE | No | No | No |
| --- | --- | --- | --- | --- |

| Company | Agree as is (from which release); Agree with changes; To capture it in the meeting minutes; Disagree | Detailed Comments |
| --- | --- | --- |
| HW | Disagree | We think the added clarification should have already been the common understanding. For other restrictions that are not indicated, e.g. allowedServingCells, allowedCG-List and allowedPHY-PriorityIndex, each restriction is already associated with a UE capability. Therefore, this lcp-Restriction is only applied to restrictions that are not explicitly indicated from UE. The current spec should already be clear enough, and if needed, this kind of clarification should be captured into the rapporteur CR as there is no functional change. |
| OPPO | Agree | We are ok on this clarification. |
| Qualcomm | Agree with change | Some other LCP restriction parameters seem to be missing in the proposed text, e.g. allowed serving cells, CG list (R16), PHY-priority index (R16). Another concern is that it is not very future proof, i.e. whenever we add a new LCP restriction in the future release, we have to update this list again. We wonder if companies would consider replacing the TP with a reference to 38.321 instead. |
| MediaTek | Agree | We think the clarification is useful. Otherwise, people may misunderstood that lcp-Restriction cover all LCP parameters. |
| Xiaomi | Agree |  |

Conclusion:

TBD

## 3.2	CSI reporting for DRX (Rel-15 and 16)

CSI reporting

R2-2101773	Correction on CSI reporting when CSI masking is setup	Huawei, HiSilicon	CR	Rel-15	38.321	15.11.0	1052	-	F	NR_newRAT-Core

R2-2101774	Correction on CSI reporting when CSI masking is setup	Huawei, HiSilicon	CR	Rel-16	38.321	16.3.0	1053	-	F	NR_newRAT-Core

These CRs propose to add a case that is motivated the the past discussions that the CSI multiplexed with other overlapping UCI maybe reported outside the “DRX Active Time” and it is up to UE implementation whether to report or not. Similar to the CSI mask case, where the p-CSI multiplexed with other overlapping UCI is outside the “On duration” and these CRs propose to align it with above behaviour.

| Company | Agree as is (which CR; from which release); Agree with changes; To capture it in the meeting minutes; Disagree | Detailed Comments |
| --- | --- | --- |
| HW | Agree as is (from Rel-15) | We confirm that the case mentioned in this CR is valid when CSI mask is setup and the UE behaviour should be aligned to other discussed cases. Since it proposes to leave it up to UE implementation, so the backward compatibility issue can be eliminated. |
| OPPO | Disagree | We think the current note may have already capture the case proposed by the CRs, in our minde, “outside DRX Active Time” also includes “outside onduration”. |
| Qualcomm | Agree as is |  |
| MediaTek | Agree as is |  |
| Xiaomi | Disagree | If the onDurationTimer is not running, it is obvious that the UEis outside the DRX active time. |

Conclusion:

TBD

## 3.3	MAC inactivity timers at empty scheduling (Rel-16 only)

MAC inactivity timers at empty scheduling

Moved from 6.1.3

R2-2100317	Configuration and capability signaling for not starting MAC timers	Qualcomm Incorporated		CR	Rel-16	38.331	16.3.0	2320	-	F	TEI16

R2-2100315	Correction to MAC timer procedures	Qualcomm Incorporated	CR	Rel-16	38.321	16.3.0	1013	-	F	TEI16

R2-2100316	UE capability for not starting MAC timers	Qualcomm Incorporated	CR	Rel-16	38.306	16.3.0	0484	-	F	TEI16

These CRs propose that UE UE does not re-/start drx-InactivityTimer, bwp-InactivityTimer and sCellDeactivationTimer if it skips a dynamic UL grant for new data or it transmits a MAC PDU without any MAC SDU in Rel-16.

1) please indicate your answer to the MAC CR (R2-2100315)

| Company | Agree as is; Agree with changes; To capture it in the meeting minutes; Disagree | Detailed Comments |
| --- | --- | --- |
| HW | Disagree | Not essential but significant complexity added to UE implementation. The UE has to check each time about the outcome of UL skipping. In addition, it also brings the riks of misalignment between UE and NW with respect to the “timer” status. And the interaction may impact the time point of taking effect of the corresponding timers, e.g. BWP inactivity timer, which should be consulted with RAN1 and RAN4. |
| OPPO | Disagree | We think the current behaviour is clear and the proposed change may bring extra implementation complexity for UE. |
| Qualcomm | Agree as is | This change is not complicated to implement by UE, i.e. only when UE skips a UL grant, it adjusts the residual life of MAC timers. And there is little impact by state misalignment between UE and gNB, i.e. if network misses a UL Tx by UE, UE stays in active time longer than network expects it does; and it is an extremely rare event that UE does not transmit anything but network successfully receives a TB. |
| MediaTek | - | We have sympathy with the CRs and we believe this can bring power saving gain. However, we share same view with HW that this feature may bring more risk of NW/UE misalignment and indeed will create extra implementation complexity in both UE and NW side, when considering UL skipping. To have a compreshensive evaluation on the impact, we suggest to postpone the discussion to R17 and allow companies to have more time to check. |
| Xiaomi |  | It seems very difficutl to configure the value of the “skipMAC-TimerRestart” as the UL traffic would be very unpredictable. If the uplink traffic is known by the gNB, then the gNB should configure a shorter value of onDurationTimier or inactivityTimer. |

2) If you answered “agree” in 1), please indicate your answer to the RRC CR (R2-2100317)

| Company | Agree as is; Agree with changes; To capture it in the meeting minutes; Disagree | Detailed Comments |
| --- | --- | --- |
| Qualcomm | Agree as is |  |

3) If you answered “agree” in 1), please indicate your answer to the UE capability CR (R2-2100316)

| Company | Agree as is; Agree with changes; To capture it in the meeting minutes; Disagree | Detailed Comments |
| --- | --- | --- |
| Qualcomm | Agree as is |  |

Conclusion:

TBD

## 3.4	Clarification to RLC PDU polling at HO (Rel-15 and16)

Text Enhancement

R2-2101441	Clarification to RLC PDU Polling at Handover	Ericsson	CR	Rel-16	38.322	16.2.0	0038	-	F	NR_newRAT-Core

R2-2101442	Clarification to RLC PDU Polling at Handover	Ericsson	CR	Rel-15	38.322	15.5.0	0039	-	F	NR_newRAT-Core

These CRs propose to reflect the RRC statement “the UE should perform the reconfiguration with sync as soon as possible following the reception of the RRC message triggering the reconfiguration with sync, which could be before confirming successful reception (HARQ and ARQ) of this message” in RLC as well.

| Company | Agree as is (from which release); Agree with changes; To capture it in the meeting minutes; Disagree | Detailed Comments |
| --- | --- | --- |
| HW | Disagree | First there is a CR in this meeting to revise RRC CR to clarify the same thing and we think both CRs are not needed. The exising RRC spec has already specified how to handle the RLC/HARQ feedback for RRC signalling, and the user plane handling depends on the indication of reestablishRLC.Anyway, this kind of clarification should be taken into account in RRC spec and we should not duplicate the text in RLC spec as normally RRC messages and procedures should be transparent to RLC. |
| OPPO | Disagree | If it’s already captured in the RRC, there is no need to further clarify in RLC. |
| Qualcomm | Disagree | We think the current spec is clear and no further clarification is needed. |
| MediaTek | Disagree | Agree with QC. |
| Xiaomi | Disagree | Agree with QC and OPPO that the RRC already reflects such UE behaviour. No need to duplicate the texts in RLC. |

Conclusion:

TBD

## 3.5	RoHC handling during PDCP re-establishment (Rel-15 and 16)

R2-2101775	Discussion about RoHC handling during PDCP re-establishment	Huawei, HiSilicon	discussion	Rel-15	NR_newRAT-Core

During PDCP re-establishment, the UE may retransmit the PDCP data from the first missing SDU. However, there is a risk that the receiver side may discard the duplicated PDCP data carrying the new RoHC context. In this case, RoHC context state is misaligned between transmitter and receiver side. This paper would like to identify this issue and propose one solution similar to the DAPS that the transmitter should maintain IR state for retransmitted PDCP SDU during PDCP re-establishment.

1) please indicate your view on the issue identified in this paper (R2-2101775)

| Company | Agree with the issue; Disagree | Detailed Comments |
| --- | --- | --- |
| HW | Agree with the issue | We confirm the issue is valid when, for instance, drb-ContinueROHC is enabled. Without a specific solution in the spec, it is likely that the RoHC context could be misaligned between UE and NW during PDCP retransmissions, i.e. upon PDCP re-establishment. |
| OPPO | Agree with the issue |  |
| Qualcomm | Agree with the issue | We agree this is a genuine issue and needs to be fixed. |
| MediaTek | Agree with the issue |  |
| Xiaomi | Agree with the issue |  |

2) If you answered “agree” in 1), please indicate your views on the proposed solution in the annext TP in this paper (R2-2101775)

| Company | Agree as is (from which release); Agree with changes; To capture it in the meeting minutes; Disagree | Detailed Comments |
| --- | --- | --- |
| HW | Agree as is, But open to other solutions | We understand that, the proposed solution is motivated by the DAPS RoHC handling that has been discussed in Rel-16. So we slightly prefer to extend the solution to the PDCP re-establishment case without introducing a brand new one. However, as long as the issue can be resolved, we are open to other suggested solutions either by NW side or UE side or both sides. |
| OPPO | The note is ok to us |  |
| Qualcomm | Disagree | The proposed solution, i.e. falling back to IR state upon re-establishment, seems a fix. However, it is not a desired solution for us because: - Not needed when RoHC is reconfigured, as anyway RoHC will start from IR state - For other case, i.e. ContinueRoHC was enabled, - ContinueROHC loses its value as continuity is not maintained, when UE starts from IR states. That defies the purpose of the ContinueRoHC feature; - It introduces undesired complexity at the UE; - In addition, RoHC has an existing mechanism for recovery (feedback system) in place. We’d suggest to have further discussions to find a more efficient solution. |
| MediaTek | Agree | We think adding a note is ok, and we are fine for discussing other solution(s) if any. |
| Xiaomi | Disagree | In Rel-15 we agree not to decompress the duplicated PDCP PDU by changing the LTE PDCP behaviours, and this implies that the transmitter should not change the IR state which will lead to the decompression failure. However it was agreed that this is up-to the transmitter’s implementation, and no need to capture anything in the specification. |

Conclusion:

TBD

# 4	Conclusion

TBD

# 5	References

[1]	RAN2 113-e Chairman Notes 2021-01-25 0900 UTC.docx