3GPP TSG-RAN WG2 Meeting #113e	R2-21XXXXX

# Electronic Meeting, 25th Jan – 5th Feb 2021

Title:	[Draft] LS Reply on RRC based BWP switch

Work Item:	NR_newRAT-Core

Source:	Apple Inc [To be RAN2]

To:	RAN WG4

Cc:

Contact Person:

#### Name:	Naveen Palle

E-mail Address:	naveen dot palle at apple dot com

Attachments:

- Overall Description:

RAN2 thanks RAN4 for their enquiry on the topic of RRC based BWP switching and would like to provide the below responses to the questions asked.

- On whether RRC reconfiguration can change any parameter of the already active BWP of an activated SCell or SpCell.

Response: According to the RAN2 specification the NW can change any parameters of an already active BWP of an SPCell or an SCell with the exception that the modification of firstActiveDownlinkBWP-Id and firstActiveUplinkBWP-Id  for an SCell is not allowed.

- On whether this RRC reconfiguration without modification of firstActiveDownlinkBWP-Id or firstActiveUplinkBWP-Id for an activated SCell or SpCell can trigger a BWP switch.

Response: RAN2 confirms that an RRC message with a firstActiveDownlinkBWP-Id and firstActiveUplinkBWP-Id that is different from the UE’s current BWP ID, results in a BWP switch for an SpCell. And in Rel-15 or in Rel-16, the BWP switching for SCell using RRC message is not be possible.

RAN2 has not specified whether there is a BWP switch or not for the NW change of any parameters (with the exception of firstActiveDownlinkBWP-Id or firstActiveUplinkBWP-Id) of an already active BWP of an SPCell or an SCell.

RAN2 notes that It is also possible that an SCell can be released and added again with a different BWP in a single RRC message, but this is not considered as a BWP switch according to RAN2.

Further more, RAN2 would like to inform RAN4 that during the RRCResume/RRCSetup procedure for the PCell, the active BWP parameters change for the UE or the BWP can be switched.

2. To RAN WG2 group.

ACTION: RAN2 requests RAN4 to take the above responses into consideration.

3. Date of Next TSG-RAN WG4 Meetings:

TSG RAN WG2   Meeting #113-bis-e                   Apr 12 – 20 , 2021	         Online

TSG RAN WG2   Meeting #114-e            	   May 19–  27 , 2021            Online