3GPP TSG-RAN WG2 Meeting #113 Electronic	R2-210xxxx

Elbonia, 25 January – 05 February 2021

Agenda item:	5.4.1.4

Source:	Nokia

Title:	Summary of [AT113-e][007][NR15] Inter Node RRC (Nokia)

WID/SID:	NR_newRAT-Core, TEI16

Document for:	Discussion and Decision

# 1	Introduction

This document is the report of the following email discussion:

#### 5.4.1.4	Inter-Node RRC messages

[AT113-e][007][NR15] Inter Node RRC (Nokia)

Scope: Treat R2-2100586, R2-2100772, R2-2100773, R2-2101934, R2-2101347, R2-2101705, R2-2101935, R2-2101936, R2-2101944, R2-2101021, R2-2101022

Phase 1, determine agreeable parts, Phase 2, for agreeable parts Work on CRs.

Intended outcome: Report and Agreed CRs.

Deadline: A first round with Deadline for comments Thursday Jan 28 1200 UTC to settle scope what is agreeable

SN initiated SCG release

R2-2100586	Clarification on inter node signalling upon SN initiated SCG release		Samsung Telecommunications	CR	Rel-16	38.331	16.3.1	2340	-	F	NR_newRAT-Core

Band combination selection

R2-2100772	Clarification on band combination selection over inter-node message	NTT DOCOMO INC.	discussion	Rel-15	NR_newRAT-Core

R2-2100773	Clarification on band combination selection over inter-node message	NTT DOCOMO INC.	CR	Rel-15	38.331	15.12.0	2353	-	F	NR_newRAT-Core

R2-2101934	Clarification on band combination selection over inter-node RRC message	NTT DOCOMO INC.	CR	Rel-16	38.331	16.3.1	2453	-	A	NR_newRAT-Core

Message size

R2-2101347	Discussion on inter-node coordination of message size in MR-DC	Samsung Telecommunications	discussion	NR_newRAT-Core

MN and SN configuration restrictions

R2-2101705	Discusson on the usage of MN and SN configuration restrictions	Huawei, HiSilicon	discussion	Rel-15	NR_newRAT-Core

R2-2101935	Clarification to usage of MN and SN configuration restrictions	Nokia, Nokia Shanghai Bell	CR	Rel-15	38.331	15.12.0	2035	2	F	NR_newRAT-Core	R2-2011224

R2-2101936	Clarification to usage of MN and SN configuration restrictions	Nokia, Nokia Shanghai Bell	CR	Rel-16	38.331	16.3.0	2036	2	A	NR_newRAT-Core	R2-2011225

ASN.1

R2-2101944	Lack of late non-critical extensions in inter-node messages	Nokia, Nokia Shanghai Bell	discussion	Rel-15	NR_newRAT-Core

Intra-band EN-DC

Move from 6.1.1

R2-2101021	Companion paper for CR proposed for intra-band EN-DC deployment issue	Nokia, Nokia Shanghai Bell	discussion	Rel-16	TEI16

R2-2101022	Inter-node messaging for supporting intra-band EN-DC scenarios	Nokia, Nokia Shanghai Bell	CR	Rel-16	38.331	16.3.1	2377	-	B	TEI16

# 2	Discussion

Topic 1: SN initiated SCG release

R2-2100586	Clarification on inter node signalling upon SN initiated SCG release		Samsung Telecommunications	CR	Rel-16	38.331	16.3.1	2340	-	F	NR_newRAT-Core

There is ambiguity in current specification regarding inter-node signalling for the following case: SN

initiated release of SCG configuration while keeping some SN terminated DRBs. It seems that the MN

may not  initiate SCG release towards UE while SN releases SCG, or MN may initiate SCG release

unintentionally depending on how one interprets the signalling. In EN-DC, the issue can be solved by

RAN3 via setting X2AP::SGNB MODIFICATION REQUIRED with SCG resources == not present, which

SN can inform MN to release SCG resource.However, in XnAP, no such IE can be found in XnAP::S-

NODE MODIFICATION REQUIRED.

Question 1: Do companies agree to the CR in R2-2100586?

| Answers to Question 1 |  |  |
| --- | --- | --- |
| Company | Yes/No | Comments (e.g. changes required to be acceptable, why the CR is or is not needed) |
| ZTE | No with comments | We think it is a valid scenario, i.e. releasing the lower layer configuration of SCG. However, before discussing the detailed solution on how to inform MN, we think we should first discuss which node is responsible for making such decision? MN or SN? At least for SN addition procedure, it is up to MN to decide whether SN can only setup PDCP without any radio bearer (e.g. does not candidateCellInfoListSN in CG-ConfigInfo). However, in the scenario raised in the CR, maybe it is more appropriate to first send “request” (e.g. Inactivity indicator) to MN , and then let MN to decide whether to release SN’s lower layer, or release entire SN, or trigger bearer type change...etc. So we suggest to let RAN3 to discuss this issue first, if RAN3 confirms the releasing of SCG lower layer can be initiated by SN directly, we can then discuss in RAN2 how to achieve this. |
| Google |  | We understand this issue but we prefer to solve this issue in RAN3 as EN-DC. |
| Ericsson | No but the issue is valid | We acknowledge that the issue is valid and current signalling may bring to confusion when the SN wants to release the SCG lower layers. However, the option proposed in the CR is still not crystal clear and it assumes that the UE needs to check the presence/absence of two or more fields at once. This is not a future proof solution since the meaning or usage of those fields may change in next releases. For us, a clean and simple solution to address this issue would be to introduce an indicator in the INM so that the SN can inform the MN about the SCG release. |

Summary 1: TBD.

Proposal 1: TBD.

Topic 2: Band combination selection

R2-2100772	Clarification on band combination selection over inter-node message	NTT DOCOMO INC.	discussion	Rel-15	NR_newRAT-Core

R2-2100773	Clarification on band combination selection over inter-node message	NTT DOCOMO INC.	CR	Rel-15	38.331	15.12.0	2353	-	F	NR_newRAT-Core

R2-2101934	Clarification on band combination selection over inter-node RRC message	NTT DOCOMO INC.	CR	Rel-16	38.331	16.3.1	2453	-	A	NR_newRAT-Core

In current spec when MN sends SN with allowedBC-ListMRDC in CG-ConfigInfo there exists problem that since eNB has no reference to NR capability if MN narrow down a list of Allowed band combinations and transmits it to SN there is no band combination available for SN to select. For instance if MN received the following BC1 and BC2 from UE and narrow down the band combination list to BC1 only then send to SN. Suppose SN only support channel bandwidth of 100MHz operation then there is no band combination available for SN to select.

To resolve the problem mentioned above it is proposed that the MN may increase the probability that the SN finds a suitable SCG configuration by including in this field all entries that comprise at least the PCell band of MN.

Question 2: Do companies agree to the issue in R2-2101934? If yes, then please continue with the CR discussion as well if they are agreeable or not.

| Answers to Question 2 |  |  |
| --- | --- | --- |
| Company | Yes/No | Comments (e.g. changes required to be acceptable, why the CR is or is not needed) |
| ZTE | Acceptable | In our understanding, the intention of CR is correct. Although it is more related to network implementation, we can accept the clarification if it helps the discussion happened in other organization (e.g. O-RAN). Regarding the added sentence, we suggest to remove “of MN” for simplicity. The MN may increase the probability that the SN finds a suitable SCG configuration by including in this field all entries that comprise at least the PCell band of MN. |
| Google |  | The issue can be resolved by the MN implementations. The current text does not prevent the proposal in the MN implementation. If companies think such clarification is needed, we suggest the wording is revised as as: “To make the SN easier find a suitable SCG configuration, the MN can include in the field the entries that comprise at least the PCell band”. |
| Ericsson | Yes | Same view as ZTE, we are also fine with the correction ZTE proposes and prefer to go with their suggested phrase. |

Summary 2: TBD.

Proposal 2: TBD.

Topic 3: Message size

R2-2101347	Discussion on inter-node coordination of message size in MR-DC	Samsung Telecommunications	discussion	NR_newRAT-Core

RAN2 discussed DL RRC segmentation in case of Dual Connectivity and handover. In particular whether the SN (in case of Dual Connectivity) and the target (in case of handover) would need to be aware of the MN (in case of Dual Connectivity) and source (in case of handover) supports DL RRC segmentation.

The motivation for this is that the SN/target can provide RRC messages (SN-configuration and handover command respectively) to the MN/source which are beyond the PDCP limit only if the MN/source supports DL RRC segmentation.

RAN2 discussed whether the MN/source can indicate this to the SN/target in the SCG-ConfigInfo/AS-context. But based on the discussion it was identified that this is better to be discussed in and potentially addressed by RAN3.

The discussion in RAN2 was limited to segmentation. We however think that in DC there is an issue unrelated to support of DL segmentation. i.e. in case of DC the maximum size has to be shared between MN and SN. i.e. MN may initiate a reconfiguration that given UE capability limitations can only be done if SN performs a re-configuration at the same time. In such case there may be a need for MN to indicate what size is remaining for SN to use. Likewise, there may be cases in which SN initiates a re-negotiation i.e. requesting a larger share of the UE capabilities that may require MN to perform a reconfiguration at the same time. In both cases it is desirable for MN and SN to perform the reconfiguration simultaneously i.e. as it is desirable to have joint/ success failure.

Proposal	RAN2 is requested to discuss whether any R15 changes are required to coordinate sharing of the RRC message size between MN and SN or whether this can be addressed by RAN3 as part of the R16 discussions related to DL segmentation (as in the LS)

Question 3: Do companies agree to the proposal in R2-2101347?

| Answers to Question 3 |  |  |
| --- | --- | --- |
| Company | Yes/No | Comments |
| ZTE | No | In our understanding, the message size goes beyond limitation mainly happens when both MN and SN are adding SCells at the same time. However, this may happen rarely. And from MN’s perspective, when MN receives the RRCReconfiguration message embedded in CG-Config, MN knows the size of this message. So MN can determine whether MN can also include other MCG configuration in MN RRCReconfiguration message or not. So we prefer to do nothing now. However, if most companies think this is urgent, we are open to further discuss it. |
| Google |  | We prefer this is addressed by RAN3 as part of the R16 discussions. |
| Ericsson | No | We think this is not an issue and what is proposed is an optimization to integrate in the INM signalling the RRC segmentation feature that was standardized in Rel-16. Along this line, this does not look even a Rel-15 issue. Anyway, we are not so keen to have any change now. |

Summary 3: TBD.

Proposal 3: TBD.

Topic 4: MN and SN configuration restrictions

R2-2101705	Discusson on the usage of MN and SN configuration restrictions	Huawei, HiSilicon	discussion	Rel-15	NR_newRAT-Core

Observation1: configRestrictInfo is not allowed to be included in SgNB Modification Request procedure during an ongoing SN triggered Modification procedure. Proposal 1: If companies want to enhance the procedure to allow the MN to re-negotiate the configuration restriction in SN triggered modification procedure send an LS to RAN3 to ask whether SgNB Modification Refuse should be used to suggest the configuration restriction for the next round of SN triggered modification.

R2-2101935	Clarification to usage of MN and SN configuration restrictions	Nokia, Nokia Shanghai Bell	CR	Rel-15	38.331	15.12.0	2035	2	F	NR_newRAT-Core	R2-2011224

R2-2101936	Clarification to usage of MN and SN configuration restrictions	Nokia, Nokia Shanghai Bell	CR	Rel-16	38.331	16.3.0	2036	2	A	NR_newRAT-Core	R2-2011225

The fields CG-ConfigInfo::configRestrictInfo and CG-Config::configRestrictModReqSCG have different roles in handling the MN/SN configuration restrictions: The MN field configRestrictInfo can be used in both MN and SN-initiated procedures, but the SN field configRestrictModReqSCG is only used in SN-initiated procedures. However, since this is not catured in the field descriptions there could be confusion as to how these fields are used: For example, if SN is allowed to include configRestrictModReqSCG in response to SN addition procedure, how should MN interpret this: Does SN accept the addition conditionally, or is it simply indicating a preference for other values? Either would break the Rel-15 MR-DC principle of not allowing "negotiation" during the procedures (i.e. only accept or reject is allowed), so this can cause inter-operability issues in case networks comprehend these differently.

Question 4A: Do companies agree to Observation 1 and Proposal 1 in R2-2101705? Please comment.

| Answers to Question 4A |  |  |
| --- | --- | --- |
| Company | Yes/No | Comments |
| Huawei, HiSilicon (Proponent) | Yes | Observation 1 is to indicate that according to 36.423, if the MN triggers modification in an SN-initiated procedure, the motivation does not include re-negotiation on the configuration restriction. Therefore there’s no ambiguity, and the question is whether we need any enhancement. On Proposal 1, we are ok with not having any optimization (i.e. MN does not include configRestrictInfo in SN-initiated procedures). If RAN2 wants to have any kind of enhancement, RAN3 spec and the stage 2 spec need to be modified as well. And from our perspective, enhancing the usage of SgNB Modification Refuse is a better way to go (as indicated in the latter part of Proposal 1). |
| ZTE | No | First, we think the issue discussed in R2-2101705 is different from Nokia’s paper. Regarding the scenario raised in HW’s paper, in our understanding, if MN cannot accept the value requested by SN, MN can directly send SgNB Modification Refuse message without including a new value in it. From SN perspective, as long as SN receives SgNB Modification Refuse message, the SN knows the “request” is rejected by MN, and the SN has to use the previous value set by the MN. If the MN wants to allocate a new value to the SN, the MN can then initiate a new SgNB Modification Request procedure. So there is no need to include new value in SgNB Modification Refuse message. And no need to send LS to RAN3. Regarding observation 1, we think it is too strict to add such restriction. For instance, if MN accepts the value requested by SN, MN should be allowed to indicate the new value in configRestrictInfo, and send it to SN in response to SN initiated procedure. |
| Google |  | We agree with observation 1. Regarding proposal 1, we wonder such optimization is needed. |
| Ericsson | Yes | We basically agree with the proposal but we also want to echo Huawei comment regarding that the MN should not provide further restriction as response to an SN-initiate procedure. In such a case, the way how it work should be what is described by ZTE. We also think that no LS to RAN3 is needed. |

Summary 4A: TBD.

Proposal 4A: TBD.

Question 4B: Do companies consider that the CRs in R2-2101935 and R2-2101935 are agreeable? Please provide your comments on the CR

| Answers to Question 4B |  |  |
| --- | --- | --- |
| Company | Yes/No | Comments |
| Huawei, HiSilicon | No | First we would like to make sure whether the CRs want MN to carry configRestrictInfo in Step 2 or Step 6? If carried in Step 6, since the Uu signalling has been transferred to UE in Step5, it is possible that UE and SN will maintain different configuration (if SN accepts the restriction). If carried in Step 2, it is uncertain whether SN must accept this restriction. We believe MN should not force SN to accept the restriction, rather, it is a suggestion to SN. We think this suggestion process is an optimization because MN is already allowed to configure such restriction in MN-initiated procedures, so the preferred way is to stick to the current spec (MN does not include configRestrictInfo in SN-initiated procedures) |
| ZTE | Yes to Rel-15 CR, Partially Yes to Rel-16 CR | We agree with the clarification sentences added in field description in both Rel-15 CR and Rel-16 CR. But we don’t think the below new fields are needed in Rel-16 CR. For measurement configuration, it may change dynamically based on local RRM strategy and UE’s movement, the only thing we need to do is to ensure the measIDs configured by MN and SN won’t exceed UE’s capability. So from SN perspective, the SN only needs to know the maximum number it can configure to UE. There is no need to inform the MN the exact number of measIDs currently used by the SN in real time. CG-Config-v16xy-IEs ::= SEQUENCE { maxInterFreqMeasIdSCG-r16 INTEGER(1..maxMeasIdentitiesMN) OPTIONAL, maxIntraFreqMeasIdSCG-r16 INTEGER(1..maxMeasIdentitiesMN) OPTIONAL, nonCriticalExtension SEQUENCE {} OPTIONAL } |
| Google | Yes | We agree with the intent to capture the Rel-15 MR-DC principle of not allowing "negotiation" during the procedures. |
| Ericsson | Yes |  |

Summary 4B: TBD.

Proposal 4B: TBD.

Topic 5: ASN.1

R2-2101944	Lack of late non-critical extensions in inter-node messages	Nokia, Nokia Shanghai Bell	discussion	Rel-15	NR_newRAT-Core

Observation 1: None of the INMs have introduced OCTET STRING for late NCE corrections to Rel-15. Observation 2: The INM CG-Config CG-ConfigInfo and MeasurementTimingConfiguration have already been extended in Rel-16 making introduction of late NCEs to Rel-15 difficult. And proposed the following: Proposal 1: RAN2 to discuss how to handle late NCEs (for Rel-15 and Rel-16) to the inter-node messages

Question 5: Do companies agree to observations and proposal made in R2-2101944?

| Answers to Question 5 |  |  |
| --- | --- | --- |
| Company | Yes/No | Comments |
| ZTE | No strong view | Agree to observations, no strong view to solution. |
| Google |  | We can postpone discussing it until we encounter a real problem, that is, we need to add something to Rel-15. |
| Ericsson | No | We acknowledge that this was a mistake that we made in Rel-15 but our preference is to do nothing for now. We can introduce late NCEs in Rel-17 if we want to avoid to carry the same mistake. |

Summary 5: TBD.

Proposal 5: TBD.

Topic 6: Intra-band EN-DC

Move from 6.1.1

R2-2101021	Companion paper for CR proposed for intra-band EN-DC deployment issue	Nokia, Nokia Shanghai Bell	discussion	Rel-16	TEI16

R2-2101022	Inter-node messaging for supporting intra-band EN-DC scenarios	Nokia, Nokia Shanghai Bell	CR	Rel-16	38.331	16.3.1	2377	-	B	TEI16

Companies agreed unanimously last meeting in the email discussion on the proposals based on which the CRs are now implemented. See Annex B for the whole discussion and companies input.

Question 6: Is the intent of the CR in R2-2101022 agreeable? The discussion paper in R2-2101021 attempts to explain the changes in the CR.

| Answers to Question 6 |  |  |
| --- | --- | --- |
| Company | Yes/No | Comments (e.g. changes required to be acceptable, why the CR is or is not needed) |
| Huawei, HiSilicon | Yes but | We think the issue is valid, and the IEs to be exchanged should be: carrier center frequency, bandwidth, and band indication. |
| ZTE | Yes with comments | Regarding whether to exchange “carrier center +BW” or reusing scs-SpecificCarrier, we actually think there is no big difference, anyway, one node (sending node or receiving node) needs to derive the “carrier center” information based on configured UE-specific channel BW. We slightly prefer to reuse scs-SpecificCarrier structure, so the calculation of carrier center is up to NW implementation, no need to capture it in specification. Regarding the issues raised in discussion paper, we think below Q2 is the most important that related to inter-operability, so we would like to confirm whether all companies have the same understanding. But the answer should have no impact to ASN.1 design. Q2 : Whether network needs to exchange the BWs of all configured SCSs? This relates to RAN4's formula, as we known for a given serving cell, network can configure multiple BWs for different SCSs, then for RAN4's formula, which BW should be used (e.g. for BWNR_channel) in calculating the required Nominal Channel spacing. A2: In our understanding this is only the configured channel bandwidth and SCS which is active at a given time. Any change of this requires a new procedure towards the UE and should result in informing also the peer node. Regarding the CR, we would suggest to highlight in field description that the fields are referring to “UE-specific channel BW”, not “cell-specific channel BW”. |
| Ericsson | Yes but | We agree with Huawei that carrier center frequency and bandwidth could be exchanged instead. For band indication, this may probably not be needed as the consequence of the UE capability coordination, i.e. MN sends allowedBC-ListMRDC and receives selectedBandCombination. |

Summary 6: TBD.

Proposal 6: TBD.

# 4	Conclusion

Always echo the list of observations and proposals.

# Annex A – Contact Points

Respondents to the email discussion are kindly asked to fill in the following table.

| Company | Name | Email Address |
| --- | --- | --- |
| Nokia | Amaanat | amaanat.ali@nokia.com |
| Huawei, HiSilicon | Lili Zheng | zhenglili4@huawei.com |
| ZTE | LiuJing | liu.jing30@zte.com.cn |
| Google | Frank Wu | frankwu@google.com |
| Ericsson | Antonino Orsino | antonino.orsino@ericsson.com |

# Annex B – Nokia contribution in RAN2#112-e was R2-2010976 Intra-band EN-DC deployment issue

Proposal 1 : RAN2 to clarify in the description of the scellFrequenciesSN-EUTRA and scellFrequenciesSN-NR what the "frequency" means (i.e. carrier center frequency or the SSB frequency).

Q2.1: Companies are invited to provide views on the above proposal.

| Company | Carrier centre frequency/ SSB frequency | Comments |
| --- | --- | --- |
| Qualcomm Incorporated | SSB frequency for NR / Center frequency for EUTRA | At least the combination of a serving frequency and measuredFrequency should be used by MN to identify the type of measurements configured by SN, i.e. either intra-frequency or inter-frequency. This is mentioned in section 7.2 of 37.340. There could be other intended purposes. |
| Ericsson (Tony) | SSB frequency for NR / Center frequency for EUTRA | Agree with QC understanding |
| ZTE (LiuJing) | SSB frequency for NR/ Center frequency for EUTRA | Agree with QC’s understanding. |
| Xiaomi (Yumin) | SSB frequency for NR / Center frequency for EUTRA |  |
| vivo (Wenming) | SSB frequency for NR / Center frequency for EUTRA | But it seems this paper was withdrawn. R2-2010976 Intra-band EN-DC deployment issue Nokia, Nokia Shanghai Bell discussion NR_newRAT-Core => withdrawn |
| Samsung | SSB frequency for NR / Center frequency for EUTRA | As indicated by others, the field was introduced for coordination of measurements and from this perspective SSB frequency is appropriate |
| Huawei | SSB frequency for NR/ Center frequency for EUTRA | Agree with QC’s understanding. |
| Intel |  | Document is withdrawn as per chair’s notes and we should not discuss it officially. |
| NEC | SSB frequency for NR/ Center frequency for EUTRA | We also agree with QC |
| Apple | SSB frequency for NR / Center frequency for EUTRA |  |
| CATT (Jing) | SSB frequency for NR / Center frequency for EUTRA |  |

Proposal 2: RAN2 to discuss how to exchange PSCell/Scell(s) carrier center frequency and channel bandwidth to ensure UE capability is respected in intra-band EN-DC deployments.

Q2.2: Companies are invited to provide views on the above proposal.

| Company | Comments |
| --- | --- |
| Qualcomm Incorporated | The identified problem looks real. NR uses somewhat complicated way of expressing the exact location of channel bandwidth, like point A, SCS specifics and so on. Something similar to FrequencyInfoDL/UL may need to be added. |
| Ericsson (Lian) | The problem is valid, one could exchange point A and channel bandwidth to this end. |
| ZTE (LiuJing) | Seems this was discussed before, but no conclusion was made at that time (see below history). R2-1909971 Inter Node Message impacts due to intra-band EN-DC Nokia, Nokia Shanghai Bell discussion Rel-15 - DOCOMO think this was discussed in February. Think these parameters such as channel raster and operating BW can be configured by operator O&M and do not need to be configured by INMs. - Huawei have the same view as DOCOMO. ZTE think the OAM BW will be the cell channel BW but for intra-band EN-DC we need to refer to the UE channel BW to ensure it is contiguous across LTE and NR. So think the Nokia issue is valid. DOCOMO think referring to the band combination set index then the channel bandwidth is also understood. ZTE think it is the bandwidth and location that is important, not just the BW. => Noted Still, we think the issue is valid, and we are open to further discuss the solution. |
| Xiaomi (Yumin) | This is a valid issue. We can discuss further on how to exchange the frequency and bandwidth. |
| vivo (Wenming) | We are open to discuss the issue. But it seems this paper was withdrawn. R2-2010976 Intra-band EN-DC deployment issue Nokia, Nokia Shanghai Bell discussion NR_newRAT-Core => withdrawn |
| Samsung | We agree there seems to be an issue, although(as indicated by ZTE and discussed earlier) the need depends on actual network deployment. Anyhow, we think existing signalling should not be affected but are open to consider introduction of extensions to address the issue. |
| Huawei | The issue is valid. RAN2 can consider adding carrier center frequency and channel bandwidth information into the inter-node message, maybe band information is also needed. (Having point A and BW may not be sufficient because point A itself does not help to deduce the center frequency.) |
| Intel | Document is withdrawn as per chair’s notes and we should not discuss it officially. |
| NEC | Our understanding is, similar to ZTE, that almost the same issue was discussed and concluded no specific action is taken, which would mean to assume OAM-based approach. Indeed, we are also open for further discussion (but from Rel-16) |
| Apple | We also agree this is a valid issue. |
| CATT (Jing) | Agree with the issue. Channel bandwidth, SCS specifics and point A can be considered to exchange for intra-band EN-DC deployments. |