3GPP TSG-RAN WG2 Meeting #113 Electronic	R2-210xxxx

Elbonia, 25 January – 05 February 2021

Agenda item:	6.5.2

Source:	Nokia, Nokia Shanghai Bell

Title:	[DRAFT] Summary of Email Discussion [AT113-e][025][IIOT] RRC (Nokia)

WID/SID:	NR_IIOT-Core - Release 16

Document for:	Discussion and Decision

# 1	Introduction

This email discussion aims to collect company views on Rel-16 RRC corrections that have been proposed for NR IIoT in RAN2#113e. The scope of this email discussion is:

[AT113-e][025][IIOT] RRC (Nokia)

Scope: Treat R2-2100712, R2-2101340, R2-2101941

Phase 1, determine agreeable parts, Phase 2, for agreeable parts Work on CRs.

Intended outcome: Agreed CRs if any is agreeable.

Deadline: Schedule A

The papers to be considered in this email discussion are listed below:

R2-2100712 Configuration of AutonomousTX and cg-retransmission timer Nokia, Nokia Shanghai Bell CR Rel-16 38.331 16.3.1 2349 - F NR_IIOT-Core R2-2101340 Correction on the configuration of Type 1 configured grant Huawei, HiSilicon CR Rel-16 38.331 16.3.1 2404 - F NR_IIOT-Core R2-2101941 LCP restriction for allowedCG-List and configuredGrantType1Allowed ASUSTeK CR Rel-16 38.331 16.3.1 2435 1 F NR_IIOT-Core R2-2101743

Please provide your contact information when responding:

| Company | Contact Name | Email |
| --- | --- | --- |
| Nokia | Ping-Heng Wallace Kuo | Ping-Heng.Kuo@nokia.com |
| Ericsson | Zhenhua Zou | Zhenhua.Zou@Ericsson.com |

# 2	Discussion

## 2.1	Joint Configuration of Autonomous Transmission and CG Retransmission Timer

R2-2100712 considers the following agreement made in RAN2 #112e during discussion of Rel-17 NR IIoT/URLLC:

RAN2 #112e Agreement: - The assumption for Rel-16 is that the network will not configure autonomousTx and cg-RetransmissionTimer simultaneously per cell. No optimizations will be pursued to allow the two features be configured together in Rel-16. No CR is needed for this for now.

It was observed that, if these two features are configured together in Rel-16, it would lead to some undefined UE behaviour. Therefore, most companies do not think these will be jointly configured. In particular, when autonomousTx is configured, it was agreed that the configured grant timer should be stopped upon de-prioritization of a PUSCH; nevertheless, it creates some ambiguity for cg-RetransmissionTimer. As there is no intention for further optimization in Rel-16, it might be better to disallow the joint configuration in specification to avoid potential misconfiguration. On the other hand, in Rel-16 IIoT features including autonomousTx are typically used in licensed band, while the applicability of cg-RetransmissionTimer is restricted to unlicensed spectrum. Thus, it was proposed to have the following modification in the field description of autonomousTx:

autonomousTx If this field is present, the Configured Grant configuration is configured with autonomous transmission, see TS 38.321 [3]. This field is not configured when cg-RetransmissionTimer is configured in any Configured Grant configuration in the same serving cell.

Question 1: Do you agree modifying the field description of autonomousTx to ensure it is NOT jointly configured with cg-RetransmissionTimer in Rel-16 ?

| Company | YES/NO | Comments |
| --- | --- | --- |
| Nokia | Yes | Although the agreement says that CR is not needed for now, on the safe side we think it is better to clarify at this stage to avoid further confusion. It does not harm at all to have such clarification, while providing a clearer guideline for the product implementation – the developers do not have to read through meeting notes to find the agreed assumptions. |
| Ericsson | No | The Rel-17 discussion did not conclude that any feature is broken when two are configured together. It points to the direction that, if some optimizations are needed after the Rel-17 discussion, then RAN2 does not need to go back and change the Rel-16 spec. For the particular example mentioned above, the UE behaviour is clear on when the cg-RetransmissionTimer is (re)-started or stopped, although it might not be optimal. On another general comment, Ericsson does not prefer adding unnecessary network configuration restrictions. It is up-to network implementation to judge and consider if a particular configuration makes sense or not and in the case of undefined UE behaviour, the understanding is that it is up-to UE implementation and the corresponding network implementation can handle all cases. For the network that choose not to handle all those cases, they can simply choose not to configure them together. |

## 2.2	Configuration of Type-1 Configured Grant

R2-2101340 considers the following agreement made in RAN2 #109e:

- Two CGs of any type, one activated in UL and another activated in SUL, are not time-overlapping by the control of the network. This can be captured in the stage-2 spec.

Based on this agreement, configured grants can be configured in both SUL and NUL, as long as time-overlapping between these grants on SUL and NUL could be avoided via gNB scheduling. This is different to the current RRC specification, wherein it forbids simultaneous configuration of Type-1 configured grant on both SUL and NUL. Hence, the CR suggests that the field description of rrc-ConfiguredUplinkGrant should be modified as following by removing the sentence “Type 1 configured grant may be configured for UL or SUL, but not for both simultaneously”:

rrc-ConfiguredUplinkGrant Configuration for "configured grant" transmission with fully RRC-configured UL grant (Type1). If this field is absent the UE uses UL grant configured by DCI addressed to CS-RNTI (Type2). Type 1 configured grant may be configured for UL or SUL, but not for both simultaneously.

Question 2: Do you agree modifying the field description of rrc-ConfiguredUplinkGrant to remove the restriction such that Type-1 CG cannot be configured in NUL and SUL simultaneously ?

| Company | YES/NO | Comments |
| --- | --- | --- |
| Nokia | Yes | It resolves the gap between agreement and RRC specification. |
| Ericsson | Yes | This was agreed in RAN2#109e. The impact on this field was simply missed in the RRC running CR discussion. |

## 2.3	Allowed CG List

R2-2101941 considers the issue of potential ambiguity caused by configurations of configuredGrantType1Allowed and allowedCG-List. For instance, the CR has identified two cases:

- If the field configuredGrantType1Allowed is present but there is no CG indicated in the sequence of allowedCG-List since it’s not present, UE may be confused whether all CG Type 1 configurations can be used for UL MAC SDUs from this logical channel since nothing is actually indicated in the sequence.

- If the field configuredGrantType1Allowed is not present, it’s true that this sequence does not include any configured grant type 1 configuration. But UE may be confused whether this configuration means all CG Type 2 configurations can be used for UL MAC SDUs from this logical channel, since the absence of allowedCG-List means all configured grant configurations are allowed for this logical channel.

Therefore, the CR proposes the following change in the field description of allowedCG-List :

allowedCG-List This restriction applies only when the UL grant is a configured grant. If present, UL MAC SDUs from this logical channel can only be mapped to the indicated configured grant configuration. If the size of the sequence is zero, then UL MAC SDUs from this logical channel cannot be mapped to any configured grant configurations. If the field is not present, ignore the field configuredGrantType1Allowed and UL MAC SDUs from this logical channel can be mapped to any configured grant configurations. If the field configuredGrantType1Allowed is present, only those configured grant type 1 configuration indicated in this sequence are allowed for use by this logical channel; otherwise, this sequence shall not include any configured grant type 1 configuration. Corresponds to "allowedCG-List" as specified in TS 38.321 [3].

Question 3: Do you agree modifying the field description of allowedCG-List such that the LCH should ignore the field of configuredGrantType1Allowed when allowedCG-List is not present ?

| Company | YES/NO | Comments |
| --- | --- | --- |
| Nokia | No | We do not see any problem with current text. The two cases that the CR proponent has mentioned are indeed the intended behaviour: - If the field configuredGrantType1Allowed is present but there is no CG indicated in the sequence of allowedCG-List since it’s not present, the LCH can be mapped to all CGs that are configured, including Type-1 CGs. - If the field configuredGrantType1Allowed is not present, the LCH can be mapped to all Type-2 CGs that are configured. We are not sure what are the issues that this CR tries to resolve. In any case we believe proper gNB implementation can avoid any confusion at the UE side. |
| Ericsson | No on this particular change; but there is a need for further clarifications and possible changes. | In my understanding, UE shall meet all the LCP conditions in the subclause 5.4.3.1.2 of the MAC spec: The MAC entity shall, when a new transmission is performed: 1> select the logical channels for each UL grant that satisfy all the following conditions: 2> the set of allowed Subcarrier Spacing index values in allowedSCS-List, if configured, includes the Subcarrier Spacing index associated to the UL grant; and 2> maxPUSCH-Duration, if configured, is larger than or equal to the PUSCH transmission duration associated to the UL grant; and 2> configuredGrantType1Allowed, if configured, is set to true in case the UL grant is a Configured Grant Type 1; and 2> allowedServingCells, if configured, includes the Cell information associated to the UL grant. Does not apply to logical channels associated with a DRB configured with PDCP duplication within the same MAC entity (i.e. CA duplication) when CA duplication is deactivated for this DRB in this MAC entity; and 2> allowedCG-List, if configured, includes the configured grant index associated to the UL grant; and 2> allowedPHY-PriorityIndex, if configured, includes the priority index (as specified in clause 9 of TS 38.213 [6]) associated to the dynamic UL grant.. The field description in RRC is more a clarification on each sub-condition and in this case the one related with allowedCG-List. The proposed CR has changed the intention in the MAC spec. For example, if allowedCG-List is not configured (i.e., not present), the legacy configuredGrantType1Allowed shall still be applied as shown in the MAC. Technically it is possible to configure both CG type 1 and type 2 in one BWP. Suppose allowedCG-List is not configured. If configuredGrantType1Allowed is configured, then the MAC spec tells that the LCH can be mapped to any Configured Grant (including type 2). If configuredGrantType1Allowed is not configured, then the MAC spec tells that the LCH can only be mapped to Configured Grant type 2. I believe this is the correct intention and also Nokia’s understanding. What is worthwhile to discuss is the below additional clarification text. The highlighted word “only” may be misunderstood as that type 2 is not allowed: - If the field configuredGrantType1Allowed is present, only those configured grant type 1 configuration indicated in this sequence are allowed for use by this logical channel; otherwise, this sequence shall not include any configured grant type 1 configuration. My recollection is that this was added in the running CR discussion but was not carefully revised in the later revisions. It is also okay for Ericsson to remove completely this clarification part, as it is a description on configuredGrantType1Allowed. |

# 3	Conclusion

[TBD]