3GPP TSG-RAN WG2 Meeting #113 Electronic	R2-210XXXX

25 January – 05 February 2021

Agenda item:	8.16.2

Source:	Nokia

Title:	Summary of [AT113-e][031][eNPN] SNPN with subscription or credentials by a separate entity

WID/SID:	NG_RAN_PRN_enh-Core - Release 17

Document for:	Decision

1	Introduction

This document is the summary of the following email discussion:

[AT113-e][031][eNPN] SNPN with subscription or credentials by a separate entity (Nokia)

Scope: Take into account documents submitted to this section, 1st pass: identify what is required to be supported by AS and determine the RAN2 impact, if possible. Identify common views / potential initial agreements, Identify points that need further discussion. Can also gather comments on the need to ask questions to other group.

Intended outcome: Report with agreeable proposals and discussion points (not too many, preferably < 10) for treatment on-line

Deadline: 1st Deadline for Comments: Friday Jan 29 1000 UTC. Other deadline if needed by rapporteur. Report Ready for treatment on-line Feb 3.

Contact person(s) for each participating company

| Company | Name | Email address |
| --- | --- | --- |
| Nokia | Gyorgy Wolfner | gyorgy.wolfner@nokia.com |
| ZTE | Wenting Li | li.wenting@zte.com.cn |

2	Discussion

Documents considered during this email discussion (papers submitted to agenda item 8.16.2):

R2-2100543	Overview of RAN2 impacts to support SNPN with 3rd party subscription	Nokia, Nokia Shanghai Bell	discussion	Rel-17	NG_RAN_PRN_enh

R2-2101717	Support SNPN along with credentials owned by an entity separate from the SNPN	CMCC	discussion	Rel-17	NG_RAN_PRN_enh-Core

R2-2100241	Initial Discussion on Credential by a Separate Entity	OPPO	discussion	Rel-17	NG_RAN_PRN_enh-Core

R2-2100277	Consideration on SNPN with Subscription or Credentials by a Separate Entity	CATT	discussion	Rel-17	NG_RAN_PRN_enh-Core

R2-2100289	Discussion of credentials by a separate entity in SNPN	China Telecommunication	discussion	Rel-17

R2-2100431	Consideration on the Separate Entity Supporting	ZTE Corporation, Sanechips	discussion	Rel-17	NG_RAN_PRN_enh-Core

R2-2100441	Access to SNPN with credentials from a different entity	Qualcomm Incorporated	discussion

R2-2100490	SNPN and Service Provider (SP) separation	Ericsson	discussion	Rel-17	NG_RAN_PRN_enh-Core

R2-2100634	RAN2 impact on support SNPN along with subscription / credentials owned by an entity separate from the SNPN	Intel Corporation	discussion	Rel-17	NG_RAN_PRN_enh-Core

R2-2100838	Support SNPN with subscription or credentials by a separate entity	vivo	discussion

R2-2100918	SIB info for third party credentials and UE onboarding	Sony	discussion	Rel-17	NG_RAN_PRN_enh-Core

R2-2101001	Discussion on RAN2 impact of supporting SNPN with credentials owned by a separate entity	Huawei, HiSilicon, China Telecom	discussion	Rel-17	NG_RAN_PRN_enh-Core

R2-2101515	Support of SNPN with subscription or credentials by a separate entity	LG Electronics	discussion	Rel-17

2.1	Broadcasting related issues

Q1.1a: Do you agree that an indicator that "access using credentials from a separate entity is supported" should be broadcasted?

Q1.1b: Do you agree that the indicator should be broadcasted per SNPN in shared cells?

Q1.1c: Do you have a proposal in which SIB the indicator should be broadcasted?

| Company | Answer Q1.1a | Answer Q1.1b | Answer Q1.1c | Comments |
| --- | --- | --- | --- | --- |
| ZTE | Yes | Yes | Yes(SIB1) | As concluded in the SA2, - For a UE configured to use SNPNs, automatic SNPN selection is performed in the following order: - UE selects and attempts to register with the SNPN it was last registered with (if available). - UE selects and attempts to register with the available SNPN identified by a PLMN ID and NID for which the UE has SUPI and credentials (i.e. as in Rel-16). - UE selects an available and allowable SNPN, which broadcasts "access using credentials from a separate entity is supported" indication and an SNPN ID contained in the user-controlled list (if available) - UE selects an available and allowable SNPN which broadcasts "access using credentials from a separate entity is supported" indication and a GID contained in the separate entity-controlled list (if available). Considering that this indication will affect the network/ cell selection, we prefer to include it in the SIB1, FFS for the NPN-IdentityInfo-r16 or add a new list to the CellAccessRelatedInfo |

Q1.2a: Do you agree that the Supported Group IDs (GIDs) should be broadcasted?

Q1.2b: Do you agree that the Supported Group IDs (GIDs) should be broadcasted per SNPN in shared cells?

Q1.2c: Do you have a proposal on the maximum number of Supported Group IDs (GIDs) to be broadcasted?

Q1.2d: Do you have a proposal in which SIB the Supported Group IDs (GIDs) should be broadcasted?

| Company | Answer Q1.2a | Answer Q1.2b | Answer Q1.2c | Answer Q1.2d | Comments |
| --- | --- | --- | --- | --- | --- |
| ZTE | Yes | Yes | Yes(See comments) | Yes(See comments) | For the Q1.2c, we think the total number of the Group IDs can be counted independently from the legacy network IDs.E.g. the total number of the Group IDs of all of the SNPNs does not exceed 12 ( meanwhile, the total number of the legacy Network IDs including (PLMNs, PNI-NPNs, and SNPN) does not exceed 12) For the Q1.2d, similar to question 1, considering that this indicate will affect the network/ cell selection, we prefer to include it in the SIB1, FFS for the NPN-IdentityInfo-r16 or add a new list to the CellAccessRelatedInfo. |

Q1.3a: Do you agree that an indicator whether "the SNPN allows registration attempts from UEs that are not explicitly configured to select the SNPN" should be broadcasted?

Q1.3b: Do you agree that the indicator should be broadcasted per SNPN in shared cells?

Q1.3c: Do you have a proposal in which SIB the indicator should be broadcasted?

| Company | Answer Q1.3a | Answer Q1.3b | Answer Q1.3c | Comments |
| --- | --- | --- | --- | --- |
| ZTE | Yes | Yes | Yes(See comments) | Similar to question 1, considering that this indicate will affect the network/ cell selection, we prefer to include it in the SIB1, FFS for the NPN-IdentityInfo-r16 or add a new list to the CellAccessRelatedInfo. |

Q1.4: Other broadcasting related proposals (e.g. other parameters to be broadcasted) to support SNPN with subscription or credentials by a separate entity?

| Company | Proposal |
| --- | --- |

2.2	Impacts on cell selection and reselection

Q2.1: Do you agree that the AS should report to the NAS about the broadcasted parameters listed in Q1.1, Q1.2 and Q1.3?

| Company | Answer | Comments |
| --- | --- | --- |
| ZTE | Yes | At least for the Q1.3, it shall be reported to the NAS, for that in the SA2’s interim conclusion, on the indication of Q1.3, it said “ - If the UE detects a plurality of such SNPNs, the order in which the UE selects and attempts a registration with an SNPN is implementation specific” we think the AS can send such kind of the SNPNs list and relaed indication in Q1.3 to the NAS and left the NAS to make the final decision. For the Q1.1/1.2, it also depends on the interaction between NAS and AS, at least for the case that the NAS layer indicate a list of SNPNs in the user-controlled list, or a list of Group IDs, the UE AS should report to the NAS about the broadcasted parameters of the corresponding SNPNs /Group IDs explicitly. For the case that the NAS layer indicate only one SNPN or only one Group ID, the parameter in Q1.1, Q1.2 can be reported implicitly. Anyway, the AS should report to the NAS about the broadcasted parameters listed in Q1.1, Q1.2 and Q1.3 explicitly or implicitly |

Q2.2: Do you see any impacts on cell selection or cell reselection (e.g. a need to change suitable cell criteria) to support SNPN with subscription or credentials by a separate entity?

| Company | Answer | Comments (e.g. details on the impacts) |
| --- | --- | --- |
| ZTE | Yes | We think at least it will affect the suitable cell definition |

2.3	Other issues

Q3.1: Do you see any impacts on connected mobility to support SNPN with subscription or credentials by a separate entity?

| Company | Answer | Comments (e.g. details on the impacts) |
| --- | --- | --- |
| ZTE | No | Up to now, we don’t see any impacts on the connected state mobility. |

Summary:

Rapporteur's Proposal:

Q3.2: Other issues that are not covered by previous questions related to support SNPN with subscription or credentials by a separate entity to be discussed in RAN2

| Company | Issue |
| --- | --- |
| ZTE | For the SNPN that only support group ID, whether the indication in the Q1.1 shall be set? Though in the SA2 interim conclusion, it said that “- UE selects an available and allowable SNPN which broadcasts "access using credentials from a separate entity is supported" indication and a GID contained in the separate entity-controlled list (if available)” , we still have concern on the case that the SNPN only support Group ID. For example, for cell A, the SNPN 1 only support Group ID1, then the UE (with HSP 1) that include the SNPN1 in user-controlled list may try to access Cell A to access it’s corresponding HSP 1, but for the Cell A, it only support HSP with Group ID1, it doesn’t support HSP1. |

Summary:

Rapporteur's Proposal:

2.4	Proposed questions to other WGs

Q4: Proposed questions to other WGs related to SNPN with subscription or credentials by a separate entity (please provide only a question if you think it is important to send an LS to other WGs from this RAN2 meeting)

| Company | WG(s) | Question |
| --- | --- | --- |
| ZTE | RAN3/CT1/SA2 | Whether the Ran node need the additional information for the AMF selection? |

Summary:

Rapporteur's Proposal:

3	Conclusions

3.1	Proposals that may be agreed

3.2	Proposals/Issues that requires further discussion

3.3	Potential questions to other WGs