3GPP TSG-RAN WG2 Meeting #113-e                                                             R2-2102014

Electronic, Jan 25th – Feb 5th, 2021

Agenda Item:	8.10.3.1

Source: 	CMCC

Title:  	Summary of email discussion [AT113-e][104][NTN] TAC update (CMCC)

Document for:	Discussion and Decision

1	Introduction

This document is for the following offline discussion, particularly for topics in 8.10.3.1:

[AT113-e][104][NTN] TAC update (CMCC)

Scope: Discuss TAC update procedure, based on R2-2101607, R2-2100259, R2-2100742, R2-2100820, R2-2101406

Initial intended outcome: Summary of the offline discussion with e.g.:

§  List of proposals for agreement (if any)

§  List of proposals that require online discussions

Initial deadline (for companies' feedback): Monday 2021-02-01 11:00 UTC

Initial deadline (for rapporteur's summary in R2-2102014): Monday 2021-02-01 17:00 UTC

Proposals marked "for agreement" in R2-2102014 not challenged until Tuesday 2020-02-02 11:00 UTC will be declared as agreed by the session chair. For the rest the discussion will continue online.

To address this issue of frequent TAU procedure triggered by the satellite motion, the “fixed tracking area” concept is proposed and has been captured in TR 38.821 (section 7.3.1.3) [1], in which the tracking area code (TAC) is fixed on ground while the  cells is sweeping on the ground. And it means that while the cells sweep on the ground, the tracking area code (i.e. TAC) broadcasted is changed when the cell arrives to the area of next planned earth fixed tracking area. Two approaches were discussed during the study item on how to effectively update the TACs that the cells/satellites are broadcasting they move across TAs, one is hard TAC update, another is soft TAC update, which is summarized in TR 38.821 [1] and previous email discussion [2] before RAN2#112 meeting:

Hard TAI update means that each cell can broadcast only on tracking area code. When this is combined with Earth fixed tracking area, it will create fluctuation at the border areas of these Earth fixed tracking areas, as depicted in Figure 1.

Figure 1 Tracking area update for Earth moving beams with hard TAI update

Soft TAI update requires the network to broadcast more than one TAI for a cell and PLMN. The cell adds the new TAC in its system information in addition to the old and removes the old a bit later. If there is a chain of TAs, the TA list adds one TA more and removes one old while the cell sweeps the ground. This also reduces the amount of TAUs for UEs that happen to be located at the border area, as depicted in Figure 2.

Figure 2 Tracking area update for Earth moving beams with soft TAI update

In [1][2][3][4][5][6][7], the pros and cons of the two approached are illustrated. In a summary, Soft TAI update solution can mitigate the increasing TAI update signaling with the problem of paging overhead, while the hard TAI update option has the problem of signaling overhead and boundary fluctuation, as shown in figure3. From another perspective, we could say that this issue mainly requires a compromise between paging load and signaling overhead actually.

# Figure 3: TAC fluctuation at the border area

To progress the topic, this document provides questions with respect to the two approaches and companies are requested to provide their views on those.

2	Discussion

As mentioned in [1][2][3][4][5], although there is the concern about the increasing paging load resulting from a cell broadcasting multiple TAIs. However, the satellite/cell will remove the old TAC once the satellite/cell cover the most geography area corresponding to the new TAC after it adds the new TAC in its system information in addition to the old one. And considering the short duration for the transition stage of the satellite passing over the boundary area, the increasing paging signaling only occurs in a short period. On the other hand, as VDF mentioned, how the MNOs and Satellite service providers jointly arrive at a suitable and practical Tracking Areas to suit their networks’ needs is up to deployment and implementation, not a standards issue.

Question 1: Do companies assumes that in soft TAI update approach, the multiple TACs only broadcasted just during the transition stage of the satellite passing over the boundary area, or up to deployment and implementation? This means the paging load caused by multiple TACs broadcasted in one cell is limited and is under controlled.

| Company | Yes/No | Comment / alternative proposal |
| --- | --- | --- |

Regarding hard TAI update, since only one TAC per PLMN ID is broadcasted in SI, when gNB switches its TAC during satellite’s cell sweep on the ground, a stationary UE may need to perform TAU because of the different TAs at different timings, which results in signalling-intensive situation and increases the UE’s power consumption.

Question 2: Do companies agree that hard TAI update will result in frequent TAU signalling overhead and increases the UE’s power consumption?

| Company | Yes/No | Comment / alternative proposal |
| --- | --- | --- |

In section 1, the pros and cons of the two approaches illustrated in [1][2][3][4][5][6][7] was summarized. Based on the comparison, companies are invited to answer this Question:

Question 3: Do companies have a preference on supporting either hard or soft TAI update, or both?

| Company | Yes/No | Comment / alternative proposal |
| --- | --- | --- |

Additionally, in NTN, to avoid having TAU performed frequently by the UE triggered by the satellite motion, as indicated in TR 38.821, the tracking area may be designed to be fixed on ground. As mentioned in [1][3][6], a relative issue is how does the UE determine the TA it is currently located, which can be categorized into two groups:

- current TA is determined based on the broadcasted radio coverage,

- current TA is determined based on UE’s geographical location and satellite’s ephemeris’ information.

Question 4: which option is companies’ preference, option 1 or option 2?

| Company | Yes/No | Comment / alternative proposal |
| --- | --- | --- |

As mentioned in [3][6], in the soft TAI update scheme, it is beneficial to allow gNB to not trigger the SI update when the list of broadcast TACs changed, since it is not necessary for the UE staying in the same cell to know whether a new TAC is added or an existing TAC is deleted from the TAC list due to the satellite movement. UE will have to check the TAC list only when it just enters into a new cell. Moreover, if the option 2 in question 4 is adopted, it is straightforward that there is no necessity of SI update due to satellite motion.

Question 5: do companies agree that such kind of TAC change in SI caused by satellite motion will not trigger paging for system information change?

| Company | Yes/No | Comment / alternative proposal |
| --- | --- | --- |

In [5], the paper have pointed out that soft TAC update procedure has some NAS impact. For example, AS reports multiple TACs per PLMN to NAS for cell selection and NAS has to determine whether to trigger registration update based on reported multiple TACs per PLMN (unlike today where only one reported TAC per PLMN is used). The RAN can determine and report one TAC for a UE in the NG ULI as for TN.

- Question 6: Do compaines agree that the soft TAC update has NAS impact and LS to CT1 and SA2 asking for any concern on soft TAC update mechanism is needed.

| Company | Yes/No | Comment / alternative proposal |
| --- | --- | --- |

3	Conclusion

TBD

4	References

- R2-2001627	Impact of CG/SPS with periodicities non dividing HF length	Sequans Communications3GPP TR 38.821 Solutions for NR to support non-terrestrial networks (NTN), version 16.0.0

- R2-209820, [POST111e][910][NTN] Impacts of earth fixed and moving beams (Ericsson)

- R2-2101607	Considerations on Soft TAI Update	CMCC	discussion	Rel-17

- R2-2100259	Improving Tracking Area Updates in NR-NTN	MediaTek Inc.	discussion

- R2-2100742	TAC update procedure	Qualcomm Incorporated	discussion	Rel-17

- R2-2100820	Fixed Tracking Area and the Tracking Area Code in NTN	PANASONIC R&D Center Germany	discussion	R2-2009120

- R2-2101406	TAI update for earth moving cell	NEC Telecom MODUS Ltd.	discussion

6 Proposals in summary contribution